3GPP TSG-RAN5 Meeting #90-e	R5-210932

# Electronic Meeting, 22th Feb– 5th Mar 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1169 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating Additional spurious emisisons testing for SUL band n83 and n84 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_lic_bands_BW_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2020-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NS_18 is defined for n83 and NS_48 NS_49 are defined for n84 in R17 core spec 38.101-1. The additional spurious emisisons for SUL testing need to be updated in aligment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating 6.5C.3.3 for NS_18 for band n83 and NS_48 NS_49 for n84. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case 6.5C.3.3 can’t be tested for SUL band n83 and n84. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5C.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Table 6.5.3.3.4.1-23 for NS_49 in 6.5.3.3 is introduced in R5-210741 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

6.5C.3.3	Additional spurious emissions for SUL

6.5C.3.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.5C.3.3.2	Test applicability

This test applies to all types of NR UE release 15 and forward that support SUL operating on the SUL bands.

6.5C.3.3.3	Minimum conformance requirements

The additional spurious emission requirements specified in 6.5.3.3.3 apply to the UE operating on SUL bands.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.5.3.3

6.5C.3.3.4	Test description

Same test description as specified in clause 6.5.3.3.4 with following exceptions:

-	Instead of table 5.3.5-1  use Table 5.5C-1

-	For NS_05, instead of table 6.5.3.3.4.4-1 use Table 6.5C.3.3.4-1

-	For NS_43, instead of table 6.5.3.3.4.1-5 use Table 6.5C.3.3.4-2

-	For NS_18, instead of table 6.5.3.3.4.1-3 use Table 6.2C.3.3.4-3

-	For NS_48, instead of table 6.5.3.3.4.1-5 use Table 6.2C.3.3.4-4

-	For NS_49, instead of table 6.5.3.3.4.1-23 use Table 6.2C.3.3.4-5

<Unchanged Text Skipped>

Table 6.5C.3.3.4-2: Test Configuration Table (network signalled value “NS_43”)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Use carrier center frequency (Fc) as specified in test parameters for SUL carrier Mid range for Non-SUL carrier |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 5 MHz, 10 MHz, 15 MHz for SUL carrier Lowest for Non-SUL carrier |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15 kHz for both SUL carrier and Non-SUL carrier |  |
| Additional Spurious for SUL test parameters for NS_43 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | SUL Ch BW (MHz) | Downlink Configuration | Uplink Configuration | SUL Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | 910 | 10 | N/A | N/A | DFT-s--OFDM | PI/2 BPSK |  | Outer_Full (A2) |
| 2 | 907.5 | 15 |  |  |  | PI/2 BPSK |  | Edge_1RB_Left (A6) |
| 3 | 907.5 | 15 |  |  |  | PI/2 BPSK |  | Outer_Full (A6) |
| 4 | 902.5 | 5 |  |  |  | QPSK |  | Outer_Full (A1) |
| 5 | 910 | 10 |  |  |  | QPSK |  | Outer_Full (A4) |
| 6 | 907.5 | 15 |  |  |  | QPSK |  | Edge_1RB_Left (A6) |
| 7 | 907.5 | 15 |  |  |  | QPSK |  | Outer_Full (A6) |
| 8 | 910 | 10 |  |  |  | 16 QAM |  | Outer_Full (A5) |
| 9 | 907.5 | 15 |  |  |  | 16 QAM |  | Edge_1RB_Left (A6) |
| 10 | 907.5 | 15 |  |  |  | 16 QAM |  | Outer_Full (A6) |
| 11 | 910 | 10 |  |  |  | 64 QAM |  | Outer_Full (A3) |
| 12 | 907.5 | 15 |  |  |  | 64 QAM |  | Edge_1RB_Left (A6) |
| 13 | 907.5 | 15 |  |  |  | 64 QAM |  | Outer_Full (A6) |
| 14 | 907.5 | 15 |  |  |  | 256 QAM |  | Edge_1RB_Left (A6) |
| 15 | 907.5 | 15 |  |  |  | 256 QAM |  | Outer_Full (A6) |
| 16 | 902.5 | 5 |  |  | CP-OFDM | QPSK |  | Outer_Full (A1) |
| 17 | 910 | 10 |  |  |  | QPSK |  | Outer_Full (A5) |
| 18 | 907.5 | 15 |  |  |  | QPSK |  | Edge_1RB_Left (A6) |
| 19 | 907.5 | 15 |  |  |  | QPSK |  | Outer_Full (A6) |
| 20 | 902.5 | 5 |  |  |  | 16 QAM |  | Outer_Full (A1) |
| 21 | 910 | 10 |  |  |  | 16 QAM |  | Outer_Full (A5) |
| 22 | 907.5 | 15 |  |  |  | 16 QAM |  | Edge_1RB_Left (A6) |
| 23 | 907.5 | 15 |  |  |  | 16 QAM |  | Outer_Full (A6) |
| 24 | 910 | 10 |  |  |  | 64 QAM |  | Outer_Full (A3) |
| 25 | 907.5 | 15 |  |  |  | 64 QAM |  | Edge_1RB_Left (A6) |
| 26 | 907.5 | 15 |  |  |  | 64 QAM |  | Outer_Full (A6) |
| 27 | 907.5 | 15 |  |  |  | 256 QAM |  | Edge_1RB_Left (A6) |
| 28 | 907.5 | 15 |  |  |  | 256 QAM |  | Outer_Full (A6) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 2: AMPR requirement for NS_43 is specified in 6.2.3.3.6. |  |  |  |  |  |  |  |  |

Table 6.5C.3.3.4-3: Test Configuration Table (network signalled value “NS_18”)

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | Low range for SUL carrier Mid range for Non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  | 5MHz 10MHz 20MHz 30MHz for SUL carrier Lowest for Non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  | 15 kHz for all carriers |  |  |
| A-MPR test parameters for NS_18 |  |  |  |  |  |  |
| Test ID | ChBw SCS | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) | Comment |
| 1 | 5MHz, 10MHz, 20MHz | N/A | DFT-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 2 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 3 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 4 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 5 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 6 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 7 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 8 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 9 |  |  | CP-s OFDM | QPSK | Edge_1RB_Left | A1, A2 |
| 10 |  |  |  | QPSK | Outer_Full | A1, A2 |
| 11 |  |  |  | 16 QAM | Edge_1RB_Left | A1, A2 |
| 12 |  |  |  | 16 QAM | Outer_Full | A1, A2 |
| 13 |  |  |  | 64 QAM | Edge_1RB_Left | A1, A2 |
| 14 |  |  |  | 64 QAM | Outer_Full | A1, A2 |
| 15 |  |  |  | 256 QAM | Edge_1RB_Left | A1, A2 |
| 16 |  |  |  | 256 QAM | Outer_Full | A1, A2 |
| 17 | 30MHz |  | DFT-s OFDM | 256 QAM | 81@70 | A3 |
| 18 |  |  |  | 256 QAM | Outer_Full | A4 |
| 19 |  |  | CP-s OFDM | 256 QAM | 87@73 | A3 |
| 20 |  |  |  | 256 QAM | Outer_Full | A4 |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |

Table 6.5C.3.3.4-4: Test Configuration Table (network signalled value “NS_48”)

| Initial Conditions |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | 15kHz for all carriers |  |  |
| A-MPR test parameters for NS_48 |  |  |  |  |  |  |  |  |
| Test ID | Fc (MHz) | Ch BW (MHz) | Uplink Configuration | Downlink Configuration | SUL Configuration |  |  |  |
|  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |
| 1 | Default | 25 | N/A. | N/A | DFT-s-OFDM |  | QPSK | Outer_Full (A3) |
| 2 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 3 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 4 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 5 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 6 | Default | 40 |  |  |  |  | QPSK | 90@0 (A3) |
| 7 | Default | 40 |  |  |  |  | QPSK | 150@0 (A4) |
| 8 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 9 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 10 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 11 | Default | 50 |  |  |  |  | QPSK | 32@0 (A2) |
| 12 | Default | 50 |  |  |  |  | QPSK | 108@0 (A4) |
| 13 | Default | 50 |  |  |  |  | QPSK | 225@0 (A2) |
| 14 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 15 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 16 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 17 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 18 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 19 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 20 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 21 | Default | 40 |  |  |  |  | 256 QAM | 90@0 (A3) |
| 22 | Default | 40 |  |  |  |  | 256 QAM | 150@0 (A4) |
| 23 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 24 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 25 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 26 | Default | 50 |  |  |  |  | 256 QAM | 32@0 (A2) |
| 27 | Default | 50 |  |  |  |  | 256 QAM | 108@0 (A4) |
| 28 | Default | 50 |  |  |  |  | 256 QAM | 225@0 (A2) |
| 29 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 30 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 31 | Default | 25 |  |  | CP-OFDM |  | QPSK | Outer_Full (A3) |
| 32 | Default | 25 |  |  |  |  | QPSK | Edge_1RB_Right (A3) |
| 33 | Default | 30 |  |  |  |  | QPSK | Outer_Full (A3) |
| 34 | Default | 30 |  |  |  |  | QPSK | Edge_1RB_Right (A5) |
| 35 | Default | 40 |  |  |  |  | QPSK | 16@0 (A2) |
| 36 | Default | 40 |  |  |  |  | QPSK | 95@0 (A3) |
| 37 | Default | 40 |  |  |  |  | QPSK | 152@0 (A4) |
| 38 | Default | 40 |  |  |  |  | QPSK | 192@0 (A2) |
| 39 | Default | 40 |  |  |  |  | QPSK | 5@187 (A3) |
| 40 | Default | 40 |  |  |  |  | QPSK | Outer_Full (A1) |
| 41 | Default | 50 |  |  |  |  | QPSK | 34@0 (A2) |
| 42 | Default | 50 |  |  |  |  | QPSK | 115@0 (A4) |
| 43 | Default | 50 |  |  |  |  | QPSK | 228@0 (A2) |
| 44 | Default | 50 |  |  |  |  | QPSK | 5@223 (A5) |
| 45 | Default | 50 |  |  |  |  | QPSK | Outer_Full (A1) |
| 46 | Default | 25 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 47 | Default | 25 |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |
| 48 | Default | 30 |  |  |  |  | 256 QAM | Outer_Full (A3) |
| 49 | Default | 30 |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |
| 50 | Default | 40 |  |  |  |  | 256 QAM | 16@0 (A2) |
| 51 | Default | 40 |  |  |  |  | 256 QAM | 95@0 (A3) |
| 52 | Default | 40 |  |  |  |  | 256 QAM | 152@0 (A4) |
| 53 | Default | 40 |  |  |  |  | 256 QAM | 192@0 (A2) |
| 54 | Default | 40 |  |  |  |  | 256 QAM | 5@187 (A3) |
| 55 | Default | 40 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| 56 | Default | 50 |  |  |  |  | 256 QAM | 34@0 (A2) |
| 57 | Default | 50 |  |  |  |  | 256 QAM | 115@0 (A4) |
| 58 | Default | 50 |  |  |  |  | 256 QAM | 228@0 (A2) |
| 59 | Default | 50 |  |  |  |  | 256 QAM | 5@223 (A5) |
| 60 | Default | 50 |  |  |  |  | 256 QAM | Outer_Full (A1) |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |  |

Table 6.5C.3.3.4-5: Test Configuration Table (network signalled value “NS_49”)

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |  |  |  |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range for SUL carrier Mid range for non-SUL carrier |  |  |  |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | 25 MHz, 30MHz, 40MHz, 50MHz for SUL carrier Lowest for non-SUL carrier |  |  |  |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | 15 kHz for all carriers |  |  |  |  |  |  |
| A-MPR test parameters for NS_49 |  |  |  |  |  |  |  |
| Test ID |  | Fc (MHz) |  | Ch BW (MHz) |  | Downlink Configuration |  |  | Uplink Configuration |  | SUL Configuration |  |  |  |
|  |  |  |  |  |  |  |  |  |  | Modulation (Note 2) |  |  | RB allocation (Note 1) |  |  |
| 1 | Default |  | 25 |  | N/A. |  |  | N/A |  | DFT-s-OFDM |  | QPSK | Outer_Full (A3) |  |  |
| 2 | Default |  | 25 |  |  |  |  |  |  |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 3 | Default |  | 25 |  |  |  |  |  |  |  |  | QPSK | Edge_1RB_Left (A3) |  |  |
| 4 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 20@0 (A1) |  |  |
| 5 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 36@0 (A5) |  |  |
| 6 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 80@0 (A3) |  |  |
| 7 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 120@0 (A4) |  |  |
| 8 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | Outer_Full (A2) |  |  |
| 9 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 10 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 40@0 (A1) |  |  |
| 11 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 5@53 (A5) |  |  |
| 12 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 100@0 (A4) |  |  |
| 13 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 159@33 (A2) |  |  |
| 14 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 5@187 (A5) |  |  |
| 15 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 192@0 (A1) |  |  |
| 16 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | Outer_Full (A1) |  |  |
| 17 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 5@75 (A5) |  |  |
| 18 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 5@215 (A5) |  |  |
| 19 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 175@45 (A2) |  |  |
| 20 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 220@0 (A1) |  |  |
| 21 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | Outer_Full (A1) |  |  |
| 22 | Default |  | 25 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 23 | Default |  | 25 |  |  |  |  |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 24 | Default |  | 25 |  |  |  |  |  |  |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 25 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 20@0 (A1) |  |  |
| 26 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 36@0 (A5) |  |  |
| 27 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 80@0 (A3) |  |  |
| 28 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 120@0 (A4) |  |  |
| 29 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 30 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 31 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 40@0 (A1) |  |  |
| 32 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 5@53 (A5) |  |  |
| 33 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 100@0 (A4) |  |  |
| 34 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 159@33 (A2) |  |  |
| 35 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 5@187 (A5) |  |  |
| 36 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 192@0 (A1) |  |  |
| 37 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 38 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 5@75 (A5) |  |  |
| 39 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 5@215 (A5) |  |  |
| 40 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 175@45 (A2) |  |  |
| 41 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 220@0 (A1) |  |  |
| 42 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 43 | Default |  | 25 |  |  |  |  |  |  | CP-OFDM |  | QPSK | Outer_Full (A3) |  |  |
| 44 | Default |  | 25 |  |  |  |  |  |  |  |  | QPSK | Edge_1RB_Right (A3) |  |  |
| 45 | Default |  | 25 |  |  |  |  |  |  |  |  | QPSK | Edge_1RB_Left (A3) |  |  |
| 46 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 20@0 (A1) |  |  |
| 47 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 36@0 (A5) |  |  |
| 48 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 80@0 (A3) |  |  |
| 49 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | 120@0 (A4) |  |  |
| 50 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | Outer_Full (A2) |  |  |
| 51 | Default |  | 30 |  |  |  |  |  |  |  |  | QPSK | Edge_1RB_Right (A5) |  |  |
| 52 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 40@0 (A1) |  |  |
| 53 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 5@53 (A5) |  |  |
| 54 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 100@0 (A4) |  |  |
| 55 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 150@33 (A2) |  |  |
| 56 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 5@187 (A5) |  |  |
| 57 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | 192@0 (A1) |  |  |
| 58 | Default |  | 40 |  |  |  |  |  |  |  |  | QPSK | Outer_Full (A1) |  |  |
| 59 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 5@75 (A5) |  |  |
| 60 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 5@215 (A5) |  |  |
| 61 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 175@45 (A2) |  |  |
| 62 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | 216@0 (A1) |  |  |
| 63 | Default |  | 50 |  |  |  |  |  |  |  |  | QPSK | Outer_Full (A1) |  |  |
| 64 | Default |  | 25 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A3) |  |  |
| 65 | Default |  | 25 |  |  |  |  |  |  |  |  | 256 QAM | Edge_1RB_Right (A3) |  |  |
| 66 | Default |  | 25 |  |  |  |  |  |  |  |  | 256 QAM | Edge_1RB_Left (A3) |  |  |
| 67 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 20@0 (A1) |  |  |
| 68 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 36@0 (A5) |  |  |
| 69 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 80@0 (A3) |  |  |
| 70 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | 120@0 (A4) |  |  |
| 71 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A2) |  |  |
| 72 | Default |  | 30 |  |  |  |  |  |  |  |  | 256 QAM | Edge_1RB_Right (A5) |  |  |
| 73 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 40@0 (A1) |  |  |
| 74 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 5@53 (A5) |  |  |
| 75 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 100@0 (A4) |  |  |
| 76 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 150@33 (A2) |  |  |
| 77 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 5@187 (A5) |  |  |
| 78 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | 192@0 (A1) |  |  |
| 79 | Default |  | 40 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A1) |  |  |
| 80 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 5@75 (A5) |  |  |
| 81 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 5@215 (A5) |  |  |
| 82 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 175@45 (A2) |  |  |
| 83 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | 216@0 (A1) |  |  |
| 84 | Default |  | 50 |  |  |  |  |  |  |  |  | 256 QAM | Outer_Full (A1) |  |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. |  |  |  |  |  |  |  |

-	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.1.4 for TE diagram and section A.3.2 for UE diagram.

-	The parameter setting for the cell are set up according to the TS 38.508-1 [5] subclause 4.4.3

-	Downlink signals are initially setup according to Annex C.0, C.1, C.2 and uplink signals according to Annex G.0, G.1, G.2, and G.3.0 with consideration of supplementary uplink physical channels.

<End of modified section 1>