3GPP TSG RAN WG5 Meeting #90-e	R5-211348

# Electronic Meeting, 22nd February - 5th March 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5000 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial updates of TS 36.523-1 Section 9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2021-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The way to represent Void test cases is inconsistent within 35.523-1 but also between 36.523-1 and 36.523-2. In addition from the problem from standards editorial point of view they also represent inconsistency with the titles used in 36.523-2 where the issues do not exist. Other minor editorial issues have been found around the places with the issues described above (i.e. the entire spec was not deliberately searched for editorial issues): The style applied to a TP number/header is incorrect, syntactically incorrect TPs (PRD13) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Void test cases titles which have more than the word Void have been corrected with the additional text moved to Notes under the title. Missing Voids have been added. Style on one TP number/header has been changed to H6. Test Purposes text has been modified to follow the agreed RAN5 way of specifying a TP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Difficulties finding exact matches between tests definition and test applicability. Poor quality of 3GPP specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.2.1, 9.1.2.3.1, 9.2.1.1.21, 9.2.2.2.3 to 9.2.2.2.13, 9.3.1.8 to 9.3.1.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The present CR is an update of R5-210147 followed by one intermediate revision r1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 9.1	EMM common procedures

### 9.1.1	Void

#### 9.1.1.1	Void

#### 9.1.1.2	Void

### 9.1.2	Authentication procedure

#### 9.1.2.1	Void

NOTE:	The present test (Authentication accepted) is superseded by test case 9.1.2.3 (i.e. all requirements which the present test verified are verified in test case 9.1.2.3).

#### 9.1.2.2	Void

#### 9.1.2.3	Authentication not accepted by the network / GUTI used / Authentication reject and re-authentication

9.1.2.3.1	Test Purpose (TP)

(1)

with { UE having sent an initial NAS message with type of identity GUTI }

ensure that {

when { UE receives an AUTHENTICATION REJECT message as a result of failure of an Authentication procedure initiated by the network  }

then { UE shall set the update status to EU3 ROAMING NOT ALLOWED, delete the stored GUTI, TAI list, last visited registered TAI and KSIASME and enter state EMM-DEREGISTERED }

}

(2)

with { UE having a NAS signalling connection existing }

ensure that {

when { UE receives an AUTHENTICATION REQUEST message }

then { UE responds with a correct AUTHENTICATION RESPONSE message and establishes correct EPS security context }

}

[Text skipped here]

##### 9.2.1.1.21	Void

NOTE:	The present test (Attach / Abnormal case / Success after several attempts due to no network response) is superseded by test case 9.2.1.1.22 (i.e. all requirements which the present test verified are verified in test case 9.2.1.1.22).

[Text skipped here]

##### 9.2.2.2.3	Void

##### 9.2.2.2.4	Void

##### 9.2.2.2.5	Void

##### 9.2.2.2.6	Void

##### 9.2.2.2.7	Void

##### 9.2.2.2.8	Void

##### 9.2.2.2.9	Void

##### 9.2.2.2.10	Void

##### 9.2.2.2.11	Void

##### 9.2.2.2.12	Void

##### 9.2.2.2.13	Void

#### 9.3.1.8	Void

#### 9.3.1.9	Void

#### 9.3.1.10	Void

#### 9.3.1.11	Void

#### 9.3.1.12	Void

[End of changes]