3GPP TSG RAN meeting #91e										RP-210567

Electronic Meeting, Mach 22 - 26, 2021

## Status Report to TSG

Agenda item:	9.7.9

| WI / SI Name | Further Multi-RAT Dual-Connectivity enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | LTE_NR_DC_enh2 |  |  |  |  |
| Unique ID | 860049 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201040 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: - | Core part: 03/2022 | Performance part: 09/2022 | Testing part: - |  |
| Overall Completion level | Study Item: - | Core part: 40% | Performance Part: 0% | Testing part: - |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | David Lecompte |
|  | Company | Huawei |
|  | Email | david.lecompte@huawei.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#102-E agreements:

Working Assumption:

- At least for the case of known cell, temporary RS is supported to expedite the activation process during the SCell activation procedure for efficient SCell activation for both FR1 and FR2:

  - The temporary RS should provide at least the functionalities of AGC settling and time/frequency tracking during SCell activation procedure.

  - FFS potential functionalities of CSI measurement/acquisition and cell search

Agreements:

- TRS is selected as temporary RS for Scell activation

  - If more functionalities are confirmed to be supported by temporary RS, other RS candidates, e.g. aperiodic CSI-RS, P/SP-CSI RS, SRS and RS based on SSS/PSS, are not precluded.

  - The TRS should be triggered by DCI or MAC-CE. FFS which exact triggering command.

Agreements:

- UEs measure the triggered temporary RS during Scell activation procedure no earlier than a slot m:

  - FFS timeline values m which may need coordination with RAN4.

  - FFS if the triggered temporary RS can be associated with a BWP, then the measurement above is independent of the activation state of the BWP.

Agreements:

- Companies are encouraged to provide design details of temporary RS next meeting, at least including:

  - TRS structure, e.g. whether to fully reuse existing Rel-15/16 TRS structure and configuration restriction (refer to S5.1.6.1.1 of TS 38.214), or any modification

  - QCL information, if any

  - Triggering command: DCI format/fields or MAC-CE fields

  - Triggering timeline/scheduling offset

RAN1#103-E agreements:

Working Assumption:

- With respect to efficient SCell activation, reuse existing Rel-15/16 TRS structure for temporary RS

- FFS: how many burst/symbols are required for both AGC settling and Time/Frequency tracking for different cases, e.g. FR1 and FR2, known and unknown SCell

  - A burst of temporary RS is notated as in S5.1.6.1.1 of TS 38.214

    - “2-slot with four CSI-RSs resources (4 samples)” for FR1

    - either “1-slot with two CSI-RSs resources (2 samples)” or “2-slot with four CSI-RSs resources (4 samples)” for FR2

- The working assumption can be confirmed after RAN4 check. (A LS for such request is planned).

Agreements:

For efficient SCell activation, discuss and agree from the following alternatives at RAN1#104-e

- Alt 1:  the trigger of temporary RS is integrated into a single triggering signalling with the trigger of SCell activation transmitted on an activated cell.

  - FFS detailed design of this integrated triggering signalling.

  - Potential examples of single triggering signalling for further discussions

    - A PDSCH TB, e.g. containing two respective MAC-CEs for both triggers, one MAC-CE for both triggers

    - A DCI for both triggers

    - A PDSCH TB and its scheduling DL grant, e.g. MAC-CE for activation and DL grant for temporary RS

    - A DL grant and a UL grant received in the same slot/OFDM symbols of PDCCH where the DL grant is scheduling a MAC-CE for SCell activation and the UL grant is triggering the RS.

    - Rel-15/16 SCell activation MAC-CE and a specific configuration of temporary RS being implicitly triggered as well

- Alt2: Triggering of temporary RS separately from SCell activation command is not precluded and both ‘separate’ triggers (examples below) and ‘integrated’ triggers (examples in Alt 1) are considered for SCell activation

  - FFS detailed design of separate triggering signaling.

  - Potential examples of separate triggering signaling for further discussions

    - Rel-15/16 SCell activation MAC-CE and Rel 15/16 DCI triggering

    - Rel-15/16 SCell activation MAC-CE and new DCI triggering for temporary RS

- Note: temporary RS should be triggered by DCI or MAC-CE.

- Note: the final mechanism of trigger signaling targets at applicability to one or more SCell activation.

- FFS handling of  SCell activation by existing Rel15/16 CA activation command when temporary RS is configured and triggered/not triggered

Draft LS (R1-2009786) to RAN4 is approved. Final LS in R1-2009798.

RAN1#104-E agreements:

Working Assumptions:

For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell

- FFS: QCL type

- FFS: the case of unknown SCell

- FFS: other QCL source, e.g. the SSB/P-TRS of another active cell

Agreements:

For efficient activation of SCells, down select at least one option from below:

- Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including timeline design for receiving temporary RS

- Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s)

  - Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS

  - FFS: The same DCI for SCell deactivation

- Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline

  - Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS

- Note: Companies are encouraged to provide complete solutions for fast SCell activation.

- Note: the previous agreement on the definitions of Alt 1 and Alt 2 is still effective

#### 2.1.2	Remaining Open issues

Further details of TRS mechanism for efficient SCell activation.

Reduction of SCell activation time for the case of unknown cell.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#111-E agreements:

Efficient activation/de-activation mechanism for one SCG and SCells:

- FFS how signalling and inter-node interaction works at activation deactivation (e.g. MN triggered, SN triggered, UE triggered, signalling mechanism, which node is in control etc)

- FFS if for deactived SCG, the UE stop monitoring PDCCH for PSCell and SCells of the SCG

- FFS if For the PSCell in deactivated SCG, the UE performs CSI/RRM measurement and report; AGC; beam management; RLM

Conditional PSCell change/addition:

- R2 assumes that the work Will follow what is in the WID, and initially focus on CPA and Inter-SN CPC

- R2 assumes for now that LTE SCG is not included.

RAN2#112-E agreements:

Efficient activation/de-activation mechanism for one SCG and SCells:

- The work will focus on a single deactivated SCG.

- SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell addition/change, RRC resume or HO

  - After the session closed, Samsung commented offline that the agreement 1 is not clear and its intention was as follows:

    - SCG activation state (activated/deactivated) can be configured at PSCell addition/change, RRC resume or HO.

  - The WI rapporteur (Huawei) indicated this was the intention, so this wording is recommended to clarify the meaning of the original agreement 1 above.

- Continue RAN2 work with the assumption that when the SCG is deactivated, the UE does not monitor PDCCH on the PSCell. This assumption can be reconsidered if issues are found.

- While the SCG is deactivated, PSCell mobility is supported.

  - When the SCG is in deactivated state, the UE can receive an SCG RRCReconfiguration message embedded in an MCG RRC(Connection)Reconfiguration message on SRB1, like when the SCG is activated, and then the UE

    - processes the SCG RRCReconfiguration message according to Rel-15/16 procedures (FFS if any restriction/difference)

    - sends an SCG RRCReconfigurationComplete message in the MCG RRC(Connection)ReconfigurationComplete message according to Rel-15/16 procedures

  - The SCG RRCReconfiguration can change the PSCell.  FFS if the UE does RACH towards the target PSCell, in that case.

- While the SCG is deactivated:

  - there can be SCG SCells in deactivated state

  - there cannot be SCG SCells in activated state

  - it is FFS whether there can be SCells in SCG dormant state.

  - FFS whether SCell can be added/reconfigured/released while the SCG is deactivated or this can be done only at SCG activation or after SCG activation.

- MN- and SN-configured measurements are supported for deactivated SCG

  - As a baseline, MN-configured RRM measurement/reporting procedures do not depend on the SCG activation state (deactivated or activated). Further optimisations are not precluded.

  - When the SCG is in deactivated state, the UE sends MeasurementReport messages for measurement results of SN-configured measurements embedded in the E-UTRA (if the MCG is EUTRA) or in the NR (if the MCG is NR) ULInformationTransferMRDC message via SRB1

  - FFS: Details on the performed measurements (e.g. all SN configured measurements or subset based on certain criteria, restrictions on inter-frequency/RAT)

    - It is FFS whether the network can configure the UE stop certain configured RRM measurements while the SCG is deactivated, or can release certain RRM measurements at SCG deactivation.

    - Relaxation of RRM measurement requirements (as compared with non-DRX activated cell requirements) while the SCG is deactivated is FFS.

- RAN2 assumes that UE will not perform SRS transmission while the SCG is deactivated. This assumption can be reconsidered if issues are found.

Conditional PSCell change/addition:

- General/procedure

  - Maintain Rel-15 principle that only one PScell is active at a time even with conditional PScell addition/change.

  - Usage of CPAC is decided by the network. The UE evaluates when the condition is valid.

  - The baseline operation for CPAC procedure assumes the RRC Reconfiguration message contains SCG addition/change triggering condition(s) and the RRC configuration(s) for candidate target PSCells. The UE accesses the prepared PSCell when the relevant condition is met.

  - CPAC execution condition and/or candidate PSCell configuration can be updated by modifying the existing CPAC configuration.

  - Support configuration of one or more candidate cells for CPAC.

  - UE is not required to continue evaluating the triggering condition of other candidate PSCell(s) during CPC/CPA execution.

  - For FR1 and FR2, leave it up to UE implementation to select the candidate PSCell if more than one candidate cell meets the triggering condition. UE may consider beam information in this.

  - No additional optimizations with multi-beam operation are introduced to improve RACH performance for CPAC completion with multi-beam operation.

- Trigger/ condition related

  - For conditional PSCell addition, the MN decides on the conditional PSCell addition execution condition. FFS for PSCell Change.

  - The execution condition for CPAC is defined by a measurement identity which identifies a measurement configuration.

  - For conditional PSCell change, A3/A5 execution condition should be supported while for conditional PSCell addition, A4/B1 like execution condition should be supported.

  - Allow having multiple triggering conditions (using “and”) for CPAC execution of a single candidate cell. Only single RS type per CPAC candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously.

  - Cell level quality is used as baseline for CPAC execution condition;

  - Only single RS type (SSB or CSI-RS) per candidate PSCell is supported for PSCell change.

  - TTT is supported for CPAC execution condition (as per legacy configuration)

- Signalling related

  - Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CPAC configuration to UE following Rel-16 signalling.

  - Multiple candidate PSCells can be sent in either one or multiple RRC messages.

  - As part of the CPAC configuration to be sent to the UE, the RRC container is used to carry candidate PSCell configuration, and the MN is not allowed to alter any content of the configuration from the PSCell. Moreover, in case of SN change, source SN is not allowed to alter any content of the configuration from the target SN. FFS on which RRC format is used (can be considered in stage-3)

  - For conditional PSCell addition, the MN transmits the final RRCReconfiguration/ RRCConnectionReconfiguration message to the UE. FFS how the encapsulation is done exactly (can be considered in Stage-3).

  - In MN initiated inter-SN CPC and CPA, the MN is not required to indicate the execution condition(s) to other involved entities (e.g. target SN, source SN).

  - For CPA and MN initiated Inter-SN CPC, the MN generates and transmits the conditional configuration message (i.e. RRCReconfiguration/RRCConnectionReconfiguration message) to the UE.  The RRCReconfiguration provided by the candidate PSCell(s) is encapsulated in the final conditional reconfiguration message to the UE. The MN is not allowed to alter the RRCReconfiguration provided by the candidate PSCell(s).

  - For the generation of conditional reconfiguration for SN initiated inter-SN conditional PSCell change, the MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s).

  - Send LS to RAN3 informing

    - RAN2 agreements

    - RAN2 findings on the limitation of providing addition/modification of multiple CPC candidate cells in inter-node RAN3 message (i.e. XnAP fields, not in RRC INM)

- FFS issues

  - FFS whether we need coordination on exact execution conditions or just measurements.

  - FFS whether source or target SN knows the condition

  - FFS in which exact cases the condition needs to be indicated

  - FFS how many candidate cells (UE and network impacts should be clarified). FFS whether the number of candidate cells for CPAC different from that of CHO.

  - FFS on UE capability for triggering quantities

RAN2#113-E agreements:

Efficient activation/de-activation mechanism for one SCG and SCells:

- SCG activation can be requested by MN/SN/UE. FFS on how to accept/reject the procedure. FFS which signalling is used.

- SCG deactivation can be requested by MN/SN. FFS whether UE can request deactivation. FFS on how to accept/reject the procedure. FFS which signalling is used.

- RRC signalling is defined for the interaction between UE/MN and MN/SN in SCG activation/deactivation. FFS if lower-layer signalling is needed.

- Confirm that there is no PUSCH transmission on deactivated SCG. FFS if any other UL is allowed towards SCG.

- Confirm that there is no PDCCH monitoring on PSCell of the deactivated SCG.

- Confirm that there is no support of SCell dormancy for SCG SCells within a deactivated SCG.

- NW-triggered SCG activation is indicated to the UE via the MCG.

- NW-triggered SCG deactivation can be indicated to the UE via the MCG. FFS via SCG.

- The UE behaviour when the SCG activation is indicated to the UE via the MCG is one or more of the following options:

  - option 1)	similar to reconfiguration with sync, i.e. the UE always initiates random access to the PSCell.

  - option 2)	in certain cases:

    - the UE does not initiate random access and monitors PDCCH on the PSCell (at the latest after the specified processing time).

    - the SCG can schedule data transmission on the PDCCH

    - The UE decides not to perform random access (one option to be selected):

      - option 2a) if the TA timer is still running and possibly other conditions (FFS how TAT starts)

      - option 2b) based on the contents of the SCG activation indication

      - FFS for option 2a): in the SCG deactivated state, the UE monitors some DL beams (FFS if the same as BFD or RLM) and, if the UE sees that the beams are not good enough (details FFS), the UE either (one of the options to be selected):

        - will perform random access upon reception of the next SCG activation indication from the MCG

        - reports measurement results (details FFS) via the MCG and wait for reconfiguration.

- Further discuss the format and content of the SCG activation indication from the MCG to the UE after there is more progress on solution 2.

- Continue to discuss whether some kind of beam monitoring (similar to RLM/BFD) should be supported when the SCG is deactivated. FFS if this only applies to when TAT is running.

- Clarify the meaning of "the UE maintains DL sync while the SCG is deactivated" (e.g. whether that is a consequence of doing RRM measurements of the PSCell or something more is needed).

- Further discuss the comparison between

  - define a mechanism for SCG activation upon UL data arrival on SCG bearers

  - use split bearer with primary path on MCG (network sees UL data and can initiate activation)

- It is FFS whether the UE can provide some assistance information for deactivation of the SCG (but there is no proposal so far).

- Confirm that there is no PUSCH transmission on deactivated SCG. FFS if any other UL is allowed towards SCG.

- Confirm that there is no PDCCH monitoring on PSCell of the deactivated SCG.

- Confirm that there is no support of SCell dormancy for SCG SCells within a deactivated SCG.

Conditional PSCell change/addition:

- In SN initiated CPC with MN involvement, the source SN transfers the execution condition(s) to the MN. FFS whether MN needs to comprehend the execution condition set by the source SN. FFS on stage-3 detail of coding of execution condition(s) in the final message.

- Only SRB1 can be used in CPA and Inter-SN CPC scenarios in Rel-17. The complete message upon CPAC execution for CPA and Inter-SN CPC in Rel-17 should be provided to the MN via SRB1.

- For the transmission of CPAC configuration, upon reception of RRCReconfiguration/RRCConnectionReconfiguration message with CPAC configuration, the UE shall reply the RRCReconfigurationComplete/RRCConnectionReconfigurationComplete message to the MN to inform that the message has been received. FFS if the message contains an embedded RRC complete message to the SN.

- UE checks the validity of CPAC execution criteria configuration immediately on receiving the CPAC Reconfiguration message.

- Compliance check for embedded RRCReconfiguration may be delayed until execution (up to UE implementation).

- At least the following two options should be discussed for the transmission of RRC complete message upon the CPAC execution.

  - Option 1: If SRB1 is used for the transmission, in CPA and Inter-SN CPC, upon execution of CPAC, the UE shall reply the RRCReconfigurationComplete/RRCConnectionReconfigurationComplete message to the MN including an embedded RRC complete message to the SN, and then the MN informs the target SN. This assumes the scenario where the MCG configuration is/can be changed upon triggering the CPA and/or inter-SN CPC.

  - Option 2:  If SRB1 is used for the transmission, in CPA and Inter-SN CPC, upon execution of CPAC, the ULInformationTransferMRDC should be used to transfer the complete message (as for intra-SN CPC). This assumes the scenario where the MCG configuration is not changed upon triggering the CPA and/or inter-SN CPC.

- FFS if the configurations of all candidates PSCell configurations for CPA and Inter-SN PSCell change are released upon the successful completion of CPAC, conventional PSCell change or conventional PSCell addition.

- FFS if SCGFailureInformation procedure can be taken as the baseline for CPAC failure handling in Rel-17 scenarios. For CPC initiated by MN, A4/B1 like execution condition should be supported.

- For CPC initiated by MN, A4/B1 like execution condition should be supported.

- Non-conditional SCG RRC Reconfiguration can be sent in the same MN generated RRCReconfiguration message, which carries execution conditions and target candidate configurations. i.e. ‎the secondaryCellGroup can be sent in the same configuration message with the ‎conditionalReconfiguration for inter-SN CPC.

- In case of

  - CPA and MN initiated Inter-SN CPC, upon reception of ‎RRCReconfiguration/RRCConnectionReconfiguration message with CPAC configuration, UE responds with RRCReconfigurationComplete / RRCConnectionReconfigurationComplete message to the MN to inform ‎that the message has been received. The message does not include an embedded RRC complete message for source SN.

  - SN initiated Inter-SN CPC, upon reception of ‎RRCReconfiguration/RRCConnectionReconfiguration message with CPAC configuration, UE responds with RRCReconfigurationComplete / RRCConnectionReconfigurationComplete message to MN. This message can include an embedded RRC complete message for source SN.

- The message carrying ‎conditionalReconfiguration for CPA/CPC is in MN format (i.e. contains ‎both MCG and SCG re-configurations). For the following cases: a). MN-Initiated CPA b). MN-Initiated inter-SN CPC c). SN-initiated inter-SN CPC.

- In CPA and Inter-SN CPC, upon execution of CPAC, ‎the UE ‎shall ‎reply the RRCReconfigurationComplete/RRCConnectionReconfigurationComplete ‎message to ‎the MN ‎including an embedded RRC complete message to the SN, and then the MN ‎informs the ‎target SN.

- Working assumption: the configurations of all candidates PSCell configurations for CPA and Inter-SN PSCell change are ‎released upon the successful completion of CPAC, conventional PSCell change or conventional PSCell ‎addition.‎ This can be revisited if critical issues found in a later stage.

- SCGFailureInformation procedure can be taken as the baseline for CPAC failure ‎handling in Rel-17 ‎scenarios.‎

  - FFS on the exact content of the message.

  - FFS if time allows on further ‎enhancements to CPAC failure handling‎

- Send an LS to RAN3 informing RAN2 agreements.

#### 2.2.2	Remaining Open issues

Efficient activation/de-activation mechanism for one SCG and SCells:

- signalling and inter-node interaction at SCG activation/deactivation

- certain aspects of UE behaviour for deactivated SCG e.g.

  - Handling of SCells

  - RRM measurement details

  - Whether there are L1 measurements/reporting

  - Whether there is RLM/BFD

  - Need for RACH at activation and at PSCell change

Conditional PSCell change/addition:

- certain aspects of signalling and inter-node interaction for the supported scenarios e.g.

  - handling of multiple candidate cells

  - UE capability for triggering quantities

  - CPAC execution

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#110-e agreements:

Signalling Support for Efficient Activation/Deactivation for One SCG and SCells

- MN initiated SN modification procedure can be used for support of SCG (de)activation, and SN can decide whether to accept or reject SCG (de)activation request after receiving SN modification request message.

- Activity Notification message sent from SN to MN, can be used for the MN to make final decision on SCG (de)activation. It is FFS whether no spec impacts or the Activity Notification message shall be enhanced, e.g., add a new SCG (de)activation suggestion IE.

- MN can initiate SCG (de)activation during SN addition procedure, SN can decide whether to accept or reject SCG (de)activation request after receiving SN addition request message, FFS on how to reject it.

Signalling Support for Conditional PSCell Change/Addition

- RAN3 discuss CPAC in (NG) EN-DC and NR-DC.

- Start to Focus on CPA, MN initiated inter-SN CPC, and SN initiated inter-SN CPC, if time allows, other cases can be discussed pending to RAN2 progress

- Start CPAC discussion based on the conventional DC procedures:

  - CPA: SN addition procedure for CPA

  - MN initiated inter SN CPC: MN initiated SN Change procedure, i.e. CPA + SN release

  - SN initiated inter SN CPC: SN initiated SN Change procedure

  - FFS on direct inter-SN communication

- Target SN to make the decision on the prepared PSCell or PSCells (if decided to be allowed).

- WA: target SN to provide the prepared PSCell id (or PSCell ids, if decided to be allowed) to the MN for CPA, MN initiated inter-SN CPC, and SN initiated inter-SN CPC

- WA: Support Early Data Forwarding in CPAC.

- WA: in case of MN initiated inter-SN CPC, to support early data forwarding, the MN needs to inform source SN about CPC triggered (i.e. the successful reconfiguration of CPC at UE), details FFS.

- Support Late Data Forwarding in CPAC.

- WA: in case of both MN and SN initiated inter-SN CPC, to support late data forwarding, it is needed to inform the source SN about the successful CPC execution and UE accesses to the target SN, details FFS. RAN3 waits for RAN2 progress before discussing further details.

RAN3#111-e agreements:

Signalling Support for Efficient Activation/Deactivation for One SCG and SCells

- Xn interface: MN initiated SN addition procedure:

  - Add a new IE in the SN addition request message to indicate at least the de-activation, while the detail code of this new IE is FFS.

    - E.g., if the IE is set to 1 or not existed, the SCG is requested to activate.  If the IE is set to 0, the SCG is requested to de-activate.

  - Add a new IE in the SN addition response message to indicate at least the de-activation result, while the detail code of this new IE is FFS.

    - E.g., if the IE is set to 0, the SCG is de-activated. If the IE is set to 1, the SCG is activated.

- Open issue 1: During SN addition procedure, if the request of SCG (de)activation is rejected:

1) SN uses the response message including “SCG deactivation” result is sufficient;

2) or SN allows to use the reject message including new Cause value;

3) or SN allows to uses the reject message as legacy (without new Cause)

- MN initiated SN modification procedure

  - Add a new IE, e.g., “SCG activation requested” with two codepoints in the SN modification request message in order to indicate the SCG is requested to activate or de-activate.

  - Add a new IE, e.g., “SCG activation result” with two codepoints in the SN modification response message in order to indicate the SCG is activated or de-activated.

- Open issue 2: During SN modification procedure, if the request of SCG (de)activation is rejected:

1) SN uses the response message including “SCG (de)activation” is sufficient;

2) or SN allows to use the reject message including new Cause value;

3) or SN allows to use the reject message as legacy (without new Cause).

- FFS: Whether X2/Xn Handover procedure needs to be enhanced to support of SCG (de)activation.

- F1 interface: UE context setup procedure

  - Add a new IE in the UE context setup request message to indicate at least the de-activation, while the detail code of this new IE is FFS.

    - E.g., if the IE is set to 1 or not existed, the SCG is requested to activate.  If the IE is set to 0, the SCG is requested to de-activate.

  - Add a new IE in the UE context setup response message to indicate at least the de-activation result, while the detail code of this new IE is FFS.

    - E.g., if the IE is set to 0, the SCG is de-activated. If the IE is set to 1, the SCG is activated.

- Open issue 3: During UE context setup procedure, if the request of SCG (de)activation is rejected:

1) gNB-DU uses the response message including “SCG (de)activation” is sufficient;

2) or gNB-DU allows to use the reject message including new Cause value;

3) or gNB-DU allows to use the reject message as legacy (without new Cause).

- F1 interface: UE Context Modification

  - Add a new IE, e.g., “SCG activation requested” with two codepoints in the UE Context Modification request message in order to indicate the SCG is requested to activate or de-activate.

  - Add a new IE, e.g., “SCG activation result” with two codepoints in the UE Context Modification response message in order to indicate the SCG is activated or de-activated.

- Open issue 4: During UE Context Modification procedure, if the request of SCG (de)activation is rejected:

1) gNB-DU uses the response message including “SCG (de)activation” is sufficient;

2) or gNB-DU allows to use the reject message including new Cause value;

3) or gNB-DU allows to use the reject message as legacy (without new Cause).

- Open issue 5: Whether E1AP shall be enhanced to support of SCG (de)activation, if included, the Bearer Context Setup procedure enhancement shall be aligned with X2/Xn/F1AP.

- Open issue 6: Whether E1AP shall be enhanced to support of SCG (de)activation, if included, the Bearer Context Modification enhancement shall be aligned with X2/Xn/F1AP.

- Open issue 7: Introduce a new Cause value for class1 procedure failure case, e.g., “Requested SCG state not available” is defined as “The action failed because the requested SCG state is not accepted.”MN initiated SN modification procedure can be used for support of SCG (de)activation, and SN can decide whether to accept or reject SCG (de)activation request after receiving SN modification request message.

Signalling Support for Conditional PSCell Change/Addition

- WA: Prepare multiple PSCells in one CPAC procedure.Do not provide Location Information and Resource Coordination information in CPAC, use same parameters for other IEs in the response message for different PSCells, FFS for single RRC container or multiple RRC containers which is pending to RAN2.

- WA: Initiating node to make the decision on how many PSCells may be configured for UE.

- FFS whether the initiating node send the suggested PSCell number and/or the maximum number of PSCells to the peer node.

- WA: In case of SN initiated inter-SN CPC, prepare multiple PSCells in one target SN by one SN Change procedure is the baseline.

- In CPA and MN initiated inter-SN CPC, MN does not send execution condition(s) to the Target SN, Target SN provides the prepared PSCell id(s) and the corresponding RRC container(s) (RRCReconfiguration) to the MN, and then the MN generates and transmits the conditional configuration message to the UE.

- Direct communication between S-SN and T-SN is not supported.

- Early data forwarding in CPAC is supported

#### 2.3.2	Remaining Open issues

- Support of Efficient activation/de-activation mechanism for one SCG

  - Inter-node signalling to support SCG activation/deactivation

- Support of Conditional PSCell change/addition

  - Inter-node signalling to support CPA

  - Inter-node signalling to support CPC

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#98-E agreements:

On efficient SCell activation:

RAN4 replied in R4-2104067 to R1-2009798 the following conclusions:

- SCell to be activated is known and belongs to FR1

  - If SCell measurement cycle is equal to or smaller than 160ms

    - temporary RS can be used for time/frequency tracking

      - 1 burst (2-slot with four CSI-RS resources) is required based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798.

  - If SCell measurement cycle is larger than 160ms

    - temporary RS can be used for AGC

      - 1 burst (2-slot with four CSI-RS resources) is required

    - temporary RS can be used for time/frequency tracking

      - 1 separate burst (2-slot with four CSI-RS resources) is required in addition to the one burst required for AGC

  - The agreements above apply based on RAN1 working assumptions on temporary RS design provided in the LS R1-2009798.

  - FFS: whether minimum gap between the RS symbol(s) for AGC and the RS symbols for time/frequency acquisition is considered to account for UE AGC application time delay

    - The minimum gap length is FFS

- SCell to be activated belongs to FR2

  - If there is at least one active serving cell on that FR2 band and temporary RS for the target SCell is provided, no matter whether the SCell to be activated is known or unknown

    - temporary RS can be used for time/ frequency tracking

      - The number of temporary RS symbols is under discussion

  - If there is no active serving cell on that FR2 band, and the SCell to be activated is known to UE

    - temporary RS can be used for fine timing tracking

      - The number of temporary RS symbols is under discussion

- RAN1 working assumption for temporary RS (i.e., reuse existing Rel-15/16 TRS structure) can provide reduction in maximum allowed activation delay at least for some cases as listed above. RAN4 will continue the discussion if there are any suggested changes from RAN4 perspective and will provide additional agreements (if any) in RAN4 #99-e.

#### 2.4.2	Remaining Open issues

Work has not started for any of the objectives (RAN4 only replied to RAN1 LS).

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

RAN1#102-E contributions:

- R1-2006415	Work plan for Further MR-DC Enhancements	Huawei

- R1-2005411	Discussion on efficient activation/de-activation mechanism for Scells	vivo

- R1-2005442	Discussion on Support Efficient Activation De-activation Mechanism for SCells in NR CA	ZTE

- R1-2005629	On supporting efficient activation mechanism for SCells in NR CA	MediaTek Inc.

- R1-2005698	Disucssion on efficient activation/de-activation mechanism for Scell in NR CA	CATT

- R1-2005908	On low latency Scell activation	Nokia, Nokia Shanghai Bell

- R1-2006065	Efficient activation/de-activation for Scell	OPPO

- R1-2006178	On efficient activation/de-activation mechanism for Scells	Samsung

- R1-2006283	Discussion on efficient activation/de-activation mechanism for SCells in NR CA	Spreadtrum Communications

- R1-2006511	Views on Rel-17 DSS SCells efficient activation/de-activation	Apple

- R1-2006673	Reduced Latency SCell Activation	Ericsson

- R1-2006751	Discussion on efficient activation/deactivation mechanism for SCells	NTT DOCOMO, INC.

- R1-2006754	Efficient activation/deactivation of SCell	ASUSTEK COMPUTER (SHANGHAI)

- R1-2006835	Views on efficient activation/de-activation mechanism for SCells in NR CA	Qualcomm Incorporated

- R1-2006927	Discussion on efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

RAN1#103-E contributions:

- R1-2008832	Discussion on Support Efficient Activation De-activation Mechanism for SCells in NR CA			ZTE

- R1-2007548	Support efficient activation/de-activation mechanism for Scells	FUTUREWEI

- R1-2007697	Discussion on efficient activation/de-activation mechanism for Scells	vivo

- R1-2007841	Disucssion on efficient activation/de-activation mechanism for Scell in NR CA	CATT

- R1-2008112	Discussion on efficient activationde-activation mechanism for SCells in NR CA	Spreadtrum Communications

- R1-2008197	On efficient activation/de-activation mechanism for Scells	Samsung

- R1-2008286	Discussion on efficient activation/de-activation for Scell	OPPO

- R1-2008322	Discussion on efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

- R1-2008453	On efficient SCell Activation/Deactivation	Apple

- R1-2008713	Efficient activation/deactivation of SCell	ASUSTeK

- R1-2008849	Discussion on efficient activation mechanism for SCells	NEC

- R1-2008968	On supporting efficient activation mechanism for SCells in NR CA	MediaTek Inc.

- R1-2009005	On efficient activation/de-activation for SCells	Intel Corporation

- R1-2009048	On low latency Scell activation	Nokia, Nokia Shanghai Bell

- R1-2009197	Discussion on efficient activation/deactivation mechanism for SCells	NTT DOCOMO, INC.

- R1-2009208	Reduced Latency SCell Activation	Ericsson

- R1-2009279	Views on efficient activation/de-activation mechanism for SCells in NR CA	Qualcomm Incorporated

- R1-2009283	On work plan for SI on XR Evaluations for NR	Qualcomm Incorporated

RAN1#104-E contributions:

- R1-2100045	Support efficient activation/de-activation mechanism for Scells	FUTUREWEI

- R1-2100112	Discussion on Support Efficient Activation De-activation Mechanism for SCells in NR CA	ZTE

- R1-2100188	Discussion on efficient activation/de-activation for Scell	OPPO

- R1-2100192	Discussion on efficient activation/de-activation mechanism for SCells	Huawei, HiSilicon

- R1-2100360	Discussion on efficient activation and de-activation mechanism for SCell in NR CA	CATT

- R1-2100475	Discussion on efficient activation/de-activation mechanism for Scells	vivo

- R1-2100679	On efficient activation/de-activation for SCells	Intel Corporation

- R1-2100695	Discussion on efficient activation mechanism for SCells	NEC

- R1-2100721	On low latency Scell activation	Nokia, Nokia Shanghai Bell

- R1-2100795	Discussion on efficient activation/de-activation mechanism for SCells in NR CA	Spreadtrum Communications

- R1-2101067	Discussion on efficient activation/de-activation mechanism for SCells	CMCC

- R1-2101239	On efficient activation/de-activation mechanism for Scells	Samsung

- R1-2101294	Fast SCell Activation	InterDigital, Inc.

- R1-2101364	On Efficiency Activation/De-activation for SCells in CA	Apple

- R1-2101492	Efficient activation/de-activation mechanism for SCells in NR CA	Qualcomm Incorporated

- R1-2101563	Reduced Latency SCell Activation	Ericsson

- R1-2101566	Efficient activation/deactivation of SCell	ASUSTeK

- R1-2101634	Discussion on efficient activation/deactivation mechanism for SCells	NTT DOCOMO, INC.

RAN2#111-E contributions:

- R2-2007676	Work plan for R17 Further MR-DC enhancements WI	Huawei

- R2-2007677	Status of the work on efficient SCell activation/deactivation	Huawei

- R2-2006756	On Support of Activation/Deactivation for SCG	InterDigital

- R2-2006806	Discussion on SCG suspension	OPPO

- R2-2006900	Framework of SCG deactivation and activation	ZTE Corporation, Sanechips

- R2-2007009	Efficient Activation/Deactivation Mechanism for SCG and Scells	CATT

- R2-2007046	Discussion on efficient activation mechanism for one SCG	Spreadtrum Communications

- R2-2007068	On fast deactivation and activation of one SG and SCells	Nokia, Nokia Shanghai Bell

- R2-2007109	Scoping the usage of SCG suspension	Apple

- R2-2007215	Efficient activation and deactivation mechanism for SCG and SCells	vivo

- R2-2007236	Enhancements for Rel-17 efficient activation/de-activation	Intel Corporation

- R2-2007439	Consideration on dormant SCG	CMCC

- R2-2007598	Efficient SCG/SCell (de)activation	Ericsson

- R2-2007623	Further enhancements regarding deactivation and resumption for R17	Samsung Telecommunications

- R2-2007678	Discussion on SCG deactivation and activation	Huawei, HiSilicon

- R2-2007748	Efficient SCG activation/deactivation in MR-DC	Qualcomm Incorporated

- R2-2007867	Discussion on SCG suspension	MediaTek Inc.

- R2-2007986	Time-efficient SCG Activation mechanism	LG Electronics

- R2-2007994	Discussion of SCG activation/deactivation	SHARP Corporation

- R2-2006695	Scope and scenario for CPAC	vivo

- R2-2006757	Coexistence of CHO and CPC at the UE	InterDigital

- R2-2006805	Discussion on conditional PSCell change and addition	OPPO

- R2-2006901	Discussion on conditional PSCell addition/change	ZTE Corporation, Sanechips

- R2-2006976	Overview of conditional PSCell addition	NEC

- R2-2006977	Inter-SN Conditional PSCell Change	NEC

- R2-2007010	Scope and basic procedure for Conditional PSCell Addition/Change ??(CPAC)?	CATT

- R2-2007052	Discussion on conditional PSCell addition or change	Spreadtrum Communications

- R2-2007089	Discussion on conditional PSCell change and addition	Apple

- R2-2007130	Scenarios and General Principles of CPAC	ETRI

- R2-2007237	Rel-17 Conditional PSCell Addition	Intel Corporation

- R2-2007364	On the scope of Rel-17 CPAC	Nokia, Nokia Shanghai Bell

- R2-2007438	Discussion on CPAC scenarios	CMCC

- R2-2007553	Inter node CPAC procedure and configuration discussion	Futurewei

- R2-2007599	Conditional reconfigurations	Ericsson

- R2-2007624	Further enhancements on conditional configuration for R17	Samsung Telecommunications

- R2-2007679	Discussion on Conditional PSCell addition/change	Huawei, HiSilicon

- R2-2007749	Conditional PSCell addition/change	Qualcomm Incorporated

- R2-2007839	Conditional PSCell addition and change in MR-DC	Potevio

- R2-2007985	Considerations of CPAC in Rel-17	LG Electronics

- R2-2008079	Remaining issues of Conditional PSCell Addition	NTT DOCOMO INC.

RAN2#112-E contributions:

- R2-2008870	Discussion on SCG suspension or deactivation	OPPO

- R2-2008920	Considerations on fast (de)active of Scell	KDDI Corporation

- R2-2009150	Discussion on efficient activation mechanism for one SCG	Spreadtrum Communications

- R2-2009246	Further consideration on SCG activation and deactivation	ZTE Corporation, Sanechips

- R2-2009259	On Support of Activation/Deactivation for SCG	InterDigital

- R2-2009284	Further discuss the issues with SCG fast activation	Futurewei

- R2-2009357	Efficient Activation/Deactivation Mechanism for SCG	CATT

- R2-2009439	Discussion on SCG suspension	MediaTek Inc.

- R2-2009531	Open items on SCG deactivation feature	Apple

- R2-2009547	On fast deactivation and activation of one SG and SCells	Nokia, Nokia Shanghai Bell

- R2-2009590	Discussion on efficient deactivation mechanism for the SCG	China Unicom

- R2-2009814	SCG deactivation upon SCG addition	NEC

- R2-2009867	On SCG deactivatoin and activation	Lenovo, Motorola Mobility

- R2-2009913	Discussion on efficient SCG activation/deactivation	China Telecommunications

- R2-2009942	Signalling for Rel-17 efficient SCG de-activation/re-activation	Intel Corporation

- R2-2010062	Efficient SCG (de)activation	Ericsson

- R2-2010087	Progressing SCG deactivation and resumption for R17	Samsung Telecommunications

- R2-2010123	[Post111-e][919][eDCCA] Efficient activation deactivation of SCG Discussion on SCG deactivation and activation	Huawei

- R2-2010124	Discussion on SCG deactivation and activation	Huawei, HiSilicon

- R2-2010132	Efficient SCG activation/deactivation in MR-DC	Qualcomm Incorporated

- R2-2010231	Signalling for SCG activation	SHARP Corporation

- R2-2010283	Efficient SCG Activation mechanism	LG Electronics

- R2-2010290	Activation and deactivation mechanism for SCG and SCells	vivo

- R2-2010372	Considerations on SCG activation or deactivation	CMCC

- R2-2010683	Progressing SCG deactivation and resumption for R17	Samsung Telecommunications

- R2-2010733	[AT112-e][230][eDCCA] Progressing FFS points of efficient SCG activation and deactivation (Huawei)	Huawei

- R2-2009088	Conditional PSCell change / addition	vivo

- R2-2009158	CPC configuration number restriction	Spreadtrum Communications

- R2-2009260	Coexistence of CHO and CPC	InterDigital

- R2-2009285	CPAC failure handling discussio	Futurewei

- R2-2009358	Discussion on Further CPAC Enhancements	CATT

- R2-2009359	Introduction of CPA and MN Initiated Inter-SN CPC	CATT

- R2-2009360	Summary of  [Post111-e][920][eDCCA] Conditional PSCell Change and Addition (CATT)	CATT

- R2-2009379	Discussion on conditional PSCell addition/change	ZTE Corporation, Sanechips

- R2-2009475	Discussion on conditional PSCell change and addition	Apple

- R2-2009592	Discussion on inter-SN conditional PSCell change (SN initiated)	China Unicom

- R2-2009596	Discussion on conditional PSCell change and addition	OPPO

- R2-2009771	On Rel-17 Conditional PSCell Addition and Change (CPAC)	Nokia, Nokia Shanghai Bell

- R2-2009815	Conditional PSCell addition procedure	NEC

- R2-2009816	Framework of Inter-SN Conditional PSCell change	NEC

- R2-2009868	Issues on inter-SN CPC	Lenovo, Motorola Mobility

- R2-2010003	Conditional PSCell Change / Addition	Ericsson

- R2-2010088	Progressing conditional configuration for R17	Samsung Telecommunications

- R2-2010125	Discussion on support of conditional PSCell change/addition	Huawei, HiSilicon

- R2-2010130 Configuration of Conditional PSCell addition/change	Qualcomm Incorporated

- R2-2010248	Discussion on SN initiated CPC and CPAC Execution	ETRI

- R2-2010282	Considerations of CPAC in Rel-17	LG Electronics

- R2-2010373	Discussions about CPAC procedures	CMCC

- R2-2010529	Regarding inter MN-SN signaling design for Conditional PSCell Addition	Intel Corporation

- R2-2010626	Further consideration for Conditional PSCell addition and change	NTT DOCOMO INC.

- R2-2010734	[AT112-e][231][eDCCA] Progressing conditional reconfiguration for SN initiated inter-SN CPC (CATT)	CATT

- R2-2010850	LS on Conditional PSCell Aaddition/Change agreements	RAN2

- R2-2007676	Work plan for R17 Further MR-DC enhancements WI	Huawei

RAN2#112-E contributions:

- R2-2100136	Discussion on SCG deactivation and activation	OPPO

- R2-2100137	Discussion on TRS activation for fast SCell activation	OPPO

- R2-2100426	Discussion on SCG deactivation	China Telecom

- R2-2100568	Further consideration on SCG deactivation and activation	ZTE Corporation, Sanechips

- R2-2100589	Progressing SCG deactivation and resumption for R17	Samsung Telecommunications

- R2-2100632	Further discuss the issues with SCG fast activation	Futurewei

- R2-2100640	Further considerations on SCG deactivation	NEC

- R2-2100641	SCG (de)activation initiation	NEC

- R2-2100647	Considerations on Time Alignment Timer for SCG deactivation	KDDI Corporation

- R2-2100667	Discussion on efficient activation mechanism for one SCG and SCells	Spreadtrum Communications

- R2-2100729	Power-efficient SCG (De)activation mechanism	LG Electronics

- R2-2100730	Time-fficient SCG (De)activation mechanism	LG Electronics

- R2-2101014	UE behavior for SCG deactivation	vivo

- R2-2101015	Signaling aspect of SCG activation and deactivation	vivo

- R2-2101077	Deactivated SCG handling	Nokia, Nokia Shanghai Bell

- R2-2101078	MN and SN responsibilities for SCG deactivation	Nokia, Nokia Shanghai Bell

- R2-2101094	Mobility and RRM for deactivated SCG	Ericsson

- R2-2101095	On the need for random access during SCG activation	Ericsson

- R2-2101096	SCG (de)activation procedure	Ericsson

- R2-2101121	General issues on SCG activation and deactivation	Lenovo, Motorola Mobility

- R2-2101122	[Draft] LS on SCG activation and deactivation	Lenovo, Motorola Mobility

- R2-2101123	SCell states configuration in the same RRC message to activate/deactivate SCG	Lenovo, Motorola Mobility

- R2-2101235	Further Considerations on Efficient SCG Activation/Deactivation	CATT

- R2-2101312	On Support of Activation/Deactivation for SCG	InterDigital

- R2-2101464	Remaining open items on SCG deactivation feature	Apple Inc

- R2-2101481	UE behaviour on deactivated SCG	Huawei, HiSilicon

- R2-2101482	SCG activation and deactivation procedure	Huawei, HiSilicon

- R2-2101483	Selection of SCG activation state at mobility and resume	Huawei, HiSilicon

- R2-2101541	Consideration for some remaining FFSes	Intel Corporation

- R2-2101807	Discussion on SCG deactivation	MediaTek Inc.

- R2-2101865	LS RAN2 decisions for SCG deactivation	Nokia, Nokia Shanghai Bell

- R2-2101871	UE behaviour in SCG deactivated state	Qualcomm Incorporated

- R2-2101876	Further discussion for SCG deactivation	SHARP Corporation

- R2-2101883	Considerations on SCells in SCG deactivation	CMCC

- R2-2101884	Signallings of SCG activation and deactivation	CMCC

- R2-2101915	Further consideration on SCG activation and deactivation	NTT DOCOMO INC.

- R2-2101969	Summary of [AT113-e][230][eDCCA] Solution alternatives for SCG activation and deactivation (Huawei)	Huawei

RAN3#110-e contributions:

- R3-205947	Enhancements related to the conditional PSCell change	Nokia, Nokia Shanghai Bell	discussion

- R3-205948	Enabling inter-SN conditional PSCell change	Nokia, Nokia Shanghai Bell	CR	38.423

- R3-205949	Enabling inter-SN conditional PSCell change	Nokia, Nokia Shanghai Bell	CR	36.423

- R3-205963	Inter-node signaling to support activation/deactivation of SCG	InterDigital	discussion

- R3-205990	Discussion on SCG deactivation and activation	ZTE	discussion

- R3-205991	Draft CR37340 for SCG deactivation and activation	ZTE	draftCR	37.340

- R3-205992	Discussion on CPA and CPC	ZTE	discussion

- R3-205993	Draft CR37340 for support of CPAC	ZTE	draftCR	37.340

- R3-206062	Discussion on Conditional PScell Mobility procedures	China Telecommunications	discussion

- R3-206076	CR on 38.423 for introducing CPC indicator in SN Addition procedure	China Telecommunications	CR	38.423

- R3-206079	CR on 36.423 for introducing CPC indicator in SgNB Addition procedure	China Telecommunications	CR	36.423

- R3-206155	CPA Signalling flow and discussion	NEC	discussion

- R3-206178	Conditional PSCell Addition and Change	Qualcomm Incorporated	discussion

- R3-206179	Signaling for Efficient SCG activation/deactivation	Qualcomm Incorporated	discussion

- R3-206282	Consideration on support of conditional PScell Change/Addition	LG Electronics	discussion

- R3-206337	Consideration on Activation/Deactivation for One SCG and SCells	LG Electronics	discussion

- R3-206341	Discussion on support of efficient activation/de-activation mechanism for one SCG and Scells	Huawei	discussion

- R3-206342	Activation and de-activation of SCG	Huawei	draftCR	37.340

- R3-206343	Activation and de-activation of SCG	Huawei	CR	38.401

- R3-206344	Discussion on Support of conditional PSCell change addition	Huawei	discussion

- R3-206345	Conditional PSCell change addition	Huawei	draftCR	37.340

- R3-206346	Conditional PSCell change addition	Huawei	CR	38.401

- R3-206355	Discussion on efficient Activation/Deactivation Mechanism for SCG	CATT	discussion

- R3-206356	CR for TS 38.423 on efficient Activation/Deactivation Mechanism for SCG	CATT	CR	38.423

- R3-206357	CR for TS 38.473 on efficient Activation/Deactivation Mechanism for SCG	CATT	CR	38.473

- R3-206358	Consideration on Scenarios for Conditional PSCell Change/Addition	CATT	discussion

- R3-206359	CR for 37.340 on Conditional PSCell Change/Addition	CATT	draftCR	37.340

- R3-206479	On support of CPAC	Lenovo, Motorola Mobility	discussion

- R3-206480	On support for SCG deactivation and activation	Lenovo, Motorola Mobility	discussion

- R3-206497	Support of SCG activation/de-activation mechanism	Ericsson	discussion

- R3-206498	Support of SCG activation/deactivation for MR-DC	Ericsson	CR	38.423

- R3-206499	Support of SCG activation/deactivation for MR-DC	Ericsson	CR	38.473

- R3-206623	Support of conditional PSCell change-addition	Ericsson	discussion

- R3-206624	Conditional SN Addition	Ericsson	CR	38.423

- R3-206625	Conditional SN Addition	Ericsson	draftCR	37.340

- R3-206626	CHO in MR-DC operation	Ericsson	discussion

- R3-206754	Discussion on CPAC based on the conventional DC scenarios	Samsung	discussion

- R3-206755	Discussion on messages and IEs to support CPA	Samsung	discussion

- R3-206756	(TP for LTE_NR_DC_enh2-Core for TS 36.423) CPA related X2 messages	Samsung	other

- R3-206828	Discussion on MN coordinated SGC (de-)activation	Samsung	discussion

- R3-207003	CB: # MRDC1-SCG_activation_deactivation - Summary of email discussion	ZTE - moderator	discussion

- R3-207004	CB: # MRDC2-PSCell_Change_Addition - Summary of email discussion	Huawei - moderator	discussion

- R3-206894	CB: # MRDC3-Others - Summary of email discussion	Ericsson - moderator	discussion

RAN3#111-e contributions:

- R3-210045	LS on Conditional PSCell Addition/Change agreements	RAN WG2

- R3-210082	Discussion on further details of the inter-SN CPC	Nokia, Nokia Shanghai Bell

- R3-210083	Enabling inter-SN conditional PSCell change	Nokia, Nokia Shanghai Bell

- R3-210084	Enabling inter-SN conditional PSCell change	Nokia, Nokia Shanghai Bell

- R3-210085	Principles for activation and deactivation of SCG resources	Nokia, Nokia Shanghai Bell

- R3-210086	Enabling fast SCG activation/deactivation	Nokia, Nokia Shanghai Bell

- R3-210087	Enabling fast SCG activation/deactivation	Nokia, Nokia Shanghai Bell

- R3-210106	Discussion on Conditional PScell Addition/Change procedures	China Telecommunication

- R3-210107	CR on 36.423 for introducing CPAC information in SgNB Addition procedure	China Telecommunication

- R3-210108	CR on 38.423 for introducing CPAC information in SN Addition procedure	China Telecommunication

- R3-210136	MN initiated SCG activation and deactivation	Huawei, InterDigital

- R3-210137	Activation and de-activation of SCG	Huawei, InterDigital

- R3-210138	Activation and de-activation of SCG	Huawei, InterDigital

- R3-210180	Further discussion on SCG deactivation and activation	ZTE

- R3-210181	CR 37.340 for support of SCG activation/deactivation	ZTE

- R3-210182	CR 38.423 for support of SCG activation/deactivation	ZTE

- R3-210183	Discussion on CPA and CPC	ZTE

- R3-210184	CR 36.423 for support of CPA and CPC	ZTE

- R3-210185	CR 38.423 for support of CPA and CPC	ZTE

- R3-210186	[draft] Reply LS on Conditional PSCell Addition/Change	ZTE

- R3-210239	SCG Activation / deactivation discussion	NEC

- R3-210240	discussion on Conditional PSCell Addition and PSCell Change	NEC

- R3-210266	Discussion on SCG activation and deactivation	Samsung

- R3-210267	CR on SCG (de-)activation over F1	Samsung

- R3-210268	CR on SCG (de-)activation over E1	Samsung

- R3-210352	Signaling support for SCG deactivation/activation	Qualcomm Incorporated

- R3-210353	CPA and MN initiated Inter-SN CPC procedures: preparation, execution, and data forwarding	Qualcomm Incorporated

- R3-210354	SN initiated Inter-SN CPC procedure: preparation, execution, and data forwarding	Qualcomm Incorporated

- R3-210403	Support of SCG activation/de-activation	Ericsson

- R3-210404	Support of SCG activation/de-activation over X2	Ericsson

- R3-210405	Support of SCG activation/de-activation over Xn	Ericsson

- R3-210502	(TP to TS 38.401) Consideration on conditional PSCell change/addition	Huawei

- R3-210503	Support of conditional PSCell change/addition	Huawei

- R3-210504	Support of conditional PSCell change/addition	Huawei

- R3-210535	Open issues on conditional PScell Change/Addition	LG Electronics

- R3-210537	Open issues on Activation Deactivation for One SCG and SCells	LG Electronics

- R3-210542	TP for modification and release to prepared CPAC	Google Inc.

- R3-210597	Left issues on SCG activation and deactivation	Lenovo, Motorola Mobility

- R3-210598	SCG activation and deactivation impact on E1 interface	Lenovo, Motorola Mobility

- R3-210599	Support of SCG Activation and Deactivation	Lenovo, Motorola Mobility

- R3-210600	Left issues on CPAC	Lenovo, Motorola Mobility

- R3-210601	[DRAFT] Reply LS on Conditional PSCell Addition/Change agreements	Lenovo, Motorola Mobility

- R3-210744	Support of conditional PSCell change/addition	Ericsson

- R3-210745	Conditional SN Addition	Ericsson

- R3-210746	Conditional SN Addition	Ericsson

- R3-210765	Discussion on efficient Activation/Deactivation Mechanism for SCG	CATT

- R3-210766	CR for TS 38.423 on efficient Activation/Deactivation Mechanism for SCG	CATT

- R3-210767	CR for TS 38.473 on efficient Activation/Deactivation Mechanism for SCG	CATT

- R3-210768	Some Considerations on How to Support CPAC	CATT

- R3-210769	Introduction of support CPAC	CATT

- R3-210796	Work plan for Rel-17 Further Multi-RAT Dual-Connectivity enhancements	Huawei

- R3-210857	Discussion on handling multiple PSCell candidates for CPAC	Samsung

- R3-210861	Correction on the preparation of one PSCell in one CPAC procedure	Samsung

- R3-210862	Correction on the preparation of multiple PSCells in one CPAC procedure	Samsung

- R3-210877	Correction on the preparation of one PSCell in one CPAC procedure	Samsung

- R3-210945	Regarding inter MN-SN signaling design for CPAC	Intel Corporation

- R3-211008	CB: # MRDC1-work_plan - Summary of email discussion	Huawei - moderator

- R3-211009	CB: # MRDC2-SCG_activation_deactivation - Summary of email discussion	ZTE - moderator

- R3-211010	CB: # MRDC3-PSCell_Change_Addition - Summary of email discussion	Huawei - moderator

- R3-211040	LS on Conditional PSCell Addition/Change agreements	RAN WG2

- R3-211132	CB: # MRDC2-SCG_activation_deactivation - Summary of email discussion	ZTE - moderator

- R3-211133	CB: # MRDC3-PSCell_Change_Addition - Summary of email discussion	Huawei - moderator

- R3-211134	Reply LS on Conditional PSCell Addition/Change agreements	Samsung

- R3-211338	Reply LS on Conditional PSCell Addition/Change agreements	Samsung

RAN4#98-e contributions:

- R4-2100236	Discussion on RAN1 LS on temporary RS for efficient SCell activation in NR CA	Apple

- R4-2101543	Reply LS on temporary RS for efficient SCell activation in NR CA	OPPO

- R4-2101709	Discussion on temporary RS for efficient SCell activation in NR CA	Huawei, HiSilicon

- R4-2102894	Discussion on temporary RS for efficient SCell activation in NR CA	Qualcomm Incorporated

- R4-2104067	Reply LS on temporary RS for efficient SCell activation in NR CA	RAN4

- R4-2104083	WF on temporary RS for efficient SCell activation in NR CA	Huawei, HiSilicon