3GPP TSG RAN Meeting #91e	RP-210734

Electronic Meeting, March 16 - 26, 2021

Agenda item:	9.7.25

Source: 		Reliance Jio, CEWiT, Huawei, Indian Institute of Technology Hyderabad, Indian Institute of Technology Madras, ISRO, Qualcomm, Radisys, Saankhya Labs, Tejas Networks

Title: 	Extending NavIC A-GNSS support to NR Rel-17

Document for:        Discussion and Agreement

# Introduction

A-GNSS support for NavIC navigation system was introduced in LTE Release16 [1]. The respective changes however are yet to be extended to NR.

The objective of extending the NavIC A-GNSS support in NR Release 17 was discussed during initial formulation of nr_pos_enh item before RAN#86. It was deprioritised till the start of normative work with the understanding that the same will be considered for the potential work item when the nr_pos_enh SI is completed.

The goal of this discussion is an agreement on the scope and correct way to extend the NavIC A-GNSS support to NR in Rel-17.

# Impact of NavIC A-GNSS support extension to NR

It has been identified that major part of the technical specification updates required to enable NavIC GNSS assistance in NR are already in place. Following table summarises the impact of NavIC A-GNSS support extension to NR and corresponding status in existing specifications.

| Impact of extending NavIC support to NR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Status | Remarks |
| 38.455 | NavIC GNSS ID & posSIBs to be updated in Assistance Information Meta Data definitions | Updates in-place v16.1.0 onwards | RAN3: NRPPa As defined in TS 36.455 [6], |
| 37.355 | - NavIC assistance data elements - NavIC capability Information elements - NavIC assistance data request - Broadcast of assistance data – - Mapping of posSib2Type to assistance NavIC data element | Updates in-place by default | RAN2: Common TS for LTE and NR with LPP definitions.[3] |
| 38.331 | - Reference specification for NavIC. - navic to GNSS ID IE and respective posSIBs to message definition IEs. - NavIC references to be added to text as applicable. | To be done | RAN2: NR RRC Protocol definition As defined in TS 36.331 [4], |
| 38.305 | - Adding reference specification for NavIC. - NavIC to be added to all mentions of supported GNSS systems | To be done | RAN2: As included in TS 36.305 [5], |
| 38.171 | - Nominal accuracy requirement for UE supporting NavIC constellation. - Appendix E.2 GNSS Assistance Data to be updated | To be done | RAN4: As defined in TS 36.171 [7] |

# Proposals

We propose to discuss and arrive at an agreement on best way to undertake the pending normative work for extending NavIC A-GNSS support in NR Rel-17

Option 1: To be done as TEI17 CRs in RAN2 & RAN4 respectively

Option 2: To be done as part of nr_pos_enh WI

# References

[1]	RP-192408, “Revised WID, Support for NavIC in LTE”

[2]	RP-200604, “Summary of WI LCS_NAVIC”

[3]	R2-2002238, “CR of TS 37.355 for introducing NavIC in LTE”

[4]	R2-2002239, “CR of TS 36.331 for introducing NavIC in LTE”

[5]	R2-2002240, “CR of TS 36.305 for introducing NavIC in LTE”

[6]	R3-201438, “CR of TS 36.455 for introducing NavIC in LTE”

[7]	R4-2002274, “CR of TS 36.171 for introducing NavIC in LTE – performance part”