3GPP TSG RAN Meeting #91-e							   		     RP-210751

Electronic Meeting, March 16-26, 2021

Status Report to TSG

Agenda item:			9.2.2

| WI / SI Name | Enhancements on MIMO for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_eMIMO |  |  |  |  |
| Unique ID | 800085 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-192271 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2020 (changed from 06/2020) | Performance part: 03/2021 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 100% | Performance Part: 100% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Eko Onggosanusi |
|  | Company | Samsung |
|  | Email | eko.o@samsung.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

Completed

2.1.2	Remaining Open issues

Completed

2.2	RAN2

2.2.1	Agreements

Completed

2.2.2	Remaining Open issues

Completed

2.3	RAN3

n/a

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

The following agreements and conclusions were made in RAN4 #98-e:

- RF core requirement:

Completed

- RRM core requirement and maintenance:

RRM core requirement has been completed in RAN4 #86-e.

For RRM core requirement maintenance, the following correction CRs are agreed:

- CR (R4-2104055) for TS38.133: Link Recovery requirements (Rel-16), with Cat-A CR (R4-2100206) for Rel-17.

- CR (R4-2103589) for TS38.133: Pathloss activation delay requirements (Rel-16), with Cat-A CR (R4-2100208) for Rel-17.

- CR (R4-2100758) for TS38.133: Correction on the measurement restriction for CSI-IM resource (Rel-16), with Cat-A CR (R4-2100759) for Rel-17.

- CR (R4-2103590) for TS38.133: L1-SINR measurement requirement (Rel-16), with Cat-A CR (R4-2100934) for Rel-17.

- CR (R4-2103591) for TS38.133: Maintaining L1-SINR measurement requirements Rel-16, with Cat-A CR (R4-2101672) for Rel-17.

For the clarification on applicability of requirements to Multi-TRxP, the following achieved agreement has been captured in approved Way Forward (R4-2104068):

Agreement

- Clarification on applicability of requirements to Multi-TRxP

  - Proposal: Capture agreements from RAN4#95e below in spec:

    - No RRM core requirement impact identified on MRTD/MTTD values specified in Rel-15;

    - It is RAN4 common understanding that MRTD/MTTD requirements in clauses 7.5.3, 7.6.3 and 7.6.4 is sufficient for support the deployment with multi-DCI based and single-DCI based multi-TRxP transmission.

  - Option 1: Capture agreements in RAN4#95e in applicability section in 38.133

    - Option 1a:

3.6.11 Applicability of requirements to Multi-TRxP Unless explicitly stated otherwise the Maximum Transnsmission Timing Difference (MTTD) and Maximum Receive Timing Difference (MRTD) requirements in clauses 7.5.3, 7.6.3 and 7.6.4 are applicable to Multi-TRxP deployment on NR carrier.

    - Option 1b:

3.6.11 Applicability of MRTD/MTTD requirements in intra-band DC/CA Unless explicitly stated otherwise the Maximum Transnsmission Timing Difference (MTTD) and Maximum Receive Timing Difference (MRTD) requirements in clauses 7.5.3, 7.6.3 and 7.6.4 for co-located deployment are applicable when - The network configures MIMO or TX diversity - When UE is configured to receive multiple PDCCH - When UE is configured by repetitionScheme set to one of ' fdmSchemeA', ' fdmSchemeB' and 'tdmSchemeA'

  - Option 2: Minor revision in 38.133 for clarification purpose only without core requirement changes

  - Option 3: No updates are needed in TS 38.133.

- RRM performance requirement:

WF was approved in R4-2104068, the simulation results summary of L1-SINR accuracy was noted in R4-2103726, and the following dCRs were endorsed:

- Draft CR (R4-2101675) for TS38.133: IMR configuration for L1-SINR measurement tests

- Draft CR (R4-2101676) for TS38.133: L1-SINR measurement procedure tests with SSB CMR and dedicated IMR

- Draft CR (R4-2103592) for TS38.133: Adding L1-SINR accuracy requirements

- Draft CR (R4-2103593) for TS38.133: L1-SINR measurement accuracy tests with CSI-RS based CMR and dedicated IMR

- Draft CR (R4-2103598) for TS38.133: Test case CR on measurement performance of L1-SINR for CSI-RS-based CMR and no dedicated IMR

- Draft CR (R4-2103595) for TS38.133: Introduction of test cases for BFD and link recovery procedure for Scell

- Draft CR (R4-2103596) for TS38.133: Introduction of test case of link recovery with link recovery requests,

- Draft CR (R4-2103594) for TS38.133: L1-SINR measurement accuracy tests with SSB CMR and dedicated IMR

Based upon the above draft CRs, the big CR (R4-2100938) to TS38.133 on introduction of Rel-16 NR eMIMO RRM performance requirements and test cases is agreed in post-meeting email approval, with category-A CR (R4-2100939) agreed for Rel-17.

Specifically, the following agreements were achieved in RAN4 Chairman Notes and approved way forward for each topic.

L1-SINR measurement accuracy requirement:

Agreement

- Methodology for deriving baseband accuracy from link-level simulation

  - Deriving baseband accuracy values from 99.9% confidence interval of simulation results and define the baseband L1-SINR accuracy as follows:

    - Scenario 1A: ±4.5dB

    - Scenario 2A: ±3.5dB

    - Scenario 2B: ±3.5dB

    - Scenario 2C: ±3.0dB

    - Scenario 2D: ±3.0dB

- L1-SINR measurement accuracy under extreme condition

  - L1-SINR accuracy requirement is 1dB higher under extreme condition than normal condition as SS-SINR accuracy

- Whether to introduce the relative accuracy requirement

  - Introduce the relative accuracy requirement of L1-SINR measurement in Rel-16.

- The definition of relative L1-SINR accuracy requirement

  - Refer to the definition of relative L1-RSRP accuracy requirement (The same methodology as L1-RSRP)

- How to derive the L1-SINR accuracy requirement

  - Deriving L1-SINR accuracy requirement from the simulation results as no significant impact of implementation margin on relative accuracy.

- How to derive the L1-SINR accuracy requirement from the simulation results

  - Use the same methodology as absolutely accuracy, 99.9% interval of the simulation results as relative L1-SINR accuracy.

- Additional FR2 margin for CMR+IMR L1-SINR measurement Scenario

  - No additional margin for CMR+IMR L1-SINR measurement is introduced for FR2

  - The requirements are defined under the following assumption

    - CMR/IMR in the test come from the same direction

    - CMR/IMR are scheduled in the same slot

      - Note: capture the respective assumptions in the specification

- Other implementation margins for L1-SINR measurement accuracy

  - Add 1dB margin for L1-SINR measurement accuracy in FR1/FR2 to take into account the fading conditions.

- Relative L1-SINR accuracy requirement from the simulation results

  - Use the same methodology as absolutely accuracy, 99.9% interval of the simulation results to define relative L1-SINR accuracy.

  - Companies can bring additional results in the next meeting and further discuss the final relative accuracy values.

- Update the spec for L1-SINR measurement requirement

  - CR to update the TS 38.133 Annex B.2 for L1-SINR measurement requirement.

  - Other necessary update or correction is not preclude.

Test Case for L1-SINR Measurement:

Agreement

- Introducing CSI-IM and CSI-RS for L1-SINR measurement procedures tests

  - Introduce the following CSI-IM configurations and CSI-RS configurations for L1-SINR measurement procedure tests.

    - One set of periodic CSI-IM resources with 20ms periodicity for Scenario 2A

    - One set of aperiodic CSI-IM resources for Scenario 2B

    - One set of periodic CSI-RS resources with 20ms periodicity for Scenario 2C

    - One additional set of aperiodic CSI-RS resources for Scenario 2D

- Introducing CSI-IM and CSI-RS for L1-SINR measurement accuracy test

  - Introduce the following CSI-IM configurations and CSI-RS configurations for L1-SINR measurement accuracy tests

    - One set of periodic CSI-IM resources with 10ms periodicity for Scenario 2B

    - One additional set of periodic CSI-RS resources with 10ms periodicity for Scenario 2D

- Discussion on the revised CR on test cases for L1-SINR accuracy

  - For L1-SINR measurement accuracy test, define two tests for FR1 (high Io and low Io) and one test for FR2 (high Io only).

Test case for SCell beam failure recovery:

Agreement

- Test cases defined for link recovery in SCell

  - RAN4 defines two scenarios for the test case of SCell Beam Failure Recovery

    - Scenario 1: Network does not configure PUCCH for SR for BFR MAC CE

    - Scenario 2: Network configures PUCCH for SR for BFR MAC CE

- The setting of cases to be defined for each scenario

  - Test setup of two scenarios, e.g., time duration, q0/q1 configuration, are common for both Scenario 1 and Scenario 2 in SCell BFR test cases.

- The methodology of Scell Beam Failure Recovery test cases

  - For Scenario 1 it verifies UE transmits SR with random access procedure. The test procedure also includes the reported BFR MAC CE contains a beam associated with the candidate beam set q1.

  - For Scenario 2 it verifies UE transmits PUCCH with an LRR. The test procedure also includes the reported BFR MAC CE contains a beam associated with the candidate beam set q1.

- Core requirement update for link recovery in SCell is needed

  - Revise TS38.133 8.5.9 to capture the case the gNB does not configure the dedicated PUCCH for SCell BFR MAC CE.

Test case of pathloss RS activation delay:

Agreement

- Test Case for Pathloss RS Activation Delay

  - RAN4 further study the testability of PL RS activation delay requirement.

    - Study on the feasibility of the PHR-based test configuration of changing pathloss by RS switching in FR1 and FR2.

    - Study on the possibility of PHR triggered due to causes other than PL changing in the test.

    - Study on the feasibility of test methodologies (e.g., OTA or conducted) for FR1 and FR2.

  - Test case for MAC-CE based pathloss RS activation delay shall be updated in TS38.133 provided it is testable with proposed feasible test method, or otherwise no test case is for pathloss RS activation delay.

- Demodulation and CSI performance requirements

Following way forward and simulation assumption for PDSCH requirements, PMI requirement ware approved, and the following CRs are agreed:

- R4-2017529	WF on NR eMIMO demodulation and CSI requirement PDSCH requirement

- R4-2103826           CR  to TS38.101-4 : general and applicability rule for eMIMO demod requirements;

- R4-2103830           CR to TS38.101-4 : FRC for eMIMO sDCI/mDCI-based PDSCH transmission

- R4-2103828           CR to TS38.101-4: Introduction of PDSCH requirement with Single-DCI based SDM scheme

- R4-2103986           CR to TS38.101-4: Introduction of PDSCH requirement with Multi-DCI based multi-TRP

transmission schemes

- R4-2103827          CR to TS38.101-4: Performance requirements single-DCI based multi-TRP Repetition Tx schemes

- R4-2103833          CR to TS38.101-4: Introduction of PMI test cases with Rel-16 eType II codebook

- R4-2103954          CR to TS38.101-4: Applicability of PMI reporting test of eType II codebook

Agreement:

PDSCH requirement

- Abbreviation of TRP in 38.101-4:

  - Modify TRP to TRxP and add the abbreviation into sub clause 3.3 of 38.101-4 spec as ‘Transmission and Reception Point’

- FRC

  - FRC for single-DCI based FDM scheme A

    - It’s RAN4 common understanding, TBS determination for FRC of single DCI based FDM scheme A based on full resource allocation BW

  - FRC for single-DCI based inter-slot TDM scheme

    - Introduce new FRC for single-DCI based inter-slot TDM scheme with two TRS resources configuration every 20ms with additional note as

      - Throughput is calculated under the assumption of repetition number 2

    - Reference channel

      - R.PDSCH.1-11.2 FDD: sDCI based Inter-slot TDM in FDD

      - R.PDSCH.1-16.2 TDD: sDCI based Inter-slot TDM in TDD

- Test Parameters

  - Antenna port index for FDM scheme A

    - {1000,1001} DMRS antenna port index for TCI state#1 and TCI state#2

  - Antenna port index for inter-slot TDM scheme

    - {1000} DMRS antenna port index for TCI state#1 and TCI state#2

  - Note: For more details please refer to R4-2101449

- Requirement definition for 38.101-4

  - SNR= average of impairment results +extra margin

  - Extra margin

    - 64 QAM : 0.8 dB

    - 16 QAM : 0.5 dB

- PDSCH requirement derive

  - Introduce SNR requirements with [] based on the simulation summary R4-2100903 submitted in RAN4#98-e meeting

  - Companies are encouraged to further align the results and update the simulation results in RAN4#98bis-e meeting

    - In RAN4#98bis-e, for the test cases that the result span in ideal results among companies’ larger than [2.5]dB, RAN4 does not consider the farthest result(s) from the ideal AVERAGE value, until the span becomes within [2.5]dB. The final requirements are derived from AVERAGE impairment results with corresponding ideal results whose span is within [2.5]dB.

    - Note: The threshold, [2.5]dB, can be discussed in RAN4#98bis-e meeting

Agreement:

CSI requirements

- Introduce Rel-16 eType II codebook requirements with test metric as following PMI (eTypeII codebook)/Random PMI (Type I codebook)

- Test Parameters

  - Number of CSI-RS ports:

    - 16 ports with (N1,N2) = (4,2) and (O1,O2)=(4,4)

  - Number of PMI Sub-bands per CQI Sub-band

    - R = 1

  - Codebook parameter configuration

    - paramCombination-r16: 6, with L =4, pν =1/2, β=1/2

  - Sub-band Size:

    - 4 for FDD with 15kHz SCS, 10MHz CBW

    - 8 for TDD with 30kHz SCS, 40MHz CBW

  - MIMO correlation

    - XP Medium

  - Test point

    - 90% of TP

  - Channel Model

    - TDLA30-5

  - MCS and Rank

    - MCS 20 (64QAM Table), Rank 2

  - Beam-Steering Model

    - How to specify beam steering model into specification.

      - Same as specified in B.2.3B.4A of TS 36.101

  - Beam-Steering Approach

    - Enable two independent beam directions applying the beam for both eType II and Type I cookbook simulation

  - The remaining parameters will be same as for Rel-15 Type II codebook simulation assumptions in R4-2103846

- Gamma value

  - FDD

    - 16x2: [2.2]

    - 16x4: [2.2]

  - TDD

    - 16x2: [2.2]

    - 16x4: [2.2]

  - Introduce requirement with gamma value in [] based on simulation results summary R4-2100903 submitted in RAN4#98-e meeting.

  - Companies are encouraged to further align the results. Further update results and values in RAN4#98bis-e meeting not excluded

2.4.2	Remaining Open issues

- Core requirements (RF and RRM core)

Completed

- RRM performance part

Completed

- Demodulation and CSI performance requirement (performance part)

Completed

2.5	RAN5

n/a

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

n/a

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

n/a

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

[1] 3GPP, RAN1, RAN1#94bis Chairman Notes

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template