#3GPP TSG RAN Meeting #91-e						RP-210839

Electronic Meeting, March 16 – 26, 2021

______________________________________________________________________Agenda item: 9.6.2

Source: Moderator (LG Electronics)

Title: pCR for TR 38.845: Positioning requirements for V2X and public safety

Document for: Approval

- Introduction

This pCR proposes to add the positioning requirements for V2X and public safety use cases to TR 38.845 whose skeleton is provided in [1].

- References

- RP-210282, “Draft TR 38.845 v0.0.0,” LG Electronics.

- Proposed changes

*** Unchanged text is omitted ***

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905 : "Vocabulary for 3GPP Specifications".

[2]	3GPP RP-201518 : "Revised SID on Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases".

[3]	3GPP TS 22.261 : "Service requirements for the 5G system".

[4]	3GPP TS 22.186 : "Enhancement of 3GPP support for V2X scenarios".

[5]	3GPP RP-210040 : "Reply LS to RP-201390 on requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases," (source: 5GAA).

[6]	3GPP RP-210036 : "Reply LS to 3GPP TSG RAN on requirements of in-coverage, partial coverage and out-of-coverage positioning use cases," (source: SAE Advanced Applications Technical Committee)

[7]	3GPP TS 22.280 : "Mission Critical Services Common Requirements (MCCoRe)"

3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

absolute position: absolute position is an estimate of the UE position in 2D/3D geographic coordinates (e.g., latitude, longitude, elevation) within a coordinate system

relative position: relative position is an estimate of the UE position relative to other network elements or relative to other UEs

positioning service availability: percentage value of the amount of time the positioning service is delivering the required position-related data within the performance requirements, divided by the amount of time the system is expected to deliver the positioning service according to the specification in the targeted service area.

positioning service latency: time elapsed between the event that triggers the determination of the position-related data and the availability of the position-related data at the system interface.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GAA	5G Automotive Association

SAE AA TC	SAE Advanced Applications Technical Committee

# 4	Positioning use cases and requirements

## 4.0	Introduction

# This clause summarizes the positioning use cases and requirements defined for V2X and public safety in 3GPP specifications and input from other organizations. Sources used in this summary are provided in Annex A for information.

## 4.1	V2X

In 3GPP specifications, V2X positioning requirements can be found in [3] and [4]. TS 22.261 [3] specifies the high accuracy positioning requirements for the 5G system and these requirements are summarized in its Clause 7.3.2.2 with a note that these requirements include V2X. Seven different positioning service levels are defined in Table 7.3.2.2-1 [3] in terms of the horizontal and vertical accuracy, positioning service availability, and positioning service latency. TS 22.186 [4] specifies the relative lateral positioning requirement for general V2X use cases and the relative longitudinal positioning requirement for the platooning use case in its Clause 5.1 and 5.2.

5GAA provides positioning requirements for 58 V2X services that were summarized into three groups [5]; the first group with tens of meters accuracy, the second with lane level accuracy, and the third with sub-meter level accuracy. The positioning requirements can be on the 3D/2D coordinates (absolute position) or on the distance and/or angle (relative position) to a reference point, e.g. another UE.

SAE AA TC informs that it is necessary to have a 3GPP positioning technology which supports advanced V2X applications working even in various out-of-coverage scenarios [6].

It is observed that the positioning requirements in V2X depend on the service the UE operates. Also, the requirements are applicable to relative and absolute positioning depending on the use case or the positioning service level. In terms of the horizontal or lateral/longitudinal accuracy, the requirements for the absolute position or relative position can be categorised into three sets as follows by incorporating the requirements from the sources mentioned above:

- Set 1: 10 – 50 m with 68 – 95 % confidence level. This includes Group 1 in [5] and Service level 1 in [3].

- Set 2: 1 – 3 m with 95 – 99 % confidence level. This includes Group 2 in [5], Service level 2, 3, 4 in [3].

- Set 3: 0.1 – 0.5 m with 95 – 99 % confidence level. This includes Group 3 in [5], Service level 5, 6, 7 in [3], the requirements in [4].

Requirements for other performance metrics are also defined in a range depending on the positioning service level in [3]; 2 – 3 m (absolute) or 0.2 m (relative) vertical accuracy, 95 – 99.9% positioning service availability, 10 ms – 1 s positioning service latency.

Positioning service should be provided in indoor, outdoor, tunnel areas. The UE velocity up to 250 km/h needs to be supported for outdoor and tunnel areas. As long as the UE operates a V2X use case having the corresponding positioning requirements, the requirements should be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage. The requirements should be also fulfilled when the GNSS-based positioning is not available or not accurate enough.

## 4.2	Public safety

Public safety positioning requirements are defined in [3] and [7]. TS 22.261 [3] provides numerical positioning requirements for the “1st responders” use case in Table B.1-1 in [3]; 1 m horizontal accuracy, 2 m (absolute) or 0.3 m (relative) vertical accuracy, 95 – 98 % positioning service availability. TS 22.280 [6] specifies some qualitative positioning requirements in its Clause 5.11, 6.12, and 7.8. These requirements are applicable to both relative and absolute positioning.

Positioning service should be provided both in indoor and outdoor areas. As long as the UE operates a public safety use case having the corresponding positioning requirements, the requirements should be fulfilled when the UE is inside the network coverage as well as when it is outside the network coverage. The requirements should be also fulfilled when the GNSS-based positioning is not available or not accurate enough.

*** Unchanged text is omitted ***

Annex A:
Sources of positioning requirements for V2X and public safety

Sources in TS 22.261 [3]

The 5G system shall be able to provide positioning services with the performance requirements reported in Table 7.3.2.2-1.

NOTE: 	The requirements do not preclude any type of UE, including specific UE such as for example V2X, MTC.

Table 7.3.2.2-1 Performance requirements for Horizontal and Vertical positioning service levels

| Positioning service level | Absolute(A) or Relative(R) positioning | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Coverage, environment of use and UE velocity |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Horizontal Accuracy | Vertical Accuracy (note 1) |  |  | 5G positioning service area | 5G enhanced positioning service area (note 2) |  |
|  |  |  |  |  |  |  | Outdoor and tunnels | Indoor |
| 1 | A | 10 m | 3 m | 95 % | 1 s | Indoor - up to 30 km/h Outdoor (rural and urban) up to 250 km/h | NA | Indoor - up to 30 km/h |
| 2 | A | 3 m | 3 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 3 | A | 1 m | 2 m | 99 % | 1 s | Outdoor (rural and urban) up to 500 km/h for trains and up to 250 km/h for other vehicles | Outdoor (dense urban) up to 60 km/h Along roads up to 250 km/h and along railways up to 500 km/h | Indoor - up to 30 km/h |
| 4 | A | 1 m | 2 m | 99,9 % | 15 ms | NA | NA | Indoor - up to 30 km/h |
| 5 | A | 0,3 m | 2 m | 99 % | 1 s | Outdoor (rural) up to 250 km/h | Outdoor (dense urban) up to 60 km/h Along roads and along railways up to 250 km/h | Indoor - up to 30 km/h |
| 6 | A | 0,3 m | 2 m | 99,9 % | 10 ms | NA | Outdoor (dense urban) up to 60 km/h | Indoor - up to 30 km/h |
| 7 | R | 0,2 m | 0,2 m | 99 % | 1 s | Indoor and outdoor (rural, urban, dense urban) up to 30 km/h Relative positioning is between two UEs within 10 m of each other or between one UE and 5G positioning nodes within 10 m of each other (note 3) |  |  |
| NOTE 1: The objective for the vertical positioning requirement is to determine the floor for indoor use cases and to distinguish between superposed tracks for road and rail use cases (e.g. bridges). NOTE 2: Indoor includes location inside buildings such as offices, hospital, industrial buildings. NOTE 3: 5G positioning nodes are infrastructure equipment deployed in the service area to enhance positioning capabilities (e.g. beacons deployed on the perimeter of a rendezvous area or on the side of a warehouse). |  |  |  |  |  |  |  |  |

The 5G system shall be able to provide the 5G positioning services with a TTFF less than 30 s and, for some 5G positioning services, shall support mechanisms to provide a TTFF less than 10 s.

- NOTE 1: 	In some services, a TTFF of less than 10s may only be achievable at the expense of a relaxation of some other performances (e.g. horizontal accuracy may be 1 m or 3 m after 10 s TTFF, and reach a steady state accuracy of 0,3 m after 30 s).

The 5G system shall support a mechanism to determine the UE's velocity with a positioning service availability of 99%, an accuracy better than 0,5 m/s for the speed and an accuracy better than 5 degree for the 3-Dimension direction of travel.

The 5G system shall support a mechanism to determine the UE's heading with an accuracy better than 30 degrees (0,54 rad) and a positioning service availability of 99,9 % for static users and with an accuracy better than 10 degrees (0,17 rad) and a positioning service availability of 99 % for users up to 10 km/h.

The 5G system shall support positioning technologies that allow the UE to operate at Service Level 1 for at least 12 years using less than 1800 mWh of battery capacity, assuming multiple position updates per hour.

- NOTE 2:	This requirement aims energy-efficient positioning technologies draining a minimal energy on the UE battery. It derives from use cases, such as asset tracking, with a small form-factor battery representative of an IoT device. This requirement may translate into an energy consumption for the UE’s positioning functions in the order of 20 mJ per fix.

- NOTE 3:	This requirement does not preclude the use of higher energy consumption to fulfil higher position update rates than the one above, or other KPIs than those of Service Level 1 (e.g. more accurate service levels).

Table B.1-1 Typical needs to support example use cases from vertical industries.

| Use cases | Description | Main KPIs range and drivers |
| --- | --- | --- |
| 1st responders | Tracking and guidance of 1st responders, with requirements for high-accuracy in the horizontal domain and vertical domain, as well as accurate awareness of height variation to detect falls, combined to a high level of availability and reliability | Service area is both indoor and outdoor Accuracy: < 1m horizontal, < 2m vertical (indoor for floor detection) and < 0,3 m vertical (relative) to detect changes in height of the UE holder. Availability > 95 % (98% outdoor) Other KPI drivers include: MCX, confidence, event-triggered report |
| Road | Use cases involving road vehicles such as traffic monitoring, road-user charging (e.g. Road-Tolling, insurance mechanisms, etc.) which require positioning or tracking of vehicles at lane level, but also some awareness of position in the vertical domain (e.g. bridges) Service area is outdoor, but may include tunnels. | Accuracy: 1-3m horizontal (with <1m across-track for lane detection), <2,5 m vertical, Velocity < 2m/s, Availability: 95-99% Other KPI drivers include: tampering detection and prevention (typically for Road User Charging) |

Sources in TS 22.186 [4]

[R.5.1-007] The 3GPP system shall support relative lateral position accuracy of 0.1 m between UEs supporting V2X application.

[R.5.2-003] The 3GPP system shall support relative longitudinal position accuracy of less than 0.5 m for UEs supporting V2X application for platooning in proximity.

Sources in the input from 5GAA [5]

The positioning requirements according to the 5GAA TRs on Use Cases Vol. I and II can be summarize into three main groups, all at the highest speed a participating vehicle might be driving at, depending on the type of road environment and/or safety rules (e.g. 250kph on German Autobahn)

- Relatively lax, 10s of meters with 1 sigma (information provisioning)

- Traffic Jam Warning - Urban Scenario on Road Warning

- Traffic Jam Warning - Rural Scenario on Road Warning

- Traffic Jam Warning - Highway Scenario on Road Warning

- Rural Scenario on Route Information

- Highway Scenario on Route Information

- Software Update - Conventional-Routine/Urgent, Autonomous-Routine

- Software Update - Autonomous-Urgent

- Software Update - Without Infrastructure, Vehicle to Workshop

- Remote Automated Driving Cancellation

- HD Content Delivery - High-End Service for cars

- HD Content Delivery - Low-End Service for cars

- HD Content Delivery – Bus Passenger Service

- Software Update of Reconfigurable Radio System

- Patient Transport Monitoring

- Automated Valet Parking (Wake Up)

- Lane level accuracy, 1.5 m with 3 sigma (e.g. day1 safety use cases)

- Cross-Traffic Left-Turn Assist

- Intersection Movement Assist

- Emergency Break Warning

- Lane Change Warning - lagging vehicle, leading vehicle, Highway

- Lane Change Warning - lagging vehicle, leading vehicle, urban

- Lane Change Warning - not permitted case, rural

- Vehicle Health Monitoring

- Speed Harmonization

- See-Through for Pass Maneuver

- Obstructed View Assist via CCTV

- Obstructed View Assist via Remote Vehicles

- Continuous Traffic Flow via Green Lights Coordination

- Vehicle collects hazard and road event for AV

- Vehicles Platooning in Steady State

- Cooperative Lane Merge

- Autonomous Vehicle Disengagement Report

- Accident Report

- Awareness Confirmation

- Coordinated, Cooperative Driving Manoeuvre - Cooperative Lane Change

- Coordinated, Cooperative Driving Manoeuvre - Road Blockage

- Bus Lane Sharing Request

- Bus Lane Sharing Revocation

- Vehicle Decision Assist - RV Waiting for a Short Period of Time, RV Broken Down, Bus Having to Wait

- Vehicle Decision Assist - Slow Vehicle en Route

- Below meter level accuracy, e.g. 0.1 m with 3 sigma (e.g. automated driving or teleoperated driving)

- High Definition Sensor Sharing

- Vulnerable Road User - Awareness near potentially dangerous situations Urban

- Vulnerable Road User - Collision risk warning

- Real-Time Situational Awareness & High-Definition Maps

- Group Start

- Tele-Operated Driving (TOD)

- TOD support

- TOD for Automated Parking

- Cooperative Manoeuvres of Autonomous Vehicles for Emergency Situations

- High definition map collecting & sharing

- Automated Intersection crossing

- Infrastructure Assisted Environment Perception - Data Distribution about Objects on the Road

- Infrastructure Assisted Environment Perception - Individual Data Transmission in Form of Trajectories or Actuation Commands)

- Infrastructure based Tele-Operated Driving

- Automated Valet Parking – Joint Authentication and Proof of Localisation

- Coordinated, Cooperative Driving Manoeuvre - Pedestrian Crossing

- Cooperative Traffic gap

- Cooperative Lateral Parking

- Cooperative Curbside Management

Sources in 3GPP TS 22.280 [7]

[R-5.11-001] The MCX Service shall support obtaining and conveying Location information describing the position of the MCX UE.

[R-5.11-002] The MCX Service should support obtaining and conveying high accuracy Location information describing the position of the MCX UE.

[R-5.11-002a] The MCX Service shall be able to provide a mechanism for obtaining high accuracy Location information by integrating position information from multiple external sources (e.g. magnetometers, orientation sensors, GNSS)

[R-5.11-003] The MCX Service shall provide for the flexibility to convey future formats of Location information.

[R-5.11-004] The MCX Service shall provide a means for MCX Service Administrators to manage the privacy of Location information for MCX Users within their authority.

[R-5.11-005] An authorized MCX User shall be able to control the supplying of Location information by the MCX UE for MCX Service communications.

[R-5.11-006] The conveyed Location information shall be the most recently obtained information about the position of the MCX UE at the time of the Location information conveyance.

[R-5.11-007] The MCX Service shall be capable of configuring and re-configuring one or more Location information update triggers (i.e., identified conditions that, when satisfied, cause the MCX UE to report its current Location information).

[R-5.11-008] The MCX Service shall be able to modify Location information update triggers of an MCX User while the MCX User is on the network.

[R-5.11-009] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever a trigger condition is satisfied (e.g., initial registration, distance travelled, elapsed time, cell change, tracking area change, PLMN change, MCX Service communication initiation).

[R-5.11-010] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever the MCX User initiates an MCX Service Emergency Alert.

[R-5.11-011] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever the MCX User initiates an MCX Service Emergency Group Communication.

[R-5.11-012] The MCX Service shall provide a means for an MCX UE to send a Location information update if the MCX User is in an MCX Service Emergency State and a configured amount of time has passed since the previous location information update.

[R-5.11-013] The MCX Service shall provide a means for an MCX UE to send a Location information update whenever a trigger condition is satisfied while the MCX User is in MCX Service Emergency State (e.g., initial registration, distance travelled, elapsed time, cell change, tracking area change, PLMN change, MCX Service communication initiation).

NOTE 1:	The Location information update triggers for an MCX User in an MCX Service Emergency State might be different than the Location information update triggers used when the MCX User is not in an MCX Service Emergency State.

[R-5.11-014] The MCX Service shall provide a means for an MCX Service Administrator to define geographical areas to be used for Location information update triggers for MCX Users within their authority.

[R-5.11-015] The MCX Service shall provide a means for an MCX UE in a predefined area to send a Location information update whenever a trigger condition configured in an MCX User's active MCX Service User Profile is satisfied (e.g., initial registration, distance travelled, elapsed time, cell change, tracking area change, PLMN change, MCX Service communication initiation).

NOTE 2:	The Location information update triggers for an MCX User in a predefined area might be different than the Location information update triggers used when the MCX User is not in a predefined area.

[R-6.12-001] The MCX Service shall provide Location information of the transmitting MCX UE to receiving MCX UEs subject to privacy restrictions.

[R-6.12-002] The MCX Service shall support conveyance of Location information provided by 3GPP location services.

[R-6.12-003] The MCX Service shall provide a means for an authorized MCX User to restrict the dissemination of his Location information.

[R-6.12-004] The MCX Service shall provide end-to-end confidentiality of Location information.

[R-6.12-005] The MCX Service shall provide authentication of messages carrying Location information.

[R-6.12-006] The MCX Service shall provide a means for an authorized MCX User to activate a one-time Location information report of an MCX User and periodic Location information update reports of an MCX User or a specific Functional Alias.

[R-6.12-007] The MCX Service shall provide a means for an authorized MCX User to deactivate periodic Location information update report of an MCX User.

[R-7.8-001] An MCX UE shall be capable of transmitting its Location, if known, to other MCX UEs when operating off the network, subject to privacy restrictions.

[R-7.8-002] An MCX UE shall be capable of providing a mechanism for obtaining the distance between the MCX UE and other MCX UEs within communication range.

[R-7.8-003] The Off-Network MCX Service shall provide a means for an authorized MCX User to activate a one-time Location information report of a particular target MCX User within communication range.

*** Unchanged text is omitted ***