3GPP TSG RAN#91e	RP-21xxxx

e-Meeting, March 16th – 26th, 2021

Agenda item:

Source: 	3GPP TSG RAN1 Chairman

Title: 	Email Summary on the scope of Coverage Enhancements WI

Document for:	Discussion/Decision

# Introduction

In this document, we will provide a summary on the scope of Rel-17 Coverage Enhancements WI discussion, using the following contributions as a starting point:

- RP-210455	Revised WID on NR coverage enhancements	China Telecom

- RP-210499	Views on the scope of NR coverage enhancements WI	Apple Inc.

- RP-210602	Views on PUSCH and Msg3 enhancements for Redcap UEs in NR CE WI	ZTE, Sanechips

# Discussion

The email discussion is organized as follows:

- Initial Round till Tuesday 23rd March 11:59h UTC: Final deadline for comments on Initial email discussions

- Intermediate Round till

  - Wednesday 24th March 10:59h UTC: Deadline for comments on Intermediate Summaries

  - Thursday 25th March 11:59h UTC: Final deadline for technical comments

- Final Round till Friday 26th March 10:59h UTC: Deadline for final comments

## Initial Round

RP-210455 has the following proposed update (along with other editorial updates)

- Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

  - Based on the mechanism(s) specified to enable joint channel estimation for PUSCH

RP-210499 has the following proposal:

- PUSCH/Msg3 coverage recovery for Redcap device is to be handled under Rel-17 NR coverage enhancement WI.

RP-210602 has the following proposal:

- Proposal 1: The PUSCH enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific PUSCH enhancements are needed.

- Proposal 2: Msg3 PUSCH repetition based enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific Msg3 enhancements are needed.

- Proposal 3: Specify UE features for Rel-17 CE PUSCH and Msg3 enhancements for RedCap UEs in WI phase.

- Proposal 4: Discuss whether/how to differentiate between Rel-17 CE UEs and Redcap UEs before Msg3 transmission in WI phase.

- Proposal 5: Add one note in CE WID to clarify that coverage recovery on PUSCH and Msg3 for RedCap UEs are included in Rel-17 CE WI.

Note that proposal 5 above discusses a similar issue as in RP-210499.

### DM-RS Bundling for PUCCH

Question:

- Is the update in RP-210455 agreeable? (copied below for easy reference)

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - Based on the mechanism(s) specified to enable joint channel estimation for PUSCH

| Company | Views |
| --- | --- |
| Nokia, NSB | The proposed sentence is a bit too strong in our view. One alternative is to say “Based on similar mechanism(s) as specified to enable joint channel estimation for PUSCH” |
| Ericsson | We agree that PUCCH DMRS bundling can be based on PUSCH joint channel estimation. We’re not sure if ‘specified’ is the most appropriate wording here, though. We expect this should not mean that RAN1 and/or RAN4 must first agree to a PUSCH joint channel estimation mechanism before deciding or discussing the PUCCH mechanism. Would it be more clear to say - Based on the mechanism(s) specified to enable joint channel estimation for PUSCH |
| Intel | It is not clear to us whether all the solutions for joint channel estimation specified for PUSCH enhancement would apply for PUCCH enhancement. For instance, DMRS optimization, which is under the discussion for PUSCH enhancement, is already deprioritized for PUCCH enhancement. Our view is that even without this sub-bullet, RAN1 still proceeds along with the same direction for PUCCH enhancement. So we think this update may not be needed. |
| vivo | In our understanding similar principle including restrictions/requirements on gNB scheduling/UE behavior to enable DMRS bundling for PUSCH is applicable for enabling joint channel estimation across PUCCH repetition. As Intel commented RAN1 proceeds along the same direction, however some clarification would be helpful, e.g. based on similar principle to enable joint channel estimation for PUSCH. |
| CMCC | We share a similar idea that the mechanisms discussed in PUSCH joint channel estimation can be reused in the PUCCH DMRS bundling, which could avoid duplicated work and smooth the discussions. Considering the PUCCH could not fully reuse the specification of PUSCH joint channel estimation, the update wording is proposed as below. Reuse the mechanism(s) specified to enable joint channel estimation for PUSCH as much as possible. |
| DOCOMO | We support the update |
| CATT | We share the same view that the mechanisms studied for PUSCH JCE can be applied to PUCCH DMRS bundling as well. In our understanding, the scheduling restrictions, the conditions to keep phase continuity and the potential use cases are common for PUSCH JCE and PUCCH DMRS bundling. We support the update. |
| InterDigital | We agree with Intel that this update may not be needed although we understand the intention of the proposed bullet. Whether PUCCH can incorporate PUSCH DMRS bundling mechanism will become clearer as the RAN1 discussion progresses. |
| Qualcomm | We agree with the Nokia input. Having commonality with the solution for PUSCH is a good thing but making the PUCCH and PUSCH solutions identical may not be possible or beneficial. |
| Apple | The followings were agreed in RAN1#104-e meeting Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling. - FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16. - Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible So same design principle could apply to DMRS bundling without frequency hopping. The clarification to WID is helpful, the updates from CMCC are fine for us. |

Proposals:

- TBD

### Coverage Recovery for RedCap

Question:

- Should be consider adding the following note to the CovEnh WI:

  - Note: for PUSCH enhancement and Msg3 PUSCH repetition, the 3dB coverage recovery for RedCap UE is to be considered.

| Company | Views |
| --- | --- |
| Nokia, NSB | This depends on the outcome of email thread [38][RedCap_scope], and it is too early to consider any such addition in CovEnh WI. In any case, CovEnh WI is working on solutions that are generic enough to be incorporated by future RedCap devices, if so desired, and hence we foresee no need for specific optimizations to be done here. |
| Ericsson | We think it is premature to add RedCap coverage recovery specific considerations to NR coverage enhancement. There is no agreement yet in RedCap that 3 dB coverage recovery (e.g. motivated by antenna efficiency) is needed. Furthermore, our understanding is that Rel-15/16 Msg3 retransmission already provides coverage and that Msg3 repetition in coverage enhancement is more motivated for PDCCH overhead and/or latency considerations. |
| Intel | Agree with the comments that this proposal is now pre-mature until the consideration of antenna efficiency relaxation is resolved as part of [RedCap_scope] email discussion. While the general applicability of CE features to RedCap UEs need to be addressed, this is mainly subject of the next discussion point (1.1.3) and the “3dB coverage recovery for RedCap” would need to wait until resolved in RedCap discussions. It should be noted that currently, in the Rel-17 WI on CE, the CE features are being developed without particular absolute coverage target(s), and thus, the “3dB coverage recovery” may not even be of much relevance if the same coverage enhancement solution is to be made available for non-RedCap and RedCap UEs. |
| FUTUREWEI | This enh should be able to provide at least 3dB, no need for a special note. RedCap can decide later whether any of the Rel-17 CE features are mandatory, optional, etc. There is a good chance that at least some RedCap UEs would want to use this feature. |
| vivo | Coverage enhancement include recovery for RedCap can be discussed in CE and the schemes specified for coverage enhancement are equally applicable for RedCap. Regarding “3dB” value for coverage recovery depends on outcome of RedCap discussion |
| CMCC | No strong need to add this note, or update the notes as “coverage enhancement mechanisms can be reused for the coverage recovery of RedCap UEs”, as the repetition based mechanisms could satisfy the “3 dB” requirement. In addition, the coverage gap such as 3 dB should be based on the agreements or the output from RedCap. |
| DOCOMO | We support to add the note in principle. As it depends on the outcome of the discussion on the number of Rx antenna ports in [91E][38][RedCap_scope] (e.g., whether 3dB coverage recovery due to antenna efficiency loss is necessary to be considered for FR1 TDD bands), fine tuning may be necessary when the issue is resolved. |
| CATT | Coverage enhancement mechanisms is generic enough and can be applied to RedCap UEs(which is related to the discussion under section 1.1.3). If the intention is not to define some RedCap-specific solutions, it would be more reasonable to capture the note in RedCap WID such as ‘the coverage enhancement mechanisms can be reused’. Furthermore, the statement ‘the 3dB coverage recovery for RedCap UE is to be considered’ is quite unclear on how to consider the 3 dB gap considering we don’t have an absolute target even for coverage enhancement. |
| Qualcomm | Ok to add the note above for R17 RedCap; however, we suggest revising the wording as: - Note: for coverage recovery of PUSCH/Msg3, the 3dB loss of transmit antenna efficiency for RedCap UE is to be considered. |
| Apple | The coverage issue for PUSH and Msg3 was already identified in the TR38.875 due to 3dB antenna efficiency loss of wearable device. In the coverage evaluation, the msg3 is the bottleneck for the scenario of Rural 700MHz FDD NLOS O2I, it’s 4.49dB coverage gap to the target MCL. For RedCap devices, the gap is about 7.5dB. We think it’s the common understanding that the schemes specified under coverage enhancement WI could be reused by RedCap device. In addition, if the additional 3dB loss is not compensated, the coverage gap is still there between RedCap device and Rel.17 CovEnh UE. Note: Now, there is no coverage recovery discussion under email thread [38][RedCap_scope]. |

Proposals:

- TBD

### Features for CE UEs vs. RedCap UEs

Questions: Regarding the following three proposals

- The PUSCH enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific PUSCH enhancements are needed.

- Msg3 PUSCH repetition based enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific Msg3 enhancements are needed.

- Specify UE features for Rel-17 CE PUSCH and Msg3 enhancements for RedCap UEs in WI phase.

It seems that these can be further discussed in a later part of the release, instead of aiming for a conclusion now? Any thoughts?

| Company | Views |
| --- | --- |
| Nokia, NSB | Similarly as in the question in section 1.1.2, this depends on further details of RedCap scope and it is difficult to make a conclusion now. It should be noted though that a conclusion on first two bullets seem to imply no need for adding objectives to CovEnh WID regarding RedCap UEs, which would be fine too. |
| Ericsson | Agree with the moderator's suggestion to discuss these aspects later in the release. We'd like to stress that UL coverage recovery for RedCap UEs may not be needed unless RAN decides to relax the antenna efficiency for RedCap wearables. Therefore, aspects such as UE capability and if PUSCH enhancements or Msg3 repetition for Cov Enh are required for RedCap should be FFS for now. |
| Intel | While we agree with the spirit of the three bullets, they should be left FFS for now. For now, if we’d prefer to say something, an alternative could be to capture a note as following: - Note: It is expected that PUSCH and Msg3 repetition-based enhancements, to be specified for CE UEs, can be supported for RedCap UEs. The applicability of the CE solutions and corresponding UE features for RedCap UEs to be addressed during the WI phase in either RedCap or CE WIs (FFS at RAN #92-E). |
| FUTUREWEI | OK to discuss later or have a note as Intel suggests. |
| vivo | In general agree with three bullets, it can be discussed in later phase as moderator’s suggestion. |
| CMCC | Agree with moderator’s suggestion to discuss this later. Whether the Redcap UE should support mechanisms of CE as mandate or based on the claim of UE capability more depends on the conclusions and the detailed designs of RedCap. At least, the mechanism of CE could be reused for Redcap. Once the Redcap UE claims that they are CE UEs, the solutions should apply to them. |
| DOCOMO | We support the suggestion that these can be discussed in later part of the WI phase, as it is still unclear whether PUSCH/Msg3 enhancements to be specified in CovEnh WI can be applied to RedCap UEs without any adjustments or any adjustments for RedCap UEs are necessary. |
| CATT | Agree with moderator that it is too early to have a conclusion. It should be discussed later under the umbrella of RedCap. |
| InterDigital | Agree with moderator’s suggestion to discuss it in the later release. |
| Qualcomm | For UL coverage recovery of PUSCH/msg3, we think RedCap UE can use the solution(s) developed for non-RedCap UEs in the R17 CovEnh WI. We are ok with keeping the 3rd bullet since it can cover the 3dB antenna efficiency loss for RedCap UEs. |
| Apple | In General, we agree the same mechanism defined in CE WI can be applied to RedCap devices. What we are concerning about is how to quantify the value range of number of repetitions, we prefer it should be determined based on link budget gap from RedCap devices rather than from eMBB devices, i.e., taking into account 3dB antenna efficiency loss in link budget gap. |

Proposals:

- TBD

### Differentiation between CE and RedCap UEs before Msg3 Tx

Question:

- Is there a need to differentiate between Rel-17 CE UEs and Redcap UEs before Msg3 transmission in WI phase? If so, how? Should we clarify it accordingly (in CE or RedCap WI?)

| Company | Views |
| --- | --- |
| Nokia, NSB | Any related discussion should take place in [38][RedCap_scope], as it is not a coverage enhancement aspect in itself. |
| Ericsson | There is already discussion in Cov Enh on differentiating UE support for Msg3 and in RedCap on early UE identification before Msg3 transmission. While solutions can be similar, these are somewhat different issues, and can be considered in parallel in the two different WIs. If it is found that differentiation is needed, any incompatibilities between identified solutions in RedCap and Cov Enh can be resolved at that time. However, it should be kept in mind in the RedCap and Cov Enh discussions that UEs may support both RedCap and Cov Enh, and so if resources such as PRACH are used to independently indicate support for either feature, the overhead will be proportionately higher. |
| Intel | We think this is also related to the discussion in RedCap whether/how early identification is needed to differentiate RedCap UEs and non-RedCap UEs. In our view, this should be discussed under [RedCap_scope]. |
| FUTUREWEI | If both early identification and this CE feature are supported, there could be the issue Ericsson mentions. Perhaps it could be resolved later as part of the applicability discussion whether some small modification is needed. |
| vivo | Agree with Nokia’s comment, it should be discussed in RedCap_scope thread. A general comment, we do see potential issue as Ericsson mentioned above and should be carefully studied to avoid excessive overhead or lower system efficiency. |
| CMCC | This is a similar issue as 1.1.3 which is considering the combination of features from two separate WIs. We could discuss this later when the Redcap have a clear conclusion on when to identify a RedCap UE. |
| DOCOMO | Yes, we think differentiation is necessary between Rel-17 CE UEs and RedCap UEs (and also legacy UEs) before Msg2/3 transmission. Coverage recovery for Msg2 is necessary for RedCap UEs in FR1 TDD bands while it may not be necessary for legacy UEs or Rel-17 CE UEs. The amount of coverage enhancement/recovery for Msg3 may be different between Rel-17 CE UEs and RedCap UEs. Early identification of these UEs should be specified either CE or RedCap WI. We are fine with either WI as long as overlapping is avoided. |
| CATT | Share the same view with Nokia. |
| InterDigital | Agree with Nokia |
| Qualcomm | We agree with the comment of Nokia. For RedCap UEs, we think early indication before msg3 is necessary, because NW needs to identify (at least) the following aspects during initial access: - reduced BW (for DL and UL) - reduced DL coverage (for msg2 PDCCH/PDSCH) - reduced UL coverage (for msg3) Considering 1) and 2) are not the objectives of the UL CovEnh WI for non-RedCap UE, we do see a need to differentiate R17 RedCap UE and R17 CE for non-RedCap UE before msg3 transmission, and this is not purely a UL CovEnh aspect. |
| Apple | We share the view that this issue should be discussed under RedCap scope. |

Proposals:

- TBD

## Intermediate Round

## Final Round

# Conclusion

Based on the email discussion, the following are proposed:

TBD

# References

RP-210455	Revised WID on NR coverage enhancements	China Telecom

RP-210499	Views on the scope of NR coverage enhancements WI	Apple Inc.

RP-210602	Views on PUSCH and Msg3 enhancements for Redcap UEs in NR CE WI	ZTE, Sanechips