3GPP TSG RAN#91e	RP-210867

e-Meeting, March 16th – 26th, 2021

Agenda item:

Source: 	3GPP TSG RAN1 Chair

Title: 	Email Summary on the scope of Coverage Enhancements WI

Document for:	Discussion/Decision

# Introduction

In this document, we will provide a summary on the scope of Rel-17 Coverage Enhancements WI discussion, using the following contributions as a starting point:

- RP-210455	Revised WID on NR coverage enhancements	China Telecom

- RP-210499	Views on the scope of NR coverage enhancements WI	Apple Inc.

- RP-210602	Views on PUSCH and Msg3 enhancements for Redcap UEs in NR CE WI	ZTE, Sanechips

# Discussion

The email discussion is organized as follows:

- Initial Round till Tuesday 23rd March 11:59h UTC: Final deadline for comments on Initial email discussions

- Intermediate Round till

  - Wednesday 24th March 10:59h UTC: Deadline for comments on Intermediate Summaries

  - Thursday 25th March 11:59h UTC: Final deadline for technical comments

- Final Round till Friday 26th March 10:59h UTC: Deadline for final comments

## Initial Round

RP-210455 has the following proposed update (along with other editorial updates)

- Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

  - Based on the mechanism(s) specified to enable joint channel estimation for PUSCH

RP-210499 has the following proposal:

- PUSCH/Msg3 coverage recovery for Redcap device is to be handled under Rel-17 NR coverage enhancement WI.

RP-210602 has the following proposal:

- Proposal 1: The PUSCH enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific PUSCH enhancements are needed.

- Proposal 2: Msg3 PUSCH repetition based enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific Msg3 enhancements are needed.

- Proposal 3: Specify UE features for Rel-17 CE PUSCH and Msg3 enhancements for RedCap UEs in WI phase.

- Proposal 4: Discuss whether/how to differentiate between Rel-17 CE UEs and Redcap UEs before Msg3 transmission in WI phase.

- Proposal 5: Add one note in CE WID to clarify that coverage recovery on PUSCH and Msg3 for RedCap UEs are included in Rel-17 CE WI.

Note that proposal 5 above discusses a similar issue as in RP-210499.

### DM-RS Bundling for PUCCH

Question:

- Is the update in RP-210455 agreeable? (copied below for easy reference)

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - Based on the mechanism(s) specified to enable joint channel estimation for PUSCH

| Company | Views |
| --- | --- |
| Nokia, NSB | The proposed sentence is a bit too strong in our view. One alternative is to say “Based on similar mechanism(s) as specified to enable joint channel estimation for PUSCH” |
| Ericsson | We agree that PUCCH DMRS bundling can be based on PUSCH joint channel estimation. We’re not sure if ‘specified’ is the most appropriate wording here, though. We expect this should not mean that RAN1 and/or RAN4 must first agree to a PUSCH joint channel estimation mechanism before deciding or discussing the PUCCH mechanism. Would it be more clear to say - Based on the mechanism(s) specified to enable joint channel estimation for PUSCH |
| Intel | It is not clear to us whether all the solutions for joint channel estimation specified for PUSCH enhancement would apply for PUCCH enhancement. For instance, DMRS optimization, which is under the discussion for PUSCH enhancement, is already deprioritized for PUCCH enhancement. Our view is that even without this sub-bullet, RAN1 still proceeds along with the same direction for PUCCH enhancement. So we think this update may not be needed. |
| vivo | In our understanding similar principle including restrictions/requirements on gNB scheduling/UE behavior to enable DMRS bundling for PUSCH is applicable for enabling joint channel estimation across PUCCH repetition. As Intel commented RAN1 proceeds along the same direction, however some clarification would be helpful, e.g. based on similar principle to enable joint channel estimation for PUSCH. |
| CMCC | We share a similar idea that the mechanisms discussed in PUSCH joint channel estimation can be reused in the PUCCH DMRS bundling, which could avoid duplicated work and smooth the discussions. Considering the PUCCH could not fully reuse the specification of PUSCH joint channel estimation, the update wording is proposed as below. Reuse the mechanism(s) specified to enable joint channel estimation for PUSCH as much as possible. |
| DOCOMO | We support the update |
| CATT | We share the same view that the mechanisms studied for PUSCH JCE can be applied to PUCCH DMRS bundling as well. In our understanding, the scheduling restrictions, the conditions to keep phase continuity and the potential use cases are common for PUSCH JCE and PUCCH DMRS bundling. We support the update. |
| InterDigital | We agree with Intel that this update may not be needed although we understand the intention of the proposed bullet. Whether PUCCH can incorporate PUSCH DMRS bundling mechanism will become clearer as the RAN1 discussion progresses. |
| Qualcomm | We agree with the Nokia input. Having commonality with the solution for PUSCH is a good thing but making the PUCCH and PUSCH solutions identical may not be possible or beneficial. |
| Apple | The followings were agreed in RAN1#104-e meeting Agreements: Subject to the prerequisite of DMRS bundling for PUCCH repetitions, enhance inter-slot frequency hopping pattern for PUCCH repetitions with DMRS bundling. - FFS: details in inter-slot frequency hopping pattern enhancement, e.g., additional frequency hopping patterns than Rel-16. - Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible So same design principle could apply to DMRS bundling without frequency hopping. The clarification to WID is helpful, the updates from CMCC are fine for us. |
| ZTE | The update is fine for us. We share with above companies that there is no much difference between the mechanisms for enabling joint channel estimation for PUSCH and DMRS bundling for PUCCH. But, we’d like to clarify what would the update impact the WG discussion. Is a correct understanding that no discussion would be needed for DMRS bundling for PUCCH unless some PUCCH specific issues are identified (e.g., specific signaling/capabilities at a later stage)? |
| Samsung | It is understood that the intention is to maximize the commonality of joint channel estimation between PUSCH and PUCCH. Like other comments above, it would be better to soften the wording. |
| Lenovo, Motorola Mobility | Yes, we agree with the moderator proposal that similar mechanisms for joint channel estimation as for PUSCH should be specified for PUCCH as well |
| OPPO | We are open to support similar solutions from PUSCH. However, the proposed bullet is very restrictive and may not be fully realized by PUCCH structure. Thus we would prefer to use the similar mechanism as from PUSCH if possible. |
| MediaTek | We are fine with the update. |
| Sharp | We generally agree with the proposed direction that mechanism to support DMRS bundling across PUCCH repetitions should be based on the ones to enable joint channel estimation for PUSCH. In fact, RAN1 has already made some agreement (e.g. “Strive for common design for PUSCH/PUCCH with DMRS bundling as much as possible.”) along with this direction. However, as pointed out by Intel, we can’t say that all the solutions for PUSCH are applicable to PUCCH. Therefore, we are wondering if the proposed sentence needs to be captured in the WID. |
| Panasonic | The same principle should be used but may not be necessary to be exactly same. Therefore, wording suggested by vivo looks reasonable. |
| Xiaomi | We agree with Nokia’s and Vivo’s view. It is better to say “Based on similar mechanism(s) as specified to enable joint channel estimation for PUSCH”. On the one hand, we are not sure all the solutions for joint channel estimation for PUSCH enhancement would apply for PUCCH enhancement, such as DMRS pattern, time window size or configuration granularity. For example, joint channel estimation may be triggered and configured per UE for PUSCH, while per PUCCH format configuration for PUCCH DMRS bundling. On the other hand, some similar principle can be used to enable DMRS bundling for PUSCH and joint channel estimation across PUCCH repetition. So, we can reuse the mechanism(s) specified to enable joint channel estimation for PUSCH as much as possible. |
| Huawei, HiSilicon | Considering the discussion above, it seems that adding this sentence now may be too early, because the detail of the PUSCH technique(s) are not yet settled in RAN1, and there are detail differences where PUSCH is scheduled but PUCCH is based on RRC configuration. This makes it tough to come up with suitable wording for a WID. In the end, such a ‘clarification’ may not give any additional guidance to RAN1. |

It seems that it is a general understanding that similar mechanisms for joint channel estimation for PUSCH and PUCCH should be used. A majority of companies think that some update of the WID is necessary to clarify this direction, while there are a few companies believe it’s not necessary or too early (since more details need to be developed in RAN1). Based on the comments so far, the following is proposed:

Proposals:

Update the CovEnh WID as follows:

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

### Coverage Recovery for RedCap

Question:

- Should be consider adding the following note to the CovEnh WI:

  - Note: for PUSCH enhancement and Msg3 PUSCH repetition, the 3dB coverage recovery for RedCap UE is to be considered.

| Company | Views |
| --- | --- |
| Nokia, NSB | This depends on the outcome of email thread [38][RedCap_scope], and it is too early to consider any such addition in CovEnh WI. In any case, CovEnh WI is working on solutions that are generic enough to be incorporated by future RedCap devices, if so desired, and hence we foresee no need for specific optimizations to be done here. |
| Ericsson | We think it is premature to add RedCap coverage recovery specific considerations to NR coverage enhancement. There is no agreement yet in RedCap that 3 dB coverage recovery (e.g. motivated by antenna efficiency) is needed. Furthermore, our understanding is that Rel-15/16 Msg3 retransmission already provides coverage and that Msg3 repetition in coverage enhancement is more motivated for PDCCH overhead and/or latency considerations. |
| Intel | Agree with the comments that this proposal is now pre-mature until the consideration of antenna efficiency relaxation is resolved as part of [RedCap_scope] email discussion. While the general applicability of CE features to RedCap UEs need to be addressed, this is mainly subject of the next discussion point (1.1.3) and the “3dB coverage recovery for RedCap” would need to wait until resolved in RedCap discussions. It should be noted that currently, in the Rel-17 WI on CE, the CE features are being developed without particular absolute coverage target(s), and thus, the “3dB coverage recovery” may not even be of much relevance if the same coverage enhancement solution is to be made available for non-RedCap and RedCap UEs. |
| FUTUREWEI | This enh should be able to provide at least 3dB, no need for a special note. RedCap can decide later whether any of the Rel-17 CE features are mandatory, optional, etc. There is a good chance that at least some RedCap UEs would want to use this feature. |
| vivo | Coverage enhancement include recovery for RedCap can be discussed in CE and the schemes specified for coverage enhancement are equally applicable for RedCap. Regarding “3dB” value for coverage recovery depends on outcome of RedCap discussion |
| CMCC | No strong need to add this note, or update the notes as “coverage enhancement mechanisms can be reused for the coverage recovery of RedCap UEs”, as the repetition based mechanisms could satisfy the “3 dB” requirement. In addition, the coverage gap such as 3 dB should be based on the agreements or the output from RedCap. |
| DOCOMO | We support to add the note in principle. As it depends on the outcome of the discussion on the number of Rx antenna ports in [91E][38][RedCap_scope] (e.g., whether 3dB coverage recovery due to antenna efficiency loss is necessary to be considered for FR1 TDD bands), fine tuning may be necessary when the issue is resolved. |
| CATT | Coverage enhancement mechanisms is generic enough and can be applied to RedCap UEs(which is related to the discussion under section 1.1.3). If the intention is not to define some RedCap-specific solutions, it would be more reasonable to capture the note in RedCap WID such as ‘the coverage enhancement mechanisms can be reused’. Furthermore, the statement ‘the 3dB coverage recovery for RedCap UE is to be considered’ is quite unclear on how to consider the 3 dB gap considering we don’t have an absolute target even for coverage enhancement. |
| Qualcomm | Ok to add the note above for R17 RedCap; however, we suggest revising the wording as: - Note: for coverage recovery of PUSCH/Msg3, the 3dB loss of transmit antenna efficiency for RedCap UE is to be considered. |
| Apple | The coverage issue for PUSH and Msg3 was already identified in the TR38.875 due to 3dB antenna efficiency loss of wearable device. In the coverage evaluation, the msg3 is the bottleneck for the scenario of Rural 700MHz FDD NLOS O2I, it’s 4.49dB coverage gap to the target MCL. For RedCap devices, the gap is about 7.5dB. We think it’s the common understanding that the schemes specified under coverage enhancement WI could be reused by RedCap device. In addition, if the additional 3dB loss is not compensated, the coverage gap is still there between RedCap device and Rel.17 CovEnh UE. Note: Now, there is no coverage recovery discussion under email thread [38][RedCap_scope]. |
| ZTE | Support the proposed note. According to output of RedCap SI, 3dB coverage recovery of PUSCH and Msg3 for RedCap UE is needed. It’s our understanding that the discussion on the number of Rx branches in [91E][38][RedCap_scope] only has impacts on DL coverage while not for UL. If companies have concerns on adding an absolute target value, we would be also ok to delete ‘3 dB’ in the note. We would also like to highlight that coverage recovery is currently not discussed under [38][RedCap_scope]. |
| Samsung | No need to add the proposed note: The solutions developed under coverage enhancement WI can be also applicable for RedCap UE. On the other hand, any RedCap UE specific optimization can be discussed under RedCap WI. |
| Lenovo, Motorola Mobility | This should be discussed after conclusion in [RedCap_scope] email discussion. Generally, techniques specified for Coverage Enhancement can be reused for RedCap. |
| OPPO | The 3dB coverage recovery for UL is quite important to us, as the study phase already identify the need. In the current coverage enhancement WI, there is no target and seems the enhancement will be lower than the target. Thus, it is very useful to state the target for the PUSCH channel. We support he added bullet. |
| MediaTek | There is no need to consider the 3dB antenna inefficiency in the CE WI. The 3dB is used in RAN1 for evaluation purposes, to compare the coverage of RedCap UE and legacy NR UEs. It is expected that the CE WI will introduce enhancements that goes beyond this (3dB) antenna inefficiency gap. |
| Sharp | We also think it is premature to add the proposed note and prefer to wait for the outcomes from [RedCap_scope] discussions. Moreover, in our view CovEnh WID does not need to state any linkage between RedCap UEs and any specific function developed in this WI. |
| Panasonic | The solution developed in CovEnh WI can be applicable to RedCap. There is no need to have explicit text. |
| Xiaomi | We support to add the note. Actually we are more interested in how to handle the antenna efficiency gain and how to capture the description in the WID of CE. In our understanding, it also depends on which release (R15/R16 or R17) is the target coverage performance for Redcap. If the coverage of R15 UE is the target, then maybe the solution and the number of repetition set in CE is sufficient. While, on the other hand, if the coverage of R17 UE is the target, then there is certain impact on the determination of the maximum number of repetitions when we consider antenna efficiency loss. |
| Huawei, HiSilicon | According to the RedCap WID: “the appropriate WI for handling of any potential coverage recovery aspects related to RedCap UEs devices will be considered at RAN#91e”. we suggest the following - Note: for PUSCH enhancement and Msg3 PUSCH repetition, the 3dB potential coverage recovery for RedCap UE is to be considered. |

It seems that the views on adding the notes are diverging. Some commented that it’s premature as it is also related to the scope of RedCap WI discussion, some commented that there is no such a need for the note, while other supported adding the note (with some variations of the wording). It seems that there is a general understanding that the coverage enhancement techniques developed in CovEnh WI can be applicable to RedCap UEs. Considering that the RedCap scope is still being finalized, it seems better to revisit this topic in the next RAN plenary.

Proposed conclusion:

- It is generally understood that the techniques developed in the CovEnh WI can be applicable to RedCap UEs

- Revisit in RAN#92-e whether or not to add a note in CovEnh WID regarding the issue of potential coverage recovery for PUSCH enhancement and Msg 3 PUSCH repetition for RedCap UEs raised in RP-210602

### Features for CE UEs vs. RedCap UEs

Questions: Regarding the following three proposals

- The PUSCH enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific PUSCH enhancements are needed.

- Msg3 PUSCH repetition based enhancements to be specified for CE UEs are supported for RedCap UEs, and no additional RedCap-specific Msg3 enhancements are needed.

- Specify UE features for Rel-17 CE PUSCH and Msg3 enhancements for RedCap UEs in WI phase.

It seems that these can be further discussed in a later part of the release, instead of aiming for a conclusion now? Any thoughts?

| Company | Views |
| --- | --- |
| Nokia, NSB | Similarly as in the question in section 1.1.2, this depends on further details of RedCap scope and it is difficult to make a conclusion now. It should be noted though that a conclusion on first two bullets seem to imply no need for adding objectives to CovEnh WID regarding RedCap UEs, which would be fine too. |
| Ericsson | Agree with the moderator's suggestion to discuss these aspects later in the release. We'd like to stress that UL coverage recovery for RedCap UEs may not be needed unless RAN decides to relax the antenna efficiency for RedCap wearables. Therefore, aspects such as UE capability and if PUSCH enhancements or Msg3 repetition for Cov Enh are required for RedCap should be FFS for now. |
| Intel | While we agree with the spirit of the three bullets, they should be left FFS for now. For now, if we’d prefer to say something, an alternative could be to capture a note as following: - Note: It is expected that PUSCH and Msg3 repetition-based enhancements, to be specified for CE UEs, can be supported for RedCap UEs. The applicability of the CE solutions and corresponding UE features for RedCap UEs to be addressed during the WI phase in either RedCap or CE WIs (FFS at RAN #92-E). |
| FUTUREWEI | OK to discuss later or have a note as Intel suggests. |
| vivo | In general agree with three bullets, it can be discussed in later phase as moderator’s suggestion. |
| CMCC | Agree with moderator’s suggestion to discuss this later. Whether the Redcap UE should support mechanisms of CE as mandate or based on the claim of UE capability more depends on the conclusions and the detailed designs of RedCap. At least, the mechanism of CE could be reused for Redcap. Once the Redcap UE claims that they are CE UEs, the solutions should apply to them. |
| DOCOMO | We support the suggestion that these can be discussed in later part of the WI phase, as it is still unclear whether PUSCH/Msg3 enhancements to be specified in CovEnh WI can be applied to RedCap UEs without any adjustments or any adjustments for RedCap UEs are necessary. |
| CATT | Agree with moderator that it is too early to have a conclusion. It should be discussed later under the umbrella of RedCap. |
| InterDigital | Agree with moderator’s suggestion to discuss it in the later release. |
| Qualcomm | For UL coverage recovery of PUSCH/msg3, we think RedCap UE can use the solution(s) developed for non-RedCap UEs in the R17 CovEnh WI. We are ok with keeping the 3rd bullet since it can cover the 3dB antenna efficiency loss for RedCap UEs. |
| Apple | In General, we agree the same mechanism defined in CE WI can be applied to RedCap devices. What we are concerning about is how to quantify the value range of number of repetitions, we prefer it should be determined based on link budget gap from RedCap devices rather than from eMBB devices, i.e., taking into account 3dB antenna efficiency loss in link budget gap. |
| ZTE | Support the proposals. In our view, the PUSCH and Msg3 enhancements specified for CE UEs would be sufficient for 3dB coverage recovery for PUSCH and Msg3 of RedCap UEs. Thus, no additional RedCap-specific enhancements are needed. It’s our understanding that it is more efficient to let the same group of researchers/delegates who are familiar with CE enhancements to discuss above aspects when it comes to WG level discussion. Thus, we suggest the discussion on above aspects could be carried out in RAN1 if companies don’t want to rush into any detailed conclusions in this RAN plenary meeting. |
| Samsung | The principle from the proponent is fine. However, as moderator said, no specific action is required in this meeting -- Further discuss in a later part of the release. |
| Lenovo, Motorola Mobility | Yes, we agree to consider these proposals at a later stage in Rel-17. Once the enhancements are independently finalized for each WI, then it will be a relatively efficient to discuss these proposals |
| OPPO | We think the moderator’s proposal is reasonable. The bullet is also OK for us. But it may not have to be added into the WID instantly. |
| MediaTek | We are fine with the first two bullet points. We don’t see a need to discuss the third bullet point, unless otherwise specified, all NR features can be supported by RedCap UEs. |
| Sharp | We agree with moderator. These can be discussed later. |
| Panasonic | We agree moderator's suggestion. |
| Xiaomi | We general agree with it. |
| Huawei, HiSilicon | If there is no concern to make this conclusion now, it would clear up the question for both WIs. (Similar as in 1.1.2). |

It seems that the principle of the three proposals is generally agreeable, although it seems a bit early to make a concrete conclusion. This seems to be connected with the discussion in Section 1.1.2 as well. It is thus suggested to combine the two sections into a single one for the next round of discussion.

Proposals:

- Please see Section 1.2.2 for more details.

### Differentiation between CE and RedCap UEs before Msg3 Tx

Question:

- Is there a need to differentiate between Rel-17 CE UEs and Redcap UEs before Msg3 transmission in WI phase? If so, how? Should we clarify it accordingly (in CE or RedCap WI?)

| Company | Views |
| --- | --- |
| Nokia, NSB | Any related discussion should take place in [38][RedCap_scope], as it is not a coverage enhancement aspect in itself. |
| Ericsson | There is already discussion in Cov Enh on differentiating UE support for Msg3 and in RedCap on early UE identification before Msg3 transmission. While solutions can be similar, these are somewhat different issues, and can be considered in parallel in the two different WIs. If it is found that differentiation is needed, any incompatibilities between identified solutions in RedCap and Cov Enh can be resolved at that time. However, it should be kept in mind in the RedCap and Cov Enh discussions that UEs may support both RedCap and Cov Enh, and so if resources such as PRACH are used to independently indicate support for either feature, the overhead will be proportionately higher. |
| Intel | We think this is also related to the discussion in RedCap whether/how early identification is needed to differentiate RedCap UEs and non-RedCap UEs. In our view, this should be discussed under [RedCap_scope]. |
| FUTUREWEI | If both early identification and this CE feature are supported, there could be the issue Ericsson mentions. Perhaps it could be resolved later as part of the applicability discussion whether some small modification is needed. |
| vivo | Agree with Nokia’s comment, it should be discussed in RedCap_scope thread. A general comment, we do see potential issue as Ericsson mentioned above and should be carefully studied to avoid excessive overhead or lower system efficiency. |
| CMCC | This is a similar issue as 1.1.3 which is considering the combination of features from two separate WIs. We could discuss this later when the Redcap have a clear conclusion on when to identify a RedCap UE. |
| DOCOMO | Yes, we think differentiation is necessary between Rel-17 CE UEs and RedCap UEs (and also legacy UEs) before Msg2/3 transmission. Coverage recovery for Msg2 is necessary for RedCap UEs in FR1 TDD bands while it may not be necessary for legacy UEs or Rel-17 CE UEs. The amount of coverage enhancement/recovery for Msg3 may be different between Rel-17 CE UEs and RedCap UEs. Early identification of these UEs should be specified either CE or RedCap WI. We are fine with either WI as long as overlapping is avoided. |
| CATT | Share the same view with Nokia. |
| InterDigital | Agree with Nokia |
| Qualcomm | We agree with the comment of Nokia. For RedCap UEs, we think early indication before msg3 is necessary, because NW needs to identify (at least) the following aspects during initial access: - reduced BW (for DL and UL) - reduced DL coverage (for msg2 PDCCH/PDSCH) - reduced UL coverage (for msg3) Considering 1) and 2) are not the objectives of the UL CovEnh WI for non-RedCap UE, we do see a need to differentiate R17 RedCap UE and R17 CE for non-RedCap UE before msg3 transmission, and this is not purely a UL CovEnh aspect. |
| Apple | We share the view that this issue should be discussed under RedCap scope. |
| ZTE | No strong preference from our side on whether to differentiate between Rel-17 CE UEs and Redcap UEs before Msg3 transmission. - As mentioned above, Msg3 repetition should also be made applicable for RedCap UEs for coverage recovery. With a same set of Msg3 enhancements, it could be considered to not differentiate between UEs supporting Msg3 repetition before Msg3 transmission, regardless it is a CE UE or RedCap UE. The RedCap type could be further reported and identified by Msg3 transmission. This could reduce specification effort and less resource segmentation (e.g., PRACH partition if PRACH is used for differentiation from normal UEs). On the other hand, it may also limit the supported maximum BW for CE UEs no larger than 20 MHz as defined for RedCap UEs. If no need for differentiation, then a note could be added in CE WI to clarify this. During WG discussion in CE WI, the differentiation mechanism would only focus on UEs supporting Msg3 repetition and legacy UEs. If differentiation is needed, a conclusion may be sufficient to clarify the situation and corresponding discussion for differentiation of CE UEs/RedCap CEs from legacy UEs could be discussed separately in different WIs. It can also be further discussed in RAN1 to decide whether differentiation is needed. Depending on the RAN1 discussion, we may come back to this issue in June if RAN guidance is needed. |
| Samsung | The discussion depends on the progress of each WI. We don’t see the need for clarification in this meeting. |
| Lenovo, Motorola Mobility | The differentiation and early identification between RedCap and non-RedCap UEs should be discussed under [RedCap_scope]. Similar mechanisms can be used for CE UEs, if needed. |
| OPPO | We agree there would be a deification of RedCap UE in the initial access procedure. This will be in RedCap WI We also OK to discuss identification of Coverage Enhanced UE in the CE WI. The differentiation of CE and RedCap UE seems could be the results. Thus, may be we can just explicitly state in CE WI that the identification of CE UE is considered in WID. |
| MediaTek | Early identification of RedCap UEs is discussed within the RedCap WI in RAN1 and RAN2. The pros, cons and motivations for early identification goes beyond the coverage recovery, hence, it is best to be discussed in the RedCap WI or under [RedCap_scope] email discussion. There is no needed to discuss this issue in the CE WI. |
| Sharp | In our view (1) differentiating UE support of CovEnh function for Msg3 and (2) early indication of RedCap ability can be discussed in parallel under CovEnh and RedCap WIs, respectively, unless any difficulty to apply both of (1) and (2) for a single UE is identified. |
| Panasonic | We agree the comment from Ericsson that the differentiation of CovEnh and the differentiation of RedCap can be considered in parallel for respective WIs. The combination case handling can be discussed later. |
| Xiaomi | Agree with Nokia and VIVO. This should be discussed in Redcap thread. |
| Huawei, HiSilicon | In the RedCap WID, we need to differentiate between RedCap UEs, and non-RedCap UEs, before Msg3. In the CovEnh WID, we need to differentiate between UEs with and without CE, before Msg3. |

It seems that that there is a common understanding that potential differentiation between Rel-17 CE UEs and Redcap UEs before Msg3 transmission should be discussed in the RedCap WI.

Proposals:

- Potential differentiation between Rel-17 CE UEs and Redcap UEs before Msg3 transmission should be discussed in the RedCap WI.

## Intermediate Round

### DM-RS Bundling for PUCCH

Question: is the following update agreeable?

Update the CovEnh WID as follows:

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

| Company | Views |
| --- | --- |
| Ericsson | Support |
| MediaTek | Fine with the update. |
| Intel | We are fine with the update. |
| CATT | Support |
| vivo | support |
| CMCC | General fine with the update |
| InterDigital | We still think the change is not necessary as RAN1 can make that decision as similar to the inter-slot frequency hopping agreement which is made in the previous RAN1 meeting. But, considering that the updated change is relaxed and not mandated to use PUSCH mechanism for PUCCH repetition, we can accept the change if this doesn’t delay the progress of PUCCH repetition design. |
| Qualcomm | We are ok with the update. |
| ZTE | Support |
| DOCOMO | We support the update |
| Samsung | Fine with the update. |
| Lenovo, Motorola Mobility | Support |
| Apple | We are fine with the updated proposal. |
| OPPO | We are OK with the bullet updated. |
| Sharp | We are fine with the update. |
| Nokia, NSB | We support the proposal |
| Panasonic | Support |
| Xiaomi | Support |
| Huawei, HiSilicone | We doubt this gives much clarity due to so much caveat, but OK |

Based on the discussion, it seems that the proposal is agreeable. No further discussion is necessary.

Proposed Agreement:

Update the CovEnh WID as follows:

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

### Coverage Recovery for RedCap, and Features for CE UEs vs. RedCap UEs

Note this section combines the discussion in Section 1.1.2 and 1.1.3. As a result, the proposed conclusion below reflects the discussion in Section 1.1.2 and 1.1.3.

Is the following proposal agreeable?

Proposed conclusion:

- It is generally understood that the techniques developed in the CovEnh WI can be applicable to RedCap UEs

  - Detailed discussion on which/how features developed in the CovEnh WI (e.g., PUSCH enhancements, Msg3 PUSCH repetition based enhancements) that are supported by RedCap UEs can be done, e.g., in RAN#92-e

- Revisit in RAN#92-e whether or not to add a note in CovEnh WID regarding the issue of potential coverage recovery for PUSCH enhancement and Msg 3 PUSCH repetition for RedCap UEs raised in RP-210602

| Company | Views |
| --- | --- |
| Ericsson | Agree that techniques developed in the CovEnh WI can be applicable to RedCap UEs. However, since we discuss CE features that are applicable, and not necessarily required, for RedCap UEs, these features ‘may be supported’. We also think these discussions can be done in RAN1, and if needed discussed later in RAN. Regarding adding a note to the CE WID, this is dependent on which features are applicable and if coverage recovery is found needed. We prefer to have some outcome of these discussions before it is concluded that they should be revisited in RAN. So we’d suggest for the first bullet and subbullet: - It is generally understood that the techniques developed in the CovEnh WI can be applicable to RedCap UEs - Detailed discussion on which/how features developed in the CovEnh WI (e.g., PUSCH enhancements, Msg3 PUSCH repetition based enhancements) that may be supported by RedCap UEs can be done in RAN1 and if needed in RAN plenary And to drop the last subbullet on adding the note. |
| FUTUREWEI | We are fine to assume that the Rel-17 CE solutions are available by default to RedCap UEs (i.e., optional or mandatory), where exceptions can be discussed later on a case-by-case basis after we see what those features actually look like. The exception would be if there is some sort of incompatibility with RedCap UEs, where we would decide to either not support the solution or make a small compatibility modification such that it works directly with e.g. RedCap early identification. However if the incompatibility is too large we can decide that the feature is not supported. So, please add "by default" to the first main bullet of the proposal. The subbullet is not needed as the discussion would likely not be so early as RAN#92 and would first occur in the WG under RedCap. The second main bullet is not needed, no need to revisit. |
| MediaTek | For the first bullet point, we are supportive of the modification from Ericsson. We don’t see a need for the second bullet point, and we suggest to remove it. |
| Intel | We are fine with the update from Ericsson for the first bullet. Our view is that the solutions for PUSCH and Msg3 PUSCH enhancement developed in the CE WI can be applicable for RedCap UEs. We are also okay to remove the second bullet. |
| CATT | For the first bullet, as it has been announced ‘which/how features developed in the CovEnh WI …’, we don’t think ‘may be’ is needed. We are fine with the other update, i.e. it should be firstly handled in RAN1, from Ericsson. For the second bullet, we share the same views that it is not needed. |
| vivo | We are fine with original proposal at it says discuss it in RAN#92e. We are fine with the second bullet as well. |
| CMCC | We are fine with the 1st main bullet. For the subbullet, we share a similar idea that “may be” is duplicated, as the “which/how” means that parts or all of the CE features could be integrated with Redcap UEs. At least, it is one way for Redcap UE to support CE feature that a Redcap UE claims that it supports the UE capability of CE. We support that the discussion could come back to RAN1. And if there are agreements or it is still in controversy, we can return to RAN plenary to update the WID or seeking for a solution. For the discussion where it happens in RAN1, our initial thinking is under the agenda of RedCap. |
| InterDigital | We are ok with the proposal. Also fine with update from Ericsson and removing 2nd bullet. |
| Qualcomm | We are ok with the update. Also ok with the additional modifications by Ericsson. |
| ZTE | We are fine with the proposal. In RAN#92-e, we can first discuss whether some high-level features in CE can be supported by RedCap UEs, e.g., the three enhancements for PUSCH and type A repetition of Msg3. Then, further detailed sub-features could be discussed in later phase in RAN1 or RAN plenary if needed. If some of the features would be agreed to be supported by RedCap UEs, we need to add a note in CovEnh WID to clarify this. This could be re-visited in RAN#92-e. |
| DOCOMO | For the 1st bullet, we support the modification from Ericsson For the 2nd bullet, we don’t see the need |
| Samsung | Fine with moderator’s proposed conclusion. |
| Lenovo, Motorola Mobility | Agree |
| Apple | We support the updated conclusion. For the second bullet, the moderator provides a reasonable way forward, there is no discussion on coverage recovery issue in the ongoing RedCap discussion. Thus, it can be discussed further in RAN#92 meeting to close this left issue. |
| Sharp | We are fine with the proposal. Also fine with the update from Ericsson and removing the 2nd bullet. |
| Nokia, NSB | We are OK with the conclusion in general, though we agree with some comments above that the second bullet is not needed, as RAN Plenary can always discuss such issues when needed. |
| Panasonic | We support the update from Ericsson. |
| Xiaomi | From our viewpoint, all the CE solutions should be developed such that they can be applicable to both normal UEs and Redcap UEs. Therefore we do not see the need of the subbullet of 1st bullet and suggest to remove it. On the other hand, we wonder what additional information is needed so the decision on whether/how features in CE are supported by Redcap UEs can be made in the next RAN-P meeting. We would suggest to agree the general principle that the techniques developed in the CovEnh WI can be applicable to RedCap UEs in RAN-P, and any potential exception(s) can be further discussed in WG level. |
| Huawei, HiSilicon | No the proposal should not be agreed while targeting RAN#92, as RAN should not once again delay clarity. The proposal in the RedCap email thread is to assume applicability of Rel-17 CE features to RedCap UEs, and that proposal should be agreed. Then this discussion in CE is automatically settled, and in this meeting. |

Based on the discussion, it seems that the 2nd bullet is not necessary and can be removed. For the 1st bullet, it seems that there is no need to come back to this topic in the next RAN meeting – either it can be done as part of the normative RAN1 work (majority view) and be concluded now (as proposed by some companies). Considering the overall inputs so far, the proposed conclusion is updated as follows:

Proposed conclusion:

- It is generally understood that the techniques developed in the CovEnh WI can be applicable to RedCap UEs by default

  - Detailed discussion on which/how features developed in the CovEnh WI (e.g., PUSCH enhancements, Msg3 PUSCH repetition based enhancements) that are supported by RedCap UEs is expected to be part of the normative work in RAN WGs can be done, e.g., in RAN#92-e

Any further comments?

| Company | Views |
| --- | --- |
| Ericsson | Support the conclusion |
| MediaTek | Support the proposed conclusion. |
| OPPO | The current bullet is from RedCap discussion. We wonder if the RedCap discussion have further update of the bullet, should we reflect that in the summary? There was a specific coverage recovery target were proposed by some companies. That target could be needed to captured in CE WID, since only CE will discuss coverage enhancement solution. We may not nee the no further discussion conclusion, but we can wait for RedCap summary. |

This topic is also being discussed in the RedCap email thread. After further coordination, this topic will no longer be discussed any further herein, but will be handled in the RedCap email thread.

### Differentiation between CE and RedCap UEs before Msg3 Tx

Is the following agreeable?

Proposed conclusion:

- Potential differentiation between Rel-17 CE UEs and Redcap UEs before Msg3 transmission should be discussed in the RedCap WI.

| Company | Views |
| --- | --- |
| Ericsson | Conclusion is OK |
| FUTUREWEI | For the specific CE solution of PUSCH Msg 3 repetition, we are fine to conclude now that it is supported by RedCap UEs, and conclude that the any compatibility modification of the solution (e.g., taking into account early identification) is discussed in RedCap. |
| MediaTek | Support the Conclusion. |
| Intel | We are fine with the conclusion. |
| CATT | Support the conclusion. |
| vivo | Support the conclusion |
| CMCC | Fine with the conclusion. The issues related to differentiations or (early) identifications should be discussed in the RedCap WI. |
| InterDigital | Support the conclusion |
| Qualcomm | We are ok with the conclusion. |
| ZTE | Based on the discussion in initial round , it seems companies agree that the differentiation between RedCap UEs and non-RedCap UEs (potentially including Rel-17 CE UEs) before Msg3 transmission should be discussed in RedCap WI, and differentiation between UEs with and without coverage enhancements before Msg3 transmission should be discussed in CE WI. Regarding the potential incompatibilities (if any) between the differentiation mechanisms could be further revisited in later phase. Thus, we suggest the following proposed conclusion: Proposed conclusion: The differentiation between RedCap UEs and non-RedCap UEs (potentially including Rel-17 CE UEs) before Msg3 transmission should be discussed in RedCap WI, and the differentiation between UEs with and without coverage enhancements before Msg3 transmission should be discussed in CE WI. - Potential incompatibilities (if any) between the differentiation mechanisms could be addressed in later phase. |
| DOCOMO | We are fine with the proposed conclusion regarding potential differentiation between Rel-17 CE UEs and Redcap UEs. However, it is unclear whether potential differentiation between Legacy UEs and Rel-17 CE UEs will be discussed in RedCap WI as well. |
| Samsung | Prefer to leave it up to WG discussion for now. However, if RAN intend to conclude something…. 1) Differentiation/identification can be either ‘before’ msg3 transmission or ‘during/after’ msg3 transmission. There is related on-going RAN1 discussion for Msg3 PUSCH repetition under CE WI. 2) The discussion can be parallelized in RedCap and CE WI, i.e. does not fall into single WI (either RedCap or CE). Therefore, our suggestion is as follows: - Potential differentiation between Rel-17 CE UEs and Redcap UEs before Msg3 transmission during initial access should be discussed in the RedCap WI separately in each WI striving for avoiding any conflict. |
| Lenovo, Motorola Mobility | Agree |
| Apple | Our understanding is this proposal will not impact ongoing Msg3 discussion under CovEnh WI. If any differentiation is required between type of UEs, it can be done in RedCap WI. |
| OPPO | Fine with the proposal, even our preference is clarify the target here. 😊 |
| Sharp | OK with the proposed conclusion. |
| Nokia, NSB | We support the proposal |
| Panasonic | We support the conclusion. |
| Xiaomi | We are supportive to the intention of the proposal. However, in Redcap discussion the potential differentiation between R-17 CE and Redcap UEs can be either by Msg1 or Msg3 transmission. Therefore, we suggest to make the following revision to make the conclusion more complete. Hope it can be acceptable. Proposed conclusion: Potential differentiation between Rel-17 CE UEs and Redcap UEs before and by Msg3 transmission should be discussed in the RedCap WI. |
| Huawei, HiSilicon | This needs to be more precise as follows to avoid being unclear on exactly which UEs are in each set: Potential differentiation between Rel-17 CE non-RedCap UEs, and RedCap UEs will be discussed in the RedCap WI. This leaves to the CE WI any potential differentiation between non-RedCap UEs with/without CE. |

Based on the discussion, it seems that the proposed conclusion should be modified as follows:

Proposed conclusion:

- Potential differentiation between Rel-17 CE non-RedCap UEs (with or without Rel-17 CE) and vs. Redcap UEs before Msg3 transmission during initial access should will be discussed in the RedCap WI.

- Potential differentiation for non-RedCap UEs with vs. without Rel-17 CE during initial access will be discussed in the CE WI

Any further comments?

| Company | Views |
| --- | --- |
| Ericsson | Support the conclusion |
| Huawei, HiSilicon | This conclusion is fine. |
| Intel | We are fine with the conclusion. |
| FUTUREWEI | Support |
| MediaTek | Support the proposed conclusion. |
| Samsung | Fine with the conclusion. For our clarification, does 1st bullet also care Redcap UE with Rel-17 CE? |
| CMCC | Fine with the proposal |
| CATT | Support |
| DOCOMO | Support the conclusion |
| Qualcomm | We are ok with the proposed conclusion. |
| Lenovo, Motorola Mobility | We support the conclusion |
| ZTE | Fine with the intention of the proposed conclusion. To be more precise, we suggest to make the following revisions. - Potential differentiation between non-RedCap UEs (with or without Rel-17 CE) vs. Redcap UEs (with or without Rel-17 CE) during initial access will be discussed in the RedCap WI. - Potential differentiation for non-RedCap UEs with vs. without Rel-17 CE during initial access will be discussed in the CE WI As for the potential incompatibilities (if any) between the differentiation mechanisms, it could be addressed in later phase. |
| Huawei, HiSilicon #2 | Quick reply (hope to save some time): To ZTE: The suggested change would put some, but not all, RedCap UEs into both discussions, when the aim here is to separate them and avoid loops in RAN1 discussion. Wanshi’s version is cleaner by avoiding that. The answer to Samsung’s question is: yes, the wording is clear and general over all RedCap UEs. |
| Xiaomi | We support the proposal. |
| Nokia, NSB | We support the proposal |
| OPPO | The update of proposal is better. We are fine. |
| Sharp | Support |

It seems that the proposed conclusion is agreeable. No further discussion is deemed necessary.

Proposed conclusion:

- Potential differentiation between Rel-17 CE non-RedCap UEs (with or without Rel-17 CE) and vs. Redcap UEs before Msg3 transmission during initial access should will be discussed in the RedCap WI.

- Potential differentiation for non-RedCap UEs with vs. without Rel-17 CE during initial access will be discussed in the CE WI

## Final Round

No need. The email thread can be concluded earlier.

# Conclusion

Based on the email discussion, the following are proposed and these proposals are stable for endorsement:

- Update the CovEnh WID as follows:

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Potential differentiation between non-RedCap UEs (with or without Rel-17 CE) vs. Redcap UEs during initial access will be discussed in the RedCap WI

- Potential differentiation for non-RedCap UEs with vs. without Rel-17 CE during initial access will be discussed in the CE WI

# References

RP-210455	Revised WID on NR coverage enhancements	China Telecom

RP-210499	Views on the scope of NR coverage enhancements WI	Apple Inc.

RP-210602	Views on PUSCH and Msg3 enhancements for Redcap UEs in NR CE WI	ZTE, Sanechips