3GPP RAN 5G-ACIA Evaluations week 2

December 14th – 18th 2020

Source:	Ericsson

Title:		Simulation Results for 5G-ACIA (First round)

# 1	Introduction

At RAN#89, a work plan [2] had been agreed to provide evaluations for 5G-ACIA [1]. After first round of discussion, a set of simulation assumptions and the following features had been agreed [3]:

For the Rel-15 baseline, the following is agreed:

- Rel-15 URLLC features included in the baseline are as follows, while it is up to each proponent to decide which Rel-15 features are used, and detail this when providing the results:

  - UE Processing capability 2

  - UL Configured grant

  - DL Semi-persistent scheduling

Regarding Rel-16 features, the following is agreed:

- It is up to each proponent to decide on which Rel-16 features to provide simulations results for in addition to the Rel-15 baseline

- This can be revisited after the first round of simulations have been provided in December.

In this document we provide our simulation results derived according to agreed assumptions.

# 2	Scenario and results

## 2.1	Scenario description 4GHz

In general, motion control use case description is given in [4] and simulation methodology is extensively described in 5G-ACIA LS [1], thus, we see no need to repeat it in this document. For brevity, we focus on the most important aspect of simulation methodology we used.

Due to 1 ms latency target, we use the shortest transmission duration (2 os, as in Rel-15 PDSCH) and assume short processing times at gNB. Only one initial HARQ transmission is enabled. Before transmission of a packet, gNB checks the delay budget left and removes packet from queue if packet delay budget is going to expire before it can be decoded by UE.

For scheduling UL CG and DL SPS are used. This helps to avoid PDCCH blocking and DCI errors. Since packet arrival is known by gNB, allocation in time and periodicity is optimized so that the alignment delay is minimized.

The full set of simulation parameters for 4GHz carrier are summarized in Appendix A.

## 2.2	Simulation results for 4GHz

We provide CDF for service availability in figure 1 and for latency in figure 2.

Figure 1. CDF of communication service availability for all UEs in factory hall with a total of 120 UEs.

Figure 2. CDF of delay distribution of all UEs in factory hall (DL and UL) with a total of 120 UEs.

In addition, coupling loss and geometry are provided for calibration purposes in figures 3 and 4.

Figure 3. Coupling loss for 4GHz factory hall

Figure 4. Geometry for 4GHz factory hall

# 3	Conclusions

As it was demonstrated, high service availability can be achieved at low load. Results for higher load is FFS.

For 4GHz, it is possible to achieve 99.9999% CSA for more than 99% of users in low load scenario.

# References

RP-201279, “LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation”, 5G-ACIA

RP-202069, “Way forward on RAN work for 5G ACIA requested simulations“, Ericsson

Agreements week 1 5G-ACIA, Moderator (Ericsson)

3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains, V16.4.0, December 2019.

# Appendix A. Simulation assumptions for 4GHz

| Parameter name | Parameter Value |
| --- | --- |
| Factory hall size | 120x50 m |
| Room height | 10 m |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. |
| BS/TRP antenna height | 8m |
| Layout – BS/TRP deployment | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| BS antenna mount | Option 1 (1 sector per BS) from 38.824 is used |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5dB |
| BS antenna configurations | 4 Tx/4 Rx antenna ports and 8 Tx/8 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Tx/4 Rx antenna ports; dH = dV = 0.5 λ |
| BS Tx power | 24 dBm per 20 MHz => 31 dBm in total |
| BS receiver | MMSE-IRC |
| UE Tx power | 23dBm |
| UE antenna configuration | 2 Tx/2 Rx antenna ports Isotropic |
| UE antenna height | 1.5 m |
| UE antenna gain | 0 dBi as starting point |
| UE speed | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| UE receiver noise figure | 9 dB |
| Channel model | InF-DH |
| Carrier frequency and simulation bandwidth | TDD, 4 GHz: 100 MHz |
| SCS | 30 kHz |
| TDD DL-UL configuration | 1 slot DL : 1 slot UL, no guard symbols |
| Number of UEs per service area | 10 UEs per service area |
| UE distribution | All UEs randomly distributed within the respective service area. |
| Message size | 48 bytes |
| DL traffic model | DL traffic arrival with option-1 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 |
| CSA requirements | 99.9999% |
| E2E latency & air interface latency | E2E latency: 1 ms; Air interface latency: 1ms |
| UE power control | Open-loop PC |
| HARQ/repetition | Single HARQ transmission (only initial tx is allowed) |
| Channel estimation | Ideal |
| Allocation in time domain | 2 os |
| MIMO | Up to Rank 2 in DL Up to Rank 1 in UL |
| Packets simulated | 300 k packets per UE. |