3GPP TSG RAN WG1 email discussion [5G-ACIA]

December 14 – 18, 2020

Source:	Nokia, Nokia Shanghai Bell

Title:	First round of simulation results for 5G-ACIA evaluation

Document for:		Discussion and Decision

# Introduction

In RAN1#102-e, RAN1 received an LS from 5G-ACIA requesting RAN1 to carry out an URLLC and IIOT performance evaluation, using Rel-16 URLLC/IIOT features as a baseline [1]. The LS was treated in RAN Plenary #89, which agreed on the following way forward [2]:

RP-202069 Way forward and RAN work for 5G ACIA requested simulations Ericsson result of email discussion [89E][03][5GACIA_simulation] proposal: - Start an offline email-based activity to provide evaluation results for 5G-ACIA - One company volunteers as moderator - Proposes a work plan to follow - Ericsson is willing do this - Discussions are on the RAN1_NR reflector - Email activity only during short periods (< week) distributed across the time allocated to the activity - No email activity in weeks before/during/after RAN1 meetings or RAN defined inactive periods - All companies should strive to limit email activity as much as possible - Outcome of the offline discussion will directly go to RAN without need for discussion in RAN1 nor need for LS from RAN1 to RAN - Target completion by RAN#91 - At RAN#91, RAN will decide on a response LS to 5G-ACIA

Afterwards, the moderator (Ericsson) shared the following schedule in the RAN1_NR reflector:

- 12-16 October 2020

  - Discussion on which URLLC features to include in the evaluations and simulation assumptions

- 14-18 December 2020

  - First round of simulation results

- 22-26 February 2021

  - Second round of simulation results

- 8-12 March 2021

  - Finalization of the report to RAN#91

The discussion regarding the simulation assumptions and URLLC features took place in 12-16 of October as described above, where the following agreements where made. For brevity, the table with the agreed simulation assumptions is placed in Table 3 in Appendix A of this contribution.

Agreements:

- The simulation assumptions given in the table are agreed [see Appendix A]

- Additional simulation parameters are taken from TR 38.824.

For the Rel-15 baseline, the following is agreed:

- Rel-15 URLLC features included in the baseline are as follows, while it is up to each proponent to decide which Rel-15 features are used, and detail this when providing the results:

  - UE Processing capability 2

  - UL Configured grant

  - DL Semi-persistent scheduling

Regarding Rel-16 features, the following is agreed:

- It is up to each proponent to decide on which Rel-16 features to provide simulations results for in addition to the Rel-15 baseline

- This can be revisited after the first round of simulations have been provided in December.

In this contribution, we provide initial simulation results following the previous agreements on the 5G-ACIA evaluation. Section 2 presents simulation results on frequency range 1 (FR1), whereas Section 3 include results for FR2. Finally, conclusions are presented in Section 4.

# Simulation results for 30 kHz SCS (FR1)

Simulation assumptions can be found in Table 4 in the appendix B. For the initial round of simulation results, we focus on the case with random uniformly-distributed traffic arrival (i.e. option-1 as described in the LS [1]) and symmetric TDD configuration consisting of 1 special (S) slot with S = DDDDDDGGUUUUUU. Since there is no time for 1 HARQ retransmission within the 1 ms latency interval (even with 2-symbol transmission duration), the UL and DL scheduling is performed with a mini-slot duration of 6 symbols. For each 6-symbol transmission, it is further assumed that 2 OFDM symbols are used by control and reference signals, resulting in 33% relative overhead.

For each PDSCH and PUSCH transmission, the gNB selects the MCS according to the UE’s CQI reports and SRS transmissions, respectively, where the first transmission BLER target is 1E-5. One key challenge identified while performing the simulations is related to the fast interference fluctuations which make it difficult to accurate select a proper MCS fulfilling the BLER target. For these reasons, some improvements were applied to the gNB link adaptation algorithms as follows:

- In DL, the UE is configured to periodically report a CQI per sub-band. A conservative link-adaptation implementation at the gNB side is considered where the worst CQI, out of the N reported sub-band CQIs is used to derive the MCS for the PDSCH.

  - On top, the gNB applies an offset (in dB) to the UE’s worst CQI report and also limits the maximum MCS for DL scheduling to a certain max_MCS. Thus, the PDSCH MCS is selected as follows:

MCS_PDSCH = min(worst_CQI - offset , max_MCS).

- In UL, similar method is applied but in SINR domain. That is, the gNB estimates UL SINR from the UL SRS transmission, and applies an offset as well as a max_MCS restriction before selecting the MCS for the PUSCH transmissions (note that the applied offset could be different for DL and UL link adaptation). For power control, we assume an aggressive power control strategy where the UE’s transmission power is fixed to 23 dBm (i.e. maximum power), and the power spectral density varies according to the number of PRBs allocated to the PUSCH.

Performance results are presented in the following. We simulate 100 UE drops, where each drop has 50 UEs per service area and both the traffic offsets and the UE positions are randomized across drops (i.e. there are 100*50*12 = 60.000 UEs in total). For each drop, we collect a total of ~20.000 latency samples per UE.

Figure 1 shows cumulative distribution function (CDF) of the per-packet latency collected from all UEs in both downlink (DL) and (UL) directions. The 1 ms latency target is achieved with 99.999% reliability in both UL and DL, whereas a fraction of approximately 3E-6 in DL and 6E-6 in UL of the packets are not correctly received due to bad SINR conditions, high interference, etc.

In Table 1, we show the average PRB utilization as well as the percentage of UEs reaching a Communication Service Availability of 100%. The latter essentially corresponds to the percentage of UEs that do not experience more than 1 consecutive packet error across the simulation (note that we don’t show the CDF of CSA distribution due to the limited number of samples per UE). It is observed that, although the overall per-packet latency is generally very good (as discussed for Figure 1), there is still a small chance that some UEs  experience consecutive packet errors (i.e. violate the CSA constraint). Finally, the PRB utilization shows that the system can reach up to 40%-50% load while still achieving reasonable performance. This relatively high load is partly due to the uniform periodic traffic model which is significantly less bursty as compared as e.g. traditional FTP3 traffic. The PRB% is lower in UL than in DL likely due to the quite different power control scheme.

Observation 1: For the FR1 deployment with InF-DH and random traffic offset (option 1), the network can support 50 UEs in both DL and UL directions, while keeping the 99.999% percentile of the latency CDF (collected from all the UEs) below the 1 ms target. Under such load conditions, at least 0.015% and 0.002% of the UEs in DL and UL, respectively, experience two or more consecutive transmission errors and thus do not fulfil the CSA requirement.

Figure 1: CDF of the per-packet latency collected from all the simulated UEs. Not-correctly received packets have infinite delay. FR1 – 30 kHz SCS.

Table 1: CSA and PRB utilization statistics. FR1 – 30 kHz SCS.

| Setting | Percentage of UEs reaching a communication service availability (CSA) of 100%* | Average PRB utilization [%] |
| --- | --- | --- |
| DL, 50 UEs per service area | 99.985% | 50.4% |
| UL, 50 UEs per service area | 99.998% | 40.4% |
| *We simulate 100 UE drops (i.e. 100*50*12 = 60.000 UEs in total) and collect a total of ~20.000 latency samples per UE. The reported statistic corresponds to the percentage of UEs that do not experience more than 1 consecutive packet error across the simulation. |  |  |

# Simulation results for 120 kHz SCS (FR2)

Simulation assumptions can be found in Table 5 in the appendix C. We assume the same TDD configuration S = DDDDDDGGUUUUUU and link adaptation enhancements as described in Section 2. Figure 2 shows the CDF of the UL and DL delay for different amount of UEs per service area. In constrast to the FR1 results, the latency performance is significantly better in DL than in UL direction, which is a consequence of poorer channel quality/SINR in UL and a more conservative UL link adaptation strategy to ensure a latency outage probability below 1E-5. Particularly, up to 50 and 30 UEs per service area are supported in DL and UL, respectively, while still keeping the 99.999% percentile of the latency below the 1 ms target. Regarding the packet error rate, a fraction of approximately 9E-6 in UL and between 1E-6 and 3E-6 in DL of the transmitted packets are not correctly received within the 1 ms target.

Table 2 shows the average PRB utilization as well as the percentage of UEs reaching a CSA of 100%. Note that, as compared to FR1 results, the system ends up operating at lower PRB utilization to be able to fulfil the 1 ms latency with 99.999% reliability target. This is because the latency performance is impacted not only by queuing delay and interference but also by limitations to user multiplexing imposed by beamforming operation itself. A multiplanel antenna and increased number of antenna ports could be used to mitigate this.

Observation 2: For the FR2 deployment with InF-DH and random traffic offset (option 1), the network can support 50 and 30 UEs per service area in DL and UL, respectively, while keeping the 99.999% percentile of the latency CDF (collected from all the UEs) below the 1 ms target. Under such load conditions, at least 0.07% and 0.96% of the UEs in DL and UL, respectively, experience two or more consecutive transmission errors and thus do not fulfil the CSA requirement.

Figure 2: CDF of the per-packet latency collected from all the simulated UEs. Not-correctly received packets have infinite delay. FR2 – 120 kHz SCS.

Table 2: CSA and PRB utilization statistics. FR2 – 120 kHz SCS.

| Setting | Percentage of UEs reaching a communication service availability (CSA) of 100%* | Average PRB utilization [%] |
| --- | --- | --- |
| DL, 30 UEs per service area | 99.86% | 14.4% |
| DL, 40 UEs per service area | 99.59% | 20.4% |
| DL, 50 UEs per service area | 99.04% | 25.6% |
| UL, 30 UEs per service area | 99.93% | 45.9% |
| *We simulate 100 UE drops (i.e. 100*50*12 = 60.000 UEs in total) and collect a total of ~10.000 latency samples per UE. The reported statistic corresponds to the percentage of UEs that do not experience more than 1 consecutive packet error across the simulation. |  |  |

# Conclusions

In this contribution, we have provided initial system-level simulation results following the agreements related to the 5G-ACIA performance evaluation. The obtained results are summarized with the following observations:

Observation 1: For the FR1 deployment with InF-DH and random traffic offset (option 1), the network can support 50 UEs in both DL and UL directions, while keeping the 99.999% percentile of the latency CDF (collected from all the UEs) below the 1 ms target. Under such load conditions, at least 0.015% and 0.002% of the UEs in DL and UL, respectively, experience two or more consecutive transmission errors and thus do not fulfil the CSA requirement.

Observation 2: For the FR2 deployment with InF-DH and random traffic offset (option 1), the network can support 50 and 30 UEs per service area in DL and UL, respectively, while keeping the 99.999% percentile of the latency CDF (collected from all the UEs) below the 1 ms target. Under such load conditions, at least 0.07% and 0.96% of the UEs in DL and UL, respectively, experience two or more consecutive transmission errors and thus do not fulfil the CSA requirement.

# References

- R1-2006953	LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation, 5G ACIA

- RP-202069	Way forward and RAN work for 5G ACIA requested simulations, Ericsson

# Appendix A: Simulation assumptions agreed during week 1 discussions

Table 3: Simulation assumptions agreed during week 1 discussions

| Parameters | 5G-ACIA LS | Agreement |
| --- | --- | --- |
| Factory hall size | 120x50 m | As in 5G-ACIA LS |
| Room height | 10 m | As in 5G-ACIA LS |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. | According to proposed layout below |
| BS/TRP antenna height | 1.5 m for InF-SL and InF-DL 8m for InF-SH and InF-DH | As in 5G-ACIA LS |
| Layout – BS/TRP deployment | Depending on the number of TRPs | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| Channel model | UC-2: InF-DH > InD-DL > InF-SH > InF-SL | Mandatory: InF-DH Optional: InD-DL, InF-SH, InF-SL |
| Carrier frequency and simulation bandwidth | TDD 4 GHz: 100 MHz 30 GHz: 160 MHz | As in 5G-ACIA LS |
| TDD DL-UL configuration | Simulation company should report the used DL-UL configuration. | Companies should report the used DL-UL configuration. 1:1 DL-UL configuration is recommended. |
| Number of UEs per service area | Up to 50 per service area, e.g., 10, 20, 40, and 50 | As in 5G-ACIA LS |
| UE distribution | All UEs randomly distributed within the respective service area. | As in 5G-ACIA LS |
| Message size | 48 bytes | 48 bytes |
| DL traffic model | DL traffic arrival with option-1, option-2, and option-3. | 5G-ACIA Option 1 is mandatory. Companies are also encouraged to provide results for option 3 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 and option-2 | As in 5G-ACIA LS with Option 1 as mandatory |
| CSA requirements | UC-#2: 99.9999% | UC-#2: 99.9999% |
| Performance metrics | 1) CSA: single CDF of CSA distribution of all UEs in factory hall 2) Latency: single CDF of latency distribution of all UEs in factory hall 3) Percentage of UEs satisfying requirements 4) resource utilization | As in 5G-ACIA LS with 3) and 4) as low priority Note: For metric 2) it is clarified that a packet transmission cannot be performed after the latency deadline. The collected statistics cannot exceed the latency requirement. The packets exceeding the deadline are visible in the UE packet error statistics |
| E2E latency & air interface latency | E2E latency: 1 ms for UC#2 | E2E latency: 1 ms for UC#2 Air interface latency: 1ms |
| UE speed | Linear movement | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| BS antenna mount |  | Option 1 (1 sector per BS) from 38.824 is used |

# Appendix B: Simulation assumptions for 30 kHz/FR1 case

Table 4: Simulation assumptions for 30 kHz/FR1 case

| Parameters​ | Value​ |
| --- | --- |
| Inter-BS distance​ | 20m​ |
| Carrier frequency​ | 4 GHz​ |
| UE Tx power​ | 23dBm​ |
| BS antenna element gain + connector loss​ | 5 dBi​ |
| BS receiver noise figure​ | 5dB​ |
| BS antenna configurations​ | (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Tx/4 Rx antenna ports​ |
| UE antenna configuration​ | 4 Rx antenna ports ​ (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Rx;​ |
| UE antenna height​ | Follow the modelling of TR 38.901 (e.g. 1.5m)​ |
| UE antenna gain​ | 0dBi as starting point​ |
| BS Tx power​ | 27 dBm​ |
| BS receiver​ | MMSE-IRC as the baseline receiver​ |
| UE receiver noise figure​ | 9 dB​ |
| SCS ​ | 30 kHz, i.e. 28 symbols per TTI​ |
| Simulation bandwidth ​ | 100 MHz @ 4GHz​ |
| Layout​ | Single layer as defined in 38.802​ Indoor floor:12 BSs per 120 m x 50 m​ |
| Channel model ​ | InF-DH (dense clutter 0.6, high BS 8 m)​ |
| Traffic model​ | 1 ms packet inter-arrival time. Random offset​ |
| Number of UEs per service area (12 service areas)​ | 50 |
| UE distribution​ | 100% of users are indoor, 75 km/h​ |
| UE power control​ | Constant max power |
| HARQ/repetition​ | No HARQ/repetition​ |
| TDD config​ | TDD frame with single slot S = DDDDDDGGUUUUUU and 6-symbol mini-slot scheduling |

# Appendix C: Simulation assumptions for 120 kHz/FR2 case

Table 5: Simulation assumptions for 120 kHz/FR2 case

| Parameters | Value |
| --- | --- |
| Inter-BS distance | 20m |
| Carrier frequency | 30 GHz |
| UE Tx power | 23dBm |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 7 dB |
| BS antenna configurations | 2 Tx/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) |
| UE antenna configuration | 2 Tx/Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1) 2 UE panels facing opposite directions |
| UE antenna height | Follow the modelling of TR 38.901 (e.g. 1.5m) |
| UE antenna gain | 0dBi as starting point |
| BS Tx power | 26 dBm |
| BS receiver | MMSE-IRC as the baseline receiver |
| UE receiver noise figure | 10 dB |
| SCS | 120 kHz |
| Simulation bandwidth | 160 MHz |
| Layout | Single layer as defined in 38.802 Indoor floor:12 BSs per 120 m x 50 m |
| Channel model | InF-DH (dense clutter 0.6, high BS 8 m) |
| Traffic model | 1 ms packet inter-arrival time. Random offset |
| Number of UEs per service area (12 service areas) | 30, 40, 50 |
| UE distribution | 100% of users are indoor, 3 km/h |
| UE power control | Constant max power |
| HARQ/repetition | No HARQ/repetition |
| TDD config | TDD frame with single slot S = DDDDDDGGUUUUUU and 6-symbol mini-slot scheduling |