3GPP RAN 5G-ACIA Evaluations week 3

February 22nd – 26th 2021

Source:	Ericsson

Title:		Simulation Results for 5G-ACIA (Second round)

# 1	Introduction

At RAN#89, a work plan [2] had been agreed to provide evaluations for 5G-ACIA [1]. After first round of discussion, a set of simulation assumptions and the following features had been agreed [3]:

For the Rel-15 baseline, the following is agreed:

- Rel-15 URLLC features included in the baseline are as follows, while it is up to each proponent to decide which Rel-15 features are used, and detail this when providing the results:

  - UE Processing capability 2

  - UL Configured grant

  - DL Semi-persistent scheduling

Regarding Rel-16 features, the following is agreed:

- It is up to each proponent to decide on which Rel-16 features to provide simulations results for in addition to the Rel-15 baseline

- This can be revisited after the first round of simulations have been provided in December.

In this document we provide our simulation results derived according to agreed assumptions.

# 2	Scenario and results

## 2.1	Scenario description

In general, motion control use case description is given in [5] and simulation methodology is extensively described in 5G-ACIA LS [1], thus, we see no need to repeat it in this document. For brevity, we focus on the most important aspect of simulation methodology we used.

Due to 1 ms latency target, we use the shortest transmission duration (2 os, as in Rel-15 PDSCH) and assume short processing times at gNB. Only one initial HARQ transmission is enabled. Before transmission of a packet, gNB checks the delay budget left and removes packet from queue if packet delay budget is going to expire before it can be decoded by UE.

For scheduling in 4 GHz carrier frequency, UL CG and DL SPS are used. This helps to avoid PDCCH blocking and DCI errors. Since packet arrival is known by gNB, allocation in time and periodicity is optimized so that the alignment delay is minimized.

For scheduling in 30 GHz carrier frequency, UL CG and DL dynamic scheduling are used. One UE per mini-slot is scheduled both in UL and DL due to analog beamforming selected implementation.

Latency calculation assumptions are according to TR 37.910. Alignment delay is accounted for latency calculations.

The full set of simulation parameters for 4GHz carrier frequency are summarized in Appendix A, and for 30GHz carrier frequency in Appendix B.

## 2.2		Simulation results for 4GHz

We provide CDF for downlink communication service availability in Figure 1 and for uplink communication service availability in Figure 2 for the two scenarios 10 UEs/SA and 20 UEs/SA. Figure 3 shows the CDF of packet delay for received packets, combined for uplink and downlink.

Figure 1. CDF of downlink communication service availability for all UEs in factory hall.

Figure 2. CDF of uplink communication service availability for all UEs in factory hall.

Figure 1 shows that the downlink communication service availability for the case of 10 UE/SA is worse than 99.999% for around 10% of the UEs, while the rest UEs experience CSA higher than 99.9999%. For the 20 UE/SA case, CSA is worse than 99.999% for 16% UEs, while the rest experience CSA higher than 99.9999%. This aligns with the tabulated values for CSA we have provided in the enclosed excel sheet, where it is noted that higher than 99.999% CSA in downlink is achieved for 90.71% of UEs in 10 UE/SA scenario and 84.05% of UEs in the 20 UE/SA case.

For 4GHz scenario, higher than 99.9999% downlink CSA is achieved by 90.71% UEs for 10 UEs/SA case, and 84.05% for 20 UEs/SA case.

Figure 2 shows that higher than 99.9999% CSA is achieved for all UEs in uplink both in 10 UE/SA and 20 UE/SA cases, which is supported by the values given in the simulation assumption excel sheet.

For 4GHz scenario, uplink CSA is excellent, achieving 100% CSA for both, 10 UEs/SA and 20 UEs/SA cases.

Figure 3. CDF of delay distribution of all UEs in factory hall (DL and UL).

Delay distribution shown in Figure 3 shows relatively evenly distributed delays below 1 ms for successfully received packets after an initial processing delay of around 0.4 ms. Slot based TDD scheduling causes some stepwise pattern in the delay distribution.

For 4GHz scenario, packet delay for successfully received packets is relatively evenly distributed between 0.3 ms and 1 ms.

CDFs of observed packet error rates can be seen from Figures 4 and 5 for downlink and uplink respectively.

Figure 4a & 4b. CDF of downlink packet error rate

For the downlink it can be seen from the figures above that around 99.5% of UEs experiences packet error rate lower than 10-3, which is quite similar for both cases. This indicates that most of the UEs have good link performance, while few experience worse packet losses probability than 10-3.

For 4GHz scenario, 99.5% UEs have packet error rate lower than 10-3 in downlink, while the few remaining experience inferior packet error rates.

Figure 5. CDF of uplink packet error rate

Figure 5 shows very good performance in uplink, since packet error rate seems to be close to 0 for all UEs.

For 4GHz scenario, uplink packet error ratio is excellent, showing close to zero packet error ratios for all UEs.

In addition, coupling loss and geometry are provided for calibration purposes in Figures 6 and 7. They are slightly different from those in the last Round due to HPBW setting, which is 65 degrees in the current version.

Figure 6. Coupling loss for 4GHz factory hall

Figure 7. Geometry for 4GHz factory hall

## 2.3	Simulation results for 30GHz

For 30GHz evaluations, we provide CDF for downlink communication service availability in Figure 8 and for uplink communication service availability in Figure 9 for the two scenarios 10 UEs/SA and 20 UEs/SA. Figure 10 shows the CDF of packet delay for received packets, combined for uplink and downlink.

Figure 8. CDF of downlink communication service availability for all UEs in factory hall.

Figure 9. CDF of uplink communication service availability for all UEs in factory hall.

Figure 8 shows that for the case of 10 UE/SA, 94.17% has downlink communication service availability higher or equal than 99.9999%. For the 20 UE/SA case, 83.7% has downlink communication service availability higher or equal than 99.9999%. This aligns with the tabulated values for CSA we have provided in the enclosed excel sheet

For 30 GHz scenario, higher than 99.9999% downlink CSA is achieved by 94.17% UEs for 10 UEs/SA case, and 83.7% for 20 UEs/SA case.

Figure 9 shows that for the case of 10 UE/SA, 98.56% has uplink communication service availability higher or equal than 99.9999%. For the 20 UE/SA case, 90.48% has uplink communication service availability higher or equal than 99.9999%. This aligns with the tabulated values for CSA we have provided in the enclosed excel sheet

For 30 GHz scenario, higher than 99.9999% uplink CSA is achieved by 98.56% UEs for 10 UEs/SA case, and 90.48% for 20 UEs/SA case.

Figure 10. CDF of delay distribution of all UEs in factory hall (DL and UL).

CDFs of observed packet error rates can be seen from Figure 11 and Figure 12 for downlink and uplink respectively.

Figure 11a & 11b. CDF of downlink packet error rate

For the downlink it can be seen from the figures above that around 99.87% of UEs experience packet error rate lower than 10-3, which is quite similar for both cases. This indicates that most of the UEs have good performance, while few experience worse packet losses probability than 10-3.

For 30GHz scenario, 99.87% UEs have packet error lower than 10-3 in downlink, while the few remaining experience inferior packet error rates.

Figure 12a & 12b. CDF of uplink packet error rate

For the uplink it can be seen from the figures above that around 99.6% of UEs for 10UEs/SA case have packet error rate lower than 10-3, and around 98.3% of UEs for 20UEs/SA case have packet error rate lower than 10-3. This indicates that most of the UEs have good performance, while few experience inferior packet losses.

For 30GHz scenario, 99.6% UEs for 10UEs/SA case and 98.3% UEs for 20UEs/SA case have packet error lower than 10-3 in uplink, while the few remaining experience inferior packet error rates.

In addition, coupling loss and geometry are provided for calibration purposes in Figure 13 and Figure 14 respectively.

Figure 13 Coupling loss for 30GHz factory hall

Figure 14 Geometry for 30GHz factory hall

# 3	Conclusions

In the previous sections we made the following observations:

Observation 1	For 4GHz scenario, higher than 99.9999% downlink CSA is achieved by 90.71% UEs for 10 UEs/SA case, and 84.05% for 20 UEs/SA case.

Observation 2	For 4GHz scenario, uplink CSA is excellent, achieving 100% CSA for both, 10 UEs/SA and 20 UEs/SA cases.

Observation 3	For 4GHz scenario, packet delay for successfully received packets is relatively evenly distributed between 0.3 ms and 1 ms.

Observation 4	For 4GHz scenario, 99.5% UEs have packet error rate lower than 10-3 in downlink, while the few remaining experience inferior packet error rates.

Observation 5	For 4GHz scenario, uplink packet error ratio is excellent, showing close to zero packet error ratios for all UEs.

Observation 6	For 30 GHz scenario, higher than 99.9999% downlink CSA is achieved by 94.17% UEs for 10 UEs/SA case, and 83.7% for 20 UEs/SA case.

Observation 7	For 30 GHz scenario, higher than 99.9999% uplink CSA is achieved by 98.56% UEs for 10 UEs/SA case, and 90.48% for 20 UEs/SA case.

Observation 8	For 30GHz scenario, 99.87% UEs have packet error lower than 10-3 in downlink, while the few remaining experience inferior packet error rates.

Observation 9	For 30GHz scenario, 99.6% UEs for 10UEs/SA case and 98.3% UEs for 20UEs/SA case have packet error lower than 10-3 in uplink, while the few remaining experience inferior packet error rates.

# References

RP-201279, “LS on 3GPP NR Rel-16 URLLC and IIoT performance evaluation”, 5G-ACIA

RP-202069, “Way forward on RAN work for 5G ACIA requested simulations“, Ericsson

Agreements week 1 5G-ACIA, Moderator (Ericsson)

Final summary of week 2 5G-ACIA, Moderator (Ericsson)

3GPP TS 22.104, Service requirements for cyber-physical control applications in vertical domains, V16.4.0, December 2019.

# Appendix A. Simulation assumptions for 4GHz

| Parameter name | Parameter Value |
| --- | --- |
| Factory hall size | 120x50 m |
| Room height | 10 m |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. |
| BS/TRP antenna height | 8m |
| Layout – BS/TRP deployment | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| BS antenna mount | Option 1 (1 sector per BS) from 38.824 is used |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5 dB |
| BS antenna configurations | 4 Tx/4 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) for 4 Tx/4 Rx antenna ports; dH = dV = 0.5 λ |
| BS Tx power | 24 dBm per 20 MHz => 31 dBm in total |
| BS receiver | MMSE-IRC |
| UE Tx power | 23dBm |
| UE antenna configuration | 1 Tx/2 Rx antenna ports Isotropic. We observed that most of UEs currently deployed have 1TX/2RX. |
| UE antenna height | 1.5 m |
| UE antenna gain | 0 dBi |
| UE speed | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| UE receiver noise figure | 9 dB |
| Channel model | InF-DH |
| Carrier frequency and simulation bandwidth | TDD, 4 GHz: 100 MHz |
| SCS | 30 kHz |
| TDD DL-UL configuration | 1 slot DL : 1 slot UL, no guard symbols |
| Number of UEs per service area | 10 UEs per service area |
| UE distribution | All UEs randomly distributed within the respective service area. |
| Message size | 48 bytes |
| DL traffic model | DL traffic arrival with option-1 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 |
| CSA requirements | 99.9999% |
| E2E latency & air interface latency | E2E latency: 1 ms; Air interface latency: 1ms |
| UE power control | Open-loop PC |
| HARQ/repetition | Single HARQ transmission (only initial tx is allowed) |
| Channel estimation | Ideal |
| Allocation in time domain | 2 os |
| MIMO | Up to Rank 2 in DL Up to Rank 1 in UL |
| Packets simulated | 300 k packets per UE. |

# Appendix B. Simulation assumptions for 30GHz

| Parameter name | Parameter Value |
| --- | --- |
| Factory hall size | 120x50 m |
| Room height | 10 m |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. |
| BS/TRP antenna height | 8m |
| Layout – BS/TRP deployment | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| BS antenna mount | Option 1 (1 sector per BS) from 38.824 is used |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 7 dB |
| BS antenna configurations | 2 Tx/2 Rx antenna ports (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) for 2 Tx/2 Rx antenna ports; dH = dV = 0.5 λ |
| BS Tx power | 23 dBm per 80 MHz => 26 dBm in total |
| BS receiver | MMSE-IRC |
| UE Tx power | 23dBm |
| UE antenna configuration | 1 Tx/2 Rx antenna ports Isotropic We observed that most of UEs currently deployed have 1TX/2RX. |
| UE antenna height | 1.5 m |
| UE antenna gain | 5 dBi |
| UE speed | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| UE receiver noise figure | 10 dB |
| Channel model | InF-DH |
| Carrier frequency and simulation bandwidth | TDD, 30 GHz: 160 MHz |
| SCS | 120 kHz |
| TDD DL-UL configuration | 1 slot DL : 1 slot UL, no guard symbols |
| Number of UEs per service area | 10 / 20 UEs per service area |
| UE distribution | All UEs randomly distributed within the respective service area. |
| Message size | 48 bytes |
| DL traffic model | DL traffic arrival with option-1 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 |
| CSA requirements | 99.9999% |
| E2E latency & air interface latency | E2E latency: 1 ms; Air interface latency: 1ms |
| UE power control | Open-loop PC |
| HARQ/repetition | Single HARQ transmission (only initial tx is allowed) |
| Channel estimation | Ideal |
| Allocation in time domain | 2 os |
| MIMO | Up to Rank 2 in DL Up to Rank 1 in UL |
| Packets simulated | 300 k packets per UE. |