3GPP TSG RAN WG1 email discussion [5G-ACIA]

e-Meeting, February 22th –26th, 2021

Source:	vivo

Title:	5G-ACIA 2nd round URLLC evaluation results

Agenda Item:

Document for:	Discussion and Decision

- Introduction

According to previous agreements in Annex II [1]-[4], we provide InF-DH scenario calibration results, evaluation methodologies and updated evaluation results for 2nd round 5G-ACIA URLLC evaluation. In this contribution, both coordination and non-coordination transmission are evaluated.

- Calibration results

The CDF of coupling loss and DL geometry for both FR1and FR2 in InF-DH scenario are depicted in below Figure 1 and 2 for inter-company calibration.

| - Coupling loss | - Geometry |
| --- | --- |

Figure 1. CDF of coupling loss and geometry for FR1 in InF-DH scenario

| - Coupling loss | - Geometry |
| --- | --- |

Figure 2. CDF of coupling loss and geometry for FR2 in InF-DH scenario

- Evaluation assumptions

The detailed simulation assumptions for 5G-ACIA URLLC evaluation can be found in the Table I-1 of the Annex I. In addition, the following R15/R16 URLLC features and transmission strategies are applied.

- Low SE MCS and CQI table with 1e-5 target BLER

- Transmission scheme

  - One-shot transmission for each TB (without retransmission) for FR1

  - With HARQ retransmission for FR2

- DL SPS and CG PUSCH periodicity

  - 2 slots (1ms) period for FR1

  - 8 slots (1ms) period for FR2

- Scheduler

  - Survival time aware scheduler, i.e.

    - The gNB will timely re-configure the resource allocation and MCS for the UE when the PDSCH or PUSCH is incorrectly received.

    - The gNB applies an offset (e.g. 2dB) to the reported DL CQI or the measured UL SINR to deduce a more conservative MCS for the UE with NACK packets.

  - SU-MIMO

- The control channel overhead is 1 symbol for PDCCH in downlink and 1 symbol for PUCCH in uplink.

The following evaluation assumptions are considered for E2E latency analysis.

- TDD DL-UL configuration 6D:2S:6U

- PUSCH and PDSCH processing capability 2

For DL E2E latency, it can be observed from the below Table1 and Table 2,

- At least 11 OS processing time is required for one-shot transmission for FR1.

- At least 73 OS processing time is required if retransmission is enabled for FR2.

Note1: [X, Y] in the table means the time needed for the best case and worst case.

Note2: Since the OS number of 1ms for FR2 is 112, UE may not be able to perform retransmission in some cases.

For UL E2E latency, same observations can be obtained as DL.

Table 1. DL user plane latency for FR1

| ID | Component | Value(OS) |
| --- | --- | --- |
| 1 | DL data transfer | [11, 25) |
| 1.1 | BS processing delay | 5.5/2 |
| 1.2 | DL Frame alignment | [0, 14) |
| 1.3 | TTI for DL data packet transmission | 6 |
| 1.4 | UE processing delay | 4.5/2 |
|  | Total no retransmission user plane latency for DL | [11, 25) |

Table 2. DL user plane latency for FR2

| ID | Component | Value(OS) |
| --- | --- | --- |
| 1 | DL data transfer | [26, 40) |
| 1.1 | BS processing delay | 22/2 |
| 1.2 | DL Frame alignment | [0, 14) |
| 1.3 | TTI for DL data packet transmission | 6 |
| 1.4 | UE processing delay | 18/2 |
| 2 | HARQ retransmission | [47, 75) |
| 2.1 | UE processing delay | 18/2 |
| 2.2 | UL frame alignment | [0, 14) |
| 2.3 | TTI for ACK/NACK transmission | 1 |
| 2.4 | BS processing delay | 22/2 |
| 2.5 | Repeat DL data transfer from 1.1 to 1.4 | [26, 40) |
|  | Total one retransmission user plane latency for DL | [73, 115) |

For non-coordination transmission, compared to our 1st round simulation results, more conservative MCS selection schemes are adopted to enhance the CSA performance.

For coordination transmission, all 12 BSs are jointly scheduled by FDMed way, each packet can be accommodated within 1 PRB. For FR1, since the maximum number of supported resources are 273 PRBs for 100 MHz bandwidth, and there are total 2 slots within 1ms, the maximum number of UEs can be FDMed transmission should be no more than 546. Follow the same principle, for FR2, the maximum number of UEs can be FDMed transmission should be no more than 856.

- Evaluation results

Based on the above evaluation assumptions and analyzation, the evaluation results for both non-coordination and coordination transmission are provided.

  - Non-coordination transmission evaluation results

For non-coordination transmission in InF-DH FR1, the CDF of PER, CSA and latency for DL and UL are given in Figure 3, Figure 4 and Figure 5, respectively. The percentage of UEs satisfying 1ms latency and 99.9999% CSA requirement and resource utilization are given in Table 3.

Figure 3. InF-DH FR1 PER performance with non-coordination transmission

Figure 4. InF-DH FR1 CSA performance with non-coordination transmission

Figure 5. InF-DH FR1 latency performance with non-coordination transmission

Table 3 Percentage of UEs satisfying requirements for non-coordination transmission in FR1

| UE number per service area | Percentage of UEs satisfying requirements (%) |  | Resource utilization (%) |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| 10 | 100 | 100 | 5.9 | 6.1 |
| 20 | 100 | 100 | 11.4 | 13.1 |
| 30 | 100 | 100 | 17.9 | 22.6 |
| 40 | 100 | 99.8 | 26.0 | 30.0 |
| 50 | 100 | 99.7 | 40.0 | 43.0 |

From the above simulation results, we have the following observation,

Observation 1:  For non-coordination transmission for FR1,

- The PER performance is degraded with increasing the number of UEs per service area, all PERs are below 1e-4 both for DL and UL.

- There is a neglectable impact on the latency performance with the increasing of the number of UEs per service area.

- For DL with less than or equal to 50 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%.

- For UL with less than or equal to 30 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%. In the case of 40/50 UEs per service area, the percentage of UEs unsatisfying the requirement is less than 1%.

- The resource utilization is growing rapidly with increasing the number of UEs per service area.

  - Coordination transmission evaluation results

    - Evaluation results for FR1

For coordination transmission in FR1 for both DL and UL, the CSA is 100% for UEs per service area up to 40 due to the fully FDMed transmission. The CDF of latency for DL and UL is given in Figure 6. The percentage of UEs satisfying 1ms latency and 99.9999% CSA requirement and resource utilization are given in Table 4.

Figure 6. InF-DH FR1 latency performance with coordination transmission

Table 4 Percentage of UEs satisfying requirements for coordination transmission in FR1

| UE number per service area | Percentage of UEs satisfying requirements (%) |  | Resource utilization (%) |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| 10 | 100 | 100 | 1.8 | 1.8 |
| 20 | 100 | 100 | 3.7 | 3.7 |
| 30 | 100 | 100 | 5.5 | 5.5 |
| 40 | 100 | 100 | 7.3 | 7.3 |

From above simulation results, we have the following observation,

Observation 2:  For coordination transmission for FR1,

- For DL and UL with less than or equal to 40 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%.

    - Evaluation results for FR2

For coordination transmission in FR2, since multi-beam  transmission is adopted in FR2, and all UEs are uniformly distributed within per service area without considering uniformly distributed in each beam, some UEs may not be fully FDMed within a beam with the increasing of UEs per service area. The CDF of PER, CSA and latency for DL and UL are given in Figure 7, Figure 8 and Figure 9, respectively. The percentage of UEs satisfying 1ms latency and 99.9999% CSA requirement and resource utilization are given in Table 5.

Figure 7. InF-DH FR2 PER performance with coordination transmission

Figure 8. InF-DH FR2 CSA performance with coordination transmission

Figure 9. InF-DH FR2 latency performance with coordination transmission

Table 5 Percentage of UEs satisfying requirements for coordination transmission in FR2

| UE number per service area | Percentage of UEs satisfying requirements (%) |  | Resource utilization (%) |  |
| --- | --- | --- | --- | --- |
|  | DL | UL | DL | UL |
| 10 | 100 | 100 | 1.3 | 1.1 |
| 20 | 100 | 100 | 2.8 | 2.3 |
| 30 | 100 | 100 | 4.0 | 3.4 |
| 40 | 100 | 100 | 5.3 | 4.5 |
| 50 | 100 | 100 | 6.6 | 5.5 |
| 60 | 97.8 | 100 | 13.1 | 6.8 |
| 70 | 96.3 | 99.8 | 17.6 | 8.0 |

From above simulation results, we have the following observation,

Observation 3:  For coordination transmission for FR2,

- For DL and UL with less than or equal to 50 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%.

- For DL and UL with 60/70 UEs per service area, some UEs may not be fully FDMed due to ununiform UE distribution within multi-beam.

- Conclusion

In this contribution, we provide our 2nd round 5G-ACIA URLLC evaluation results with the following observations:

Observation 1:  For non-coordination transmission for FR1,

- The PER performance is degraded with increasing the number of UEs per service area, all PERs are below 1e-4 both for DL and UL.

- There is a neglectable impact on the latency performance with the increasing of the number of UEs per service area.

- For DL with less than or equal to 50 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%.

- For UL with less than or equal to 30 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%. In the case of 40/50 UEs per service area, the percentage of UEs unsatisfying the requirement is less than 1%.

- The resource utilization is growing rapidly with increasing the number of UEs per service area.

Observation 2:  For coordination transmission for FR1,

- For DL and UL with less than or equal to 40 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%.

Observation 3:  For coordination transmission for FR2,

- For DL and UL with less than or equal to 50 UEs per service area, the percentage of UEs satisfying the 1ms latency and 99.9999% CSA requirement is 100%.

- For DL and UL with 60/70 UEs per service area, some UEs may not be fully FDMed due to ununiform UE distribution within multi-beam.

- Reference

- RP-202069, 3GPP TSG RAN Meeting #89e, September 14 - 18, 2020

- RP-202097, 3GPP TSG RAN Meeting #89e, September 14 - 18, 2020

- Agreements on URLLC features and simulation assumptions for 5G-ACIA, 3GPP RAN 5G-ACIA Evaluations Week 1, October 12th – 16th 2020

- Summary of discussions in Week 2，3GPP RAN 5G-ACIA Evaluations Week2, December 14th – 18th 2020

# Annex I

Table I-1. Detailed simulation assumptions

| Parameters | Values |
| --- | --- |
| Factory hall size | 120x50 m |
| Room height | 10 m |
| Inter-BS/TRP distance | 20 m |
| BS/TRP antenna height | 8 m InF-DH |
| Layout – BS/TRP deployment | 12 TRPs |
| Channel model | InF-DH |
| Cluster information | Clustter density: 0.6 Cluster height: 6 m Cluster size: 2 m |
| Carrier frequency, simulation bandwidth and SCS | 4 GHz: 100 MHz, 30 KSCS 30 GHz: 160MHz, 120 KSCS |
| TDD DL-UL configuration | S (6D:2S:6U) |
| Number of UEs per service area | Up to 50 per service area 10, 20, 30, 40, and 50 |
| UE distribution | All UEs randomly distributed within the respective service area. |
| Message size | 48 bytes |
| DL traffic model | DL traffic arrival with option-1 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 |
| Simulation packets | 2E6 for DL 2E6 for UL |
| UE speed | Linear movement, 75 km/h |
| UE Tx power | 23 dBm |
| BS/TRP Tx power | 4 GHz: 24 dBm per 20 MHz 30 GHz: 23 dBm per 80 MHz |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 4 GHz: 5 dB 30 GHz: 7 dB |
| UE antenna gain | 4 GHz: 0 dBi 30 GHz: 5 dBi |
| UE receiver noise figure | 9 dB |
| BS antenna configurations | 4 GHz: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH = dV = 0.5 λ 30 GHz: (M, N, P, Mg, Ng; Mp, Np) = (4, 4, 2, 1, 1; 1, 1) dH = dV = 0.5λ |
| UE antenna configuration | 4 GHz: (M, N, P, Mg, Ng; Mp, Np) = (1, 2, 2, 1, 1; 1, 2) dH = dV = 0.5 λ 30 GHz: (M, N, P, Mg, Ng; Mp, Np) = (2, 4, 2, 1, 2; 1, 1) dH = dV = 0.5 λ |
| Receiver | MMSE-IRC |

# Annex II

Agreements of Week1:

- The simulation assumptions given in the table are agreed

- Additional simulation parameters are taken from TR 38.824.

| Parameters | 5G-ACIA LS | Agreement of Week 1 |
| --- | --- | --- |
| Factory hall size | 120x50 m | As in 5G-ACIA LS |
| Room height | 10 m | As in 5G-ACIA LS |
| Inter-BS/TRP distance | Depending on the number of TRPs, which are evenly deployed in the factory hall. Simulation company should provide the number of BSs/TRPs used in the simulation. | According to proposed layout below |
| BS/TRP antenna height | 1.5 m for InF-SL and InF-DL 8m for InF-SH and InF-DH | As in 5G-ACIA LS |
| Layout – BS/TRP deployment | Depending on the number of TRPs | 12 TRPs within area with the same 2D placement as in TR 38.901 and TR 38.824. |
| Channel model | UC-2: InF-DH > InD-DL > InF-SH > InF-SL | Mandatory: InF-DH Optional: InD-DL, InF-SH, InF-SL |
| Carrier frequency and simulation bandwidth | TDD 4 GHz: 100 MHz 30 GHz: 160 MHz | As in 5G-ACIA LS |
| TDD DL-UL configuration | Simulation company should report the used DL-UL configuration. | Companies should report the used DL-UL configuration. 1:1 DL-UL configuration is recommended. |
| Number of UEs per service area | Up to 50 per service area, e.g., 10, 20, 40, and 50 | As in 5G-ACIA LS |
| UE distribution | All UEs randomly distributed within the respective service area. | As in 5G-ACIA LS |
| Message size | 48 bytes | 48 bytes |
| DL traffic model | DL traffic arrival with option-1, option-2, and option-3. | 5G-ACIA Option 1 is mandatory. Companies are also encouraged to provide results for option 3 |
| UL traffic model | UL traffic is symmetric with DL, and DL-UL traffic arrival time relationship with option-1 and option-2 | As in 5G-ACIA LS with Option 1 as mandatory |
| CSA requirements | UC-#2: 99.9999% | UC-#2: 99.9999% |
| Performance metrics | 1) CSA: single CDF of CSA distribution of all UEs in factory hall 2) Latency: single CDF of latency distribution of all UEs in factory hall 3) Percentage of UEs satisfying requirements 4) resource utilization | As in 5G-ACIA LS with 3) and 4) as low priority Note: For metric 2) it is clarified that a packet transmission cannot be performed after the latency deadline. The collected statistics cannot exceed the latency requirement. The packets exceeding the deadline are visible in the UE packet error statistics |
| E2E latency & air interface latency | E2E latency: 1 ms for UC#2 | E2E latency: 1 ms for UC#2 Air interface latency: 1ms |
| UE speed | Linear movement | Linear movement: 75 km/h No explicit UE mobility (nor handovers) are modeled in the evaluations. |
| BS antenna mount |  | Option 1 (1 sector per BS) from 38.824 is used |

For the Rel-15 baseline, the following is agreed:

- Rel-15 URLLC features included in the baseline are as follows, while it is up to each proponent to decide which Rel-15 features are used, and detail this when providing the results:

  - UE Processing capability 2

  - UL Configured grant

  - DL Semi-persistent scheduling

Regarding Rel-16 features, the following is agreed:

- It is up to each proponent to decide on which Rel-16 features to provide simulations results for in addition to the Rel-15 baseline

- This can be revisited after the first round of simulations has been provided in December.

Conclusions of Week2:

Conclusion on colleting simulation assumptions:

- The final Excel sheet can be found here.

Conclusion on FR2 antenna assumptions:

- 2RX/TX is still the baseline

- Results for additional configurations can be provided

Conclusion on cell coordination:

- No coordination is baseline

- Results with cell coordination can be provided

Conclusion on MU-MIMO:

- SU-MIMO is baseline

- Results with MU-MIMO can be provided

Agreement for latency:

- For FR1 companies are encouraged to provide simulation results for one-shot transmission

- Noted proposal for latency

- For the E2E latency, following assumptions are made:

  - Components from table 5.7.1.1.1.-1 for DL and table 5.7.1.1.2.-1 for UL from TR 37.910 are used to calculate the E2E latency

    - In case re-tx is simulated, the alignment delay for the re-TX at the gNB side (which is not included in the tables from the TR 37.910) should also be added to the latency

  - Companies report the UE processing delay and gNB processing delay, for other components, the values from table 5.7.1.1.1.-1 for DL and table 5.7.1.1.2.-1 for UL from TR 37.910 are assumed

- Supported by Intel, Huawei/HiSilicon, vivo, ITRI, ZTE

Conclusion on additional simulation assumptions:

- No consensus on CSA metric with no consecutive errors is mandatory

- Narrow down channel model to InF-DH explicitly

- Option-1 for DL traffic and Option-1 for UL traffic relationship to DL is still baseline. Additional results can be submitted

- Number of samples, minimum number of packets per UE and minimum number of UEs / network drops modelled are left to companies’ choice

Conclusions on format for submissions to round 2

- Companies will provide

  - CDF of packet error rate for UL and DL

  - CDF of CSA for UL and DL

  - Tabulated values for percentage of UEs satisfying 1ms latency and 99.9999% reliability/CSA requirement for each simulated case

  - CDF for coupling loss and geometry for calibration