3GPP TSG-RAN5 Meeting #91-e	R5-212101

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2097 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of SMS over NAS TC 9.1.8.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As step 21a1 refer to step 20a1 in Table 4.5.2.2-2, no need to release RRC connection again in 21A and the steps could be simplified. The RRC connections steps should be added for the service request procedure in Steps 38 & 49. To perform step 22-24, SS should wait for the expiry of T3512, not just start it in Step 21A. The step descriptions could be simplifed when referring to the generic procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test steps and step descriptions were simplified with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test steps will be imcomplete and mislead case implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.8.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

9.1.8.1	SMS over NAS / MO and MT SMS over NAS - Idle mode

9.1.8.1.1	Test Purpose (TP)

(1)

with { the UE in switched off state with valid USIM inserted }

ensure that {

when { the UE requests initial registration for SMS over NAS }

then { the UE shall send REGISTRATION REQUEST message with SMS requested bit of the 5GS update type IE "SMS over NAS supported" }

}

(2)

with { the UE in 5GMM-REGISTERED state }

ensure that {

when { the UE initiates a periodic registration update and the requirements to use SMS over NAS transport have not changed in the UE}

then { the UE sets the SMS requested bit of the 5GS updatetype IE in the REGISTRATION REQUEST message to the same value as indicated by the UE in the last REGISTRATION REQUEST message }

}

(3)

with { the UE in 5GMM_Connected state with NR RRC_IDLE mode and the UE has sent a SERVICE REQUEST message triggered by initiating MO SMS}

ensure that {
  when { UE receives a SERVICE ACCEPT message from SS }

then { UE sends CP-DATA containing RP-DATA RPDU (SMS SUBMIT TPDU) encapsulated in an Uplink NAS transport message }

}

(4)

with { UE has sent CP-DATA containing an RP-DATA RPDU (SMS SUBMIT TPDU) encapsulated in an Uplink NAS transport message }

ensure that {
  when { UE receives a CP-DATA containing an RP-ACK RPDU encapsulated in a Downlink NAS transport message }

then { UE sends a CP-ACK encapsulated in an Uplink NAS Transport message }

}

(5)

with { the UE in 5GMM-REGISTERED state with NR RRC_IDLE mode, UE has received a paging request and UE has completed a SERVICE REQUEST procedure}

ensure that {
  when { UE receives a CP-DATA containing an RP-DATA RPDU (SMS DELIVER TPDU) encapsulated in a Downlink NAS transport message }

then { UE sends a CP-ACK encapsulated in an Uplink NAS transport message followed by a CP-DATA containing an RP-ACK RPDU encapsulated in an Uplink NAS transport message}

}

9.1.8.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 5.5.1.2.2, 5.5.1.2.4, 5.5.1.3.2, 5.5.1.3.4, 9.11.3.6 and 9.11.3.9A. Unless otherwise stated these are Rel-15 requirements.

[TS 24.501 clause 5.5.1.2.2]

The UE in state 5GMM-DEREGISTERED shall initiate the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE performs initial registration for 5GS services;

b)	when the UE performs initial registration for emergency services;

c)	when the UE performs initial registration for SMS over NAS; and

d)	when the UE moves from GERAN to NG-RAN coverage or the UE moves from a UTRAN to NG-RAN coverage.

…

The UE initiates the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF, starting timer T3510. If timer T3502 is currently running, the UE shall stop timer T3502. If timer T3511 is currently running, the UE shall stop timer T3511.

…

If the UE requests the use of SMS over NAS, the UE shall include the 5GS update type IE in the REGISTRATION REQUEST message with the SMS requested bit set to “SMS over NAS supported”.  When the 5GS update type IE is included in the REGISTRATION REQUEST for reasons other than requesting the use of SMS over NAS, and the UE does not need to register for SMS over NAS, the UE shall set the SMS requested bit of the 5GS update type IE to “SMS over NAS not supported” in the REGISTRATION REQUEST message.

[TS 24.501 clause 5.5.1.2.4]

If the initial registration request is accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE.

…

If the 5GS update type IE was included in the REGISTRATION REQUEST message with the SMS requested bit set to “SMS over NAS supported”, and SMSF selection is successful, then the AMF shall send the REGISTRATION ACCEPT message after the SMSF has confirmed that the activation of the SMS service was successful. When sending the REGISTRATION ACCEPT message, the AMF shall:

a)	set the SMS allowed bit of the 5GS registration result IE to “SMS over NAS allowed” in the REGISTRATION ACCEPT message, if the UE has set the SMS requested bit of the 5GS registration type IE to “SMS over NAS supported” in the REGISTRATION REQUEST message and the network allows the use of SMS over NAS for the UE; and

b)	store the SMSF address and the value of the SMS allowed bit of the 5GS registration result IE in the UE 5GMM context and consider the UE available for SMS over NAS.

[TS 24.501 clause 5.5.1.3.2]

The UE in state 5GMM-REGISTERED shall initiate the registration procedure for mobility and periodic registration update by sending a REGISTRATION REQUEST message to the AMF,

l)	when the UE needs to register for SMS over NAS, indicate a change in the requirements to use SMS over NAS, or de-register from SMS over NAS;

…

The UE in state 5GMM-REGISTERED shall initiate the registration procedure for mobility and periodic update by sending a REGISTRATION REQUEST message to the AMF when the UE needs to request the use of SMS over NAS transport or the current requirements to use SMS over NAS transport change in the UE. The UE shall set the SMS requested bit of the 5GS update type IE in the REGISTRATION REQUEST message as specified in subclause 5.5.1.2.2.

When initiating a registration procedure for mobility and periodic registration update and the UE needs to send the 5GS update type IE for a reason different than indicating a change in requirement to use SMS over NAS, the UE shall set the SMS requested bit of the 5GS update type IE in the REGISTRATION REQUEST message to the same value as indicated by the UE in the last REGISTRATION REQUEST message.

If the UE no longer requires the use of SMS over NAS, then the UE shall include the 5GS update type IE in the REGISTRATION REQUEST message with the SMS requested bit set to “SMS over NAS not supported”.

[TS 24.501 clause 5.5.1.3.4]

If the registration update request has been accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE.

…

If the 5GS update type IE was included in the REGISTRATION REQUEST message with the SMS requested bit set to “SMS over NAS not supported” and:

a)	the SMSF address is stored in the UE 5GMM context and:

1)	the UE is considered available for SMS over NAS; or

2)	the UE is considered not available for SMS over NAS and the SMSF has confirmed that the activation of the SMS service is successful; or

b)	the SMSF address is not stored in the UE 5GMM context, the SMSF selection is successful and the SMSF has confirmed that the activation of the SMS service is successful;

then the AMF shall set the SMS allowed bit of the 5GS registration result IE in the REGISTRATION ACCEPT message as specified in subclause 5.5.1.2.4. If the UE 5GMM context does not contain an SMSF address or the UE is not considered available for SMS over NAS, then the AMF shall:

a)	store the SMSF address in the UE 5GMM context if not stored already; and

b)	store the value of the SMS allowed bit of the 5GS registration result IE in the UE 5GMM context and consider the UE available for SMS over NAS.

If SMSF selection in the AMF or SMS activation via the SMSF is not successful, or the AMF does not allow the use of SMS over NAS, then the AMF shall set the SMS allowed bit of the 5GS registration result IE to “SMS over NAS not allowed" in the REGISTRATION ACCEPT message.

If the 5GS update type IE was included in the REGISTRATION REQUEST message with the SMS requested bit set to "SMS over NAS not supported", then the AMF shall:

a)	mark the 5GMM context to indicate that the UE is not available for SMS over NAS; and

NOTE 2:	The AMF can notify the SMSF that the UE is deregistered from SMS over NAS based on local configuration.

b)	set the SMS allowed bit of the 5GS registration result IE to "SMS over NAS not supported" in the REGISTRATION ACCEPT message.

When the UE receives the REGISTRATION ACCEPT message, if the UE is also registered over another access to the same PLMN, the UE considers the value indicated by the SMS allowed bit of the 5GS registration result IE as applicable for both accesses over which the UE is registered.

[TS 24.501 clause 9.11.3.6]

The purpose of the 5GS registration result information element is to specify the result of a registration procedure.

The 5GS registration result information element is coded as shown in figure 9.11.3.6.1 and table 9.11.3.6.1.

The 5GS registration result is a type 4 information element with a length of 3 octets.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS registration result IEI |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of 5GS registration result contents |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | SMS allowed |  | 5GS registration result value |  |  |  | octet 3 |  |

Figure 9.11.3.6.1: 5GS registration result information element

Table 9.11.3.6.1: 5GS registration result information element

| 5GS registration result value (octet 3, bits 1 to 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 1 |  | 3GPP access |
| 0 | 1 | 0 |  | Non-3GPP access |
| 0 | 1 | 1 |  | 3GPP access and non-3GPP access |
| 1 | 1 | 1 |  | reserved |
| All other values are unused and shall be treated as "3GPP access", if received by the UE. |  |  |  |  |
| SMS over NAS transport allowed (SMS allowed) (octet 3, bit 4) |  |  |  |  |
| Bit |  |  |  |  |
| 4 |  |  |  |  |
| 0 |  |  |  | SMS over NAS not allowed |
| 1 |  |  |  | SMS over NAS allowed |
| Bits 5 to 8 of octet 3 are spare and shall be coded as zero. |  |  |  |  |

[TS 24.501 clause 9.11.3.9A]

The purpose of the 5GS update type IE is to allow the UE to provide additional information to the network when performing a registration procedure.

The 5GS update type information element is coded as shown in figure 9.11.3.9A.1 and table 9.11.3.9A.1.

The 5GS update type is a type 4 information element.

| 8 |  | 7 |  | 6 |  | 5 |  | 4 |  | 3 |  | 2 |  | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS update type IEI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 1 |  |
| Length of 5GS update type |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | octet 2 |  |
| 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | 0 Spare |  | NG-RAN-RCU |  | SMS requested |  | octet 3 |  |

Figure 9.11.3.9A.1: 5GS update type information element

Table 9.11.3.9A.1: 5GS update type information element

| SMS over NAS transport requested (SMS requested) (octet 3, bit 1) |  |  |
| --- | --- | --- |
| Bit |  |  |
| 1 |  |  |
| 0 |  | SMS over NAS not supported |
| 1 |  | SMS over NAS supported |
| NG-RAN Radio Capability Update (NG-RAN-RCU) (octet 3, bit 2) |  |  |
| Bits |  |  |
| 2 |  |  |
| 0 |  | NG-RAN radio capability update not needed |
| 1 |  | NG-RAN radio capability update needed |
| Bits 3 to 8 of octet 3 are spare and shall be coded as zero. |  |  |

9.1.8.1.3	Test description

9.1.8.1.3.1	Pre-test conditions

System Simulator:

NGC Cell A belongs to Home PLMN and TAI1;

UE:

The UE does not have any stored SMS message.

Preamble:

The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 [4].

9.1.8.1.3.2	Test procedure sequence

Table 9.1.8.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched ON. | - | - | - | - |
| 2 - 4 | UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS 38.508-1 [4]. | - | - | - | - |
| 5 | Check: Does UE transmit a REGISTRATION REQUEST message including 5GS update type IE with SMS requested bit set to "SMS over NAS supported"? | --> | REGISTRATION REQUEST | 1 | P |
| 6 - 14 | Steps 5 to 13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4]are performed. | - | - | - | - |
| 15 | SS transmits REGISTRATION ACCEPT message including 5GS registration result with SMS allowed bit set to “SMS over NAS allowed” and T3512 value set to 3 minutes. | <-- | REGISTRATION ACCEPT | - | - |
| 16 -21a1 | Steps 15 to 20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] . | - | - | - | - |
| 21A | Wait for the expiry of T3512. | - | - | - | - |
| 22 - 24 | UE establishes RRC connection by executing steps 2-4 of Table 4.5.2.2-2 in TS38.508-1 [4]. | - | - | - | - |
| 25 | Check: Does UE perform periodic Registration, including 5GS update type IE with SMS requested bit set to "SMS over NAS supported"? | --> | REGISTRATION REQUEST | 2 | P |
| 26 - 34 | Void. | - | - | - | - |
| 35 | SS transmits REGISTRATION ACCEPT message including 5GS registration result with SMS allowed bit set to “SMS over NAS allowed” and T3512 value set to 3 minutes. | <-- | REGISTRATION ACCEPT | - | - |
| 35A | The UE transmits a REGISTRATION COMPLETE message. | --> | REGISTRATION COMPLETE | - | - |
| 36 | The SS transmits an RRCRelease message. | - | - | - | - |
| 37 | Sending of a 160 character MO SMS is initiated at the UE via MMI or AT command. | - | - | - | - |
| 38 | The UE establishes RRC connection and transmits a SERVICE REQUEST message by executing steps 2-4 of Table 4.5.4.2-2 in TS 38.508-1 [4]. | --> | SERVICE REQUEST |  |  |
| 39 -42 | Steps 5 to 8 of Table 4.5.4.2-3 of the generic procedure in TS 38.508-1 [4]are performed. | - | - | - | - |
| 43 | The UE transmits a CP-DATA containing an RP-DATA RPDU (SMS SUBMIT TPDU) encapsulated in an Uplink NAS transport message. | --> | UPLINK NAS TRANSPORT | 3 | P |
| 44 | The SS transmits a CP-ACK encapsulated in a Downlink NAS Transport message. | <-- | DOWNLINK NAS TRANSPORT | - | - |
| 45 | The SS transmits a CP-DATA containing an RP-ACK RPDU encapsulated in a Downlink NAS transport message | <-- | DOWNLINK NAS TRANSPORT | - | - |
| 46 | Check: Does the UE transmit a CP-ACK encapsulated in an Uplink NAS Transport message? | --> | UPLINK NAS TRANSPORT | 4 | P |
| 47 | The SS transmits an RRCRelease message | - | - | - | - |
| 48 | The SS pages the UE using NG-5G-S-TMSI. | - | - | - | - |
| 49 | The UE establishes RRC connection and transmits a SERVICE REQUEST message by executing steps 2-4 of Table 4.5.4.2-3 in TS 38.508-1 [4]. | --> | SERVICE REQUEST | - | - |
| 50-53 | Steps 5 to 8 of Table 4.5.4.2-3 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 54 | The SS transmits a CP-DATA containing a RP-DATA RPDU (SMS DELIVER TPDU) encapsulated in a Downlink NAS transport message to the UE. | <-- | DOWNLINK NAS TRANSPORT | - | - |
| 55 | Check: Does the UE transmit a CP-ACK encapsulated in an Uplink NAS transport message? | --> | UPLINK NAS TRANSPORT | 5 | P |
| 56 | Check: Does the UE transmit a CP-DATA containing a RP-ACK RPDU encapsulated in an Uplink NAS transport message? | --> | UPLINK NAS TRANSPORT | 5 | P |
| 57 | The SS transmits a CP-ACK encapsulated in a Downlink NAS transport message to the UE. | <-- | DOWNLINK NAS TRANSPORT | - | - |

9.1.8.1.3.3	Specific message contents

Table 9.1.8.1.3.3-1: REGISTRATION REQUEST (step 5, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘001’B |  | INITIAL |
| 5GS update type |  |  |  |  |
| SMS requested |  | SMS over NAS supported |  |  |

Table 9.1.8.1.3.3-2: REGISTRATION ACCEPT (steps 15 and 35, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration result |  |  |  |  |
| SMS allowed |  | SMS over NAS allowed |  |  |
| T3512 value |  |  |  |  |
| Timer value |  | ‘00011’B |  |  |
| Unit |  | ‘101’B |  |  |

Table 9.1.8.1.3.3-3: REGISTRATION REQUEST (step 25, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| 5GS registration type |  |  |  |  |
| 5GS registration type value |  | ‘011’B |  | PERIODIC |
| 5GS update type |  |  |  |  |
| SMS requested |  | SMS over NAS supported |  |  |

Table 9.1.8.1.3.3-3A: SERVICE REQUEST (step 38, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-16 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Service type |  |  |  |  |
| Service type value |  | ‘0000’B | signalling |  |

Table 9.1.8.1.3.3-4: UL NAS TRANSPORT (step 43, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-DATA | RP-DATA RPDU |  |

Table 9.1.8.1.3.3-5: DL NAS TRANSPORT (step 44, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-ACK |  |  |

Table 9.1.8.1.3.3-6: DL NAS TRANSPORT (step 45, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-DATA | RP-ACK RPDU |  |

Table 9.1.8.1.3.3-7: UL NAS TRANSPORT (step 46, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-ACK |  |  |

Table 9.1.8.1.3.3-8: DL NAS TRANSPORT (step 54, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-DATA | RP-DATA |  |

Table 9.1.8.1.3.3-9: UL NAS TRANSPORT (step 55, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-ACK |  |  |

Table 9.1.8.1.3.3-10: UL NAS TRANSPORT (step 56, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-10 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-DATA | RP-ACK |  |

Table 9.1.8.1.3.3-11: DL NAS TRANSPORT (step 57, Table 9.1.8.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-11 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Payload container type |  | ‘0010’B | SMS |  |
| Payload container |  | CP-ACK |  |  |

[End of change]