3GPP TSG-RAN5 Meeting # 91-e	R5-213159

# Electronic Meeting, 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-3 | CR | 2585 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | FDD Inter Frequency Absolute RSRQ Accuracy CA Idle Mode Measurements for Non-Overlapping Carrier |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_euCA-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add RSRQ absolute accuracy RRM testcase for CA Idle mode measurements for inter frequency non-overlapping carrier. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding RRM testcase for CA Idle mode measurements inter frequency non-overlapping carrier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Absolute accuracy of RSRQ for CA Idle mode measurement inter frequency non-overlapping carrier won’t be verified against requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 36.521-2, 36.521-3 CR 0963, 2582, 2583, 2584, 2586 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------------------------------------------------------- Omitted chapters ------------------------------------------------------------

--------------------------------------------------------------- Start of changes ------------------------------------------------------------

##### 9.2.3.1_2.5	Test requirement

Table 9.2.3.1_2.5-2 defines the primary level settings including test tolerances for all tests.

Each RSRQ FDD - FDD inter frequency absolute accuracy test shall meet the reported values test requirements in Table 9.2.3.1_2.5-3.

Table 9.2.3.1_2.5-1: Void

Table 9.2.3.1_2.5-2: Cell Specific Test requirement Parameters for RSRQ FDD - FDD inter frequency absolute accuracy

| Parameter |  | Unit |  | Test 1 |  |  |  | Test 2 |  |  | Test 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 1 | Cell 2 |  |  | Cell 1 | Cell 2 |  | Cell 1 |  | Cell 2 |  |
| E-UTRA RF Channel Number |  |  |  | 1 | 2 |  |  | 1 | 2 |  | 1 |  | 2 |  |
| BWchannel |  | MHz |  | 10 | 10 |  |  | 10 | 10 |  | 10 |  | 10 |  |
| Gap Pattern Id |  |  |  | 0 | - |  |  | 0 | - |  | 0 |  | - |  |
| Measurement bandwidth |  |  | 22—27 |  |  |  |  | 22—27 |  |  | 22—27 |  |  |  |
| PDSCH Reference measurement channel defined in A.1.1 |  |  | R.0 FDD |  |  |  | - | R.0 FDD |  | - | R.0 FDD |  |  | - |
| PDSCH allocation |  |  | 13—36 |  |  |  | - | 13—36 |  | - | 13—36 |  |  | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.2.1 |  |  | R.6 FDD |  |  |  |  | R.6 FDD |  |  | R.6 FDD |  |  |  |
| OCNG Patterns defined in D.1.1 (OP.1 FDD) and D.1.2 (OP.2 FDD) |  |  | OP.1 FDD |  |  | OP.2 FDD |  | OP.1 FDD |  | OP.2 FDD | OP.1 FDD |  |  | OP.2 FDD |
| PBCH_RA |  | dB |  | 0 | 0 |  |  | 0 | 0 |  | 0 | 0 |  |  |
| PBCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz |  | -80.0 | -81.1 |  |  | -104.7 | -104.7 |  | -119.5 |  | -119.2 |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  | -118.5 |  | -118.2 |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  | -118 |  | -117.7 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  | -117.5 |  | -117.2 |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |  | -116.5 |  | -116.2 |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  | -116 |  | -115.7 |  |
|  |  | dB |  | -1.75 | -1.75 |  |  | -4.0 | -3.2 |  | -4.0 |  | -3.2 |  |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz |  | -81.75 | -82.85 |  |  | -108.70 | -107.90 |  | -123.5 |  | -122.4 |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  | -122.5 |  | -121.4 |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  | -122.0 |  | -120.9 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  | -121.5 |  | -120.4 |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |  | -120.5 |  | -119.4 |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  | -120.0 |  | -118.9 |  |
| RSRQNote3 | Bands FDD_A | dB |  | -14.76 | -14.76 |  |  | -16.25 | -15.69 |  | -16.25 |  | -15.69 |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz |  | -50.00 | -51.10 |  |  | -75.46 | -75.22 |  | -90.26 |  | -89.72 |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |  |  | -89.26 |  | -88.72 |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |  |  | -88.76 |  | -88.22 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |  |  | -88.26 |  | -87.72 |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |  |  | -87.26 |  | -86.72 |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |  |  | -86.76 |  | -86.22 |  |
|  |  | dB |  | -1.75 | -1.75 |  |  | -4.0 | -3.2 |  | -4.0 |  | -3.2 |  |
| Propagation condition |  | - |  | AWGN |  |  |  | AWGN |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in TS 36.133 [4] section 3.5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 9.2.3.1_2.5-3: RSRQ FDD - FDD inter frequency absolute accuracy requirements
for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRQ_02 | RSRQ_00 | RSRQ_00 |
| Highest reported value (Cell 2) | RSRQ_18 | RSRQ_18 | RSRQ_18 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRQ_00 | RSRQ_00 | RSRQ_00 |
| Highest reported value (Cell 2) | RSRQ_21 | RSRQ_19 | RSRQ_19 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

--------------------------------------------------------- If approved, testcase 9.2.3.1_3 -------------------------------------------------

#### 9.2.3.1_4	FDD - FDD Inter Frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements for Non-overlapping Carriers

##### 9.2.3.1_4.1	Test purpose

To verify that the FDD - FDD inter frequency absolute RSRQ measurement accuracy for CA idle mode measurements for non-overlapping carriers is within the specified limit for all bands.

##### 9.2.3.1_4.2	Test applicability

This test applies to E-UTRA FDD UE release 15 and forward capable of IDLE mode measurements for CA. This information is received from UECapabilityInformation.

##### 9.2.3.1_4.3	Minimum conformance requirements

The absolute accuracy of RSRQ is defined as the RSRQ measured from a cell that has different carrier frequency from the serving cell.

The accuracy requirements in table 9.2.3.1_4.3-1 are valid under the following conditions:

Cell specific reference signals are transmitted either from one, two or four antenna ports.

Conditions defined in TS 36.101 [2] clause 7.3 for reference sensitivity are fulfilled.

RSRP|dBm according to Annex I.3.3 for a corresponding Band.

Table 9.2.3.1_4.3-1: RSRQ FDD - FDD inter frequency absolute accuracy requirements for CA idle mode measurements for non-overlapping carriers

| Accuracy |  | Conditions |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |
|  |  |  | E-UTRA operating band groups Note 4 | Minimum Io | Maximum Io |
| dB | dB | dB |  | dBm/15kHz Note 3 | dBm/BWChannel |
| 5 | 6.5 | -3 dB | FDD_A, TDD_A | -121 | -50 |
|  |  |  | FDD_B1, FDD_B2 | -120.5 | -50 |
|  |  |  | FDD_C, TDD_C | -120 | -50 |
|  |  |  | FDD_D | -119.5 | -50 |
|  |  |  | FDD_E, TDD_E | -119 | -50 |
|  |  |  | FDD_F | -118.5 | -50 |
|  |  |  | FDD_G | -118 | -50 |
|  |  |  | FDD_H | -117.5 | -50 |
|  |  |  | FDD_N | -114.5 | -50 |
| 6 | 6.5 | -6 dB | Note 2 | Note 2 | Note 2 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 3: The condition level is increased by ∆>0, when applicable, as described in Sections B.4.2 and B.4.3. NOTE 4: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |

The reporting range of RSRQ is defined from -19.5 dB to -3 with 0.5 dB resolution. The mapping of measured quantity is defined in Table 9.2.3.1_4.3-2.

Table 9.2.3.1_4.3-2: RSRQ FDD - FDD Inter frequency absolute accuracy measurement report mapping

| Reported value | Measured quantity value | Unit |
| --- | --- | --- |
| RSRQ_00 | RSRQ -19.5 | dB |
| RSRQ_01 | -19.5 RSRQ < -19 | dB |
| RSRQ_02 | -19 RSRQ < -18.5 | dB |
| … | … | … |
| RSRQ_32 | -4 RSRQ < -3.5 | dB |
| RSRQ_33 | -3.5 RSRQ < -3 | dB |
| RSRQ_34 | -3 RSRQ | dB |

The normative reference for this requirement is TS 36.133 [4] clause 9.1.6B.3 and A.9.2.62.

##### 9.2.3.1_4.4	Test description

###### 9.2.3.1_4.4.1	Initial conditions

Test Environment: Normal, TL/VL, TL/VH, TH/VL, TH/VH; as defined in TS 36.508 [7] clause 4.1.

Frequencies to be tested: According to Annex E table E-1 and TS 36.508 [7] clauses 4.4.2 and 4.3.1.1.

Channel Bandwidth to be tested: 10 MHz as defined in TS 36.508 [7] clause 4.3.1.1.

1. Connect the SS (node B emulator) and AWGN noise sources to the UE antenna connectors as shown in TS 36.508 [7] Annex A figure A.14 for UE with 2Rx RF band and Annex A, Figure A.78 (without using the faders) for 4Rx capable UE without any 2Rx RF bands.

2. Propagation conditions are set according to Annex B clause B.0.

3. Message contents are defined in clause 9.2.3.1_4.4.3.

4. There are two E-UTRA FDD carriers and one cell on each carrier specified in each test. Cell 1 is the cell used for connection setup with the power level set according to Annex C.0 and C.1 for this test.

###### 9.2.3.1_4.4.2	Test procedure

The three tests consist of Cell 1, serving cell and Cell 2, target cell. The absolute accuracy of RSRQ is defined as the RSRQ measured from a cell that has different carrier frequency from the serving cell, Cell 1.

1.	System information combination 3 as defined in TS 36.508 [7] clause 4.4.3.1 is used in E-UTRA cells.

2.	The UE is in state Generic RB Established (state 3) on Cell 1 according to TS 36.508 [7].

3.	The SS transmits an RRCConnectionRelease message including the measIdleConfig on cell 1.

4.	Wait for 30 s for the UE to enter E-UTRA RRC_IDLE state on Cell 1 and do measurements over cells 1 and 2.

5.	The SS sends RRC Paging message with UE S-TMSI on the cell 1.

6.	The UE shall transmit an RRCConnectionRequest message on the cell 1.

7.	The SS transmits a RRCConnectionSetup message.

8.	The UE shall transmit a RRCConnectionSetupComplete message with idleMeasAvailable.

9.	The SS transmits a SecurityModeCommand message to activate AS security.

10.	The UE shall transmit a SecurityModeComplete message.

11.	The SS transmits a RRCConnectionReconfiguration message to establish the default bearer with condition SRB2-DRB(1, 0) .

12.	The UE shall transmit an RRCConnectionReconfiguration complete message.

13.	The SS transmits UEInformationRequest message with idleModeMeasurementReq included.

14.	The UE shall transmit UEInformationResponse message including measResultListIdle.

15.	SS shall check the RSRQ value of Cell 1 and Cell 2 from measResultListIdle message. The RSRQ value of Cell 2 reported by the UE is compared to the actual RSRQ value according to the Table 9.2.3.1_4.5.2. If the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one.

16.	SS shall check the MeasurementReport messages transmitted by the UE until the confidence level according to Tables G.2.3-1 in Annex G.2 is achieved.

17.	Repeat step 1-16 for each sub-test in Table 9.2.3.1_4.5.2 as appropriate.

###### 9.2.3.1_4.4.3	Message contents

Message contents are according to TS 36.508 [7] values 4.6 with the following exceptions:

Table 9.2.3.1_4.4.3-1: RRCConnectionRelease

| Derivation Path: 36.508 Table 4.6.1-15 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionRelease ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionRelease-r8 SEQUENCE { |  |  |  |  |
| releaseCause |  | other |  |  |
| redirectedCarrierInfo |  | Not present |  |  |
| idleModeMobilityControlInfo |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measIdleConfig-r15 |  | MeasIdleConfigDedicated-r15 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.1_4.4.3-2: MeasIdleConfigDedicated-r15

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasIdleConfigDedicated-r15 ::= SEQUENCE { |  |  |  |
| measIdleCarrierListEUTRA-r15 | EUTRA-CarrierList-r15 |  |  |
| measIdleDuration-r15 | sec60 |  |  |
| } |  |  |  |

Table 9.2.3.1_4.4.3-3: EUTRA-CarrierList-r15

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EUTRA-CarrierList-r15 ::= SEQUENCE ( | 2 entries |  |  |
| MeasIdleCarrierEUTRA-r15[1] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 1 |  |  |
| allowedMeasBandwidth-r15 | mbw50 | 10 MHz carrier |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | Not present |  |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| MeasIdleCarrierEUTRA-r15[2] SEQUENCE { |  |  |  |
| carrierFreq-r15 | Same downlink EARFCN as used for Cell 2 |  |  |
| allowedMeasBandwidth-r15 | mbw50 | 10 MHz carrier |  |
| validityArea-r15 | Not present |  |  |
| measCellList-r15 | Not present |  |  |
| reportQuantities | both |  |  |
| qualityThreshold-r15 SEQUENCE { |  |  |  |
| idleRSRP-Threshold-r15 | Not present |  |  |
| idleRSRQ-Threshold-r15 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 9.2.3.1_4.4.3-4: RRCConnectionSetupComplete

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCConnectionSetupComplete ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE { |  |  |  |  |
| rrcConnectionSetupComplete-r8 SEQUENCE { |  |  |  |  |
| selectedPLMN-Identity |  | 1 |  |  |
| registeredMME SEQUENCE {} |  | Not checked |  |  |
| dedicatedInfoNAS SEQUENCE {} |  | Present but contents not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| up-CIoT-EPS-Optimisation-r13 |  | true |  | UP-CIoT |
| cp-CIoT-EPS-Optimisation-r13 |  | true |  | CP-CIoT |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| idleMeasAvailable-r15 |  | true | Value expected |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.1_4.4.3-5: UEInformationRequest

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationRequest-r9 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-DL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationRequest-r9 SEQUENCE { |  |  |  |  |
| rach-ReportReq-r9 |  | FALSE |  |  |
| rlf-ReportReq-r9 |  | FALSE |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UEInformationRequest-v930-IEs |  |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UEInformationRequest-v1020-IEs |  |
| logMeasReportReq-r10 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UEInformationRequest-v1130-IEs |  |
| connEstFailReportReq-r11 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UEInformationRequest-v1250-IEs |  |
| mobilityHistoryReportReq-r12 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  | UEInformationRequest-v1530-IEs |  |
| idleModeMeasurementReq-r15 |  | true |  |  |
| flightPathInfoReq-r15 SEQUENCE {} |  | Not present |  |  |
| nonCriticalExtension SEQUENCE {} |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.1_4.4.3-6: UEInformationResponse

| Derivation Path: 36.331 clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| UEInformationResponse-r9 ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier-UL |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueInformationResponse-r9 SEQUENCE { |  |  |  |  |
| rach-Report-r9 SEQUENCE {} |  | Not checked |  |  |
| rlf-Report-r9 SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| logMeasReport-r10 SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  | Not checked |  |  |
| connEstFailReport-r11 SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mobilityHistoryReport-r12 SEQUENCE {} |  | Not checked |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measResultListIdle-r15 |  | MeasResultListIdle-r15 |  |  |
| flightPathInfoReport-r15 |  | Not checked |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

Table 9.2.3.1_4.4.3-6 MeasResultListIdle-r15

| Derivation Path: 36.331, clause 6.3.5 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| MeasResultListIdle-r15::= SEQUENCE { |  | List Measurements |  |  |
| MeasResultIdle-r15 SEQUENCE { |  | Cell 1 measurements |  |  |
| measResultServingCell-r15 SEQUENCE { |  |  |  |  |
| rsrpResult-r15 |  | RSRP-Range, |  |  |
| rsrqResult-r15 |  | RSRQ-Range-r13 |  |  |
| } |  |  |  |  |
| measResultNeighCells-r15 CHOICE { |  |  |  |  |
| measResultIdleListEUTRA-r15 SEQUENCE { |  |  |  |  |
| MeasResultIdleEUTRA-r15 SEQUENCE { |  |  |  |  |
| carrierFreq-r15 |  | ARFCN-ValueEUTRA-r9 | Cell2 freq |  |
| physCellId-r15 |  |  |  |  |
| measResult-r15 SEQUENCE { |  |  |  |  |
| rsrpResult-r15 |  | RSRP-Range |  |  |
| rsrqResult-r15 |  | RSRQ-Range-r13 |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

##### 9.2.3.1_4.5	Test requirement

Table 9.2.3.1_4.5-1 defines the primary level settings including test tolerances for all tests.

Each RSRQ FDD - FDD inter frequency absolute accuracy test shall meet the reported values test requirements in Table 9.2.3.1_4.5-2.

Table 9.2.3.1_4.5-1: Cell Specific Test requirement Parameters for RSRQ FDD - FDD inter frequency absolute accuracy for CA idle mode measurements for non-overlapping carriers

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| E-UTRA RF Channel Number |  |  | 1 |  | 1 |  | 1 |  |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Measurement bandwidth |  |  | 22—27 |  | 22—27 |  | 22—27 |  |
| PDSCH Reference measurement channel defined in A.1.1 |  |  | R.0 FDD | - | R.0 FDD | - | R.0 FDD | - |
| PDSCH allocation |  |  | 13—36 | - | 13—36 | - | 13—36 | - |
| PDCCH/PCFICH/PHICH Reference measurement channel defined in A.2.1 |  |  | R.6 FDD |  | R.6 FDD |  | R.6 FDD |  |
| OCNG Patterns defined in D.1.1 (OP.1 FDD) and D.1.2 (OP.2 FDD) |  |  | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD | OP.1 FDD | OP.2 FDD |
| PBCH_RA |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| PBCH_RB |  |  |  |  |  |  |  |  |
| PSS_RA |  |  |  |  |  |  |  |  |
| SSS_RA |  |  |  |  |  |  |  |  |
| PCFICH_RB |  |  |  |  |  |  |  |  |
| PHICH_RA |  |  |  |  |  |  |  |  |
| PHICH_RB |  |  |  |  |  |  |  |  |
| PDCCH_RA |  |  |  |  |  |  |  |  |
| PDCCH_RB |  |  |  |  |  |  |  |  |
| PDSCH_RA |  |  |  |  |  |  |  |  |
| PDSCH_RB |  |  |  |  |  |  |  |  |
| OCNG_RANote1 |  |  |  |  |  |  |  |  |
| OCNG_RBNote1 |  |  |  |  |  |  |  |  |
| Note2 | Bands FDD_A | dBm/15 kHz | -80 | -81.1 | -104.7 | -104.7 | -119.5 | -119.2 |
|  | FDD_B1, FDD_B2 |  |  |  |  |  | -119 | -118.7 |
|  | Bands FDD_C |  |  |  |  |  | -118.5 | -118.2 |
|  | Bands FDD_D |  |  |  |  |  | -118 | -117.7 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -117.5 | -117.2 |
|  | Bands FDD_G |  |  |  |  |  | -116.5 | -116.2 |
|  | Bands FDD_H |  |  |  |  |  | -116 | -115.7 |
|  |  | dB | -1.75 | -1.75 | -4 | -3.2 | -4 | -3.2 |
| RSRPNote3 | Bands FDD_A | dBm/15 kHz | -81.75 | -82.85 | -108.7 | -107.9 | -123.5 | -122.4 |
|  | FDD_B1, FDD_B2 |  |  |  |  |  | -123 | -121.9 |
|  | Bands FDD_C |  |  |  |  |  | -122.5 | -121.4 |
|  | Bands FDD_D |  |  |  |  |  | -122 | -120.9 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -121.5 | -120.4 |
|  | Bands FDD_G |  |  |  |  |  | -120.5 | -119.4 |
|  | Bands FDD_H |  |  |  |  |  | -120 | -118.9 |
| RSRQNote3 | Bands FDD_A | dB | -14.76 | -14.76 | -16.25 | -15.69 | -16.25 | -15.69 |
|  | FDD_B1, FDD_B2 |  |  |  |  |  |  |  |
|  | Bands FDD_C |  |  |  |  |  |  |  |
|  | Bands FDD_D |  |  |  |  |  |  |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  |  |  |
|  | Bands FDD_G |  |  |  |  |  |  |  |
|  | Bands FDD_H |  |  |  |  |  |  |  |
| IoNote3 | Bands FDD_A | dBm/9 MHz | -50 | -51.1 | -75.46 | -75.22 | -90.26 | -89.72 |
|  | FDD_B1, FDD_B2 |  |  |  |  |  | -89.76 | -89.22 |
|  | Bands FDD_C |  |  |  |  |  | -89.26 | -88.72 |
|  | Bands FDD_D |  |  |  |  |  | -88.76 | -88.22 |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -88.26 | -87.72 |
|  | Bands FDD_G |  |  |  |  |  | -87.26 | -86.72 |
|  | Bands FDD_H |  |  |  |  |  | -81.99 | -86.22 |
|  |  | dB | -1.75 | -1,75 | -4.0 | -3.2 | -4.0 | -3.2 |
| Propagation condition |  | - | AWGN |  | AWGN |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRQ, RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP and RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: For Band 26, the tests shall be performed with the carrier frequency of the assigned E-UTRA channel bandwidth within 865-894 MHz. Note 6: E-UTRA operating band groups are as defined in Section 3.5. |  |  |  |  |  |  |  |  |

Table 9.2.3.1_4.5-2: RSRQ FDD - FDD inter frequency absolute accuracy requirements
for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRQ_00 | RSRQ_00 | RSRQ_00 |
| Highest reported value (Cell 2) | RSRQ_21 | RSRQ_21 | RSRQ_21 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | RSRQ_00 | RSRQ_00 | RSRQ_00 |
| Highest reported value (Cell 2) | RSRQ_24 | RSRQ_22 | RSRQ_22 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

#### 9.2.3.2	FDD - FDD Inter Frequency Relative Accuracy of RSRQ

##### 9.2.3.2.1	Test purpose

To verify that the FDD - FDD inter frequency relative accuracy measurement of RSRQ is within the specified limit for all bands.

---------------------------------------------------------------- End of changes ------------------------------------------------------------