3GPP TSG-RAN5 Meeting #91-e	R5-212683

# Electronic Meeting, 17th May 2021 – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5021 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add test case 7.3.5.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add new test case PDCP handover / DAPS handover with key change / Status reporting in accordance with work plan. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The test case 7.3.5.6 has been initiated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work plan cannot be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### 7.3.5.6	PDCP handover / DAPS handover with key change / Status reporting

7.3.5.6.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state and supporting DAPS handover }

ensure that {

when { UE receives an RRCConnectionReconfiguration message includes the mobilityControlInfo for DAPS handover with key change }

then { PDCP entity associated with a DAPS bearer shall perform ciphering/deciphering under two sets of keys }

}

(2)

with { UE in RRC_CONNECTED state and supporting DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for DAPS handover with key change }

ensure that {

when { upper layers request uplink data switching during DAPS handover }

then { UE shall send a PDCP status report for the DAPS bearer }

}

(3)

with { UE in RRC_CONNECTED state and supporting DAPS handover and having received RRCConnectionReconfiguration message includes the mobilityControlInfo for handover with key change }

ensure that {

when { upper layers reconfigure the PDCP entity to release DAPS and daps-SourceRelease is configured }

then { UE shall send a PDCP status report for the DAPS bearer }

}

7.3.5.6.2	Conformance requirements

FFS

7.3.5.6.2	Test description

7.3.5.6.2.1	Pre-test conditions

System Simulator:

-	Cell 1 and Cell 2.

UE:

-	None.

Preamble:

-	The UE is in state Loopback Activated (state 4) according to [18].

7.3.5.6.2.2	Test procedure sequence

FFS

7.3.5.6.2.3	Specific message contents

FFS

<End of modified section>