3GPP TSG-RAN5 Meeting #91-e	R5-214021

# Online, , 17th May 2021 - 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1199 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating clause 7.3C for Rel-17 SUL combinations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecommunications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 7.3C needs to be updated to align with the latest release 17 38.101-1 for the combination of “CA_n1A_SUL_n78A-n80A”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updating 7.3C about the combination of “CA_n1A_SUL_n78A-n80A” to align with core spec. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The combination can not be tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3C.0, 7.3C.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-212042r1. |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

7.3C.0.3	ΔRIB,c for SUL

7.3C.0.3.1	General

For a UE supporting a SUL configuration, the ΔRIB,c applies for both SC and SUL operation.

7.3C.0.3.2	SUL band combination

For the UE which supports SUL band combination, the minimum requirement for reference sensitivity in subclause 7.3C.0 shall be increased by the amount given in ΔRIB,c defined in subclause 7.3C.0.3 for the applicable operating bands. Unless otherwise stated, ΔRIB,c is set to zero.

In case the UE supports more than one of band combinations for CA, SUL or DC, and an operating band belongs to more than one band combinations then

-	When the operating band frequency range is ≤ 1 GHz, the applicable additional ΔRIB,c shall be the average value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14], truncated to one decimal place that apply for that operating band among the supported band combinations. In case there is a harmonic relation between low band UL and high band DL, then the maximum ΔRIB,c among the different supported band combinations involving such band shall be applied

-	When the operating band frequency range is > 1 GHz, the applicable additional ΔRIB,c shall be the maximum value for all band combinations defined in subclause 7.3A, 7.3B, 7.3C in this specification and 7.3A, 7.3B in TS 38.521-3 [14] for the applicable operating bands.

7.3C.0.3.2.1	ΔRIB,c  for two bands

Table 7.3C.0.3.2.1-1: ΔRIB,c due to SUL (two bands)

| Band combination for SUL | NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| SUL_n78-n80 | n78 | 0.5 |
| SUL_n78-n81 | n78 | 0.5 |
| SUL_n78-n82 | n78 | 0.5 |
| SUL_n78-n83 | n78 | 0.5 |
| SUL_n78-n84 | n78 | 0.5 |
| SUL_n78-n86 | n78 | 0.5 |

7.3C.0.3.2.2	ΔRIB,c for three bands

Table 7.3C.0.3.2.2-1: ΔRIB,c due to SUL (three bands)

| Band combination for SUL | NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| CA_n1_SUL_n78-n80 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n1_SUL_n78-n84 | n1 | 0.2 |
|  | n78 | 0.5 |
| CA_n28_SUL_n41-n83 | n28 | 0.2 |

The normative reference for this requirement is TS 38.101-1 [2] clause 7.3C.2 and 7.3C.3.

<Unchanged Text Skipped>

7.3C.3	Reference sensitivity power level for SUL (3CC)

Editor’s Note:

- No test points defined for Reference sensitivity power level testing for SUL with DL CA. This test case is covered by 7.3.2 and 7.3C.2.

7.3C.3.1	Test purpose

The test purpose is to verify the ability of the UE to receive data with a given average throughput for a specified reference measurement channel, under SUL and 2 DL CA operation and conditions of low signal level, ideal propagation and no added noise.

7.3C.3.2	Test applicability

This test case applies to all types of NR UE release 15 and forward that supports SUL operation on the SUL bands with downlink 2CA.

7.3C.3.3	Minimum conformance requirement

The minimum conformance requirements are defined in clause 7.3C.0.

7.3C.3.4	Test description

NOTE:	No testing needs to be performed since the testing has been covered in test case 7.3.2 and 7.3C.2.

For band combination CA_nX_SUL_nY-nZ, test the REFSENS of SUL configuration or NR band as listed in table 7.3C.3.4-1.

Table 7.3C.3.4-1: Test band combinations and configuration

| Band configuration | Verifying REFSENS of SUL configurations/ NR band | Subtest case | Table with test parameters to select |
| --- | --- | --- | --- |
| CA_n1A_SUL_n78A-n80A | SUL_n78A-n80A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n1_SUL_n78-n84 | SUL_n78A-n84A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n1 | 7.3.2 | Table 7.3.2.4.1-1 |
| CA_n28_SUL_n41-n83 | SUL_n41A-n83A | 7.3C.2 | Table 7.3C.2.4.1-1 |
|  | n28 | 7.3.2 | Table 7.3.2.4.1-1 |

7.3C.3.5	Test requirement

Same test requirement as clause 7.3.2 and 7.3C.2 for each band or band combinations listed in table 7.3C.3.4-1.

<Unchanged Text Skipped>