3GPP TSG-RAN5 Meeting #91-e	R5-213095

# Electronic Meeting, 17th – 28th May 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0078 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding test applicability for new test cases introduced in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New test cases 7.6C.2_1 and 7.6C.3_1 are introduced into TS 38.521-1. The test applicabilities should be updated accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introducing new test applicabilities in tables 4.0-1 and 4.0-3 for newly added test cases in table 4.1.1-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test applicability for new test cases in TS 38.521-1 is absent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.0; 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2 and Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A

C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C002 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C003 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C004 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C005 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.1-2/4 AND A.4.3.2A.1-1/1 AND A.4.1-3/1 THEN R ELSE N/A

C006 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1) THEN R ELSE N/A

C006a IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/4) AND A.4.3.5-1/1 THEN R ELSE N/A

C006b IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C007 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/4) AND A.4.3.2-1/22 THEN R ELSE N/A

C008 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/4) AND NOT(A.4.3.2-1/22) THEN R ELSE N/A

C009 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C009a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C009z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C010 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C010a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C010z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C011 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C011a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C011z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C012 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C012a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C012b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/2 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A

C012c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/3 AND A.4.3.2B.2.0-2A/3 THEN R ELSE N/A

C012d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/4 AND A.4.3.2B.2.0-2A/4 THEN R ELSE N/A

C012e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1/2 AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C012z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C013 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C014 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C015 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND A.4.3.2-1/33 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C016 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016y IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND A.4.3.2-1/20 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C017 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C017b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/6 THEN R ELSE N/A

C017x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.9-1/1 THEN R ELSE N/A

C017y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/33 THEN R ELSE N/A

C018 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C019 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C019b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.2-1/6 THEN R ELSE N/A

C019x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/4 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.9-1/1 THEN R ELSE N/A

C020 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C021 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C021a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C022 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C023 IF A.4.1-4/5 AND A.4.1-3/2 THEN R ELSE N/A

C023a IF A.4.1-4/5 AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C024 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C025 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 THEN R ELSE N/A

C025a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C026 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND 4.3.6-1/11 THEN R ELSE N/A

C027 Void

C028 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.6-1/11 THEN R ELSE N/A

C029 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C030 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND 4.3.2-1/9 THEN R ELSE N/A

C031 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C032 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C033 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/2 THEN R ELSE N/A

C034 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/6 THEN R ELSE N/A

C035 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C036 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/3 THEN R ELSE N/A

C037 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C037a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C038 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C038a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C039 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C040 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C041 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C041a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C042a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C043 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (4.3.6-1/43 OR 4.3.6-1/44) THEN R ELSE N/A

C044 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.6-1/42 THEN R ELSE N/A

C045 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND (A.4.1-4/1 OR (A.4.1-4/3 AND A.4.3.2B.2.0-1A/2)) THEN R ELSE N/A

C046 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND (A.4.1-4/1 OR (A.4.1-4/3 AND A.4.3.2B.2.0-1A/3)) THEN R ELSE N/A

C047 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/4 AND (A.4.1-4/3 AND A.4.3.2B.2.0-1A/4) THEN R ELSE N/A

C048 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND A.4.1-4/1 THEN R ELSE N/A

C049 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND A.4.1-4/1 THEN R ELSE N/A

C050 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/36 THEN R ELSE N/A

C051 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/37 THEN R ELSE N/A

C052 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.11-1/1 THEN R ELSE N/A

C0yy IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) AND A.4.1-4A/1 AND A.4.3.2A.1-1/1 THEN R ELSE N/A

NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25).

For the purposes of the present document, the tested bands selection criteria given in Table 4.0-2 apply. The ICS proformas used in Table 4.0-2 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-2: Tested Bands Selection Criteria

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| D001 | (A.4.3.1-1 OR A.4.3.1-2) AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FR1 Bands without SUL/SDL bands |
| D002 | A.4.3.1-4 | All supported FR1 PC2 Bands |
| D003 | A.4.3.1-5 | All supported FR1 SUL Bands |
| D004 | {1,2,3,5,7,8,12,14,20,25,28,30,34,38,39,40,41,50,51,65,66,70,71,74,75,76} | UE supported bands among n1,n2,n3,n5,n7,n8,n12,n14,n20,n25,n28,n30,n34,n38,n39,n40,n41,n50,n51,n65,n66,n70,n71,n74,n75,n76 |
| D005 | A.4.3.1-3 | All supported FR2 Bands |
| D006 | A.4.3.1-1 OR A.4.3.1-2 | All supported FR1 Bands |
| D007 | A.4.3.1-1 OR A.4.3.1-2 OR A.4.3.1-3 | All supported NR Bands |
| D008 | ANY((A.4.3.1-1 OR A.4.3.1-2) AND 10MHz) | Any band within the set supporting 10 MHz UE Channel BW |
| D009 | ANY((A.4.3.1-2) AND 20MHz) | Any TDD FR1 band within the set supporting 20 MHz UE Channel BW |
| D010 | ANY((A.4.3.1-2) AND 40MHz) | Any TDD FR1 band within the set supporting 40 MHz UE Channel BW |
| D011 | A.4.3.9-4a OR A.4.3.9-4b | All supported 4 Rx antenna ports Bands |
| D012 | A.4.3.1-1 AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FDD bands without SUL/SDL bands |
| NOTE 1: Band Selection is based on set theory. For each feature, item number shall correspond to the Band number. The result is the set of bands for which the test shall be conducted. The following operators are used: AND: Set intersection ( ). {1,2} AND {2,3} = {2} OR: Set union ( ). {1,2} OR {2,3} = {1,2,3} NOT: Set complement (\), full set being all bands. NOT{1} = {2 ...256} Also note that this is set without repetitions so {1} AND {1} = {1} The following basic sets are used: {1,2}: Explicitly given band set 10MHz: All bands supporting 10 MHz The following sets derived from pro-forma tables are also used: TBD |  |  |

For the purposes of the present document, the tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The ICS proformas used in Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-3: Tested CA/DC Configuration Selection Criteria

| Code | Tested CA Configuration Selection Criteria | Comment |
| --- | --- | --- |
| E001 | A.4.3.2A.2.1-3 AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.2.1-2) | All supported intra-band contiguous CA Configurations with 2 carriers in DL but no CA in UL |
| E002 | A.4.3.2A.4.1-1 AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.4.1-2) | All supported inter-band CA Configurations with 2 carriers in DL but no CA in UL |
| E003 | UL(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band contiguous EN-DC configurations in FR1(2UL CCs) |
| E003a | A.4.3.2B.2.1-2 AND A.4.3.2B.2.0-1/1 | All supported Intra-band contiguous EN-DC configurations in FR1(2DL CCs) |
| E004 | UL(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1(2UL CCs) |
| E004a | A.4.3.2B.2.2-2 AND A.4.3.2B.2.0-1/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1(2DL CCs) |
| E005 | UL(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations within FR1 (2UL CCs) |
| E005a | A.4.3.2B.2.3.1-2 AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations within FR1 (2DL CCs) |
| E006 | (A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2) AND A.4.3.2B.2.0-1/2 | All supported EN-DC configurations within FR1 (3DL CCs) |
| E007 | (A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2) AND A.4.3.2B.2.0-1/3 | All supported EN-DC configurations within FR1 (4DL CCs) |
| E008 | (A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2) AND A.4.3.2B.2.0-1/4 | All supported EN-DC configurations within FR1 (5DL CCs) |
| E009 | (A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/5 | All supported EN-DC configurations within FR1 (6DL CCs) |
| E010 | UL(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2 OR A.4.3.2B.2.3.10-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations including FR2 (2UL CCs) |
| E010a | A.4.3.2B.2.3.6-2 AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations including FR2 (2DL CCs) |
| E011 | UL(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3) AND A.4.3.2B.2.0-2/2 AND A.4.3.2B.2.0-2A/2 | All supported Inter-band EN-DC configurations including FR2 (3UL CCs(2NR UL CCs)) |
| E011a | (A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3) AND A.4.3.2B.2.0-1/2 AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations including FR2 (3DL CCs(2NR DL CCs)) |
| E012 | UL(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2) AND A.4.3.2B.2.0-2/3 AND NR_A.4.3.2B.2.0-2A/3 | All supported Inter-band EN-DC configurations including FR2 (4UL CCs(3NR UL CCs)) |
| E012a | (A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2) AND A.4.3.2B.2.0-1/3 AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations including FR2 (4DL CCs(3NR DL CCs)) |
| E013 | UL(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2) AND A.4.3.2B.2.0-2/4 AND A.4.3.2B.2.0-2A/4 | All supported Inter-band EN-DC configurations including FR2 (5UL CCs(4NR UL CCs)) |
| E013a | (A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2) AND A.4.3.2B.2.0-1/4 AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations including FR2 (5DL CCs(4NR DL CCs)) |
| E014 | (A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/5 AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations including FR2 (6DL CCs(5NR DL CCs)) |
| E015 | UL(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2B.2.0-2/1 | All supported FR1 2UL CA configurations |
| E016 | (A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3) AND A.4.3.2B.2.0-1/1 | All supported FR1 2DL CA configurations |
| E017 | (A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2B.2.0-1/2 | All supported FR1 3DL CA configurations |
| E018 | (A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2B.2.0-1/3 | All supported FR1 4DL CA configurations |
| E019 | ULSwitching(A.4.3.2A.4.1-3) | All supported FR1 2UL CA configurations with ULTxSwitching capability |
| E0yy | SUL with intra-band contiguous DL CA (A.4.3.2C.3-2) | All supported FR1 intra-band contiguous 2DL CA with SUL in uplink Configurations |

4.1	RF conformance test cases

NOTE:	To determine applicability of a test case, supported CBW and SCS in the RF-Parameters IE (see TS 38.331 [11]) which conveys RF related capabilities for NR operation is taken into account.

4.1.1	FR1 standalone conformance test cases

Table 4.1.1-1: Applicability of RF SA FR1 conformance test cases, ref. TS 38.521-1 [1]

| Clause | TC Title | Release | Applicability |  | Tested Bands/CA-Configurations Selection | Branch | Additional Information |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Condition | Comment |  |  |  |
| 6 | Transmitter Characteristics |  |  |  |  |  |  |
| 6.2.1 | UE maximum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC1 PC2 PC3 |  |
| 6.2.2 | Maximum Power Reduction (MPR) | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed. Skip TC 6.2.2 if UE supports NSA and TS 38.521-3 TC 6.2B.2.3 has been executed. |
| 6.2.3 | UE additional maximum output power reduction | Rel-15 | C001 | UEs supporting 5GS FR1 PC3 | D001 | PC2 PC3 | Test execution is not necessary if TS 38.521-1 6.5.2.3 and 6.5.3.3 are executed. Skip TC 6.2.3 if UE supports NSA and TS 38.521-3 TC 6.2B.3.3 has been executed. |
| 6.2.4 | Configured transmitted power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.2A.1.1 | UE maximum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.2A.2.1 | Maximum Power Reduction (MPR) for CA (2 UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  | Test execution is not necessary if TS 38.521-1 6.5A.2.4.1.1 is executed. |
| 6.2A.3.1 | UE additional maximum output power reduction for CA (2 UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  | Test execution is not necessary if TS 38.521-1 6.5A.2.3 and 6.5A.3.3 are executed. |
| 6.2A.4.1 | Configured transmitted power for CA (2 UL CA) | Rel-15 | C004 | Ues supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.2C.1 | Configured transmitted power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.3 | UE maximum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.4 | UE maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2C.5 | UE additional maximum output power reduction for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.2D.1 | UE maximum output power for UL-MIMO | Rel-15 | N/A | Not recommended due to no test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4. | D001 |  | Maximum Output Power for UL-MIMO is tested as part of the MPR test case with using MPR=1.5dB suggested by RAN4. |
| 6.2D.2 | UE maximum output power reduction for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  | Test execution is not necessary if TS 38.521-1 6.5D.2.4.1 is executed. |
| 6.2D.3 | UE additional maximum output power reduction for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  | Test execution is not necessary if TS 38.521-1 6.5D.2.3 and 6.5D.3.3 are executed. |
| 6.2D.4 | Configured transmitted power for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.3.1 | Minimum output power | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.3.1 if UE supports NSA and TS 38.521-3 TC 6.3B.1.1 or 6.3B.1.2 or 6.3B.1.3 has been executed. |
| 6.3.3.2 | General ON/OFF time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.3.3.2 if UE supports NSA and TS 38.521-3 TC 6.3B.3.1 or 6.3B.3.2 or 6.3B.3.3 has been executed. |
| 6.3.3.4 | PRACH time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.3.3.4 if UE supports NSA and TS 38.521-3 TC 6.3B.4.1 or 6.3B.4.2 or 6.3B.4.3 has been executed. |
| 6.3.3.6 | SRS time mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.4.2 | Absolute power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.4.3 | Relative power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3.4.4 | Aggregate power tolerance | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.3A.1.1 | Minimum output power for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.3.1 | Transmit ON/OFF time mask for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.3.1_1 | Time mask for switching between two uplink carriers | Rel-16 | C051 | UEs supporting 5GS FR1 and Inter-band CA (2UL CA) and dynamic UL Tx switching | E019 |  | NOTE 1 |
| 6.3A.4.1 | Absolute power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.4.2 | Power Control Relative power tolerane for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3A.4.3 | Aggregate power tolerance for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | E015 |  |  |
| 6.3C.1 | Minimum output power for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.3 | Transmit ON/OFF time mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.4.1 | Absolute power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.4.2 | Power Control Relative power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3C.4.3 | Aggregate power tolerance for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.3D.1 | Minimum output power for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.3D.3 | Transmit ON/OFF time mask for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.3D.4.1 | Absolute power tolerance for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.3D.4.2 | Relative power tolerance for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.3D.4.3 | Aggregate power tolerance for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4.1 | Frequency error | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.1 if UE supports NSA and TS 38.521-3 TC 6.4B.1.1 or 6.4B.1.2 or 6.4B.1.3 has been executed. |
| 6.4.2.1 | Error Vector Magnitude | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.1 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.1 or 6.4B.2.2.1 or 6.4B.2.3.1 has been executed. |
| 6.4.2.2 | Carrier leakage | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.2 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.2 or 6.4B.2.2.2 or 6.4B.2.3.2 has been executed. |
| 6.4.2.3 | In-band emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.3 if UE supports NSA and TS 38.521-3 TC 6.4B.2.2.3 or 6.4B.2.3.3 has been executed. |
| 6.4.2.4 | EVM equalizer spectrum flatness | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.4.2.4 if UE supports NSA and TS 38.521-3 TC 6.4B.2.1.4 or 6.4B.2.2.4 or 6.4B.2.3.4 has been executed. |
| 6.4.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK | Rel-15 | C050 | UEs supporting 5GS FR1 and pi/2-BPSK modulation scheme for power boosting in FR1 | D001 |  |  |
| 6.4A.1.1 | Frequency error for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4A.2.1.1 | Error Vector Magnitude for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4A.2.2.1 | Carrier leakage for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4A.2.3.1 | In-band emissions for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.4C.1 | Frequency error for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.1 | Error Vector Magnitude for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.2 | Carrier leakage for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.3 | In-band emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.4 | EVM equalizer spectrum flatness for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.4C.2.5 | EVM equalizer spectrum flatness for Pi/2 BPSK for SUL | Rel-16 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  | NOTE 1 |
| 6.4D.1 | Frequency error for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4D.2.1 | Error Vector Magnitude for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4D.2.2 | Carrier leakage for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4D.2.3 | In-band emissions for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4D.2.4 | EVM equalizer spectrum flatness for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4D.3 | Time alignment error for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.4D.4 | Requirements for coherent UL MIMO | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.4E.2.2.1 | Error Vector Magnitude for V2X / non-concurrent operation | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 6.5.1 | Occupied bandwidth | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.1 if UE supports NSA and TS 38.521-3 TC 6.5B.1.2 or 6.5B.1.3 has been executed. |
| 6.5.2.2 | Spectrum Emission Mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC2 PC3 | Skip TC 6.5.2.2 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.1 or 6.5B.2.3.1 has been executed. |
| 6.5.2.3 | Additional spectrum emission mask | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC2 PC3 | NOTE 1 Skip TC 6.5.2.3 if UE supports NSA and TS 38.521-3 TC 6.5B.2.3.2 has been executed. |
| 6.5.2.4.1 | NR ACLR | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | PC2 PC3 | Skip TC 6.5.2.4.1 if UE supports NSA and TS 38.521-3 TC 6.5B.2.2.3 or 6.5B.2.3.3 has been executed. |
| 6.5.2.4.2 | UTRA ACLR | Rel-15 | C001a | UEs supporting 5GS FR1 PC3 | D001 |  |  |
| 6.5.3.1 | General spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.3.1 if UE supports NSA and TS 38.521-3 TC 6.5B.3.1.1 or 6.5B.3.2.1 has been executed. |
| 6.5.3.2 | Spurious emission for UE co-existence | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 6.5.3.3 | Additional spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.3.3 if UE supports NSA and TS 38.521-3 TC 6.5B.4.3 has been executed. |
| 6.5.4 | Transmit intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 6.5.4 if UE supports NSA and TS 38.521-3 TC 6.5B.5.3 has been executed. |
| 6.5A.1.1 | Occupied bandwidth for CA (2UL CA) | Rel-15 | C004 | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.2.2.1 | Spectrum emission mask for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.2.4.1.1 | NR ACLR for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.2.4.2.1 | UTRA ACLR for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.3.1.1 | General spurious emissions for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.3.2.1 | Spurious emissions for UE co-existence for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5A.4.1 | Transmit intermodulation for CA (2UL CA) | Rel-15 | FFS | UEs supporting 5GS FR1 and CA (2UL CA) | FFS |  | NOTE 1 |
| 6.5C.1 | Occupied bandwidth for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.2 | Spectrum Emission Mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.3 | Additional spectrum emission mask for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.4.1 | NR ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.2.4.2 | UTRA ACLR for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.3.1 | General spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.3.2 | Spurious emission for UE co-existence for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.3.3 | Additional spurious emissions for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5C.4 | Transmit intermodulation for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 6.5D.1 | Occupied bandwidth for UL-MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.1_1 | Occupied bandwidth for UL MIMO (Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL-MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.2.2 | Spectrum Emission Mask for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.2.3 | Additional spectrum emission mask for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.2.4.1 | NR ACLR for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.2.4.2 | UTRA ACLR for UL-MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.2_1.4.2 | UTRA ACLR for UL MIMO (Rel-16 onward) | Rel-16 | FFS | FFS | FFS |  | NOTE 1 |
| 6.5D.3.1 | General spurious emissions for UL-MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.3.2 | Spurious emission for UE co-existence for UL-MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.3.3 | Additional spurious emissions for UL-MIMO | Rel-15 only | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 6.5D.3_1.1 | General spurious emissions for UL-MIMO (Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL-MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.3_1.2 | Spurious emission for UE co-existence for UL-MIMO (Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL-MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.3_1.3 | Additional spurious emissions for UL-MIMO(Rel-16 onward) | Rel-16 | FFS | UEs supporting 5GS FR1 and UL-MIMO and ULFPTx | FFS |  | NOTE 1 |
| 6.5D.4 | Transmit intermodulation for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D001 |  |  |
| 7 | Receiver Characteristics |  |  |  |  |  |  |
| 7.3.2 | Reference sensitivity power level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 | 2Rx 4Rx |  |
| 7.3A.1 | Reference sensitivity power level for 2DL CA without exception | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.3A.1_1 | Reference sensitivity power level for 2DL CA exceptions | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.3A.2 | Reference sensitivity level for CA (3DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3A.3 | Reference sensitivity level for CA (4DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.3C.2 | Reference sensitivity power level for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.3D.2 | Reference sensitivity power level for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.4 | Maximum input level | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.4 if UE supports NSA and TS 38.521-3 TC 7.4B.3 or 7.4B.4 has been executed. |
| 7.4A.1 | Maximum input level for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.4A.2 | Maximum input level for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.4A.3 | Maximum input level for CA (4DL CA) | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.4D | Maximum input level for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.5 | Adjacent channel selectivity | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | NOTE 1 Skip TC 7.5 if UE supports NSA and TS 38.521-3 TC 7.5B.2 or 7.5B.3 has been executed. |
| 7.5A.1 | Adjacent channel selectivity for 2DL CA | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.5A.2 | Adjacent channel selectivity for 3DL CA | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5A.3 | Adjacent channel selectivity for 4DL CA | FFS | FFS | FFS | FFS |  | NOTE 1 |
| 7.5D | Adjacent channel selectivity for UL-MIMO | Rel-15 | C003 | UEs supprtoing 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.6.2 | Inband Blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.6.2 if UE supports NSA and TS 38.521-3 TC 7.6B.2.2 or 7.6B.2.3 has been executed. |
| 7.6.3 | Out-of-band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 7.6.4 | Narrow band blocking | Rel-15 | C001 | UEs supporting 5GS FR1 | D004 |  | Skip TC 7.6.4 if UE supports NSA and TS 38.521-3 TC 7.6B.4.2 or 7.6B.4.3 has been executed. |
| 7.6A.2.1 | In-band Blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.2.2 | Inband blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.2.3 | Inband blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | NOTE 1 Skip TC 7.6A.2.3 if UE supports NSA and TS 38.521-3 TC 7.6B.2.3_1.3 has been executed. |
| 7.6A.3.1 | Out-of-band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.3.2 | Out-of-band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.3.3 | Out-of-band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |
| 7.6A.4.1 | Narrow band blocking for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.6A.4.2 | Narrow band blocking for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.6A.4.3 | Narrow band blocking for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  | Skip TC 7.6A.4.3 if UE supports NSA and TS 38.521-3 TC 7.6B.4.3_1.3 has been executed. |
| 7.6C.2 | Inband blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.6C.2_1 | Inband Blocking for SUL with DL CA | Rel-17 | C0yy | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E0yy |  |  |
| 7.6C.3 | Out-of-band blocking for SUL | Rel-15 | C002 | UEs supporting 5GS FR1 and SUL | D003 |  |  |
| 7.6C.3_1 | Out-of-band blocking for SUL with DL CA | Rel-17 | C0yy | UEs supporting 5GS FR1 and SUL and intra-band contiguous CA (2DL CA) | E0yy |  |  |
| 7.6D.2 | Inband blocking for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.6D.3 | Out-of-band blocking for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.6D.4 | Narrow band blocking for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.7 | Spurious response | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  |  |
| 7.7A.1 | Spurious response for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.7A.2 | Spurious response for CA (3DL CA) | Rel-16 | FFS | UEs supporting 5GS FR1 and CA (3DL CA) | FFS |  | NOTE 1 |
| 7.7A.3 | Spurious response for CA (4DL CA) | Rel-16 | FFS | UEs supporting 5GS FR1 and CA (4DL CA) | FFS |  | NOTE 1 |
| 7.7D | Spurious response for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.8.2 | Wide band Intermodulation | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.8.2 if UE supports NSA and TS 38.521-3 TC 7.8B.2.2 or 7.8B.2.3 has been executed. |
| 7.8A.2.1 | Wide band Intermodulation for CA (2DL CA) | Rel-15 | C031 | UEs supporting 5GS FR1 and CA (2DL CA) | E016 |  |  |
| 7.8A.2.2 | Wide band Intermodulation for CA (3DL CA) | Rel-16 | C033 | UEs supporting 5GS FR1 and CA (3DL CA) | E017 |  |  |
| 7.8A.2.3 | Wide band Intermodulation for CA (4DL CA) | Rel-16 | C036 | UEs supporting 5GS FR1 and CA (4DL CA) | E018 |  |  |
| 7.8D.2 | Wide band Intermodulation for UL-MIMO | Rel-15 | C003 | UEs supporting 5GS FR1 and UL-MIMO | D012 |  |  |
| 7.9 | Spurious emissions | Rel-15 | C001 | UEs supporting 5GS FR1 | D001 |  | Skip TC 7.9 if UE supports NSA and TS 38.521-3 TC 7.9B.1 or 7.9B.2 or 7.9B.3 has been executed. |
| 7.9A.1 | Spurious emission for 2DL CA | Rel-15 | C005 | UEs supporting 5GS FR1 and inter-band 2DL CA with a DL-only band | E002 |  |  |
| NOTE 1: The test case/branch is incomplete for any Band/CA/DC Configuration but has basic test configurations already. NOTE 1 can be removed only when the test case/branch is complete for at least one Band / CA/DC Configuration for at least one feature included in the test case/branch. Detailed completion status can be found in the corresponding test case section in 38.521-1. |  |  |  |  |  |  |  |

Table 4.1.1-1a: Void

Table 4.1.1-1b: Void

Table 4.1.1-1c: Void

<End of Changes>