3GPP TSG-RAN WG4 Meeting # 99-e	R4-2110336

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.174 | CR | 0015 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on maintenance on sharing factor of RLM and link recovery for IAB-MT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-05-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There are misalignments of the Psharing factor in SSB-based and CSI-RS based RLM/BFD/CBD. In SSB-based RLM/BFD/CBD requirements, the current requirements of the conditions for Psharing factor = 1 are not correct as the corresponding part has been updated in TS 38.133. It is proposed to align the Psharing factor in SSB-based RLM/BFD/CBD requirements as the way in CSI-RS based RLM/BFD/CBD. There is gap related requirements which should be removed based on previous agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define Psharing factor in SSB-based RLM/BFD/CBD in the way in CSI-based requirements. Remove gap related requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There are misalignments in the requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.3.1.2.2 ,12.3.2.2.2, 12.3.2.5.2 and 12.3.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

12.3.1.2.2	Minimum requirement

IAB-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_out_SSB [ms] period becomes worse than the threshold Qout_SSB within TEvaluate_out_SSB [ms] evaluation period.

IAB-MT shall be able to evaluate whether the downlink radio link quality on the configured RLM-RS resource estimated over the last TEvaluate_in_SSB [ms] period becomes better than the threshold Qin_SSB within TEvaluate_in_SSB [ms] evaluation period.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 12.3.1.2.2-1 for FR1 with scaling factor K1 = 5.

TEvaluate_out_SSB and TEvaluate_in_SSB are defined in Table 12.3.1.2.2-2 for FR2 with scaling factor N=8 and K2 = 3.

For FR1,

-	P = 1

For FR2,

-	P=1, when the RLM-RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when the RLM-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when RLM-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

<End of Change 1>

<Start of Change 2>

12.3.2.2.2	Minimum requirement

IAB-MT shall be able to evaluate whether the downlink radio link quality on the configured SSB resource in set  estimated over the last TEvaluate_BFD_SSB ms period becomes worse than the threshold Qout_LR_SSB within TEvaluate_BFD_SSB ms period.

The value of TEvaluate_BFD_SSB is defined in Table 12.3.2.2.2-1 for FR1.

The value of TEvaluate_BFD_SSB is defined in Table 12.3.2.2.2-2 for FR2 with scaling factor N= 8.

For FR1,

-	P=1.

For FR2,

-	P=1, when the BFD-RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when the BFD-RS resource is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when the BFD-RS resource is fully overlapped with SMTC period (TSSB = TSMTCperiod).

<End of Change 2>

<Start of Change 3>

12.3.2.5.2	Minimum requirement

Upon request the IAB-MT shall be able to evaluate whether the L1-RSRP measured on the configured SSB resource in set  estimated over the last TEvaluate_CBD_SSB ms period becomes better than the threshold Qin_LR provided SSB_RP and SSB Ês/Iot are according to Annex Table in B.2.4.1 [6] for a corresponding band.

The IAB-MT shall monitor the configured SSB resources using the evaluation period in table 12.3.2.5.2-1 and 12.3.2.5.2-2 which is applicable to the non-DRX mode only.

The value of TEvaluate_CBD_SSB is defined in Table 12.3.2.5.2-1 for FR1.

The value of TEvaluate_CBD_SSB is defined in Table 12.3.2.5.2-2 for FR2 with scaling factor N=8.

Where,

For FR1,,

-	P = 1.

For FR2,

-	P=1, when the candidate beam detection RS resource is not overlapped with SMTC occasion.

-	P=11-TSSBTSMTCperiod, when candidate beam detection RS is partially overlapped with SMTC occasion (TSSB < TSMTCperiod).

-	P = 3, when candidate beam detection RS is fully overlapped with SMTC period (TSSB = TSMTCperiod).

<End of Change 3>

<Start of Change 4>

12.3.2.6.2	Minimum requirement

Upon request the IAB-MT shall be able to evaluate whether the L1-RSRP measured on the configured CSI-RS resource in set  estimated over the last TEvaluate_CBD_CSI-RS [ms] period becomes better than the threshold Qin_LR within TEvaluate_CBD_CSI-RS [ms] period provided CSI-RS Ês/Iot is according to Annex Table in B.2.4.2 [6] for a corresponding band.

The IAB-MT shall monitor the configured CSI-RS resources using the evaluation period in table 12.3.2.6.2-1 and 12.3.2.6.2-2 which is applicable to the non-DRX mode only.

The value of TEvaluate_CBD_CSI-RS is defined in Table 12.3.2.6.2-1 for FR1.

The value of TEvaluate_CBD_CSI-RS is defined in Table 12.3.2.6.2-2 for FR2 with scaling factor N=8.

For FR1,

-	P = 1.

For FR2,

-	P = 1, when candidate beam detection RS is not overlapped with SMTC occasion.

-	P=11-TCSI-RSTSMTCperiod, when candidate beam detection RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P = 3, when candidate beam detection RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

If the IAB-MT is not capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤2 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

If the IAB-MT is capable of 4 SMTC configurations per frequency [15], and is provided with higher layer signaling of smtcj, where 1≤j≤4 [15], then TSMTCperiod follows smtcjmax where jmax is the maximum value of all j for which smtcj has been configured.

Longer evaluation period would be expected if the CSI-RS is on the same OFDM symbols with RLM, BFD, BM-RS, or other CBD-RS, according to the measurement restrictions defined in clause 12.3.2.6.3.

The values of MCBD used in Table 12.3.2.6.2-1 and Table 12.3.2.6.2-2 are defined as

-	MCBD = 3, if the CSI-RS resource configured in the set  is transmitted with Density = 3.

<End of Change 4>