3GPP TSG-RAN WG4 Meeting # 99-e 	R4-2110000

Electronic Meeting,19-27th May, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.809 | CR | 0003 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for update on TR38.809 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB-Core |  |  |  |  |  | Date: |  |  | 2021-5-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To include CR endorsed in RAN4#98bis-e to TR38.809. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The content, which in below draft CR endorsed in RAN4#98bis-e,is introduced in TR38.809: R4-2106041 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The further agreement will not be updated to TR . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5.2.2, 9.6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Start of the changes >

7.5.2.2	Error Vector Magnitude

EVM performance is the SNR performance of the transmitted signal. In order to have the same link performance, IAB-MT output signal quality should have the same performance as UE then guarantee the link quality. UE requirements are reused by IAB-MT EVM requirements. The difference is that BPSK EVM requirement is removed considering BPSK modulation is not likely to be used by the backhaul link. As IAB-MT is part of IAB node which is a network node, the principle of EVM frame structure for IAB-MT measurement can reuse BS EVM frame structure. It is agreed that only CP-OFDM signal is tested for IAB-MT and only PUSCH channel is measured for EVM requirement. Both BS and UE EVM measurement procecure can be used by IAB-MT for the Tx uplink signal EVM requirement.

< Next change >

9.6.2.2	Error Vector Magnitude

IAB-MT OTA EVM requirement analysis is the same as the conducted requirement in 7.5.2.2. IAB-MT type1-O EVM requirements should be the same with conducted requirements. IAB-MT type2-O EVM requirement reuses UE FR2 EVM requirements with the exception that BPSK requirement is removed. The IAB-MT frequency must be within a certain error limit relative to of the parent node's center frequency. It is agreed that only CP-OFDM signal is tested for IAB-MT and only PUSCH channel is measured for EVM requirement.Both BS and UE EVM measurement procecure can be used by IAB-MT for the Tx uplink signal EVM requirement.

< End of the changes >