# 3GPP TSG-RAN WG4 Meeting # 99-e	R4-2108420

# Electronic Meeting, May. 19-27, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1935 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduce the SCell beam failure recovery without the dedicated PUCCH resource in R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-Core |  |  |  |  |  | Date: |  |  | 2021-5-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to the agreement in RAN4 #98 shown as follows Agreement: - For Scenario 1 it verifies UE transmits SR with random access procedure. The test procedure also includes the reported BFR MAC CE contains a beam associated with the candidate beam set q1. - For Scenario 2 it verifies UE transmits PUCCH with an LRR. The test procedure also includes the reported BFR MAC CE contains a beam associated with the candidate beam set q1.. - It has been agreed that the Scenario 1 and 2 should be defined in performance part. Thus, the core requirement should be revised to capture the secnario 1. |
| Summary of change: |  | - Add the scenario 1, i.e., “UE transmits SR with random access procedure”, in clause 8.5.9 of TS 38.133. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The core requirement is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2109644 |  |  |  |  |  |  |  |  |

>>>>>Start of Change 1<<<<<

8.5.9	Requirements for Beam Failure Recovery in SCell

8.5.9.1	Introduction

For the UE provided with a configuration of PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 in TS 38.213 [3], if beam recovery procedure is triggered for any of SCells, the UE shall transmit SR for UL resouce, followed by MAC CE providing one index for at least one corresponding SCell with radio link quality is worse than Qout,LR, and the index qnew for a periodic CSI-RS configuration or for a SSB provided by higher layer, as described in clause 5.17 of TS38.321 [7], if any, for a corresponding SCell.

For the UE not provided with a configuration of PUCCH transmission with a link recovery request (LRR) as described in clause 9.2.4 in TS 38.213 [3], if beam recovery procedure is triggered for any of SCells, the UE shall transmit preamble for UL-SCH resource application, followed by MAC CE providing one index for at least one corresponding SCell with radio link quality is worse than Qout,LR, and the index qnew for a periodic CSI-RS configuration or for a SSB provided by higher layer, as described in clause 5.17 of TS38.321 [7], if any, for a corresponding SCell.

8.5.9.2	Requirement

Provided that UE is configured by schedulingRequestIDForBFR a configuration for LRR in a PUCCH transmission, after BFR is triggered on any of SCells as described in clause 5.17 of TS38.321 [7], UE shall be capable of transmit PUCCH with a LRR within a period of T, where

-	T = T1 x Ceil((T2+D) /T1) in which T1, T2 and D are defined as

-	T1 is equal to the periodicity of PUCCH configured with schedulingRequestIDForBFR.

-	T2 = TEvaluate_CBD is the evaluation period specified in clause 8.5.5 or 8.5.6 for SSB or CSI-RS based candidate beam detection, that is TEvaluate_CBD_SSB or TEvaluate_CBD_CSI-RS, depending on the applicable reference signal configured for candidate beam detection.

-	D = 2ms is the UE Processing time.

>>>>>End of Change 1<<<<<