3GPP TSG RAN Meeting #92-e	RP-211149

# Electronic Meeting, June 14 - 18, 2021

Agenda item:	9.11

Source:	vivo

Title:	New WID on legacy RRM requirements improvements

Document for:	Approval

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Information about Work Items can be found at http://www.3gpp.org/Work-Items

Title: Incomplete RRM requirements enhancement

Acronym:

Unique identifier:

NOTE:	For new WIs/SIs leave the Unique identifier empty or you can make a proposal for an Acronym.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  |  |  |
| No | X |  | X | X | X |
| Don't know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, we define them as work tasks. If you are in doubt, please contact MCC.

2.2	Parent and child Work Items

{For a Feature: list here the children Building Blocks (optional) and Work Tasks (optional)}

{For a Building Block: list here the parent Feature (mandatory) and children Work Tasks (optional)}

{For a Work Task: list here the parent Building Block (mandatory)}

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
|  |  | {mandatory text: "parent WID" or "child WID"} |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one WID. Therefore the table above should just include the feature WI Unique ID and title and Nature of relationship is "parent WID".

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure, such as preceding SI or a preceding WI (e.g. if you further enhance a topic).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

NOTE:	Also related or dependent WIs in other TSGs should be indicated.

3	Justification

Both FR1+FR2 NR-DC and FR1+FR1 NR-DC are supported in R16. However, RRM requirements for FR1+FR1 NR-DC are incomplete in Rel-16. For example, PSCell addition delay requirements for FR1+FR1 NR-DC are missing in Rel-16. The missing requirements are supposed to be the baseline to specify HO with PSCell requirements for this scenario in Rel-17 further RRM enhancement WI. Furthermore, complete RRM requirements in Rel-16 are expected for FR1+FR1 NR-DC as there is interest in this deployment from operators.

There is another issue on RRM requirements for UE capability ‘intraFreq-needForGap-r16’ and ‘interFreq-needForGap-r16’.  The UE capability was introduced in Rel-16 as follows.

The IE NeedForGapsInfoNR indicates whether measurement gap is required for the UE to perform SSB based measurements on an NR target band while NR-DC or NE-DC is not configured. NeedForGapsInfoNR information element -- ASN1START -- TAG-NeedForGapsInfoNR-START NeedForGapsInfoNR-r16 ::= SEQUENCE { intraFreq-needForGap-r16 NeedForGapsIntraFreqlist-r16, interFreq-needForGap-r16 NeedForGapsBandlistNR-r16 }

The existing RRM requirements for the UE capabilities are only specified for intra-frequency SSB based measurements without gaps.

However, there are other RRM requirements that need to be studied and specified for the UE capabilities in order to have complete requirements for the feature which are to ensure the feature can really work in field for all possible UE implementations. In addition, there may be impact to the work related to NCSG of R17 MG enhancement WI.

4	Objective

4.1	Objective of Core part WI

The objective of this work item is to specify missed NR RRM requirements for FR1+FR1 NR-DC and for UE capability ‘intraFreq-needForGap-r16’ and ‘interFreq-needForGap-r16’.

- FR1+FR1 NR-DC [RAN4]

  - Specify RRM requirements for FR1+FR1 NR-DC

    - PSCell addition delay requirements

    - CCSF for SSB-based and CSI-RS based L3 measurements performed outside gaps

    - CCSF for SSB-based and CSI-RS-based L3 measurements performed within gaps

  - Requirements are specified in release independent from Rel-16

- UE capability ‘NeedForGap’ [RAN4]

  - RRM requirements for UE capability ‘NeedForGap’ are applied to NR SA only.

  - The measurements related to ‘NeedForGap’ are limited to SSB based measurements only.

  - Study and specify RRM requirements for UE capability ‘NeedForGap’

    - Study whether additional interruption is allowed when UE reporting ‘no gap’.

      - Specify interruption requirements, if interruption is allowed.

    - Study CSSF for measurements with ‘no gap’ in ‘NeedForGap’ reporting, and specify requirements if needed.

    - Study scheduling restriction for measurements with ‘no gap’ in ‘NeedForGap’ reporting, and specify requirements if needed.

    - Study measurement period for measurements with ‘no gap’ in ‘NeedForGap’ reporting, and specify requirements if needed.

  - Decide if requirements are specified in release independent from Rel-16.

4.2	Objective of Performance part WI

  - Specify test cases to verify core requirements introduced in the WI, if needed.

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |
| {Possible values: "TS" or "Internal TR" or "External TR". See Note 1} | {E.g. "22.XXX"} | {Title of the specification (as per TR 21.801 §6.1.1), to be aligned as much as possible with the WI/SI title} | {E.g. "TSG#87"} | {E.g. "TSG#89"} | {e.g.: rapporteur: <FamilyName>, <GivenName>, <Company>, <email address>. See Note 2} |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS38.133 | RRM core requirements | RAN #95 | Core part |
| TS38.133 | Test cases | RAN #97 | Performance part |

NOTE:	If this is a RAN WID including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

Qian Yang

Company: vivo

Email: qian9.yang@vivo.com

7	Work item leadership

RAN WG4

8	Aspects that involve other WGs

{Specify all the other WG(s) to be involved and, if necessary, their task. E.g.: "SA2, SA3, SA5. CT6 for storage, and potentially SA4". If not applicable, indicate "None" or "None identified yet".}

NOTE:	For RAN WIDs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval.}

Supporting IM name

vivo