3GPP TSG-RAN WG1 Meeting #105-e	R1-2106293

e-Meeting, May 10th – 27th, 2021

| CHANGE REQUEST |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38.214 | CR | 0198 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on DL PRS periodicity and muting repetition factor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Ericsson), OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-Core |  |  |  |  |  | Date: |  |  | 2021-05-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For the configuration of the PRS, the product of DL PRS resource periodicity, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the lengthsize of the bitmap of dl-PRS-MutingOption1 cannot exceeds 2^μ×10240 (where μ=0,1,2,3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively). The current specification does not include this limitation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | the correction clarifies the UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Condition for proper configuration of the periodicity and muting parameter for PRS are not well defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---- Unchanged texts omitted ----

#### 5.1.6.5	PRS reception procedure

<Unchanged parts are omitted>

A DL PRS resource set is configured by NR-DL-PRS-ResourceSet, consists of one or more DL PRS resources and it is defined by:

-	nr-DL-PRS-ResourceSetID defines the identity of the DL PRS resource set configuration.

-	dl-PRS-Periodicity-and-ResourceSetSlotOffset defines the DL PRS resource periodicity and takes values TperPRS∈2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively and the slot offset for DL PRS resource set with respect to SFN0 slot 0. All the DL PRS resources within one DL PRS resource set are configured with the same DL PRS resource periodicity. The UE does not expect that the product of DL PRS resource periodicity TperPRS, the higher layer parameter dl-prs-MutingBitRepetitionFactor and the size of the bitmap of dl-PRS-MutingOption1 exceeds 2μ×10240, where μ=0, 1, 2, 3 for dl-PRS-SubcarrierSpacing=15, 30, 60 and 120 kHz respectively.

---- Unchanged texts omitted ----