3GPP TSG-RAN WG1 Meeting #104bis-e	R1- 2104019

# E-meeting, April 12th – 20th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0216 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on MR-DC Uplink Power Control in 38.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Apple), Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_CA_enh-Core |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In EN-DC, RRC parameter tdm-PatternConfig2 was agreed to replace tdm-PatternConfig-r16 as Rel-16 TDM pattern RRC parameter in CR R2-2006349, while tdm-PatternConfig-r16 under RRCConnectionResume-v1610-IEs is now referring to the Rel-15 TDM pattern instead of the Rel-16 one. tdm-PatternConfig2 This field is used for dual UL transmission in EN-DC with LTE FDD PCell and for single UL transmission in EN-DC with LTE FDD/TDD PCell, as specified in TS 38.101-3 [101] and TS 38.213 [88]. The network sets at most one of tdm-PatternConfig and tdm-PatternConfig2 to setup. When this field is configured in EN-DC with LTE TDD PCell, it is not applicable if TDD configuration is sa0 or sa6 in SIB1.. RRCConnectionReconfiguration-v1510-IEs ::= SEQUENCE { nr-Config-r15 CHOICE { release NULL, setup SEQUENCE { endc-ReleaseAndAdd-r15 BOOLEAN, nr-SecondaryCellGroupConfig-r15 OCTET STRING OPTIONAL, -- Need ON p-MaxEUTRA-r15 P-Max OPTIONAL -- Need ON } } OPTIONAL, -- Need ON sk-Counter-r15 INTEGER (0.. 65535) OPTIONAL, -- Need ON nr-RadioBearerConfig1-r15 OCTET STRING OPTIONAL, -- Need ON nr-RadioBearerConfig2-r15 OCTET STRING OPTIONAL, -- Need ON tdm-PatternConfig-r15 TDM-PatternConfig-r15 OPTIONAL, -- Cond FDD-PCell nonCriticalExtension RRCConnectionReconfiguration-v1530-IEs OPTIONAL } RRCConnectionResume-v1610-IEs ::= SEQUENCE { idleModeMeasurementReq-r16 ENUMERATED {true} OPTIONAL, -- Need ON restoreMCG-SCells ENUMERATED {true} OPTIONAL, -- Need ON restoreSCG ENUMERATED {true} OPTIONAL, -- Cond EarlySec sCellToAddModList-r16 SCellToAddModList-r16 OPTIONAL, -- Cond EarlySec sCellToReleaseList-r16 SCellToReleaseListExt-r13 OPTIONAL, -- Need ON sCellGroupToReleaseList-r16 SCellGroupToReleaseList-r15 OPTIONAL, -- Need ON sCellGroupToAddModList-r16 SCellGroupToAddModList-r15 OPTIONAL, -- Cond EarlySec nr-SecondaryCellGroupConfig OCTET STRING OPTIONAL, -- Cond RestoreSCG p-MaxEUTRA-r16 P-Max OPTIONAL, -- Cond SCG p-MaxUE-FR1-r16 P-Max OPTIONAL, -- Cond SCG tdm-PatternConfig-r16 TDM-PatternConfig-r15 OPTIONAL, -- Cond FDD-PCell tdm-PatternConfig2-r16 TDM-PatternConfig-r15 OPTIONAL, -- Need OR nonCriticalExtension SEQUENCE {} OPTIONAL }. tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].. But in the latest version of TS 38.213, the tdm-PatternConfig-r16 is still used for Rel-16 TDM pattern, which is clearly not in line with TS 36.331. If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]). Similar issue can be found in the NE-DC. TS 36.331 has defined another RRC configuration tdm-PatternConfigNEDC for NE-DC, which is independent of the TDM pattern RRC configurations for EN-DC. tdm-PatternConfigNE-DC This field is used when power control or IMD issues require single UL transmission in NE-DC as specified in TS 38.101-3 [101] and TS 38.213 [88].. But in the latest version of TS 38.213, tdm-PatternConfig-r15 has been used for NE-DC, which is misaligned as well. If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 in [13, TS 36.213]). Thirdly, the following highlighted UE capability is incorrectly used in the latest version of TS 38.213. Because the uplinkTxSwitching-OptionSupport is a UE capability for another Rel-16 feature UL Tx switching. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability uplinkTxSwitching-OptionSupport = dualUL in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.. Instead, the right UE capability that should be refered here is tdm-restrictionDualTX-FDD-endc-r16 as defined in the latest version of specification TS 38.306: tdm-restrictionDualTX-FDD-endc-r16 Indicates whether the UE supports TDM restriction to LTE FDD PCell in (NG)EN-DC for dual UL transmission operation when tdm-PatternConfig2-R16 is configured, as specified in TS 36.331 [17]. UE indicates support this feature shall also indicate support of tdm-Pattern.. |
| Summary of change: |  | - Correct the RRC parameter for TDM pattern used in uplink powr control of EN-DC - Correct the RRC parameter for TDM pattern used in uplink powr control of NE-DC - Correct TDM restriction for dual UL transmission UE capability in uplink power control of EN-DC - tdm-PatternConfig-r15 is updated to tdm-PatternConfig - tdm-PatternConfig-r16 is updated to tdm-PatternConfig2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment between the RRC configuration/UE capability reference in the specification for EN-DC and NE-DC uplink power control. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.1, 7.6.1A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Isolated Impact Analysis: This CR has isolated impact on EN-DC/NE-DC with respect to the determination of transmission power procedure. Since it is a straightforward specification correction for EN-DC/NE-DC, it is not expected to impact on gNB/UE implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts are omitted>

## 7.6	Dual connectivity

### 7.6.1	EN-DC

If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power  for transmissions on the MCG by p-MaxEUTRA and a maximum power  for transmissions in FR1 on the SCG by p-NR-FR1.

The UE determines a transmission power for the MCG as described in [13, TS 36.213] using  as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described Clauses 7.1 through 7.5 using  as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described Clauses 7.1 through 7.5.

A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration.

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows.

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig or by tdm-PatternConfig2 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-restrictionDualTX-FDD-endc-r16 in [16, TS 38.306], and is configured with tdm-PatternConfig2, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and

-	if UE transmission(s) in subframe  of the MCG overlap in time with UE transmission(s) in slot  of the SCG in FR1, and

-	if  in any portion of slot  of the SCG,

the UE reduces transmission power in any portion of slot  of the SCG so that  in any portion of slot , where  and  are the linear values of the total UE transmission powers in subframe  of the MCG and in slot  of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot  of the SCG if  would need to be reduced by more than the value provided by XSCALE in order for  in any portion of slot  of the SCG. The UE is required to transmit in slot  of the SCG if  would not need to be reduced by more than the value provided by XSCALE in order for  in all portions of slot .

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE expects to be configured with reference TDD configuration for E-UTRA by tdm-PatternConfig in [13, TS 36.213].

### 7.6.1A	NE-DC

If a UE is configured with an MCG using NR radio access and with a SCG using E-UTRA radio access, the UE is configured a maximum power  for transmissions in FR1 on the MCG by p-NR-FR1 and a maximum power  for transmissions on the SCG by p-MaxEUTRA.

The UE determines transmission power for the MCG in FR1 as described Clauses 7.1 through 7.5 using  as the maximum transmission power for . The UE determines transmission power for the MCG in FR2 as described Clauses 7.1 through 7.5.

If the UE is not provided tdd-UL-DL-ConfigurationCommon for the MCG, the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power.

If at least one symbol of slot  of the MCG that is indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated overlaps with subframe  of the SCG

-	for subframe , the UE determines a transmission power for the SCG as described in [13, TS 36.213] using  as the maximum transmission power

otherwise

-	the UE determines a transmission power for the SCG as described in [13, TS 36.213] without considering  as the maximum transmission power

If a UE is configured with , where  is the linear value of ,  is the linear value of , and  is the linear value of a configured maximum transmission power for NE-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the MCG as follows

-	If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213])

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE does not expect to transmit in a slot on the MCG in FR1 when a corresponding subframe on the SCG is an UL subframe in the reference TDD configuration.

-	If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for NE-DC and

-	if the UE transmission(s) in slot  of the MCG in FR1 overlap in time with UE transmission(s) in subframe  of the SCG, and

-	if  in any portion of slot  of the MCG,

the UE reduces transmission power in any portion of slot  of the MCG so that  in all portions of slot , where  and  are the linear values of the total UE transmission powers in slot  of the MCG in FR1 and in subframe  of the SCG, respectively.

-	If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for NE-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfigNE-DC-r15 in [13, TS 36.213]).

<Unchanged parts are omitted>