3GPP TSG RAN meeting #92e										RP-211267

Electronic Meeting, June 14 – 18, 2020

## Status Report to TSG

Agenda item:			9.7.1.2

| WI / SI Name | Study on supporting NR from 52.6 GHz to 71 GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_ext_to_71GHz |  |  |  |  |
| Unique ID | 860041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210862 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: 06/2022 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 25% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jing Sun (RAN1); Jiwoo Kim (RAN4) |
|  | Company | Qualcomm; Intel Corporation |
|  | Email | jingsun@qti.qualcomm.com; Jiwoo.kim@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

### 2.1	RAN1

#### 2.1.1	Agreements

Agreements for RAN1 #104bis-e

[104b-e-NR-52-71GHz-01] Email discussion/approval on initial access aspects with checkpoints for agreements on Apr-15, Apr-20 – Daewon (Intel)

Agreement:

For the case where SSB location and SCS are explicitly provided to the UE (non-initial access) and SSB does not configure Type-0 PDCCH, support 480 kHz and 960 kHz numerologies for the SSB

- Note: Strive to minimize specification impact due to the new SCS for SSB

Agreement:

- For operation with shared spectrum channel access of NR 52.6 – 71 GHz, support discovery burst (DB) and define the DB same as in Rel-16 37.213 Section 4.0

- FFS: Support discovery burst transmission window (DBTW) at least for SSB with 120 kHz SCS with the following requirements

  - PBCH payload size is no greater than that for FR2

  - Duration of DBTW is no greater than 5 ms

  - Number of PBCH DMRS sequences is the same as for FR2

  - FFS: applicability of DBTW design for 120kHz to SSB with 480kHz and 960kHz SCS

  - Support mechanism to indicate or inform that DBTW is enabled/disabled for both IDLE and CONNECTED mode UEs

    - FFS: how to support UEs performing initial access that do not have any prior information on DBTW.

    - FFS: details of the mechanism for enabling/disabling DBTW considering LBT exempt operation and overlapping licensed/unlicensed bands

    - FFS: details of how to inform UEs of the configuration of DBTW

Agreement:

For SSB with 120kHz SCS for NR 52.6 GHz to 71 GHz,

- 120 kHz SCS: the first symbols of the candidate SS/PBCH blocks have indexes {4, 8,16, 20} + 28×n, where index 0 corresponds to the first symbol of the first slot in a half-frame.

- For carrier frequencies within 52.6 GHz to 71GHz, support at least 𝑛 = 0, 1, 2, 3, 5, 6, 7, 8, 10, 11, 12, 13, 15, 16, 17, 18.

  - Other values of n (if any) are FFS, and support of additional n values are subject to support of DBTW for 120kHz SSB

Agreement:

- PRACH configuration for 480/960 kHz SCS (if agreed)

  - The minimum PRACH configuration period is 10 ms (as in FR2)

  - For RO configuration for PRACH with 480/960kHz SCS,

    - FFS: details of how to configure the 480/960 kHz PRACH ROs using [60 or 120 kHz] reference slot considering at least:

      - location of 480/960 kHz PRACH slot per reference slot

      - location of duration containing 480/960khz PRACH slot pattern within 10ms

      - potential impact to RA-RNTI calculation

[104b-e-NR-52-71GHz-02] Email discussion/approval on PDCCH monitoring enhancements with checkpoints for agreements on Apr-15, Apr-20 – Alex (Lenovo)

Agreement:

Previous agreement is modifed as follows:

Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately

  - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X

  - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation between the start of two consecutive spans

  - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots]

  - Y <= X

  - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary.

  - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots.

  - FFS: Further definition of capabilities

- Alt 3: Use a sliding window of X slots as the baseline to define the new capability.

  - The capability indicates the BD/CCE budget within the sliding window

  - The sliding unit of the sliding window is [1] slot.

  - FFS: Further definition of capabilities

- Specific numbers for X, Y may depend on UE capability and gNB configuration

  - Examples:

    - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

Conclusion:

For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed.

Agreement:

For 120 kHz SCS in 52.6-71 GHz, the BD/CCE budget is the same as that for 120 kHz in FR2.

[104b-e-NR-52-71GHz-03] Email discussion/approval on PUCCH format 0/1/4 enhancements with checkpoints for agreements on Apr-15, Apr-20 – Steve (Ericsson)

Agreement:

- The maximum values for the configured number of RBs, NRB, for enhanced PF0/1/4 are at least:

  - 12 RBs for 120 kHz SCS

  - 3 RBs for 480 kHz SCS

  - 2 RBs for 960 kHz SCS

- FFS: Whether or not the above values need to be revised to support larger values (and any associated signaling impact), e.g., to support lower UE Tx beamforming gain and/or larger UE EIRP and conducted power limits for different UE power classes, different from those in the agreed evaluation assumptions

Agreement:

Down select to one of the following two alternatives for the configuration of the number of RBs, NRB, for enhanced PUCCH formats 0/1/4:

- Alt-1:

  - For enhanced PF0/1

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS

  - For enhanced PF4

    - Support configuration of all integer values in the range [1 .. max(NRB)] for each SCS that fulfill the requirement NRB=2α2∙3α3∙5α5 where α2,α3,α5 is a set of non-negative integers.

- Alt-2:

  - Same as Alt-1, but with coarser granularity, i.e., not all integer values of NRB can be configured

  - FFS: Which values of NRB are supported values in the range [1 .. max(NRB)]

Agreement:

For UCI of enhanced PF4, support pre-DFT blockwise spreading using OCCs of length 2 and 4 only, as in Rel-15/16.

Agreement:

For DMRS of enhanced PF4, a Type-1 low PAPR sequence of length equal to the total number of mapped REs of of the PUCCH resource is used. Cyclic shifts are defined in the same was as Rel-15/16 for PF4 (Alt-1 in agreement from RAN1#104-e).

Agreement:

For UCI of enhanced PF4, support pre-DFT blockwise spreading performed across all allocated RBs (Alt-1 in agreement from RAN1#104-e).

Agreement:

For addressing the FFS from the prior agreement in RAN1#104bis-e on the maximum values for the configured number RBs, send an LS to RAN4 asking for feasible maximum values for UE_EIRP and UE_P for operation in 52.6-71 GHz.

R1-2104061	LS to RAN4 on maximum UE conducted power and maximum UE EIRP for operation in the 52.6 – 71 GHz band

Agreement:

User-multiplexing can be considered but as lower priority compared to maximum isotropic loss for PUCCH as a design criterion.

[104b-e-NR-52-71GHz-04] Email discussion/approval on beam management for new SCSs with checkpoints for agreements on Apr-15, Apr-20 – Youngwoo (InterDigital)

Agreement:

Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH

Agreement:

For timeDurationForQCL, beamSwitchTiming and beamReportTiming,

- Following candidate values of FR2 are reused for 120 kHz:

  - timeDurationForQCL: 14 and 28 symbols

  - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols

  - beamReportTiming: 14, 28 and 56 symbols

- For 480 kHz

  - Support at least the candidate values for 120 kHz scaled by 4x

  - FFS: Support for additional candidate value(s)

- For 960 kHz

  - Support at least the candidate values for 120 kHz scaled by 8x

  - FFS: Support for additional candidate values(s)

- FFS: UE capability signaling details

- Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16).

Agreement:

For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI

- FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

[104b-e-NR-52-71GHz-05] Email discussion/approval on defining maximum bandwidth for new SCSs, timeline related aspects adapted to each of the new numerologies 480kHz and 960kHz and reference signals with checkpoints for agreements on Apr-16, Apr-20 – Huaming (Vivo)

Agreement:

A model-based approach is not used to derive the timelines for single PDSCH/PUSCH and multi-PDSCH/PUSCH scheduling for NR operation in 52.6 GHz to 71 GHz.

Agreement:

Continue study at least the following aspects for potential PTRS enhancement for DFT-s-OFDM for NR operation in 52.6 to 71 GHz

- The need of potential PTRS enhancement

- PTRS pattern with more PTRS groups within one DFT-s-OFDM symbol when a large number of PRBs is scheduled

  - (Ng = 8, Ns = 4, L = 1), (Ng = 16, Ns = 2, L = 1), (Ng = 16, Ns = 4, L = 1),

  - Note: Ng number of PT-RS groups, Ns number of samples per PT-RS group, and PTRS every L number of DFT-s-OFDM symbols

  - Other patterns are not precluded

- Other aspects of PTRS enhancements are not precluded from further study

Agreement:

- It is recommended to strictly follow and evaluate at least based on assumptions which are not optional in previous agreed LLS assumptions for study of potential RS enhancements for NR operation in 52.6 to 71 GHz.

  - Note: evaluation based on optional model/scenario/parameter values are not precluded from being considered for discussion and decisions

- Companies are encouraged to report results (along with previously reported aspects and cubic metric for power boosting aspects) at least for SINR in dB achieving PDSCH/PUSCH BLER of 10% in a numerical and tabular way (e.g. adapted from LLS result report template in SI).

  - Note: other ways of presentation of results (e.g. BLER curve) is not precluded

Agreement:

- In Rel-17, for NR operation in 52.6 – 71 GHz, conclude that increased PTRS frequency density is not supported for CP-OFDM at least for Rel-15 PTRS pattern when the allocated number of RB > 32

- Companies are encouraged to study whether to increase PTRS frequency density for small RB allocations for CP-OFDM for NR operation in 52.6 to 71 GHz with respect to phase noise compensation performance

  - CPE and ICI PN compensation

    - Note: Results for CPE compensation-only are to be reported for reference

  - (K = 0.5, L = 1), (K = 1, L = 1), (K = 2, L = 1),

    - Note: PTRS per K number of PRBs, and PTRS every L number of OFDM symbols

  - Number of RBs: 8, 16, 32

  - Other values of K and number of RBs are not precluded

- Study on other aspects of potential PTRS enhancement (e.g., decreased PTRS frequency density) is not precluded

[104b-e-NR-52-71GHz-06] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on Apr-16, Apr-20 – Seonwook (LGE)

Agreement:

- The maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz.

  - FFS: Further restrictions for 480 kHz to 4

  - FFS: A UE capability to select between 4 and 8 for 480 kHz SCS

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

- The maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8.

  - FFS: Further restrictions for 120 kHz and 480 kHz SCS

  - FFS: A UE capability to select between different values for 120 kHz and 480 kHz SCS

Agreement:

For a DCI that can schedule multiple PDSCHs,

- MCS for the 1st TB: This appears only once in the DCI and applies commonly to the first TB of each PDSCH

- NDI for the 1st TB: This is signaled per PDSCH and applies to the first TB of each PDSCH

- RV for the 1st TB: This is signaled per PDSCH, with 2 bits if only a single PDSCH is scheduled or 1 bit for each PDSCH otherwise and applies to the first TB of each PDSCH

- HARQ process number: This applies to the first scheduled PDSCH and is incremented by 1 for subsequent PDSCHs (with modulo operation, if needed)

- FFS:

  - MCS/NDI/RV for the 2nd TB for each PDSCH, including whether scheduling of the 2nd TB for each PDSCH can be supported or not

  - Details of resource allocation related fields such as VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, and ZP CSI-RS trigger

  - Whether/how to signal CBGFI/CBGTI if CBGFI/CBGTI is supported for multi-PDSCH scheduling

  - Details of fields that are common with multi-PUSCH scheduling, e.g., TDRA, FDRA, priority indicator, including potential enhancements

Agreement:

- For a DCI that can schedule multiple PUSCHs,

  - TDRA: Alt 2 (TDRA table is extended such that each row indicates up to 8 multiple PUSCHs (that can be non-continuous in time-domain). Each PUSCH has a separate SLIV and mapping type. The number of scheduled PUSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.), as per agreement made in RAN1#104-e

    - FFS: signaling details

  - Note: Alt 2 does not preclude continuous resource allocation in time-domain.

- For a DCI that can schedule multiple PDSCHs,

  - TDRA: TDRA table is extended such that each row indicates up to 8 multiple PDSCHs (that can be non-continuous in time-domain). Each PDSCH has a separate SLIV and mapping type. The number of scheduled PDSCHs is implicitly indicated by the number of indicated valid SLIVs in the row of the TDRA table signalled in DCI.

    - FFS: signaling details

  - Note: This does not preclude continuous resource allocation in time-domain.

  - Note: Multi-PDSCH scheduling for the case of 120 kHz SCS is still FFS as per prior agreement. This case can be addressed after this FFS has been decided.

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the following options can be considered,

- Option 1: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table and based on extension of K1 set

- Option 1a: The set of candidate PDSCH reception occasions is determined according to each SLIV of each row in the TDRA table

- Option 2: The set of candidate PDSCH reception occasions is determined according to the last SLIV of each row in the TDRA table

- FFS: Codebook generation details, including how to handle the collision with TDD DL/UL configuration and whether/how to extend K1 set based on K1 and slot offset between last PDSCH and other PDSCHs in a row in the TDRA table

Conclusion:

The following is observed for alternative 1 from prior agreement.

- For Alt 1 (C-DAI/T-DAI is counted per DCI) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Same DAI overhead with Rel-16 single-PDSCH DCI

  - T-DAI in UL DCI:

    - In case of single codebook handling feedback for both single and multi-PDSCH scheduling, same DAI overhead with Rel-16 UL DCI

    - In case of separate sub-codebooks, need additional DAI field (with same bit-width of DAI with Rel-16 UL DCI), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

      - Note that DAI field increment for this case is similar for the case in Rel-15 where CBG is configured

  - HARQ-ACK codebook generation:

    - A separate sub-codebook can be generated when multi-PDSCH DCI is configured for a serving cell, similar to the way as 2nd sub-codebook is defined to handle CBG-based scheduling

      - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

      - FFS: how many sub-codebooks are generated when multi-PDSCH DCI is configured for a serving cell and CBG is configured for the serving cell and/or the other serving cell(s)

    - HARQ-ACK payload size is increased compared to single PDSCH scheduling only, since the number of HARQ-ACK bits corresponding to each DAI of the (sub-)codebook for multi-PDSCH DCI in case of separate sub-codebooks (or for all DL DCIs in case of single codebook) depends on the maximum configured number of PDSCHs for multi-PDSCH DCI across serving cells belonging to the same PUCCH cell group.

    - The number of HARQ-ACK bits for multi-PDSCH DCI in case of separate sub-codebooks, or for all DL DCIs in case of single codebook, does not depend on the number of actually scheduled PDSCHs, rather, it is fixed as the maximum configured number of PDSCHs.

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 2 from prior agreement.

- For Alt 2a (C-DAI/T-DAI is counted per PDSCH with a single codebook) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - C-DAI/T-DAI in DL DCI: Bit-width can be increased (FFS: by how much), in DL DCI not only for multi-PDSCH DCI but also for single-PDSCH DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - T-DAI in UL DCI: Bit-width can be increased (FFS: by how much), in UL DCI for all serving cells including a serving cell not configured with multi-PDSCH DCI

  - C-DAI/T-DAI in DL DCI and T-DAI in UL DCI shall be designed such that at most 3 consecutive DCI missing can be resolved, same as in Rel-15/16 NR.

    - FFS: details on increment of DAI field size

    - FFS: whether/how to handle the case where different DCI formats (e.g., DCI format 1_0 and DCI format 1_1) have different field sizes for C-DAI/T-DAI

  - HARQ-ACK codebook generation:

    - The number of HARQ-ACK bits depends on the number of scheduled PDSCHs.

    - FFS: ordering of the PDSCHs for DAI counting

    - FFS: time domain bundling of HARQ-ACK feedback, as per agreement in RAN1#104-e

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

Conclusion:

The following is observed for alternative 3 from prior agreement.

- For Alt 3 (C-DAI/T-DAI is counted per M scheduled PDSCH(s), where M is configurable) of generating type-2 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs,

  - If M equals to the maximum configured number of PDSCHs, Alt 3 is the same with Alt 1, if the same number of codebooks is assumed.

  - Else if M equals to 1, Alt 3 is the same with Alt 2.

  - Otherwise (i.e., 1<M<the maximum configured number of PDSCHs), Alt 3 is similar to Alt 2, except that

    - The number of HARQ-ACK bits corresponding to each DAI increases by M times.

    - NACK bits may be padded if the number of scheduled PDSCHs is not an integer multiple of M.

    - FFS: details on DAI field size

    - FFS: whether single codebook or separate sub-codebooks is(are) generated when multi-PDSCH DCI is configured for a serving cell

  - In addition, new RRC parameter to configure M needs to be introduced.

  - Note that multi-PDSCH DCI refers to a DL DCI where at least one entry of the TDRA table allows scheduling more than one PDSCH

[104b-e-NR-52-71GHz-07] Email discussion/approval on channel access mechanism with checkpoints for agreements on Apr-16, Apr-20 – Jing (Qualcomm)

Working assumption:

For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT.

Agreement:

- Contention Exempt Short Control Signaling rules can be applicable to the transmission of SS/PBCH.

  - FFS: What are the other DL signals and channels that can be multiplexed with SS/PBCH transmission under Contention Exempt Short Control Signaling rule

  - FFS: Whether this can be applied to all supported SCS or specific SCS.

  - FFS: Extension to discovery burst if it is defined including signals other than SS/PBCH

  - Note: Restriction for short control signalling transmissions apply (10% over any 100ms interval)

- FFS: Other DL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as PDCCH, broadcast PDSCH, PDSCH without user plain data, CSI-RS, PRS, etc

Working assumption:

For energy measurement in 5us observation slot, when performing single measurement, the location of the measurement within the 5us is left for implementation, i.e., anywhere within the 5us.

Agreement:

For LBT for single carrier transmission, continue down selection between

- Alt SC.1. gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth)

- Alt SC.3. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth

For LBT for multi-carrier transmission in intra-band CA, continue down selection between

- Alt CA.1. gNB/UE performs multiple LBT, one for each channel bandwidth separately

- Alt CA.2. gNB/UE performs single LBT over all CCs

- Alt CA.5. Define a unit of LBT bandwidth and gNB/UE performs LBT in all the LBT units (to be transmitted in) in the channel bandwidth in each CC

Agreement:

For a COT with MU-MIMO (SDM) transmission, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed in TDM fashion

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

Within a COT with TDM of beams with beam switching, when independent per-beam LBT sensing at the start of COT is performed for beams used in the COT (Alt 2 or Alt 3 in earlier agreement) is considered, the following alternatives are further considered

- Alt A: The per-beam LBT for different beams is performed one after another in time domain

  - Alt A-1: The node completes one eCCA on one beam, and directly move on to the eCCA on the other beam, with no transmission in the middle

  - Alt A-2: The node completes one eCCA on one beam, start transmission with the beam to occupy the COT, then move on to the eCCA on the other beam

  - Alt A-3: The node performs eCCA of the different beams simultaneous, round robin between different beams

- Alt B: The per-beam LBT for different beams is performed simultaneously in parallel, assuming the node has the capability to simultaneously sense in different beams

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode. Down-select between

- Alt 1. Support cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) gNB indication

- Alt 2. Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

- FFS: Whether the indication of the decision on applying LBT mode or no-LBT  mode is per beam (can be different for different UEs in different beams or can be different for different beam pairs between gNB and the UE) or per cell (can be different for different cells for a UE in carrier aggregation)

- FFS: Whether a gNB and its UE(s) can have different mode

- FFS: Whether L1 signalling can be used for both Alt 1 and Alt 2 for gNB indication

Agreement:

For contention exemption short control signalling based DL transmission of SS/PBCH, further consider if the following signals/channels can be multiplexed with SS/PBCH block transmission.

- RMSI PDCCH and RMSI PDSCH

- Other broadcast PDSCH

- PDSCH without user-plane data

- PDCCH

- CSI-RS

- PRS

- Other signals/channels contained in Discovery Burst (i.e., exemption applies to Discovery Burst)

Note: Total exempted signals/channels should meet the restriction of 10% over any 100ms interval.

FFS: If contention exemption short control signalling based DL transmission is allowed when not multiplexed with SS/PBCH block transmission.

Agreements for RAN1 #105-e

[105-e-NR-52-71GHz-01] Email discussion/approval on initial access aspects with checkpoints for agreements on May-24, May-27 – Daewon (Intel)

Agreement:

For 480kHz/960kHz SSB, select one of the following alternatives:

- ALT 1) First symbols of the candidate SSB have index {X, Y} + 14*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

  - value of X and Y are identical for 480kHz and 960kHz

    - FFS: exact value of X and Y

- ALT 2) First symbols of the candidate SSB have index {4, 8, 16,20} + 28*n, where index 0 corresponds to the first symbol of the first slot in a half-frame

- Values of n for 480kHz and 960kHz for ALT 1 and 2

  - FFS: whether number of values for ‘n’ depend on LBT operation (i.e. LBT vs no-LBT)

  - FFS: exact values of ‘n’ for each SCS

  - Values of ‘n’ for one mode of operation shall be strictly a subset of values for another mode of operation, if two mode of operation exist for number of candidate SSBs

  - FFS: whether values of ‘n’ shall not be all consecutive integer values (i.e. non-candidate SSB slots are positioned every few candidate SSB slots)

Proposal:

In addition to 120kHz, support 480 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints.

- Limited sync raster entry numbers

  - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

- only 480kHz CORESTE#0/Type0-PDCCH SCS supported for 480 kHz SSB SCS.

- SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

Formal objection sustained by: Huawei, MediaTek (would like to discuss at next meeting)

Proposal:

In addition to 120kHz, support both 480 and 960 kHz SSB for initial access with support of CORESET0/Type0-PDCCH configuration in the MIB with following constraints.

- Limited sync raster entry numbers

  - It is assumed that RAN4 supports a channelization design which results in the total number of synchronization raster entries considering both licensed and unlicensed operation in a 52.6 – 71 GHz band no larger than 665 (Note: the total number of synchronization raster entries in FR2 for band n259 + n261 is 602). If the assumption cannot be satisfied, it’s up to RAN4 to decide its applicability to bands in 52.6 – 71 GHz.

- only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS i.e., (480,480) and (960,960).

- SSB time domain candidate resource pattern (within a slot or pair of slots) for 480 and 960kHz SSB are identical

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

Formal objection sustained by: Huawei, MediaTek (object to 960 kHz)

Proposal:

To support ANR and PCI confusion detection for 480/960kHz SCS based SSB, support CORESET#0/Type0-PDCCH configuration in MIB of 480 and 960kHz SSB

- FFS: additional method(s) to enable support to obtain neighbor cell PCI and SIB1 contents related to CGI reporting

- Only 1 CORESTE#0/Type0-PDCCH SCS supported for each SSB SCS, i.e., (480,480) and (960,960).

- Prioritize support SSB-CORESET0 multiplexing pattern 1. Other patterns discussed on a best effort basis.

- Note: Strive to minimize specification impact by reusing tables for CORESET#0 and type0-PDCCH CSS set configuration defined for FR2 in Rel-15, as much as possible

- Note: From UE perspective, ANR detection for 480/960kHz SCS based SSB is not supported if the UE does not support 480/960 SCS for SSB.

- Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Formal objection sustained by: Huawei

Agreement:

For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access)

- Support configuring CORESET#0/Type0-PDCCH for the purpose of ANR/PCI confusion detection by down selecting from the following two alternatives

  - Alt 1) Using dedicated signaling

  - Alt 2) Using configuration in MIB

    - Note: for ANR, when reading the MIB, the cell containing the SSB is known to the UE, as defined in 38.133 specification.

Agreement:

For 480kHz and 960kHz PRACH,

- Down-select among option 1 and 2

  - Option 1) The reference slot duration corresponds to 60 kHz SCS. A PRACH slot index, nslotRA , corresponds to one of the starting 480/960 kHz PRACH slots within the reference slot.

    - FFS: supported values of the starting PRACH slot index nslotRA  within reference slot and whether or not the ROs for a given PRACH configuration can span more than one PRACH slot if gaps between consecutive ROs are supported for LBT and/or beam switching purposes

  - Option 2) Each 120kHz RO corresponds to 4 and 8 candidate RO positions for 480kHz and 960kHz PRACH, respectively. Information about the number and locations of 480/960kHz candidate RO(s) are configured or pre-selected within each 120kHz RO. The reference 120kHz RO is determined by the current PRACH configuration method in Rel-15/16 specification.

- Following alternatives are considered on PRACH density

  - ALT 1) At least the same density (i.e. number of PRACH slots per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher PRACH slot density (number of PRACH slots per reference slot)

  - ALT 2) at least the same RO density (i.e. number of RO per reference slot) as for 120kHz PRACH in FR2 is supported

    - FFS: support for higher RO density

  - An “example” illustration of PRACH slots for 480/960kHz is shown below:

- FFS: whether and how to account for LBT in RO configuration (if needed)

- FFS: whether and how to account for beam switching gap in RO configuration (if needed)

Agreement:

FFS: Support DBTW at least for 120kHz

- FFS whether DBTW will be applicable for 480/960 kHz SSB SCS

  - If DBTW is supported for 480/960kHz SSB:

    - For the case agreed in RAN1 #104bis-e where 480/960 kHz SSB location and SCS are explicitly provided to the UE (non-initial access), indication of DBTW configuration (e.g. enable/disable of DBTW,  NSSBQCL, and DBTW length) are supported by dedicated signaling.

- For 120kHz SSB, support mechanism to distinguish at least the following scenarios:

  - Case 1) (Unlicensed with LBT off) + DBTW disabled

  - Case 2) (Unlicensed with LBT on) + DBTW enabled

  - Case 3) (Unlicensed with LBT on) + DBTW disabled

  - Case 4) (Licensed) + DBTW disabled

  - FFS: Whether/how LBT on/off is indicated in MIB

    - If not indicated in MIB, then FFS whether/how the UE determines different sizes of DCI 1_0 with CRC scrambled by SI-RNTI

  - FFS: whether any case(s) can be combined for DBTW signaling design and how to handle implications to DCI 1_0 size ambiguity if is not distinguished in signaling

  - FFS: whether all above cases need an explicit indication

  - FFS: Whether a single indication can be used for combination of more than one cases

- For 120 kHz SSB, enable/disable of DBTW is indicated by one or more of the following methods:

  - Option 1) signaling in MIB

    - Option 1-1) disabling DBTW is jointly coded with NSSBQCL

    - Option 1-2) indicated by other bit fields in MIB

    - FFS: among options 1-1 and 1-2

  - Option 2) distinct GSCN used by the SSB

  - Option 3) By comparing the value of  NSSBQCL in MIB and DBTW length after UE reads SIB1 or by comparing the value of  NSSBQCL in MIB and default DBTW length of 5 ms before UE reads SIB1.

  - FFS: whether to support option 1, 2, 3, or any combination of the options.

  - Note: enable/disable signaling of DBTW by MIB or GSCN does not preclude other signaling methods

Agreement:

If DBTW is supported,

- Working assumption: MIB signaling to support

  - Alt A) indication of NSSBQCL at least for 120kHz SSB

    - In this case, the total number of values of NSSBQCL to not exceed 4

  - Alt B) Explicit indication of SSB index and/or SSB candidate location

    - FFS on the details of signaling

  - FFS between Alt A, or B, or supporting both

- Supported DBTW lengths

  - Alt 1) 0.5, 1, 2, 3, 4, 5 msec

    - Note: same as Rel-16 FR1 NR-U

  - Alt 2) maximum 5 msec

    - FFS other values

  - FFS between Alt 1 and 2

- Number of candidate positions when DBTW is enabled

  - For 120kHz SSB

    - FFS between 64 or 80

  - If DBTW is additionally supported for 480/960kHz SSB

    - FFS between 64 or 128

[105-e-NR-52-71GHz-02] Email discussion/approval on PUCCH format 0/1/4 enhancements with checkpoints for agreements on May 24, May 27 – Steve (Ericsson)

Agreement:

- For 120 kHz SCS:

  - Support at least Alt-1 for enhanced PF0/1 for both PUCCH resources before and after dedicated PUCCH resource configuration

  - FFS: Whether or not Alt-2 is additionally supported for PF0/1 for either or both of the following:

    - PUCCH resources before dedicated PUCCH resource configuration

    - PUCCH resources after dedicated PUCCH resource configuration

  - FFS: Supported RE mapping scheme(s) amongst {Alt-1, Alt-2} for enhanced PF4 including design details

- Notes:

  - Alt-1 = all REs within each RB are mapped

  - Alt-2 = a subset of REs within each RB are mapped (sub-PRB interlaced mapping)

  - Which RE mapping scheme(s) to support for PF0/1/4 to be concluded in RAN1#106

- Note: No further enhancements on RB shortage issue and frequecy hopping distance issue should be considered for PUCCH resource sets prior to RRC configuration.

[105-e-NR-52-71GHz-03] Email discussion/approval on scheduling particularly w.r.t. multi-PDSCH/PUSCH with a single DCI, HARQ, with checkpoints for agreements on May 25, May 27 – Seonwook (LGE)

Agreement:

- Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling.

- Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI.

- Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI.

Conclusion:

For a DCI that can schedule multiple PUSCHs,

- CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2.

Agreement:

- If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH.

  - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

- If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH.

  - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering)

- The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated.

Agreement:

For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs),

- A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots.

  - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs

  - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH

  - FFS: Details to introduce the gap between PDSCHs or between PUSCHs

Agreement:

For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots.

- The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

- The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

  - FFS: details of further pruning of the set of SLIVs

  - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot

  - FFS impact of time domain bundling, if supported

Agreement:

- At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs,

  - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16.

- FFS:

  - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs

  - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields

Agreement:

If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- At least two sub-codebooks are generated for a PUCCH cell group where

  - The first sub-codebook is for the following cases:

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV

    - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH

  - The second sub-codebook is for the following case:

    - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs

      - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs

      - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases

  - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH

- FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group

- FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI

Agreement:

If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs,

- PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly.

- The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR.

- Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR

  - This shall not impose additional gNB’s scheduling restriction.

- In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on

  - The number of SLIVs associated with the row indexes in TDRA table

    - FFS: details

- FFS: the case with configuration of CBG retransmission

- FFS: the number of sub-codebooks

- FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

[105-e-NR-52-71GHz-04] Email discussion/approval on channel access mechanism with checkpoints for agreements on May 25, May 27 – Jing (Qualcomm)

Agreement:

For energy measurement in 8us deferral period, continue down-selection between the following alternatives

- Alt 1. Two energy measurements are required, with one measurement in the first 3us and one measurement in the last 5us

- Alt 2. One measurement is required

  - FFS where the measurement is located

Note: By implementation, it is possible to support longer than 8us deferral period (Intend to cover Alt 3 as implementation choice for either Alt 1 or Alt 2)

Agreement:

On maximum gap within a COT to allow COT sharing without LBT, down-select or support both of the following two alternatives

- Alt 1. No maximum gap defined. A later transmission can share the COT without LBT with any gap within the maximum COT duration

- Alt 3. Define a maximum gap Y, such that a later transmission can share the COT without LBT only if the later transmission starts within Y from the end of the earlier transmission. If the later transmission starts after Y from the end of the earlier transmission, an one-shot LBT is needed to share the COT

Agreement:

For regions where LBT is not mandated, gNB should indicate to the UE this gNB-UE connection is operating in LBT mode or no-LBT mode

- Support both cell specific (common for all UEs in a cell as part of system information or dedicated RRC signalling or both) and UE specific (can be different for different UEs in a cell as part of UE-specific RRC configuration) gNB indication

Agreement:

- Contention Exempt Short Control Signaling rules apply to the transmission of msg1 for the 4 step RACH and MsgA for the 2-step RACH for all supported SCS.

  - Note restriction for short control signalling transmissions apply (10% over any 100ms intervals)

  - Alt 1: The 10% over any 100ms interval restriction is applicable to all available msg1/msgA resources configured (not limited to the resources actually used) in a cell

  - Alt 2: The 10% over any 100ms interval restriction is applicable to the msg1/msgA transmission from one UE perspective

- FFS: Other UL signals/channels can be transmitted with Contention Exempt Short Control Signaling rule, such as msg3, SRS, PUCCH, PUSCH without user plain data, etc

[105-e-NR-52-71GHz-05] Email discussion/approval focusing on analysis or recommendation to RAN#92e (June) on how to introduce the 52.6-71GHz frequency range with checkpoints for agreements on May 25, May 27 – Alex (Lenovo)

Final LS endorsed in R1-2106277

Conclusion:

Regarding the impact on specifications maintained by RAN1, there are only relatively small differences between potential options

Conclusion:

RAN1 can adapt to other groups' preferences on the notation for 52.6-71 GHz.

Conclusion:

Regardless of if the frequency range 52.6 to 71 GHz is an extension of FR2 or a new FR, the related UE capabilities and their applicability to the frequency range 52.6 to 71 GHz will have to be analyzed on a case by case basis

Conclusion:

From RAN1's perspective, an easy distinction between features, capabilities and other characteristics in the specifications that apply specifically to the following ranges may be useful:

- 24.25-52.6 GHz (NOTE: this is the current definition of FR2)

- 52.6-71 GHz

- 24.25-71 GHz

NOTE: The distinction can be realized with or without a designated frequency range (FRx) definition.

Include above conclusion with the note in the LS to RAN.

Conclusion:

Regardless of if the frequency range 52.6 to 71 GHz is an extension of FR2 or a new FR, the application of any of the UE feature introduced for 52.6-71 GHz to existing FR1/FR2 should be discussed case by case.

NOTE: This conclusion will not be included in the LS to RAN.

#### 2.1.2	Remaining Open issues

RAN1 objectives from WID

- Physical layer aspects including [RAN1]:

  - In addition to 120kHz SCS, specify new SCS, 480kHz and 960kHz, and define maximum bandwidth(s), for operation in this frequency range for data and control channels and reference signals, only NCP supported.

Note: Except for timing line related aspects, a common design framework shall be adopted for 480kHz to 960kHz

  - Time line related aspects adapted to 480kHz and 960kHz, e.g., BWP and beam switching timeing, HARQ timing, UE processing, preparation and computation timelines for PDSCH, PUSCH/SRS and CSI, respectively.

  - Support of up to 64 SSB beams for licensed and unlicensed operation in this frequency range.

  - Supports 120kHz SCS for SSB and 120kHz SCS for initial access related signals/channels in an initial BWP.

    - Study and specify, if needed, additional SCS (240kHz, 480kHz, 960kHz) for SSB, and additional SCS(480kHz, 960kHz) for initial access related signals/channels in initial BWP.

    - Study and specify, if needed, additional SCS (480kHz, 960kHz) for SSB for cases other than initial access.

    - Note: coverage enhancement for SSB is not pursued.

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e

  - Support enhancement for PUCCH format 0/1/4 to increase the number of RBs under PSD limitation in shared spectrum operation.

  - Support enhancements for multi-PDSCH/PUSCH scheduling and HARQ support with a single DCI

Note: coverage enhancement for multi-PDSCH/PUSCH scheduling is not pursued

  - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

  - Specify support for PRACH sequence lengths (i.e. L=139, L=571 and L=1151) and study, if needed, specify support for RO configuration for non-consecutive RACH occasions (RO) in time domain for operation in shared spectrum

  - Evaluate, and if needed, specify the PTRS enhancement for 120kHz SCS, 480kHz SCS and/or 960kHz SCS, as well as DMRS enhancement for 480kHz SCS and/or 960kHz SCS.

- Physical layer procedure(s) including [RAN1]:

  - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz.

    - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified.

    - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access

    - Study, and if needed specify, energy detection threshold enhancement

### 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

RAN2 objectives from WID:

- Radio interface protocol architecture and procedures [RAN2]:

  - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified.

### 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

### 2.4	RAN4

#### 2.4.1	Agreements

Agreements for RAN4 #98bis-e

Agreement (R4-2105410) BS RF TX requirements

Channels, bandwidth, and regulatory

- Agreed to define a band [66-71] GHz, based on which the system parameters discussion can proceed with an aim to harmonize for both licensed and unlicensed bands

  - Agreed Focus unlicensed band work on 57 – 71 GHz

  - Agreed Min channel BWs: 100 MHz for 120 kHz SCS, 200 MHz for 480 kHz SCS, 400 MHz for 960 kHz SCS (subject to review of any licensed block sizes)​

  - Agreed Max channel BWs: 400 MHz 120 kHz SCS, 1600 MHz 480 kHz SCS​

  - Max channel BWs 960 kHz SCS FFS

Agreement (R4-2106113) BS RF TX requirements

General and output power

  - The radiated transmitter characteristics requirements applying to the BS type 2-O should be considered for NR operation in 52.6 – 71 GHz range as the starting point for discussion.

  - The BS output power for NR operation in 52.6 – 71 GHz range is declared by the manufacturer. Additional regional requirements can be added to specification to align with regulatory requirements.

  - Total power dynamic range requirement based on 10*log10(Nrb), similar to current FR2, can be applied to 52.6-71 GHz.

Timing and signal quality

  - Re-use frequency error requirements from current FR2.

  - New EVM window length is defined for new SCS.

  - Modulations up to 64 QAM are supported.

Timing and signal quality

  - For licensed operation, adjust OBUE for 52.6 – 71 GHz. Additional regional requirements can be added to specification to align with regulatory requirements.

  - Re-use OTA occupied bandwidth from current FR2. Further discuss the measurement step sizes in conformance phase for wider channel bandwidths.

Agreement (R4-2106114) BS RF RX requirements

  - The radiated receiver characteristics requirements applying to the BS type 2-O should be considered as the for NR operation in 52.6 – 71 GHz range as starting point for discussion. Final requirement values need further considerations.

  - Sensitivity is declared, FRCs will be discussed when minimum and maximum carrier bandwidths are known.

  - ACS and in-band blocking interferer bandwidth are adjusted taking into account applicable channel bandwidths

  - For receiver intermodulation the interferer levels for general receiver intermodulation for NR operation in 52.6 – 71 GHz range can be derived by applying an offset below the in-band blocking levels.

  - The current methodology to derive ICS value for BS type 2-O can be used as baseline to calculate the wanted and interfering signal levels for NR operation in 52.6 – 71 GHz range, but the value may be adjusted for this higher frequency range.

  - New FRCs should be defined for the larger SCSs with the wider channel bandwidth for NR operation in 52.6 – 71 GHz range than those currently defined in FR2, where the allocated RBs within the new FRCs should be scaled according to the target SCSs and channel bandwidth.

Reply LS to RAN1 on max/min CBW and channelization for NR operation in 52.6 – 71 GHz (R4-2105411)

  - For minimum channel bandwidths,

    - For 120 kHz SCS, 100 MHz

    - For 480 kHz SCS, 400 MHz

    - For 960 kHz SCS, 400 MHz

  - For maximum channel bandwidths,

    - For 120 kHz SCS, 400 MHz

    - For 480 kHz SCS, 1600 MHz

    - For 960 kHz SCS, TBD MHz

RF work plan was approved (R4-2105412)

RRM Agreements for RAN4 #99-e

RRM Agreements on Impact of higher SCS on RRM requirements: (R4-2108354)

  - Define new RRM requirements due to higher data/SSB SCS for at least the following topics:

    - UE transmit timing, Timing advance (TA), Interruptions, Active BWP switching delay, Interruption time

  - Study impact on RRM requirements due to higher SSB SCS for at least the Intra-frequency and Inter-frequency measurement

RRM Agreements scaling factor for RX beam sweeping (R4-2108354)

  - RAN4 to study whether the changes on scaling factor for RX beam sweeping are required for this frequency range support

RRM Agreements Operation in licensed/unlicensed band (R4-2108354)

  - RAN4 to initially focus on RRM requirements for both licensed and unlicensed bands without considering LBT and start working on LBT impact on RRM requirements in unlicensed band based on more conclusions from RAN1.

RF Agreements for RAN4 #99-e

BS TX (R4-2108638)

  - Re-use FR2 EIRP accuracy as baseline, further study next meeting

  - Re-use fractional bandwidth concept from current FR2, with 6% breakpoint FFS

  - Re-use OFF power requirement of -36 dBm/MHz from FR2.

  - Consider transient period together with transmit OFF power, and also together with UE requirement

  - Total power dynamic range requirement is based on 10*log10(Nrb) for all SCS and ChBW.

  - Study CA TAE and MIMO TAE further

  - Consider BS type 2-O EVM requirements, but allow further time to check.

  - Apply FR2 approach for spurious emissions, with necessary adaptations to emissions mask step frequencies.

  - Occupied bandwidth measurement step size postponed to performance part

  - For licensed operation, further consider how to adjust OBUE for 52.6 – 71 GHz taking to account the larger channel bandwidth

  - Further consider EN 303 722 for unwanted emissions in unlicensed operation in Europe where the regulatory requirements are clear

BS RX (R4-2108639)

  - Sensitivity levels

    - Use the same principle for derivation of sensitivity level requirements as for current FR2 (i.e. declaration of sensitivity level),

  - FRCs

    - FRCs for 52.6 GHz – 71 GHz include 100 MHz/120 kHz, 400 MHz/480 kHz, 400 MHz/960 MHz are to be defined

  - Inband blocking

    - In-band blocking level of BS type 2-O can be used as baseline for NR operation in 52.6 – 71 GHz range, but consideration should be placed to ensure alignment between in-band selectivity and ACS.

    - Re-evaluate the interferer signal type, including its SCS, channel bandwidth

  - OOB blocking

    - Blocker level

      - Reuse current existing FR2 OOB blocker level for 52.6-71GHz

  - Frequency definitions

    - For out-of-band blocking RF core requirement define the interferer signal upper frequency limit to 2nd harmonic similar as for FR2

    - For receiver blocking further consider ΔfOBUE and decide if ΔfOOB needs to be aligned.

    - Postpone measurement step size discussion to performance part.

UE System Parameters and Other (R4-2107997)

  - FR definition: RAN4 agrees FR3 is not be introduced in any RAN4 specifications for the frequency range 52.6 – 71 GHz.; LS was sent to RAN (R4-2107879)

  - Max CCBW for 960 kHz SCS: 2000 MHz for both licensed and unlicensed bands while further checking any issue with unlicensed (e.g. 802.11 ad/ay)

  - Operation Scenario: Standalone, CA, or DC operation: RAN4 aims to finish a single band requirement first. CA/DC can be discussed in parallel.

  - Band numbering: Reuse the reserved band numbers in FR2 for 60 GHz band

  - Regulatory for unlicensed band: Consider EN 302 567, EN 303 753, and TR 38.805 (FCC 47 CFR 15.255) for mobile application in unlicensed band.

  - ITS: No need to exclude ITS band for unlicensed band

  - 60 GHz OTA testing: Agree to recommend RAN Plenary the need and urgency of studying the UE OTA test method for 52.6 – 71 GHz. LS was sent to RAN (R4-2107878)

UE TX requirements (R4-2107973)

  - UE power class framework: Power classes will be a package of four parameters – Minimum peak EIRP, EIRP spherical coverage, Maximum TRP, and Maximum EIRP (regulatory defined, captured for reference); FFS whether EIRP PSD limit needs to be included.

  - UE TX power classes for spec: Further discuss which, if any, of the existing power classes in 38.101-2 can be reused for an unlicensed NR band or a new power class is needed. As basis for power class definition, it is beneficial to discuss what are representative antenna array sizes in this frequency range.

  - UE types: Consider handheld, FWA, and vehicular types. Other types not precluded.

  - Maximum UL modulation order: Study on MPR analysis for different modulation schemes and Link level performance evaluation; FFS on optionality of 64 QAM and pending on the outcome of the study

UE RX requirements (chairman report on R4-2107956)

  - The framework of FR2 UE Rx requirements can be reused, i.e., REFSENS, spherical coverage EIS, maximum input level, ACS, and in-band blocking are considered as baseline Rx requirements. The maximum DL modulation order can be further discussed.

Coexistence-related requirements – ACLR and ACS (R4-2107915)

  - WF Rationale for 60 GHz band coexistence simulation: Companies are welcome to further discuss coexistence simulation parameters before providing simulation results while keeping in mind the difference to the assumptions and parameters considered in TR 38.803.   The new simulations results would need to provide technical justifications (e.g., preliminary simulation results) in next RAN4 meeting, showing the impact of the new results on the required ACIR comparing to the current ones in TR 38.803.

  - WF on coexistence simulation parameters in Table 1. A preliminary list of parameters for indoor and dense urban deployment is proposed in Table 1.  Further discussions on parameters are FFS in next RAN4 meeting.  Companies are welcome to provide simulation results based on the simulation assumptions in Table 1 in next RAN4 meeting.

Time-related issues (R4-2107972)

  - For NR operation in the 52.6 – 71 GHz range, the Rx-Tx and Tx-Rx transition time shall reuse the FR2 value of 13792 Tc. (7.015 usec) for 120 kHz SCS; FFS on other SCS

  - GNB Beam switch time (beam direction switch only) : RAN4 tentatively agrees [59 ns] with the understanding that the value can be confirmed once open issues related to BS output power are resolved

  - TX ON-ON and TX ON-OFF transient period: Re-use UE transient time from current FR2 for 120 kHz SCS; FFS on other SCS

LS on UE OTA test method in 52.6 – 71 GHz (R4-2107878) was approved.

LS on FR definition on 52.6 – 71 GHz (R4-2107879) was approved.

LS reply on maximum UE EIRP and conducted power (R4-2107950) was approved.

#### 2.4.2	Remaining Open issues

RAN4 objectives from WID:

- Core specifications for UE, gNB and RRM requirements [RAN4]:

  - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range.

  - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1).

  - Specify RRM/RLM/BM core requirements.

- Specify the following requirements [RAN4]

  - Base station demodulation performance requirements

  - UE demodulation performance requirements

  - Radio Resource Management performance requirements, including RRM/RLM test cases

  - Base station conformance testing

### 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

### 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1-104bis-e

- R1-2102327	Initial access signals and channels for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2102385	Discussion on initial access aspects	OPPO

- R1-2102448	Discussion on initial access aspects for NR for 60GHz	Spreadtrum Communications

- R1-2102514	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

- R1-2102558	Initial access aspects	Nokia, Nokia Shanghai Bell

- R1-2102621	Initial access aspects for up to 71GHz operation	CATT

- R1-2102688	Discussion on initial access of 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2102715	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

- R1-2102772	Further considerations on initial access for additional SCS in Beyond 52.6GHz	FUTUREWEI

- R1-2102788	Initial Access Aspects	Ericsson

- R1-2102977	On initial access aspects for NR from 52.6GHz to 71GHz	Xiaomi

- R1-2102996	Initial access aspects for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2103021	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

- R1-2103096	Discussion on Initial access signals and channels	Apple

- R1-2103157	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2103229	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2103294	Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz	Sony

- R1-2103339	Initial access aspects to support NR above 52.6 GHz	LG Electronics

- R1-2103411	NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2103442	Further Discussion of Initial Access Aspects	AT&T

- R1-2103448	Discussions on initial access aspects	InterDigital, Inc.

- R1-2103472	Initial access aspects	Sharp

- R1-2103487	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

- R1-2103519	Discussion on initial access aspects supporting NR from 52.6 to 71 GHz	NEC

- R1-2103567	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2103691	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

- R1-2103801	Issue Summary for initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2103802	Summary #1 of email discussion on initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2104029	Summary #2 of email discussion on initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2102328	Enhancement on PDCCH monitoring	Huawei, HiSilicon

- R1-2102386	Discussion on PDCCH monitoring	OPPO

- R1-2102449	Discussion on PDCCH monitoring enhancement for NR beyond 52.6 GHz	Spreadtrum Communications

- R1-2102515	Discussions on PDCCH monitoring enhancements for NR operation from 52.6GHz to 71GHz			vivo

- R1-2102559	PDCCH monitoring enhancements	Nokia, Nokia Shanghai Bell

- R1-2102622	PDCCH monitoring enhancements for up to 71GHz operation	CATT

- R1-2102704	PDCCH monitoring enhancement for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2102773	Further considerations on PDCCH monitoring enhancements	FUTUREWEI

- R1-2102789	PDCCH Monitoring Enhancements	Ericsson

- R1-2102809	PDCCH monitoring enhancement for NR from 52.6GHz to 71GHz	Panasonic

- R1-2102978	PDCCH monitoring enhancement for NR 52.6-71GHz	Xiaomi

- R1-2102997	PDCCH monitoring enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2103022	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2103097	Discussion on PDCCH Enhancements for above 52.6 GHz	Apple

- R1-2103158	PDCCH monitoring enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2103230	PDCCH monitoring enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2103295	PDCCH enhancement for NR from 52.6GHz to 71GHz	Sony

- R1-2103340	PDCCH monitoring enhancements to support NR above 52.6 GHz	LG Electronics

- R1-2103412	PDCCH Monitoring Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2103449	Discussions on PDCCH monitoring enhancements	InterDigital, Inc.

- R1-2103488	Discussion on the PDCCH monitoring enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2103512	Discussion on PDCCH monitoring enhancements supporting NR from 52.6GHz to 71 GHz			NEC

- R1-2103568	PDCCH monitoring enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2103689	Feature lead summary for B52.6 GHz PDCCH monitoring enhancements	Moderator (Lenovo)

- R1-2104057	Feature lead summary #2 for [104b-e-NR-52-71GHz-02] Email discussion/approval on PDCCH Monitoring Enhancements	Moderator (Lenovo)

- R1-2102329	Enhancement on PUCCH formats	Huawei, HiSilicon

- R1-2102387	Discussion on enhancements for PUCCH format 0/1/4	OPPO

- R1-2102450	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

- R1-2102516	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

- R1-2102560	Enhancements for PUCCH formats	Nokia, Nokia Shanghai Bell

- R1-2102623	Enhancements for PUCCH formats for up to 71GHz operation	CATT

- R1-2102706	On Enhancements for PUCCH formats 0/1/4	MediaTek Inc.

- R1-2102774	Considerations on PUCCH Channel enhancements	FUTUREWEI

- R1-2102790	PUCCH enhancements	Ericsson

- R1-2102998	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2103023	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2103098	Discussion on Enhancements for PUCCH formats 0/1/4	Apple

- R1-2103159	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2103231	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2103296	Considerations on enhancements for PUCCH formats 0/1/4	Sony

- R1-2103341	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

- R1-2103450	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

- R1-2103489	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2103569	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2103692	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

- R1-2103784	FL Summary for Enhancements for PUCCH formats 0/1/4	Moderator (Ericsson)

- R1-2104001	FL Summary for Enhancements for PUCCH formats 0/1/4	Moderator (Ericsson)

- R1-2102330	Discussion on the beam management procedures for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2102388	Discussion on beam management	OPPO

- R1-2102451	Discussion on beam manangement for above 52.6GHz	Spreadtrum Communications

- R1-2102517	Discussions on beam management for new SCSs for NR operation from 52.6GHz to 71GHz			vivo

- R1-2102561	Beam Management Aspects	Nokia, Nokia Shanghai Bell

- R1-2102624	Beam management for new SCSs for up to 71GHz operation	CATT

- R1-2102705	Beam management discussion for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2102775	Beam management for shared spectrum access in Beyond 52.6GHz	FUTUREWEI

- R1-2102791	Beam Management for New SCSs	Ericsson

- R1-2102979	Discussion on beam management in NR from 52.6 GHz to 71GHz	Xiaomi

- R1-2102999	Beam-management enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2103024	Discussion on Beam management aspects for extending NR up to 71 GHz	Intel Corporation

- R1-2103099	Beam Management for New SCSs	Apple

- R1-2103160	Beam management for new SCS for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2103232	Beam management for new SCSs for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2103297	Beam management enhancement for NR from 52.6GHz to 71GHz	Sony

- R1-2103342	Enhancements for beam management to support NR above 52.6 GHz	LG Electronics

- R1-2103413	On Beam Management for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2103451	Discussions on beam management for new SCSs	InterDigital, Inc.

- R1-2103490	Discussion on the beam management for 52.6 to 71GHz	ZTE, Sanechips

- R1-2103570	Beam based operation for new SCSs for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2103821	FL Summary for Beam Management for new SCSs	Moderator (InterDigital, Inc.)

- R1-2103911	Discussion Summary #1 for Beam Management for new SCSs	Moderator (InterDigital Inc.)

- R1-2103912	Discussion Summary #2 for Beam Management for new SCSs	Moderator (InterDigital Inc.)

- R1-2102331	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2102389	Discussion on PDSCH/PUSCH enhancements	OPPO

- R1-2102452	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

- R1-2102518	Discussions on PDSCH/PUSCH enhancements for NR operation from 52.6GHz to 71GHz			vivo

- R1-2102562	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2102569	Discussions on scheduling enhancements for PDSCH and PUSCH	CAICT

- R1-2102625	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

- R1-2102716	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

- R1-2102776	Considerations on PDSCH/PUSCH enhancements	FUTUREWEI

- R1-2102792	PDSCH-PUSCH Enhancements	Ericsson

- R1-2102980	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

- R1-2103000	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2103012	PT-RS enhancements for NR from 52.6GHz to 71GHz	Mitsubishi Electric RCE

- R1-2103025	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2103100	Discussion on PDSCH/PUSCH enhancements for above 52.6 GHz	Apple

- R1-2103161	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2103233	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2103298	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Sony

- R1-2103343	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

- R1-2103407	Discussion on PDSCH and PUSCH enhancements for 52.6GHz – 71GHZ band	CEWiT

- R1-2103414	PDSCH Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2103452	Discussions on PDSCH/PUSCH enhancements for 52.6 GHz to 71 GHz Band	InterDigital, Inc.

- R1-2103463	Discussion on multi-PDSCH/PUSCH scheduling for NR 52.6-71 GHz	Panasonic Corporation

- R1-2103491	Discussion on the data channel enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2103513	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

- R1-2103571	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2103693	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz	WILUS Inc.

- R1-2103726	PDSCH-PUSCH Enhancement Aspects for NR beyond 52.6 GHz	Charter Communications

- R1-2103818	Discussion summary #1 of [104b-e-NR-52-71GHz-05]	Moderator (vivo)

- R1-2103916 	Discussion summary #2 of [104b-e-NR-52-71GHz-05]	Moderator (vivo)

- R1-2104041	Discussion summary #3 of [104b-e-NR-52-71GHz-05]	Moderator (vivo)

- R1-2103344	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

- R1-2103927	Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

- R1-2104042	Summary #3 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

- R1-2102332	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

- R1-2102390	Discussion on channel access mechanism	OPPO

- R1-2102453	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

- R1-2102519	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

- R1-2102563	Channel access mechanism	Nokia, Nokia Shanghai Bell

- R1-2102570	Discussions on channel access mechanism enhancements for 52.6G-71 GHz	CAICT

- R1-2102626	Channel access mechanism for up to 71GHz operation	CATT

- R1-2102689	On the channel access mechanisms for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2102717	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

- R1-2102777	Further considerations on channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

- R1-2102793	Channel Access Mechanisms	Ericsson

- R1-2102981	Channel access mechanism for NR on 52.6-71 GHz	Xiaomi

- R1-2103001	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2103026	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

- R1-2103101	Channel access mechanisms for unlicensed access above 52.6GHz	Apple

- R1-2103162	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2103234	Channel access mechanism for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2103299	channel access mechanism for 60 GHz unlicensed spectrum	Sony

- R1-2103345	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

- R1-2103415	Channel Access for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2103427	Channel access for multi-beam operation	Panasonic

- R1-2103443	Further Discussion of Channel Access Mechanisms	AT&T

- R1-2103453	Discussion on channel access mechanisms	InterDigital, Inc.

- R1-2103492	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

- R1-2103520	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

- R1-2103572	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2103631	Discussion on multi-beam operation	ITRI

- R1-2103694	Discussion on channel access mechanism for NR from 52.6GHz to 71GHz	WILUS Inc.

- R1-2103727	Channel access mechanisms for above 52.6 GHz	Charter Communications

- R1-2103791	Email discussion summary for channel access mechanism for 52.6GHz-71GHz band, ver01			Moderator (Qualcomm Incorporated)

- R1-2103941	Email discussion summary for channel access mechanism for 52.6GHz-71GHz band, ver02			Moderator (Qualcomm Incorporated)

- R1-2104040	Email discussion summary for channel access mechanism for 52.6GHz-71GHz band, ver03			Moderator (Qualcomm Incorporated)

- R1-2103102	On How to Introduce the 52.6-71GHz Frequency Range	Apple

- R1-2103493	Discussion on the frequency range definition for above 52.6GHz	ZTE, Sanechips

- R1-2102391	Discussion on other aspects	OPPO

- R1-2102520	Discussions on frequency range definition and sync raster for 52.6-71GHz	vivo

- R1-2102564	Extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R1-2102627	Views on Frequency Range for 52.6  to 71 GHz	CATT

- R1-2102794	NR channelization for 52.6 – 71 GHz band	Ericsson

- R1-2103235	Discussion on defining frequency range for NR from 52.6 to 71 GHz	Samsung

- R1-2103346	Link level evaluation results for NR above 52.6 GHz	LG Electronics

- R1-2103393	Link level and system evaluation results for 52-71 GHz	Huawei, HiSilicon

- R1-2103418	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

- R1-2103454	Efficient beam management for NR in 52.6 – 71 GHz	InterDigital, Inc.

- R1-2103573	Views on how to introduce 52.6 - 71 GHz frequency range in NR	NTT DOCOMO, INC.

RAN1-105-e

- R1-2104210	Initial access for Beyond 52.6GHz	FUTUREWEI

- R1-2104273	Initial access signals and channels for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2104348	Discussions on initial access aspects for NR operation from 52.6GHz to 71GHz	vivo

- R1-2104416	Discussion on initial access aspects for NR for 60GHz	Spreadtrum Communications

- R1-2104452	Initial access aspects	Nokia, Nokia Shanghai Bell

- R1-2104460	Initial Access Aspects	Ericsson

- R1-2104507	Initial access aspects for up to 71GHz operation	CATT

- R1-2104659	Initial access aspects for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2104765	Discusson on initial access aspects	OPPO

- R1-2104833	Discussion on the initial access aspects for 52.6 to 71GHz	ZTE, Sanechips

- R1-2104894	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

- R1-2104950	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105007	Discussion on initial access aspects for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105061	Considerations on initial access for NR from 52.6GHz to 71 GHz	Fujitsu

- R1-2105092	Discussion on Initial access signals and channels	Apple

- R1-2105156	Considerations on initial access aspects for NR from 52.6 GHz to 71 GHz	Sony

- R1-2105260	Discussion on initial access aspects supporting NR from 52.6 to 71 GHz	NEC

- R1-2105297	Initial access aspects for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2105370	Discussion on initial access of 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2105419	Initial access aspects to support NR above 52.6 GHz	LG Electronics

- R1-2105495	Initial access aspects for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2105555	On initial access aspects for NR from 52.6GHz to 71 GHz	Xiaomi

- R1-2105581	Discussions on initial access aspects	InterDigital, Inc.

- R1-2105592	NR Initial Access from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2105630	Initial access aspects	Sharp

- R1-2105660	On the importance of inter-operator PCI confusion resolution and ANR support in 52.6 GHz and beyond	AT&T

- R1-2105688	Initial access aspects for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2105786	Further details of initial access for NR above 52.6 GHz	Charter Communications

- R1-2105868	Discussion on initial access aspects for NR beyond 52.6GHz	WILUS Inc.

- R1-2105977	Issue Summary for initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2105978	Summary #1 of email discussion on initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2106082	Summary #2 of email discussion on initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2106311	Summary #3 of email discussion on initial access aspects of NR extension up to 71 GHz	Moderator (Intel Corporation)

- R1-2104211	Enhancements of PUCCH formats for Beyond 52.6GHz	FUTUREWEI

- R1-2104349	Discussions on PUCCH enhancements for NR operation from 52.6GHz to 71GHz	vivo

- R1-2104417	Discussion on enhancements for PUCCH format 0/1/4 for above 52.6GHz	Spreadtrum Communications

- R1-2104453	Enhanced PUCCH formats 0/1/4	Nokia, Nokia Shanghai Bell

- R1-2104461	PUCCH enhancements	Ericsson

- R1-2104508	Enhancements for PUCCH formats for up to 71GHz operation	CATT

- R1-2104660	Enhancements for PUCCH for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2104766	Discussion on enhancements for PUCCH format 0/1/4	OPPO

- R1-2104834	Discussion on the PUCCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2104895	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2104951	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105008	Discussion on PUCCH enhancements for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105093	Discussion on Enhancements for PUCCH formats 0/1/4	Apple

- R1-2105157	Further considerations on enhancements for PUCCH formats 0/1/4	Sony

- R1-2105298	Enhancements for PUCCH format 0/1/4 for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2105369	On Enhancements for PUCCH formats 0/1	MediaTek Inc.

- R1-2105420	Enhancements for PUCCH formats 0/1/4 to support NR above 52.6 GHz	LG Electronics

- R1-2105496	Enhancements to PUCCH formats 0/1/4 for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2105582	Discussions on enhancements for PUCCH formats 0/1/4	InterDigital, Inc.

- R1-2105689	PUCCH format 0/1/4 enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2105869	Discussion on PUCCH enhancement for PUCCH format 0/1/4	WILUS Inc.

- R1-2105929	Enhancement on PUCCH formats	Huawei, HiSilicon

- R1-2105961	FL Summary for Enhancements for PUCCH formats 0/1/4	Moderator (Ericsson)

- R1-2106068	FL Summary for [105-e-NR-52-71GHz-02] Email discussion/approval	Moderator (Ericsson)

- R1-2105422	Summary #1 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

- R1-2104212	Enhancements to support PDSCH/PUSCH for Beyond 52.6GHz	FUTUREWEI

- R1-2104274	PDSCH/PUSCH enhancements for 52-71GHz spectrum	Huawei, HiSilicon

- R1-2104350	Discussions on multi-PDSCH/PUSCH scheduling for NR operation from 52.6GHz to 71GHz			vivo

- R1-2104418	Discussion on PDSCH and PUSCH enhancements for above 52.6GHz	Spreadtrum Communications

- R1-2104454	PDSCH/PUSCH enhancements	Nokia, Nokia Shanghai Bell

- R1-2104462	PDSCH-PUSCH Enhancements	Ericsson

- R1-2104509	PDSCH/PUSCH enhancements for up to 71GHz operation	CATT

- R1-2104661	PDSCH/PUSCH enhancements for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2104767	Discussion on PDSCH/PUSCH enhancements	OPPO

- R1-2104835	Discussion on the PDSCH/PUSCH enhancements for 52.6 to 71GHz	ZTE, Sanechips

- R1-2104896	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- R1-2104952	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105009	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105062	Considerations on multi-PDSCH/PUSCH with a single DCI and HARQ for NR from 52.6GHz to 71 GHz	Fujitsu

- R1-2105094	Discussion on multi-PxSCH and HARQ Codebook Enhancements	Apple

- R1-2105158	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Sony

- R1-2105259	Discussion on PDSCH enhancements supporting NR from 52.6GHz to 71 GHz	NEC

- R1-2105299	PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2105372	HARQ codebook design for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2105396	Discussion on PDSCH/PUSCH enhancements for NR 52.6-71 GHz	Panasonic Corporation

- R1-2105421	PDSCH/PUSCH enhancements to support NR above 52.6 GHz	LG Electronics

- R1-2105497	PDSCH/PUSCH scheduling enhancements for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2105556	PDSCH and PUSCH enhancements for NR 52.6-71GHz	Xiaomi

- R1-2105583	Enhancing PDSCH/PUSCH Scheduling for 52.6 GHz to 71 GHz Band	InterDigital, Inc.

- R1-2105596	PDSCH Considerations for Supporting NR from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2105690	PDSCH/PUSCH enhancements for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2105784	PDSCH-PUSCH Enhancement for NR beyond 52.6 GHz	Charter Communications

- R1-2105870	Discussion on multi-PDSCH/PUSCH scheduling for NR from 52.6GHz to 71GHz	WILUS Inc.

- R1-2106105	Summary #2 of PDSCH/PUSCH enhancements (Scheduling/HARQ)	Moderator (LG Electronics)

- R1-2104213	Channel access for shared spectrum Beyond 52.6 GHz	FUTUREWEI

- R1-2104275	Channel access mechanism for 60 GHz unlicensed operation	Huawei, HiSilicon

- R1-2104351	Discussions on channel access mechanism for NR operation from 52.6GHz to 71 GHz	vivo

- R1-2104419	Discussion on channel access mechanism for above 52.6GHz	Spreadtrum Communications

- R1-2104455	Channel access mechanism	Nokia, Nokia Shanghai Bell

- R1-2104463	Channel Access Mechanisms	Ericsson

- R1-2104510	Channel access mechanism for up to 71GHz operation	CATT

- R1-2104662	Channel access mechanism for NR in 52.6 to 71GHz band	Qualcomm Incorporated

- R1-2104720	Discussions on channel access mechanism enhancements for 52.6G-71 GHz	CAICT

- R1-2104768	Discussion on channel access mechanism	OPPO

- R1-2104836	Discussion on the channel access for 52.6 to 71GHz	ZTE, Sanechips

- R1-2104897	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

- R1-2104953	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105010	Discussion on channel access mechanism for extending NR up to 71 GHz	Intel Corporation

- Withdrawn

- R1-2105063	Considerations on channel access mechanism for NR  from 52.6GHz to 71 GHz	Fujitsu

- R1-2105095	Channel access mechanism	Apple

- R1-2105145	Channel access for multi-beam operation	Panasonic

- R1-2105159	Channel access mechanism for 60 GHz unlicensed spectrum	Sony

- R1-2105261	Discussion on channel access mechanism supporting NR from 52.6 to 71GHz	NEC

- R1-2105300	Channel access mechanism for NR from 52.6 GHz to 71 GHz	Samsung

- R1-2105371	On the channel access mechanisms for 52.6-71 GHz NR operation	MediaTek Inc.

- R1-2105423	Channel access mechanism to support NR above 52.6 GHz	LG Electronics

- R1-2105498	Channel access mechanisms for NR from 52.6 GHz to 71GHz	Lenovo, Motorola Mobility

- R1-2105557	Discussion on channel access mechanism for NR on 52.6-71 GHz	Xiaomi

- R1-2105584	Discussion on channel access mechanisms	InterDigital, Inc.

- R1-2105597	On Channel Access Mechanism for NR from 52.6 GHz to 71 GHz	Convida Wireless

- R1-2105661	On receiver assisted channel access and directional LBT	AT&T

- R1-2105691	Channel access mechanism for NR from 52.6 to 71 GHz	NTT DOCOMO, INC.

- R1-2105755	Discussion on multi-beam operation	ITRI

- R1-2105785	Channel access mechanisms for above 52.6 GHz	Charter Communications

- R1-2105871	Discussion on channel access mechanism for NR from 52.6GHz to 71GHz	WILUS Inc.

- R1-2105986	Contribution summary of channel access mechanism for 52.6GHz-71GHz band, ver01	Moderator (Qualcomm Incorporated)

- R1-2106165	Contribution summary of channel access mechanism for 52.6GHz-71GHz band, ver02	Moderator (Qualcomm Incorporated)

- R1-2106193	Contribution summary of channel access mechanism for 52.6GHz-71GHz band, ver03	Moderator (Qualcomm Incorporated)

- R1-2104352	Discussions on frequency range definition and sync raster for 52.6-71GHz	vivo

- R1-2104456	Extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R1-2104464	Introduction of the 52.6 to 71 GHz frequency range	Ericsson

- R1-2104511	Views on Frequency Range for 52.6 to 71 GHz	CATT

- R1-2104769	Discussion on other aspects	OPPO

- R1-2104837	Discussion on the frequency range definition for above 52.6GHz	ZTE, Sanechips

- R1-2104898	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

- R1-2104954	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

- Withdrawn

- R1-2105011	Discussion on frequency range determination for NR extension up to 71GHz	Intel Corporation

- Withdrawn

- R1-2105096	On How to Introduce the 52.6-71GHz Frequency Range	Apple

- R1-2105301	Discussion on defining frequency range for NR from 52.6 to 71 GHz	Samsung

- R1-2105373	Frequency range naming discussion for 52.6-71 GHz	MediaTek Inc.

- R1-2105424	Discussion on frequency range definition to support NR above 52.6 GHz	LG Electronics

- R1-2105533	Discussion on the frequency range for bands in 52.6-71 GHz	Huawei, HiSilicon

- R1-2105585	Efficient beam management for NR in 52.6 – 71 GHz	InterDigital, Inc.

- R1-2105692	Views on how to introduce 52.6 - 71 GHz frequency range in NR	NTT DOCOMO, INC.

- R1-2105787	Discussion on how to introduce the 52.6-71GHz frequency range	Charter Communications

RAN2#114e

- R2-2106446		RAN2 Datapath impact from >52GHz	Apple Inc

RAN4-98bis-e

- R4-2104439	Simulation results for NR coexistence study indoor deployment at 60GHz	Qualcomm CDMA Technologies

- R4-2104441	Simulation results for NR coexistence study: dense urban micro deployment at 60GHz	Qualcomm CDMA Technologies

- R4-2104456	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2104457	Proposals on BS receiver requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2104534	Discussion on band plan for B52.6GHz	vivo

- R4-2104535	Further discussion on the minimum and maximum channel bandwidths for B52.6G	vivo

- R4-2104536	General discussion on channel raster and sync raster for B52.6G	vivo

- R4-2104537	Draft Reply LS on bandwidth and channelization	vivo

- R4-2104588	On extension of FR2 or new Frequency Range (FR3) to introduce 52.6-71 GHz	Ericsson France S.A.S

- R4-2104594	Discussion on system parameters for NR in 52.6GHz ~ 71GHz	CMCC

- R4-2104595	Discussion on frequency range terminology for 52.6GHz ~ 71GHz	CMCC

- R4-2104683	On BS receiver requirements for 52-71 GHz	Ericsson

- R4-2104731	Discussion on BS TX RF requirements for 52.6-71GHz	CATT

- R4-2104732	Discussion on BS RX RF requirements for 52.6-71GHz	CATT

- R4-2104802	Discussion on the band plan of 52.6-71 GHz	CATT

- R4-2104803	Discussion on the frequency range of 52.6-71 GHz	CATT

- R4-2104804	Discussion on the reply LS on the CBW and channelization	CATT

- R4-2104805	Discussion on the reply LS on beam switching gap	CATT

- R4-2104806	Discussion on the co-exit simulation assumption	CATT

- R4-2104821	Discussion on system parameters for NR operation in 52.6GHz - 71GHz	Apple

- R4-2104835	On frequency range definition between 52.6GHz and 71GHz	Apple

- R4-2104885	Regulatory status for the frequency range 52.6 GHz to 71 GHz and potential band plan	Apple

- R4-2104895	Testability aspects for the 52.6 GHz to 71 GHz frequency range	Apple

- R4-2105090	On UE TX requirements for operations up to 71 GHz	Ericsson

- R4-2105138	Reply LS to RAN1: LS on beam switching gap for 60 GHz band	Ericsson

- R4-2105172	Simulation results for NR coexistence study indoor deployment at 60GHz	Qualcomm CDMA Technologies

- R4-2105173	Simulation results for NR coexistence study: dense urban micro deployment at 60GHz	Qualcomm CDMA Technologies

- R4-2105210	Email discussion summary for [98-bis-e][137] NR_ext_to_71GHz_Part_1_NWM	Moderator (Intel)

- R4-2105211	Email discussion summary for [98-bis-e][138] NR_ext_to_71GHz_Part_2	Moderator (Qualcomm)

- R4-2105328	[98-bis-e][137] NR_ext_to_71GHz_Part_1_NWM - Version 0.0.3 RAN4	Moderator (Intel)

- R4-2105410	Way forward on WF on [137] NR_ext_to_71GHz_Part1	Intel

- R4-2105411	Reply LS to RAN1 on max/min CBW and channelization for NR operation in 52.6 - 71 GHz	Intel

- R4-2105412	NR 52.6 - 71 GHz work plan	Intel

- R4-2105413	Way forward on 60GHz beam switching	Apple

- R4-2105414	LS out on 60GHz beam switching	Apple

- R4-2105415	Way forward on system simulations for 60 GHz	Qualcomm

- R4-2105474	Email discussion summary for [98-bis-e][137] NR_ext_to_71GHz_Part_1_NWM	Moderator (Intel)

- R4-2105475	Email discussion summary for [98-bis-e][138] NR_ext_to_71GHz_Part_2	Moderator (Qualcomm

- R4-2105496	Email discussion summary for [98-bis-e][138] NR_ext_to_71GHz_Part_2	Moderator (Qualcomm)

- R4-2105983	Email discussion summary for [98-bis-e][312] NR_exto71GHz_BSRF	Moderator (Nokia)

- R4-2106113	WF on BS RF TX requirements for 52.6 – 71 GHz	Nokia

- R4-2106114	WF on BS RF RX requirements for 52.6 – 71 GHz	Ericsson

- R4-2106152	Email discussion summary for [98-bis-e][312] NR_exto71GHz_BSRF	Moderator (Nokia)

- R4-2106294	on frequency range definition for NR 52_71GHz	Xiaomi

- R4-2106300	work plan of extending NR to 71GHz	Xiaomi

- R4-2106305	Discussion on minimum and maximum bandwidth for NR 52.6 to 71 GHz	LG Electronics Finland

- R4-2106355	On BS transmitter aspects extending NR to 71 GHz	Ericsson

- R4-2106464	NR 52.6 - 71 GHz work plan	Intel Corporation

- R4-2106465	On spectrum situation and band plan for 52.6 - 71 GHz	Intel Corporation

- R4-2106466	On system parameters for NR 52.6 - 71 GHz	Intel Corporation

- R4-2106467	On frequency range definition on 52.6 - 71 GHz	Intel Corporation

- R4-2106587	Discussion on band plan for 52.6-71GHz	ZTE Corporation

- R4-2106588	Discussion on system parameters for 52.6-71GHz	ZTE Corporation

- R4-2106589	Discussion on BS Tx requirements for 52.6-71GHz	ZTE Corporation

- R4-2106590	Discussion on BS Rx requirements for 52.6-71GHz	ZTE Corporation

- R4-2106591	Discussion on switching delay for 52.6-71GHz	ZTE Corporation

- R4-2106592	Discussion on frequency range definition for 52.6-71GHz	ZTE Corporation

- R4-2106665	Work plan considerations for NR extension to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2106666	On frequency range definition for 52.6 – 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2106691	52.6-71 GHz system parameters	Ericsson

- R4-2106692	On 52.6 to 71 GHz minimum/maximum channel bandwidth, draft LS to RAN1		Ericsson

- R4-2106909	On open loop TPC and transient requirements for NR in the 52.6 - 71 GHz frequency range	Apple

- R4-2107076	bandplan for 52-71GHz frequency range	Ericsson

- R4-2107189	Bandplan for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2107190	System parameters for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2107191	On UE Tx RF aspects for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2107192	On UE Rx RF aspects for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2107210	60 GHz UE switching times	Qualcomm Incorporated

- R4-2107211	60 GHz UE TX EVM	Qualcomm Incorporated

- R4-2107271	On beam switching for 60GHz Band	HiSilicon Technologies Co. Ltd

- R4-2107286	on channel bandwidth for >52.6GHz Band	HiSilicon Technologies Co. Ltd

- R4-2107316	Discussion on frequency range definition for 52-71GHz	Skyworks Solutions Inc.

- R4-2107342	UE Tx requirements for NR extension to 71GHz	Intel Corporation

- R4-2107343	UE Rx requirements for NR extension to 71GHz	Intel Corporation

RAN4-99e

- R4-2108786	Discussions on system simulations and results for 60GHz 	Qualcomm CDMA Technologies

- R4-2109011	Views on UE Array and EIRP level at 60 GHz	Sony

- R4-2109014	Discussion on the system parameters for 52.6-71 GHz	CATT

- R4-2109015	Co-existence simulation assumptions and some simulation results	CATT

- R4-2109114	Discussion on BS TX RF requirements for 52 6-71GHz	CATT

- R4-2109115	Discussion on BS RX RF requirements for 52 6-71GHz	CATT

- R4-2109291	RRM considerations for extension to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2109325	System parameters for NR operation in 52.6GHz - 71GHz	Apple

- R4-2109375	On frequency range definition between 52.6GHz and 71GHz	Apple

- R4-2109383	Proposals on coexistence simulation for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2109384	Proposals on BS transmitter requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2109385	Proposals on BS receiver requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2109433	Overview of the regulatory parameters the 52.6 GHz to 71 GHz frequency range	Apple

- R4-2109445	Testability aspects for the 52.6 GHz to 71 GHz frequency range	Apple

- R4-2109446	On UE RF requirements for NR in the 52.6 - 71 GHz frequency range	Apple

- R4-2109474	60 GHz UE switching times	Qualcomm Incorporated

- R4-2109475	60 GHz channel bandwidths, raster, and carrier aggregation	Qualcomm Incorporated

- R4-2109476	60GHz UE TX emissions and EIRP	Qualcomm Incorporated

- R4-2109479	On system parameters for NR in 52.6GHz ~ 71GHz	CMCC

- R4-2109480	Discussion on RRM impacts for 52.6GHz ~ 71GHz	CMCC

- R4-2109696	Further discussion on band plan for B52.6GHz	vivo

- R4-2109697	Further discussion on channel bandwidths and corresponding spectrum utilization for B52.6GHz	vivo

- R4-2109698	Further discussion on channel raster and sync raster for B52.6GHz	vivo

- R4-2109835	On frequency range definition for 52.6 - 71 GHz	Nokia, Nokia Shanghai Bell

- R4-2109870	On BS RF transmitter requirements for the frequency range 52 to 71 GHz	Ericsson

- R4-2109871	On BS RF receiver requirements for the frequency range 52 to 71 GHz	Ericsson

- R4-2109874	Draft LS to RAN1 on beam switching gap for the frequency range 52 to 71 GHz	Ericsson

- R4-2109944	RRM impacts overview for extending NR operation to 71GHz	vivo

- R4-2109981	On UE TX requirements for operations up to 71 GHz	Ericsson

- R4-2110001	View on Maximum Channel Bandwidth for 960kHz SCS	Samsung

- R4-2110023	on channelization for licensed and un-licensed band	Xiaomi

- R4-2110030	on UE TX requirement and regulation requirement	Xiaomi

- R4-2110171	On system parameters in 60 GHz NR	Intel Corporation

- R4-2110172	On UE TX requirements in 60 GHz NR	Intel Corporation

- R4-2110173	On frequency range definition for 60 GHz NR	Intel Corporation

- R4-2110348	Discussion on RRM impact of Rel-17 NR_ext_to_71GHz	Huawei, HiSilicon

- R4-2110410	UE timing	Ericsson

- R4-2110414	Reply LS to RAN1: LS on beam switching gap for 60 GHz band	Ericsson

- R4-2110483	52.6-71 GHz System Parameters	Ericsson

- R4-2110484	On 52.6 to 71 GHz maximum channel bandwidth for 960 kHz, draft LS to RAN1	Ericsson

- R4-2110600	Discussion on system parameters for 52.6-71GHz	ZTE Corporation

- R4-2110601	Discussion on BS Tx requirements for 60GHz	ZTE Corporation

- R4-2110602	Discussion on BS Rx requirements for 60GHz	ZTE Corporation

- R4-2110603	Discussion on frequency range definition for 52.6-71GHz	ZTE Corporation

- R4-2110604	Discussion on UE power class for 60GHz	ZTE Corporation

- R4-2110605	Discussion on switching delay for 52.6-71GHz	ZTE Corporation

- R4-2110684	Bandplan for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2110685	System parameters for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2110686	On UE Tx RF aspects for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2110687	On UE Rx RF aspects for a NR band in the range 52.6GHz – 71GHz	Nokia, Nokia Shanghai Bell

- R4-2110828	Reply LS of max UE conducted power and max UE EIRP for operation in the 52.6 – 71 GHz band	OPPO

- R4-2110950	NR 52.6 -71 GHz workplan (RRM)	Intel Corporation

- R4-2110951	Discussion on the RRM scope for NR 52.6 – 71 GHz support	Intel Corporation

- R4-2110960	Discussion on maximum conducted and radiated output power	LG Electronics Finland

- R4-2110977	Discussion on maximum conducted and radiated output power	LG Electronics Finland

- R4-2110992	Discussion maximum bandwidth for NR 52.6 to 71 GHz	LG Electronics Finland

- R4-2111057	Further discussion on the FR2-extension vs. FR3 introduction for NR operation in 52.6 - 71 GHz range	Huawei

- R4-2111058	Analysis of the exemplary new band introduction to TS 38.104 specification	Huawei

- R4-2111059	Draft CR to TS 38.104: exemplary implementation of new bands with FR2.1 and FR2.2 frequency ranges	Huawei

- R4-2111060	Draft LS to RAN: RAN4 recommendation for the 52.6 - 71 GHz frequency range designation	Huawei

- R4-2111152	Further consideration on extension of FR2 or new Frequency Range (FR3) to introduce 52.6-71 GHz	Ericsson

- R4-2111170	Channelization aspects for 57-71GHz unlicensed band	MediaTek Inc.

- R4-2111352	on 60GHz UE Tx RF requirements	Huawei, HiSilicon

- R4-2111379	on beam switching for 60GHz Band	Huawei, HiSilicon

- R4-2111510	UE OTA test methods for 52.6 to 71 GHz frequency range	Intel Corporation

- R4-2111517	RRM requirements for NR operation in 57-71GHz	Qualcomm Incorporated

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template