3GPP TSG RAN meeting #92e										RP-211289

Electronic Meeting, June 14-18, 2021

Status Report to TSG

Agenda item:			9.7.2.10

| WI / SI Name | Enhancement of RAN slicing for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_slice-Core |  |  |  |  |
| Unique ID | 911107 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210912 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 30% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Ningyu Chen |
|  | Company | CMCC |
|  | Email | chenningyu@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN2

2.1.1	Agreements

SMBR enforcement in RAN

RAN2#113bis-e

Postponed: contributions on this topic requested for May meeting as it may impact SA2 work

RAN2#114-e

Can consider documenting SMBR enforcement in Stage-2 as conclusion of the slicing WI.

Cell reselection

RAN2#113bis-e

Agreements

1	RAN2 aligns with SA2 assumption that support of slices in a TA is homogenous also for Rel-17 (i.e. all cells within a TA supports the same slice availability). If SA2 decides to support heterogeneous deployments, RAN2 can revisit this.

2	The criteria for determining the cell reselection priority for inter-frequency cell reselection should not be left to UE implementation, but should be defined in the specification (just like cell reselection priorities currently). The details of slice info and how the UE determines its priority list from slice info is FFS.

2b	FFS how to define slice priorities for reselection and how to handle conflicts between different priorities (e.g. broadcast vs. dedicated slice-specific priorities)

5	UE is only configured with either the existing dedicated priority configuration or the slice info in RRC Release.

3	In the case that slice info is also provided to the UE in the RRC Release message while SIB also provides the slice info, UE follows the dedicated slice info from RRC Release while T320-like timer is running and only if it expires that it follows the slice info in the SIB

4	In the case that existing dedicated priority configuration is provided to the UE in the RRC Release message while SIB also provides the slice info, UE follows the dedicated priority configuration while T320 is running as per legacy and only if it expires that it follows the slice info in the SIB

6	 For UE supporting slice based cell reselection, the UE should use slice info in the SIB for cell reselection if both slice info and existing cell reselection priority is broadcast in the SIB.

1: With regard the main solution for prioritisation for slice based cell reselection, the following topics to be the initial focus for discussion: Details of slice availability in terms of Slice grouping and frequency priority information for broadcast and RRC Release message,  usage of “intended slice” (FFS whether we use this term in specification), UE prioritisation of slice when there is more than one intended slice and how UE determines frequency priority for inter-frequency cell reselection based on these.

2: Following topics are only considered after some progress on the main solution for prioritisation for slice based cell reselection: which SIB(s) to carry slice availability, whether an LS to SA3 is needed (if SST/SD is agreed for slice info), whether SIB segmentation/on-demand is required (if new SIB is defined).

3: Other topics that have some support and could be discussed further depending on companies providing more details on the motivation and level of support: slice based reselection for MO, different RSRP/RSRQ thresholds for inter and intra-frequency slice based cell reselection, need for Validity area in RRC Release

RAN2#114-e

1: Frequency priority mapping for each slice (slice -> frequency(ies) -> absolute priority of each of the frequency) is provided to a UE.

- Note: Signaling optimizations are not excluded.

- Note: "slice may also mean "slice group"

1b: Frequency priority mapping for each of the slice (slice -> frequency(ies) -> absolute priority of each of the frequency) is part of the “slice info” agreed to be provided to the UE using both broadcast and dedicated signaling.

2: RAN2 kindly allow one more meeting cycle for understanding the necessity of Slice priority along with the following shortlisted solution directions for Idle mode mobility:

- a)	Option 4): Slice priority first looping over slice-frequency combination

- b)	Option 5): Maximize slice support

- c)	Option 6): Frequency priority of highest priority slice with adjustment based on actually supported slice(s) in best ranked cell, without multiple iterations of cell reselection

- d)	Option 7): Perform legacy cell reselection mechanism based on slice specific frequency priority

3: RAN2 consider a scenario in its work for slice specific cell (re)selection where it is possible that (Suitable) cells on the same frequency belonging to different TAs support different Slice(s).

4: Working assumption: The Best cell principle according to absolute priority reselection criteria specified in clause 5.2.4.5 of TS38.304 needs to be met also for slice specific cell (re)selection.

RACH

RAN2#113bis-e

Agreements

1	RAN2 aims to support both RO partition and preambles partition.

2	scalingFactorBI and powerRampingStepHighPriority can be configured at least in SIB (FFS for dedicated RRC signalling).

3	Network can configure slices with 4-step or 2-step (or both) RA resources.

4	Legacy 2-step RA fallback mechanism is supported.

2: RAN2 will prioritize the discussion for slice specific RACH for IDLE and INACTIVE mode. And CONNECTED mode is down prioritized and can be considered if time allows.

3: Slice specific RACH (including RACH isolation and RACH prioritization) is only applied for CBRA but not for CFRA.

4: To ensure the backward compatibility, it is RAN2’s common understanding that common RACH resource should be configured in initial BWP if the slice specific RACH resource is configured in initial BWP.

6: RAN2 confirms that the issue of prioritization parameter collision with MPS/MCS need to be resolved. There is UE based solution (option 1, fixed rule) or network based solution (option 2, configurable rule) or both. Discussion on pros and cons can be left to next meeting.

5.1: RACH type selection between 2-step slice specific RACH and 4-step slice specific RACH is based on a RSRP threshold.

FFS to introduce a slice specific threshold or reuse the legacy threshold.

FFS UE should first select between slice specific RA and common RA or UE should first select RA type between 2-step RA and 4-step RA

5.2: The table from R2-2104322 can be used for further discussion.

Slice specific RACH is only applicable if there is slice information (e.g., slice group or slice related operator defined access category) available for AS layer when access. FFS on details of slice group.

RAN2#114-e

4: RAN2 confirm for a slice group, separated RO and/or separate preamble can be configured within the existing RACH-ConfigCommon and RACH-ConfigCommonTwoStepRA

5: Same as NR Rel-15 conclusion, RAN2 conclude that there is no RA-RNTI collision between slice specific RACH and legacy RACH in shared RO

6: Same as NR Rel-15 conclusion, RAN2 conclude that the RA-RNTI collision between slice specific RACH and legacy RACH may happen in separate RO.

Working assumption: this can be left to network implementation to resolve it (e.g. network configure RO in different time)

FFS how many slice groups we can have and how they are indicated.

2.1.2	Remaining Open issues

Cell reselection

The following aspects are FFS:

- Content of “Slice Info” – to what extent the information needs to be and should be provided to support the Principle in proposal 5 [91]

- If used, who provides the “Slice priority” (NAS/ AS, UE/ Network)

- Can RAN2 continue to use “intended” slice for initial registration and idle-mode mobility

- How UE in each of the solutions from proposal 2 [91] uses slice info for cell reselection if both slice info and existing cell reselection priority is signaled (in the SIB and/ or dedicated signaling)

RACH

The configuration details RACH partitioning:

- What is the configuration needed for slice-specific RACH?

- Which parameters need to be separated for slices (or slice groups)?

- How does the RACH prioritization work with existing RACH prioritization (e.g. MPS/MCS)?

- What information is needed to help design the "common" Rel-17 RACH prioritization scheme?

2.2	RAN3

2.2.1	Agreements

2.2.2	Remaining Open issues

To be added after RAN3 objectives are approved in RAN plenary.

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

N/A

4.	References

RAN2#113bis-e

- R2-2102696	Slice specific cell reselection	Qualcomm Incorporated

- R2-2102697	Slice specific RACH	Qualcomm Incorporated

- R2-2102761	Considerations on slice based RACH configuration	Beijing Xiaomi Software Tech

- R2-2102762	Considerations on slice based cell reselection	Beijing Xiaomi Software Tech

- R2-2102773	Considerations on contents of slice based reselection	KDDI Corporation

- R2-2102831	slice specific cell reselection	Intel Corporation

- R2-2102832	Considerations of slice based RACH	Intel Corporation

- R2-2102988	Considerations on slice-based cell reselection	Lenovo, Motorola Mobility

- R2-2102989	Considerations on slice-based PRACH configuration	Lenovo, Motorola Mobility

- R2-2103089	Slice based RACH configuration	Samsung

- R2-2103159	Discussion on slice based cell reselection	China Telecommunication

- R2-2103213	Consideration on slice-specific cell reselection	OPPO

- R2-2103214	Consideration on slice-specific RACH	OPPO

- R2-2103239	Discussion on slice based cell reselection	Spreadtrum Communications

- R2-2103240	Consideration on slice based RACH configuration	Spreadtrum Communications

- R2-2103269	Cell (re)selection for RAN slicing	Asia Pacific Telecom co. Ltd, FGI

- R2-2103375	Slice based cell reselection	vivo

- R2-2103376	Slice based RACH configuration	vivo

- R2-2103548	RACH prioritisation for slices	Nokia, Nokia Shanghai Bell

- R2-2103589	Slice based Cell Reselection	Sony Europe B.V.

- R2-2103621	Discussion on slice based cell reselection	LG Electronics UK

- R2-2103646	On solution for RAN slicing enhancement	Ericsson

- R2-2103647	SMBR enforcement in RAN	Ericsson

- R2-2103668	Slice-based cell reselection information	Nokia, Nokia Shanghai Bell

- R2-2103694	Work Plan for RAN Slicing WI	CMCC

- R2-2103695	Discussion on slice based cell reselection	CMCC

- R2-2103696	Discussion on slice based RACH configuration	CMCC

- R2-2103745	Slice-specific system information for cell reselection	Google Inc.

- R2-2103881	Discussion on slice based cell reselection	Apple

- R2-2103882	Discussion on slice based RACH	Apple

- R2-2103961	System information contents for slice-aware cell reselection	Sharp

- R2-2104004	Discussion on slice based cell reselection under network control	Huawei, HiSilicon

- R2-2104005	Discussion on slice based RACH configuration	Huawei, HiSilicon

- R2-2104019	Analysis on slice based RACH configuration	CATT

- R2-2104032	Discussion on slice based Cell Reselection	CATT

- R2-2104063	Discussion on slice-aware cell reselection	ZTE corporation, Sanechips

- R2-2104064	Discussion on slice specific RACH resources and RACH prioritization	ZTE corporation, Sanechips

- R2-2104099	Slice-specific RA procedure	LG Electronics UK

- R2-2104176	Discussion on slice based cell reselection	Samsung Electronics Co., Ltd

- R2-2104321	Summary of [AT113bis-e][251][NR] Slice-specific cell reselection (Intel)	Intel

- R2-2104322	Summary of [AT113bis-e][252][NR] Slice-specific RACH (CMCC)	CMCC

RAN2#114-e

- R2-2104740	Further discussion on slice specific cell reselection	Qualcomm Incorporated

- R2-2104741	Further discussion on slice specific RACH	Qualcomm Incorporated

- R2-2104743	Draft LS to SA2 on slice grouping and slice priority	Qualcomm Incorporated

- R2-2104744	Discussion on SMBR enforcement	Qualcomm Incorporated

- R2-2104782	Considerations on slice based cell reselection	Beijing Xiaomi Software Tech

- R2-2104789	Considerations on slice based RACH configuration	Beijing Xiaomi Software Tech

- R2-2104791	Discussion on slice aware cell reselection	ZTE corporation, Sanechips

- R2-2104792	Slice specific RACH resources and RACH prioritization	ZTE corporation, Sanechips

- R2-2104873	Frequency prioritization for slice specific cell (re)selection	Intel Corporation

- R2-2104874	Further considerations of slice based RACH	Intel Corporation

- R2-2105109	Discussion on slice based cell reselection	Apple

- R2-2105110	Discussion on slice based RACH	Apple

- R2-2105203	Discussion on frequency priority for inter-frequency cell reselection	China Telecommunication

- R2-2105212	Further discussion on slice-based cell reselection	Lenovo, Motorola Mobility

- R2-2105213	Further discussion on slice-based PRACH configuration	Lenovo, Motorola Mobility

- R2-2105239	Discussion on Uplink SMBR enforcement	Nokia, Nokia Shanghai Bell

- R2-2105240	Slice specific cell reselection	Nokia, Nokia Shanghai Bell

- R2-2105331	Discussion on slice-based reselection	vivo

- R2-2105332	Discussion on slice-based RACH configuration	vivo

- R2-2105345	Slice specific RACH configuration	Samsung

- R2-2105438	Discussion on slice based cell reselection	Samsung Electronics Co., Ltd

- R2-2105475	Slice-specific RACH prioritisation	Nokia, Nokia Shanghai Bell

- R2-2105533	Discussion on slice based cell reselection	Spreadtrum Communications

- R2-2105534	Consideration on slice based RACH configuration	Spreadtrum Communications

- R2-2105568	Consideration on slice-specific cell reselection	OPPO

- R2-2105569	Consideration on slice-specific RACH	OPPO

- R2-2105630	Cell (re)selection for RAN slicing	FGI

- R2-2105631	Cell (re)selection for RAN slicing	Asia Pacific Telecom, FGI

- R2-2105697	Slice based Cell Reselection and intended slice	Sony

- R2-2105738	Considerations on contents of slice related cell selection info	KDDI Corporation

- R2-2105880	Discussion on slice aware cell reselection	LG Electronics UK

- R2-2105942	SMBR enforcement in RAN	Ericsson

- R2-2105943	Cell re-selection enhancements for slicing	Ericsson

- R2-2105944	RACH for RAN slicing enhancement	Ericsson

- R2-2106013	Slice-based cell/frequency prioritization	NEC Telecom MODUS Ltd.

- R2-2106014	RAN Slicing remaining RACH issues	NEC Telecom MODUS Ltd.

- R2-2106087	Consideration on slice-based cell reselection	SHARP Corporation

- R2-2106155	Discussion on SMBR enforcement	Huawei, HiSilicon

- R2-2106156	Discussion on slice based cell reselection under network control	Huawei, HiSilicon

- R2-2106157	Discussion on slice based RACH configuration	Huawei, HiSilicon

- R2-2106175	Discussion on Slice-based Cell Reselection	CATT

- R2-2106184	Analysis on slice based RACH configuration	CATT

- R2-2106223	Discussion on SMBR enforcement in RAN	CMCC

- R2-2106224	Discussion on slice based cell reselection	CMCC

- R2-2106225	Discussion on slice based RACH configuration	CMCC

- R2-2106373	UL SMBR enforcement	Samsung

- R2-2106374	UL SMBR enforcement	Samsung

- R2-2106375	Discussion on slice-specific RACH operation	LG electronics

- R2-2106418	SMBR enforcement in RAN	Intel Corporation

- R2-2106501	Summary of [AT114-e][250][Slicing] Usage of slice priorities for cell reselection (Lenovo)	Lenovo

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template