3GPP RAN TSG Meeting #92e		RP-211444

Electronic meeting, 14 – 18th June 2021

Agenda item:	9.6.1

Source:	Apple Inc.

Title:	Regulatory update for the 6GHz frequency range

WI/SI:	FS_6GHz_LTE_NR

Release:	Rel-17

Document for:	Approval

# 1	Introduction

Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations is the RAN level study item, which aims at capturing the latest information and status of the regulatory decisions for the 6GHz frequency range. In this discussion paper we present a text proposal for the corresponding TR 37.890 that captures new regulatory decisions in the ITU region 1 (UK, Saudi Arabia) and 2 (Canada, Brazil).

# 2	Text proposal

-------------------------------------------------- TP BEGIN --------------------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- [1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

- [2] 	RP-172804: “Feasibility Study on 6 GHz for LTE and NR”, Ericsson, Verizon Wireless, Qualcomm Incorporated.

[3]					ITU-R Radio Regulations, Articles, Edition 2016;

- [4]		FCC ONLINE TABLE OF FREQUENCY ALLOCATIONS, 47 C.F.R. § 2.106, December 13, 2017;

- [5]	FCC 17-104, Notice of Inquiry, “Expanding Flexible Use in Mid-Band Spectrum Between 3.7 and 24 GHz”;

- [6]	Comments of IEEE 802, in GN Docket No. 17-183;

- [7]	APPLE INC., BROADCOM LIMITED,,CISCO SYSTEMS, INC., FACEBOOK, INC., GOOGLE LLC, HEWLETT PACKARD ENTERPRISE, INTEL CORPORATION,MEDIATEK INC., MICROSOFT CORPORATION, and QUALCOMM INCORPORATED, in GN Docket No. 17-183;

- [8] 	Reply Comments of the Wireless Internet Service Providers Association, in GN Docket No. 17-183;

- [9]	Comments of Ericsson, in GN Docket No. 17-183;

- [10]	Comments of T-Mobile USA, in GN Docket No. 17-183;

- [11]	Comments of Verizon, in GN Docket No. 17-183;

- [12]	Reply Comments of the Satellite Industry Association, in GN Docket No. 17-183;

- [13]	Reply Comments of the Fixed Wireless Communications Coalition, in GN Docket No. 17-183;

- [14]	Comments of Dynamic Spectrum Alliance, in GN Docket No. 17-183;

- [15]	Comments of the National Spectrum Management Association, in GN Docket No. 17-183;

- [16]	Comments of CTIA, in GN Docket No. 17-183;

- [17]	Reply Comments of Cisco Systems, Inc., in GN Docket No. 17-183;

- [18] 	Reply Comments of WI-FI Alliance, in GN Docket No. 17-183;

- [19]	PART 15 - Radio Frequency Devices, Title 47 of electronic Code of Federal Regulations;

- [20]	The European Table of Frequency Allocations and applications in the frequency range 8.3 kHz and 3000 GHz (ECA Table), October 2017;

- [21]	RSCOM17-53rev1- Mandate to CEPT to study and identify harmonised compatibility and sharing conditions for wireless access systems including radio local area networks in the band 5925-6425 MHz for the provision of wireless broadband services.

- [22]	ETSI TR 103 524 System Reference document (SRdoc), “Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 5 925 MHz to 6 725 MHz”, v1.1.1, October 2018

- [23]	CEPT/ERC/REC 74-01: “Unwanted Emissions in the Spurious Domain”;

- [24]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz, May 2019.

- [25]	FCC Notice of Proposed Rulemaking. FCC 18-147. October 24, 2018

- [26]	ETSI TR 103 612, "IMT cellular networks; Mobile/Fixed Communication Network (MFCN) in the frequency range 6 425 - 7 125 MHz", v1.1.1, December 2019

- [27]	ETSI TR 103 631, "Wireless Access Systems including Radio Local Area Networks (WAS/RLANs) in the band 6 725 MHz to 7 125 MHz", v1.1.1, March 2019

- [28]	ECC Report 302, “Sharing and compatibility studies related to Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) in the frequency band 5925-6425 MHz”

- [29]	CCSA-TC5-WG8-2019-003 Project Proposal on the feasibility study of IMT system using 5925-7125MHz frequency band, http://www.ccsa.org.cn/tc/meeting.php?meeting_id=6243#

- [30]	World Radiocommunication Conference 2019 (WRC-19) Provisional Final Acts, ITU-R https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.13-2019-PDF-E.pdf

- [31] Report and order and further notice of proposed rulemaking, FCC 20-51

- [32] ECC Report 316, “Sharing studies assessing short-term interference from Wireless Access Systems including Radio Local Area Networks (WAS/RLAN) into Fixed Service in the frequency band 5925-6425 MHz”, 21 May 2020

- [33]	CEPT Report 075, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report B: Harmonised technical parameters for WAS/RLANs operating on a coexistence basis with appropriate mitigation techniques and/or operational compatibility/coexistence conditions, operating on the basis of a general authorisation. , November 2020

- [34]	ECC Decision (20)01; “On the harmonised use of the frequency bands 5945 to 6425 MHz for the implementation of Wireless Access Systems including Radio Local Area Networks (WAS/RLAN)”, November 2020

- [35]	ETSI TR 103 524, "System Reference document (SRDoc); Wireless access systems including radio local area networks (WAS/RLANs) in the band 5925 MHz to 6725 MHz"

- [36]	EN 303 687, “"6 GHz RLAN Harmonised Standard for access to radio spectrum", Draft

- [37]	doc 2.1_LS to 3GPP TSG RAN – ENG (18th meeting of the RCC Commission on Spectrum and Satellite Orbits), [to be updated]

- [38]	CEPT Report 073, “to study feasibility and identify harmonised technical conditions for Wireless Access Systems including Radio Local Area Networks in the 5925-6425 MHz band for the provision of wireless broadband services”; Report A: Assessment and study of compatibility and coexistence scenarios for WAS/RLANs in the band 5925-6425 MHz, Approved on 6 March 2020 by ECC.

- [39]	Korea’s Ministry of Science and ICT, "Technical standards for radio equipment for radio stations", URL: https://www.law.go.kr/admRulLsInfoP.do?admRulSeq=2100000196974

- [40]	Ofcom,  "Improving spectrum access for Wi-Fi; Spectrum use in the 5 GHz and 6 GHz bands", July 2020, URL: https://www.ofcom.org.uk/__data/assets/pdf_file/0036/198927/6ghz-statement.pdf

- [41]	Communications & Information Technology Commission, "Spectrum Outlook for Commercial and Innovative Use 2021- 2023", January 2021, URL: https://www.citc.gov.sa/ar/new/publicConsultation/Documents/Spectrum%20Outlook%20for%20Commercial%20and%20Innovative%20(2021-2023).pdf

- [42]	Innovation, Science and Economic Development Canada, "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band", May 2021, URL: https://www.ic.gc.ca/eic/site/smt-gst.nsf/eng/sf11698.html

- [43]	National Telecommunications Agency (ANATEL), "ACT NO. 1306", February 2021, URL: https://sei.anatel.gov.br/sei/modulos/pesquisa/md_pesq_documento_consulta_externa.php?eEP-wqk1skrd8hSlk5Z3rN4EVg9uLJqrLYJw_9INcO7uvjUt3vSOwT_4Z5fukj9yIzPErY4KWH5cpE9W_9hcTZkCG-vLPIdpXyuhgMG-L9M-uBLoSdAAXO0clb3SIt1i

-------------------------------------------------- next section --------------------------------------------------

## 4.1 	ITU Region 1

-------------------------------------------------- text omitted --------------------------------------------------

### 4.1.x 	UK

On 24th July 2020, UK Ofcom has decided to make the lower 6 GHz band (5925-6425 MHz) available for the license exempt RLANs for indoor use with a maximum EIRP of 250mW and outdoor use with a maximum EIRP of 25mW, with the maximum mean EIRP density of 10mW/MHz 12.6mW/MHz in any 1 MHz band [40].

Equipment must not form part of a fixed outdoors installation when operating in 5925- 6425MHz. Aeronautical mobile use is not permitted. The Low Power Indoor apparatus may only be used within a building, onboard an aircraft or in any other enclosed space with attenuation characteristics at least as strong as those of either a building or an aircraft. ‘Onboard aircraft’ means the use of radio links for communications purposes inside an aircraft. Airborne use of the relevant equipment will be permitted within an aircraft only to establish a connection with a station or apparatus within the same aircraft.

### This band will be made available on a licence-exempt (non-protected and non- interference) basis with technical parameters that provide adequate protection for other users. The technical conditions are technology neutral so, as well as Wi-Fi, we expect other similar RLAN technologies, such as licence-exempt technologies based on 3GPP standards, to be able to access the spectrum we are making available.

### 4.1.y 	Saudi Arabia

On 28th January 2021, Saudi Arabia Communications & Information Technology Commission announced that it will allocate the entire 5925 – 7125 MHz band license-exempt for the following reasons [41]:

-	Importance of WLAN use in the Kingdom and substantial amount of Wi-Fi traffic, which was exemplified during the COVID-19 lockdowns, and the emergence of a promising device ecosystem that can be taken advantage of starting from 2021.

-	The substantial amount of licensed TDD mid band spectrum already being made available for IMT and 5G. With the release of the 3800 – 4000 MHz band, a total of 890 MHz will be available in large contiguous channels for exclusive IMT use across 2300 MHz, 2600 MHz and 3400 – 4000 MHz. CITC believes that this bandwidth will be sufficient to cover the mid-band spectrum needs of IMT for the foreseeable future. We note that the situation is different in the EU where less exclusive mid-band spectrum (in particular in TDD configuration) is available for IMT. On the other hand, countries with substantial exclusive mid-band spectrum for IMT (such as South Korea) have decided to release the entire 6 GHz band for license-exempt use.

-	The existing mid-bands for exclusive IMT use have robust ecosystems already as well as superior propagation characteristics. If mobile operators want to access the 6 GHz band, they can do so on a license-exempt basis using NR-U (which 3GPP has defined as band n96).

### Nevertheless, given that segments of the band are considered for a possible IMT identification in Region 1, CITC will consult with relevant stakeholders before finalizing our decision.

## 4.2 	ITU Region 2

### 4.2.1 	USA

-------------------------------------------------- text omitted --------------------------------------------------

### 4.2.x 	Canada

On May 21st 2021, Innovation, Science and Economic Development (ISED) department of Canada published "Decision on the Technical and Policy Framework for Licence-Exempt Use in the 6 GHz Band" [42] with the following key aspects:

-	D1: ISED will allow licence-exempt RLAN use in the 5925-7125 MHz band;

-	D3: Standard-power RLANs under the control of an AFC system will be permitted to operate on a licence-exempt basis in the 5925-6875 MHz frequency range:

-	standard-power devices would be subject to a maximum permitted EIRP of 36 dBm per channel;

-	a maximum permitted power spectral density of 23 dBm/MHz and that standard-power RLAN operation would only be permitted on specific frequency channels, with EIRP dictated by the AFC system;

-	to address coexistence with fixed-satellite services (FSS), ISED sought comments to limit the standard-power RLAN’s maximum EIRP to 125 mW per channel at elevation angles above 30 degrees over the horizon.

-	D4: Low-power indoor-only RLANs will be permitted to operate on a licence-exempt basis across the 5925-7125 MHz band with the use of a contention-based protocol (e.g. listen-before-talk):

-	subject to a maximum permitted EIRP of 30 dBm per channel; and

-	a maximum permitted power spectral density of 5 dBm/MHz.

-	D5: Indoor and outdoor very low-power RLAN devices will be permitted to operate on a licence-exempt basis across the 5925-7125 MHz band with the use of a contention-based protocol (e.g. listen-before- talk):

-	subject to a maximum permitted EIRP of 14 dBm per channel; and

-	a maximum permitted power spectral density of -8 dBm/MHz.

-	D6-D7: ISED will adopt an AFC system that is practical, consistent with the Canadian public interest and harmonized with the U.S. to the maximum extent possible. Canadian AFC rules will include a requirement to protect licensed fixed service systems and radioastronomy sites.

### 4.2.y 	Brazil

Act 1306 lists the technical requirements for Broadband Wireless Access System for Local Area Networks operating 5.925 – 7.125 GHz band with the following key points [43].

For indoor access points and subordinate access points, they must meet the following conditions:

-	Average power is limited to a maximum of 30 dBm EIRP.

-	Average power spectral density is limited to a maximum of 5 dBm / MHz EIRP.

-	The equipment must be used in an indoor environment only.

-	Cannot be battery powered.

For client equipment operating under the control of an indoor access point, they must meet the following conditions:

-	Average power is limited to a maximum of 24 dBm EIRP.

-	Average power spectral density is limited to a maximum of -1 dBm / MHz EIRP.

-	The equipment must be used in an indoor environment only.

For very low power devices, the following conditions must be met:

-	Average power is limited to a maximum of 17 dBm EIRP.

-	Average power spectral density is limited to a maximum of -5 dBm / MHz EIRP.

-	Allowed for both indoor and outdoor environment.

-------------------------------------------------- TP END --------------------------------------------------