3GPP TSG RAN meeting #92e							       RP-21xxxx

Electronic Meeting, June 14 - 18, 2021                                                (revision of RP-211127)

- Source:	China Telecom

- Title:	Revised WID on NR coverage enhancements

- Document for:	Approval

Agenda Item:	9.7.1.5

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: NR coverage enhancements

## Acronym: NR_cov_enh

## Unique identifier: 900061

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: {Rel-17}.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG-approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan.

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  | X |  |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work Items |  |
| --- | --- |
| Unique ID | Title |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860036 | Study on NR coverage enhancements |  |
| 860035 | Study on support of reduced capability NR devices |  |

## 3	Justification

Coverage is one of the key factors that an operator considers when commercializing cellular communication networks due to its direct impact on service quality as well as CAPEX and OPEX. Many countries are making available more spectrum in FR1, such as 3.5GHz, which is typically in higher frequencies than for LTE or 3G. Furthermore, Compared to LTE, NR is designed to operate at much higher frequencies such as 28GHz or 39GHz in FR2. Due to the higher frequencies, it is inevitable that the wireless channel will be subject to higher path-loss making it more challenging to maintain an adequate quality of service that is at least equal to that of legacy RATs. One key mobile application of particular importance is voice service for which a typical subscriber will always expect a ubiquitous coverage wherever she/he is located.

The Rel-17 study item 860036 “Study on NR coverage enhancements” evaluated the baseline performance for both FR1 and FR2. The following channels have been identified as the potential bottleneck channels for FR1:

- 1st priority

  - PUSCH for eMBB (for FDD and TDD with DDDSU, DDDSUDDSUU and DDDDDDDSUU)

  - PUSCH for VoIP (for FDD and TDD with DDDSU, DDDSUDDSUU)

- 2nd priority

  - PRACH format B4

  - PUSCH of Msg.3

  - PUCCH format 1

  - PUCCH format 3 with 11bit

  - PUCCH format 3 with 22bit

  - Broadcast PDCCH (BS with 24dBm/MHz Tx power)

- The following channels have been identified as the potential bottleneck channels for Urban 28 GHz scenario:

  - PUSCH eMBB (DDDSU and DDSU)

  - PUSCH VoIP (DDDSU and DDSU)

  - PUCCH F3 11bits

  - PUCCH F3 22bits

  - PRACH B4

  - PUSCH of Msg3

- The Rel-17 study item 860036 “Study on NR coverage enhancements” studied the enhancements for PUSCH, PUCCH and other channels/signals. The enhancements for PUSCH, PUCCH and Msg3 PUSCH were proposed to be specified in Rel-17 coverage enhancements WI.

- It is noted that techniques for enhancing coverage specified in this WI are also applicable for NTN use cases; however, there is no specific optimization of coverage enhancement techniques for NTN in this work item.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to specify enhancements for PUSCH, PUCCH and Msg3 PUSCH for both FR1 and FR2 as well as TDD and FDD.

The detailed objectives of the work item are as follows:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1, RAN2]

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Specify the following performance requirements [RAN4]

- Base station demodulation performance requirements

- Base station conformance testing

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | NR; Physical channels and modulation | RAN#94 | Core Part |
| 38.212 | NR; Multiplexing and channel coding | RAN#94 | Core Part |
| 38.213 | NR; Physical layer procedures for control | RAN#94 | Core Part |
| 38.214 | NR; Physical layer procedures for data | RAN#94 | Core Part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#95 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#95 | Core Part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | RAN#95 | Core Part |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#95 | Core part |
| 38.101-2 | NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone | RAN#95 | Core part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#97 | Perf. Part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#97 | Perf. part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#97 | Perf. part |

## 6	Work item Rapporteur(s)

Jianchi ZHU

Company: China Telecom

Email: zhujc@chinatelecom.cn

## 7	Work item leadership

Primary: RAN WG1

Secondary: RAN WG2, RAN WG4

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name

China Telecom

Apple

AT&T

BT

CAICT

CATT

CEWIT

Charter Communications

China Unicom

CMCC

Ericsson

Facebook

Fujitsu

Huawei

HiSilicon

Intel

InterDigital

IITH

IITM

KT

Lenovo

MediaTek

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

NTT DOCOMO

OPPO

Orange

Panasonic

Rakuten Mobile

Reliance Jio

Samsung

Sanechips

Sierra Wireless

Sharp

Softbank

Spreadtrum

Tejas Networks

Telecom Italia

Qualcomm

Verizon

Vivo

WILUS

Xiaomi

ZTE