3GPP TSG RAN Meeting #92-e	RP-21xxxx

- Electronic Meeting, June 14 – 18, 2021

Source:	Moderator (Huawei)

Title:	Moderator Summary of [92-e-29-newLTE-UECat-1Rx]

Agenda item:	10.11

Document for:	Discussion

# Introduction

CAICT, China Telecom, China Unicom, CMCC, Guangdong Genius, Honor, Huawei, HiSilicon, OPPO, Samsung, Spreadtrum, vivo and Xiaomi have provided a joint proposal in RP-211493 (revised from RP-211466).

The proposals are:

Proposal 1: Define new LTE UE DL categories 3bis and 4bis with single Rx antenna based on UE category 3 and 4 in TS 36.306 and TS 36.331.

Proposal 2: For LTE Cat 3bis and Cat 4bis, reuse the existing RF, RRM and performance requirements specified for DL 1bis to verify single Rx antenna performance, and add additional SDR tests to verify the peak date rate.

Proposal 3: Task RAN2 and RAN4 to finalize the work for LTE Cat 3bis and Cat 4bis in TEI17

Proposal 4: UE reporting Category 3bis/4bis can report and therefore fall back to Cat 1bis when it roams to other regions if needed

For the initial email discussion companies are asked to provide their comments in Section 2. Companies are encouraged to provide comments with detailed reasoning (not just “support” or “object”) in order to facilitate the next round of discussion.

Please also add the contact person for your company in Section 4 when responding.

Please update the document’s name following this model before uploading:

RP-21xxxx [92-e-29-newLTE-UECat-1Rx] v001-Moderator

RP-21xxxx [92-e-29-newLTE-UECat-1Rx] v002-Moderator-CompanyA

RP-21xxxx [92-e-29-newLTE-UECat-1Rx] v003-CompanyA-CompanyB

https://www.3gpp.org/ftp/tsg_ran/TSG_RAN/TSGR_92e/Inbox/Drafts/92-e-29-newLTE-UECat-1Rx

# Initial email discussion (Mon 08:00h UTC – Tues 11:00h UTC)

Proposal 1: Define new LTE UE DL categories 3bis and 4bis with single Rx antenna based on UE category 3 and 4 in TS 36.306 and TS 36.331.

| Company | Comments and questions on proposal 1 in RP-211493 (e.g. support/no support with detailed motivations/reasons) |
| --- | --- |
| Lenovo | Which UL category should be signaled by UEs supporting the new DL categories 3bis and 4bis? How can the peak rate of DL 100/150 Mbps be achieved w/o CA and MIMO? |
| Apple | We support the proposal to define new LTE UE categories, which can help to ensure consistent user experiences (e.g. peak data rate >50Mbps), when 1Rx UE is roaming between LTE and NR networks. Also, the expected standardization efforts can be limited too. |
| DOCOMO | We are not supportive for introducing new categories which overlaps with Rel-17 NR RedCap from usecase/requirement perspectives. |
| OPPO | We support this proposal. R17 NR Redcap is interesting , but it is too late for the market. |
| Ericsson | A performance reduction is expected when removing an Rx antenna. For IOT/wearable devices using LTE cat1-bis, this was acceptable due to the low amount of generated traffic volume. However, LTE cat3/4-bis modems will create more traffic and will likely end up in low-end (smart)phones, which would degrade LTE network performance. LTE cat3/4-bis devices are of very similar capability as NR RedCap devices. We prefer that the wearables eco system is prospering on NR rather than on LTE, hence 3GPP should continue to work on and promote NR RedCap instead of LTE. |
| Telecom Italia | We are not supportive of the proposal. We completely share the issues raised by Ericsson and we are repeating the discussion we had on RedCap. These kind of devices will cause a severe degradation in the LTE networks. |
| Qualcomm | We have made a similar proposal in RAN#86 but it was not accepted. Since then, we have been planning without assuming Cat3/4-bis introduction. The attractiveness of changing this now is unclear. |
| Spreadtrum | Due to the urgent commercial market demand and the form factor restriction on high-end wearable devices, we agree to introduce new LTE UE DL category configuring single Rx antenna and providing higher date rate than Cat 1bis. Compared with NR RedCap devices, LTE wearable devices with single antenna are more mature and can be more quickly commercialized and meanwhile the LTE network can provide wider coverage than NR in the near future. Considering the current definitions of UE DL Cat 3 and Cat4 in TS 36.306, when single antenna is configured, the maximum number of a DL TB size will be reduced to 75376 as shown in the third column of the table below. The new defined UE DL category can provide a peak data rate of ～75Mbps. We support to introduce a new UE DL category with peak data rate of ～75Mbps. From our perspective, Cat 4bis shall be with high priority, while we are open to define Cat 3bis and Cat 4bis together. UE Category; Maximum number of DL-SCH transport block bits received within a TTI (Note 1); Maximum number of bits of a DL-SCH transport block received within a TTI; Total number of soft channel bits; Maximum number of supported layers for spatial multiplexing in DL. Category 1; 10296; 10296; 250368; 1. Category 2; 51024; 51024; 1237248; 2. Category 3; 102048; 75376; 1237248; 2. Category 4; 150752; 75376; 1827072; 2. |
| Deutsche Telekom | We do not support the definition of such categories for the reasons outlined by Telecom Italia and (partly) Ericsson. Customers will not accept degradation in performance also for devices which might have restricted possibilities in terms of antennas. We also think that instead of asking for relaxation in terms of antennas, companies should invest in good antenna design in order to provide a positive user experience while keeping the system efficiency. |
| Samsung | We support this proposal of new UE category considering the form factor limitation of wearable devices |
| China Telecom | Ok with the proposal. Considering the market demand, 3GPP requirements are needed for the certification of such devices. |
| Intel | The proposed LTE Cat 3/4bis device category overlaps with NR RedCap use cases and may cause market fragmentation. In LTE time the introduction of Cat1bis devices affected the adoption of the LTE Cat M2 type of devices. Potential overlap of device categories from 3GPP perspective is undesirable. Further clarifications from the proponents on possible market fragmentation and how it can be avoided are encouraged. |
| Huawei, HiSilicon | Considering the urgent market need for high-end wearable device and impact to the certification process in China, we support to introduce the new DL UE categories 3bis and 4bis with 1Rx. The corresponding max DL throughput is reduced by half compared to UE Categories 3 and 4. The max UL throughput can be same as UE categories 3 and 4, i.e. UL Category 3. As stated in the contribution, the main motivation to introduce UE DL categories 3bis and 4bis with 1Rx is due to the form factor restriction, but the modem inside is still UE Category 3 and 4. The new LTE UE DL categories 3bis and 4bis is immediate urgent market need, but RedCap specification is still under discussion, it cannot meet the current market need. Investigation on benefits to reuse 2Rx was conducted for wearable device by vendors, unfortunately bad performance was observed. The impact on legacy network can be made ZERO by the use of LTE UE category fallback, something which is normal for LTE but doesn't exist in NR. So operators and network vendors that do not like to have this type of devices in their network are totally “safe”. On the other hand we see 14 companies (including operators, network vendors, UE and chipset vendors) asking for this. |
| Nokia, Nokia Shanghai Bell | In general we share the views from Ericsson and Qualcomm here. Has there also been consideration of similar solutions as discussed with RedCap to control access of such devices to the network? |

Proposal 2: For LTE Cat 3bis and Cat 4bis, reuse the existing RF, RRM and performance requirements specified for DL 1bis to verify single Rx antenna performance, and add additional SDR tests to verify the peak date rate.

| Company | Comments and questions on proposal 2 in RP-211493 (e.g. comments on the expected specification impact) |
| --- | --- |
| Apple | Same understanding as the proposal 2 that RAN4 related work can be limited. |
| OPPO | To reduce the workload, reusing the existing RF/RRM/performance requirements where possible is a good approach. |
| Spreadtrum | Since the single Rx antenna performance has been specified for Cat 1bis, all the existing requirements for Cat 1bis are also applicable to the new defined LTE UE DL category. Additional SDR test is needed to verify the peak data rate higher than Cat1 bis. Thus, we support Proposal 2. |
| Deutsche Telekom | No need (see our answer to 1) |
| Samsung | We shall limit RAN4 effort by reusing the existing requirements |
| China Telecom | Fine with Proposal 2 to only add SDR tests to limit the overall workload. |
| Intel | In case the TEI/WI is approved, then the scope is reasonable |
| Huawei, HiSilicon | From the requirements defined for Cat 1bis, it is feasible to reuse the existing RF/RRM/Demodulation requirements, at the same time, the workload can be reused. Higher peak data rate is supported for new DL UE categories 3bis and 4bis, new SDR test can be defined, but SDR test is conducted under AWGN, no simulation is needed, only FRC (Fixed Reference Channel) needs to be defined |

Proposal 3: Task RAN2 and RAN4 to finalize the work for LTE Cat 3bis and Cat 4bis in TEI17

| Company | Comments and questions on proposal 3 in RP-211493 |
| --- | --- |
| Lenovo | For RAN2 it should be feasible to complete the work in just one 1 meeting since the impacts may be only on new UE category signaling. For RAN4 we are not sure whether the work can be completed in one meeting. |
| Apple | OK with TEI17 considering the scope of the work. |
| OPPO | Yes and we think early implementation in Rel16 should be also allowed due to the fact that market is actually ready. |
| Spreadtrum | Since the urgent commercial market demand and the minor specification work which is mainly on the UE category signaling and additional SDR test, we support to finalize the work in TEI 17. |
| Deutsche Telekom | No need (see our answer to 1) |
| Samsung | OK with TEI17 given the scope of work is quite clear and limited |
| China Telecom | With only SDR requirements to be defined, the RAN4/2 workload would be limited. |
| Huawei, HiSilicon | As stated for Proposal 3, considering the clear work scope and the feasibility to reuse the existing RF/RRM/Demodulation requirements except SDR test, we prefer to finalize the related work in TEI17. |

Proposal 4: UE reporting Category 3bis/4bis can report and therefore fall back to Cat 1bis when it roams to other regions if needed

| Company | Comments and questions on proposal 4 in RP-211493 |
| --- | --- |
| Lenovo | Proposal looks bit odd since UE capability signaling is normally not dependent on the region where the UE is located. But to ensure backwards-compatibility and allow the UE to be operated in legacy networks it looks ok that the UE reporting Category 3bis/4bis shall also report Cat 1bis. |
| Apple | Fallback mode can be further discussed in working group level. No hurry to make decision now. |
| DOCOMO | Fallback category to Cat1bis should be supported irrespective of when roaming or not to avoid any impacts on legacy NW. Current proposal is not sufficient to ensure NW-UE alignment on fallback category as NW does not know whether the UE fallbacks or not. If there is misalignment between NW and UE, rate matching for turbo coding is affected as it is determined based on the soft-buffer size, which depends on UE category. |
| OPPO | We understand that it means UE reporting category 3bis/4bis shall also support UE category 1bis and it should be done in explicit way i.e. category 1bis should be always reported along with UE category 3bis/4bis. For the region where 3bis/4bis category are supported it is up to network to decide on UE category. For region where 3bis/4bis category are not supported, network can only use category 1bis. |
| Ericsson | The proposal sounds as if LTE cat3/4-bis should be limited to a certain region (i.e. China). Is that the intention? If so, how can that be done? |
| Spreadtrum | Introducing fallback to Cat 1bis can help the legacy network supporting Cat 1bis to know that the UE is configured with single Rx antenna, which gives the operators flexibility to do some network optimization. So we support Proposal 4. But we are also fine with Apple’s suggestion to discuss fallback mode in WGs. |
| Deutsche Telekom | No need (see our answer to 1) What are “other regions” ???? |
| Samsung | It shall be discussed in RAN4 |
| Intel | Signalling of a fallback category for use by networks that do not support the Cat3/4bis will have to be considered as part of work. For the solution, we would prefer not to specify any region-specific UE behaviour. In our view the mention of 'roaming to other regions' in the proposal is not relevant, as fallback signalling has to be considered for any case, where the network doesn’t understand the Cat3/4bis signalling. |
| Huawei, HiSilicon | To ensure backwards compatibility and follow the principle defined for DL Cat 1bis, UE indicating support of Cat 3bis/4bis should also report support of Cat 3/4. To address the concerns from some companies, we just proposed another approach i.e. UE supporting of Cat 3bis/4bis can fall back to report Cat 1bis when it roams to certain regions. Having read the above comments, it makes more sense to have a simple behavior whereby the Cat1bis category is always reported by those Cta3bis/Cat4bis UEs. This will ensure that any legacy network have no problem to handle those devices and the issues mentioned above by some companies are solved. I guess we can continue to discuss with an updated Proposal 4 based on the above. |

# Summary of initial discussion

On proposal 1, the following companies responded to the initial round discussion:

- Agree (7): Apple, OPPO, Spreadtrum, Samsung, China Telecom, Huawei, HiSilicon

- Disagree (7): Docomo, Ericsson, Telecom Italia, Qualcomm, Deutsche Telekom, Nokia, Nokia Shanghai Bell

- Uncertain (2): Lenovo, Intel

13 companies co-sourced RP-211493 where proposal 1 was provided, among which 6 companies continued agreeing with the proposal in the initial round discussion, while the other 7 companies did not respond to the initial round.

Main concerns expressed by companies:

- Overlap with RedCap use cases

- Impact on the LTE network performance

A few questions were asked for clarifications, and clarifications provided in subsequent responses:

- Which UL category should be signaled by UEs supporting the new DL categories 3bis and 4bis?

  - Clarification from Huawei: The max UL throughput can be same as UE categories 3 and 4, i.e. UL Category 3.

- How can the peak rate of DL 100/150 Mbps be achieved w/o CA and MIMO?

  - Clarification from Spreadtrum: comments explained how/why the peak rate is halved compared to DL 100/150 Mbps

- Further clarifications on possible market fragmentation and how it can be avoided are encouraged

  - Clarification from proponents: The new LTE UE DL categories 3bis and 4bis is immediate urgent market need, but RedCap specification is still under discussion, it cannot meet the current market need

- Consideration of similar solutions as discussed with RedCap to control access of such devices to the network?

  - Clarification from Huawei: The impact on legacy network can be made ZERO by the use of LTE UE category fallback, something which is normal for LTE but doesn't exist in NR

Discussion on proposal 4 is related to the impact on the LTE network performance.

On proposal 2: the understanding of the supporting companies on the specification impact seems to be aligned with proposal 2. No further discussion on proposal 2 seems needed for the time-being.

On proposal 3: it was generally understood that 1 or maybe 2 meetings would be needed to complete the work. There was a preference from the supporting companies to consider the work as a TEI17. No further discussion on proposal 3 seems needed for the time-being.

On proposal 4:

- A question for clarification asked “what are other regions”? It was noted that UE capability signaling is normally not dependent on the region and it was asked how certain UE categories could be limited to a certain region.

- Some companies commented that fallback to Cat1bis shall be supported in any region, at least to avoid impact to legacy networks.

It was therefore proposed that UE reporting Category 3bis/4bis shall also always report Cat 1bis.

# Intermediate email discussion (Tue 15:00h UTC – Wed 11:00h UTC)

The intermediate email discussion targets further clarifications on the main concerns, i.e. overlap with RedCap use cases, and impact on the LTE network performance.

Regarding impact to LTE network performance, proposal 4 is updated in proposal 4b where fallback to Cat1bis allows a legacy network or a network not wanting to handle the peak rate of those devices with 1Rx to handle them as Cat1bis UEs.

Proposal 4b: UE reporting DL Cat 3bis/4bis shall also always report DL Cat 1bis.

Question 1: please provide comments on proposal 4b, and indicate whether such fallback mechanism doesn’t address the concerns on the impact on the LTE network performance.

| Company | Comments and questions on proposal 4b |
| --- | --- |
| China Unicom | From our view, the fallback mechanism will not impact on LTE network performance. If network doesn’t support for LTE cat 3bis/4bis, the network side could treat UE as LTE cat 1bis. |
| vivo | Sorry for not being able to comment in the initial round of discussion. Vivo support the proposal to specify LTE Cat 3bis/4bis devices since we see the clear market urgency in China due to the certification process issue. We agree with the fact that Proposal 4b can completely remove the concern from the operators or NW vendors who does not like to serve such devices in their NW/market, since in such case the device will be scheduled as Cat1bis. |
| Huawei, HiSilicon | To effectively utilize the higher capability of Cat.3bis/4bis for supporting high definition video, we think that UE shall also report Cat 3/4 besides Cat.1bis and Cat 3bis/4bis. In our understanding this is what the majority of those devices deployed in the market in China (see vivo input below) do today. Which UE category to be used is up to network selection, of course, so once again there is not impact on the network or on the operator. It is proposed to revise the proposal as below: Modified Proposal 4b: UE reporting DL Cat 3bis (resp. 4bis) shall also always report DL Cat 1bis and DL Cat 3 (resp. 4). [Moderator: let’s call this modified proposal 4b] |
| Xiaomi | We support the proposal 4b due to the current market demands we see, sorry for missing the first round discussion. We also think the above revised proposal by Huawei is a good compromise, it works for operator who like to serve the LTE Cat 3bis/4bis devices or not. |
| DOCOMO | We are fine with Proposal 4b to avoid any impact on legacy NW. We don’t agree with the revision from Huawei. Legacy NW cannot control whether the UE fallbacks to Cat3/4 or Cat1bis, which causes NW-UE misalignment on fallback category. |
| OPPO | We think proposal 4b is enough to keep backward compatibility. Not sure whether UE should also report Cat3/4 considering Cat3bis/4bis UE can be only equipped with 1 antenna while the maximum supported layer of Cat3/4 is 2. |
| Spreadtrum | To specify LTE Cat 3bis/4bis devices is to meet the urgent commercial market demand especially in China, and we have the same observation with vivo on the situation of China. We agree with proposal 4b by moderator; with this non-regional fallback mechanism, operators have more flexibility to treat new Cat 3bis/4bis as an old and familiar type(s) based on their network operation strategy. Also, we are open to further improvement on proposal 4b. |
| Deutsche Telekom | We don’t agree with the proposal on fallback as we do not see that Cat3bis/Cat4bis are agreeable in 3GPP. We do not understand the Huawei propose above (modified Proposal 4b) ?! Also, we do not want to see an increasing number of 1 Rx devices in our networks due to the negative system efficiency impacts they create. |
| China Telecom | Ok with Proposal 4b. |
| Telenor | We share the concerns raised by Ericsson, TIM, DT and more on network performance by introducing lower capable devices. We would avoid an increased number of 1RX devices in our networks as those reduced capability devices have negative systems performance impacts and hits back on us as an operator. We share the same view as DT that Cat3bis/Cat4bis are not agreeable by 3GPP. |
| Telia Company | Telia Company sees 1RX UEs causing negative performance impacts to network operators and their customers. Not to be agreed in 3GPP. Telia Company agreed with DT, TIM and Telenor views. |
| Orange | Orange also share the same concerns as Ericsson, TIM, DT and Telenor. 1 Rx devices have a strong detrimental impact on spectral efficiency of the network. 1Rx devices in LTE should rely on the existing Cat1bis category, and higher categories should rely on at least 2 Rx. Introducing Cat3bis and Cat4bis categories is not agreeable for Orange. |
| Telecom Italia | We do not agree with this new proposal, and share the concerns raised by DT and Telenor. Moreover, with proposal 4b, the burden is to operators who want to bar these devices since a modification is required on the network side. And in general, it is not agreeable to develop non standard equipment and then force 3GPP to react when they are put in operation |
| Intel | The original proposal 4b addresses backward compatibility issues and is technically correct. For Huawei Modified proposal 4b, it is not clear how UE can report DL Cat 3/4. The LTE DL Cat 3/4 implies support of 2 DL MIMO layers, which are not supported by the proposed device types. Further clarifications are needed. |
| BT | We share the same concerns as other European operators, and we do not agree with the modified proposal 4b because the legacy network cannot control whether the UE fallbacks to Cat3/4 or Cat1bis, which causes NW-UE misalignment on fallback category. |

Regarding the overlap with RedCap use cases, the proponents clarified that the new LTE UE DL categories 3bis and 4bis is immediate urgent market need, but RedCap specification is still under discussion so it cannot meet the current market need.

Question 2: other comments (e.g. on market urgency vs. RedCap commercial availability) can be provided below.  Please indicate whether the clarifications from the proponents addressed the concerns on market fragmentation.

| Company | Other comments |
| --- | --- |
| China Unicom | We support to introduce the new UE categories to fulfill the market urgency. Moreover, we understand company’s concerns on market fragmentation of Redcap, while we are optimistic for commercial NR Redcap in the next 2 or 3 years. We are also supportive for NR Redcap WI as well as the urgent market demands for NR wearable device. As the operator, we have to satisfy our customer’s demands for the high-end wearable device in a high priority. |
| Vivo | As commented before, this proposal is to address the recent change of certification process issue for single Rx LTE devices which already exists in the market for a long time. NR Redcap, however, is still far from commercialization. There have been millions of LTE based smart watch in the market already, not sure why defining performance requirement for such already-existing devices will fragment the market for NR Redcap which has not be implemented yet. Rather, the proposal is beneficial for operators as now the reasonable performance for such single RX LTE based high-end smart wearables can be ensured. Finally, if such device is not welcome by an NW, it can be treated as Cat1bis according to the proposal 4b above. |
| Qualcomm | Surprised to see the moderator’s arithmetic. Our count is the following: - Agree (6): Apple, OPPO, Spreadtrum, Samsung, Huawei, HiSi - Disagree (8): Docomo, Ericsson, Telecom Italia, Qualcomm, Deutsche Telekom, Intel, Nokia, Nokia Shanghai Bell - Uncertain (1): Lenovo While the moderator’s intermediate summary implied majority support of the proposal. Even if we add CU’s and vivo’s support above, there is still no majority support at present. The discrepancy is due to counting by the moderator companies as supporters who haven’t entered any comments to the discussion. The point of the email discussion is to collect the views expressed, in light of the current arguments, as opposed to counting co-signing companies of input contributions. We think the actual current level of support should be established first, rather than discussing any specification details. |
| Huawei, HiSilicon | There is current immediate urgent market request for LTE Cat 3bis/4bis, however NR RedCap will be ready in the next 2 or 3 years and cannot fulfill the current market request. In addition, the deployment of NR network also needs some time to reach the same coverage as LTE. The introduction of Cat.3bis/4bis does not bring market fragmentation. |
| Xiaomi | NR Redcap devices depending on the NR network deployment and might be ready in the coming 2-3 years. However, this proposal solve the current market demands which are addressed by many UE companies. So we think the proposal just fill the gap before the mature of NR market. |
| DOCOMO | Given the quite strong request from some companies to introduce these new LTE categories, we won’t object to introducing them. However, we are not so optimistic and we don’t think market fragmentation can be completely avoided, which means these new LTE Ues are not on the market when Rel-17 RedCap Ues are ready for the market. |
| OPPO | We don’t share the concern on degradation of LTE network. The LTE terminal market is such mature that no one will produce low-end LTE smart phone based on Cat 3bis/4bis in our opinion. If NR Redcap UE is specified in Rel16 then we would intend to agree with the concern on the market fragment. But wearable device based on Rel17 Redcap will not be there in the coming few years while the current market is demanding such LTE wearable devices which are ready actually. |
| Spreadtrum | Introducing a new LTE UE category “Cat-4bis” with single Rx antenna is used to meet the urgent commercial market demand for high-end wearable devices, but NR RedCap may be commercialized at least several years later to provide attractive reduced latency and better performance. In addition, we have another observation for LTE high-end wearable devices with 1RX. During the initial stage, 5G network cannot achieve full coverage in all regions as that of LTE network. A dual-mode LTE&NR Redcap UE is expected to be more welcomed due to more uniform and good experience on data rate and mobility. For dual-mode LTE/NR Redcap Ues, single antenna configuration for both LTE and NR Redcap can really reduce the space size compared to two antennas for LTE and single antenna for NR Redcap. From this perspective, introducing new LTE UE category with single antenna will potentially accelerate the commercialization of NR Redcap instead of market fragmentation. |
| Deutsche Telekom | We do not see any “urgent” market requirements which requires the introduction of Cat3/4bis devices with just single antenna. What should a device link smart watch (which is often mentioned in the context) do with > 10 .. 20 Mbps ? Further as explained earlier, the customer expects the same coverage with a smart watch as with a smartphone – he will complain to the operator that the coverage is not good enough, but in reality the root cause for this is either the missing 2 Rx or a bad antenna design of the smart watch .. hence this proposal just invites even worse customer experience than what we have today. Hence, we are objecting the introduction of any further 1 Rx category for LTE (like Cat3bis/Cat4bis). 1 |
| China Telecom | As commented by several companies, LTE Cat 3bis/4bis devices, which are based on the existing chipset of Cat ¾, will come soon. In addition, they are served by LTE network in lower frequencies such as 800MHz/1800MHz and with good coverage. As an operator with large-scale deployment of NR SA network, we are also very supportive of NR RedCap which will come to the market later. |
| Telenor | We do not see any “urgent” market requirements for Cat3bis/Cat4bis devices with 1 RX. This is well explained by DT above, we share the same viewpoints on this. Hence, we are objecting the introduction of any further 1 Rx category for LTE (like Cat3bis/Cat4bis). |
| NordicSemi | Sorry for joining the discussion late. We would like to echo Apple previous comment that 4G will be here still for quite some time. Furthermore, since RedCap supports also HD-FDD Ues, having LTE “bis” categories to support also HD-FDD TYPE A, would provide even more commercial motivation for this WID. |
| Telia Company | Telia Company does not support these “urgent” market requirements for Cat3bis/Cat4bis 1RX UE. No new 1RX UE categories are needed for LTE. Telia Company agreed with DT, TIM and Telenor views. |
| Orange | Same view as DT and Telenor. No urgent need market requirement for these categories. 1 Rx device can rely on Cat1bis. For higher throughput requirements, 2 Rx is necessary from a network performance perspective. We are opposed to the introduction of Cat3bis / Cat4bis. |
| Telecom Italia | Same view as DT, Telenor, Telia and Orange |
| Vodafone | We do not yet have a final position on this topic, but we wish to make the following observations and suggest a way forward. - Given the large number of companies co-signing the input contribution and the support from another company (coupled with the recent discussions on the number of rx antennas for RedCap wearables), we expect that a significant number of LTE devices with category >1 but with only one rx antenna are already active within our networks. - Currently we cannot identify which cat 4/3 devices have just one antenna. Adding some signaling to provide this information to the network could be beneficial. - It would be good to avoid market fragmentation between RedCap and “cat 4-bis”, but it seems clear that the cat 4/3-bis devices are already widespread in at least one major market. - Having cat 4/3 bis smartphones with only 1 rx antenna would be a significant degradation of LTE network performance. As the issue seems to relate to device certification, a possible way forward is to only allow cat 4-3bis to be used in devices that have a small form factor, e.g. less than 5 cm*5cm*1cm or displaced volume less than 10 cubic cm. |
| Intel | We would prefer to have a clear differentiation of RedCap and new LTE categories in terms of supported data rate for the same segments of devices. A clear overlap of supported data rates and device categories will not be helpful for NR/LTE technologies. |
| BT | We agree with others that we don’t see any “urgent market requirements” for such devices, and we are concerned about the potential for market fragmentation, extra complexity, new coverage assessments for 1Rx, new access control, and time constraint due to WG being completely overloaded, if this work is progressed. |

# Final email discussion (Wed. 15:30h UTC – Thu 11:00h UTC)

# Summary

# Contact information

| Company | Contact person | Email address |
| --- | --- | --- |
| Lenovo | Hyung-Nam Choi | hchoi5@lenovo.com |
| Apple | Yang Tang | yang.tang@apple.com |
| DOCOMO | Shinya Kumagai | shinya.kumagai@docomo-lab.com |
| Ericsson | Christian Hoymann | christian.hoymann@ericsson.com |
| Qualcomm | Peter Gaal | pgaal@qti.qualcomm.com |
| Spreadtrum | Yu Ding | yu.ding@unisoc.com |
| Deutsche Telekom | Axel Klatt | Axel.Klatt@telekom.de |
| Samsung | Xutao Zhou | xutao.zhou@samsung.com |
| China Telecom | Shan Yang | yangshan@chinatelecom.cn |
| Intel | Andrey Chervyakov | andrey.chervyakov@intel.com |
| Vivo | Xueming Pan | panxueming@vivo.com |
| Huawei, HiSilicon | Simone Provvedi | simone.provvedi@huawei.com |
| Xiaomi | Yang Liu | Liuyang24@xiaomi.com |
| Telenor | Rune Rækken | Rune-harald.rakken@telenor.com |
| BT | Johnny Dixon | Johnny.dixon@bt.com |