3GPP TSG RAN meeting #92e										RP-211508

Electronic Meeting, June 14 – 18, 2020

## Status Report to TSG

Agenda item:			9.8.4.12

| WI / SI Name | Study on optimizations of pi/2 BPSK uplink power in NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_FR2_FWA_Bn259 |  |  |  |  |
| Unique ID | 911015 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210875 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2021 | Performance part: 03/2022 | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: N/A | Core part: 80% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Sumant Iyer (RAN4) |
|  | Company | Qualcomm |
|  | Email | sumanti@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

### 2.1	RAN4

#### Agreements

Agreements pertaining to UE RF

- The min. peak EIRP requirement:

  - Alt-1: Based on scaling from existing requirements

    - [26.7 to 27.5] dBm

  - Alt-2: From averaging UE Tx budget-based proposals.

    - Average power: 28.0 dBm

  - Agreement: FFS

- The REFSENS requirement for 50 MHz CBW, -1 dB SNR target

  - Alt-1: Based on scaling from existing requirements

    - [-89.2 to -89.7] dBm

  - Alt-2: From averaging UE Rx budget-based proposals.

    - Average power: -89.7 dBm

  - Agreement: FFS

- Multi-band Relaxations

  - ∆MBP,n (dB) = 0.5 dB

  - ∆MBS,n (dB) = 0.5 dB

- Spherical coverage requirement shall be derived based on following gain drop:

  - 8 dB degradation @ 85th%ile point

- Beam correspondence requirements

  - Option 1: Bit 0 OK to introduce

  - Option 2: Bit 0 not OK to introduce

  - Agreement: FFS

#### 2.1.2	Remaining Open issues

- Remaining core part (UE RF requirements):

  - Min. peak EIRP requirement

  - REFSENS requirement

- Completion of requirements in 38.101-3

- Completion of requirements in 38.101-4

- Completion of requirements 38.133

- Completion of requirements 38.307

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

N/A

## 4.	References

WF from RAN4#98-Bis-e: R4-2105385

WF from RAN4#99-e: R4-2107839

WID from RAN#91-e: RP-210875