3GPP TSG-RAN5 Meeting #92-e	R5-215936

# Electronic Meeting, 16th – 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2005 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of SIB2 to add messages for relaxed RRM measurement |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Configuration of relaxed RRM measurement is introduced into SIB2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the relaxed RRM measurement configuration into SIB2 structure of 38.508-1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The message couldn’t be refered by test cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-215066 |  |  |  |  |  |  |  |  |

<< Beginning of changes >>

–	SIB2

SIB2 contains cell re-selection information common for intra-frequency, inter-frequency and/ or inter-RAT cell re-selection (i.e. applicable for more than one type of cell re-selection but not necessarily all) as well as intra-frequency cell re-selection information other than neighbouring cell related.

Table 4.6.2-1: SIB2

| Derivation Path: TS 38.331 [6], clause 6.3.1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| nrofSS-BlocksToAverage | 2 |  |  |
| absThreshSS-BlocksConsolidation SEQUENCE{ |  |  |  |
| thresholdRSRP | RSRP-Range | Table 4.6.3-152 |  |
| thresholdRSRQ | Not present |  |  |
| thresholdSINR | Not present |  |  |
| } |  |  |  |
| rangeToBestCell | dB0 |  |  |
| q-Hyst | dB0 | To reduce interference between intra-frequency multiple cells |  |
| speedStateReselectionPars | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | Not present |  |  |
| s-NonIntraSearchQ | Not present |  |  |
| threshServingLowP | 0 | Actual value of threshold = field value * 2 [dB] |  |
| threshServingLowQ | Not present |  |  |
|  | 3 | 3dB | QBASED |
| cellReselectionPriority | 4 | A middle value in the range has been selected |  |
| cellReselectionSubPriority | Not present |  |  |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| q-RxLevMin | -70 | -140dBm |  |
|  | -55 | -110dBm | SIG |
| q-RxLevMinSUL | Not Present |  |  |
|  | -70 | -140dBm | SUL |
|  | -55 | -110dBm | SUL AND SIG |
| q-QualMin | Not present |  |  |
|  | -20 | -20dB | QBASED |
| s-IntraSearchP | 31 | Actual value of threshold = field value * 2 [dB] |  |
| s-IntraSearchQ | Not present |  |  |
| t-ReselectionNR | 0 |  |  |
| frequencyBandList | Not present |  |  |
| frequencyBandListSUL | Not present |  |  |
| p-Max | Not present |  |  |
| smtc | SSB-MTC | Table 4.6.3-185 |  |
| ss-RSSI-Measurement | Not present |  |  |
| ssb-ToMeasure | SSB-ToMeasure |  |  |
| deriveSSB-IndexFromCell | false |  | FDD |
|  | true |  | TDD |
| t-ReselectionNR-SF | Not present |  |  |
| smtc2-LP-r16 | Not present |  |  |
| ssb-PositionQCL-Common-r16 | Not present |  |  |
| } |  |  |  |
| relaxedMeasurement-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SUL | For test cases using SUL frequency for the serving cell, Qrxlevmin is obtained from q-RxLevMin-sul. |
| QBASED | This condition applies to Quality based signalling test cases. |

<< End of changes >>