3GPP TSG-RAN5 Meeting #92-e	R5-216073

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0341 | rev | 1 | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_pow_sav-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE power saving in NR WI requires addition of 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added test case for 4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the WP will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3.2.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR 0083 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-214567 and the outcome of 1 revision to this document |  |  |  |  |  |  |  |  |

5.3.3.2.3	4Rx TDD FR1 PDCCH 1 Tx antenna performance for power saving

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	Table 5.3.3.2.3.4.3.1-7 is FFS.

5.3.3.2.3.1	Test Purpose

This test verifies the demodulation performance of PDCCH under 4 receive antenna conditions and with a given SNR for which the average probability of miss-detection of the Downlink Scheduling Grant (Pm-dsg), shall be below the specified value in Table 5.3.3.2.3.3-2 after recept wake-up indication in the DCI format 2_6 PDCCH in DRX off state. The downlink physical setup is in accordance with Annex C.2.1.

5.3.3.2.3.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

This test also applies to all types of EUTRA UE release 16 and forward supporting EN-DC and 4 Rx antenna ports and Long DRX Cycle and DRX adaptation.

5.3.3.2.3.3	Minimum conformance requirements

During the test the UE shall monitor the DCI format 2_6 PDCCH in DRX off state and decide whether to receive the following PDCCH in DRX on period.

For the parameters specified in Table 5.3.3.2.3.3-1, the average probability of a missed downlink scheduling grant (Pm-dsg) observed on PDCCH during DRX on shall be below the specified value in Table 5.3.3.2.3.3-2. The downlink physical setup is in accordance with Annex C.3.1.

Table 5.3.3.2.3.3-1: Test Parameters

| Parameter |  | Unit | 1 Tx Antenna |
| --- | --- | --- | --- |
| TDD UL-DL pattern |  |  | FR1.30-1 |
| CCE to REG mapping type |  |  | interleaved |
| Interleaver size |  |  | 3 |
| REG bundle size |  |  | 2 |
| Shift Index |  |  | 0 |
| DRX cycle |  | ms | 10 |
| ps-WakeUp-r16 |  |  | absent |
| Wake-up indication bit in DCI format 2_6 |  |  | 1 |
| PDCCH DCI format 2_6 configuration | PS-offset |  | (TminimumTimeGap+1)/2μ/0.125 |
|  | Number of PDCCH candidates |  | 1 |
|  | Frequency domain resource allocation for CORESET |  | Start from RB = 0 with contiguous RB allocation |
|  | TCI state |  | TCI state #1 |
| Slots for PDCCH monitoring |  |  | Each slot during DRX-on period |
| Note: TminimumTimeGap is signaled as a part of drx-Adaptation-r16 UE capability. |  |  |  |

Table 5.3.3.2.3.3-2: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | -0.9 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.3.3.2.

5.3.3.2.3.4	Test description

5.3.3.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-2 of 38.521-1 [7].

Configurations of DRX, DCP, PDSCH and PDCCH before measurement are specified in 5.3.3.2.3.4.3.1 and Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise sources to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.3 for TE diagram and section A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.1.3-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without Release On, Test Mode On) for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.3.3.2.1.4.3.

5.3.3.2.3.4.2	Test procedure

1.	SS transmits PDCCH with DCI format2_6 as specified in PDCCH Reference Channel for C_RNTI within DRX off state. The Wake-up indication bit in PDCCH is set to 1.

2.	SS transmits PDSCH via PDCCH with DCI format1_1 as specified in PDCCH Reference Channel for C_RNTI R.PDCCH. 5-1.2 TDD to transmit the DL RMC according to Table 5.3.3.2.3.3-2 in DRX on period. The details of PDCCH are specified in Table 5.3-1, Table 5.3.3.2-1 and Table 5.3.3.2.3.3-2 respectively. The details of PDSCH are specified in Table A.3.3.2.2-3. The SS sends downlink MAC padding bits on the DL RMC.

3.	Set the parameters of the propagation condition, antenna configuration, the correlation matrix and the SNR according to Table 5.3.3.2.3.3-2 as appropriate.

4.	Measure the Pm-dsg for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL PUCCH during each subtest interval. Pm-dsg is the ratio (statDTX)/(NACK+ACK+statDTX). If Pm-dsg is less than the value specified in table 5.3.3.2.3.5-1, pass the UE. Otherwise fail the UE.

5.	Repeat steps from 2 to 4 for each subtest in Table 5.3.3.2.3.3-2 as appropriate.

5.3.3.2.3.4.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.3.3.2.3.4.3.1	Message exceptions for SA

Table 5.3.3.2.3.4.3.1-1: DRX-Config

| Derivation Path: TS 38.508-1 [4],Table 4.6.3-56 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DRX-Config ::= SEQUENCE { |  |  |  |
| drx-onDurationTimer CHOICE { |  |  |  |
| milliSeconds | ms1 |  |  |
| } |  |  |  |
| drx-InactivityTimer | ms1 |  |  |
| drx-HARQ-RTT-TimerDL | 0 |  |  |
| drx-HARQ-RTT-TimerUL | 0 |  |  |
| drx-RetransmissionTimerDL | sl1 |  |  |
| drx-RetransmissionTimerUL | sl1 |  |  |
| drx-LongCycleStartOffset CHOICE { |  |  |  |
| ms10 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-2: DCP-Config

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-106 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| dcp-Config-r16 CHOICE { |  |  |  |
| setup SEQUENCE { |  |  |  |
| ps-Offset-r16 | 40 |  |  |
| sizeDCI-2-6-r16 | 2 |  |  |
| ps-PositionDCI-2-6-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-3: PDCCH-Config

| Derivation Path: TS 38.508-1 [6],Table 4.6.3-95 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-Config::= SEQUENCE { |  |  |  |
| controlResourceSetToAddModList SEQUENCE(SEQUENCE(SIZE (1..3)) OF ControlResourceSet ::= SEQUENCE { | 2 entry |  |  |
| ControlResourceSet[1] | ControlResourceSet1 |  |  |
| ControlResourceSet[2] | ControlResourceSet2 |  |  |
| } |  |  |  |
| searchSpacesToAddModList SEQUENCE(SIZE (1..10)) OF SearchSpace ::= SEQUENCE { | 1 entry |  |  |
| SearchSpace[1] | SearchSpace |  |  |
| } |  |  |  |
| searchSpacesToAddModListExt-r16 SEQUENCE(SIZE (1..10)) OF SearchSpace { | 1 entry |  |  |
| searchSpaceExt-r16[1] | SearchSpaceExt |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-4: PDCCH-ControlResourceSet1

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { | Null |  |  |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-5: PDCCH-ControlResourceSet2

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| frequencyDomainResources | 11111111 11111111 10000000 00000000 00000000 00000 | CORESET to use the least significant 102 RBs of the BWP |  |
| Duration | 1 | SearchSpace duration of 1 symbol |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| Interleaved SEQUENCE { |  |  |  |
| reg-BundleSize | n2 |  | 1 Tx |
| interleaverSize | n3 |  | TDD |
| } |  |  |  |
| } |  |  |  |
| tci-StatesPDCCH-ToAddList { | 1 | TCI State #1 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-6: PDCCH Search Space

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | 2 | SearchSpaceId with condition USS | USS |
| controlResourceSetId | 1 | ControlResourceSetId |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| nrofCandidates SEQUENCE { |  |  |  |
| aggregationLevel4 | n1 | AL4 |  |
| } |  |  |  |
| } |  |  |  |
| searchSpaceType CHOICE { |  |  |  |
| common SEQUENCE { |  |  | CSS, SISS |
| ue-Specific SEQUENCE { |  |  | USS |
| dci-Formats | formats0-1-And-1-1 | DCI Format 1_1 | Long_DCI |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.3.3.2.3.4.3.1-7: PDCCH Search Space Ext

FFS

Table 5.3.3.2.1.4.3.1-8: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| TCI-State[1] | TCI-StateId 0 |  |  |
| qcl-type1 { |  | Type 1 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type C |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | Ssb : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| TCI-State[2] | TCI-StateId 1 | Type 1 QCL information |  |
| qcl-type1 { |  |  |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | CSI-RS # 0 |  |
| Qcl-Type | Type A |  |  |
| } |  |  |  |
| qcl-type2 { |  | Type 2 QCL information |  |
| Cell | ServCellIndex |  |  |
| Bwp-id | 1 | BWP ID |  |
| referenceSignal | csi-rs : 0 | SSB # 0 |  |
| Qcl-Type | Type D |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

5.3.3.2.3.4.3.2	Message exceptions for NSA

Same as 5.3.3.2.3.4.3.1.

5.3.3.2.3.5	Test requirement

Table 5.3.3.2.3.5-1 defines the primary level settings.

For the parameters specified in Table 5.3-1 the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 5.3.3.2.3.5-1.

Table 5.3.3.2.3.5-1: Minimum performance with 30 kHz SCS

| Test number | Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 40 | 102 | 1 | 4 | R.PDCCH. 2-1.2 TDD | TDLC300- 100 | 1x4 Low | 1 | 0 |
|  |  |  |  | 8 | R.PDCCH. 2-1.4 TDD |  |  |  |  |