3GPP TSG-RAN5 Meeting #92-e	R5-215663

# Electronic Meeting, 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0394 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to FR1 2DLCA PDSCH demodulation with power imbalance test case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Korea |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TC 5.2A.2.2.1 is currently missing many aspects. Incorrect sub-section numbering Section 5.1.1.6 is missing |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Updated Annex G reference to point to the section used for single carrier PDSCH test cases - Corrected sub-section numbering - Added missing section 5.1.1.6 - Added test requirements - Updated test procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This aspect of the test plan will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2A.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 5.1.1.5.3	Antenna connection for CA tests with 4 RX

FFS

#### 5.1.1.6	Applicability and test rules for PDSCH performance requirements with power imbalance for intra-band contiguous CA

For UE passing the FDD and TDD CA power imbalance performance requirements with 2 DL CCs as defined in sections 5.2A.2.2 and 5.2A.3.2, the test coverage can be considered fulfilled with FDD or TDD intra-band contiguous CA with 3 or more DL CCs supported by the UE. During the test, UE is required to test the supported intra-band contiguous CA configurations with 2 DL CCs covering the lowest and highest operating bands.

The channel bandwidth combination for testing is determined by following procedure:

-	First select the bandwidth combinations with the same bandwidth in each carrier.

-	If there is no such bandwidth combination, select the bandwidth combinations with smallest bandwidth difference between the two carriers, and the carrier with smaller bandwidth will be used for test.

-	Among the bandwidth combinations selected, select the CA combination with largest aggregated bandwidth combination.

<many sections skipped>

#### 5.2A.2.2	Requirements for 2RX PDSCH carrier aggregation with power imbalance

##### 5.2A.2.2.0	Minimum conformance requirements for 2RX PDSCH CA with power imbalance

The performance requirements are specified in Table 5.2A.2.2.0-3 and Table 5.2A.2.2.0-4, with the addition of test parameters in Table 5.2A.2.2.0-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2A.2.2.0-1.

Table 5.2A.2.2.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only |  |

Table 5.2A.2.2.0-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD and TDD |
| Active DL BWP index |  |  | 1 |
| Propagation condition |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration |  |  | 1x2 |
| PDSCH configuration | Length (L) |  | FDD: 12TDD: 12 for DL slot, 4 for special slot |
|  | PRB bundling size |  | WB |
| Modulation and code rate |  |  | 64QAM, MCS 26 |
| Number of HARQ Processes |  |  | FDD: 4 TDD: 8 |
| Maximum number of HARQ transmission |  |  | 1 |
| Redundancy version coding sequence |  |  | {0} |
| TDD UL-DL pattern |  |  | 30kHz SCS: FR1.30-1 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | As defined in Table A.1.2-2 for FR1.30-1 |
| PUCCH format for HARQ-ACK feedback |  |  | PUCCH format 1 |
| Overhead for TBS determination |  |  | 0 |
| SSB transmission |  |  | Slot#0 with periodicity 20ms |
| RB assignment |  |  | Full applicable test bandwidth as defined in Table 5.3.5-1 of TS 38.101-1 [6] |

Table 5.2A.2.2.0-3: Minimum performance for FDD CA with 15 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

Table 5.2A.2.2.0-4: Minimum performance for TDD CA with 30 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2A.2.2.

##### 5.2A.2.2.1	2Rx PDSCH Demodulation Performance for CA with power imbalance (2DL CA)

Editor’s note: Following aspects in this clause are incomplete

-	Connection Diagram is in []

- 	Test point selection criteria is TBD

- 	Message contents is TBD

-	Annex F update is TBD

5.2A.2.2.1.1	Test Purpose

To verify the ability of an intra-band adjacent carrier aggregation UE to demodulate the signal transmitted by the PCell or SCell in the presence of a stronger SCell or PCell signal on an adjacent frequency. Throughput is measured on the PCell or SCell only.

5.2A.2.2.1.2	Test applicability

This test applies to all types of NR UE release 15 and forward that supports 2DL CA.

5.2A.2.2.1.3	Test description

5.2A.2.2.1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure [A.3.1.7.1] for TE diagram and clause [A.3.2.2] for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2A-1 to Table 5.2A-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.1.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2A.2.1.1.3.3.

5.2A.2.2.1.3.2	Test procedure

1.	Configure SCC according to Annex C.0, C.1 and C.2 for all downlink physical channels.

2.	The SS shall configure SCC as per TS 38.508-1 [6] clause 5.5.1. Message contents are defined in clause 5.2A.2.2.1.3.3.

3.	SS activates SCC by sending the activation MAC-CE (Refer TS 38.321 [18], clauses 5.9, 6.1.3.10). Wait for at least 1 second (Refer TS 38.133[19], clause9.3).

4.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the MCS according to Table 5.2A.2.2.0-2 on PCC. The SS sends downlink MAC padding bits on the DL RMC.

5.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to 5.2A.2.2.0-2 to 5.2A.2.2.0-4 as appropriate on both PCC and SCC.

6.	Measure the average throughput on PCC for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

5.2A.2.2.1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

FFS

5.2A.2.2.1.4	Test Requirement

Table 5.2A.2.2.0-2, Table 5.2A.2.2.1.4-1 and Table 5.2A.2.2.1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the MCS scheduled as specified in Table 5.2A.2.2.0-2 for the throughput test shall meet or exceed the specified value in Table 5.2A.2.2.1.4-1 for the specified cell power levels.

Table 5.2A.2.2.1.4-1: Minimum performance for FDD CA with 15 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |

Table 5.2A.2.2.1.4-2: Minimum performance for TDD CA with 30 kHz SCS

| Test Number | Bandwidth (MHz) |  | Reference channel |  | Power at antenna port (dBm/Hz) |  | Reference value Fraction of Maximum Throughput (%) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PCell | SCell | PCell | SCell | for PCell | for Scell | PCell | SCell |
| 1 | Selected Channel bandwidth as per section 5.1.1.6 |  | Derived as per section 5.1.3.2 of TS 38.214 [12] | NA | -112 | -106 | 85 | NA |