3GPP TSG-RAN5 Meeting #92-e	R5-215970

# Electronic Meeting, 16 - 27 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 1993 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of test frequencies for CA_n71(2A) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Dish Network |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CADC_NR_LTE_DC_R17-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of test frequencies for CA_n71(2A). Applicable sub-Block CBW combinations are (TS 38.101-1 v17.2.0, , Table 5.5A.2-1)): Applicable CBWs vs SCS are (TS 38.101-1 v17.2.0, Table 5.3.5-1): This result in following CBW combinations for TS 38.508-1 for SCS=15kHz and SCS=30kHz, taking maximum aggregated bandwidth for CA_n71(2A) of 30 MHz into account: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test frequencies for CA_n71(2A) for all applicable CBW combinations and SCS=15 kHz and 30 kHz have been added. As there is no UL CA specified for CA_n71(2A) then there is no limitations to the gap between the sub-blocks (TS 38.101-1, clause 5.3A.5) and the full bandwidth of n71A can be used to set Max Wgap (SB1 Low range, SB2 High range). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testing of CA_n71(2A) not possible. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.5.70, 4.3.1.1.5.71 (new), 4.3.1.1.5.72 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | The present CR is an update of R5-214928 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 4.3.1.1.5.67 – 4.3.1.1.5.70	FFS

###### 4.3.1.1.5.71	CA_n71(2A)

Editor’s note:	Test frequencies for CA_n71(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz; and SCS CC1=30kHz and SCS CC2=15 kHz is FFS.

Table 4.3.1.1.5.71-1: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=15 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (5MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 5+5 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 5+10 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 5+15 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 5+20 | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=5 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (10MHz) + n71A (5-20MHz) |  |  |  |  |  |  |
| 10+5 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 10+10 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=10 MHz |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (5-15MHz) |  |  |  |  |  |  |
| 15+5 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 15+10 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=15 MHz |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (5-10MHz) |  |  |  |  |  |  |
| 20+5 | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=5 MHz |
| 20+10 | SB1 | 10 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-1: Low range for CBW=20 MHz |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.71-1: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.71-2: NR Intra-Band non-contiguous CA configuration CA_n71(2A) without UL CA, SCS=30 kHz, Max Wgap

| SB CBW combination | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A); n71A (10MHz) + n71A (10-20MHz) |  |  |  |  |  |  |
| 10+10 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 10+15 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| 10+20 | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=10 MHz |
|  | SB2 | 20 | 51 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=20 MHz |
| CA_n71(2A); n71A (15MHz) + n71A (10-15MHz) |  |  |  |  |  |  |
| 15+10 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| 15+15 | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=15 MHz |
|  | SB2 | 15 | 38 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=15 MHz |
| CA_n71(2A); n71A (20MHz) + n71A (10MHz) |  |  |  |  |  |  |
| 20+10 | SB1 | 10 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.71-2: Low range for CBW=20 MHz |
|  | SB2 | 10 | 24 | Downlink & Uplink |  | Table 4.3.1.1.1.71-2: High range for CBW=10 MHz |
| Note 1: CA_n71(2A) is specified in [7] 38.101-1 without uplink CA. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

###### 4.3.1.1.5.72 – 4.3.1.1.5.76	FFS