3GPP TSG-RAN5 Meeting #92-e	R5-215995

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5341 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to A-MPR test cases for LTE band 24 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New NS value and A-MPR table were introduced for LTE band 24 in TS 36.101 to comply with the regulatory updates |  |  |  |  |  |  |  |  |
| Summary of change: |  | Additions related to A-MPR conformance requirements, and placeholders for test configuration and test requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformance requirements will not reflect the recent updates for LTE band 24 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4, 6.2.4.2, 6.2.4.3, 6.2.4.4, 6.2.4.4.3, 6.2.4.5, 6.2.4_2, 6.2.4_2.2, 6.2.4_2.4.1, 6.2.4_2.5, 6.2.4_3, 6.2.4_3.2, 6.2.4_3.4.1, 6.2.4_3.5, 6.2.4_4, 6.2.4_4.2, 6.2.4_4.4.1, 6.2.4_4.5, 6.2.4B, 6.2.4B.2, 6.2.4B.4.1, 6.2.4B.5, 6.2.4_s.2, 6.2.4_s.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | Rel-17 to trigger spec upgrade |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Rev1: Checkmark against ME on the coversheet is removed. |  |  |  |  |  |  |  |  |

### 6.2.4	Additional Maximum Power Reduction (A-MPR)

Editor’s note: The following aspects are either missing or not yet determined:

- TT and test points are TBD.

- TT, test points/test configuration and test requirements related to NS_56 are FFS.

<<Unchanged text skipped>>

#### 6.2.4.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA Power Class 3 UE release 8 and forward.

The requirements of this test apply in test case 6.6.3.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_28, NS_29, NS_30, NS 31, NS_36, NS_38, NS_39, NS_45, NS_56 to all types of E-UTRA Power Class 3 UE release 8 and forward.

NOTE:	As a result TC 6.2.4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4.3	Minimum conformance requirements

For UE Power Class 3 the specific requirements and identified sub-clauses are specified in Table 6.2.4.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.3-1 to 6.2.4.3-31 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2.5 apply.

Table 6.2.4.3-1: Additional Maximum Power Reduction (A-MPR) / Spectrum Emission requirements

| Network Signalling value | Requirements (sub-clause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.2-1 | 1.4,3,5,10,15,20 | Table 5.4.2-1 | N/A |  |
| NS_03 | 6.6.2.2.3.1 | 2,4,10, 23, 25,35,36, 66, 70 | 3 | >5 | ≤ 1 |  |
|  |  |  | 5 | >6 | ≤ 1 |  |
|  |  |  | 10 | >6 | ≤ 1 |  |
|  |  |  | 15 | >8 | ≤ 1 |  |
|  |  |  | 20 | >10 | ≤ 1 |  |
| NS_04 | 6.6.2.2.3.2 | 41 | 5 | >6 | ≤1 |  |
|  |  |  | 10, 15, 20 | Table 6.2.4.3-2 |  |  |
| NS_04 | 6.6.2.2.3.2, 6.6.3.3.3.1 | 41 | 5,10, 15, 20 | Table 6.2.4.3-3, Table 6.2.4.3-4 |  |  |
| NS_05 | 6.6.3.3.3.2 | 1 | 10,15,20 | ≥ 50 1 | ≤ 1 1 |  |
|  |  |  | 15, 20 | Table 6.2.4.3-5 2 |  |  |
|  |  | 65 3 | 10,15,20 | ≥ 50 | ≤ 1 1 |  |
|  |  |  | 15,20 | Table 6.2.4.3-5 2 |  |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.4.2-1 | N/A |  |
|  |  | 85 | 5,10 | Table 5.4.2-1 | N/A |  |
| NS_07 | 6.6.2.2.3.3 6.6.3.3.3.3 | 13 | 10 | Table 6.2.4.3-6 | Table 6.2.4.3-6 |  |
| NS_08 | 6.6.3.3.3.4 | 19 | 10, 15 | > 44 | ≤ 3 |  |
| NS_09 | 6.6.3.3.3.5 | 21 | 10, 15 | > 40 | ≤ 1 |  |
|  |  |  |  | > 55 | ≤ 2 |  |
|  |  | 74 6 | 10, 15 | > 40 | ≤ 1 |  |
|  |  |  |  | > 55 | ≤ 2 |  |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4.3-7 | Table 6.2.4.3-7 |  |
| NS_11 | 6.6.2.2.1 6.6.3.3.3.6 | 231 | 1.4, 3, 5, 10,15,20 | Table 6.2.4.3-8 | Table 6.2.4.3-8 |  |
| NS_12 | 6.6.3.3.3.7 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-9, Table 6.2.4.3-10 | Table 6.2.4.3-9, Table 6.2.4.3-10 |  |
| NS_13 | 6.6.3.3.3.8 | 26 | 5 | Table 6.2.4.3-11 | Table 6.2.4.3-11 |  |
| NS_14 | 6.6.3.3.3.9 | 26 | 10, 15 | Table 6.2.4.3-12 | Table 6.2.4.3-12 |  |
| NS_15 | 6.6.3.3.3.10 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4.3-13 Table 6.2.4.3-14 | Table 6.2.4.3-13, Table 6.2.4.3-14 |  |
| NS_16 | 6.6.3.3.3.11 | 27 | 3, 5, 10 | Table 6.2.4.3-15, Table 6.2.4.3-16, Table 6.2.4.3-17 |  |  |
| NS_17 | 6.6.3.3.3.12 | 28 | 5, 10 | Table 5.4.2-1 | N/A |  |
| NS_18 | 6.6.3.3.3.13 | 28 | 5 | ≥ 2 | ≤ 1 |  |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |  |
| NS_19 | 6.6.3.3.3.14 | 44 | 10, 15, 20 | Table 6.2.4.3-18 | Table 6.2.4.3-18 |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.3.15 | 23 | 5, 10, 15, 20 | Table 6.2.4.3-19 | Table 6.2.4.3-19 |  |
| NS_21 | 6.6.2.2.3.1 6.6.3.3.3.16 | 30 | 5, 10 | Table 6.2.4.3-20 | Table 6.2.4.3-20 |  |
| NS_22 | 6.6.3.3.3.17 | 42, 43 | 5, 10, 15, 20 | Table 6.2.4.3-21 |  |  |
| NS_23 | 6.6.3.3.3.18 | 42, 43 | 5, 10, 15, 20 | N/A |  |  |
| NS_24 | 6.6.3.3.3.19 | 65 4 | 5, 10, 15, 20 | Table 6.2.4.3-22 |  |  |
| NS_25 | 6.6.3.3.3.20 | 65 4 | 5, 10, 15, 20 | Table 6.2.4.3-23 |  |  |
| … |  |  |  |  |  |  |
| NS_27 | 6.6.2.2.3.4, 6.6.3.3.3.21 | 48 | 5, 10, 15, 20 | Table 6.2.4.3-24 |  |  |
| NS_28 | 6.2.2A, 6.6.3.3.22 | 46 (NOTE 5) | 20 | Table 6.2.4-25 | NS_28 |  |
| NS_29 | 6.2.2A, 6.6.2.3.1a, 6.6.3.3.23 | 46 (NOTE 5) | 20 | Table 6.2.4-26 | NS_29 |  |
| NS_30 | 6.2.2A, 6.6.3.3.24 | 46 (NOTE 5) | 20 | Table 6.2.4-27 | NS_30 |  |
| NS_31 | 6.2.2A, 6.6.3.3.25 | 46 (NOTE 5) | 20 | Table 6.2.4-28 | NS_31 |  |
| NS_32 | - | - | - | - | - |  |
| NS_35 | 6.6.2.2.3.5 | 71 | 5, 10, 15, 20 | N/A |  |  |
| NS_36 |  | 68 | 5, 10, 15 | Table 6.2.4.3-29 |  |  |
| … |  |  |  |  |  |  |
| NS_38 | 6.6.3.3.3.27 | 74 | 3, 5, 10, 15, 20 | Table 6.2.4.3-30 |  |  |
| NS_39 | 6.6.3.3.3.28 | 74 | 10, 15, 20 | Table 6.2.4.3-31 |  |  |
| … |  |  |  |  |  |  |
| NS_45 | 6.6.3.3.3.29 | 53 | 1.4, 3, 5, 10 | N/A |  |  |
| .. |  |  |  |  |  |  |
| NS_56 | 6.6.3.3.3.31 | 24 | 5, 10 | Table 6.2.4.3-32 |  |  |
| Note 1 Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in Note2 as the emissions requirement in 6.6.3.3.3.2 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. Note 2 Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. Note 3: Applicable when the E-UTRA carrier is within 1920-1980 MHz. Note 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. Note 5: Applicable only for an LAA Scell configured in Band 46. |  |  |  |  |  |  |

<<Unchanged text skipped>>

Table 6.2.4.3-32: A-MPR for "NS_56"

| Channel bandwidth confined to 1627.5- 1637.5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region A | Region B | Region C | Region D | Region E | Region F | Region G |
| 5 MHz | 1630.0, 1630.3 | RBstart | ≤ 8 | ≤ 8 | N/A | N/A | N/A | N/A | N/A |
|  |  | LCRB [RBs] | ≤ 8 | > 8 | N/A | N/A | N/A | N/A | N/A |
|  |  | A-MPR [dB] | 8 | 2 | N/A | N/A | N/A | N/A | N/A |
|  | 1635.0 | No A-MPR needed |  |  |  |  |  |  |  |
|  | 1649.0 |  |  |  |  |  |  |  |  |
|  | 1654.0 |  |  |  |  |  |  |  |  |
| 10 MHz | 1632.5 | RBstart | ≤ 5 | ≤ 18 | ≤ 18 | ≥ 35 | ≥ 35 | ≥ 40 | ≥ 40 |
|  |  | LCRB [RBs] | ≤ 5 | ≤ 12 | > 12 | ≤ 7 | > 7 | ≤ 7 | > 7 |
|  |  | A-MPR [dB] | 7 | 5 | 7 | 4 | 2 | 5 | 3 |
|  | 1651.5 | No A-MPR needed |  |  |  |  |  |  |  |

<<Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4.4	Test Description

<<Unchanged text skipped>>

Table 6.2.4.4.1-26: Test Configuration Table (network signalled value "NS_36")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 700.5 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 |  | 700.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 |  | 700.5 | 5MHz |  |  | QPSK | 20 | 1 |
| 4 |  | 700.5 | 5MHz |  |  | QPSK | 24 | 1 |
| 5 |  | 700.5 | 5MHz |  |  | QPSK | 12 | 0 |
| 6 (Notes 2,3) |  | 700.5 | 5MHz |  |  | 16QAM | 12 | 0 |
| 7 |  | 705.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 8 (Notes 2,3) |  | 705.5 | 5MHz |  |  | 16QAM | 25 | 0 |
| 9 |  | 703 | 10MHz |  |  | QPSK | 50 | 0 |
| 10 |  | 703 | 10MHz |  |  | QPSK | 1 | 0 |
| 11 |  | 703 | 10MHz |  |  | QPSK | 40 | 9 |
| 12 |  | 703 | 10MHz |  |  | QPSK | 36 | 13 |
| 13 |  | 703 | 10MHz |  |  | QPSK | 10 | 40 |
| 14 |  | 703 | 10MHz |  |  | QPSK | 6 | 44 |
| 15 |  | 708 | 10MHz |  |  | QPSK | 50 | 0 |
| 16 (Notes 2,3) |  | 708 | 10MHz |  |  | 16QAM | 50 | 0 |
| 17 |  | 705.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 18 |  | 705.5 | 15MHz |  |  | QPSK | 50 | 17 |
| 19 |  | 705.5 | 15MHz |  |  | QPSK | 50 | 24 |
| 20 |  | 705.5 | 15MHz |  |  | QPSK | 1 | 57 |
| 21 |  | 705.5 | 15MHz |  |  | QPSK | 10 | 17 |
| 22 |  | 705.5 | 15MHz |  |  | QPSK | 18 | 24 |
| 23 (Notes 2,3) |  | 705.5 | 15MHz |  |  | 16QAM | 18 | 24 |
| 24 |  | 710.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 25 |  | 710.5 | 15MHz |  |  | 16QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |

Table 6.2.4.4.1-27: Test Configuration Table (network signalled value "NS_56")

FFS

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

##### 6.2.4.4.3	Message contents

<<Unchanged text skipped>>

6.2.4.4.3.30	Message contents exceptions (network signalled value "NS_45")

1.	Information element additionalSpectrumEmission is set to NS_45. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.30-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 45 (NS_45) |  |  |

6.2.4.4.3.31	Message contents exceptions (network signalled value "NS_56")

1.	Information element additionalSpectrumEmission is set to NS_56. This can be set in the SystemInformationblockType2 as part of the cell broadcast message. This exception indicates that the UE shall meet the additional spurious emission requirement for a specific deployment scenario.

Table 6.2.4.4.3.31-1: SystemInformationBlockType2: Additional spurious emissions requirement

| Derivation Path: TS 36.508 [7] clause 4.4.3.3, Table 4.4.3.3-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| additionalSpectrumEmission | 56 (NS_56) |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4.5-1 to Table 6.2.4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

<<Unchanged text skipped>>

Table 6.2.4.5-29: UE Power Class 3 test requirements (network signalled value "NS_36")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 2 | 0 | 21 | 2 | 25.7 | 18.3 |
| 2 | 1 | 4 | 0 | 18 | 4 | 25.7 | 13.3 |
| 3 | 1 | 1 | 0 | 21 | 2 | 25.7 | 18.3 |
| 4 | 1 | 3 | 0 | 19 | 3.5 | 25.7 | 14.8 |
| 5 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 6 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 7 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 8 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 9 | 1 | 10 | 0 | 12 | 6 | 25.7 | 5.3 |
| 10 | 0 | 10 | 0 | 13 | 5 | 25.7 | 7.3 |
| 11 | 1 | 8 | 0 | 14 | 5 | 25.7 | 8.3 |
| 12 | 1 | 6 | 0 | 16 | 5 | 25.7 | 10.3 |
| 13 | 0 | 6 | 0 | 17 | 5 | 25.7 | 11.3 |
| 14 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 15 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 16 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 17 | 1 | 10 | 0 | 12 | 6 | 25.7 | 5.3 |
| 18 | 1 | 9 | 0 | 13 | 5 | 25.7 | 7.3 |
| 19 | 1 | 7 | 0 | 15 | 5 | 25.7 | 9.3 |
| 20 | 0 | 6 | 0 | 17 | 5 | 25.7 | 11.3 |
| 21 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 22 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 23 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| 24 | 1 | 0 | 0 | 22 | 2 | 25.7 | 19.3 |
| 25 | 2 | 0 | 0 | 21 | 2 | 25.7 | 18.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4.5-29: UE Power Class test requirements (network signalled value "NS_56")

FFS

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4_2	Additional Maximum Power Reduction (A-MPR) for UL 64QAM

Editor's note:

- Test requirements values in this section need to be reviewed in order to take into account notes in Table 6.2.5.3-2: ΔTIB,c regarding ΔTIB,c and asymmetric lower tolerances in table 6.2.2.3-1.

- Test points/test configuration and test requirements related to NS_56 are FFS.

<<Unchanged text skipped>>

#### 6.2.4_2.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_1 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

The requirements of this test apply in test case 6.6.3.3_1 Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_38, NS_39, NS_56 to all types of E-UTRA UE from release 13 and forward that support UL 64QAM.

NOTE:	As a result TC 6.2.4_2 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4_2.4	Test description

##### 6.2.4_2.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_2.4.1-1 through table 6.2.4_2.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged text skipped>>

Table 6.2.4_2.4.1-27: Test Configuration Table (network signalled value "NS_36")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 700.5 | 5MHz | N/A for A-MPR testing |  | 64QAM | 25 | 0 |
| 2 |  | 705.5 | 5MHz |  |  | 64QAM | 25 | 0 |
| 3 |  | 703 | 10MHz |  |  | 64QAM | 50 | 0 |
| 4 |  | 708 | 10MHz |  |  | 64QAM | 50 | 0 |
| 5 |  | 705.5 | 15MHz |  |  | 64QAM | 75 | 0 |
| 6 |  | 710.5 | 15MHz |  |  | 64QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Void Note 3: Void Note 4: The 64QAM configurations shall not apply to UEs for which TC 6.2.4_4 is executed with network signalled value NS_36. |  |  |  |  |  |  |  |  |

Table 6.2.4_2.4.1-28: Test Configuration Table (network signalled value "NS_56")

FFS

<<Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4_2.5	Test requirements

<<Unchanged text skipped>>

Table 6.2.4_2.5-29: UE Power Class 3 test requirements (network signalled value "NS_36")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 4 | 0 | 16 | 5 | 25.7 | 10.3 |
| 2 | 3 | 0 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 3 | 3 | 10 | 0 | 10 | 6 | 25.7 | 3.3 |
| 4 | 3 | 0 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| 5 | 3 | 10 | 0 | 10 | 6 | 25.7 | 3.3 |
| 6 | 3 | 0 | 0 | 20 | 2.5 | 25.7 | 16.8 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4_2.5-30: UE Power Class test requirements (network signalled value "NS_56")

FFS

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4_3	Additional Maximum Power Reduction (A-MPR) with PUSCH frequency hopping

Following aspects missing or not yet determined:

- Emissions test cases covering this test is missing

- HARQ configuration for re-transmissions is missing

- Test requirements related to NS_56 are FFS

##### 6.2.4_3.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used. When PUSCH inter-subframe frequency hopping is configured the UE will use different resource blocks for HARQ retransmissions compared to the previous transmission resulting in potentially different A-MPR.

The purpose of this test is to ensure the UE is using A-MPR in a correct way fulfilling emissions requirements also when PUSCH frequency hopping is enabled.

##### 6.2.4_3.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_3 Additional Spectrum Emission Mask for network signalled values NS_07, NS_11 and NS_20 to all types of E-UTRA UE release 8 and forward.

The requirements of this test apply in test case TBD Additional Spurious Emissions for network signalled values NS_07, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_19, NS_20 and NS_56 to all types of E-UTRA UE release 8 and forward.

NOTE:	As a result TC 6.2.4_3 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

##### 6.2.4_3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2.4.3

##### 6.2.4_3.4	Test description

6.2.4_3.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_3.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.4_3.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  | Normal |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Mid range |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  | Test dependant |  |
| Test Parameters |  |  |  |  |  |  |
|  |  |  | Downlink Configuration |  | Uplink Configuration |  |
| Configuration ID | NS-value | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD |
| 1 | NS_07 | 10 MHz | N/A for A-MPR testing |  | QPSK | 1 |
| 2 | NS_10 | 20 MHz |  |  | QPSK | 1 |
| 3 (Note 3) | NS_11 | 15 MHz |  |  | QPSK | 1 |
| 4 | NS_12 | 5 MHz |  |  | QPSK | 1 |
| 5 | NS_13 | 5 MHz |  |  | QPSK | 1 |
| 6 | NS_14 | 15 MHz |  |  | QPSK | 1 |
| 7 (Note 4) | NS_15 | 15 MHz |  |  | QPSK | 1 |
| 8 (Note 4) | NS_16 | 10 MHz |  |  | QPSK | 1 |
| 9 | NS_19 | 20 MHz |  |  | QPSK | 1 |
| 10 (Note 4) | NS_20 | 10 MHz |  |  | QPSK | 1 |
| 11 | NS_56 | 10 MHz |  |  | QPSK | 1 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_3.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: The RBstart of partial RB allocation shall be RB# 0 of the channel bandwidth. Note 3: High range test frequency to be used. Note 4: Low range test frequency to be used. |  |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in Figure TS 36.508 [7] Annex A, Figure A.3.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] clause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to the applicable table from Table 6.2.4_3.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF according to TS 36.508 [7] clause 5.2A.2. Message contents are defined in clause 6.2.4.4.3.

6.2.4_3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0 with hopping bit(s) set to 1 (PUSCH type 2 hopping) for C_RNTI to schedule the UL RMC according to the applicable Table 6.2.4_3.4.1-1. SS sends always NACK on PHICH to trigger UE retransmissions needed for mirroring of RBs. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode in a TTI. The period of measurement shall be one sub-frame (1ms) excluding the 20 us transient periods in beginning and end of the subframe.

4.	Measure the mean power of the UE in the channel bandwidth of the radio access mode in the next TTI of the same HARQ process. The period of measurement shall be one sub-frame (1ms). For TDD slots with transient periods are not under test.

6.2.4_3.4.3	Message contents

Same message contents as in 6.2.4.4.2

##### 6.2.4_3.5	Test requirements

The maximum output power, derived in step 3 and 4 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table 6.2.4_3.5-1 and 6.2.4_3.5-2. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

Table 6.2.4_3.5-1: UE Power Class 3 test requirements (for TTI where CURRENT_TX_NB mod2=0 hence without mirroring)

| Configuration ID | NS-value | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NS_07 | 0 | 12 | 0 | 11 | 6 | 25.7 | 4.3 |
| 2 | NS_10 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| 3 | NS_11 | 0 | 10 | 0 | 13 | 5 | 25.7 | 7.3 |
| 4 | NS_12 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| 5 | NS_13 | 0 | 3 | 0 | 20 | 2,5 | 25.7 | 16.8 |
| 6 | NS_14 | 0 | 3 | 0 | 20 | 2,5 | 25.7 | 16.8 |
| 7 | NS_15 | 0 | 4 | 1.5 | 17.5 | 5 | 25.7 | 11.8 |
| 8 | NS_16 | 0 | 5 | 0 | 18 | 4 | 25.7 | 13.3 |
| 9 | NS_19 | 0 | 2 | 0 | 21 | 2 | 25.7 | 18.3 |
| 10 | NS_20 | 0 | 16 | 0 | 7 | 7 | 25.7 | -0.7 |
| 11 | NS_56 | 0 | FFS | FFS | FFS | FFS | FFS | FFS |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

Table 6.2.4_3.5-2: UE Power Class 3 test requirements (for TTI where CURRENT_TX_NB mod2=1 hence with mirroring)

| Configuration ID | NS-value | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | NS_07 | 0 | 3 | 0 | 20 | 6 | 25.7 | 13.3 |
| 2 | NS_10 | 0 | 0 | 0 | 23 | 4 | 25.7 | 18.3 |
| 3 | NS_11 | 0 | 6.5 | 0 | 16.5 | 5 | 25.7 | 10.8 |
| 4 | NS_12 | 0 | 0 | 0 | 23 | 4 | 25.7 | 18.3 |
| 5 | NS_13 | 0 | 0 | 0 | 23 | 2.5 | 25.7 | 19.8 |
| 6 | NS_14 | 0 | 0 | 0 | 23 | 2.5 | 25.7 | 19.8 |
| 7 | NS_15 | 0 | 9 | 0 | 14 | 5 | 25.7 | 8.3 |
| 8 | NS_16 | 0 | 0 | 0 | 23 | 4 | 25.7 | 18.3 |
| 9 | NS_19 | 0 | 0 | 0 | 23 | 2 | 25.7 | 20.3 |
| 10 | NS_20 | 0 | 6 | 0 | 17 | 7 | 25.7 | 9.3 |
| 11 | NS_56 | 0 | FFS | FFS | FFS | FFS | FFS | FFS |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |  |

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4_4	Additional Maximum Power Reduction (A-MPR) for UL 256QAM

Editor's note: Some test requirements for NS_24 and NS_25 - for E-UTRA band 65 - need further investigation, due to unclear conformance requirements.

Editor’s note: TT, test points/test configuration and test requirements related to NS_56 are FFS.

<<Unchanged text skipped>>

#### 6.2.4_4.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_2 Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

The requirements of this test apply in test case 6.6.3.3_2 Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_56 to all types of E-UTRA UE from release 14 and forward that support UL 256QAM.

NOTE:	As a result, TC 6.2.4_4 has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4_4.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.2.4.3

#### 6.2.4_4.4	Test description

##### 6.2.4_4.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4_4.4.1-1 through table 6.2.4_4.4.1-27. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged text skipped>>

Table 6.2.4_4.4.1-27: Test Configuration Table (network signalled value "NS_36")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 700.5 | 5MHz | N/A for A-MPR testing |  | 256QAM | 25 | 0 |
| 2 |  | 705.5 | 5MHz |  |  | 256QAM | 25 | 0 |
| 3 |  | 703 | 10MHz |  |  | 256QAM | 50 | 0 |
| 4 |  | 708 | 10MHz |  |  | 256QAM | 50 | 0 |
| 5 |  | 705.5 | 15MHz |  |  | 256QAM | 75 | 0 |
| 6 |  | 710.5 | 15MHz |  |  | 256QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4_4.5 as not all combinations are necessarily required based on the applicability of the UE. |  |  |  |  |  |  |  |  |

Table 6.2.4_4.4.1-28: Test Configuration Table (network signalled value "NS_56”)

FFS

<<Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4_4.5	Test requirements

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4_4.5-1 to Table 6.2.4_4.5-29. The allowed A-MPR values specified in Table 6.2.4.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5.3-1 apply.

<<Unchanged text skipped>>

Table 6.2.4_4.5-29: UE Power Class 3 test requirements (network signalled value "NS_36")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 4 | 0 | 14.0 | 5.0 | 25.7 | 8.3 |
| 2 | 5 | 0 | 0 | 18.0 | 4.0 | 25.7 | 13.3 |
| 3 | 5 | 10 | 0 | 8.0 | 6.0 | 25.7 | 1.3 |
| 4 | 5 | 0 | 0 | 18.0 | 4.0 | 25.7 | 13.3 |
| 5 | 5 | 10 | 0 | 8.0 | 6.0 | 25.7 | 1.3 |
| 6 | 5 | 0 | 0 | 18.0 | 4.0 | 25.7 | 13.3 |
| NOTE: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4_4.5-30: UE Power Class 3 test requirements (network signalled value "NS_56")

FFS

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4B	Additional Maximum Power Reduction (A-MPR) for UL-MIMO

### - Test points/test configuration and test requirements related to NS_56 are FFS.

#### 6.2.4B.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the sum output power at each antenna connector as specified in Table 6.2.2B.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

#### 6.2.4B.2	Test applicability

The requirements of this test apply in test case 6.6.2.2 B Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA UE release 10 and forward that support UL MIMO.

The requirements of this test apply in test case 6.6.3.3 B Additional Spurious Emissions for network signalled values NS_04, NS_05, NS_07, NS_08, NS_09, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_36, NS_38, NS_39, NS_56 to all types of E-UTRA UE release 10 and forward that support UL MIMO.

NOTE:	As a result TC 6.2.4B has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4B.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 6.2.4.3.

#### 6.2.4B.4	Test description

##### 6.2.4B.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in table 5.4.2.1-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in table 6.2.4B.4.1-1 through table 6.2.4B.4.1-23. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

<<Unchanged text skipped>>

Table 6.2.4B.4.1-23: Test Configuration Table (network signalled value "NS_36")

|  | Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment (as specified in TS 36.508 [7] subclause 4.1) |  |  |  |  |  | Normal |  |  |
| Test Frequencies (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | Refer to FUL in test parameters with following: 5 MHz CH BW: - 700.5 MHz (NUL = low range) - 705.5 MHz (NUL =132747) 10 MHz CH BW: - 703 MHz (NUL = low range) - 708 MHz (NUL = 132772) 15 MHz CH BW: - 705.5 MHz (NUL = low range) - 710.5 MHz (NUL =132797 ) |  |  |
| Test Channel Bandwidths (as specified in TS 36.508 [7] subclause 4.3.1) |  |  |  |  |  | 5MHz, 10MHz, 15MHz |  |  |
| Test Parameters for NS_36 A-MPR |  |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |
| Configuration ID |  | FUL | Ch BW | Mod'n | RB allocation FDD | Mod'n | RB allocation FDD | RBstart FDD |
| 1 |  | 700.5 | 5MHz | N/A for A-MPR testing |  | QPSK | 1 | 0 |
| 2 |  | 700.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 3 |  | 700.5 | 5MHz |  |  | QPSK | 20 | 1 |
| 4 |  | 700.5 | 5MHz |  |  | QPSK | 24 | 1 |
| 5 |  | 700.5 | 5MHz |  |  | QPSK | 12 | 0 |
| 6 (Notes 2,3) |  | 700.5 | 5MHz |  |  | 16QAM | 12 | 0 |
| 7 |  | 705.5 | 5MHz |  |  | QPSK | 25 | 0 |
| 8 (Notes 2,3) |  | 705.5 | 5MHz |  |  | 16QAM | 25 | 0 |
| 9 |  | 703 | 10MHz |  |  | QPSK | 50 | 0 |
| 10 |  | 703 | 10MHz |  |  | QPSK | 1 | 0 |
| 11 |  | 703 | 10MHz |  |  | QPSK | 40 | 9 |
| 12 |  | 703 | 10MHz |  |  | QPSK | 36 | 13 |
| 13 |  | 703 | 10MHz |  |  | QPSK | 10 | 40 |
| 14 |  | 703 | 10MHz |  |  | QPSK | 6 | 44 |
| 15 |  | 708 | 10MHz |  |  | QPSK | 50 | 0 |
| 16 (Notes 2,3) |  | 708 | 10MHz |  |  | 16QAM | 50 | 0 |
| 17 |  | 705.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 18 |  | 705.5 | 15MHz |  |  | QPSK | 50 | 17 |
| 19 |  | 705.5 | 15MHz |  |  | QPSK | 50 | 24 |
| 20 |  | 705.5 | 15MHz |  |  | QPSK | 1 | 57 |
| 21 |  | 705.5 | 15MHz |  |  | QPSK | 10 | 17 |
| 22 |  | 705.5 | 15MHz |  |  | QPSK | 18 | 24 |
| 23 (Notes 2,3) |  | 705.5 | 15MHz |  |  | 16QAM | 18 | 24 |
| 24 |  | 710.5 | 15MHz |  |  | QPSK | 75 | 0 |
| 25 |  | 710.5 | 15MHz |  |  | 16QAM | 75 | 0 |
| Note 1: The Configuration ID will be used to map the applicable Test Configuration to the corresponding Test Requirement in subclause 6.2.4.5 as not all combinations are necessarily required based on the applicability of the UE. Note 2: Applies only for UE-Categories ≥2. Note 3: The 16QAM configurations shall not apply to UEs which support UL 64QAM. |  |  |  |  |  |  |  |  |

Table 6.2.4B.4.1-24: Test Configuration Table (network signalled value "NS_56")

FFS

<<Unchanged text skipped>>

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

#### 6.2.4B.5	Test requirements

The maximum output power, derived in step 2 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4B.5-1 to Table 6.2.4B.5-27. The allowed A-MPR values specified in Table 6.2.4B.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3B. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5B.3-1 apply.

<<Unchanged text skipped>>

Table 6.2.4B.5-27: UE Power Class 3 test requirements (network signalled value "NS_36")

| Configuration ID | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 2 | 0 | 21 | 5 | 25.7 | 15.3 |
| 2 | 1 | 4 | 0 | 18 | 5 | 25.7 | 12.3 |
| 3 | 1 | 1 | 0 | 21 | 5 | 25.7 | 15.3 |
| 4 | 1 | 3 | 0 | 19 | 5 | 25.7 | 13.3 |
| 5 | 1 | 0 | 0 | 22 | 5 | 25.7 | 16.3 |
| 6 | 2 | 0 | 0 | 21 | 5 | 25.7 | 15.3 |
| 7 | 1 | 0 | 0 | 22 | 5 | 25.7 | 16.3 |
| 8 | 2 | 0 | 0 | 21 | 5 | 25.7 | 15.3 |
| 9 | 1 | 10 | 0 | 12 | 6 | 25.7 | 5.3 |
| 10 | 0 | 10 | 0 | 13 | 6 | 25.7 | 6.3 |
| 11 | 1 | 8 | 0 | 14 | 6 | 25.7 | 7.3 |
| 12 | 1 | 6 | 0 | 16 | 5 | 25.7 | 10.3 |
| 13 | 0 | 6 | 0 | 17 | 5 | 25.7 | 11.3 |
| 14 | 0 | 0 | 0 | 23 | 3 | 25.7 | 19.3 |
| 15 | 1 | 0 | 0 | 22 | 5 | 25.7 | 16.3 |
| 16 | 2 | 0 | 0 | 21 | 5 | 25.7 | 15.3 |
| 17 | 1 | 10 | 0 | 12 | 6 | 25.7 | 5.3 |
| 18 | 1 | 9 | 0 | 13 | 6 | 25.7 | 6.3 |
| 19 | 1 | 7 | 0 | 15 | 6 | 25.7 | 8.3 |
| 20 | 0 | 6 | 0 | 17 | 5 | 25.7 | 11.3 |
| 21 | 0 | 0 | 0 | 23 | 3 | 25.7 | 19.3 |
| 22 | 1 | 0 | 0 | 22 | 5 | 25.7 | 16.3 |
| 23 | 2 | 0 | 0 | 21 | 5 | 25.7 | 15.3 |
| 24 | 1 | 0 | 0 | 22 | 5 | 25.7 | 16.3 |
| 25 | 2 | 0 | 0 | 21 | 5 | 25.7 | 15.3 |
| Note 1: Lower limit is assuming ΔTIB,c is zero. If non-zero, PCMAX,c will decrease and T(PCMAX_L,c) may be higher resulting in different test requirements according to Table 6.2.5.3-1. |  |  |  |  |  |  |  |

Table 6.2.4B.5-28: UE Power Class 3 test requirements (network signalled value "NS_56")

FFS

------------------------------------------------------------- NEXT CHANGE ------------------------------------------------------

### 6.2.4_s	Additional Maximum Power Reduction (A-MPR) for subslot/slot TTI

6.2.4_s.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2_s.3-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

6.2.4_s.2	Test applicability

The requirements of this test apply in test case 6.6.2.2_s Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07, NS_11, NS_20, NS_21, NS_27 and NS_35 to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

The requirements of this test apply in test case 6.6.3.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_11, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_19, NS_20, NS_21, NS_22, NS_23, NS_24, NS_25, NS_27, NS_28, NS_29, NS_30, NS 31, NS_36, NS_38. NS_39, NS_56 to all types of E-UTRA Power Class 3 UE release 15 and forward that support subslot/slot TTI.

NOTE:	As a result TC 6.2.4_s has not been included in the test case applicability Table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

6.2.4_s.3	Minimum conformance requirements

Same minimum requirements as in clause 6.2.4.3 with the following exceptions:

-	Instead of Table 6.2.4.3-4  use Table 6.2.4_s.3-4

-	Instead of Table 6.2.4.3-6  use Table 6.2.4_s.3-6

-	Instead of Table 6.2.4.3-7  use Table 6.2.4_s.3-7

-	Instead of Table 6.2.4.3-21  use Table 6.2.4_s.3-21

The normative reference for this requirement is TS 36.101 [2] clause 6.2.4.

<<Unchanged text skipped>>

Table 6.2.4_s.3-21: A-MPR for "NS_22"

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 5 | No A-MPR is needed for 5 MHz channel bandwidth |  |  |  |  |
| 10 | RBstart | 0-13 | 0-17 | ≤ 6 | ≥12 |
|  | LCRB [RBs] | > 36 | 33-36 | ≤ 32 | ≤ 32 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥44 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 3 |
| 15 | RBstart | 0-24 | 0-38 | ≤ 14 | ≥ 23 |
|  | LCRB [RBs] | > 50 | 37-50 | ≤ 36 | ≤ 36 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥59 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| 20 | RBstart | 0-35 | 0-51 | ≤ 21 | ≥ 31 |
|  | LCRB [RBs] | > 64 | 49-64 | ≤ 48 | ≤ 48 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥79 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4_s.3-22: A-MPR for "NS_56"

| Channel bandwidth confined to 1627.5- 1637.5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region A | Region B | Region C | Region D | Region E | Region F | Region G |
| 5 MHz | 1630.0, 1630.3 | RBstart | ≤ 8 | ≤ 8 | N/A | N/A | N/A | N/A | N/A |
|  |  | LCRB [RBs] | ≤ 8 | > 8 | N/A | N/A | N/A | N/A | N/A |
|  |  | A-MPR [dB] | 8 | 2 | N/A | N/A | N/A | N/A | N/A |
|  | 1635.0 | No A-MPR needed |  |  |  |  |  |  |  |
|  | 1649.0 |  |  |  |  |  |  |  |  |
|  |  | 1654.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 MHz |  | 1632.5 |  | RBstart |  | ≤ 5 |  | ≤ 18 |  | ≤ 18 |  | ≥ 35 |  | ≥ 35 |  | ≥ 40 |  | ≥ 40 |  |
|  |  |  |  | LCRB [RBs] |  | ≤ 5 |  | ≤ 12 |  | > 12 |  | ≤ 7 |  | > 7 |  | ≤ 7 |  | > 7 |  |
|  |  |  |  | A-MPR [dB] |  | 7 |  | 5 |  | 7 |  | 4 |  | 2 |  | 5 |  | 3 |  |
|  |  | 1651.5 |  | No A-MPR needed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |  |  |  |  |

<<Unchanged text skipped>>

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------