3GPP TSG-RAN WG5 Meeting #92-e	R5-214630

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0159 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.3.24 - Generic Test Procedure for UE intitated MCPTT functional alias status determination and subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to 24.379 clause 9A.2.2.2.5, as long as all functional aliases for a user are deactivated, the notification sent to the user does not contain any functional aliases. - Default message for PIDF content shall be used - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Note added to Table 5.3.24.4-5 - Specific message content removed from Table 5.3.24.4-5 - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test procedure |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.24 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.579-1 CR 0173 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.24	Generic Test Procedure for UE initiated MCPTT functional alias status determination and subscription

5.3.24.1	Initial conditions

As specified in the test case which calls the procedure in its entirety or refers to parts of it.

5.3.24.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.24.3	Procedure

Table 5.3.24.3-1: MCPTT functional alias status determination and subscription

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Make the MCPTT User request to determine the current status of a functional alias and later notification of status changes of a functional alias. (NOTE 1) | - | - | - | - |
| - | EXCEPTION: Step 2a1 describes behaviour that depends on the E-UTRA RRC state at the time the present procedure is called. | - | - | - | - |
| 2a1 | IF in RRC_IDLE state, the E-UTRA/EPC actions which are related to the MCPTT call establishment described in clause 5.4.3 'Generic Test Procedure for MCPTT CO communication in E-UTRA' take place. | - | - | - | - |
| 3 | Check: Does the UE (MCPTT Client) send a SIP SUBSCRIBE requesting the status of any existing functional aliases? | --> | SIP SUBSCRIBE | - | P |
| 4 | The SS (MCPTT server) responds with a SIP 200 (OK) | <-- | SIP 200 (OK) | - | - |
| 5 | The SS (MCPTT server) sends a SIP NOTIFY with functional alias information | <-- | SIP NOTIFY | - | - |
| 6 | Check: Does the UE (MCPTT Client) send a SIP 200 (OK)? | --> | SIP 200 (OK) | - | P |
| - | EXCEPTION: The SS waits 2 seconds before the SS deactivates the dedicated EPS bearer and releases the RRC connection. (NOTE 2) | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI NOTE 2: The specified wait period of 2s shall ensure that lower layer signalling (TCP) is finished. |  |  |  |  |  |

5.3.24.4	Specific message contents

All message contents are as specified in clause 5.5 and in the test case calling the procedure with the following clarifications:

Table 5.3.24.4-1: SIP SUBSCRIBE (step 3, Table 5.3.24.3-1)

| Derivation Path: Table 5.5.2.14-1 with condition MCPTT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Expires |  |  |  |  |
| value | "4294967295" | to receive the current status and later notification | TS 24.379 [9] clause 9A.2.1.3 |  |
| Message-body |  |  | TS 24.379 [9] clause 9A.2.1.3 |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 5.3.24.4-2 |  |  |  |

Table 5.3.24.4-2: MCPTT-Info in SIP SUBSCRIBE (Table 5.3.24.4-1)

| Derivation Path: Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | px_MCPTT_ID_User_A |  | TS 24.379 [9] clause 9A.2.1.3 |  |

Table 5.3.24.4-3: SIP 200 (OK) (step 4, Table 5.3.24.3-1)

Derivation Path: Table 5.5.2.17.1.2-1 with condition SUBSCRIBE-RSP

Table 5.3.24.4-4: SIP NOTIFY (step 5, Table 5.3.24.3-1)

| Derivation Path: Table 5.5.2.8-1 with condition PRESENCE-EVENT |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | PIDF | TS 24.379 [9] clause 9A.2.2.2.5 |  |
| MIME-part-body | PIDF as described in Table 5.3.24.4-5 |  |  |  |

Table 5.3.24.4-5: PIDF in SIP NOTIFY (Table 5.3.24.4-4)

| Derivation Path: Table 5.5.3.5.2-1 (NOTE 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NOTE 1: PIDF document contains tuple with empty <status> element (i.e. there are no <functionalAlias> entries at all) and not containing a <p-id-fa> element |  |  |  |  |