3GPP TSG-RAN WG5 Meeting #92-e	R5-214633

# Online, , 16th August 2021 - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0162 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.3.3 – Generic Test Procedure for MCPTT pre-established session establishment CO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to response C1-213546 to LS R5-211360 support of ICE is mandatory for the client and the server |  |  |  |  |  |  |  |  |
| Summary of change: |  | Editor's note removed from Table 5.3.3.4-1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | C1-213546 is not reflected in test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 36.579-1 CR 0168 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | See also document R5w210210 discussed at TTCN Workshop #54 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.3.3	Generic Test Procedure for MCPTT pre-established session establishment CO

5.3.3.1	Initial conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [6] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document)

IUT:

-	UE (MCPTT client)

-	The UE has performed the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in clause 5.3.2 and thereby the MCPTT client is authorised for and able to use the MCPTT service including making group and private calls on- and off-network, and, the MCPTT user is registered for receiving MCPTT service through the MCPTT Client.

5.3.3.2	Definition of system information messages

The E-UTRA default system information messages as defined in TS 36.508 [6] are used.

5.3.3.3	Procedure

Table 5.3.3.3-1: MCPTT pre-established session establishment CO

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Void | - | - | - | - |
| 1A | E-UTRA/EPC signalling according to steps 2 - 8 of clause 5.4.13 'Generic Test Procedure for MCPTT radio bearer establishment for use of pre-established session' takes place | - | - | - | - |
| 2-7 | Void | - | - | - | - |
| 8 | Check: Does the UE (MCPTT Client) send a SIP INVITE message in order to create a pre-established session? | --> | SIP INVITE | - | P |
| 8A | The SS sends SIP 100 Trying | <-- | SIP 100 Trying | - | - |
| 9 | Void | - | - | - | - |
| 10 | The SS (MCPTT server) responds with a SIP 200 (OK) message. | <-- | SIP 200 (OK) | - | - |
| 10A | Check: Does the UE (MCPTT Client) respond with a SIP ACK message? | --> | SIP ACK | - | P |
| 11 | Void | - | - | - | - |
| 11A | The SS waits 2 seconds to ensure that lower layer signalling (TCP) is finished. | - | - | - | - |
| 12 | The SS transmits an RRCConnectionRelease message. | <-- | RRC: RRCConnectionRelease | - | - |

5.3.3.4	Specific message contents

Table 5.3.3.4-1: SIP INVITE (step 8, Table 5.3.3.3-1)

| Derivation Path: Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  | RFC 3261 [22 RFC 3840 [33] |  |
| feature-param | "+g.3gpp.mcptt" | This media feature tag when used in a SIP request or a SIP response indicates that the function sending the SIP message supports Mission Critical Push To Talk (MCPTT) communication. |  |  |
| feature-param | "audio" | This feature tag indicates that the device supports audio as a streaming media type. |  |  |
| Accept |  |  | RFC 3261 [22] |  |
| media-range[1] | "application/sdp” |  |  |  |
| Accept-Contact |  |  | RFC 3841 [29] |  |
| ac-value[1] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  |  |  |
| req-param | "require" |  |  |  |
| explicit-param | "explicit" |  |  |  |
| Answer-Mode | not present |  |  |  |
| Content-Type |  |  |  |  |
| media-type | "application/sdp" |  |  |  |
| Message-body |  |  |  |  |
| SDP Message | SDP message as described in Table 5.5.3.1.1-1 with conditions PRE_ESTABLISHED, INITIAL_SDP_OFFER |  |  |  |

Table 5.3.3.4-2: SIP 200 (OK) (step 10, Table 5.3.3.3-1)

| Derivation Path: Table 5.5.2.17.1.2-1 with condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Contact |  |  |  |  |
| addr-spec |  |  |  |  |
| user-info and host | tsc_MCX_SessionID_B | The URI that identifies the pre-established session |  |  |