3GPP TSG-RAN WG5 Meeting #92-e	R5-215370

# Electronic Meeting, 16th August - 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0183 | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of General extension payload in Mikey message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Solutions |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 33.180 annex E.6, General extension payload shall be an MCData Protected Payload message as defined in clause 8.5.4 |  |  |  |  |  |  |  |  |
| Summary of change: |  | ‘Key Parameters’ Payload of the MIKEY-SAKKE I_MESSAGE used for GMK distribution by the SS is now encapsulated in MCData Protected Payload message |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE/Clients might reject the GMK if an invalid data (unprotected Key Parameters) is found in General Extention Payload of the MIKEY message instead of an MCData Protected Payload |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 5.5.9.1-3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

##### -	GMK distribution (MIKEY-SAKKE sent by the SS)

Table 5.5.9.1-3: MIKEY-SAKKE I_MESSAGE (GMK distribution by the SS)

| Derivation path: RFC 6509 [23], RFC 6043 [25], RFC 3830 [24] |  |  |  |
| --- | --- | --- | --- |
| Field | Value/remark | Comment | Condition |
| MIKEY Common Header { | Any |  |  |
| version | ‘00000001’B |  |  |
| Data Type | ‘00011010’B | SAKKE msg (26) |  |
| Next payload | ‘00000101’B | Next payload is timestamp |  |
| V | ‘0’B |  |  |
| PRF func | ‘0000001’B | PRF-HMAC-SHA-256 |  |
| CSB ID | GUK-ID: 4 bit purpose tag ('0000'B for GMK) & 28 bit identifier | Group User Key Identifier Derived from GMK-ID and User Salt according to TS 33.180 [94] clause 5,2,3 |  |
| #CS | '00000000'B | no crypto sessions in the CS ID map info. |  |
| CS ID map type | 1 | empty map |  |
| CS ID map Info | Not present |  |  |
| } |  |  |  |
| Timestamp Payload (T) { |  |  |  |
| Next payload | ‘00001011’B | Next payload is RAND |  |
| TS Type | ‘00000000’B | NTP-UTC (0): 64-bits |  |
| TS Value | Current system time | 64bit UTC value representing the number of seconds since 0h on 1 January 1900 with respect to the Coordinated Universal Time (UTC) |  |
| } |  |  |  |
| RAND Payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRi |  |
| RAND len | ‘00010000’B | 16 Bytes RAND |  |
| RAND | 128-bit random number arbitrarily selected by the SS |  |  |
| } |  |  |  |
| IDRi payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRr |  |
| ID Role | 1 | Initiator (IDRi) |  |
| ID Type | 1 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_GMS_Hostname | URI of the group management server |  |
| } |  |  |  |
| IDRr payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRkmsi |  |
| ID Role | 2 | Responder (IDRr) |  |
| ID Type | 1 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | px_MCPTT_ID_User_A | MCPTT ID associated to the group management client |  |
|  | px_MCVideo_ID_User_A | MCVideo ID associated to the group management client | MCVIDEO |
|  | px_MCData_ID_User_A | MCData ID associated to the group management client | MCDATA |
| } |  |  |  |
| IDRkmsi payload { |  |  |  |
| Next payload | ‘00001110’B | Next payload is IDRkmsr |  |
| ID Role | 6 | Initiator's KMS (IDRkmsi) |  |
| ID Type | 1 | URI |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_KMS_Hostname |  |  |
| } |  |  |  |
| IDRkmsr payload { |  |  |  |
| Next payload | ‘00011010’B | Next payload is SAKKE (26) |  |
| ID Role | 7 | Responder's KMS (IDRkmsr) |  |
| ID Type | 1 |  |  |
| ID len | Length of ID Data |  |  |
| ID data | tsc_MCX_KMS_Hostname | KMS of the UE |  |
| } |  |  |  |
| SAKKE payload { |  |  |  |
| Next payload | ‘00010101’B | Next payload is General Extension |  |
| SAKKE params | 1 | Parameter Set 1 according to RFC 6509 [23], Appendix A |  |
| ID Scheme | 2 | '3GPP MCX hashed UID' (33.180 [94] E.1.2) |  |
| SAKKE data length | Length of SAKKE data (in bytes) |  |  |
| SAKKE data | Encapsulated GMK | The GMK is encapsulated by using the SAKKE public key and the UID generated from the MC Service user ID of the group management client (provided in IDRr) |  |
| } |  |  |  |
| General Extension Payload { |  |  |  |
| Next payload | ‘00000100’B | Next payload is SIGN |  |
| Type | 7 | '3GPP key parameters' See 33.180 [94] clause E.6.1 |  |
| ..Length | Length of the data (in bytes) |  |  |
| Content { |  | MCData Protected Payload message according to TS 33.180 [94] clause 8.5.4.1 |  |
| Message Type | 'C3'O | protected and authenticated DATA PAYLOAD |  |
| Date and Time | Same number of seconds as in the Timestamp Payload | UTC time in seconds since midnight UTC of January 1, 1970 |  |
| Payload ID | '00000000'O | value according to TS 33.180 [94] E.6.1 |  |
| Payload sequence number | '00'O | value according to TS 33.180 [94] E.6.1 |  |
| Payload algorithm | '01'O | AEAD_AES_128_GCM |  |
| Signalling algorithm | not present |  |  |
| IV | 'AAAAAAAAAAAAAAAA5555555555555555'O | arbitrarily selected |  |
| DPPK-ID | Same as the CSB ID in the MIKEY Common Header |  |  |
| Payload { |  | ‘Payload' element according to TS 24.282 [87] clause 15.2.13 |  |
| type | '78'O | Value as used in MCData messages in TS 24.282 [87] |  |
| length | length of the payload data |  |  |
| content type | '02'O | BINARY |  |
| Data { | Protected Payload: encrypted with AEAD algorithms | See TS 33.180 [94] clause E.6 and 8.5.4.2 |  |
| Key Type | ‘00000000’B | GMK |  |
| Status | '1' | Not-revoked |  |
| Activation Time | 0 | The time in UTC at which the associated GMK is to be made active for transmission in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). It shall be 5 octets in length. A value of 0 shall imply the activation time is the timestamp of the received MIKEY I_MESSAGE |  |
| Expiry Time | 0 | The 'Expiry time' element shall define the time in UTC at which the associated key shall no longer be used in seconds since midnight UTC of January 1, 1970 (not counting leap seconds). It shall be 5 octets in length. A value of 0 shall imply the key shall not expire. |  |
| Text | "" | no text: Text element shall contain Length sub-element with the value 0 (see TS 33.180 [94] E.6.5) |  |
| Group IDs { |  |  |  |
| Number of Group IDs | '1' |  |  |
| Group ID | px_MCPTT_Group_A_ID | The ID for the group associated with the key. |  |
|  | px_MCVideo_Group_A_ID | The ID for the group associated with the key. | MCVIDEO |
|  | px_MCData_Group_A_ID | The ID for the group associated with the key. | MCDATA |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| MIKEY_SAKKE I-MESSAGE | not present |  |  |
| } |  |  |  |
| } |  |  |  |
| SIGN (ECCSI) payload { |  |  |  |
| S type | 2 | ECCSI signature |  |
| S len | Length of the signature field (in bytes) | 12 bits |  |
| S data | Signature | The signature shall be created according to RFC 3830 [24] clause 5.2 using the algorithm according to RFC 6507 [98] clause 5.2.1 using the UID generated from the identifier associated with the group management server |  |
| } |  |  |  |

<End of modification>