3GPP TSG-RAN5 Meeting #92-e	R5-214181

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-3 | CR | 0140 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to PICS for OTDOA and ECID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test |  |  |  |  |  | Date: |  |  | 2021-07-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For OTDOA the PICS for maxSupportedPrsBandwidth, maxSupportedPrsConfigs and numberOfRXantennas and for ECID the PICS for idleStateForMeasurements are not Boolean values (yes/no) but require values to be supplied. Therefore, the current wording that states “Support of xxx” does not make sense and does not apply. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The wording for the OTDOA PICS for maxSupportedPrsBandwidth, maxSupportedPrsConfigs and numberOfRXantennas and for ECID for idleStateForMeasurements are modified accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PICS will remain confusing and may be completed incorrectly. . |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3	UE Positioning Capabilities

Table A.4.3-1: UTRA UE positioning capabilities

| Item | UTRA UE positioning capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support for IPDL | 25.306, 4.8 | R99 | pc_UE_PositioningIPDL_Sup |  |
| 2 | Support of GPS timing of cell frames | 25.306, 4.8 | R99 | pc_UE_PositioningGPS_TimingOfCellFramesSup |  |
| 3 | Support of UE-based OTDOA | 25.306, 4.8 | R99 | pc_UE_PositioningBasedOTDOA_Sup |  |
| 4 | Support of Standalone location method | 25.306, 4.8 | R99 | pc_UE_PositioningStandaloneLocMethodsSup |  |
| 5 | Support of UE-Based A-GANSS | 25.306, 4.8 | Rel-8 | pc_UEB_A_GANSS | NOTE 1 |
| 6 | Support of UE-Assisted A-GANSS | 25.306, 4.8 | Rel-8 | pc_UEA_A_GANSS | NOTE 1 |
| 7 | Support for GLONASS | 25.306, 4.8 | Rel-8 | pc_GLONASS |  |
| 8 | Support for Modernized GPS | 25.306, 4.8 | Rel-8 | pc_MGPS |  |
| 9 | Support for Galileo | 25.306, 4.8 | Rel-12 | pc_GALILEO | NOTE 2 |
| 10 | Support of UE based Network Assisted GPS L1 C/A | 25.306, 4.8 | R99 | pc_UeBasedAgps |  |
| 11 | Support of UE assisted Network Assisted GPS L1 C/A | 25.306, 4.8 | R99 | pc_UeAssistedAgps |  |
| 12 | Support of Fine Time Assistance | 25.171, 4.4 | Rel-6 |  |  |
| 13 | Support for BDS | 25.306, 4.8 | Rel-12 | pc_BDS |  |
| NOTE 1: If the capability is supported by the UE, then at least one of A.4.3-1/7, A.4.3-1/8, A.4.3-1/9 or A.4.3-1/13 must be supported as well. NOTE 2: Non-backwards compatible changes were made to the Galileo Assistance Data in RRC Rel-12, therefore testing cannot be done for earlier releases. |  |  |  |  |  |

Table A.4.3-2: E-UTRA and NR UE Positioning Capabilities

| Item | E-UTRA and NR UE Positioning Capabilities | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of UE based Assisted-GNSS | 36.355 | Rel-9 | pc_UEB_AGNSS | This implies support of LPP A.4.2-1/1 |
| 2 | Support of UE assisted Assisted-GNSS | 36.355 | Rel-9 | pc_UEA_AGNSS | This implies support of LPP A.4.2-1/1 |
| 3 | Support of GNSS Fine Time Assistance | 36.355, 37.355 | Rel-9 (E-UTRA and NR EN-DC) Rel-15 (NR NG-RAN NR) | pc_GNSS_FTA | This implies support of LPP A.4.2-1/1 |
| 4 | Support of UE assisted OTDOA | 36.355 | Rel-9 | pc_OTDOA | This implies support of LPP A.4.2-1/1 |
| 5 | Support of UE assisted ECID | 36.355 | Rel-9 (FDD) Rel-13 (TDD) NOTE 2 | pc_ECID | This implies support of LPP A.4.2-1/1 |
| 6 | Support for A-GPS L1C/A | 36.355 | Rel-9 | pc_A_GPS_L1C_A | This implies support of LPP A.4.2-1/1 |
| 7 | Support for A-GLONASS | 36.355 | Rel-9 | pc_A_GLONASS | This implies support of LPP A.4.2-1/1 |
| 8 | Support for A-GPS L1C/A and Modernized GPS | 36.355 | Rel-9 | pc_A_GPS_L1C_A_MGPS | This implies support of LPP A.4.2-1/1 |
| 9 | Support for A-Galileo | 36.355 | Rel-12 | pc_A_Galileo | This implies support of LPP A.4.2-1/1. NOTE 1 |
| 10 | Support of UE Fine Time Assistance measurements for UE-based Assisted-GNSS | 36.355, 37.355 | Rel-9 (E-UTRA and NR EN-DC) Rel-15 (NR NG-RAN NR) | pc_GNSS_FTA_UEB | This implies support of LPP A.4.2-1/1 |
| 11 | Support of UE Fine Time Assistance measurements for UE-assisted Assisted-GNSS | 36.355, 37.355 | Rel-9 (E-UTRA and NR EN-DC) Rel-15 (NR NG-RAN NR) | pc_GNSS_FTA_UEA | This implies support of LPP A.4.2-1/1 |
| 12 | Support of GNSS Acquisition Assistance | 36.355; 37.571-2, 5.4.1 | Rel-9 | pc_GNSS_AA | This implies support of LPP A.4.2-1/1 |
| 13 | Support for A-SBAS | 36.355 | Rel-9 | pc_A_SBAS |  |
| 14 | Support for A-QZSS | 36.355 | Rel-9 | pc_A_QZSS |  |
| 15 | Support of UE assisted OTDOA for Carrier Aggregation | 36.355 | Rel-10 | pc_OTDOA_CA | This implies support of LPP A.4.2-1/1 |
| 16 | Support of inter-frequency RSTD measurements that require measurement gaps | 36.355 | Rel-10 | pc_InterFreq_RSTD_withGaps | This implies support of UE assisted OTDOA A.4.3-2/4 |
| 17 | Support of inter-frequency RSTD measurements | 36.355 | Rel-10 | pc_InterFreq_RSTD | This implies support of UE assisted OTDOA A.4.3-2/4 |
| 18 | Support for A-BDS | 36.355 | Rel-12 | pc_A_BDS | This implies support of LPP A.4.2-1/1 (NOTE 3) |
| 19 | Support of UE assisted OTDOA for 3DL Carrier Aggregation | 36.355 | Rel-12 | pc_OTDOA_3DLCA | This implies support of LPP A.4.2-1/1 |
| 20 | Support for UE-Assisted MBS | 36.355 | Rel-13 | pc_UEA_MBS | This implies support of LPP A.4.2-1/1 |
| 21 | Support for UE-Assisted WLAN | 36.355 | Rel-13 | pc_WLAN | This implies support of LPP A.4.2-1/1 |
| 22 | Support for UE-Assisted Bluetooth | 36.355 | Rel-13 | pc_Bluetooth | This implies support of LPP A.4.2-1/1 |
| 23 | Support for UE-Assisted Sensor | 36.355 | Rel-13 | pc_Sens | This implies support of LPP A.4.2-1/1 |
| 24 | No support of periodical reporting for UE based Assisted-GNSS. | 36.355 | Rel-14 | pc_UEB_Noperiodic | This implies support of LPP A.4.2-1/1 |
| 25 | No support of periodical reporting for UE assisted Assisted-GNSS. | 36.355 | Rel-14 | pc_UEA_Noperiodic | This implies support of LPP A.4.2-1/1 |
| 26 | Support for UE-Based MBS | 36.355 | Rel-14 | pc_UEB_MBS | This implies support of LPP A.4.2-1/1 |
| 27 | Support for UE-Based WLAN | 36.355 | Rel-14 | pc_UEB_WLAN | This implies support of LPP A.4.2-1/1 |
| 28 | Support for UE-Based Sensor | 36.355 | Rel-14 | pc_UEB_Sens | This implies support of LPP A.4.2-1/1 |
| 29 | Support for UE-Assisted Multi-RTT | 37.355 | Rel-16 | pc_Multi_RTT | This implies support of LPP A.4.2-1/1 |
| 30 | Support for UE-Assisted DL-AoD | 37.355 | Rel-16 | pc_UEA_DL_AoD | This implies support of LPP A.4.2-1/1 |
| 31 | Support for UE-Based DL-AoD | 37.355 | Rel-16 | pc_UEB_DL_AoD | This implies support of LPP A.4.2-1/1 |
| 32 | Support for UE-Assisted DL-TDOA | 37.355 | Rel-16 | pc_UEA_DL_TDOA | This implies support of LPP A.4.2-1/1 |
| 33 | Support for UE-Based DL-TDOA | 37.355 | Rel-16 | pc_UEB_DL_TDOA | This implies support of LPP A.4.2-1/1 |
| 34 | Support for UE-Assisted NR E-CID | 37.355 | Rel-16 | pc_NR_ECID | This implies support of LPP A.4.2-1/1 |
| NOTE 1: Non-backwards compatible changes were made to the Galileo Assistance Data in LPP Rel-12, therefore testing cannot be done for earlier releases. NOTE 2: For TDD with LPP releases before Rel-13 the UE Rx - Tx time difference measurement report mapping is ambiguous and therefore testing shall not be performed. NOTE 3: If the signal type for BDS supported by the UE includes B1C then Rel-16 of LPP is required. |  |  |  |  |  |

Table A.4.3-3: Supplementary Services

| Item | Supplementary Services | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of EPC-MO-LR request for assistance data | 24.171; 24.030; 24.080 | Rel-9 | pc_EPC_MO_LR_RequestAssistanceData |  |
| 2 | Support of EPC-MO-LR request for a position estimate | 24.171; 24.030; 24.080 | Rel-9 | pc_EPC_MO_LR_RequestPositionEstimate |  |
| 3 | Support of EPC-MT-LR Location Notification | 24.171; 24.030; 24.080 | Rel-9 | pc_MT_LR_loc_notif |  |
| 4 | Support for CS-MO-LR with CS Fallback for a position estimate | 23.272 | Rel-9 | pc_CS_MO_LR_CSFallback |  |
| 5 | Support of MO-LR request for assistance data | 24.030, 5.1.1;24.080, 4.4.3.44;23.171, 8.1.1 | R99 | pc_ParamGpsAssisData | UTRA |
| 6 | Support of MO-LR request for a position estimate | 23.171, 8.1.1 | R99 | pc_ParamPosEstimate | UTRA |
| 7 | Support of MO-LR request for transfer to 3rd party | 23.171, 8.1.1 | R99 | pc_ParamXfer3rdPty | UTRA |
| 8 | Support of MT-LR LCS value added location request notification capability | 24.030;23.271 | R99 | pc_MT_LR | UTRA |

Table A.4.3-3A: OTDOA Measurements

| Item | OTDOA Measurements | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of interFreqRSTDmeasurement | 36.355, 6.5.1.7 | Rel-10 | pc_OTDOA_interFreqRSTDmeasurement |  |
| 2 | Support of additionalNeighbourCellInfoList | 36.355, 6.5.1.7 | Rel-10 | pc_OTDOA_additionalNeighbourCellInfoList |  |
| 3 | Support of prs-id | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prs_id |  |
| 4 | Support of tp-separation-via-muting | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_tp_separation_via_muting |  |
| 5 | Support of additional-prs-config | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_additional_prs_config |  |
| 6 | Support of prs-based-tbs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prs_based_tbs |  |
| 7 | Support of additionalPathsReport | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_additionalPathsReport |  |
| 8 | Support of densePrsConfig | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_densePrsConfig |  |
| 9 | maxSupportedPrsBandwidth | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_maxSupportedPrsBandwidth |  |
| 10 | Support of prsOccGroup | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prsOccGroup |  |
| 11 | Support of prsFrequencyHopping | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prsFrequencyHopping |  |
| 12 | maxSupportedPrsConfigs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_maxSupportedPrsConfigs |  |
| 13 | Support of periodicalReporting | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_periodicalReporting |  |
| 14 | Support of multiPrbNprs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_multiPrbNprs |  |
| 15 | Support of idleStateForMeasurements | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_idleStateForMeasurements |  |
| 16 | numberOfRXantennas | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_numberOfRXantennas |  |
| 17 | Support of motionMeasurements | 37.355, 6.5.1.7 | Rel-15 | pc_OTDOA_motionMeasurements |  |
| 18 | Support of interRAT-RSTDmeasurement | 37.355, 6.5.1.7 | Rel-15 | pc_OTDOA_interRAT_RSTDmeasurement |  |

Table A.4.3-4: E-CID Measurements

| Item | E-CID Measurements | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of RSRP | 36.355, 6.5.3.4 | Rel-9 | pc_ECID_Rsrp |  |
| 2 | Support of RSRQ | 36.355, 6.5.3.4 | Rel-9 | pc_ECID_Rsrq |  |
| 3 | Support of UE Rx-Tx Time Difference | 36.355, 6.5.3.4 | Rel-9 | pc_ECID_UeRxTx |  |
| 4 | Support of ueRxTxSupTDD | 36.355, 6.5.3.4 | Rel-13 | pc_ECID_ueRxTxSupTDD |  |
| 5 | Support of periodicalReporting | 36.355, 6.5.3.4 | Rel-14 | pc_ECID_periodicalReporting |  |
| 6 | Support of triggeredReporting | 36.355, 6.5.3.4 | Rel-14 | pc_ECID_triggeredReporting |  |
| 7 | idleStateForMeasurements required | 36.355, 6.5.3.4 | Rel-14 | pc_ECID_idleStateForMeasurements |  |