3GPP TSG-RAN5 Meeting #92-e	R5-216006

# Online, , 16th Aug 2021 - 27th Aug 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0208 | rev | 1 | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of updated GNSS scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Spirent Communications, Rohde & Schwarz, CATT, CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-07-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. RAN 5 has agreed that, for a number of reasons, the GNSS scenarios need to be updated. These reasons include the need to update the BDS scenarios to align with the latest version of LPP and the RAN 4 specifications, the need to include the possibility of using the BDS B1C signal and the need to update the time(s) of applicability of the scenarios. 2. In addition, a number of (minor) errors have been found: a) The IE IODA is only required for Galileo. For other GNSSs it is not relevant and should be set to “Not Present” b) In sub-clause 6.2, some of the Assistance Data for sub-tests 8 and 10 is missing. c) In clause 6.2.7.4, the GNSS-AuxiliaryInformation for sub-test 8 is incorrect or very confusing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The GNSS scenarios are replaced with new ones for UTRA, E-UTRA and NR with the following improvements. Note that currently the GPS-only scenarios for UTRA are not changed, but this could be done in a future meeting if found necessary by companies. Note that currently the QZSS scenarios are not provided (and are set to “FFS”), but this could be done in a future meeting if found necessary by companies. The new scenarios have the following changes: a) The new scenarios provided use Rinex navigation data format which gives the ephemeris (navigation model) and other useful data. Note however that this does not provide the almanac data which is set to “FFS” at this time. This almanc data will be provided at some future time if found necessary by companies. b) the BDS scenarios comply with recent changes in LPP and the RAN 4 specifications and allow for the use of the B1C signal including new tables to define the BDS satellite types. c) the time of applicability of the scenarios is set to 17th September 2020. 2. To allow for the gradual adoption of this new data and the gradual re-verification and re-validation of test cases, a 24 month ”transition period” is proposed where either this new data or the existing data can be used. 3. To avoid unnecessary duplication of data and likely associated errors, most of the Assistance Data that can be easily derived from the Rinex files has been deleted. 4. The following errors are corrected and editorial changes made: a) Some sub-clauses are now numbered for ease of reference. b) The IE IODA is only required for Galileo. For other GNSSs it is not relevant and is set to “Not Present” c) The missing Assistance Data in sub-clause 6.2 for Sub-tests 8 and 10 is added. d) In clause 6.2.7.4, table GNSS-ID-GPS-SatElement (Modernized GPS): sub-test 4 is in the wrong place and is moved. e) In clause 6.2.7.4, a new table for the GNSS-AuxiliaryInformation for sub-test 8 is added. f) Other minor clarifications made. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. The GNSS scenarios will remain out-of-date and they will become more and more unusable. The BDS scenarios will not align with the latest version of LPP and the RAN 4 specifications and will not be able to use the BDS B1C signal. 2. A number of errors will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 6.2.1, 6.2.1.1, 6.2.1.2.1, 6.2.1.2.2, 6.2.1.2.3.3, 6.2.1.2.4.5, 6.2.1.2.5, 6.2.7.1, 6.2.7.3, 6.2.7.4, Annex B, data files |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Equivalent CR for signalling Instructions to editor: 1. Remove zip file GNSS_Data_Perf_V16.zip from the zip file 2. Add the attached zip file GNSS_Orbital_Data_Perf_V1.zip into the zip file |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Result of one revision of R5-214184 |  |  |  |  |  |  |  |  |

# 4	General

## 4.1	GPS and GNSS orbital model information, assistance data and assistance data files

<<The changes in this section are repeated in the CR for signalling!>>

<< start of changes>>

The following subclauses 5 and 6 define the GPS and GNSS orbital model information, the assistance data and the assistance data files (subclause 5 only) for the test cases as follows:

Subclause 5.1: data for UTRA A-GPS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.1.1 to 6.1.3.

Subclause 5.2: data for UTRA A-GPS Minimum Performance test cases defined in TS 37.571-1 [6] subclause 5.

Subclause 6.1: data for UTRA, E-UTRA and NR A-GNSS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

Subclause 6.2: data for UTRA, E-UTRA and NR A-GNSS Minimum Performance test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

For subclause 5 the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex A.

For subclause 6 the orbital model information is defined and where appropriate is given in Rinex navigation data file format for each scenario in the appropriate data file specified in Annex B.

For subclause 5, where the assistance data is fixed or is not required on a per-satellite basis, then it is defined in the following subclauses. Where assistance data is required on a per-satellite basis, or where the values of the data also vary with time then it is specified in comma-separated-variable files in the appropriate data file specified in Annex A. These files specify the values to be used for each satellite, indexed by satellite PRN or SV ID, and, where applicable, the values to be used indexed by both time and satellite PRN or SV ID.

For subclause 6, the assistance data is defined in the following subclauses.

For the aerial GNSS scenarios defined in TS 36.508 [20], the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex B.

<< next changes>>

## 6.2	GNSS Scenarios and Assistance Data for Assisted GNSS Minimum Performance tests

### 6.2.1	General

This subclause defines the GNSS scenarios and assistance data IEs which shall be available for use as specified in all UTRA, E-UTRA and NR A-GNSS Minimum Performance test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

Subclauses 6.2.2 and 6.2.3 list the assistance data IEs required for minimum performance testing of UE-based mode, and subclauses 6.2.4 and 6.2.5 list the assistance data available for minimum performance testing of UE-assisted mode. Subclause 6.2.7 lists the values of the assistance data IE fields for all minimum performance testing.

In all cases the Assistance Data is given in the two necessary formats, RRC format for TS 37.571-1 [6] subclause 6 and LPP format for TS 37.571-1 [6] subclauses 7 and 13. Other information is also given separately for TS 37.571-1 [6] subclauses 6, 7 and 13 where it differs between the subclauses.

The A-GNSS minimum performance requirements are defined by assuming that all relevant and valid assistance data is received by the UE in order to perform GNSS measurements and/or position calculation. This subclause does not include nor consider delays occurring in the various signalling interfaces of the network.

The term SV ID used in this subclause is defined as the satellite PRN for GPS/Modernized GPS, as Code Number for Galileo, as the satellite Slot Number for GLONASS and as the Ranging Code Number for BDS.

As an alternative, the contents of clause 6.2 in version 16.5.0 of this current specification may be used until September 2023.

#### 6.2.1.1	Satellite constellations and assistance data for A-GNSS minimum performance testing

For all Assisted GNSS minimum performance tests defined in TS 37.571-1 [6] subclause 6 and for all Assisted GNSS minimum performance Sub-Test Case Numbers except Number 1 defined in TS 37.571-1 [6] subclauses 7 and 13, the satellite constellation shall consist of 24 satellites for GLONASS; 27 satellites for GPS/Modernized GPS and Galileo; 5 GEO, 3 IGSO and 27  MEO Satellites for BDS; 3 satellites for QZSS; and 2 satellites for SBAS. Almanac assistance data shall be available for all these satellites. At least 7 of the satellites per GPS/Modernized GPS, Galileo, GLONASS or BDS constellation shall be visible to the UE (that is, above 15 degrees elevation with respect to the UE). At least 1 of the satellites for QZSS shall be within 15 degrees of zenith; and at least 1 of the satellites for SBAS shall be visible to the UE. All other satellite specific assistance data shall be available for all visible satellites. In each test, signals are generated for only 6 satellites (or 7 if SBAS is included). The HDOP for the test shall be calculated using these satellites. The simulated satellites for GPS/Modernized GPS, Galileo, GLONASS and BDS shall be selected from the visible satellites for each constellation, consistent with achieving the required HDOP for the test.

For Assisted GNSS minimum performance Sub-Test Case Number 1 defined in TS 37.571-1 [6] subclauses 7 and 13, the satellite constellations for minimum performance testing shall consist of 24 satellites. Almanac assistance data shall be available for all these 24 satellites. At least 9 of the satellites shall be visible to the UE (that is above 5 degrees elevation with respect to the UE). Other assistance data shall be available for 9 of these visible satellites. In each test, signals are generated for only a sub-set of these satellites for which other assistance data is available. The number of satellites in this sub-set is specified in the test. The satellites in this sub-set shall all be above 15 degrees elevation with respect to the UE. The HDOP for the test shall be calculated using this sub-set of satellites. The selection of satellites for this sub-set shall be selected consistent with achieving the required HDOP for the test.

#### 6.2.1.2	GNSS Scenarios for A-GNSS minimum performance testing

This subclause defines the GNSS scenarios that shall be used for all Assisted GNSS minimum performance tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

The GNSS scenarios achieve the required HDOP for the Test Cases and they also satisfy the requirement that for each test instance the reference location shall change sufficiently such that the UE shall have to use the new assistance data.

The viable running time during which the scenario maintains the required HDOP or HDOPs is given. Once this time has been reached the scenario shall be restarted from its nominal start time.

The test cases include sub-test cases dependent on the GNSS supported by the UE. Each sub-test case is identified by a Sub-Test Case Number as defined below. For each GNSS scenario the parameters that vary with the sub-test are given for each sub-test.

Table 6.2.1.2-1: Sub-Test Case Number Definition for TS 37.571-1 subclause 6

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GLONASS only |
| 2 | UE supporting A-Galileo only |
| 3 | UE supporting A-GPS and Modernized GPS only |
| 4 | UE supporting A-GPS and A-GLONASS only |
| 8 | UE supporting A-GPS and A-Galileo only |
| 9 | UE supporting A-BDS only |
| 10 | UE supporting A-GPS and A-BDS only |

Table 6.2.1.2-2: Sub-Test Case Number Definition for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Supported GNSS |
| --- | --- |
| 1 | UE supporting A-GPS L1 C/A only |
| 2 | UE supporting A-GLONASS only |
| 3 | UE supporting A-Galileo only |
| 4 | UE supporting A-GPS and Modernized GPS only |
| 5 | UE supporting A-GPS and A-GLONASS only (Note) |
| 8 | UE supporting A-GPS and A-Galileo only (Note) |
| 9 | UE supporting A-BDS only |
| 10 | UE supporting A-GPS and A-BDS only (Note) |
| 11 | UE supporting A-GPS and A-GLONASS and A-BDS only (Note) |
| 12 | UE supporting A-GPS and A-Galileo and A-GLONASS only (Note) |
| 13 | UE supporting A-GPS and A-Galileo and A-BDS only (Note) |
| Note: "GPS" here means GPS L1 C/A, Modernized GPS, or both, dependent on UE capabilities. |  |

##### 6.2.1.2.1	GNSS Scenario #1

The following GNSS scenario #1 shall be used during the TTFF tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13 with the exception of the Nominal Accuracy test. The assistance data specified in the following subclauses for GNSS scenario #1 is consistent with this GNSS scenario.

Rinex navigationdata files: the required file(s) in the GNSS orbital data perf zip file specified in Annex B are given below.

Table 6.2.1.2.1-1: Rinex navigationdata files for TS 37.571-1 subclause 6

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 2 | Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 3 | Perf GNSS GPS 2020_9_17 Rinex.txt |
| 4 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Perf GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |

Table 6.2.1.2.1-2: Rinex navigationdata files for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Perf GNSS GPS 2020_9_17 Rinex.txt |
| 2 | Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 3 | Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 4 | Perf GNSS GPS 2020_9_17 Rinex.txt |
| 5 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Perf GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |
| 11 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |
| 12 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 13 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan), height: = 300m.

Nominal start time: 17th September 2020 23:40:00 (GPS time).

Viable running time to maintain specified HDOP values: 19 minutes.

Visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given below.

Table 6.2.1.2.1-3: Visible satellites for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 3, 4, 5, 10, 16, 18, 19, 20 (GLONASS) |
| 2 | 3, 5, 13, 15, 21, 27, 30 (Galileo) |
| 3 | 3, 4, 6, 9, 11, 17, 19, 22, 28 (GPS) |
| 4 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. GLONASS: 3, 4, 5, 10, 16, 18, 19, 20. |
| 8 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. Galileo: 3, 5, 13, 15, 21, 27, 30. |
| 9 | 21, 23, 28, 33, 34, 37, 38, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. BDS: 21, 23, 28, 33, 34, 37, 38, 40, 42, 43, 59, 60. |

Table 6.2.1.2.1-4: Visible satellites for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 3, 4, 6, 9, 11, 17, 19, 22, 28 (GPS) |
| 2 | 3, 4, 5, 10, 16, 18, 19, 20 (GLONASS) |
| 3 | 3, 5, 13, 15, 21, 27, 30 (Galileo) |
| 4 | 3, 4, 6, 9, 11, 17, 19, 22, 28 (GPS) |
| 5 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. GLONASS: 3, 4, 5, 10, 16, 18, 19, 20. |
| 8 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. Galileo: 3, 5, 13, 15, 21, 27, 30. |
| 9 | 21, 23, 28, 33, 34, 37, 38, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. BDS: 21, 23, 28, 33, 34, 37, 38, 40, 42, 43, 59, 60. |
| 11 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. GLONASS: 3, 4, 5, 10, 16, 18, 19, 20. BDS: 21, 23, 28, 33, 34, 37, 38, 40, 42, 43, 59, 60. |
| 12 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. Galileo: 3, 5, 13, 15, 21, 27, 30. GLONASS: 3, 4, 5, 10, 16, 18, 19, 20. |
| 13 | GPS: 3, 4, 6, 9, 11, 17, 19, 22, 28. Galileo: 3, 5, 13, 15, 21, 27, 30. BDS: 21, 23, 28, 33, 34, 37, 38, 40, 42, 43, 59, 60. |

For BDS, the satellite types are given in Table 6.2.1.2.1-4A

Table 6.2.1.2.1-4A: BDS satellite types

| Satellite type | SV IDs of Satellites |
| --- | --- |
| GEO | 59, 60 |
| IGSO | 38, 40 |
| MEO | 21, 23, 28, 33, 34, 37, 42, 43 |

The satellites to be simulated in each sub-test case have been selected in order to achieve the required HDOP. They are defined below.

Table 6.2.1.2.1-5: Satellites to be simulated for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 5, 10, 18, 19 (GLONASS) |
| 2 | 3, 5, 13, 15, 21, 27 (Galileo) |
| 3 | 3, 4, 6, 17, 19, 22 (GPS) |
| 4 | GPS: 3, 4, 28. GLONASS: 5, 18, 19. |
| 8 | GPS: 3, 4, 28. Galileo: 3, 5, 21. |
| 9 | 28, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 28. BDS: 38, 59, 60. |
| Note: The satellite simulator shall generate all the GPS, Galileo and BDS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.1-6: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | Test case dependant. See Table 6.2.1.2.1-7 |
| 2 | 3, 4, 5, 10, 18, 19 (GLONASS) |
| 3 | 3, 5, 13, 15, 21, 27 (Galileo) |
| 4 | 3, 4, 6, 17, 19, 22 (GPS) |
| 5 | GPS: 3, 4, 28. GLONASS: 5, 18, 19. |
| 8 | GPS: 3, 4, 28. Galileo: 3, 5, 21. |
| 9 | 28, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 28. BDS: 38, 59, 60. |
| 11 | GPS: 4, 28. GLONASS: 18, 19. BDS: 38, 59. |
| 12 | GPS: 4, 28. Galileo: 5, 21. GLONASS: 18, 19. |
| 13 | GPS: 4, 28. Galileo: 5, 21. BDS: 38, 59. |
| Note: The satellite simulator shall generate all the GPS, Galileo and BDS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.1-7: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13, sub-test 1

| Test case | SV IDs of Satellites to be simulated |
| --- | --- |
| Sensitivity Coarse Time Assistance | 3, 4, 6, 11, 17, 19, 22, 28 |
| Sensitivity Fine Time Assistance | 3, 4, 6, 11, 17,19, 22, 28 |
| Nominal Accuracy | 3, 4, 6, 11, 17, 19, 22, 28 |
| Dynamic Range | 3, 4, 6, 17, 19, 22 |
| Multi-Path scenario | 3, 4, 6, 17, 22 |

Ionospheric model: see values in subclause 6.2.7.

Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [17].

##### 6.2.1.2.2	GNSS Scenario #2

The following GNSS scenario #2 shall be used during the TTFF tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13 with the exception of the Nominal Accuracy test. The assistance data specified in the following subclauses for GNSS scenario #2 is consistent with this GNSS scenario.

Rinex navigationdata files: the required file(s) in the GNSS orbital data perf zip file specified in Annex B are below.

Table 6.2.1.2.2-1: Rinex navigationdata files for TS 37.571-1 subclause 6

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 2 | Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 3 | Perf GNSS GPS 2020_9_17 Rinex.txt |
| 4 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Perf GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |

Table 6.2.1.2.2-2: Rinex navigationdata files for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Perf GNSS GPS 2020_9_17 Rinex.txt |
| 2 | Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 3 | Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 4 | Perf GNSS GPS 2020_9_17 Rinex.txt |
| 5 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Perf GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |
| 11 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |
| 12 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt and Perf GNSS GLONASS 2020_9_17 Rinex.txt |
| 13 | Perf GNSS GPS 2020_9_17 Rinex.txt and Perf GNSS Galileo 2020_9_17 Rinex.txt and Perf GNSS BDS 2020_9_17 Rinex.txt |

UE location: the UE location is calculated as a random offset from the reference location using the method described in subclause 6.2.1.2.6. The reference location is: latitude: 37 degrees 24 minutes 53.391 seconds north, longitude: 122 degrees 1 minutes 3.722 seconds west, (Sunnyvale, USA), height: = 50m.

Nominal start time: 17th September 2020 23:40:00 (GPS time).

Viable running time to maintain specified HDOP values: 19 minutes.

Visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given below.

Table 6.2.1.2.2-3: Visible satellites for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 2, 3, 11, 12, 13, 17, 18, 19 (GLONASS) |
| 2 | 3, 4, 5, 9, 15, 31, 36 (Galileo) |
| 3 | 1, 3, 4, 10, 11, 21, 22, 25, 31, 32 (GPS) |
| 4 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. GLONASS: 2, 3, 11, 12, 13, 17, 18, 19 |
| 8 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. Galileo: 3, 4, 5, 9, 15, 31, 36. |
| 9 | 21, 22, 26, 34, 36, 42, 43, 44, 45 (BDS) |
| 10 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. BDS: 21, 22, 26, 34, 36, 42, 43, 44, 45 |

Table 6.2.1.2.2-4: Visible satellites for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Visible satellites |
| --- | --- |
| 1 | 1, 3, 4, 11, 21, 22, 25, 31, 32 (GPS) |
| 2 | 2, 3, 11, 12, 13, 17, 18, 19 (GLONASS) |
| 3 | 3, 4, 5, 9, 15, 31, 36 (Galileo) |
| 4 | 1, 3, 4, 10, 11, 21, 22, 25, 31, 32 (GPS) |
| 5 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. GLONASS: 2, 3, 11, 12, 13, 17, 18, 19 |
| 8 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. Galileo: 3, 4, 5, 9, 15, 31, 36. |
| 9 | 21, 22, 26, 34, 36, 42, 43, 44, 45 (BDS) |
| 10 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. BDS: 21, 22, 26, 34, 36, 42, 43, 44, 45 |
| 11 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. GLONASS: 2, 3, 11, 12, 13, 17, 18, 19. BDS: 21, 22, 26, 34, 36, 42, 43, 44, 45. |
| 12 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. Galileo: 1, 3, 4, 5, 9, 15, 31. GLONASS: 2, 3, 11, 12, 13, 17, 18, 19. |
| 13 | GPS: 1, 3, 4, 10, 11, 21, 22, 25, 31, 32. Galileo: 3, 4, 5, 9, 15, 31, 36. BDS: 21, 22, 26, 34, 36, 42, 43, 44, 45. |

For BDS, the satellite types are given in Table 6.2.1.2.2-4A

Table 6.2.1.2.2-4A: BDS satellite types

| Satellite type | SV IDs of Satellites |
| --- | --- |
| GEO |  |
| IGSO |  |
| MEO | 21, 22, 26, 34, 36, 42, 43, 44, 45 |

The satellites to be simulated in each sub-test case have been selected in order to achieve the required HDOP. They are defined below.

Table 6.2.1.2.2-5: Satellites to be simulated for TS 37.571-1 subclause 6

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 2, 3, 12, 13, 17, 18 (GLONASS) |
| 2 | 4, 5, 9, 15, 31, 36 (Galileo) |
| 3 | 1, 3, 4, 11, 21, 22, 31, 32 (GPS) |
| 4 | GPS: 21, 22, 32. GLONASS: 2, 13, 18 |
| 8 | GPS: 21, 22, 32. Galileo: 4, 5, 9. |
| 9 | 21, 34, 42, 43, 44, 45 (BDS) |
| 10 | GPS: 21, 22, 32. BDS: 21, 44, 45. |
| Note: The satellite simulator shall generate all the GPS, Galileo and BDS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.2-6: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | Test case dependant. See Table 6.2.1.2.2-7 |
| 2 | 2, 3, 12, 13, 17, 18 (GLONASS) |
| 3 | 4, 5, 9, 15, 31, 36 (Galileo) |
| 4 | 1, 3, 4, 11, 21, 22, 31, 32 (GPS) |
| 5 | GPS: 21, 22, 32. GLONASS: 2, 13, 18 |
| 8 | GPS: 21, 22, 32. Galileo: 4, 5, 9. |
| 9 | 21, 34, 42, 43, 44, 45 (BDS) |
| 10 | GPS: 21, 22, 32. BDS: 21, 44, 45. |
| 11 | GPS: 21, 32. GLONASS: 13, 18. BDS: 21, 45. |
| 12 | GPS: 21, 32. Galileo: 4, 5. GLONASS: 13, 18. |
| 13 | GPS: 21, 32. Galileo: 4, 5. BDS: 21, 45. |
| Note: The satellite simulator shall generate all the GPS, Galileo and BDS signals supported by the UE for all the simulated satellites. |  |

Table 6.2.1.2.2-7: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13, sub-test 1

| Test case | SV IDs of Satellites to be simulated |
| --- | --- |
| Sensitivity Coarse Time Assistance | 1, 3, 4, 11, 21, 22, 31, 32 |
| Sensitivity Fine Time Assistance | 1, 3, 4, 11, 21, 22, 31, 32 |
| Nominal Accuracy | 1, 3, 4, 11, 21, 22, 31, 32 |
| Dynamic Range | 1, 3, 4, 11, 21, 31 |
| Multi-Path scenario | 1, 3, 11, 21, 31 |

Ionospheric model: see values in subclause 6.2.7.

Tropospheric model: STANAG with SRI equal to 324.8, as defined in STANAG 4294 [17].

##### 6.2.1.2.3	GNSS Scenario #3

The following GNSS scenario #3 shall be used during the Nominal Accuracy test defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #3 is consistent with this GNSS scenario.

The scenario used varies dependent on the SBAS supported by the UE and also whether QZSS is supported. The scenario to be used is defined below. Where more than one SBAS is supported use the scenario for MSAS if MSAS and QZSS are supported, otherwise use the scenario for the first supported SBAS in the list.

Table 6.2.1.2.3-1: Scenarios used for Scenario #3

| SBAS supported | Scenarios used |  |
| --- | --- | --- |
|  | UE supports QZSS | UE does not support QZSS |
| None | GNSS Scenario #1 with QZSS Scenario #1 | GNSS Scenario #1 |
| WAAS | [FFS] | GNSS Scenario #2 with WAAS |
| EGNOS | [FFS] | GNSS Scenario #3A with EGNOS |
| MSAS | GNSS Scenario #1 with QZSS Scenario #1 and MSAS | GNSS Scenario #1 with MSAS |
| GAGAN | [FFS] | GNSS Scenario #3B with GAGAN |

###### 6.2.1.2.3.1	GNSS Scenario #3A

[FFS]

###### 6.2.1.2.3.2	GNSS Scenario #3B

[FFS]

###### 6.2.1.2.3.3	QZSS Scenario #1

Almanac data: FFS.

Ephemeris data: FFS.

Acquisition Assistance: FFS.

UE location: as for GNSS scenario #1.

Nominal start time: as for GNSS scenario #1.

Viable running time to maintain specified requirements: as for GNSS scenario #1.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated: PRN FFS.

###### 6.2.1.2.3.4	WAAS Scenario

Satellite positions: (PRN 135)133.0 degrees west, height: 35786037.417m, (PRN 138)107.3 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 135.

###### 6.2.1.2.3.5	EGNOS Scenario

Satellite positions: (PRN 120)15.5 degrees west, height: 35786037.417m, (PRN 124) 21.5 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 120.

###### 6.2.1.2.3.6	MSAS Scenario

Satellite positions: (PRN 129)140.0 degrees east, height: 35786037.417m, (PRN 137)145 degrees east, height: 35786037.417m

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 129.

###### 6.2.1.2.3.7	GAGAN Scenario

[FFS]

##### 6.2.1.2.4	GNSS Scenario #4

The following GNSS scenario #4 shall be used during the Nominal Accuracy test defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #4 is consistent with this GNSS scenario.

The scenario used varies dependent on the SBAS supported by the UE and also whether QZSS is supported. The scenario to be used is defined below. Where more than one SBAS is supported use the scenario for MSAS if MSAS and QZSS are supported, otherwise use the scenario for the first supported SBAS in the list.

Table 6.2.1.2.4-1: Scenarios used for Scenario #4

| SBAS supported | Scenarios used |  |
| --- | --- | --- |
|  | UE supports QZSS | UE does not support QZSS |
| None | GNSS Scenario #4D with QZSS Scenario #2 | GNSS Scenario #2 |
| WAAS | [FFS] | GNSS Scenario #4C with WAAS |
| EGNOS | [FFS] | GNSS Scenario #4A with EGNOS |
| MSAS | GNSS Scenario #4D with QZSS Scenario #2 and MSAS | GNSS Scenario #4D with MSAS |
| GAGAN | [FFS] | GNSS Scenario #4B with GAGAN |

###### 6.2.1.2.4.1	GNSS Scenario #4A

[FFS]

###### 6.2.1.2.4.2	GNSS Scenario #4B

[FFS]

###### 6.2.1.2.4.3	GNSS Scenario #4C

[FFS]

###### 6.2.1.2.4.4		GNSS Scenario #4D

[FFS]

###### 6.2.1.2.4.5	QZSS Scenario #2

Almanac data: FFS.

UE location: as for GNSS scenario #4D.

Nominal start time: as for GNSS scenario #4D.

Viable running time to maintain specified requirements: as for GNSS scenario #4D.

Satellite meeting specified requirements to be used for simulation and for which Assistance Data (other than Almanac) shall be generated: PRN [FFS].

###### 6.2.1.2.4.6	WAAS Scenario

Satellite positions: (PRN 135)133.0 degrees west, height: 35786037.417m, (PRN 138)107.3 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 138.

###### 6.2.1.2.4.7	EGNOS Scenario

Satellite positions: (PRN 120)15.5 degrees west, height: 35786037.417m, (PRN 124) 21.5 degrees west, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 124.

###### 6.2.1.2.4.8	MSAS Scenario

Satellite positions: (PRN 129)140.0 degrees east, height: 35786037.417m, (PRN 137)145 degrees east, height: 35786037.417m.

UE location: as for related GNSS scenario.

Satellite used for simulation: PRN 137.

###### 6.2.1.2.4.9	GAGAN Scenario

[FFS]

##### 6.2.1.2.5	GNSS Scenario #5

The following GNSS scenario #5 shall be used during the Moving Scenario and Periodic Update test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13. The assistance data specified in the following subclauses for GNSS scenario #5 is consistent with this GNSS scenario.

GNSS scenario #5 is as GNSS scenario #2 except as detailed below.

Table 6.2.1.2.5-1: Void

Table 6.2.1.2.5-2: Void

UE location: the UE location is given as a trajectory as shown in Figure 6.6.1 and Figure 7.1 of TS 37.571-1 [6]. The reference location is at the centre of the trajectory and is as GNSS scenario #2.

Start location: at the point between l11 and l12 in Figure 6.6.1 and Figure 7.1 of TS 37.571-1 [6], going in a clock-wise direction.

Table 6.2.1.2.5-3: Void

Table 6.2.1.2.5-4: Void

Table 6.2.1.2.5-5: Void

Table 6.2.1.2.5-6: Satellites to be simulated for TS 37.571-1 subclauses 7 and 13

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 1, 3, 11, 21, 31 |
| 2 | As Table 6.2.1.2.2-6 |
| 3 | As Table 6.2.1.2.2-6 |
| 4 | As Table 6.2.1.2.2-6 |
| 5 | As Table 6.2.1.2.2-6 |
| 8 | As Table 6.2.1.2.2-6 |
| 9 | As Table 6.2.1.2.2-6 |
| 10 | As Table 6.2.1.2.2-6 |
| 11 | As Table 6.2.1.2.2-6 |
| 12 | As Table 6.2.1.2.2-6 |
| 13 | As Table 6.2.1.2.2-6 |
| Note: The satellite simulator shall generate all the GPS, Galileo and BDS signals supported by the UE for all the simulated satellites. |  |

##### 6.2.1.2.6	UE Location for TTFF test cases

This subclause defines the method for generating the random UE locations that are required to be used for the TTFF tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

For every Test Instance in each TTFF test case, the UE location shall be randomly selected to be within 3 km of the Reference Location. The Altitude of the UE shall be randomly selected between 0 m to 500 m above WGS-84 reference ellipsoid. These values shall have uniform random distributions.

The UE location is calculated as an offset from the Reference Location.

###### 6.2.1.2.6.1	UE Location Offset

The UE location offset shall be calculated by selecting the next pair of random numbers, representing a pair of latitude and longitude offsets in degrees, from a standard uniform random number generator, with the following properties:

The ranges of the latitude and longitude offsets values shall be such that when translated onto the surface of the earth they shall lie within a 3km radius circle, centred on the Reference location specified for the GNSS scenario under consideration. For the purposes of this calculation make the following assumptions:

a)	Over the 3km radius circle at the Reference location the earth is flat and the meridians and parallels form a rectangular grid

b)	The earth is spherical with a radius of 6371141m (equal to the WGS 84 value at 35 degrees latitude)

The resolution used for the latitude and longitude offsets values shall be 90/2E23 for the latitude offset values and 360/2E24 for the longitude offset values, representing the coding resolution in degrees specified in TS 23.032.

###### 6.2.1.2.6.2	UE Altitude

The UE altitude value shall be calculated by selecting the next random number from a standard uniform random number generator, in the range 0 to 500, representing meters. The resolution used for the random number shall be 1, representing 1 meter.

### 6.2.2	Information elements required for normal UE based testing for TS 37.571-1 subclause 6

The following A-GPS and A-GANSS assistance data IEs and fields shall be present for each test as appropriate for the GNSS(s) used during the test. Fields not specified shall not be present. The values of the fields are specified in subclause 6.2.7.

The information elements are given with reference to TS 25.331 [34], where the details are defined.

# a) GPS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GPS Week |  |
| GPS Week Cycle Number | Rel-10 onwards |
| GPS TOW msec |  |
| UE Positioning GPS ReferenceTime Uncertainty |  |
| GPS TOW Assist |  |
| SatID |  |
| TLM Message |  |
| TLM Reserved |  |
| Alert |  |
| Anti-Spoof |  |

# b) GANSS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GANSS Day |  |
| GANSS Day Cycle Number | Rel-10 onwards |
| GANSS TOD |  |
| GANSS TOD Uncertainty |  |
| GANSS Time ID |  |

# c) GANSS Time Model IE This information element is only required for multiGNSS tests.

| Fields of the IE | Release |
| --- | --- |
| GANSS Time Model Reference Time |  |
| TA0 |  |
| GNSS_TOD_ID For each GNSS included in the test. |  |
| Delta_T | Rel-10 onwards |

# d) GPS Reference UE Position IE

Fields of the IE

Ellipsoid point with Altitude and uncertainty ellipsoid

# e) GANSS Reference UE Position IE

Fields of the IE

Ellipsoid point with Altitude and uncertainty ellipsoid

# f) GPS Navigation Model IE

Fields of the IE

All satellite information

# g) GANSS Navigation Model IE

Fields of the IE

All satellite information

| GANSS | Clock and Orbit Model Choice |
| --- | --- |
| Galileo | Model-1 |

# h) GANSS Additional Navigation Model IE

Fields of the IE

All satellite information

| GANSS | Clock and Orbit Model Choice |
| --- | --- |
| Modernized GPS | Model-3 |
| GLONASS | Model-4 |
| QZSS QZS-L1 C/A | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3 |
| SBAS | Model-5 |
| BDS | Model-6 |

# i) GPS Ionospheric Model IE

Fields of the IE

All

# j) GANSS Ionospheric Model IE

Fields of the IE

All

# k) GANSS Additional Ionospheric Model IE

Fields of the IE

All

# l) GPS UTC Model IE

Fields of the IE

All

# m) GANSS Auxiliary Information IE

Fields of the IE

GANSS Auxiliary Information

### 6.2.3	Information elements required for UE based Sensitivity Fine Time Assistance test case for TS 37.571-1 subclause 6

The A-GPS and A-GANSS assistance data IEs and fields that shall be present for the Sensitivity Fine Time Assistance test case shall be those specified in subclause 6.2.2 with the following exception. Fields not specified shall not be present. The values of the fields are specified in subclause 6.2.7.

# a) GPS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GPS Week |  |
| GPS Week Cycle Number | Rel-10 onwards |
| GPS TOW msec |  |
| UTRAN GPS reference time |  |
| UTRAN GPS timing of cell frames |  |
| CHOICE mode |  |
| FDD: Primary CPICH Info |  |
| TDD: cell parameters id |  |
| SFN |  |
| UE Positioning GPS ReferenceTime Uncertainty |  |
| TUTRAN-GPS drift rate |  |
| GPS TOW Assist |  |
| SatID |  |
| TLM Message |  |
| TLM Reserved |  |
| Alert |  |
| Anti spoof |  |

# b) GANSS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GANSS Day |  |
| GANSS Day Cycle Number | Rel-10 onwards |
| GANSS TOD |  |
| GANSS TOD Uncertainty |  |
| GANSS Time ID |  |
| UTRAN GANSS reference time |  |
| UTRAN GANSS timing of cell frames |  |
| CHOICE mode |  |
| FDD: Primary CPICH Info |  |
| TDD: cell parameters id |  |
| SFN |  |
| TUTRAN-GANSS drift rate |  |

### 6.2.4	Information elements available for normal UE assisted testing for TS 37.571-1 subclause 6

The following A-GPS and A-GANSS assistance data IEs and fields shall be present for each test as appropriate for the GNSS(s) used during the test. Fields not specified shall not be present. The values of the fields are specified in subclause 6.2.7.

# a) GPS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GPS Week |  |
| GPS Week Cycle Number | Rel-10 onwards |
| GPS TOW msec |  |
| UE Positioning GPS ReferenceTime Uncertainty |  |
| GPS TOW Assist |  |
| SatID |  |
| TLM Message |  |
| TLM Reserved |  |
| Alert |  |
| Anti-Spoof |  |

# b) GANSS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GANSS Day |  |
| GANSS Day Cycle Number | Rel-10 onwards |
| GANSS TOD |  |
| GANSS TOD Uncertainty |  |
| GANSS Time ID |  |

# c) GANSS Time Model IE This information element is only required for multiGNSS tests.

| Fields of the IE | Release |
| --- | --- |
| GANSS Time Model Reference Time |  |
| TA0 |  |
| GNSS_TOD_ID For each GNSS included in the test. |  |
| Delta_T | Rel-10 onwards |

# d) GPS Reference UE Position IE

Fields of the IE

Ellipsoid point with Altitude and uncertainty ellipsoid

# e) GANSS Reference UE Position IE

Fields of the IE

Ellipsoid point with Altitude and uncertainty ellipsoid

# f) GPS Almanac IE

| Fields of the IE | Release |
| --- | --- |
| Almanac Reference Week |  |
| Complete Almanac Provided | Rel-10 onwards |
| All Satellite information |  |

# g) GANSS Almanac IE

Fields of the IE

GANSS Almanac Model

| GANSS | Almanac Model Choice |
| --- | --- |
| Galileo | Model-1 |
| Modernized GPS | Model-3, 4 |
| GLONASS | Model-5 |
| QZSS QZS-L1 C/A | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3,4 |
| SBAS | Model-6 |
| BDS | Model-7 |

# h) GPS Navigation Model IE

Fields of the IE

All satellite information

# i) GANSS Navigation Model IE

Fields of the IE

All satellite information

| GANSS | Clock and Orbit Model Choice |
| --- | --- |
| Galileo | Model-1 |
| Modernized GPS | Model-3 |
| GLONASS | Model-4 |
| QZSS QZS-L1 C/A | Model-2 |
| QZSS QZS-L1C/L2C/L5 | Model-3 |
| SBAS | Model-5 |
| BDS | Model-6 |

# j) GPS Acquisition Assistance IE

| Fields of the IE | Release |
| --- | --- |
| GPS TOW msec |  |
| UE Positioning GPS ReferenceTime Uncertainty |  |
| Satellite information |  |
| SatID |  |
| Doppler (0th order term) |  |
| Extra Doppler |  |
| Doppler (1st order term) |  |
| Doppler Uncertainty |  |
| Code Phase |  |
| Integer Code Phase |  |
| GPS Bit number |  |
| Code Phase Search Window |  |
| Azimuth and Elevation |  |
| Azimuth |  |
| Elevation |  |
| Azimuth LSB | Rel-10 onwards |
| Elevation LSB | Rel-10 onwards |

# k) GANSS Reference Measurement Information IE

| Fields of the IE | Release |
| --- | --- |
| Satellite information |  |
| SatID |  |
| Doppler (0th order term) |  |
| Extra Doppler |  |
| Doppler (1st order term) |  |
| Doppler Uncertainty |  |
| Code Phase |  |
| Integer Code Phase |  |
| Code Phase Search Window |  |
| Azimuth and Elevation |  |
| Azimuth |  |
| Elevation |  |
| Azimuth LSB | Rel-10 onwards |
| Elevation LSB | Rel-10 onwards |

# l) GANSS Auxiliary Information IE

Fields of the IE

GANSS Auxiliary Information

# m) GPS UTC Model IE

Fields of the IE

GPS UTC Model

### 6.2.5	Information elements available for UE assisted Sensitivity Fine Time Assistance test case for TS 37.571-1 subclause 6

The A-GPS and A-GANSS assistance data IEs and fields that shall be available for use for the Sensitivity Fine Time Assistance test case shall be those specified in subclause 6.2.4 with the following exceptions. Fields not specified shall not be present. The values of the fields are specified in subclause 6.2.7.

# a) GPS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GPS Week |  |
| GPS Week Cycle Number | Rel-10 onwards |
| GPS TOW msec |  |
| UTRAN GPS reference time |  |
| UTRAN GPS timing of cell frames |  |
| CHOICE mode |  |
| FDD: Primary CPICH Info |  |
| TDD: cell parameters id |  |
| SFN |  |
| UE Positioning GPS ReferenceTime Uncertainty |  |
| TUTRAN-GPS drift rate |  |
| GPS TOW Assist |  |
| SatID |  |
| TLM Message |  |
| TLM Reserved |  |

# b) GANSS Reference Time IE

| Fields of the IE | Release |
| --- | --- |
| GANSS Day |  |
| GANSS Day Cycle Number | Rel-10 onwards |
| GANSS TOD |  |
| GANSS TOD Uncertainty |  |
| GANSS Time ID |  |
| UTRAN GANSS timing of cell frames |  |
| CHOICE mode |  |
| FDD: Primary CPICH Info |  |
| TDD: cell parameters id |  |
| SFN |  |
| TUTRAN-GANSS drift rate |  |

# c) GPS Acquisition Assistance IE

| Fields of the IE | Release |
| --- | --- |
| GPS TOW msec |  |
| UTRAN GPS reference time |  |
| UTRAN GPS timing of cell frames |  |
| CHOICE mode |  |
| FDD: Primary CPICH Info |  |
| TDD: cell parameters id |  |
| SFN |  |
| UE Positioning GPS ReferenceTime Uncertainty |  |
| Satellite information |  |
| SatID |  |
| Doppler (0th order term) |  |
| Extra Doppler |  |
| Doppler (1st order term) |  |
| Doppler Uncertainty |  |
| Code Phase |  |
| Integer Code Phase |  |
| GPS Bit number |  |
| Code Phase Search Window |  |
| Azimuth and Elevation |  |
| Azimuth |  |
| Elevation |  |
| Azimuth LSB | Rel-10 onwards |
| Elevation LSB | Rel-10 onwards |

### 6.2.6	Information elements available for A-GNSS test cases in TS 37.571-1 subclauses 7 and 13

The following A-GNSS assistance data elements shall be provided to the UE in the tests. The assistance data provided depends on the mode being used in the test case, the assistance data supported by the UE and the GNSSs supported by the UE. Assistance data IEs not supported by the UE shall not be sent. Assistance data IEs supported by the UE but not listed below shall not be sent. The values of the fields are specified in subclause 6.2.7.

The information elements are given with reference to TS 37.355 [8], where the details are defined.

# Table 6.2.6-1: Assistance Data to be provided to the UE for A-GNSS test cases in TS 37.571-1 subclauses 7 and 13

| Assistance Data IE supported by the UE | Mode used in test case |  |  |
| --- | --- | --- | --- |
|  | UE-based | UE-assisted, GNSS-AcquisitionAssistance supported by the UE | UE-assisted, GNSS-AcquisitionAssistance not supported by the UE |
| GNSS-Reference Time | Yes | Yes | Yes |
| GNSS-ReferenceLocation | Yes | No | Yes |
| GNSS-IonosphericModel | Yes | No | No |
| GNSS-TimeModelList | Yes for sub-tests 5, 8, 10, 11, 12 and 13 | No | Yes for sub-tests 5, 8, 10, 11, 12 and 13 |
| GNSS-NavigationModel | Yes | No | Yes |
| GNSS-AcquisitionAssistance | No | Yes | No |
| GNSS-Almanac | No | No | Yes |
| GNSS-UTC-Model | Yes for sub-tests 5, 11 and 12 | Yes for sub-tests 5, 11 and 12 | Yes for sub-tests 5, 11 and 12 |
| GNSS-AuxiliaryInformation | Yes for sub-tests 2, 5, 11 and 12 (for GLONASS). Yes for sub-test 4 (for multiple GPS signals). Yes for sub-tests 9, 10, 11 and 13 if the UE supports BDS B1C.Yes for sub-tests 5, 8, 10, 11, 12 and 13 if the UE supports multiple GPS signals | Yes for sub-tests 2, 5, 11 and 12 (for GLONASS). Yes for sub-test 4 (for multiple GPS signals). Yes for sub-tests 9, 10, 11 and 13 if the UE supports BDS B1C. Yes for sub-tests 5, 8, 10, 11, 12 and 13 if the UE supports multiple GPS signals | Yes for sub-tests 2, 4, 5, 11 and 12 (for GLONASS). Yes for sub-test 4 (for multiple GPS signals). Yes for sub-tests 9, 10, 11 and 13 if the UE supports BDS B1C. Yes for sub-tests 5, 8, 10, 11, 12 and 13 if the UE supports multiple GPS signals |

# a)	GNSS- Reference Time IE

# GNSS- Reference Time IE

| Information Element | All tests except Sensitivity Fine Time Assistance | Sensitivity Fine Time Assistance test |
| --- | --- | --- |
| GNSS-ReferenceTime |  |  |
| gnss-SystemTime |  |  |
| gnss-TimeID | Yes | Yes |
| gnss-DayNumber | Yes | Yes |
| gnss-TimeOfDay | Yes | Yes |
| gnss-TimeOfDayFrac-msec | Yes | Yes |
| notificationOfLeapSecond | Yes if gnss-TimeID = ‘glonass’ | Yes if gnss-TimeID = ‘glonass’ |
| gps-TOW-Assist | Yes if gnss-TimeID = ‘gps’ | Yes if gnss-TimeID = ‘gps’ |
| referenceTimeUnc | Yes | No |
| gnss-ReferenceTimeForOneCell | No | Yes |
| networkTime |  | Yes |
| secondsFromFrameStructureStart |  | Yes |
| fractionalSecondsFromFrameStructureStart |  | Yes |
| frameDrift |  | Yes |
| cellID |  | Yes |
| physCellId |  | Yes if TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |
| cellGlobalIdEUTRA |  | Yes if TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |
| earfcn/earfcn-v9a0 |  | Yes if TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |
| nrPhysCellId-r15 |  | Yes if TS 37.571-1 subclause 13 Test Configuration B |
| nrCellGlobalID-r15 |  | Yes if TS 37.571-1 subclause 13 Test Configuration B |
| nrARFCN-r15 |  | Yes if TS 37.571-1 subclause 13 Test Configuration B |
| referenceTimeUnc |  | Yes |

# b)	GNSS-ReferenceLocation IE

# GNSS-ReferenceLocation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-ReferenceLocation | threeDlocation |

# c)	GNSS-IonosphericModel IE

# GNSS-IonosphericModel IE

| Name of the IE |  | Fields of the IE |  |
| --- | --- | --- | --- |
| GNSS-IonosphericModel |  | KlobucharModelParameter(3) |  |
|  |  | KlobucharModel2Parameter(2) |  |
|  |  | NeQuickModelParameter(1) |  |
| Note 1: Only required if GNSSs supported include Galileo. Note 2: Only required if GNSSs supported include BDS B1C. Note 3: In the case of BDS, only required if BDS B1I supported. |  |  |  |

# d)	GNSS-TimeModelList IE This information element is only required for multiGNSS tests.

# GNSS-TimeModelList IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-TimeModelList |  |
|  | gnss-TO-ID For each GNSS included in the test. |
|  | deltaT |

# e)	GNSS-NavigationModel IE

# GNSS-NavigationModel IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-NavigationModel |  |

# GNSS Clock and Orbit Model Choices

| GNSS |  | Clock and Orbit Model Choice |  |
| --- | --- | --- | --- |
| GPS L1 C/A |  | Model-2 |  |
| Modernized GPS |  | Model-3 |  |
| GLONASS |  | Model-4 |  |
| QZSS QZS-L1 C/A |  | Model-2 |  |
| QZSS QZS-L1C/L2C/L5 |  | Model-3 |  |
| SBAS |  | Model-5 |  |
| Galileo |  | Model-1 |  |
| BDS B1I |  | Model-6 |  |
| BDS B1C |  | Model-7 |  |

# f)	GNSS-AcquisitionAssistance IE

# GNSS-AcquisitionAssistance IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AcquisitionAssistance |  |

# g)	GNSS-Almanac IE

# GNSS-Almanac IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-Almanac |  |

# GNSS Almanac Choices

| GNSS |  | Almanac Model Choice |  |
| --- | --- | --- | --- |
| GPS L1 C/A |  | Model-2 |  |
| Modernized GPS |  | Model-3,4 |  |
| GLONASS |  | Model-5 |  |
| QZSS QZS-L1 C/A |  | Model-2 |  |
| QZSS QZS-L1C/L2C/L5 |  | Model-3,4 |  |
| SBAS |  | Model-6 |  |
| Galileo |  | Model-1 |  |
| BDS B1I |  | Model-7 |  |
| BDS B1C |  | Model-3, 4 |  |

# h)	GNSS-UTC-Model IE

# GNSS-UTC-Model IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-UTC-Model |  |

# GNSS UTC Model Choices

| GNSS |  | UTC Model Choice |  |
| --- | --- | --- | --- |
| GPS L1 C/A |  | Model-1 |  |
| Modernized GPS |  | Model-2 |  |
| GLONASS |  | Model-3 |  |
| QZSS QZS-L1 C/A |  | Model-1 |  |
| QZSS QZS-L1C/L2C/L5 |  | Model-2 |  |
| SBAS |  | Model-4 |  |
| Galileo |  | Model-1 |  |
| BDS B1I |  | Model-5 |  |
| BDS B1C |  | Model-2 |  |

# i)	GNSS-AuxiliaryInformation IE

# GNSS-AuxiliaryInformation IE

| Name of the IE | Fields of the IE |
| --- | --- |
| GNSS-AuxiliaryInformation |  |

### 6.2.7	Contents of Information elements for A-GNSS Minimum performance testing

#### 6.2.7.1	General

This subclause defines the assistance data values that shall be used for all Assisted GNSS minimum performance tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13. It is given for GNSS scenarios #1, #2, #3, #4 and #5 and QZSS Scenarios #1 and #2, where it is different for each scenario; otherwise it is marked “All” where the same value is used for all scenarios.

Assistance data that is marked as “time varying” is created and used in 80ms increments.

Assistance data Information Elements and fields that are not specified shall not be used.

#### 6.2.7.2	IE Random Offset Values

This subclause defines the methods for generating the random offsets that are required to be applied to some assistance data IEs for certain tests defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

##### 6.2.7.2.1	GNSS TOW

For every Test Instance in each TTFF test case, the IE GPS TOW msec or GANSS TOD or gnss-TimeofDay plus gnss-TimeofDayFrac-msec shall have a random offset, relative to GNSS system time, within the allowed error range of Coarse Time Assistance defined in the test case. This offset value shall have a uniform random distribution.

The offset value shall be calculated by selecting the next random number from a standard uniform random number generator, in the range specified for the GNSS Coarse Time assistance error range in the Test Requirements, Test parameters table for the test under consideration. The resolution used for the random number shall be 0.01, representing 10ms.

##### 6.2.7.2.2	GNSS/cellular time offset

In addition, for every Fine Time Assistance Test Instance the IE UTRAN GPS timing of cell frames or the UTRAN GANSS timing of cell frames or fractionalSecondsFromFrameStructureStart shall have a random offset, relative to the true value of the relationship between the two time references, within the allowed error range of Fine Time Assistance defined in the test case. This offset value shall have a uniform random distribution.

The offset value shall be calculated by selecting the next random number from a standard uniform random number generator with the following properties:

For UTRAN GPS timing of cell frames the range shall be the number of UMTS chips whose duration is less than the range specified for the GNSS Fine Time assistance error range in the Test Requirements, Test parameters table for the test under consideration. For UTRAN GANSS timing of cell frames or fractionalSecondsFromFrameStructureStart the range shall be the range specified for the GNSS Fine Time assistance error range in the Test Requirements, Test parameters table for the test under consideration.

For UTRAN GPS timing of cell frames the resolution used for the random number shall be 1, representing 1 UMTS bit. For UTRAN GANSS timing of cell frames or fractionalSecondsFromFrameStructureStart the resolution used for the random number shall be 1us.

#### 6.2.7.3	Contents of Information elements for A-GNSS Minimum performance testing in TS 37.571-1 subclause 6

##### 6.2.7.3.1	Assistance Data Reference Time

Contents of UE positioning GPS reference time (sub-tests 3, 4, 8 and 10)

Reference Time (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GPS Week | Weeks | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| GPS Week Cycle Number (Rel-10 onwards) |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| GPS TOW msec | msec | Start time derived from data in clause 6.2.1.2. Add number of ms as required. (Note 1) | Start time derived from data in clause 6.2.1.2. Add number of ms as required. (Note 1) | Start time derived from data in clause 6.2.1.2. Add number of ms as required. (Note 1) |
| UTRAN GPS reference time |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| UTRAN GPS timing of cell frames |  | Note 2 | Note 2 | - |
| CHOICE mode |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | - |
| FDD: Primary CPICH Info |  | 100 | 100 | - |
| TDD: cell parameters id |  | 0 | 0 |  |
| SFN |  | Note 2 | Note 2 | - |
| UE Positioning GPS ReferenceTime Uncertainty |  | For Sensitivity Fine Time Assistance test case: ‘51’ (10.2uS). Otherwise: ‘125’ (2.127s) | For Sensitivity Fine Time Assistance test case: ‘51’ (10.2uS). Otherwise: ‘125’ (2.127s) | ‘125’ (2.127s) |
| TUTRAN-GPS drift rate |  | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| Note 1: GPS TOW msec This is the value in ms of GPS TOW msec when the GPS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GPS scenario is used, the GPS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of GPS TOW msec to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of GPS TOW msec as described in subclause 6.2.7.2. Note 2: UTRAN GPS timing of cell frames and SFN. The values of UTRAN GPS timing of cell frames (before the addition of the random offset) and SFN shall be calculated at the time the IE is required. The accuracy of the relationship between the two fields shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. A random offset is then added to the value of UTRAN GPS timing of cell frames as described in subclause 6.2.7.2. |  |  |  |  |

# Satellite Information

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 9 | 10 | 10 |

Reference Time - GPS TOW Assist (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

Reference Time - GPS TOW Assist (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| TLM Message | Bit string | Derived from data in clause 6.2.1.2 |
| TLM Reserved | Bit string | Derived from data in clause 6.2.1.2 |
| Alert | Boolean | 0 |
| Anti-Spoof | Boolean | 1 |

Contents of UE positioning GANSS reference time (sub-tests 1, 2, and 9)

# GANSS reference time: sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GANSS Day | days | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| GANSS Day Cycle Number (Rel-10 onwards) |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| GANSS TOD | seconds | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) | 125 (2.127 seconds) | 125 (2.127 seconds) |
| GANSS Time ID |  | 2 (GLONASS) | 2 (GLONASS) | 2 (GLONASS) |
| UTRAN GANSS reference time |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| UTRAN GANSS timing of cell frames |  | Note 2 | Note 2 | - |
| CHOICE mode |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | - |
| FDD: Primary CPICH Info |  | 100 | 100 | - |
| TDD: cell parameters id |  | 0 | 0 |  |
| SFN |  | Note 2 | Note 2 | - |
| TUTRAN-GANSS drift rate |  | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| Note 1: GANSS TOD This is the value in seconds of GANSS TOD when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of GANSS TOD to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1[6], shall be met. For all TTFF test cases a random offset is then added to the value of GANSS TOD as described in subclause 6.2.7.2. Note 2: UTRAN GANSS timing of cell frames and SFN. The values of UTRAN GANSS timing of cell frames (before the addition of the random offset) and SFN shall be calculated at the time the IE is required. The accuracy of the relationship between the two fields shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. A random offset is then added to the value of UTRAN GPS timing of cell frames as described in subclause 6.2.7.2. |  |  |  |  |

# GANSS reference time: sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |  |
| --- | --- | --- | --- | --- | --- |
| GANSS Day | days | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |  |
| GANSS Day Cycle Number (Rel-10 onwards) |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |  |
| GANSS TOD | seconds | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) | 125 (2.127 seconds) | 125 (2.127 seconds) |  |
| GANSS Time ID |  | Not present (Galileo) | Not present (Galileo) | Not present (Galileo) |  |
| UTRAN GANSS reference time |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |  |
| UTRAN GANSS timing of cell frames |  | Note 2 | Note 2 | - |  |
| CHOICE mode |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | - |  |
| FDD: Primary CPICH Info |  | 100 | 100 | - |  |
| TDD: cell parameters id |  | 0 | 0 |  |  |
| SFN |  | Note 2 | Note 2 | - |  |
| TUTRAN-GANSS drift rate |  | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |  |
| Note 1: GANSS TOD This is the value in seconds of GANSS TOD when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of GANSS TOD to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of GANSS TOD as described in subclause 6.2.7.2. Note 2: UTRAN GANSS timing of cell frames and SFN. The values of UTRAN GANSS timing of cell frames (before the addition of the random offset) and SFN shall be calculated at the time the IE is required. The accuracy of the relationship between the two fields shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. A random offset is then added to the value of UTRAN GPS timing of cell frames as described in subclause 6.2.7.2. |  |  |  |  |  |

# GANSS reference time: sub-test 9

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |  |
| --- | --- | --- | --- | --- | --- |
| GANSS Day | days | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |  |
| GANSS Day Cycle Number (Rel-10 onwards) |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |  |
| GANSS TOD | seconds | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |  |
| GANSS TOD Uncertainty |  | 125 (2.127 seconds) | 125 (2.127 seconds) | 125 (2.127 seconds) |  |
| GANSS Time ID |  | 3 (BDS system time) | 3 (BDS system time) | 3 (BDS system time) |  |
| UTRAN GANSS reference time |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |  |
| UTRAN GANSS timing of cell frames |  | Note 2 | Note 2 | - |  |
| CHOICE mode |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | - |  |
| FDD: Primary CPICH Info |  | 100 | 100 | - |  |
| TDD: cell parameters id |  | 0 | 0 |  |  |
| SFN |  | Note 2 | Note 2 | - |  |
| TUTRAN-GANSS drift rate |  | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | 0. Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |  |
| Note 1: GANSS TOD This is the value in seconds of GANSS TOD when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of GANSS TOD to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of GANSS TOD as described in subclause 6.2.7.2. Note 2: UTRAN GANSS timing of cell frames and SFN. The values of UTRAN GANSS timing of cell frames (before the addition of the random offset) and SFN shall be calculated at the time the IE is required. The accuracy of the relationship between the two fields shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. A random offset is then added to the value of UTRAN GPS timing of cell frames as described in subclause 6.2.7.2. |  |  |  |  |  |

##### 6.2.7.3.2	Assistance Data Time Model

Contents of UE positioning GANSS time model (sub-test 4)

# GANSS time model

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GANSS Time Model Reference Time |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| TA0 | seconds | 0 | 0 | 0 |
| GNSS_TOD_ID |  | 0 (GPS) | 0 (GPS) | 0 (GPS) |
| Delta_T (Rel-10 onwards) | seconds | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

Contents of UE positioning GANSS time model (sub-tests 8 and 10)

# GANSS time model: sub-test 8

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GANSS Time Model Reference Time |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| TA0 | seconds | 0 | 0 | 0 |
| GNSS_TOD_ID |  | 0 (GPS) | 0 (GPS) | 0 (GPS) |
| Delta_T (Rel-10 onwards) | seconds | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS time model: sub-test 10

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GANSS Time Model Reference Time |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| TA0 | seconds | 0 | 0 | 0 |
| GNSS_TOD_ID |  | 0 (GPS) | 0 (GPS) | 0 (GPS) |
| Delta_T (Rel-10 onwards) | seconds | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

##### 6.2.7.3.3	Assistance Data Reference UE Position

Contents of UE positioning GPS reference UE position (sub-tests 3, 4, 8 and 10)

# GPS reference UE position

Derived from data in clause 6.2.1.2 and the following information:

Uncertainty of the semi-major axis: 3 km.

Uncertainty of the semi-minor axis: 3 km.

Orientation of the major axis: 0 degrees.

Uncertainty of the altitude information: 500 m.

Confidence factor: 68%.

Contents of UE positioning GANSS reference UE position (sub-tests 1, 2 and 9)

# GANSS reference UE position

Derived from data in clause 6.2.1.2 and the following information:

Uncertainty of the semi-major axis: 3 km.

Uncertainty of the semi-minor axis: 3 km.

Orientation of the major axis: 0 degrees.

Uncertainty of the altitude information: 500 m.

Confidence factor: 68%.

##### 6.2.7.3.4	Assistance Data Navigation Model

Contents of UE positioning GPS navigation model (sub-tests 3, 4, 8 and 10)

# Satellite Information

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 9 | 10 | 10 |

# GPS Navigation Model (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| Satellite Status | Boolean | 0 | 0 | 0 |

# GPS Ephemeris and Clock Correction parameters (Fields occurring once per satellite)

Derived from data in clause 6.2.1.2

Contents of UE positioning GANSS navigation model (sub-tests 2 and 8)

# GANSS navigation model

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Non-Broadcast Indication | - | Not present |

# Satellite Information

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 7 | 7 | 7 |

# GANSS navigation model (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| SV Health |  | 0 | 0 | 0 |
| IOD |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS Clock Model (Fields occurring once per satellite)

Galileo Satellite clock model (“Model 1”)

Derived from data in clause 6.2.1.2

# GANSS Orbit Model (Fields occurring once per satellite)

Galileo orbit model: Keplerian Parameters (“Model 1”)

Derived from data in clause 6.2.1.2

Contents of UE positioning GANSS additional navigation models (sub-tests 1 and 4)

# GANSS additional navigation models

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Non-Broadcast Indication | - | Not present |

# Satellite Information

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 8 | 8 | 8 |

# GANSS additional navigation models (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| SV Health |  | 000000 | 000000 | 000000 |
| IOD |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS additional clock models (Fields occurring once per satellite)

GLONASS Satellite Clock Model (“Model 4”)

Derived from data in clause 6.2.1.2

# GANSS additional orbit models (Fields occurring once per satellite)

GLONASS Earth-Centered, Earth-fixed Parameters (“Model 4”)

Derived from data in clause 6.2.1.2

Contents of UE positioning GANSS additional navigation model (sub-tests 9 and 10)

# GANSS additional navigation model

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Non-Broadcast Indication | - | Not present |

# Satellite Information

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 12 | 9 | 9 |

# GANSS additional navigation model (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| SV Health |  | 0 | 0 | 0 |
| IOD |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS additional Clock Model (Fields occurring once per satellite)

BDS Satellite clock model (“Model 6”)

Derived from data in clause 6.2.1.2

# GANSS additional Orbit Model (Fields occurring once per satellite)

BDS orbit model: BDS Keplerian Parameters (“Model 6”)

Derived from data in clause 6.2.1.2

##### 6.2.7.3.5	Assistance Data Ionospheric Model

# Contents of UE positioning GPS ionospheric model (sub-tests 3 and 4)

# GPS ionospheric model

Derived from data in clause 6.2.1.2

Contents of UE positioning GANSS ionospheric model (sub-tests 2 and 8)

# GANSS ionospheric model

Derived from data in clause 6.2.1.2

Contents of UE positioning GANSS additional ionospheric model (sub-tests 1, 9 and 10)

# GANSS additional ionospheric model (QZSS)

Derived from data in clause 6.2.1.2 and the following information:

DataId: 00

# GANSS additional ionospheric model (BDS)

Derived from data in clause 6.2.1.2 and the following information:

DataId: 01

##### 6.2.7.3.6	Assistance Data Almanac

Contents of UE positioning GPS almanac (sub-tests 3, 4, 8 and 10)

# GPS Almanac (Field occurring once per message)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| WNa | Weeks | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| Complete Almanac Provided (Rel-10 onwards) |  | TRUE | TRUE | TRUE |

# Satellite Information

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Number of satellites | - | 27 |

# GPS Almanac (Fields occurring once per satellite)

FFS

Contents of UE positioning GANSS almanac (sub-tests 1, 2, 4, 8, 9, and 10)

# GANSS almanac: sub-tests 1, 4 (Field occurring once per message)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Week Number | Weeks | N/A | N/A | N/A |
| Complete Almanac Provided (Rel-10 onwards) |  | TRUE | TRUE | TRUE |

# Satellite Information GLO-KP: sub-tests 1 and 4

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Number of satellites | - | 24 |

# GANSS almanac: sub-tests 1 and 4 (Fields occurring once per satellite)

GLONASS Keplerian Parameters (“Model 5”)

FFS

# GANSS almanac: sub-tests 2 and 8 (Field occurring once per message)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Week Number | Weeks | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| Complete Almanac Provided (Rel-10 onwards) |  | TRUE | TRUE | TRUE |

# GANSS almanac: sub-tests 2 and 8 (Field occurring once per message)

Galileo Keplerian Parameters (“Model 1”)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| IODa |  | 0 | 0 | 0 |

# Satellite Information KP: sub-tests 2 and 8

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Number of satellites | - | 27 |

# GANSS almanac: sub-tests 2 and 8 (Fields occurring once per satellite)

Galileo Keplerian Parameters (“Model 1”)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SV ID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS almanac: sub-tests 2 and 8 (Fields occurring once per satellite)

Galileo Keplerian Parameters (“Model 1”)

FFS

# GANSS almanac: sub-tests 9 and 10 (Field occurring once per message)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Week Number | Weeks | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| Complete Almanac Provided (Rel-10 onwards) |  | TRUE | TRUE | TRUE |

# GANSS almanac: sub-tests 9 and 10 (Field occurring once per message)

BDS Keplerian Parameters (“Model 7”)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| IODa |  | Not present | Not present | Not present |

# Satellite Information KP: sub-tests 9 and 10

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| Number of satellites | - | 35 |

# GANSS almanac: sub-tests 9 and 10 (Fields occurring once per satellite)

BDS Keplerian Parameters (“Model 7”)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SV ID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS almanac: sub-tests 9 and 10 (Fields occurring once per satellite)

BDS Keplerian Parameters (“Model 7”)

##### FFS

##### 6.2.7.3.7	Assistance Data UTC Model

Contents of UE positioning GPS UTC model (sub-test 4)

# GPS UTC model

Derived from data in clause 6.2.1.2 and the following information:

A1: 0

A0: 0

##### 6.2.7.3.8	Assistance Data Acquisition Assistance and Reference Measurement Information

Contents of UE positioning GPS acquisition assistance (sub-tests 3, 4, 8 and 10)

# GPS Acquisition Assistance (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GPS TOW msec | msec | Start time derived from data in clause 6.2.1.2. Add number of ms as required. (Note 1) | Start time derived from data in clause 6.2.1.2. Add number of ms as required. (Note 1) | Start time derived from data in clause 6.2.1.2. Add number of ms as required. (Note 1) |
| UTRAN GPS reference time |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| UTRAN GPS timing of cell frames |  | Note 2 | Note 2 | - |
| CHOICE mode |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | - |
| FDD: Primary CPICH Info |  | 100 | 100 | - |
| TDD: cell parameters id |  | 0 | 0 |  |
| SFN |  | Note 2 | Note 2 | - |
| UE Positioning GPS ReferenceTime Uncertainty |  | For Sensitivity Fine Time Assistance test case: ‘51’ (10.2uS). Otherwise: ‘125’ (2.127s) | For Sensitivity Fine Time Assistance test case: ‘51’ (10.2uS). Otherwise: ‘125’ (2.127s) | ‘125’ (2.127s) |
| Note 1: GPS TOW msec This is the value in ms of GPS TOW msec when the GPS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GPS scenario is used, the GPS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of GPS TOW msec to be used in the Acquisition Assistance IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of GPS TOW msec as described in subclause 6.2.7.2. This “final GPS TOW msec” value is then also used to determine the value of the Acquisition Assistance Information Elements marked as “Time varying”. Note 2: UTRAN GPS timing of cell frames and SFN The values of UTRAN GPS timing of cell frames (before the addition of the random offset) and SFN shall be calculated at the time the IE is required. The accuracy of the relationship between the two fields shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. A random offset is then added to the value of UTRAN GPS timing of cell frames as described in subclause 6.2.7.2 |  |  |  |  |

# Satellite Information

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 9 | 10 | 10 |

# GPS Acquisition Assistance (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GPS Acquisition Assistance (Fields occurring once per satellite)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

Contents of UE positioning GANSS reference measurement information (sub-tests 1, 2, 4, 8, 9, and 10)

# GANSS reference measurement information: sub-tests 1 and 4 (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information: sub-tests 1 and 4

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 8 | 8 | 8 |

# GANSS reference measurement information: sub-tests 1 and 4 (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS reference measurement information: sub-tests 1 and 4 (Fields occurring once per satellite)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GANSS reference measurement information: sub-tests 2 and 8 (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information: sub-tests 2 and 8

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 7 | 7 | 7 |

# GANSS reference measurement information: sub-tests 2 and 8 (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS reference measurement information: sub-tests 2 and 8 (Fields occurring once per satellite)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GANSS reference measurement information: sub-tests 9 and 10 (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS Signal ID |  | Not present |

# Satellite Information: sub-tests 9 and 10

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 12 | 9 | 9 |

# GANSS reference measurement information: sub-tests 9 and 10 (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS reference measurement information: sub-tests 9 and 10 (Fields occurring once per satellite)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

##### 6.2.7.3.9	Assistance Data Auxiliary Information

Contents of UE positioning GANSS auxiliary information (sub-tests 1, 3, 4, 8 and 10)

# GANSS auxiliary information: sub-tests 1 and 4 (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS-ID-3 |  | Present (GLONASS) |

# Aux Info List: sub-tests 1 and 4

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 8 | 8 | 8 |

# GANSS auxiliary information: sub-tests 1 and 4 (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| Signals Available | - | 10000000 (G1) | 10000000 (G1) | 10000000 (G1) |
| Channel number | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GANSS auxiliary information: sub-tests 3, 4, 8 and 10 (Fields occurring once per message)

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS-ID-1 |  | Sub-test 3: present (Modernized GPS), sub-tests 4, 8 and 10 if the UE supports multiple GPS signals: present (Modernized GPS) |

# Aux Info List: sub-tests 3, 4, 8 and 10

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| Number of satellites | - | 9 | 10 | 10 |

# GANSS auxiliary information: sub-tests 3, 4, 8 and 10 (Fields occurring once per satellite)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| SatID | - | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| Signals Available | - | As supported by the UE | As supported by the UE | As supported by the UE |

##### 6.2.7.3.10	Assistance Data GANSS ID

Contents of GANSS ID (sub-tests 1, 2, 3, 4, 8, 9 and 10)

# GANSS ID: sub-tests 1 and 4

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS ID |  | 3 (GLONASS) |

# GANSS ID: sub-tests 2 and 8

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS ID |  | Not present (Galileo) |

# GANSS ID: sub-test 3

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS ID |  | 1 (Modernized GPS) |

# GANSS ID: sub-tests 9 and 10

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GANSS ID |  | 4 (BDS) |

#### 6.2.7.4	Contents of Information elements for A-GNSS Minimum performance testing in TS 37.571-1 subclauses 7 and 13

##### 6.2.7.4.1	GNSS REFERENCE TIME:

# GNSS-ReferenceTime (GPS): sub-tests 1, 4, 5, 8, 10, 11, 12 and 13

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-SystemTime |  |  |  |  |
| gnss-TimeID |  | 0 (gps) | 0 (gps) | 0 (gps) |
| gnss-DayNumber | days | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| gnss-TimeOfDay | s | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |
| gnss-TimeOfDayFrac-msec | ms | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
| notificationOfLeapSecond |  | Not present | Not present | Not present |
| gps-TOW-Assist |  |  |  |  |
| satelliteID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| tlmWord |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| antiSpoof |  | 1 (for all PRNs) | 1 (for all PRNs) | 1 (for all PRNs) |
| alert |  | 0 (for all PRNs) | 0 (for all PRNs) | 0 (for all PRNs) |
| tlmRsvdBits |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| GNSS-ReferenceTimeForOneCell |  |  |  |  |
| networkTime |  |  |  |  |
| secondsFromFrameStructureStart | s | Note 2 | Note 2 |  |
| fractionalSecondsFromFrameStructureStart | 250ns | Note 2 | Note 2 |  |
| frameDrift |  | 0 | 0 |  |
| cellID |  |  |  |  |
| CHOICE eUTRA |  | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |  |
| physCellId |  | 0 | 0 |  |
| cellGlobalIdEUTRA |  | ‘0000 0000’B | ‘0000 0000’B |  |
| earfcn/earfcn-v9a0 |  | Note 3 | Note 3 |  |
| CHOICE nr-r15 |  | For TS 37.571-1 subclause 13 Test Configuration B | For TS 37.571-1 subclause 13 Test Configuration B |  |
| nrPhysCellId-r15 |  | 0 | 0 |  |
| nrCellGlobalID-r15 |  | ‘0000 0000’B | ‘0000 0000’B |  |
| nrARFCN-r15 |  | Note 4 | Note 4 |  |
| referenceTimeUnc |  | ‘24’ (11.11us) | ‘24’ (11.11us) |  |
| Note 1: gnss-TimeOfDay and gnss-TimeOfDayFrac-msec. This is the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec as described in subclause 6.2.7.2. Note 2: secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart. The values of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart (before the addition of the random offset) shall be calculated at the time the IE is required. The accuracy of the values used shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of 37.571-1 [6], shall be met. A random offset is then added to the value of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart as described in subclause 6.2.7.2. Note 3: earfcn/earfcn-v9a0 is defined in TS 36.508 [20] subclause 4.3.1 for the frequency band under test (see TS 37.571-1 [6] subclause 4.4.1) Note 4: nrARFCN is defined in TS 38.508-1 [24] subclause 6.2.3 for the frequency band under test (see TS 37.571-1 [6] subclause 4.12.1) |  |  |  |  |

# GNSS-ReferenceTime (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-SystemTime |  |  |  |  |
| gnss-TimeID |  | 4 (glonass) | 4 (glonass) | 4 (glonass) |
| gnss-DayNumber | days | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| gnss-TimeOfDay | s | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |
| gnss-TimeOfDayFrac-msec | ms | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
| notificationOfLeapSecond |  | 00 | 00 | 00 |
| gps-TOW-Assist |  | Not present | Not present | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| GNSS-ReferenceTimeForOneCell |  |  |  |  |
| networkTime |  |  |  |  |
| secondsFromFrameStructureStart | s | Note 2 | Note 2 |  |
| fractionalSecondsFromFrameStructureStart | 250ns | Note 2 | Note 2 |  |
| frameDrift |  | 0 | 0 |  |
| cellID |  |  |  |  |
| CHOICE eUTRA |  | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |  |
| physCellId |  | 0 | 0 |  |
| cellGlobalIdEUTRA |  | ‘0000 0000’B | ‘0000 0000’B |  |
| earfcn/earfcn-v9a0 |  | Note 3 | Note 3 |  |
| CHOICE nr-r15 |  | For TS 37.571-1 subclause 13 Test Configuration B | For TS 37.571-1 subclause 13 Test Configuration B |  |
| nrPhysCellId-r15 |  | 0 | 0 |  |
| nrCellGlobalID-r15 |  | ‘0000 0000’B | ‘0000 0000’B |  |
| nrARFCN-r15 |  | Note 4 | Note 4 |  |
| referenceTimeUnc |  | ‘24’ (11.11us) | ‘24’ (11.11us) |  |
| Note 1: gnss-TimeOfDay and gnss-TimeOfDayFrac-msec This is the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec as described in subclause 6.2.7.2. Note 2: secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart. The values of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart (before the addition of the random offset) shall be calculated at the time the IE is required. The accuracy of the values used shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of 37.571-1 [6], shall be met. A random offset is then added to the value of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart as described in subclause 6.2.7.2. Note 3: earfcn/earfcn-v9a0 is defined in TS 36.508 [20] subclause 4.3.1 for the frequency band under test (see TS 37.571-1 [6] subclause 4.4.1) Note 4: nrARFCN is defined in TS 38.508-1 [24] subclause 6.2.3 for the frequency band under test (see TS 37.571-1 [6] subclause 4.12.1) |  |  |  |  |

# GNSS-ReferenceTime (Galileo): sub-test 3

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-SystemTime |  |  |  |  |
| gnss-TimeID |  | 3 (galileo) | 3 (galileo) | 3 (galileo) |
| gnss-DayNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| gnss-TimeOfDay |  | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |
| gnss-TimeOfDayFrac-msec |  | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
| notificationOfLeapSecond |  | Not present | Not present | Not present |
| gps-TOW-Assist |  | Not present | Not present | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| GNSS-ReferenceTimeForOneCell |  |  |  |  |
| networkTime |  |  |  |  |
| secondsFromFrameStructureStart | s | Note 2 | Note 2 |  |
| fractionalSecondsFromFrameStructureStart | 250ns | Note 2 | Note 2 |  |
| frameDrift |  | 0 | 0 |  |
| cellID |  |  |  |  |
| CHOICE eUTRA |  | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |  |
| physCellId |  | 0 | 0 |  |
| cellGlobalIdEUTRA |  | ‘0000 0000’B | ‘0000 0000’B |  |
| earfcn/earfcn-v9a0 |  | Note 3 | Note 3 |  |
| CHOICE nr-r15 |  | For TS 37.571-1 subclause 13 Test Configuration B | For TS 37.571-1 subclause 13 Test Configuration B |  |
| nrPhysCellId-r15 |  | 0 | 0 |  |
| nrCellGlobalID-r15 |  | ‘0000 0000’B | ‘0000 0000’B |  |
| nrARFCN-r15 |  | Note 4 | Note 4 |  |
| referenceTimeUnc |  | ‘24’ (11.11us) | ‘24’ (11.11us) |  |
| Note 1: gnss-TimeOfDay and gnss-TimeOfDayFrac-msec This is the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec as described in subclause 6.2.7.2. Note 2: secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart. The values of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart (before the addition of the random offset) shall be calculated at the time the IE is required. The accuracy of the values used shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of 37.571-1 [6], shall be met. A random offset is then added to the value of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart as described in subclause 6.2.7.2. Note 3: earfcn/earfcn-v9a0 is defined in TS 36.508 [20] subclause 4.3.1 for the frequency band under test (see TS 37.571-1 [6] subclause 4.4.1) Note 4: nrARFCN is defined in TS 38.508-1 [24] subclause 6.2.3 for the frequency band under test (see TS 37.571-1 [6] subclause 4.12.1) |  |  |  |  |

# GNSS-ReferenceTime (BDS): sub-test 9

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-SystemTime |  |  |  |  |
| gnss-TimeID |  | 5 (bds) | 5 (bds) | 5 (bds) |
| gnss-DayNumber | days | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| gnss-TimeOfDay | s | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) | Start time derived from data in clause 6.2.1.2. (Note 1) |
| gnss-TimeOfDayFrac-msec | ms | 0 (Note 1) | 0 (Note 1) | 0 (Note 1) |
| notificationOfLeapSecond |  | Not present | Not present | Not present |
| gps-TOW-Assist |  | Not present | Not present | Not present |
| referenceTimeUnc |  | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) Absent for Sensitivity Fine Time Assistance test case. Present otherwise | ‘117’ (2.274 seconds) |
| gnss-ReferenceTimeForCells |  | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Present for Sensitivity Fine Time Assistance test case. Absent otherwise | Absent |
| GNSS-ReferenceTimeForOneCell |  |  |  |  |
| networkTime |  |  |  |  |
| secondsFromFrameStructureStart | s | Note 2 | Note 2 |  |
| fractionalSecondsFromFrameStructureStart | 250ns | Note 2 | Note 2 |  |
| frameDrift |  | 0 | 0 |  |
| cellID |  |  |  |  |
| CHOICE eUTRA |  | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A | For TS 37.571-1 subclause 7, or subclause 13 Test Configuration A |  |
| physCellId |  | 0 | 0 |  |
| cellGlobalIdEUTRA |  | ‘0000 0000’B | ‘0000 0000’B |  |
| earfcn/earfcn-v9a0 |  | Note 3 | Note 3 |  |
| CHOICE nr-r15 |  | For TS 37.571-1 subclause 13 Test Configuration B | For TS 37.571-1 subclause 13 Test Configuration B |  |
| nrPhysCellId-r15 |  | 0 | 0 |  |
| nrCellGlobalID-r15 |  | ‘0000 0000’B | ‘0000 0000’B |  |
| nrARFCN-r15 |  | Note 4 | Note 4 |  |
| referenceTimeUnc |  | ‘24’ (11.11us) | ‘24’ (11.11us) |  |
| Note 1: gnss-TimeOfDay and gnss-TimeOfDayFrac-msec. This is the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec when the GNSS scenario is initially started in the GNSS simulator. For all TTFF test cases, each time a GNSS scenario is used, the GNSS start time shall be advanced by 120 seconds from the value last used so that, at the time the fix is made, it is at least 2 minutes later than the previous fix made with that scenario. The actual value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec to be used in the Reference Time IE (before the addition of the random offset, if applicable) shall be calculated at the time the IE is required by adding the elapsed time since the time the scenario was started in the GNSS simulator to this value. The accuracy shall be such that the Maximum Test System Uncertainty for Coarse Time Assistance, specified in Table C.1.2 of TS 37.571-1 [6], shall be met. For all TTFF test cases a random offset is then added to the value of gnss-TimeOfDay and gnss-TimeOfDayFrac-msec as described in subclause 6.2.7.2. Note 2: secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart. The values of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart (before the addition of the random offset) shall be calculated at the time the IE is required. The accuracy of the values used shall be such that the Maximum Test System Uncertainty for Fine Time Assistance, specified in Table C.1.2 of 37.571-1 [6], shall be met. A random offset is then added to the value of secondsFromFrameStructureStart and fractionalSecondsFromFrameStructureStart as described in subclause 6.2.7.2. Note 3: earfcn/earfcn-v9a0 is defined in TS 36.508 [20] subclause 4.3.1 for the frequency band under test (see TS 37.571-1 [6] subclause 4.4.1) Note 4: nrARFCN is defined in TS 38.508-1 [24] subclause 6.2.3 for the frequency band under test (see TS 37.571-1 [6] subclause 4.12.1) |  |  |  |  |

##### 6.2.7.4.2	GNSS REFERENCE LOCATION:

# GNSS-ReferenceLocation

Derived from data in clause 6.2.1.2 and the following information:

Uncertainty of the semi-major axis: 3 km.

Uncertainty of the semi-minor axis: 3 km.

Orientation of the major axis: 0 degrees.

Uncertainty of the altitude information: 500 m.

Confidence factor: 68%.

##### 6.2.7.4.3	GNSS IONOSPHERIC MODEL:

# GNSS-IonosphericModel (Klobuchar Model): sub-tests 1, 2, 4 and 5

Derived from data in clause 6.2.1.2 and the following information:

dataID: 00

neQuickModel: not present

klobucharModel2: not present

# GNSS-IonosphericModel (NeQuick Model): sub-test 3

Derived from data in clause 6.2.1.2 and the following information:

klobucharModel: not present

klobucharModel2: not present

# GNSS-IonosphericModel (Klobuchar2 Model)

Derived from data in clause 6.2.1.2 and the following information:

klobucharModel: not present

neQuickModel: not present

# GNSS-IonosphericModel: sub-tests 8 and 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-IonosphericModel |  |  |
| klobucharModel |  | See values for GNSS-IonosphericModel (Klobuchar) |
| neQuickModel |  | See values for GNSS-IonosphericModel (NeQuick) |
| klobucharModel2 |  | Not present |

# GNSS-IonosphericModel: sub-tests 9, 10 and 11

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-IonosphericModel |  |  |
| klobucharModel |  | If BDS B1I supported. See values for GNSS-IonosphericModel (Klobuchar) |
| neQuickModel |  | Not present |
| klobucharModel2 |  | If BDS B1C supported. See values for GNSS-IonosphericModel (Klobuchar2) |

# GNSS-IonosphericModel: sub-test 13

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-IonosphericModel |  |  |
| klobucharModel |  | If BDS B1I supported. See values for GNSS-IonosphericModel (Klobuchar) |
| neQuickModel |  | See values for GNSS-IonosphericModel (NeQuick) |
| klobucharModel2 |  | If BDS B1C supported. See values for GNSS-IonosphericModel (Klobuchar2) |

##### 6.2.7.4.4	GNSS TIME MODEL LIST:

# GNSS-TimeModelList (GPS – GLONASS): sub-test 5

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-TimeModelRefTime |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| tA0 |  | 0 | 0 | 0 |
| gnss-TO-ID |  | 1 (GPS) | 1 (GPS) | 1 (GPS) |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| deltaT |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-TimeModelList (GPS – Galileo): sub-test 8

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-TimeModelRefTime |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| tA0 |  | 0 | 0 | 0 |
| gnss-TO-ID |  | 1 (GPS) | 1 (GPS) | 1 (GPS) |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| deltaT |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-TimeModelList (GPS – BDS): sub-test 10

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| gnss-TimeModelRefTime |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| tA0 |  | 0 | 0 | 0 |
| gnss-TO-ID |  | 1 (GPS) | 1 (GPS) | 1 (GPS) |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| deltaT |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-TimeModelList: sub-test 11

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 4 (glonass) |
| GNSS-TimeModelList |  | See GNSS-TimeModelList (GPS – GLONASS) |
| gnss-ID |  | 5 (bds) |
| GNSS-TimeModelList |  | See GNSS-TimeModelList (GPS – BDS) |

# GNSS-TimeModelList: sub-test 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 3 (galileo) |
| GNSS- TimeModelList |  | See GNSS-TimeModelList (GPS – Galileo) |
| gnss-ID |  | 4 (glonass) |
| GNSS- TimeModelList |  | See GNSS-TimeModelList (GPS – GLONASS) |

# GNSS-TimeModelList: sub-test 13

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 3 (galileo) |
| GNSS- TimeModelList |  | See GNSS-TimeModelList (GPS – Galileo) |
| gnss-ID |  | 5 (bds) |
| GNSS- TimeModelList |  | See GNSS-TimeModelList (GPS – BDS) |

##### 6.2.7.4.5	GNSS NAVIGATION MODEL:

# GNSS-NavigationModel (GPS L1 C/A only): sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 9 | (SIZE) 9 | (SIZE) 9 |

# GNSS-NavModelSatelliteElement (GPS L1 C/A only): sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 0 | 0 | 0 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (GPS L1 C/A only): sub-test 1

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: nav-ClockModel, Model-2

GNSS-OrbitModel: nav-KeplerianSet, Model-2

# GNSS-NavigationModel (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 8 | (SIZE) 8 | (SIZE) 8 |

# GNSS-NavModelSatelliteElement (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 00000000 | 00000000 | 00000000 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (GLONASS): sub-test 2

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: glonass-ClockModel, Model-4

GNSS-OrbitModel: glonass-ECEF, Model-4

# GNSS-NavigationModel (Galileo): sub-test 3

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 7 | (SIZE) 7 | (SIZE) 7 |

# GNSS-NavModelSatelliteElement (Galileo): sub-test 3

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 0 | 0 | 0 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (Galileo): sub-test 3

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: standardClockModelList, Model-1.

standardClockModelList: (SIZE) 1 if the UE supports only Galileo E1, (SIZE) 2 if the UE supports multiple Galileo signals.

StandardClockModelElement (I/NAV):

- stanClockTgd: Not present if the UE supports multiple Galileo signals.

- stanModelID: 0 (I/NAV). Present only if the UE supports multiple Galileo signals

StandardClockModelElement (F/NAV): Present only if the UE supports multiple Galileo signals

- stanClockTgd: Not present

- stanModelID: 1 (F/NAV)

GNSS-OrbitModel: keplerianSet, Model-1

# GNSS-NavigationModel: sub-test 4

The GNSS-NavigationModel(s) to be used depends on the GNSS-NavigationModel(s) supported by the UE. The allowed NavigationModels are as follows:

GNSS-NavigationModel (GPS)

GNSS-NavigationModel (Modernized GPS)

# GNSS-NavigationModel (Modernized GPS)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 9 | (SIZE) 10 | (SIZE) 10 |

# GNSS-NavModelSatelliteElement (Modernized GPS)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 0 | 0 | 0 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (Modernized GPS)

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: cnav-ClockModel, Model-3

GNSS-OrbitModel: cnav-KeplerianSet, Model-3

# GNSS-NavigationModel: sub-test 5

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GPS) and/or GNSS-NavigationModel (Modernized GPS) depending on GNSS-NavigationModel supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GLONASS) |

# GNSS-NavigationModel (GPS)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 9 | (SIZE) 10 | (SIZE) 10 |

# GNSS-NavModelSatelliteElement (GPS)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 0 | 0 | 0 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (GPS)

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: nav-ClockModel, Model-2

GNSS-OrbitModel: nav-KeplerianSet, Model-2

# GNSS-NavigationModel: sub-test 8

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GPS) and/or GNSS-NavigationModel (Modernized GPS) depending on GNSS-NavigationModel supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (Galileo) |

# GNSS-NavigationModel: sub-test 9

The GNSS-NavigationModel(s) to be used depends on the GNSS-NavigationModel(s) supported by the UE. The allowed NavigationModels are as follows:

GNSS-NavigationModel (BDS B1I)

GNSS-NavigationModel (BDS B1C)

# GNSS-NavigationModel (BDS B1I)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 12 | (SIZE) 9 | (SIZE) 9 |

# GNSS-NavModelSatelliteElement (BDS B1I)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 0 | 0 | 0 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (BDS B1I)

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: BDS-ClockModel-r12, Model-6

GNSS-OrbitModel: BDS-KeplerianSet-r12, Model-6

# GNSS-NavigationModel (BDS B1C)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| nonBroadcastFlag |  | 0 | 0 | 0 |
| gnss-SatelliteList |  | (SIZE) 12 | (SIZE) 9 | (SIZE) 9 |

# GNSS-NavModelSatelliteElement (BDS B1C)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| svHealth |  | 0 | 0 | 0 |
| iod |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-NavModelSatelliteElement (BDS B1C)

Derived from data in clause 6.2.1.2 and the following information:

GNSS-ClockModel: bds-ClockModel2-r16, Model-7

GNSS-OrbitModel: bds-KeplerianSet2-r16, Model-7

# GNSS-NavigationModel: sub-test 10

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GPS) and/or GNSS-NavigationModel (Modernized GPS) depending on GNSS-NavigationModel supported by the UE |
| gnss-ID |  | 5 (bds) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (BDS B1I) and/or GNSS-NavigationModel (BDS B1C) depending on GNSS-NavigationModel supported by the UE |

# GNSS-NavigationModel: sub-test 11

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GPS) and/or GNSS-NavigationModel (Modernized GPS) depending on GNSS-NavigationModel supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GLONASS) |
| gnss-ID |  | 5 (bds) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (BDS B1I) and/or GNSS-NavigationModel (BDS B1C) depending on GNSS-NavigationModel supported by the UE |

# GNSS-NavigationModel: sub-test 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GPS) and/or GNSS-NavigationModel (Modernized GPS) depending on GNSS-NavigationModel supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (Galileo) |
| gnss-ID |  | 4 (glonass) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GLONASS) |

# GNSS-NavigationModel: sub-test 13

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (GPS) and/or GNSS-NavigationModel (Modernized GPS) depending on GNSS-NavigationModel supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (Galileo) |
| gnss-ID |  | 5 (bds) |
| GNSS-NavigationModel |  | See GNSS-NavigationModel (BDS B1I) and/or GNSS-NavigationModel (BDS B1C) depending on GNSS-NavigationModel supported by the UE |

##### 6.2.7.4.6	GNSS ACQUISITION ASSISTANCE:

# GNSS-AcquisitionAssistance (GPS L1 C/A only): sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |  |  |
| gnss-SignalID |  | 0 (GPS L1 C/A) | 0 (GPS L1 C/A | 0 (GPS L1 C/A) |
| gnss-AcquisitionAssistList |  | (SIZE) 9 | (SIZE) 9 | (SIZE) 9 |
| confidence-r10 | % | 98 | 98 | 98 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A only): sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A only): sub-test 1

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |  |  |
| gnss-SignalID |  | 0 (GLONASS G1) | 0 (GLONASS G1) | 0 (GLONASS G1) |
| gnss-AcquisitionAssistList |  | (SIZE) 8 | (SIZE) 8 | (SIZE) 8 |
| confidence-r10 | % | 98 | 98 | 98 |

# GNSS-AcquisitionAssistElement (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (GLONASS): sub-test 2

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

GNSS-AcquisitionAssistance: sub-test 3

The GNSS-AcquisitionAssistance(s) to be used depends on the GNSS-AcquisitionAssistance(s) supported by the UE. The allowed GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (Galileo E1)

GNSS-AcquisitionAssistance (Galileo E5A)

Data for other Galileo signals are FFS

# GNSS-AcquisitionAssistance (Galileo E1)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |  |  |
| gnss-SignalID |  | 0 (Galileo E1) | 0 (Galileo E1) | 0 (Galileo E1) |
| gnss-AcquisitionAssistList |  | (SIZE) 7 | (SIZE) 7 | (SIZE) 7 |
| confidence-r10 | % | 98 | 98 | 98 |

# GNSS-AcquisitionAssistElement (Galileo E1)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (Galileo E1)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance (Galileo E5A)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |  |  |
| gnss-SignalID |  | 1 (Galileo E5A) | 1 (Galileo E5A) | 1 (Galileo E5A) |
| gnss-AcquisitionAssistList |  | (SIZE) 7 | (SIZE) 7 | (SIZE) 7 |
| confidence-r10 | % | 98 | 98 | 98 |

# GNSS-AcquisitionAssistElement (Galileo E5A)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (Galileo E5A)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance: sub-test 4

The GNSS-AcquisitionAssistance(s) to be used depends on the GNSS-AcquisitionAssistance(s) supported by the UE. The allowed GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (GPS L1 C/A)

GNSS-AcquisitionAssistance (Modernized GPS L5)

Data for other GPS signals are FFS

# GNSS-AcquisitionAssistance (Modernized GPS L5)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |  |  |
| gnss-SignalID |  | 3 (GPS L5) | 3 (GPS L5) | 3 (GPS L5) |
| gnss-AcquisitionAssistList |  | (SIZE) 9 | (SIZE) 10 | (SIZE) 10 |
| confidence-r10 | % | 98 | 98 | 98 |

# GNSS-AcquisitionAssistElement (Modernized GPS L5)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (Modernized GPS L5)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance: sub-test 5

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GPS L1 C/A) and/or GNSS-AcquisitionAssistance (Modernized GPS L5) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GLONASS) |

# GNSS-AcquisitionAssistance (GPS L1 C/A)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AcquisitionAssistance |  |  |  |  |
| gnss-SignalID |  | 0 (GPS L1 C/A) | 0 (GPS L1 C/A | 0 (GPS L1 C/A) |
| gnss-AcquisitionAssistList |  | (SIZE) 9 | (SIZE) 10 | (SIZE) 10 |
| confidence-r10 | % | 98 | 98 | 98 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (GPS L1 C/A)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance: sub-test 8

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GPS L1 C/A) and/or GNSS-AcquisitionAssistance (Modernized GPS L5) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (Galileo E1) and/or GNSS-AcquisitionAssistance (Galileo E5A) depending on GNSS-AcquisitionAssistance supported by the UE |

# GNSS-AcquisitionAssistance: sub-test 9

The GNSS-AcquisitionAssistance(s) to be used depends on the GNSS-AcquisitionAssistance(s) supported by the UE. The allowed GNSS-AcquisitionAssistances are as follows:

GNSS-AcquisitionAssistance (BDS B1I)

GNSS-AcquisitionAssistance (BDS B1C)

# GNSS-AcquisitionAssistElement (BDS B1I)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (BDS B1I)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistElement (BDS B1C)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AcquisitionAssistElement (BDS B1C)

These fields are time varying (see clause 6.2.7.1) and are derived from data in clause 6.2.1.2 and the following information:

Doppler uncertainty: 40 m/s

Code Phase Search Window: derived for each satellite using a 3 km radius UE position uncertainty

# GNSS-AcquisitionAssistance: sub-test 10

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GPS L1 C/A) and/or GNSS-AcquisitionAssistance (Modernized GPS L5) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 5 (bds) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (BDS B1I) and/or GNSS-AcquisitionAssistance (BDS B1C) depending on GNSS-AcquisitionAssistance supported by the UE |

# GNSS-AcquisitionAssistance: sub-test 11

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GPS L1 C/A) and/or GNSS-AcquisitionAssistance (Modernized GPS L5) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GLONASS) |
| gnss-ID |  | 5 (bds) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (BDS B1I) and/or GNSS-AcquisitionAssistance (BDS B1C) depending on GNSS-AcquisitionAssistance supported by the UE |

# GNSS-AcquisitionAssistance: sub-test 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GPS L1 C/A) and/or GNSS-AcquisitionAssistance (Modernized GPS L5) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (Galileo E1) and/or GNSS-AcquisitionAssistance (Galileo E5A) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GLONASS) |

# GNSS-AcquisitionAssistance: sub-test 13

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (GPS L1 C/A) and/or GNSS-AcquisitionAssistance (Modernized GPS L5) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (Galileo E1) and/or GNSS-AcquisitionAssistance (Galileo E5A) depending on GNSS-AcquisitionAssistance supported by the UE |
| gnss-ID |  | 5 (bds) |
| GNSS-AcquisitionAssistance |  | See GNSS-AcquisitionAssistance (BDS B1I) and/or GNSS-AcquisitionAssistance (BDS B1C) depending on GNSS-AcquisitionAssistance supported by the UE |

##### 6.2.7.4.7	GNSS ALMANAC:

# GNSS-Almanac (GPS L1 C/A only): sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 24 | (SIZE) 24 | (SIZE) 24 |

# GNSS-AlmanacElement (GPS L1 C/A only): sub-test 1

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianNAV-Almanac |  | Model-2 | Model-2 | Model-2 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (GPS L1 C/A only): sub-test 1

FFS

# GNSS-Almanac (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 24 | (SIZE) 24 | (SIZE) 24 |

# GNSS-AlmanacElement (GLONASS): sub-test 2

FFS

GNSS-AlmanacElement: keplerianGLONASS (Model-5)

# GNSS-Almanac (Galileo): sub-test 3

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | 0 | 0 | 0 |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 27 | (SIZE) 27 | (SIZE) 27 |

# GNSS-Almanac (Galileo): sub-test 3

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianAlmanacSet |  | Model-1 | Model-1 | Model-1 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (Galileo): sub-test 3

FFS

kepSV-StatusFNAV: 0. Present only if the UE supports multiple Galileo signals.

# GNSS-Almanac: sub-test 4

The GNSS-Almanac(s) to be used depends on the GNSS-Almanac(s) supported by the UE. The allowed GNSS-Almanacs are as follows:

GNSS-Almanac (GPS)

GNSS-Almanac (Modernized GPS Reduced)

GNSS-Almanac (Modernized GPS Midi)

# GNSS-Almanac (Modernized GPS Reduced)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 27 | (SIZE) 27 | (SIZE) 27 |

# GNSS-AlmanacElement (Modernized GPS Reduced)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianReducedAlmanac |  | Model-3 | Model-3 | Model-3 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (Modernized GPS Reduced)

FFS

# GNSS-Almanac (Modernized GPS Midi)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 27 | (SIZE) 27 | (SIZE) 27 |

# GNSS-AlmanacElement (Modernized GPS Midi)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianMidiAlmanac |  | Model-4 | Model-4 | Model-4 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (Modernized GPS Midi)

FFS

# GNSS-Almanac: sub-test 5

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-Almanac |  | See GNSS-Almanac (GPS) and/or GNSS-Almanac (Modernized GPS Reduced) and/or GNSS-Almanac (Modernized GPS Midi) depending on GNSS-Almanac supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-Almanac |  | See GNSS-Almanac (GLONASS) |

# GNSS-Almanac (GPS)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 27 | (SIZE) 27 | (SIZE) 27 |

# GNSS-AlmanacElement (GPS)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianNAV-Almanac |  | Model-2 | Model-2 | Model-2 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (GPS)

FFS

# GNSS-Almanac: sub-test 8

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-Almanac |  | See GNSS-Almanac (GPS) and/or GNSS-Almanac (Modernized GPS Reduced) and/or GNSS-Almanac (Modernized GPS Midi) depending on GNSS-Almanac supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-Almanac |  | See GNSS-Almanac (Galileo) |

# GNSS-Almanac: sub-test 9

The GNSS-Almanac(s) to be used depends on the GNSS-Almanac(s) supported by the UE. The allowed GNSS-Almanacs are as follows:

GNSS-Almanac (BDS B1I)

GNSS-Almanac (BDS B1C Reduced)

GNSS-Almanac (BDS B1C Midi)

# GNSS-Almanac (BDS B1I)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 35 | (SIZE) 35 | (SIZE) 35 |

# GNSS-AlmanacElement (BDS B1I)

FFS

GNSS-AlmanacElement: BDS-AlmanacSet-r12 (Model-7)

# GNSS-Almanac (BDS B1C Reduced)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 35 | (SIZE) 35 | (SIZE) 35 |

# GNSS-AlmanacElement (BDS B1C Reduced)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianReducedAlmanac |  | Model-3 | Model-3 | Model-3 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (BDS B1C Reduced)

FFS

# GNSS-Almanac (BDS B1C Midi)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-Almanac |  |  |  |  |
| weekNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| toa |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| ioda |  | Not present | Not present | Not present |
| completeAlmanacProvided |  | 1 (TRUE) | 1 (TRUE) | 1 (TRUE) |
| gnss-AlmanacList |  | (SIZE) 35 | (SIZE) 35 | (SIZE) 35 |

# GNSS-AlmanacElement (BDS B1C Midi)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| keplerianMidiAlmanac |  | Model-4 | Model-4 | Model-4 |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AlmanacElement (BDS B1C Midi)

FFS

# GNSS-Almanac: sub-test 10

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 2 |
| gnss-ID |  | 0 (gps) |
| GNSS-Almanac |  | See GNSS-Almanac (GPS) and/or GNSS-Almanac (Modernized GPS Reduced) and/or GNSS-Almanac (Modernized GPS Midi) depending on GNSS-Almanac supported by the UE |
| gnss-ID |  | 5 (bds) |
| GNSS-Almanac |  | See GNSS-Almanac (BDS B1I) and/or GNSS-Almanac (BDS B1C Reduced) and/or GNSS-Almanac (BDS B1C Midi) depending on GNSS-Almanac supported by the UE |

# GNSS-Almanac: sub-test 11

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-Almanac |  | See GNSS-Almanac (GPS) and/or GNSS-Almanac (Modernized GPS Reduced) and/or GNSS-Almanac (Modernized GPS Midi) depending on GNSS-Almanac supported by the UE |
| gnss-ID |  | 4 (glonass) |
| GNSS-Almanac |  | See GNSS-Almanac (GLONASS) |
| gnss-ID |  | 5 (bds) |
| GNSS-Almanac |  | See GNSS-Almanac (BDS B1I) and/or GNSS-Almanac (BDS B1C Reduced) and/or GNSS-Almanac (BDS B1C Midi) depending on GNSS-Almanac supported by the UE |

# GNSS-Almanac: sub-test 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-Almanac |  | See GNSS-Almanac (GPS) and/or GNSS-Almanac (Modernized GPS Reduced) and/or GNSS-Almanac (Modernized GPS Midi) depending on GNSS-Almanac supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-Almanac |  | See GNSS-Almanac (Galileo) |
| gnss-ID |  | 4 (glonass) |
| GNSS-Almanac |  | See GNSS-Almanac (GLONASS) |

# GNSS-Almanac: sub-test 13

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 3 |
| gnss-ID |  | 0 (gps) |
| GNSS-Almanac |  | See GNSS-Almanac (GPS) and/or GNSS-Almanac (Modernized GPS Reduced) and/or GNSS-Almanac (Modernized GPS Midi) depending on GNSS-Almanac supported by the UE |
| gnss-ID |  | 3 (galileo) |
| GNSS-Almanac |  | See GNSS-Almanac (Galileo) |
| gnss-ID |  | 5 (bds) |
| GNSS-Almanac |  | See GNSS-Almanac (BDS B1I) and/or GNSS-Almanac (BDS B1C Reduced) and/or GNSS-Almanac (BDS B1C Midi) depending on GNSS-Almanac supported by the UE |

##### 6.2.7.4.8	GNSS UTC MODEL:

# GNSS-UTC-Model: sub-tests 5, 11 and 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-UTC-Model |  |  |
| utcModel1 |  | Model-1 |

# UTC-ModelSet1: sub-tests 5, 11 and 12

Derived from data in clause 6.2.1.2 and the following information:

gnss-Utc-A1: 0

gnss-Utc-A0: 0

##### 6.2.7.4.9	GNSS AUXILIARY INFORMATION:

# GNSS-AuxiliaryInformation (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |  |  |
| gnss-ID-GLONASS |  | (SIZE) 8 | (SIZE) 8 | (SIZE) 8 |

# GNSS-ID-GLONASS-SatElement (GLONASS): sub-test 2

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| signalsAvailable |  | G1 | G1 | G1 |
| channelNumber |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AuxiliaryInformation (Modernized GPS): sub-test 4

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |  |  |
| gnss-ID-GPS |  | (SIZE) 9 | (SIZE) 10 | (SIZE) 10 |

# GNSS-ID-GPS-SatElement (Modernized GPS): sub-test 4

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| signalsAvailable |  | As supported by the UE | As supported by the UE | As supported by the UE |

# GNSS-AuxiliaryInformation: sub-test 8

GNSS-AuxiliaryInformation is used only if multiple GPS signals supported by the UE.

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 1 if UE supports multiple GPS signals |
| gnss-ID |  | 0 (gps) if UE supports multiple GPS signals |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (Modernized GPS) |

# GNSS-AuxiliaryInformation: sub-test 9

GNSS-AuxiliaryInformation is used only if BDS B1C is supported by the UE.

# GNSS-AuxiliaryInformation (BDS B1C)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| GNSS-AuxiliaryInformation |  |  |  |  |
| gnss-ID-BDS-r16 |  | (SIZE) 12 | (SIZE) 9 | (SIZE) 9 |

# GNSS-ID-BDS-SatElement-r16 (BDS B1C)

| Information Element | Units | Value/remark GNSS #1 | Value/remark GNSS #2 | Value/remark GNSS #5 |
| --- | --- | --- | --- | --- |
| svID-r16 |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |
| satType-r16 |  | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 | Derived from data in clause 6.2.1.2 |

# GNSS-AuxiliaryInformation: sub-tests 5 and 12

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 1, or 2 if UE supports multiple GPS signals |
| gnss-ID |  | 0 (gps) if UE supports multiple GPS signals |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (Modernized GPS) |
| gnss-ID |  | 4 (glonass) |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (GLONASS) |

# GNSS-AuxiliaryInformation: sub-tests 10 and 13

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 1 if UE supports multiple GPS signals or BDS B1C, or 2 if UE supports multiple GPS signals and BDS B1C |
| gnss-ID |  | 0 (gps) if UE supports multiple GPS signals |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (Modernized GPS) |
| gnss-ID |  | 5 (bds) if UE supports BDS B1C |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (BDS B1C) |

# GNSS-AuxiliaryInformation: sub-test 11

| Information Element | Units | Value/remark GNSS All |
| --- | --- | --- |
| GNSS-GenericAssistData |  | (SIZE) 1, or 2 if UE supports multiple GPS signals or BDS B1C, or 3 if UE supports multiple GPS signals and BDS B1C |
| gnss-ID |  | 0 (gps) if UE supports multiple GPS signals |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (Modernized GPS) |
| gnss-ID |  | 4 (glonass) |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (GLONASS) |
| gnss-ID |  | 5 (bds) if UE supports BDS B1C |
| GNSS-AuxiliaryInformation |  | See GNSS-AuxiliaryInformation (BDS B1C) |

<< next changes>>

Annex A (normative): GPS data files

## A.1	GPS data files for signalling tests

The GPS data files for use in GPS signalling tests defined in TS 37.571-2 [7] subclauses 6.1.1 to 6.1.3 are contained in archive GPS_Data_Sig_V7.zip which accompanies this document.

The acquisition assistance data files contained in the archive are recommended but not mandatory.

## A.2	GPS data files for Minimum Performance tests

The GPS data files for use in GPS Minimum Performance tests defined in TS 37.571-1 [6] subclause 5 are contained in archive GPS_Data_Perf_V10.zip which accompanies this document. The different scenarios are designated with suffixes XX in the zip file, where XX is 01, 02, 03 etc. for scenarios #1, #2, #3 etc.

The acquisition assistance data files contained in the archive are recommended but not mandatory.

Annex B (normative): GNSS data files

## B.1	GNSS data files for signalling tests

The GNSS data files for use in GNSS signalling tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9 are contained in archive GNSS_Data_Sig_V16.zip which accompanies the present document.

The acquisition assistance and reference measurement information data files contained in the archive are recommended but not mandatory.

Moreover there are currently two sets of the file Sig GANSS reference measurement information subtest1_4.csv supplied in the archive designated _set1 and _set2. Either set can be used for testing.

## B.2	GNSS data files for Minimum Performance tests

The GNSS orbital data files for use in GNSS Minimum Performance tests defined in TS 37.571-1 [6] subclauses 6,  7 and 13 are contained in archive GNSS_Orbital_Data_Perf_V1.zip which accompanies the present document. The different scenarios are designated with suffixes XX in the zip file, where XX is 01, 02, 03 etc. for scenarios #1, #2, #3 etc.

## B.3	GNSS data files for aerial tests

The GNSS data files for use in GNSS signalling tests using the scenario defined in TS 36.508 [20] subclause 4.12 are contained in archive GNSS_Data_Aerial_V1.zip which accompanies the present document.

The data files contained in the archive are recommended but not mandatory.

<< end of changes>>