3GPP TSG RAN Meeting #93e									        	RP-211767

Electronic Meeting, Sept 13 - 17, 2021

## Status Report to TSG

Agenda item:			9.3.2.6

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IAB_enh |  |  |  |  |
| Unique ID | 860050 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211548 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: 09/2022 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 41% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN-2 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1 #106-e

Agreement

The following solutions are supported to handle potential indication conflict of overlapping flexible symbols between two parent IAB-nodes:

- In intra-donor DC scenarios, if the IAB MT does not support simultaneous Tx and Rx on different carriers, it does not expect to receive conflicting DCI 2_0 from different parents.

Agreement

The IAB-donor-CU can be made aware of the IAB-MT’s capability regarding simultaneous transmission and reception on multiple serving cells in a frequency band, configured by the two parent nodes in intra-donor DC scenarios.

Agreement

The semi-static configuration of H/S/NA resource type in frequency domain is provided per RB set, per D/U/F resource type within a slot.

Agreement

N is a configured number of PRBs, where the CU configures N

- N = {2, 4, 8, 16, 32, 64}

- FFS: Value(s) of N in case of multiple configured BWPs at the IAB-MT

- This agreement does not revert any existing RAN1 agreement

Agreement

For a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied

Agreement

A Reference SCS is configured for frequency domain H/S/NA configuration.

Agreement

Spatial domain restrictions from a parent node or recommendations from a child node is limited to a subset of time resources in which simultaneous operation is applied.

- FFS: Handling of frequency resources in case of FDM operation

- FFS: Support for implicit/explicit indication of the simultaneous operation mode

Agreement

The child node indication of recommended beams to the parent node can include both IAB-MT DL beams and/or IAB-MT UL beams.

- FFS: Indication via MAC-CE or UCI transmission

- FFS: Definition of IAB-MT DL beams and/or IAB-MT UL beams (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Whether indication of “not preferred” beams is supported

Agreement

MAC-CE signaling of Desired/Provided Guard Symbols is enhanced (e.g. using the same Rel-16 MAC-CE design) to support indication of guard symbols additionally required for Case #6 and Case #7 timing cases.

- FFS: Number of guard symbols associated with Case #6 and Case #7 timing modes

- FFS: Need for explicit indication of guard symbols switching between timing cases

Agreement

To support extension of CA TDD prioritization rules to NR-DC, the following resource coordination mechanisms between parents/donors are supported:

- For intra-donor and inter-donor DC scenarios, in addition to coordination at the donor CU(s), a parent-node can be made aware of the DU resource configuration (UL/DL/FL, H/S/NA) of the other peer parent node that connects to the same IAB-node.

- For intra-donor and inter-donor DC scenarios, coordinating the semi-static and/or cell-common higher layer configuration (e.g. SSB, CORESET 0, and RACH and configurations) from/for different parent nodes.

Agreement

To support soft resource availability in the frequency domain, the existing DCI 2_5 format is reused according to one of the following alternatives:

- Alt. 1: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 2: Multiple DCI format 2_5 can be received indicating availability with the granularity of one or more RB set(s) for different RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 3: A single DCI format 2_5 can be received indicating availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell.

Agreement

To support soft resource availability in the frequency domain, the existing DCI 2_5 format is reused according to one of the following alternatives:

- Alt. 1: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 2: Multiple DCI format 2_5 can be received indicating availability with the granularity of one or more RB set(s) for different RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 3: A single DCI format 2_5 can be received indicating availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell.

Agreement

MAC-CE signaling from a parent node is supported for indication of beams of an IAB-DU in the direction of which simultaneous operation is restricted

- FFS: Details of beam indication (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Applicability to other beams

Agreement

Select from the following alternatives to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 6. The IAB-MT is expected to operate according to the non-flexible configuration.

Agreement

Select from the following alternatives to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1: The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

- FFS: Consideration of the impact of parent node’s H/S/NA when specifying the prioritization rules in case of UL/DL conflict

Conclusion

Decide in RAN1#106bis-e whether the parent IAB DUs of a DC connected IAB node can have per-backhaul-link resource configurations.

- FFS: whether the per-backhaul-link configuration is provided to the dual-connected IAB node.

/

Agreement

LS to RAN3 on IAB resource multiplexing is endorsed in R1-2108494.

Agreement

The reply LS to R1-2106420 (LS on Inter-donor migration, RAN3) is endorsed in R1-2108529.

Agreement

For Case 6 timing at a given IAB-node, the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- FFS: Need for additional details with reference to support of OTA synchronization (e.g. T_delta)

Agreement

For Case 7 timing at a parent node, the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

- FFS range, granularity, and signaling details of the offset.

Agreement

For the support of DU-to-DU measurement and report:

- For DU-to-DU CLI measurement:

  - Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

- For DU-to-DU CLI report:

  - Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

Agreement

Support the exchange of semi-static Rel-16 IAB-DU H/S/NA resource configuration information and Rel-17 frequency domain IAB-DU H/S/NA resource configuration information among neighbouring IAB-nodes/IAB-donors

Agreement

An IAB-node is explicitly indicated by the parent node when Case 6 timing is performed at the IAB-node at least for specific time resources.

- FFS: whether the indication should be associated with another dimensions, e.g. multiplexing cases

- FFS whether an IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the IAB-node.

Conclusion

Details on the design of the indication of when Case 6 timing (and Case 7 timing, if agreed) is performed at the IAB-node are to be discussed under 8.10.1.

Agreement

An IAB-node is explicitly indicated by the parent node when Case 7 timing is performed at the parent node.

- FFS for signalling details

Agreement

The desired DL TX power adjustment, indicated by the IAB-MT to its parent-node to assist with the parent-node’s DL TX power allocation, is provided at least for specific time resources.

The desired DL TX power adjustment can further be associated with spatial configuration. (e.g., MT’s DL RX beams).

- FFS: signalling details, e.g. indication via MAC-CE, PUCCH, or legacy CSI framework.

Agreement

Support an IAB-node indicating adjustment to its DL TX power to a child node (e.g., in response to receiving the DL TX power assistance information from the child node) at least for specific time resources.

The DL TX power adjustment indication can further be associated with spatial configuration. (e.g., MT’s DL RX beams).

- FFS: signalling details.

Conclusion

Discuss under 8.10.1 whether CLI measurements should be included in the beam report from a node to its parent node.

Agreement

Support an IAB-node indicating its desired IAB-MT PSD range to help with its MT’s UL TX power control.

- This information is provided to the parent node

FFS: Applicability of assistance information, e.g., per multiplexing scenario, per resource, etc.

FFS: Signalling details, including the possibility to extend PHR.

Conclusion

In Rel-17 the following may be up to network implementation, no specific specification work required:

- Differentiating access and backhaul slots.

- Restricting simultaneous operation of MT and DU to DL slots.

#### 2.1.2	Remaining Open issues

60% of the items defined in the RAN1 WID objectives have been accomplished. The following items defined in the WID objectives require further work:

Duplexing enhancements:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2 #115bis-e

Agreements on enhancements to topology-wide fairness multi-hop latency and congestion mitigation

- No agreements [apart from LCG range extension].

Agreements on LCG range extension

- The length of LCG to be extended to 8 bits (i.e., at most 256 LCGs).

- New Short (Truncated) BSR format to specified that has a fixed size and consists of an 8-bit LCG ID field and an 8-bit Buffer Size field.

Agreements on topology adaptation enhancements:

- A configured threshold of available buffer size based on flow control feedback is used to determine the congestion, for the purpose of local re-routing.

- For intra-CU cases, Support inter-donor-DU re-routing at least in the scenarios of NR-DC among donor-DUs, inter-donor-DU recovery and inter-donor-DU migration.

- Support inter-CU re-routing, i.e. IAB-node re-routes the data to its original donor-CU via the alternative BAP path over the topology in target CU.

- For inter-donor-DU re-routing, support the “previous routing ID to new routing ID” BAP header rewriting.

- RAN2 to further discuss the open issues for inter-CU routing:

  - What’s the BAP address added in BAP header in the first topology (i.e. the BAP address of ingress data at the boundary node);

  - How to differentiate the concatenated traffic and non-concatenated traffic;

  - How to determine whether a data should be delivered to upper layer (for downstream);

  - How to determine whether the BAP header of a data should be rewritten (i.e. whether being routed to another topology or its own topology).

- As baseline, support the 1:1 and N:1 mapping from “previous routing ID” to “new routing ID” for BAP header rewriting at the boundary node, in inter-CU routing.

- As baseline, support the 1:1 and N:1 mapping from “ingress BH link + ingress BH RLC ID” to “egress BH link + egress BH RLC ID” for bearer mapping at the boundary node, in inter-CU routing.

Outgoing LS:

- R2-2109108:  Reply LS on reduction of service interruption for intra-donor migration to RAN3

- R2-2109143:  Reply LS on inter-donor migration to RAN3

#### 2.2.2	Remaining Open issues

35% of the items defined in the RAN2 WID objectives have been accomplished.

For the RAN2-led WID objective on topology, routing and transport, RAN2 has made progress on the topics of enhancement of LCG-range, RLF indication, and local rerouting. RAN2 could not agree on any enhancements related to topology-wide fairness, multi-hop latency and congestion mitigation. For this reason, the overall accomplishment level for the RAN2 WID objectives is rather low.

For the RAN3-led WID objective on topology adaptation enhancements, RAN2’s accomplishment level is around 50%.

For the RAN1-led WID objective on duplexing enhancements, RAN2 has not yet started any work since it depends on further progress by RAN1.

TSG RAN may consider the rescoping of the RAN2-led WID objective to reflect the lack of progress related to topology-wide fairness, multi-hop latency and congestion mitigation.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3 #113-e

Agreements on IAB-node procedure details:

- For the boundary node, the following is supported for the IP addresses assigned by CU2 (target CU):

  - Assignment: assignment of address(es) from CU2 network that replace address(es) from CU1 (source CU) network.

  - Addition: assignment of additional addresses from CU2 network, after inter-donor migration/inter-donor topology redundancy setup.

    - Inter-donor RLF recovery cases are FFS

  - Replacement: an address from CU2 network is replaced by another address from CU2 network.

    - Procedures to be used are FFS

  - Release: an address from CU2 network is released.

- The node initiating the execution of the above functionalities is

  - Assignment: CU1.

    - Clarification: CU1 initiates the assignment via an RRC container as part of Xn signalling

  - Addition: the boundary node.

  - Replacement: CU2.

  - Release: CU2.

    - It is FFS if the Release procedure can be triggered by the boundary node

  - Note: procedures are not within scope of this proposal, only the initiating node is

- For network-based IP address allocation, the existing XnAP HO signalling be used for carrying the RRC containers for IP address assignment to the boundary node.

- WA: For no IPsec/IPsec transport mode, the source CU can be notified via F1AP signalling about the network IP addresses assigned to the boundary node by CU2.

- FFS if CU1 needs to know the outer IP addresses for IPSec tunnel mode

- Xn based signalling can be considered if benefits can be proven/agreed

- No dedicated signalling is needed to enable coupling of IP addresses in CU1 and CU2 networks.

- RAN3 studies enhancements on how to avoid reconfiguration of the descendant nodes (e.g., the reconfiguration of IP addresses) in the AI 13.2.2 on reduction of service interruption.

- Regarding the processing at the boundary node:

  - RAN3 prefers that the boundary node processes access traffic in the same manner as the non-boundary access IAB-node.

  - RAN3 prefers that the boundary node performs BAP header rewriting only for traffic routed on BAP layer from a BH link in one topology to a BH link in the adjacent topology, for both UL and DL traffic.

  - FFS: In addition to BAP header rewriting, performs routing and bearer mapping in the same manner as the non-boundary intermediate IAB-node.

  - RAN2 to be liaised with respect to the points above.

- For partial inter-donor migration, the IP addresses, BAP address, BH RLC CHs and default mapping used by the boundary node for traffic in a particular topology are assigned by the CU of that topology, and they are configured via RRC.

- A dual-connected boundary node can receive a separate configuration of IP addresses, BAP address, BH RLC CHs for each topology by MN and SN, respectively.

- Partial inter-donor migration can be revoked. FFS on whether it needs enhancement to current procedures.

- In partial inter-donor migration procedure, MOBIKE may be applied to update the outer address without changing the inner address.

Agreements on reduction of service interruption:

- To reduce the service interruption during intra-donor migration, the Step 11 can be performed before Step 3 (only stage-2 impact is needed).

- The RRCReconfiguration transfer in Solution 1 and RRCReconfiguration execution in Solution 2 can take place as soon as the routing table at migrating IAB node has been updated to have one or more entries for the target path, and there is RACH success of IAB-MT of migrating IAB-node.

- Agree stg-2 TP in R3-214398 to reflect MOBIKE and Proposal 1

- In Rel-17, the following aspects aiming at avoiding unnecessary DL transmissions will not be specified:

  - the avoidance of unnecessary DL data transmission over the source path between IAB donor CU and migrating IAB node

  - the transmission of in-flight DL packets buffered at migrating IAB node and its descendant node(s), after the migration

- R3-214398: TP for BL CR for TS 38.401 was Agreed

Agreements on topology redundancy:

- RAN3 assumes that the boundary node has only one BAP address in each topology.

- RAN3 assumes that for each topology, the boundary node’s BAP address for that topology is only used to identify packets that have to be passed to upper layers.

- Liaise RAN2 to consider RAN3’s preferences when discussing BAP processing at the boundary node.

- For DL traffic, the configurations of BAP routing entry and BAP-routing-ID mapping at the boundary node need to indicate the ingress topology they refer to. For UL traffic, they need to indicate the egress topology they refer to. The indications may be implicit.

- The QoS info can be passed gradually using multiple Xn messages.

- As a baseline, RAN3 assumes that each of BAP-routing-ID mapping and BH RLC CH mapping at the boundary node are constraint to 1:1 and N:1. Support for 1:N mapping is FFS. RAN3 to liaise RAN2 on this assumption.

- For UP access traffic to the boundary node, QoS info to be passed over the Xn interface with granularity of one or multiple F1-U GTP-U tunnels.

- If IAB node establishes NRDC before F1-C, the IAB node can implicitly derive whether MN or SN is the F1-terminating donor, e.g., based on who provides the default BAP configuration.

Agreements on congestion mitigation:

- WA: per-BAP routing ID congestion indication will not be pursued in this release.

- WA: the presence of Child Node Identifier IE is Mandatory, the value of the maxnoofIABCongInd is 1024

- No consensus to TP update, BL will be kept as it is now.

Agreements on multi-hop performance:

- To address the source IP filtering during inter-Donor-DU re-routing, Option 4 (i.e. IP-based tunneling between IAB-donor-DUs) is considered. FFS on whether providing source IP address to target donor DU.

- RAN3 further discusses whether static or dynamic tunnel is established between IAB-donor-DUs for option 4.

- RAN3 discusses the enhancement related to BAP routing towards the target IAB-donor-DU, after RAN2 make a decision.

Agreements on enhancements to resource multiplexing and CLI:

- WA: The F1-terminating donor of the boundary node forwards the boundary IAB node’s multiplexing info and the boundary IAB-DU’s activated cell list to the non-F1-terminating donor, via following XnAP procedures:

  - retrieve UE context procedure,

  - handover preparation procedure,

  - SN addition procedure,

  - MN initiated SN modification procedure

  - SN initiated SN modification procedure

- Wait for RAN1 progress on whether need to forward additional information.

- WA: parent node is aware of boundary IAB-DU cell configurations via the F1AP GNB-DU RESOURCE CONFIGURATION message

Endorsement of CRs as baseline:

| R3-213155 | BL CR to XnAP on Rel-17 eIAB (Samsung, Nokia, Nokia Shanghai Bell, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics) | CR0532r5, TS 38.423 v16.6.0, Rel-17, Cat. B |
| --- | --- | --- |
| R3-213156 | CR on CP-UP separation for Rel-17 IAB (Nokia, Nokia Shanghai Bell, Samsung, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics) | CR0020r3, TS 38.420 v16.0.0, Rel-17, Cat. B |
| R3-213171 | CP-based Congestion Mitigation for IAB Network (ZTE) | CR0076r1, TS 38.470 v16.5.0, Rel-17, Cat. B |
| R3-213178 | BL CR to TS 38.401 on support of eIAB (Huawei) | CR0179r2, TS 38.401 v16.6.0, Rel-17, Cat. B |
| R3-213186 | CP-based Congestion Indication for IAB Networks (Ericsson) | CR0737r7, TS 38.473 v16.6.0, Rel-17, Cat. B |

Outgoing LS:

- R3-214476:  LS on BAP- and RRC-related agreements to RAN2

#### 2.3.2	Remaining Open issues

45% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

There are outstanding decisions to be made by RAN3 related to inter-donor IAB-DU migration (i.e., pursuance of Alt.1 and/or Alt.2) and reduction of service interruption (i.e., solution 1 vs. solution 2 for RRC Reconfiguration delivery via source path). TSG RAN may consider recommendations to RAN3 to accelerate progress on these and potentially other aspects of the WI.

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#100-e

Agreement on simultaneous operation of MT TX/DU RX and MT RX/DU TX:

No RF requirement impact identified on Rel-17 IAB simultaneous operation including MT TX/DU RX and MT RX/DU TX.

Agreed WF on RAN4 RF specification impact due to Simultaneous MT TX/DU TX for FDM operation

- No RF core requirement impact for MT TX/DU TX simultaneous operation in FDM mode  in point of view of coexistence between IAB and existing NR network

    - There is no need for additional coexistence simulation.

    - No additional effort is needed on RF impact except issues listed below

- FFS on intra-node (i.e. transmissions from the same node interfering each other) interference considering MT/DU simultaneous transmission operation with unbalanced transmitting power with below cases:

Case 1: MT and DU using the same antenna panel

Case 2: MT and DU using different antenna panes

Case 3: other possibility is not excluded.

- Based on the investigation of the intra-node (i.e. transmissions from the same node interfering each other) interference, RAN4 could decide whether exception on unwanted emission or restriction on scenario can be addressed in core spec, conformance spec or TR with below options

    - Option 1: exception on unwanted emission: EVM, relative and absolute ACLR are not applied for power controlled link

    - Option 2:  restriction on scenario: FDM operation with shared beam case is assumed/considered for the same class and/or similar power capability between IAB-DU and IAB-MT only in RAN4 spec.

    - Option 3: other options are not excluded.

- Further study on conformance testing detail on this case is not precluded in perf. part such as testability, test coverage and test configuration

Agreed WF on RAN4 RF specification impact due to Simultaneous MT RX/DU RX for FDM operation

- No RF requirement impact for MT RX/DU RX simultaneous operation in FDM mode based on proper power control, which can ensure such operation mode will not be assumed to exceed legacy requirement (such as ACS, ICS.)

- Further study on conformance testing detail on this case is not precluded in perf. part such as testability, test coverage and test configuration

Agreed WF on Timing error between IAB-DU and IAB-MT transmission within one node for timing case#6:

- Timing error between IAB-DU and IAB-MT simultaneous transmission is to be considered as new dedicated RAN4 requirement to be decided to capture it in which section in TS38.174.

- Study on associated test configuration and test model in performance part if the core requirement is agreed.

- Take 3us as starting point for maximum Timing error between IAB-DU and IAB-MT simultaneous transmission

  - To review this value if improvement needed dependent on RAN1 agreement

Agreed WF on Timing error between parent IAB-DU and child node IAB-DU transmission:

- It is acknowledged that for shared hardware architecture, the parent IAB node can tolerate the certain value of timing error uncertainty between its child IAB node and its own DL timing

  - FFS on the value of time error tolerance on parent IAB node

  - To investigate if this has already been covered in Rel-16 cell phase synch requirement and can be ensured in legacy release.

  - No RAN4 RF requirement impact is expected due to this currently.

  - This can be reviewed if update needed due to further agreement in RAN1

Agreement on RRM impact due to Rel-17 IAB enhancement

On Case 6 Timing: Impact of case 6 timing on RRM requirements can be assessed after RAN1 and RF groups agreements on case 6 timing.

On Topology redundancy: No RRM requirement is needed for topology redundancy.

CLI: The impact of CLI on RRM requirements can be assessed after CLI related agreements in RAN1/2.

Mobility: No enhancement for mobility is expected (e.g. HO, L3 neighbour cell measurement) for RRM requirements in Rel-17.

Simultaneous operation: In general simultaneous operation between IAB-MT TX and DU RX or between IAB-MT RX and DU TX is not expected to impact RRM requirements for IAB-MT or DU. RAN4 will continue assessing any possible impact on RRM requirements based on RAN1/2 agreements related to RRM.

Inter-donor migration:  Reply LS to RAN3 on inter-donor migration approved in R4-2115354

#### 2.4.2	Remaining Open issues

25% of the items defined in the RAN4 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#106e:

| TDoc | Title | Source |
| --- | --- | --- |
| R1-2106454 | Resource multiplexing between backhaul and access for IAB duplexing enhancements | Huawei, HiSilicon |
| R1-2106455 | Enhancements for simultaneous operation of MT and DU | Huawei, HiSilicon |
| R1-2106617 | Enhancement to resource multiplexing between child and parent links | vivo |
| R1-2106618 | Other enhancements for simultaneous operation of child and parent links | vivo |
| R1-2106619 | Other enhancements to Rel-17 IAB nodes | vivo |
| R1-2106828 | Enhancements for resource multiplexing among IAB backhaul and access links | Nokia, Nokia Shanghai Bell |
| R1-2106829 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | Nokia, Nokia Shanghai Bell |
| R1-2106830 | Discussion on inter-donor IAB migration and topology redundancy | Nokia, Nokia Shanghai Bell |
| R1-2106907 | Enhancements to Resource Multiplexing for NR IAB | Samsung |
| R1-2106908 | Enhancements to Timing, Power Control and CLI for NR IAB | Samsung |
| R1-2107036 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | Fujitsu |
| R1-2107188 | Resource multiplexing in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2107189 | Timing, interference, and power control in enhanced IAB systems | Lenovo, Motorola Mobility |
| R1-2107364 | Workplan for Rel-17 IAB | Qualcomm Incorporated |
| R1-2107365 | Resource management for enhanced duplexing | Qualcomm Incorporated |
| R1-2107366 | Enhancements for simultaneous operation of IAB-node's child and parent links | Qualcomm Incorporated |
| R1-2107479 | Discussion on IAB resource multiplexing enhancements | ETRI |
| R1-2107480 | Discussion on simultaneous operation enhancements for eIAB | ETRI |
| R1-2107553 | Discussions on IAB resource multiplexing enhancements | LG Electronics |
| R1-2107554 | Discussions on other enhancement for simultaneous operation | LG Electronics |
| R1-2107607 | Enhancements to resource multiplexing between child and parent links of an IAB node | Intel Corporation |
| R1-2107608 | Other Enhancements for simultaneous operations | Intel Corporation |
| R1-2107665 | Other enhancements for IAB | Huawei, HiSilicon |
| R1-2107692 | Enhancements for IAB resource multiplexing | AT&T |
| R1-2107693 | Enhancements for IAB interference management | AT&T |
| R1-2107758 | Enhanced resource multiplexing within and across IAB nodes | Apple |
| R1-2107759 | Other enhancements for simultaneous operation of an IAB node's links | Apple |
| R1-2107824 | Enhancements to the IAB resource multiplexing | ZTE, Sanechips |
| R1-2107825 | Enhancements for simultaneous operation of child and parent links | ZTE, Sanechips |
| R1-2107826 | Consideration on reference carrier for semi-static IAB-DU resource configuration | ZTE, Sanechips |
| R1-2107877 | Resource multiplexing between child and parent links of an IAB node | NTT DOCOMO, INC. |
| R1-2107878 | Other enhancements for simultaneous operation of IAB-node’s child and parent links | NTT DOCOMO, INC. |
| R1-2108040 | Discussion on simultaneous operation of IAB-node’s child and parent links | CEWiT |
| R1-2108107 | Resource multiplexing and dual connectivity in enhanced IAB | Ericsson |
| R1-2108108 | Timing, interference management and power control in enhanced IAB | Ericsson |
| R1-2108109 | Discussion on overlapping DCI format 2_5 indications in enhanced IAB | Ericsson |
| R1-2108161 | Discussions on enhancements to resource multiplexing between child and parent links of an IAB node | CEWiT |
| R1-2108277 | Feature lead summary #1 of 8.10.1 | Moderator (AT&T) |
| R1-2108278 | Feature lead summary #2 of 8.10.1 | Moderator (AT&T) |
| R1-2108279 | Feature lead summary #3 of 8.10.1 | Moderator (AT&T) |
| R1-2108339 | FL summary#1 on other enhancements for simultaneous operation of IAB-node’s child and parent links [106-e-NR-eIAB-04] | Moderator (Qualcomm) |
| R1-2108479 | FL summary#2 on other enhancements for simultaneous operation of IAB-node’s child and parent links [106-e-NR-eIAB-04] | Moderator (Qualcomm) |
| R1-2108494 | Reply LS on IAB resource multiplexing | RAN1, Ericsson |
| R1-2108525 | Draft reply LS on Inter-donor migration | Moderator (Huawei) |
| R1-2108529 | Reply LS on Inter-donor migration | RAN1, Huawei |
| R1-2108568 | FL summary#3 on other enhancements for simultaneous operation of IAB-node’s child and parent links [106-e-NR-eIAB-04] | Moderator (Qualcomm) |

RAN2#115e:

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2106948 | LS to RAN2 on reduction of service interruption during intra-donor IAB-node migration (R3-212973; contact: AT&T) | RAN3 |
| R2-2106950 | LS on Inter-donor migration (R3-212981; contact: Samsung) | RAN3 |
| R2-2107063 | Fairness Latency and Congestion | CATT |
| R2-2107064 | Remaining issue of Local Rerouting | CATT |
| R2-2107065 | On Two Logical IAB-DUs in Boundary IAB-node | CATT |
| R2-2107066 | Reducing Service Interruption during Intra-donor IAB-node Migration | CATT |
| R2-2107112 | Discussion on BH Link issue detection | CANON Research Centre France |
| R2-2107113 | Discussion on flow control for congestion mitigation | CANON Research Centre France |
| R2-2107114 | Discussion on inter-donor DU local re-routing | CANON Research Centre France |
| R2-2107115 | Discussion on RLF indication enhancements | CANON Research Centre France |
| R2-2107169 | Updated Rel-17 IAB Workplan | Qualcomm Incorporated, Samsung (WI rapporteurs) |
| R2-2107170 | BAP-layer traffic processing at the boundary node | Qualcomm Incorporated |
| R2-2107171 | Discussion of RAN3 LS on Interruption time reduction for Intra-donor IAB-node Migration | Qualcomm Incorporated, Apple |
| R2-2107172 | RAN2 aspects related to RAN3’s LS on Full Migration | Qualcomm Incorporated, Apple |
| R2-2107177 | Fairness, latency and congestion – solutions to identified issues | Samsung Electronics GmbH |
| R2-2107178 | Enhancements to LCG space and BSR triggering including pre-emptive BSR | Samsung Electronics GmbH |
| R2-2107179 | Triggers for local rerouting | Samsung Electronics GmbH |
| R2-2107250 | Enhancements for topology-wide fairness, multi-hop latency and congestion mitigation | Huawei, HiSilicon |
| R2-2107251 | Summary of [Post114-e][075][eIAB] Open Issues on Re-routing | Huawei, HiSilicon |
| R2-2107252 | Discussion on two logical DUs and service interruption reduction for RAN3 LS | Huawei, HiSilicon |
| R2-2107253 | Inter-donor routing, local rerouting and RLF indication for R17-IAB | Huawei, HiSilicon |
| R2-2107254 | F1 over NR access link and CHO | Huawei, HiSilicon |
| R2-2107289 | IAB topology-wide fairness, latency, and congestion enhancement | Intel Corporation |
| R2-2107290 | IAB dual connection, RLF and local rerouting | Intel Corporation |
| R2-2107291 | Intra-donor CU topology migration | Intel Corporation |
| R2-2107445 | Inter-donor CU Topology migration | Intel Corporation |
| R2-2107516 | Re-routing ehnancements and RLF indications in IAB | Nokia, Nokia Shanghai Bell |
| R2-2107517 | Inter-donor-DU rerouting | Nokia, Nokia Shanghai Bell |
| R2-2107518 | Analysis of RAN3 LS on Inter-donor migration | Nokia, Nokia Shanghai Bell |
| R2-2107635 | Way forward for IAB enhancements to improve topology-wide fairness multi-hop latency and congestion mitigation | Apple |
| R2-2107636 | Topology adaptation and RLF handling in eIAB networks | Apple |
| R2-2107648 | Open issues on (re-)routing | Fujitsu |
| R2-2107649 | Open issues on Type-2 BH RLF indication | Fujitsu |
| R2-2107650 | Reduction of service interruption | Fujitsu |
| R2-2107651 | UE handover during inter-donor-CU migration | Fujitsu |
| R2-2107695 | Topology optimization in IAB | NEC |
| R2-2107700 | DAPS-like handover and NR DC for IAB | NEC |
| R2-2107701 | CHO for IAB | NEC |
| R2-2107851 | An elaboration of required PDB for multi-hop latency | ITRI |
| R2-2107859 | Discussion on multi-hop latency and LCG extension issues | vivo |
| R2-2107860 | Miscellaneous issues on topology adaptation | vivo |
| R2-2107861 | On Inter-CU routing, Inter-donor-DU rerouting and local re-routing | vivo |
| R2-2107862 | Discussion on Migration and Service Interruption | vivo |
| R2-2107892 | Discussion on LCG extension for IAB | Lenovo, Motorola Mobility |
| R2-2107893 | Discussion on local rerouting and local bearer remapping for IAB | Lenovo, Motorola Mobility |
| R2-2107894 | CHO recovery in IAB | Lenovo, Motorola Mobility |
| R2-2107997 | BH RLF Indications and local rerouting for eIAB | Kyocera |
| R2-2107998 | Possible solutions for topology-wide fairness, multi-hop latency and congestion mitigation in eIAB | Kyocera |
| R2-2108026 | Topology adaptation enhancements | Samsung |
| R2-2108053 | Number of hops information to improve topology-wide fairness and latency | Sony |
| R2-2108054 | Introduce cost factor in local re-routing | Sony |
| R2-2108139 | Discussion on fairness, multi-hop latency and congestion mitigation | ZTE, Sanechips |
| R2-2108140 | Discussion on inter-donor migration and service interruption reduction | ZTE, Sanechips |
| R2-2108141 | Discussion on inter-donor topology redundancy | ZTE, Sanechips |
| R2-2108142 | Discussion on RLF indication and local re-routing | ZTE, Sanechips |
| R2-2108241 | Fairness, latency, congestion | Nokia, Nokia Shanghai Bell |
| R2-2108416 | Support for inter-donor-DU rerouting | Qualcomm Finland RFFE Oy |
| R2-2108421 | On Topology-wide Fairness, Multi-hop Latency, and Congestion in IAB Network | Ericsson |
| R2-2108422 | Boundary IAB node behaviour for partial and full inter-donor migration | Ericsson |
| R2-2108423 | On Intra-donor Migration: Reduction of service interruption and CHO | Ericsson |
| R2-2108424 | On Local Routing and Type 2/3 RLF Handling | Ericsson |
| R2-2108437 | Multi-hop scheduling enhancements for IAB | AT&T |
| R2-2108438 | Alternatives for full inter-donor migration | AT&T |
| R2-2108482 | Solutions for Inter-Donor Routing and Bearer Mapping | Futurewei Technologies |
| R2-2108483 | Enhancements to Rel. 17 IAB RLF indications and local routing | Futurewei Technologies |
| R2-2108492 | Timing information for latency enhancement in multi-hop IAB | InterDigital |
| R2-2108493 | Latency enforcement, fairness and congestion mitigation in multi-hop IAB | InterDigital |
| R2-2108494 | CHO in IAB | InterDigital |
| R2-2108495 | DAPS support in IAB | InterDigital |
| R2-2108657 | Open issues on BH RLF indications | LG Electronics |
| R2-2108658 | CHO and DAPS-like Solution for eIAB | LG Electronics |
| R2-2108743 | Discussion on topology-wide fairness, multi-hop latency and congestion mitigation | LG Electronics Inc. |
| R2-2108744 | Discussion on local routing, LCG extension, and CP-UP separation | LG Electronics Inc. |
| R2-2108753 | Rel. 17 IAB enhancements for fairness, multi-hop latency reduction, and congestion mitigation | Futurewei Technologies |
| R2-2108873 | [Draft] Reply LS on inter-donor-DU rerouting | Huawei, HiSilicon |
| R2-2109032 | Feature summary of 8.4.2 (Rel-17 IAB contributions on fairness, latency and congestion) | InterDigital |
| R2-2109106 | Report of [AT115-e][042][eIAB] fairness, latency and congestion (Interdigital= | Interdigital (rapporteur) |
| R2-2109107 | Report from [AT115-e][040][eIAB] Reply LS on reduction of service interruption for intra-donor migration (AT&T) | AT&T |
| R2-2109108 | Reply LS to RAN3 on reduction of service interruption during intra-donor IAB-node migration | RAN2 |
| R2-2109122 | [Draft] Reply LS on inter-donor migration | Samsung |
| R2-2109143 | Reply LS on inter-donor migration | RAN2 |

RAN3#113e:

| TDoc | Title | Source |
| --- | --- | --- |
| R3-213123 | LS on Topology Adaptation enhancements | RAN2 |
| R3-213132 | Reply LS to LS Usage of MOBIKE in IAB | SA3 |
| R3-213155 | BL CR to XnAP on Rel-17 eIAB | Samsung, Nokia, Nokia Shanghai Bell, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics |
| R3-213156 | CR on CP-UP separation for Rel-17 IAB | Nokia, Nokia Shanghai Bell, Samsung, Verizon, Qualcomm Incorporated, CATT, ZTE, Fujitsu, AT&T, KDDI, Lenovo, Motorola Mobility, LG Electronics |
| R3-213171 | CP-based Congestion Mitigation for IAB Network | ZTE |
| R3-213178 | BL CR to TS 38.401 on support of eIAB | Huawei |
| R3-213186 | CP-based Congestion Indication for IAB Networks | Ericsson |
| R3-213205 | Discussion on inter-donor migration in IAB | ZTE |
| R3-213206 | Further considerations on service interruption reduction | ZTE |
| R3-213207 | Discussion on Inter-topology routing in redundancy scenario | ZTE |
| R3-213208 | Discussion on inter-Donor-DU re-routing in IAB | ZTE |
| R3-213209 | Discussion on resource multiplexing in IAB | ZTE |
| R3-213210 | (TP for NR_IAB_enh BL CR for TS 38.473): Congestion indication in CP-based congestion mitigation | ZTE |
| R3-213255 | Updated Workplan for Rel-17 IAB | Qualcomm Incorporated (WI Rapporteur) |
| R3-213256 | Inter-donor topology adaptation procedures | Qualcomm Incorporated |
| R3-213257 | Reduction of service interruption during IAB migration | Qualcomm Incorporated |
| R3-213258 | Inter-donor topology transport | Qualcomm Incorporated |
| R3-213259 | Inter-donor-DU local rerouting for IAB | Qualcomm Incorporated |
| R3-213312 | (TP for IAB BL CR TS38.401) Discussion on topology redundancy for Rel-17 IAB | Samsung |
| R3-213313 | (TP for IAB BL CR TS38.401) Discussion on inter-donor IAB node migration procedure for Rel-17 eIAB | Samsung |
| R3-213326 | Inter-Donor IAB Node Migration (Procedure Details) | CATT |
| R3-213327 | Inter-Donor IAB Node Migration (CHO) | CATT |
| R3-213328 | CP-UP separation and inter-donor topology redundancy | CATT |
| R3-213329 | Congestion Mitigation for CP-based | CATT |
| R3-213330 | Inter-donor-DU re-routing | CATT |
| R3-213486 | Procedure of full migration | Fujitsu |
| R3-213487 | Handling of descendant nodes in inter-donor CHO and RLF recovery | Fujitsu |
| R3-213488 | Discussion on the inter-donor topology redundancy | Fujitsu |
| R3-213530 | discussion on Inter-Donor IAB Node Migration | Nokia, Nokia Shanghai Bell |
| R3-213531 | Discussion on unnecessary UL/DL transmission during Intra-Donor Topology Adaptation | Nokia, Nokia Shanghai Bell |
| R3-213532 | discussion on Inter-CU topology redundancy | Nokia, Nokia Shanghai Bell |
| R3-213533 | (TP for IAB BLCR 38.473) C-plane congestion indication per Routing ID | Nokia, Nokia Shanghai Bell |
| R3-213534 | Inter donor-DU re-routing | Nokia, Nokia Shanghai Bell |
| R3-213535 | Resource multiplexing | Nokia, Nokia Shanghai Bell |
| R3-213598 | IAB Inter-donor Topology Adaptation | Ericsson |
| R3-213599 | The Discussion Status in AI 13.2.1.2 | Ericsson |
| R3-213600 | Mitigation of Unnecessary DL and UL Transmissions During Intra-Donor Topology Adaptation | Ericsson |
| R3-213601 | (TP for IAB BL CR for TS 38.473): Congestion Mitigation in IAB Networks | Ericsson |
| R3-213602 | The avoidance of UL Packet Discarding due to Local Re-routing | Ericsson |
| R3-213603 | Inter-donor Resource Coordination in IAB Networks | Ericsson |
| R3-213691 | Remaining issues for IAB inter-donor migration | Lenovo, Motorola Mobility |
| R3-213692 | Discussion on IAB inter-donor topology redundancy | Lenovo, Motorola Mobility |
| R3-213693 | Discussion on UL packet discarding for inter-donor-DU re-routing | Lenovo, Motorola Mobility |
| R3-213699 | Discussion on Service Interruption Reduction for Rel-17 IAB | Samsung |
| R3-213700 | Discussion on inter-donor-DU rerouting for Rel-17 IAB | Samsung |
| R3-213932 | (TP for NR_IAB_enh BL CR for TS 38.401) Inter-CU topology update | Huawei |
| R3-213933 | Reduction of Service Interruption for IAB topology update | Huawei |
| R3-213934 | Inter-CU topology redundancy | Huawei |
| R3-213935 | Discussion on IAB congestion mitigation | Huawei |
| R3-213936 | Inter-donor re-routing for IAB and unnecessary transmission during topology update | Huawei |
| R3-213937 | (TP for NR_IAB_enh BL CR for TS 38.423) IAB resource multiplexing | Huawei |
| R3-214230 | response to R3-213313 and R3-213326 | Nokia, Nokia Shanghai Bell |
| R3-214231 | CB: # 1301_IAB_BL_CRs - Summary of email discussion | Qualcomm - moderator |
| R3-214232 | CB: # 1302_IAB_Inter_Donor_Mig - Summary of email discussion | Ericsson - moderator |
| R3-214233 | CB: # 1303_IAB_Red_Serv_Inter - Summary of email discussion | Samsung - moderator |
| R3-214234 | CB: # 1304_IAB_Top_Red - Summary of email discussion | Qualcomm - moderator |
| R3-214235 | CB: # 1305_IAB_Con_Mit - Summary of email discussion | Huawei - moderator |
| R3-214236 | CB: # 1306_IAB_Multi-hop - Summary of email discussion | ZTE - moderator |
| R3-214237 | CB: # 1307_IAB_Res_Multiplex - Summary of email discussion | Nokia - moderator |

RAN4#100e:

| R4-2112866 | RAN4 workplan for Rel-17 IAB enhancement | Samsung,Qualcomm |
| --- | --- | --- |
| R4-2112867 | Simultaneous operation on IAB-node’s child and parent links | Samsung |
| R4-2112868 | Timing enhancement on Rel-17 IAB | Samsung |
| R4-2112869 | Discussion on RAN3 LS for inter-donor migration | Samsung |
| R4-2113149 | Further discussion on RRM requirements for eIAB | Intel Corporation |
| R4-2113198 | Simultaneous operation of IAB child and parent links | ZTE Corporation |
| R4-2113199 | Timing enhancement for eIAB | ZTE Corporation |
| R4-2113681 | Requirements related to different IAB timing cases | Nokia, Nokia Shanghai Bell |
| R4-2113682 | RF requirements for simultaneous IAB-MT and IAB-DU operation | Nokia, Nokia Shanghai Bell |
| R4-2113875 | On eIAB RRM | ZTE Corporation |
| R4-2114147 | Discussion on RRM requirements for eIAB | Huawei, Hisilicon |
| R4-2114329 | RF impact analysis for simultaneous DU and MT operation | Ericsson |
| R4-2114330 | IAB MT /DU case 6 timing | Ericsson |
| R4-2114463 | Analysis of RRM requirements for enhanced IAB | Ericsson |
| R4-2114464 | LS response on Inter-donor migration | Ericsson |
| R4-2114546 | Considerations on Rel. 17 IAB enhanced RRM Core Requirements | Nokia Germany |
| R4-2115222 | Email discussion summary: [100-e][232] NR_IAB_enh_RRM | Moderator (ZTE) |
| R4-2115353 | WF on IAB enhancement RRM | ZTE Corporation |
| R4-2115354 | Reply LS on inter-donor migration | ZTE Corporation |
| R4-2115407 | Email discussion summary: [100-e][232] NR_IAB_enh_RRM | Moderator (ZTE) |
| R4-2115606 | Email discussion summary for [100-e][316] NR_eIAB | Moderator (Samsung) |
| R4-2115645 | WF on RF impact for Rel-17 eIAB | Samsung |
| R4-2115788 | Email discussion summary for [100-e][316] NR_eIAB | Moderator (Samsung) |

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template