3GPP TSG-RAN Meeting # 93-e									RP-211823

Electronic Meeting, 13th – 17th Sep. 2021

Agenda item:			9.4.1.3

Source:	Nokia, Nokia Shanghai Bell

Title:	On Introduction of lower 6GHz NR unlicensed operation for Europe

Document for:	Discussion

# Introduction

During RAN#90 a WID on introduction of lower 6GHz NR unlicensed operation for Europe (RP-210762) was agreed.

The objectives of the core part work item are:

- Depending on the details of the European regulatory requirements, determine whether they are best handled by relevant updates (if any) of band n96 or whether a new band is needed.

  - If a new band is needed, determine the band plan for unlicensed operation in the range 5945-6425 MHz

- Define or update (if needed) system parameters such as channel bandwidths and channel arrangements

- Define or update (if needed) transmitter and receiver characteristics requirements for the UE

- Define or update (if needed) transmitter and receiver characteristics requirements for the BS

The objective of the performance part work item is:

- Define or update (if needed) conformance requirements for BS testing.

According to agreed work plan (R4-2101929) the target completion time for the WI was RAN#93. Hence, since the WI is not finalized it is needed to discuss how to further proceed.

# Discussion

Referencing the objectives of the WID, the first objective being whether to introduce unlicensed operation in the range 5945-6425 MHz for Europe by reusing band n96 or by introducing a new band has been the main discussion point since the approval of the WID. This open issue has been delaying progress on all other aspects.

The discussed options are:

- Option 1: Re-using already defined band n96, for the frequency range 5945 MHz to 6425 MHz

  - FFS if additional notes and/or clarifications are needed. Regional specific requirements to be included in relevant specifications.

- Option 2: Defining a new band n[xx], for the frequency range 5945 MHz to 6425 MHz

  - On top of specific requirements provided by ECC, the new band shall reuse requirements already defined for n96, where possible.

RAN4 have concluded that both option 1 and option 2 are technically feasible and can support unlicensed operation for Europe in the frequency range 5945 MHz to 6425 MHz. An extensive comparison of the specification impact was collected during RAN4#100 as captured in a not-agreed LS draft (R4-2114752). For reference, the comparison tables have been included in Annex A.

Observation 1: 	RAN4 concludes both option 1 and option 2 are technically feasible. A comparison of specification impact to 38.101-1 and 38.104 is presented in Annex A.

As found in the comparison, both options will for the UE require specific NS(s) to be defined to contain the regulatory requirements specific for Europe. Some concerns about UEs operating according to the wrong NS resulting in violation of the regulations have been expressed during the RAN4 meetings. However, UEs not conforming to the specification or BSs broadcasting wrong NS is a potential issue with both options 1 and 2 regardless. For the BS it is specified that the BS shall follow regional regulations when operating in shared spectrum.

Observation 2: 	Specific NS(s) is needed to be defined for European NR-U operation in the 6 GHz band regardless of which option is chosen.

The regulations for unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe are defined by ECC Decision (20)01 which led to this WI being initiated. During the RAN4 meetings, it has been questioned if these regulations are sufficient to introduce unlicensed operation in the lower 6 GHz range (5945 MHz to 6425 MHz) and if further considerations should be given to the upper 6 GHz range (6425 MHz to 7125 MHz). Note, however, that no regulations have yet been defined in Europe for the upper 6 GHz range.

Observation 3: 	No European regulations have yet been defined in the upper 6 GHz range (6425 MHz to 7125 MHz).

However, if there is a strong concern for how future regulations for the upper 6 GHz range potentially might impact NR-U operation in the lower 6 GHz range (5945 MHz to 6425 MHz) perhaps it is needed to delay further progress on the WI until this has become clear. Otherwise, RAN4 has to proceed with introducing unlicensed operation in the frequency range 5945 MHz to 6425 MHz in Europe according to the only regulations that exist, namely ECC Decision (20)01 as described in the WID.

If the WI is to proceed within RAN4 it is needed to agree to the proposed extension of the WI as further discussed in the status report (RP-211822), since according to the agreed WID it is targeted for completion at RAN#93.

Observation 4: 	The WI (NR_6GHz_unlic_EU) was targeted to be finalized at RAN#93.

Since it has not been possible for RAN4 to conclude if option 1 (Re-using already defined band n96) or option 2 (Defining a new band n[xx]) shall be followed and this has delayed the WI it is suggested that RAN decides this to progress further work within RAN4.

Proposal 1: 	RAN needs to consider if the WI (NR_6GHz_unlic_EU) needs to be put on hold pending potential future regulations for the upper 6 GHz range (6425 MHz to 7125 MHz), or further extension of the WI is needed. If further WI deadline extension is preferred by RAN, RAN should decide whether option 1 (Re-using already defined band n96) or option 2 (Defining a new band n[xx]) shall be adopted.

# Conclusion

The contributions discuss how the WI for the introduction of lower 6GHz NR unlicensed operation for Europe can be proceeded.

Observation 1: 	RAN4 concludes both option 1 and option 2 are technically feasible. A comparison of specification impact to 38.101-1 and 38.104 is presented in Annex A.

Observation 2: 	Specific NS(s) is needed to be defined for European NR-U operation in the 6 GHz band regardless of which option is chosen.

Observation 3: 	No European regulations have yet been defined in the upper 6 GHz range (6425 MHz to 7125 MHz).

Observation 4: 	The WI (NR_6GHz_unlic_EU) was targeted to be finalized at RAN#93.

Proposal 1: 	RAN needs to consider if the WI (NR_6GHz_unlic_EU) needs to be put on hold pending potential future regulations for the upper 6 GHz range (6425 MHz to 7125 MHz), or further extension of the WI is needed. If further WI deadline extension is preferred by RAN, RAN should decide whether option 1 (Re-using already defined band n96) or option 2 (Defining a new band n[xx]) shall be adopted.

# References

RAN4#100:

R4-2115008	Email discussion summary for [100-e][108] NR_6GHz_unlic_EU, Nokia

R4-2114883	TP to TR 38.849 on A-MPR values for PC5 VLP deployments, Qualcomm

R4-2113692	draft TR 38.849 v0.4.0, Nokia, Nokia Shanghai Bell

R4-2113693	On system parameters for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2112342	Band plan for lower 6GHz NR unlicensed operation in EU/CEPT, Apple, Facebook, Hewlett Packard Enterprise, Skyworks Solutions Inc., Microsoft

R4-2113934	Comparison of reusing n96 and defining a new band, ZTE Corporation

R4-2114219	NR-U 6 GHz band for Europe from a UE perspective, Qualcomm Incorporated

R4-2114231	6GHz unliscenced band numbering, Huawei

R4-2114476	Band plan for unlicensed operation in 6GHz in Europe, Ericsson

R4-2114220	A-MPR for NR-U VLP in 6 GHz for Europe, Qualcomm Incorporated

RAN#92:

RP-211320	SR for introduction of lower 6GHz NR unlicensed operation for Europe, RAN4

RP-211321	Draft TR 38.849 - Introduction of lower 6GHz NR unlicensed operation for Europe. Nokia

RAN4#99:

RP-2107921	Email discussion summary for [99-e][111] NR_6GHz_unlic_EU, Nokia

R4-2108020	WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

R4-2107789	TP to TR 38.849 on MPR values for LPI deployments, Nokia

R4-2109429	Band plan for lower 6GHz NR unlicensed operation in EU/CEPT, Apple, Facebook, Hewlett Packard Enterprise, Skyworks Solutions Inc.

R4-2110691	draft TR 38.849 v0.3.0, Nokia, Nokia Shanghai Bell

R4-2110692	On system parameters for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2111165	On NR unlicensed operation for lower 6GHz in Europe, Ericsson

R4-2109430	A-MPR for 6GHz NR unlicensed band in EU/CEPT, Apple

R4-2110693	On UE RF aspects for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2110983	NR-U VLP for EU 6 GHz, Qualcomm Incorporated

R4-2110617	Discussion on BS RF requirements for Europe unlicensed 6GHz, ZTE Corporation

R4-2110618	draft CR for introduction of Europe unlicensed 6GHz, ZTE Corporation

R4-2110694	On BS RF aspects for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2109431	On LPI and VLP modes for mixed indoor/outdoor scenarios, Apple

R4-2111408	Discussion on EU band allocation, Huawei

RAN4#98bis:

RP-2105461	Email discussion summary for [98-bis-e][124] NR_6GHz_unlic_EU, Nokia

R4-2105383	WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

R4-2105384	TP to TR 38.849 on NR-ARFCN and GSCN points, Nokia

R4-2104882	Band plan for lower 6GHz NR unlicensed operation for Europe, Apple

R4-2106273	RF front-end supporting NRU in 6GHz EU spectrum with band n96, Skyworks Solutions Inc.

R4-2107196	draft TR 38.849 v0.2.0, Nokia, Nokia Shanghai Bell

R4-2107197	On system parameters for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2106274	A-MPR due to in-band PSD limit for 6GHz EU unlicensed spectrum, Skyworks Solutions Inc.

R4-2107198	On UE RF aspects for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2107351	A-MPR for European NR-U 6 GHz band, Qualcomm Incorporated

R4-2106604	Discussion on BS RF requirements for Europe unlicensed 6GHz, ZTE Corporation

R4-2106659	draft CR for introduction of Europe unlicensed 6GHz, ZTE Corporation

R4-2107199	On BS RF aspects for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2104883	On simultaneous low power and very low power operation, Apple

RAN#91:

RP-210762	WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

RP-210486	SR for Introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

RAN4#98:

RP-2103320 	Email discussion summary for [98e][133] NR_6GHz_unlic_EU, Nokia

R4-2103229 	WF on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Qualcomm, Charter Communications Inc

R4-2100514	Band plan for lower 6GHz NR unlicensed operation for Europe, Apple Inc.

R4-2100546	NRU in 6GHz EU spectrum: Band Definition and Related Emission Requirements, Skyworks Solutions Inc.

R4-2101927	Skeleton TR 38.849 v0.0.0, Nokia, Nokia Shanghai Bell

R4-2101928	draft TR 38.849 v0.1.0, Nokia, Nokia Shanghai Bell

R4-2101929	Work plan for Introduction of lower 6GHz NR unlicensed operation for Europe, Nokia, Nokia Shanghai Bell

R4-2101965	Discussion on Europe unlicensed 6GHz for NR-U, ZTE Corporation

R4-2101930	On UE RF aspects for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2102416	UE requirements for EU NR-U 6 GHz band, Qualcomm Incorporated

R4-2101931	On BS RF aspects for the lower 6GHz NR unlicensed operation, Nokia, Nokia Shanghai Bell

R4-2101966	Discussion on BS RF requirements for Europe unlicensed 6GHz, ZTE Corporation

RAN#90:

RP-202592 	WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

RP-202591 	SR for Introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

RAN#89:

RP-202116 	New WID on introduction of lower 6GHz NR unlicensed operation for Europe, Nokia

# Annex A

In the tables below it is indicated whether or not specification(s) need changes (i.e. amendments or clarifications) or not (i.e. can be re-used as is). Further, comments on actions needed or concerns have been added.

Table 1 – Comparison of UE specification (TS 38.101-1) impact for the two options

| Affected clause | Requirement | Option 1 Re-using already defined band n96 | Option 2 Defining a new band n[xx], |
| --- | --- | --- | --- |
| 5.2 | Operating bands | No changes needed Existing band n96 is re-used FUL_low: Limited to 5945 MHz FUL_high: Limited to 6424 MHz FDL_low: Limited to 5945 MHz FDL_high: Limited to 6424 MHz | Changes needed A new band [nXX] is added FUL_low: Defined as 5945 MHz FUL_high: Defined as 6424 MHz FDL_low: Defined as 5945 MHz FDL_high: Defined as 6424 MHz |
| 5.2A.1, 5.2A.2 | Intra-band CA and inter-band CA | Potential re-use of band n96 combinations limited to the frequency range available in Europe. | Potential duplicating of band combinations with new band number. |
| 5.3.5 | UE channel bandwidth per operating band | No changes needed Existing channel bandwidth will be re-used. Including 100MHz CBW. | Changes needed Duplicating and re-using of the n96 channel bandwidth for the new band in the bandwidth table. Including 100MHz CBW. |
| 5.4.2.3 | Channel raster | No changes needed Existing channel rasters are re-used (with a NOTE if needed limiting the range for the EU/CEPT region) | Changes needed Duplicating and re-using of the n96 channel rasters, that correspond to the 5945-6425MHz range |
| 5.4.3.3 | Sync raster | No changes needed Existing sync rasters are re-used (with a NOTE if needed limiting the range for the EU/CEPT region) | Changes needed Duplicating and re-using of the n96 sync rasters corresponding to the 5945-6425MHz range |
| 5.5A.1 | Configurations for intra-band contiguous CA | No changes needed Band n96 intra-band CA configurations will be re-used, including being standardised NR-U UL CA. | Changes needed Duplicating and re-using of n96 intra-band CA configurations, including being standardised NR-U UL CA. |
| 5.5A.3.1 | Configurations for inter-band CA (two bands) | Potential re-use of band n96 combinations limited to the frequency range available in Europe. | Potential duplicating of band combinations with new band number. |
| 6.2F.1 | UE maximum output power | Specification changes New NS value(s) corresponding to the EU/CEPT regulations | Specification changes New NS value(s) corresponding to the EU/CEPT regulations |
|  |  | Some companies expresses concern with having multiple NS values defined for different regions for a single band and stated that European regulatory conformance cannot be meet with this approach. However, other companies do not share these concerns and note that other bands defined within the 3GPP specification also have multiple NSs for different regions. |  |
| 6.2F.2 | UE maximum output power reduction | No changes needed | No changes needed |
| 6.2F.3 | UE additional maximum output power reduction | Changes needed New NS value(s) added but only applicable for the 5945-6425 MHz range restricting operation within this frequency range. | Changes needed New NS value(s) for the new band |
| 7.3F | Reference sensitivity | No changes needed Band n96 requirements are re-used. Including REFSENS exceptions for band combinations in 38.101-1 and 38.101-3 | Changes needed Duplicating and re-using of band n96 requirements. Including REFSENS exceptions for band combinations in 38.101-1 and 38.101-3 |
| 7.6F.2 | In-band blocking characteristics | FFS | FFS |
| 7.6F.3 | Out-of-band Blocking characteristics | FFS | FFS |

Table 2– Comparison of BS specification (TS 38.104) impact for the two options

| Affected clause | Requirement | Option 1 Re-using band n96 | Option 2 Defining a new band n[xx] |
| --- | --- | --- | --- |
| 5.2 | Operating band | No changes needed Existing band n96 is re-used FUL_low: Limited to 5945 MHz FUL_high: Limited to 6424 MHz FDL_low: Limited to 5945 MHz FDL_high: Limited to 6424 MHz | Changes needed A new band [nXX] is added FUL_low: Defined as 5945 MHz FUL_high: Defined as 6424 MHz FDL_low: Defined as 5945 MHz FDL_high: Defined as 6424 MHz |
| 5.4.2.3 | Channel raster entries for each operating band | No changes needed Existing channel bandwidth will be re-used. Including 100MHz CBW. | Changes needed Duplicating and re-using of the n96 channel bandwidth for the new band in the bandwidth table. Including 100MHz CBW. |
| 5.4.3.3 | Synchronization raster entries for each operating band | No changes needed Existing channel rasters are re-used (with a NOTE if needed limiting the range for the EU/CEPT region) | Changes needed Duplicating and re-using of the n96 channel rasters, that correspond to the 5945-6425MHz range |
| 6.6.1 | General | FFS if the definition of Foube have to be clarified for the range 5945-6425 MHz when operated in EU | FFS if Foube needs redefining for new band |
| 6.6.3.2 | ACLR | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 6.6.5.2 | UEM | FFS | FFS |
| 7.2.2 | REFSENS | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.3.2 | Dynamic range | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.4.1.2 | ACS requirement | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.4.2 | In-band blocking | Changes needed Modify the requirement for 5945-6425 so note is not based on operating band but on restricted frequency range (and potentially a modified Foube) when operating in EUFFS | Changes needed Clarify Foube requirements for new band with the range 5945-6425 MHz |
| 7.5.2 | Out-of-band blocking | Changes needed Modify the requirement for 5945-6425 so note is not based on operating band but on restricted frequency range (and potentially a modified Foobb) when operating in EUFFS | Changes needed Clarify Foobb requirements for new band with the range 5945-6425 MHz |
| 7.5.3 | Co-location minimum requirements | Changes needed Paragraph 4 and Note 3 would have to be modified to take into account the range 5945-6425 MHz when operated in EU (not the operating band as the exclusions would only exist for the allocated sub-band). | No change needed |
| 7.6.2 | Receiver spurious emissions | Changes needed Modify the requirement for 5945-6425 so note is not based on operating band but on restricted frequency range (and potentially a modified Foube) when operating in EU | No change needed |
| 7.7.2 | Intermodulation requirement | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |
| 7.8.2 | ICS requirement | No changes needed | Changes needed Duplicating and re-using from n96 and add the new band number |