3GPP TSG RAN meeting #93e										RP-211938

Electronic Meeting, September 13-17, 2021

## Status Report to TSG

Agenda item:			9.3.2.1

| WI / SI Name | Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_IIOT_URLLC_enh |  |  |  |  |
| Unique ID | 860045 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-210854 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 03/2022 | Performance part: 09/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 55% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Ping-Heng Wallace Kuo |
|  | Company | Nokia |
|  | Email | Ping-Heng.Kuo@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

| UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1.1) | Agreement The SPS HARQ-ACK deferral is enabled per SPS configuration - Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations enabled for deferral is in principle subject to deferral Agreement Definition of when to defer from the initial slot: - Alt1: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid Agreement Update the following RAN1#105-e agreement as (RED): - RAN1#105-e Agreement: For PUCCH carrier switching, the PUCCH resource configuration (i.e. pucch-Config / PUCCH-ConfigurationList) is per UL BWP (i.e. per candidate cell and UL BWP of that specific candidate cell). - FFS: CSI and SR Agreement For SPS HARQ-ACK deferral, the maximum deferral value in terms of k1+k1def is RRC configured per SPS configuration. Agreement For SPS HARQ-ACK deferral, only SPS HARQ bits subject to deferral from HARQ-ACK codebook from an initial PUCCH slot are deferred to the target PUCCH slot Agreement For SPS HARQ-ACK deferral, deferred SPS HARQ bits from more than one ‘initial PUCCH slot’ can be jointly deferred to a target PUCCH slot Agreement Confirm the following RAN1#105-e working assumption: For at least HARQ-ACK re-transmission: - Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17 - Definition of enhanced Type 3 CB: - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells) - Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17 - Details are FFS - Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities Agreement Support PHY priority handling for a PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size. - The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size. - The A/N of HARQ processes is mapped to the Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size irrespective of the PHY priority of the ‘A/N’ of the HARQ processes. - FFS: If the HARQ-ACK codebook size or structure is dependent on the PHY priority (e.g. separate configuration of CBG/NDI usage, separate configuration of HARQ IDs / CCs per priority, SPS HARQ-ACK process IDs of specific priority only for a SPS HARQ-ACK only codebook, …). Agreement Support PHY priority handling for a PUCCH carrying the Rel-16 Type 3 HARQ-ACK CB in Rel-17. - The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the Rel-16 Type 3 HARQ-ACK CB. - The A/N of HARQ processes is mapped to the Rel-16 Type 3 HARQ-ACK CB irrespective of the PHY priority of the ‘A/N’ of the HARQ processes. - The support is subject to a Rel-17 UE capability and a UE supporting this capability can be configured in Rel-17 with Rel-16 Type 3 HARQ-ACK CB and PHY prioritization. Agreement For the PHY priority handling of the enhanced Type 3 CB(s) of smaller size, the enhanced Type 3 HARQ-ACK has the same structure, size and content (in terms of HARQ-IDs, CCs) irrespective of the PHY priority. Agreement Support Rel-17 enhanced Type 3 HARQ-ACK CB of smaller size triggering using DCI format 1_2 for a UE supporting DCI format 1_2. - The triggering support for DCI format 1_2 is independently (from triggering using DCI format 1_1) RRC configured to the UE. Agreement Support Rel-16 Type 3 HARQ-ACK CB triggering using DCI format 1_2 in Rel-17 for a UE supporting DCI format 1_2. - The support is subject to a Rel-17 UE capability and a UE supporting this capability can be configured with DCI format 1_2 triggering of the Rel-16 Type 3 HARQ-ACK CB. Agreement For the enhanced Type 3 HARQ-ACK CB of smaller size triggered in a PUCCH slot, the UE is not expecting HARQ-ACK information in a Type 1 or Type 2 HARQ-ACK CB to be transmitted that cannot be mapped to the enhanced Type 3 HARQ-ACK CB of smaller size as the HARQ process is not part of the codebook. Agreement The DCI triggering (by a DL assignment) the one-shot HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB dynamically indicates the HARQ-ACK codebook(s) / PUCCH occasions to be re-transmitted. - FFS details Agreement A single DCI triggering the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB can trigger the re-transmission of HARQ-ACK information of only a single HARQ-ACK CB. Agreement The Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB is done through an explicit triggering indication in the DCI through a DCI field. Agreement Support PHY priority handling for the Rel-17 one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB. - The indicated PHY priority in the triggering DCI defines the PHY priority of the PUCCH carrying the re-transmitted HARQ-ACK information. - The indicated PHY priority in the triggering DCI is used to determine the HARQ-ACK information to be re-transmitted corresponding to the indicated PHY priority. Conclusion The dynamic repetition indication solution for slot-based PUCCH repetition from the RAN1#105-e working assumption from Cov. Enh. WI can be directly applied for dynamic repetition indication for sub-slot based PUCCH repetition. Agreement For sub-slot based PUCCH repetition for HARQ-ACK, semi-static configured PUCCH repetition (i.e. using nrofSlots) and dynamic repetition factor based operation is supported. - Sub-slot based PUCCH repetition based on semi-static configuration (i.e. using nrofSlots) and based on dynamic indication is subject to separate UE capabilities Agreement For SPS HARQ-ACK deferral, the target PUCCH slot is defined as the next PUCCH slot where sps-PUCCH-AN-List-r16 or n1PUCCH-AN PUCCH resource is regarded as valid, or a PUCCH resource (from PUCCH-ResourceSet, i.e. DG PDSCH HARQ multiplexed) is dynamically indicated - The target PUCCH slot determination is based on the total HARQ-ACK payload size including deferred SPS HARQ-ACK information and non-deferred HARQ-ACK information (if any) of a candidate target PUCCH slot - The final PUCCH resource selection in the target PUCCH slot in terms of PUCCH resource set and PUCCH resource ID follows the Rel-16 procedures. Agreement For SPS HARQ-ACK deferral, if after the target PUCCH slot determination the deferred SPS HARQ-ACK cannot be transmitted, the deferred SPS HARQ-ACK bits are not further deferred and are dropped. Agreement For SPS HARQ-ACK deferral, in the target PUCCH slot the deferred SPS HARQ-ACK bits are appended to the initial HARQ bits / Type 1 or Type 2 codebook. Agreement For SPS HARQ-ACK deferral, confirm the RAN1#104b-e working assumption with the following updates in RED: (working assumption) To handle the collision for the same HARQ process due to deferred SPS HARQ-ACK the following behaviour is to be specified: - In case the UE is expected to receives PDSCH of a certain HARQ Process ID according to TS 38.214 Sec. 5.1, the deferred SPS HARQ bit(s) for this HARQ Process ID are dropped. - Note: there is no further discussion on specific handling for the case of DG PDSCH with the same HARQ process ID Agreement For enh. Type 3 HARQ-ACK CB(s), support dynamic selection based on indication in the triggering DCI of one of at least one enh. Type 3 HARQ-ACK CB(s). - Each of the at least one enh. Type 3 HARQ-ACK CBs is at least defined by RRC configuration This includes the option to configure all DL HARQ processs of all configured CCs as one enh. Type 3 HARQ-ACK CB (resulting in same structure and size as the Rel-16 Type 3 HARQ-ACK CB) - This includes UE capability signaling (value range {1…X}) on the maximum number of supported simultaneously configured enh. Type 3 HARQ-ACK CBs that can be dynamically indicated - Details including the value of X are FFS Agreement The following enhanced Type 3 CB types of smaller size are supported, the CB to contain either: - the HARQ processes of a subset of configured CCs, or - a subset of configured HARQ processes (specific to CCs) FFS: additional enh. Type 3 CB types Agreement For Rel-17 one-shot triggering for HARQ-ACK re-transmission, the UE does not expect more than one triggering DCI for Rel-17 one-shot feedback indicating the same PUCCH slot for the re-transmission of HARQ-ACK CBs of different PUCCH slots to be re-transmitted - Note: i.e. only a single HARQ-ACK codebook / PUCCH occasion can be re-transmitted in a PUCCH slot Agreement Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2 also for single TRP operation. - The support is subject to independent UE capability indication Agreement In addition to HARQ-Ack of PDSCH dynamically scheduled by a DCI indicating a PUCCH carrier, the dynamic target carrier indication also applies to: - HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI - HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI - triggered PUCCH for Rel-16 Type 3 CB, Rel-17 enh. Type 3 CB of smaller size and Rel-17 one-shot triggering for HARQ-Ack retransmission based on the indication in the triggering DCI - FFS: Additional cases Agreement Semi-static PUCCH carrier switching is applicable to all UCI types incl. HARQ-ACK, SR and CSI. |
| --- | --- |
| CSI feedback enhancements (RAN1 AI 8.3.1.2) | Agreement For subband CQI reporting with more than 2 bits per subband - Support 4-bits CQI only Agreement For subband CQI reporting in Rel-17, RRC can configure use of legacy 2-bits D-CQI or 4-bits CQI for each CSI report configuration. - This feature is subject to UE capability - FFS: Whether wideband CQI report can be omitted Conclusion There is no consensus in RAN1 on the support of delta-MCS in Rel-17. |
| Enhancements for unlicensed band URLLC/IIoT (RAN1 AI 8.3.2) | Agreement In semi-static channel access mode, the content in a scheduling DCI that indicates the assumption on the COT-initiator for the scheduled transmission is determined based on the channel access field in the DCI. Agreement In semi-static channel access mode, - The inclusion of the channel access field in Rel-16 DCI 0_1 and 1_1 in Rel-17 DCI 0_2 and 1_2, respectively, is supported. Agreement In semi-static channel access mode, the size of channel access field in a scheduling DCI with format 0_0/1_0, 0_1/1_1, 0_2/1_2 is 2 bits. Conclusion Any UL or DL transmission that is expected to occur, should be associated to a Channel Occupancy (CO) with a corresponding FFP. When a transmission is associated to a CO with a corresponding FFP: - The association of the transmission to a CO with corresponding FFP is based on either of the following assumption: - “Initiating COT”: This assumption implies that the transmission would initiate a CO corresponding the FFP. - “Sharing COT”: This assumption implies that the transmission would share a CO corresponding to the FFP. - The association assumption is validated as follows: - “Initiating COT” assumption is validated if the transmission would start at the FFP boundary and would end before idle period of the FFP. - “Sharing COT” assumption is validated if the transmission would start after the FFP boundary and would end before idle period of the FFP and the CO corresponding to the FFP is initiated. - A transmission based on a CO association assumption can occur if the CO association assumption is validated and if the following sensing conditions are met: - For CO association assumption as “Initiating COT”: - If a CCA is successful before the transmission. - For CO association assumption as “Sharing COT” - If the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is more than 16us and if a CCA is successful before the transmission. - IF the gap between the beginning of the transmission and the end of previous one sharing the same CO in that FFP is at most 16us Agreement In semi-static channel access mode, a DL transmission burst based on sharing of a UE initiated COT corresponding to a UE FFP, shall include at least scheduled DL transmission or a DCI intended for the UE that initiated that FFP. - FFS whether/how the DL transmission burst can include transmission to any other UE in the cell than the COT initiating UE and/or broadcast transmission while ensuring that the COT initiated by the UE is not shared by any other UE in the cell for any UL transmission Agreement In semi-static channel access mode, the content of the channel access field in a DCI scheduling a UL transmission for a UE determines an index to a row in Table 1 with Alt-1 (Option 1) TABLE 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]; Initiator of a channel occupancy associated to UL transmission described in Clause x.x in TS 37.213. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0; Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2; Alt-1:gNB Alt-2: UE-initiated COT if condition A, otherwise gNB’s COT. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0; gNB. 3; 9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213; 0; UE. Note: The last row in Table 1 is only applicable when the UE can operate as an initiating device as configured by gNB. Agreement In semi-static channel access mode, when the gNB schedules by a DCI a UL transmission in a later g-FFP that is different from the g-FFP that carries the scheduling DCI: - The UE follows the indicated COT initiator as the following: - If the UE validates the indicated COT initiator assumption and satisfies the applicable sensing conditions, the transmission occurs. Otherwise, the transmission is dropped. Conclusion There is no consensus in RAN1 to support UE-initiated COT for semi-static channel occupancy in IDLE/INACTIVE mode. Agreement Do not support PUSCH repetition Type B based on NR-U Rel-16 CG for unlicensed band operation. Agreement Replace “9us sensing [within a 25us interval] as defined in Clause 4.3 in TS 37.213” with “9us sensing as defined in Clause x.x in TS 37.213” in the last row of Table 1 in the previous agreement and add the following notes to Table 1: - Note 1: The intention of Clause x.x above is to describe the LBT procedure from a UE perspective when this operates as initiating device. - Note 2: A UE operating as initiating device may transmit an UL transmission burst(s) within its u-FFP immediately after sensing the channel to be idle for at least a sensing slot duration if the gap between the UL transmission burst(s) and any previous transmission burst is more than Agreement When a UE operates as an initiating device, and the gNB shares a UE’s FFP for DL transmission, regardless of the gap between any UL and DL bursts, no restriction is imposed on the maximum duration of each of the DL bursts such that each can continue until the UE FFP idle period starts. - Note: The applicability of the EDT calculation based on the UE’s transmit power to the UE COT initiation in accordance to the UL-DL gap duration and/or the content of the DL burst is separately discussed |
| Intra-UE multiplexing enhancements (RAN1 AI 8.3.3) | Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - HP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - LP A/N reuses rate matching equation, and RE mapping rules in Rel-15 for CSI-2. Above applies at least for PUCCH format 3 and 4. Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, an additional maxCodeRate for LP HARQ-ACK can be configured in the second PUCCH-Config per PUCCH format. Conclusion Simultaneous PUCCH/PUSCH transmission on the same cell is not supported in Rel-17. Agreement In NR Rel-17, [at least] 2 new set of beta offset values can be configured to the UE to indicate separate beta_offset values for the following cases: - Multiplexing LP HARQ-ACK on HP PUSCH - Multiplexing HP HARQ-ACK on LP PUSCH Working Assumption For handling overlapping PUCCHs/PUSCHs with different priorities in R17 - Step 1: Resolve overlapping PUCCHs and/or PUSCHs with the same priority - Step 2: Resolve overlapping PUCCHs and/or PUSCHs with different priorities Note: Avoid recursive pseudo-code to implement this procedure Note: It is expected that Rel-15 intra-UE UCI multiplexing timeline will be applicable Agreement For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - PUCCH resource set determination is based on: UCI payload size = the number of HP UCI bits + the number of LP UCI bits. - FFS PRB number determination for HP A/N and LP A/N, e.g. based on their coding rates. - FFS the impact to the number of LP UCI bits due to missed DCI and potential solutions - Note: the number of LP UCI bits in the above agreement does may not necessarily mean the actual number of LP UCI bits until the second FFS is resolved |
| Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4) | Agreement SRS can be used for Rx – Tx time difference estimation at gNB side for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. Agreement Send LS to RAN4 to ask for feedback on the following questions: - Question 1: Is it feasible to support a smaller value than the current Te for the use of propagation delay compensation, assuming the existing conditions in TS 38.133 for Te requirement? If not, is it feasible under new conditions (e.g. using TRS instead of SSB)? If the answer is yes, please also provide feedback on how much it can be reduced at most. - Question 2: Is it feasible to introduce enhanced TA command indication granularity? If the answer is yes, please also provide feedback on how much it can be reduced at most (e.g. reduced to (1/16)* (16*64*Tc/2)) similar as the granularity for Rel-16 IAB based on the Timing Delta MAC CE and related condition. - Note 1: The alternatives in the working assumption achieved in RAN1#104bis-e together with the examples in Table 4.2-2 will be included in the LS to give some background for RAN4 - Note 2: The agreement “both SCS 15 kHz and 30 kHz are assumed for both control-to-control and smart grid for evaluation of the time synchronization” achieved in RAN1#102-e will be included in the LS for RAN4 information also. - Note 3: Inform RAN4 that the enhancements on Te and TA command indication granularity for propagation delay compensation may or may not have impact on normal TA related procedure, depending on which candidate option for TA-based PDC is adopted. Note that this is just for RAN4 information. - Note 4: Whether RAN1 will introduce specification enhancements is still undetermined. LS is endorsed in R1-2108635. Agreement If RTT-based propagation delay compensation is supported, - CSI-RS for tracking (TRS) can be used for Rx – Tx time difference estimation at UE side, if PRS is not configured for the UE. - PRS can be used for Rx – Tx time difference estimation at UE side, if PRS is configured for the UE. Agreement Send LS to RAN4 to ask for defining the following for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - UE Rx-Tx time difference measurement accuracy errorUE,RxTxDiff based on CSI-RS for tracking - gNB Rx-Tx time difference absolute accuracyerrorgNB,RxTxDiff errorUE,RxTxDiff based on SRS Agreement Support the following configurations for RTT-based propagation delay compensation, if RTT-based propagation delay compensation is supported. - At least one CSI-RS for tracking (TRS) configuration for Rx – Tx time difference estimation at UE side if PRS is not configured - At least one SRS configuration for Rx – Tx time difference estimation at gNB side Agreement If RTT-based propagation delay compensation is supported and performed at the UE side, the Rx-Tx measurement report provided from the gNB to the UE should include at least: - gNB Rx-Tx time difference at a given granularity - FFS whether to include SRS-Resource-ID Agreement Take the following two alternatives as the equation for evaluation of the overall time synchronization error for RTT-based propagation delay compensation. RAN1 to select one of the alternatives in RAN1#106bis-e. - Alt. 1: - errorRxTxDiff, indication is to reflect the error due to indication granularity of Rx-Tx time difference - errorgNB,RxTxDiff and errorUE, RxTxDiff reflects the measurement inaccuracy of gNB Rx-Tx time difference, and the measurement inaccuracy of UE Rx-Tx time difference, respectively. - Note: The equation may be updated after clarification on the gNB TX-RX timing difference and UE TX-RX timing difference - Alt. 2: - errorRxTxDiff,indication is to reflect the error due to indication granularity of Rx-Tx time difference - Note: Alt.2 assumes that gNB can coordinate the time of TA procedure and the time of PD compensation, so that the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to (e.g. the same as) that for the transmission of RRC signaling carrying the reference time clock Note: FFS whether / how to handle inconsistent RTT measurement in gNB and UE due a change of uplink TX timing |

#### 2.1.2	Remaining Open issues

- UE feedback enhancements for HARQ-ACK (RAN1 AI 8.3.1.1)

  - Remaining details of deferral of dropped SPS HARQ-ACK due to cancelled PUCCH for TDD

  - Remaining details of HARQ-ACK re-transmission

  - Remaining details of (sub-slot based) PUCCH repetition enhancements

  - Remaining details of Type 1 HARQ-ACK codebook for sub-slot PUCCH

  - Remaining details of PUCCH carrier switching for HARQ-ACK feedback

- CSI feedback enhancements (RAN1 AI 8.3.1.2)

  - Remaining details of 4-bit sub-band CQI reporting

- Enhancements for unlicensed band URLLC/IIoT (RAN1 AI 8.3.2)

  - Specify remaining details support for UE-initiated COT for FBE with minimum specification effort

  - Specify remaining details for harmonizing UL configured-grant enhancements in NR-U and URLLC introduced in Rel-16 to be applicable for unlicensed spectrum

- Intra-UE multiplexing enhancements (RAN1 AI 8.3.3)

  - Specify multiplexing behavior among HARQ-ACK/SR/CSI and PUSCH for traffic with different priorities, including the cases with UCI on PUCCH and UCI on PUSCH.

  - Remaining details of PHY prioritization of overlapping dynamic grant PUSCH and configured grant PUSCH of different PHY priorities on a BWP of a serving cell including the related cancelation behavior for the PUSCH of lower PHY priority, taking the solution developed during Rel-16 as the baseline

  - Details of support of simultaneous PUCCH / PUSCH

- Propagation delay compensation / Time Synchronization (RAN1 AI 8.3.4)

  - Finalization of evaluation assumptions for propagation delay compensation

  - Specify propagation delay compensation enhancements (if any)

## 2.2	RAN2

#### 2.2.1	Agreements

| Time Synchronization (RAN2 AI 8.5.2) | Agreements 1. RAN2 assumes that gNB can perform pre-compensation. RAN2 agrees to introduce signalling to enable/disable UE-side PDC. 2. The gNB can enable/disable UE-side PDC via unicast-RRC signalling for Rel-17 3. RAN2 shall wait for RAN1 to decide the measurement framework for RTT based PDC method and does not preclude UE-side PDC or gNB based pre-compensation at this point. RAN2 is expecting guidance from RAN1 on what is needed. 4. UE Assistance information from the UE which could for example be used by gNB to activate PDC is not supported 5. Implicit activation of UE-side PDC when a pre-configured threshold is met is not supported 6. UE-based trigger for TA update or RACH procedure for PDC are deprioritized for Release 17 |
| --- | --- |
| URLLC in Unlicensed (RAN2 AI 8.5.3) | Agreements 1. When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism is used for HARQ process ID and RV selection 2. When cg-RetransmissionTimer and lch-basedPrioritization are configured, for overlapping CGs that do not share HARQ processes, the MAC entity prioritizes the initial transmission of higher priority data over autonomous retransmission of lower priority data. No specification change is foreseen 3. The same HARQ PID selection rule applies to all CGs when HARQ processes are shared between multiple CG configurations with non-overlapping CG occasions and with the same TBS. No specification change is foreseen 4. It is up to NW implementation to appropriately configure CGs that share HARQ processes with autonomousTx. No specification change is foreseen 5. When lch-basedPrioritization and cg-RetransmissionTimer are both configured, the gNB can configure the UE per MAC entity whether it follows Rel-16 baseline or whether it prioritizes high priority data when selecting HARQ PID for a CG (i.e. option 2 is configurable). 6. The same HARQ PID selection rule applies to all CGs when HARQ processes are shared between multiple CG configurations with overlapping CG occasions with the same TBS. No specification change is foreseen |
| RAN enhancement for New QoS (RAN2 AI 8.5.4) | Agreements 1. RAN2 does not assume that physical HARQ-NACK messages are always available, i.e. RAN2 will not mandate explicit HARQ-NACK feedback 2. Given the application message size range under study, RAN2 will not optimize the ST design based on case of segmentation of message into multiple TBs. (This does not preclude the use of RLC segmentation; instead, it rules out optimizations for the case with RLC segmentation) 3. Following entry into the Survival Time state, PDCP duplication for ST configuration is activated. The gNB pre-configures which RLC entities can be activated for duplication when entering ST state. FFS the number of supported RLC entities. 4. RAN2 will at least continue working and discussing the HARQ NACK solution. Details are FFS. |

#### 2.2.2	Remaining Open issues

- Enhancements for support of time synchronization (RAN2 AI 8.5.2)

  - Details of signaling/messages to support propagation delay compensation mechanisms. According to the latest RAN2 agreement, RAN1 input is needed for RAN2 to proceed the related work.

- Uplink enhancements for URLLC in unlicensed controlled environments (RAN2 AI 8.5.3)

  - Some clarification about behaviour of CG retransmission timer upon deprioritization. For instance, whether autonomous transmission can be conducted to recover the deprioritized MAC PDU.

- RAN enhancements based on new QoS (RAN2 AI 8.5.4)

  - Details (RRC configuration and MAC procedures) about UE-based reactive trigger based on HARQ NACK solution.

  - Details about pre-configuration of PDCP duplication for survival time state

  - Clarifications of issues relating to resource availability upon survival time state triggering.

## 2.3	RAN3

#### 2.3.1	Agreements

| Time Synchronization (RAN3 AI 21.2) | No new agreements. |
| --- | --- |
| Enhancements based on new QoS related parameters (RAN3 AI 21.3) | - Agreements - - The working assumptions is agreed, i.e. supporting the Survival Time for both downlink and uplink. - - The granularity of the survival time is 1 us (i.e. the same as the Periodicity IE). - - The maximum value of the survival time should be at least 3 times the maximum value of the Periodicity IE. - - No RAN3 actions are needed for the TSN service in acknowledge mode, unless further action is required by other groups. - - Keep the current Survival time encoding unchanged in the BLCRs. - Baseline CRs were re-endorsed: TS 38.413 BL CR in R3-213172 TS 38.423 BL CR in R3-213173 TS 38.463 BL CR in R3-213174 TS 38.473 BL CR in R3-213175 TP for TS 38.423 BL CR agreed in R3-213774 correcting ASN.1. |

#### 2.3.2	Remaining Open issues

- RAN3 impacts of propagation delay compensation enhancements, if any.

  - For the time synchronization error budget, it is FFS for which level (e.g., UE level, PDU session level, or QoS flow level) and for its details (e.g., parameter name, parameter meaning, and value range)

  - It is FFS on whether assistance information (e.g., UE TSN timing reference, referenceTimeInfo delivery periodicity, timestamp) should be delivered during HO.

- RAN3 impacts of new QoS related parameters, e.g. survival time, burst spread, decided in SA2.

  - The maximum value for the Survival Time is FFS.

## 2.4	RAN4

#### 2.4.1	Agreements

| RRM core requirements (RAN4 AI 9.23.2, thread 239) | Agreements - WF on RRM for NR IIoT and URLLC was agreed (R4-2115371). - Continue to study the RRM impact from PUCCH carrier switching and discuss introduction of new test case(s) once the need for new RRM requirements have been concluded. - If new Te requirements are agreed in the future, these should be captured in a compatible manner (no impact on legacy devices and UEs not supporting the feature). - RAN4 waits for the PDC enhancements progress in RAN1 before discussing further. - Further discuss the wordings for the definition of reference point/downlink timing based on ‘The downlink timing is defined as the time, when the first [detected] path in time of the corresponding downlink frame from the reference cell [arrives/is received] at the UE antenna.’ |
| --- | --- |

#### 2.4.2	Remaining Open issues

RRM core requirements:

- RAN4 impacts of PUCCH carrier switching, if any.

- RAN4 impacts of propagation delay compensation enhancements, if any.

- whether it should be clarified that the reference point/downlink timing refer to the signal of the first path being received/arrives at the UE antenna.

RRM performance requirements are not started yet.

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

  - 30 normative CRs approved for stage 2 specifications - TS 23.501/23.502/503

  - 3 LS sent – 1 to ITU-T SG15 Q13, 1 to IEEE 1588, 1 to CT1 on Time Synchronization

  - All the open items identified in the exception sheet have been resolved:

    - AF influence of Time synchronization accuracy error budget (S2-2106771, S2-2106772)

    - AF influence of the 5G reference time is performed (S2-2106771, S2-2106772)

    - Pending clean up and/or fixes due to TSCTSF NF introduction, handling of PMIC (e.g. due to triggers such as validity timer expiry) when UE is in idle mode (many CRs).

#### 3.1.2	Remaining Open issues with cross-TSG impacts

All open issues are resolved, the WI of IIoT is considered as completed in SA2.

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	RAN Plenary Intervention

Based on the observations of the current status, it is proposed by the WI rapporteur that some RAN Plenary intervention is needed to provide some guidance about how WGs should proceed with the topics. In particular, for Intra-UE multiplexing and prioritization the overall progress in RAN1 so far has been rather limited and therefore some RAN guidance on focus or down-scoping could enable to still complete at least a sub-set of Intra-UE multiplexing and prioritization features in Rel-17.  Furthermore, some RAN plenary guidance may be needed to resolve the unclear situation regarding the options of propagation delay compensation mechanisms for time-synchronization in RAN1, that currently seem to hamper related progress and put the timely completion in Rel-17 at risk. On the other hand, in order to ensure the WI can complete in time, from the rappateur perspective it is also very critical that RAN2 should sharpen the focus for the objective on New QoS parameters of Survival Time, by concentrating on the solution that RAN2 has agreed to focus on. More details of the WI rapporteur view and proposals can be found in RP-211939.

## 5.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#106-e Tdocs List

R1-2106490	UE feedback enhancements for HARQ-ACK	Huawei, HiSilicon

R1-2106491	CSI feedback enhancements	Huawei, HiSilicon

R1-2106492	Intra-UE multiplexing enhancements	Huawei, HiSilicon

R1-2106493	Uplink enhancements for URLLC in unlicensed controlled environments	Huawei, HiSilicon

R1-2106586	HARQ-ACK enhancements for Rel-17 URLLC	vivo

R1-2106587	CSI feedback enhancements for Rel-17 URLLC	vivo

R1-2106588	Enhancements for unlicensed band URLLC/IIoT	vivo

R1-2106589	Intra-UE Multiplexing/Prioritization for Rel-17 URLLC	vivo

R1-2106590	Discussion on propagation delay compensation enhancements	vivo

R1-2106636	HARQ-ACK Feedback Enhancements for URLLC/IIoT	Nokia, Nokia Shanghai Bell

R1-2106637	On UL intra-UE prioritization and multiplexing enhancements	Nokia, Nokia Shanghai Bell

R1-2106638	Discussion on enhancements for propagation delay compensation	Nokia, Nokia Shanghai Bell

R1-2106639	Moderator summary #1 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2106678	HARQ-ACK Enhancements for IIoT/URLLC	Ericsson

R1-2106679	CSI Feedback Enhancements for IIoT/URLLC	Ericsson

R1-2106680	Enhancements for IIoT/URLLC on Unlicensed Band	Ericsson

R1-2106681	Intra-UE Multiplexing/Prioritization Enhancements for IIoT/URLLC	Ericsson

R1-2106682	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R1-2106697	Discussion on HARQ-ACK feedback enhancements for Rel-17 URLLC	Spreadtrum Communications

R1-2106698	Discussion on CSI feedback enhancements	Spreadtrum Communications

R1-2106699	Discussion on enhancements for unlicensed band URLLC/IIoT	Spreadtrum Communications

R1-2106700	Discussion on intra-UE multiplexing/prioritization	Spreadtrum Communications

R1-2106734	Discussion on HARQ-ACK enhancements for eURLLC	ZTE

R1-2106735	Discussion on CSI feedback enhancements for eURLLC	ZTE

R1-2106736	Discussion on unlicensed band URLLC/IIoT	ZTE

R1-2106737	Discussion on enhanced intra-UE multiplexing	ZTE

R1-2106738	Discussion on propagation delay compensation enhancements	ZTE

R1-2106764	UL enhancements for IIoT/URLLC in unlicensed controlled environment	Nokia, Nokia Shanghai Bell

R1-2106801	Considerations on HARQ-ACK enhancements for URLLC	Sony

R1-2106802	Considerations on CSI enhancements for URLLC	Sony

R1-2106803	Considerations on Unlicensed URLLC	Sony

R1-2106804	Considerations on UL Intra-UE Multiplexing	Sony

R1-2106837	Discussion on CSI Feedback Enhancements	Quectel, Langbo

R1-2106838	Discussion on Intra-UE Multiplexing/Prioritization	Quectel, Langbo

R1-2106879	On HARQ-ACK reporting enhancements	Samsung

R1-2106880	UE Feedback Enhancements for URLLC	Samsung

R1-2106881	Enhancements for unlicensed band URLLC/IIoT	Samsung

R1-2106882	Uplink intra-UE multiplexing and prioritization	Samsung

R1-2106883	Discussion for propagation delay compensation enhancements	Samsung

R1-2106962	UE feedback enhancements for HARQ-ACK	CATT

R1-2106963	CSI feedback enhancements	CATT

R1-2106964	Discussion on remaining issues on enhancements for unlicensed band URLLC/IIoT	CATT

R1-2106965	Intra-UE multiplexing and prioritization	CATT

R1-2106966	Discussion on propagation delay compensation enhancements	CATT

R1-2107013	Enhancements for unlicensed band URLLC/IIoT	NEC

R1-2107019	CSI feedback enhancements for URLLC/IIoT use cases	Nokia, Nokia Shanghai Bell

R1-2107025	Discussion on UE feedback enhancements for HARQ-ACK	Panasonic

R1-2107073	Intra-UE multiplexing and prioritization	InterDigital, Inc.

R1-2107074	CSI feedback enhancements	InterDigital, Inc.

R1-2107076	Updated NR_NTN_solutions work plan	THALES

R1-2107078	CSI feedback enhancements for URLLC	FUTUREWEI

R1-2107103	UE initiated COT for semi-static channel access	FUTUREWEI

R1-2107114	Enhancements for unlicensed band URLLC/IIoT	Panasonic Corporation

R1-2107115	Discussion on Intra-UE multiplexing of different priority	Panasonic Corporation

R1-2107132	Discussion on intra-UE multiplexing and prioritization	China Telecom

R1-2107133	Discussion on UE feedback enhancements for HARQ-ACK	China Telecom

R1-2107156	UE feedback enhancements for HARQ-ACK	NEC

R1-2107157	Discussion on Intra-UE prioritization and multiplexing	NEC

R1-2107180	HARQ-ACK feedback enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2107181	Intra-UE multiplexing enhancement for IIoT/URLLC	Lenovo, Motorola Mobility

R1-2107185	CSI feedback enhancements for URLLC/IIoT	Lenovo, Motorola Mobility

R1-2107186	Enhancements for unlicensed band URLLC/IIoT	Lenovo, Motorola Mobility

R1-2107200	Discussion on Intra-UE Multiplexing and Prioritization	Potevio Company Limited

R1-2107272	HARQ-ACK enhancements for Rel-17 URLLC/IIoT	OPPO

R1-2107273	CSI feedback enhancements for URLLC	OPPO

R1-2107274	Enhancements for unlicensed band URLLC/IIoT	OPPO

R1-2107275	Enhancements on intra-UE multiplexing/prioritization	OPPO

R1-2107276	Enhancement for support of time synchronization	OPPO

R1-2107294	Enhancements for unlicensed band URLLC/IIoT	FGI, Asia Pacific Telecom

R1-2107295	Discussion on Intra-UE multiplexing/prioritization	FGI, Asia Pacific Telecom

R1-2107296	Discussion on UE feedback enhancements for HARQ-ACK	FGI, Asia Pacific Telecom

R1-2107336	HARQ-ACK enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2107337	CSI enhancement for IOT and URLLC	Qualcomm Incorporated

R1-2107338	Uplink enhancements for URLLC in unlicensed controlled environments	Qualcomm Incorporated

R1-2107339	Intra-UE multiplexing and prioritization for IOT and URLLC	Qualcomm Incorporated

R1-2107340	Enhancements for support of time synchronization for enhanced IIoT and URLLC	Qualcomm Incorporated

R1-2107397	Discussion on UE feeback enhancements for HARQ-ACK	CMCC

R1-2107398	Discussion on CSI feeback enhancements for URLLC	CMCC

R1-2107443	Discussion on UE feedback enhancement for HARQ-ACK	LG Electronics

R1-2107444	Discussion on CSI feedback enhancements for URLLC	LG Electronics

R1-2107445	Discussion on unlicensed band URLLC/IIOT	LG Electronics

R1-2107446	Discussion on Intra-UE multiplexing/prioritization	LG Electronics

R1-2107447	Discussion on propagation delay compensation enhancements	LG Electronics

R1-2107472	UE feedback enhancements for HARQ-ACK	ETRI

R1-2107473	Enhancements for unlicensed band URLLC/IIoT	ETRI

R1-2107474	Intra-UE Multiplexing/Prioritization	ETRI

R1-2107491	On UE feedback enhancements for HARQ-ACK	MediaTek Inc.

R1-2107492	CSI feedback enhancements for URLLC	MediaTek Inc.

R1-2107493	On the enhancements for unlicensed band URLLC/IIoT	MediaTek Inc.

R1-2107494	Methods for intra-UE multiplexing and prioritization	MediaTek Inc.

R1-2107495	Discussion on propagation delay compensation for time synchronization	MediaTek Inc.

R1-2107583	Design aspects for the agreed HARQ feedback enhancements	Intel Corporation

R1-2107584	On enhanced SB CQI reporting granularity and delta-MCS reporting	Intel Corporation

R1-2107585	On the Details for Enabling URLLC/IIoT in Unlicensed Band	Intel Corporation

R1-2107586	Further details of intra-UE uplink channel multiplexing and prioritization	Intel Corporation

R1-2107587	Further analysis and design considerations for propagation delay compensation methods	Intel Corporation

R1-2107639	HARQ enhancements for IIoT and URLLC	InterDigital, Inc.

R1-2107640	Enhancements for unlicensed band URLLC/IIoT	InterDigital, Inc.

R1-2107678	Enhancements for support of time synchronization	Huawei, HiSilicon

R1-2107732	HARQ Feedback Enhancements for URLLC	Apple

R1-2107733	CSI feedback enhancements for URLLC	Apple

R1-2107734	URLLC uplink enhancements for unlicensed spectrum	Apple

R1-2107735	Design considerations on Rel-17 intra-UE multiplexing/prioritization	Apple

R1-2107791	UE feedback enhancements for HARQ-ACK	Sharp

R1-2107792	Enhancements for unlicensed band URLLC/IIoT	Sharp

R1-2107793	Enhancements of intra-UE UCI multiplexing on PUCCH and PUSCH	Sharp

R1-2107833	UE feedback enhancements for HARQ-ACK	TCL Communication Ltd.

R1-2107834	Intra-UE multiplexing/prioritization	TCL Communication Ltd.

R1-2107851	Discussion on HARQ-ACK feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2107852	Discussion on CSI feedback enhancements for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2107853	Discussion on enhancements for unlicensed band URLLC	NTT DOCOMO, INC.

R1-2107854	Discussion on intra-UE multiplexing/prioritization for Rel.17 URLLC	NTT DOCOMO, INC.

R1-2107917	UE feedback enhancements for HARQ-ACK	Xiaomi

R1-2108012	Views for Increasing Granularity of Subband CQI	ITRI

R1-2108013	Discussion on intra-UE multiplexing	ITRI

R1-2108152	Discussion on HARQ-ACK enhancement for URLLC/IIoT	WILUS Inc.

R1-2108153	Discussion on enhancement for unlicensed URLLC/IIoT	WILUS Inc.

R1-2108154	Discussion on intra-UE multiplexing/prioritization for URLLC/IIoT	WILUS Inc.

R1-2108162	UE feedback enhancements for HARQ-ACK	CAICT

R1-2108200	Feature lead summary#1 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2108235	Feature lead summary #1 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2108236	Feature lead summary #2 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital, Inc.)

R1-2108237	CSI feedback enhancements	InterDigital, Inc.

R1-2108274	Summary#1 of email thread [106-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2108301	Summary#1 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2108302	Summary#2 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2108303	Summary#3 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2108304	Summary#4 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2108305	Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2108306	Summary#6 - Enhancements for IIOT/URLLC on Unlicensed Band	Moderator (Ericsson)

R1-2108336	Summary#2 of email thread [106-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2108384	Feature lead summary#2 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2108402	Summary#3 of email thread [106-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2108440	Moderator summary #2 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2108449	Feature lead summary #3 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital)

R1-2108450	Feature lead summary #4 on CSI feedback enhancements for enhanced URLLC/IIoT	Moderator (InterDigital)

R1-2108513	Feature lead summary#3 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2108546	Moderator summary #3 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2108547	Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT	Moderator (Nokia)

R1-2108556	Summary#4 of email thread [106-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2108604	Session notes for 8.4 (Solutions for NR to support non-terrestrial networks (NTN))	Ad-Hoc Chair (Ericsson)

R1-2108617	Feature lead summary#4 on propagation delay compensation enhancements	Moderator (Huawei)

R1-2108618	Draft LS on TA-based propagation delay compensation	Moderator (Huawei)

R1-2108628	Summary#5 of email thread [106-e-NR-R17-IIoT-URLLC-04]	Moderator (OPPO)

R1-2108634	Final feature lead summary on propagation delay compensation enhancements	Moderator (Huawei)

R1-2108635	LS on TA-based propagation delay compensation	RAN1, Huawei

RAN2#115-e Tdocs List

R2-2108019	Summary of Email Discussion [Post114-e][509][URLLC/IIoT] Running Stage 2 CR review (Nokia)		Nokia, Nokia Shanghai Bell

R2-2108020	Stage-2 Running CR for Rel-17 IIoT/URLLC		Nokia, Nokia Shanghai Bell

R2-2108296   Report of email discussion [Post114-e][512][URLLC/IIoT] T-synch open issues (Intel)	Intel

R2-2107116	Triggered Synchronization Activation			CANON Research Centre France

R2-2107152	Discussion about time synchronization enhancements		Huawei, HiSilicon

R2-2107528	RE: LS on Time Synchronization	IEEE 1588 WG	LS in	To:RAN, SA	Cc:RAN2

R2-2107556	Propagation Delay Compensation for TSN		Qualcomm Incorporated

R2-2107736	Consideration on the support of time synchronization enhancement		OPPO

R2-2107741	Remaining issues on time synchronization and PDC	ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd

R2-2107800	Discussion on propagation delay compensation	vivo

R2-2107897	Left issues for propagation delay compensation	Lenovo, Motorola Mobility

R2-2108021	Time Synchronization Signalling	Nokia, Nokia Shanghai Bell

R2-2108097	Summary of PDC Issues	Ericsson

R2-2108168	Discussion on RAN enhancement to support propagation delay compensation	China Telecommunications

R2-2108258	Issues on Propagation Delay Compensation	Samsung

R2-2108436	Leftover aspects on Timing Synchronization	Intel Corporation

R2-2108547	Support of time synchronization for TSN based on RAN1 progress	CMCC

R2-2108553	Discussion on enhancements for support of time synchronization	LG Electronics Inc.

R2-2108793	Discussion on the PDC support for IDLE or CONNECTED	Xiaomi Communications

R2-2108803	Timing synchronization for UE in RRC_INACTIVE state and RRC_IDLE state	TCL Communication Ltd.

R2-2108815	Discussion on uplink time synchronization for TSN	NTT DOCOMO, INC.

R2-2108231   Summary of [Post114-e][510][URLLC/IIoT] Open issues for UCE	MediaTek Inc.

R2-2107153	Remaining issues about Uplink enhancements for URLLC in UCE	Huawei, HiSilicon

R2-2107201	Sequential processing of autonomous retransmission and lch-based prioritization	CATT

R2-2107202	Time-based HPID for gNB-scheduled dynamic retransmissions	CATT

R2-2107557	CG Harmonization for Unlicensed Controlled Environment	Qualcomm Incorporated

R2-2107737	Consideration on URLLC over NR-U		OPPO

R2-2107801	Remaining issues about autonomous re-transmission		vivo

R2-2107896	Further details on enhancements for URLLC in UCE		Lenovo, Motorola Mobility

R2-2108022	Remaining Issues of URLLC in NR-Unlicensed		Nokia, Nokia Shanghai Bell

R2-2108098	Harmonizing UL CG enhancements in NR-U and URLLC		Ericsson

R2-2108270	Further Consideration On the URLLC transmission in UCE		ZTE Corporation

R2-2108667	IIoT operation in unlicensed controlled environment		InterDigital

R2-2108674	Uplink enhancements for URLLC in unlicensed controlled environments	Intel Corporation

R2-2108748	Remaining issues of harmonizing UL CG enhancements for IIoT in UCE	III

R2-2108758	Issues on Prioritization in UCE		Samsung

R2-2108794	Remaining issues of CG harmonization	Xiaomi Communications

R2-2108810   Retransmission of UCI-only MAC PDU		LG Electronics UK

R2-2107173	Report from email discussion [Post114-e][511][URLLC/IIoT] QoS Solutions (Samsung)		Samsung Electronics GmbH

R2-2107154	Discussion on two-level PERs for survival time handling		Huawei, HiSilicon

R2-2107174	Entering, operating in, and exiting the Survival Time state	Samsung Electronics GmbH

R2-2107203	UE-based reactive solution for survival time	CATT

R2-2107558	RAN Enhancement to support Survival Time	QUALCOMM Europe Inc. - Italy

R2-2107611	Reliability enhancements for CG/SPS	Apple

R2-2107612	Further considerations on survival time for new QoS	Apple

R2-2107658	L1/L2 configuration adaptation		Fujitsu

R2-2107738	Consideration on RAN enhancement based on new QoS	OPPO

R2-2107742	Remaining issues on enhanced QoS		ZTE Corporation, Sanechips, China Southern Power Grid Co., Ltd

R2-2107802	Consideration on reactive solution for survival time	vivo

R2-2107806	Further discussions on RAN enhancements based on Survival Time		III

R2-2107895	Discuss on the mechanism to guarantee the survival time	Lenovo, Motorola Mobility

R2-2108023	Analysis of Potential RAN Enhancements for Survival Time	Nokia, Nokia Shanghai Bell

R2-2108099	RAN enhancements based on new QoS related parameters	Ericsson

R2-2108169	Discussion on RAN enhancement to support new QoS 		China Telecommunications

R2-2108435	UE-based Survival time handling		Intel Corporation

R2-2108457	ST handling with alternating CC allocations			Sequans Communications

R2-2108459	Discussion on avoiding prematurely entering Survival Time state	Futurewei Technologies

R2-2108516	Discussion on the RAN support for new QoS parameters	CMCC

R2-2108666	Enhancements based on new QoS requirements		InterDigital

R2-2108786	Progress of QoS		LG Electronics UK

R2-2108795	  Clarification on the survival time requirement	Xiaomi Communications

RAN3#113-e Tdoc List

R3-213172	Introduction of Enhanced IIoT support over NG	CATT, Nokia, Nokia Shanghai Bell, Samsung, Huawei, Ericsson, ZTE

R3-213173	Introduction of Enhanced IIoT support over Xn	Ericsson, Samsung, Huawei, ZTE,CATT, Nokia, Nokia Shanghai Bell

R3-213174	Introduction of Enhanced IIoT support over E1	ZTE, Nokia, Nokia Shanghai Bell, Samsung, CATT, Huawei, Ericsson

R3-213175	Introduction of Enhanced IIoT support over F1	Huawei, Nokia, Nokia Shanghai Bell, CATT, Ericsson, ZTE, Samsung

R3-213774	[TP for BL CR for TS 38.423]: Correction to the Baseline CR	Ericsson

R3-213241	Further discussion on Propagation Delay Compensation enhancements	ZTE

R3-213377	Discussion on Further enhanced NR-IIoT: Enhancements for support of time synchronization	Ericsson

R3-213378	Enhancements for support of time synchronization	Ericsson

R3-213379	Enhancements for support of time synchronization	Ericsson

R3-213449	Impact of handover on time synchronization	Nokia, Nokia Shanghai Bell

R3-213645	(TP for eIIoT BLCR for 38.423) Propagation delay compensation and timing accuracy during handover	Huawei

R3-213646	(TP for eIIoT BLCR for 38.413) Propagation delay compensation and timing accuracy during handover	Huawei

R3-213907	Discussion on supporting the time synchronization error budget	Samsung

R3-213951	Discussion on Propagation Delay Compensation Enhancements	CATT

R3-213242	(TP for Introduction of Enhanced IIoT support) Analysis of New QoS Related parameters	ZTE

R3-213243	(TP for Introduction of the enhanced IIoT over E1) New QoS related parameters	ZTE

R3-213447	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.413) Resolution of TSCAI open issues	Nokia, Nokia Shanghai Bell

R3-213448	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) Impact of handover on Survival Time	Nokia, Nokia Shanghai Bell

R3-213647	(TP for eIIoT BLCR for 38.473) Survival time remaining issues	Huawei

R3-213910	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.413) Discussion on the remaining issues for the Survival Time	Samsung

R3-213913	(TP for NR_IIOT_URLLC_enh BL CR for TS 38.423) The maximum value for the Survival Time	Samsung

R3-213952	Discussion on new QoS related parameters	CATT

R3-213953	(TP for [NR_IIOT_URLLC_enh-Core] BLCR for TS 38413) new QoS related parameters	CATT

R3-214186	Summary of offline: NRIIOT2-PDC	ZTE (moderator)

R3-214187	Summary of Offline Discussion on New QoS Related Parameters	Huawei (moderator)

RAN4#100-e Tdoc List

R4-2112214	Discussion on propagation delay enhancement	CMCC

R4-2112215	Discussion on reference point of UE transmit timing error	CMCC

R4-2112556	RRM impacts overview for IIoT/URLLC support for NR	vivo

R4-2112557	Discussion on propagation delay compensation enhancements	vivo

R4-2112558	Discussion on reference point for Te requirements	vivo

R4-2113144	Further discussion on RRM requirements for IIoT and URLLC in Rel-17	Intel Corporation

R4-2113288	Reference point for Te requirements for NR_IIOT_URLLC_enh	OPPO

R4-2113520	Propagation Delay Compensation Enhancements for Time Synchronization	Ericsson

R4-2113882	Discussion on the reference point for Te requirements	ZTE Corporation

R4-2113981	Propagation delay compensation enhancements	MediaTek Inc.

R4-2113982	Reference point for Te requirements	MediaTek Inc.

R4-2114009	Further discussion of the reference point for UE transmit timing requirement	Nokia, Nokia Shanghai Bell

R4-2114027	Discussion of RRM impact for PUCCH carrier switching	Nokia, Nokia Shanghai Bell

R4-2114029	Status of Propagation delay compensation enhancements and expected RAN4 impacts	Nokia, Nokia Shanghai Bell

R4-2114315	Discussion on RRM impacts of PUCCH carrier switching	Huawei, HiSilicon

R4-2114316	Discussion on RRM impacts of PDC enhancements	Huawei, HiSilicon

R4-2114317	Discussion on reference point for Te requirements	Huawei, HiSilicon

R4-2114450	LS response on UE transmit timing error	Ericsson, Intel

R4-2115371	WF on NR_IIOT_URLLC_enh_RRM	Nokia, Nokia Shanghai Bell

R4-2115414	Email discussion summary for [100-e][239] NR_IIOT_URLLC_enh_RRM Moderator (Nokia)

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template