3GPP TSG RAN meeting #93e										RP-212000

Electronic Meeting, Sep. 13-17, 2021

Status Report to TSG

Agenda item:			9.3.4.1

| WI / SI Name | Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) requirements for NR UEs |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_MIMO_OTA |  |  |  |  |
| Unique ID | 880078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201800 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: Mar/2022 | Performance part: Sep/2022 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 83% | Performance Part: 20% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG4 |
| --- | --- | --- |
| Rapporteur | Name | Zhu, Siting; Xing, Jinqiang |
|  | Company | CAICT, OPPO |
|  | Email | zhusiting@caict.ac.cn xingjinqiang@oppo.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.4	RAN4

2.4.1	Agreements

RAN4 #100-e (Aug 2021, Electronic Meeting)

- General

- WF on NR MIMO OTA was approved [2]

- 3GPP TS 38.151 v0.5.0 was approved [3]

- TP to TS38.151 v0.4.0 on Channel Model and DUT Positioning Clarifications was approved [4]

- TP to TS38.151 v0.4.0 on Minimum Number of Slots and Power Control was approved [5]

- Time plan for FR1 lab alignment and requirement development was approved [6]

- Reference Channel Emulation Curves for Validation Purposes was approved [7]

- Base Station beam configuration

- The base Station beam configuration stated in R4-2115757 is agreed.

- Channel model validation for FR1

- Reference validation targets for FR1

•	For CDL-C UMa and CDL-C UMi, reference channel emulation curves in R4-2115759 is endorsed;

•	Companies are encouraged to bring proposals and analysis for pass limit of channel validation and target to conclude in Nov 2021 RAN4 meeting.

    - A check point for offline alignment among interested parties before November is encouraged.

    - Given the reference emulation curves are based on the average approach from several companies, the pass/fail limit should consider different implementations.

- FR2 probe blocking issue

- The baseline assumption there is no remaining open blocking issue pending on further confirmation by Nov 2021 RAN4 meeting.

- The FR2 QoQZ validation is performed with Probe #1.

- FR2 probe weights

- Given probe weights belongs to implementation, no further effort on alignment of probe weights.

- Time plan for FR1 lab alignment and requirement development

- The time plan for FR1 lab alignment and requirement development in R4-2115758 is agreed.

- Measurement device handling

- RAN4 will discuss how to handle measurement device before starting the measurement campaign.

- Data processing in FR1 TRMS derivation

- Percentile value of CDF

•	The percentile value of CDF to derive final TRMS requirements will be discussed after lab alignment is finalized and before test campaign.

- Interpolation method in CDF data processing

•	Linear interpolation is selected as starting point for measurement results CDF processing.

- FR1 MU assessment

- RAN4 should discuss the preliminary MU assessment for FR1 MIMO OTA including example expanded uncertainty, final MU and TT will be decided by RAN5.

- WID is suggested to update in Sep RAN plenary meeting, to reflect this working scope extension, for easy tracing the progress in the status report of WI.

- FR2 maximum downlink power and Figure of Merit

- FR2 maximum downlink power

•	Maximum downlink power shall be specified at centre of QZ.

•	Companies’ input on this topic is encouraged.

•	Finalize max DL power level at next meeting.

- FR2 Figure of Merit

•	The FoM will be further discussed after the max downlink power is finalized.

- FR2 simulation

- Information for FR2 simulation

•	RAN4 should identify which information (except for FR2 probe weights) should be further provided and aligned to accelerate FR2 simulation activity.

- FR2 simulator alignment

•	Do a fundamental scenario simulation as Fig. 1 of R4-2113033 firstly, for FR2 MIMO OTA simulator alignment.

•	Conduct email discussion and target to finalize simulation calibration for step one as defined in R4-2110796 before RAN4 Nov. meeting.

    - The UE antenna assumptions (agreed in R4-2108085 only for simulation alignment)

      - UE antenna array: two panels 1x4 patches

      - UE antenna parameters and Beam forming: Follow TR 38.803

      - Polarization alignment: polarization aligned between UE and TE

2.4.2	Remaining Open issues

- Further discuss the pass/fail limit of channel model validation

- Further discuss the Maximum downlink RS-EPRE for FR2

- FR1/FR2 MIMO OTA lab alignment and requirement development activities

  - Lab volunteers submit channel model validation results

  - Lab volunteers finish the PADs measurement

  - Aligned labs share FR1 MIMO OTA measurement data

  - Further discuss FR2 simulation activity

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

N/A

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4#100-e

- R4-2115807	Email discussion summary for [100-e][331] NR_MIMO_OTA, Moderator (Samsung)

- R4-2115756	WF on NR MIMO OTA, vivo, CAICT, OPPO

- R4-2112976	3GPP TS 38.151 v0.5.0, vivo

- R4-2115757	TP on Channel Model and DUT Positioning Clarifications, Keysight Technologies UK Ltd

- R4-2112979	TP to TS38.151 on Minimum Number of Slots and Power Control, vivo

- R4-2115758	Time plan for FR1 lab alignment and requirement development, CAICT, OPPO, vivo

- R4-2115759	Reference Channel Emulation Curves for Validation Purposes, Keysight Technologies UK Ltd, Spirent Communications, CMCC, CAICT

- R4-2112977	Rapporteur input to TS38.151, vivo

- R4-2112573	Discussion on FR1 performance requirements, Samsung

- R4-2112980	Proposal on preliminary FR1 MIMO OTA MU value assessment, vivo,CAICT

- R4-2113914	Refinement on lab alignment activity, OPPO

- R4-2112245	Discussion on FR2 MIMO OTA performance requirements, Qualcomm Incorporated

- R4-2113033	Proposal on FR2 MIMO simulator alignment, MediaTek Beijing Inc.

- R4-2114504	Discussion on FR2 MIMO OTA simulation, Huawei, HiSilicon

- R4-2112978	TP to TS38.151 on BS beam configuration, vivo,CAICT

- R4-2114528	TP to TS 38.151 on FR1 2x2 BS beam selection, Huawei, HiSilicon

- R4-2114529	update simulation results on FR1 2x2 channel models, Huawei, HiSilicon

- R4-2114534	Discussion on probe weight, Huawei, HiSilicon

- R4-2114535	TP to TS38.151 v0.4.0 on FR2 Base Station beam configuration, Huawei, HiSilicon

- R4-2112862	Consideration on Probe#3 of FR2 MIMO OTA, CMCC

- R4-2113915	The FR2 blocking issue, OPPO

- R4-2114380	On Blocking Issue for FR2 MIMO OTA, Keysight Technologies UK Ltd

- R4-2113854	Channel Model Targets, Spirent Communications

- R4-2113858	Reference Channel Emulation Curves for FR1, CAICT, CMCC

- R4-2114382	Reference Channel Emulation Curves, Keysight Technologies UK Ltd

- R4-2114503	Discussion on FR2 channel model validation, Huawei, HiSilicon

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template