3GPP TSG RAN WG #93e                                                         RP-212004

e-Meeting, September 13th – 17th, 2021

Source: 	Intel Corporation

Title: 	Input to Conclusions for Study on Scenarios and Requirements for NR Positioning Use Cases

Agenda item:	9.2.2

Document for: 	Discussion and Decision

# Introduction

The RAN WG study item related to NR positioning support for in-coverage, partial coverage and out-of-coverage scenarios was approved with the following objectives [1]:

This study item includes the following objectives for positioning use cases, requirements, and scenarios for the UE in in-coverage, partial coverage, and out-of-coverage as per the network coverage definition in the current specifications: - Identify the positioning use cases and requirements for V2X and public safety, based on the existing 3GPP work and input from industry fora. - Identify potential deployment and operation scenarios Note: Studying the feasibility of the identified requirements is not in the scope of this SI. Note: The work in this SI shall take into account the outcomes and progresses of NR positioning study in both Rel-16 and Rel-17. Note: Work plan - RAN#91: TR skeleton, initial input on Objective 1 - RAN#92: Finalize Objective 1, initial input on Objective 2 - RAN#93: Finalize Objective 2, approve the TR (SI target completion)

RAN TSG has made good progress on study during RAN #91 and RAN #92 meetings. In this contribution, we summarize the RAN study on scenarios and requirements for NR positioning use cases as well as provide input for conclusion section of the TR 38.845.

# Summary and Proposed Updates

RAN TSG has approved v1.0.1 of the TR 38.845 in [2]. The following aspects of potential deployment and operation scenarios were captured and described in the TR:

- Network coverage:

  - In-coverage, partial coverage, out of network coverage

- Radio link:

  - Uu interface (downlink/uplink), PC5 interface (sidelink) and their combinations

- Position calculation entity:

  - Network based and UE based positioning

- UE types:

  - V2X: installed in a vehicle, a road-side unit or a device of vulnerable road user

- Spectrum:

  - V2X: ITS, licensed, unlicensed spectrum

In the next sub-sections, we propose modifications for some of the identified above aspects.

## Network coverage

The following FFS is added in editor note at the end of subsection 5.1 of the TR 38.845: “Editor’s note: FFS additional contents including a figure illustrating the three scenarios, the need for transitions between coverage states, the network coverage requirement of operation scenarios, etc.”

In our view, the current section capturing network coverage scenarios is self-contained and we do not see the strong motivation to capture additional details and therefore suggest removing editor note and FFS. One aspect that may be potentially clarified is that network coverage scenarios may change in time for mobile UEs and any of UEs can transition from in-coverage to partial coverage or out of coverage and vice versa.

Table 1: Text Proposal for section “5.1 Network coverage” of the TR 38.845

---------------------------------Unchanged text is omitted------------------------------------------- Three network coverage scenarios can be considered when at least two UEs are involved in positioning for V2X and public safety use cases. Taking the case of two UEs as an example, in-coverage scenario refers to the case where both UEs are inside the network according to TS 38.304 [8]. Partial coverage means that one UE remains inside the network coverage but the other UE is outside the network coverage. Out-of-coverage scenario refers to the case where both UEs are outside the network coverage according to TS 38.304 [8]. A UE may transition between in-coverage, partial coverage and out-of-coverage scenarios. ---------------------------------Unchanged text is omitted-------------------------------------------

  - Adopt text proposal provided in Table 1 for section “5.1 Network coverage” of the TR 38.845

## Radio link

The following FFS is added in editor note at the end of subsection 5.2 of the TR 38.845: “Editor’s note: FFS RAT independent positioning, Uu based solution with assistance of PC5 interface, PC5 based solution with assistance of Uu interface, etc.”

RAT independent solutions are valid options for UE positioning in V2X and PS use cases and can be used together with RAT dependent solutions, where applicable/available. Therefore, we propose the following update to the sub-section 5.2 of the TR 38.845:

Table 2: Text Proposal for section “5.2 Radio link” of the TR 38.845

---------------------------------Unchanged text is omitted------------------------------------------- A positioning operation can be called a Uu-based solution if it uses only measurements on Uu interface. An operation can be called a PC5-based solution if it uses only measurements on PC5 interface. An operation can be called a hybrid Uu & PC5 based solution if it uses measurements on both Uu and PC5 interfaces. In addition, combination of RAT-dependent and RAT-independent solutions (e.g., GNSS, sensors, etc.) can be used as a hybrid RAT-dependent & RAT-independent solution, where various technologies can complement each other, where it is applicable. ---------------------------------Unchanged text is omitted-------------------------------------------

  - Adopt text proposal provided in Table 2 for section “5.2 Radio link” of the TR 38.845

## Position calculation entity

The section 5.3 seems stable already and we do not see motivation for additional changes there.

## UE types

The following FFS is added in editor note at the end of subsection 5.4 of the TR 38.845: “Editor’s note: FFS additional contents including the battery capability and processing capability of different types, the power class in public safety, the vehicle types and antenna placement options defined in TR 37.885, etc.”

Considering design of power saving resource allocation schemes for sidelink communication, in our view it is worthwhile to mention this aspect in section 5.4 as it is obvious that UE installed in vehicle and RSU may have different sensitivity to power consumption comparing to a UE installed in a device of a vulnerable road user. Therefore, we propose the following modification for section 5.4 of the TR 38.845:

Table 3: Text Proposal for section “5.4 UE types” of the TR 38.845

---------------------------------Unchanged text is omitted------------------------------------------- For V2X use cases, a UE involved in positioning can be installed in a vehicle, a road side unit, or a device of a vulnerable road user. A UE installed in a device of a vulnerable road user may be more sensitive to power consumption comparing to UE installed in a vehicle or a road side unit. A UE in a vehicle or a road side unit can be equipped with a distributed antenna system where multiple antenna panels of a UE are installed in different locations. ---------------------------------Unchanged text is omitted-------------------------------------------

  - Adopt text proposal provided in Table 3 for section “5.4 UE types” of the TR 38.845

## Spectrum

The following FFS is added in editor note at the end of subsection 5.5 of the TR 38.845: “Editor’s note: FFS additional contents including the pros and cons, spectrum for public safety, etc.”. In our view, there is no strong need to have RAN level debate on the pros and cons of the specific spectrum as there is no objective to select/recommend specific spectrum and all options are applicable. Overall, positioning performance is dependent on signal bandwidth / amount of available spectrum, its utilization, etc. Therefore, we simply propose to clarify that mentioned spectrum is applicable for both Uu and PC5 air-interfaces as well as add potential spectrum for public safety use case.

Table 4: Text Proposal for section “5.5 Spectrum” of the TR 38.845

---------------------------------Unchanged text is omitted------------------------------------------- For V2X use case, the ITS-dedicated spectrum, the spectrum licensed to mobile network operators (including FR2), and the unlicensed spectrum can be considered for both Uu and PC5 interfaces, with a note that there is no mechanism corresponding to regulatory requirements to use unlicensed spectrum in Rel-17 NR sidelink. For public safety use case, the spectrum licensed to mobile network operators (including FR2), and the unlicensed spectrum can be considered for both Uu and PC5 interfaces. ---------------------------------Unchanged text is omitted-------------------------------------------

  - Adopt text proposal provided in Table 4 for section “5.5 Spectrum” of the TR 38.845

# Input to Conclusion Section of the TR 38.845

The conclusion section (Section 6) of the TR 38.845 needs to be completed. In this section, we provide input text proposal for discussion on conclusions of the study item:

Table 5: Text Proposal for section “6 Conclusions” of the TR 38.845

---------------------------------Unchanged text is omitted------------------------------------------- Technical report summarizes the positioning use cases and requirements for V2X and public safety use cases, based on the existing 3GPP work [3], [4], [7], [9] and input from industry fora [5] (please refer to Section 4.2 for summary on V2X and Section 4.3 for summary on public safety use cases). The main aspects of potential deployment and operation scenarios are provided including: - Network coverage: - In-coverage, partial coverage, out of network coverage scenarios - Radio link: - Measurements on Uu interface (downlink/uplink), PC5 interface (sidelink) and their combinations for UE positioning - Use of RAT-dependent and RAT-independent solutions - Position calculation entity: - Network based and UE based positioning solutions - UE types: - V2X: installed in a vehicle, a road-side unit or a device of vulnerable road user, wherein different UE types may have different power saving demands - Spectrum: - V2X: licensed, unlicensed spectrum for both Uu and PC5 air-interfaces and ITS for PC5 air-interface - Public safety: licensed and unlicensed for both Uu and PC5 air-interfaces. ---------------------------------Unchanged text is omitted-------------------------------------------

  - Adopt text proposal provided in Table 5 for section “6 Conclusions” of the TR 38.845

# Conclusion

In this contribution, we provided text proposals and input to conclusion section to finalize TR 38.845 prepared by RAN TSG as a part of study on scenarios and requirements for NR positioning in V2X and public safety use cases.

# References

- RP-201384, “New SID: Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases”, LGE, FirstNet, July 2020

- RP-211575, 3GPP TR “Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage NR positioning use cases”, v1.0.1, June 2021