3GPP TSG RAN meeting #93e										RP-212090

Electronic Meeting, Sept 13-17, 2021

## Status Report to TSG

Agenda item:			9.3.4.8

| WI / SI Name | Further enhancements of NR RF requirements for frequency range 2 (FR2) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RF_FR2_req_enh2 |  |  |  |  |
| Unique ID | 890059 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-211538 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 03/2022 | Performance part: 06/2022 | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: 55% | Performance Part: 25% | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Vasenkari, Petri |
|  | Company | Nokia |
|  | Email | Petri.j.vasenkari@nokia.com |

# 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

# 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4 #100e Agreements

R4-2114959    WF on Simultaneous RxTx capability for FR2 inter-band CA

R4-2114960    WF on WF on FR2 DL CA based on CBM

R4-2114961    WF on FR2 UL CA

R4-2114962    WF on DC-location

R4-2114963    WF on new FR2 CA BW class

R4-2112793    TR 38.851-0.2.0

R4-2112369    draftCR to 38.101-2 on the definition for inter-band DL CA CBM and Beam Management Reference Signal location for FR2 CA

R4-2114964    WF on UL gap for BPS

R4-2115107    LS on UL gap in FR2 RF enhancement

R4-2115332	WF on RRM requirements for FR2 Inter-band DL CA and UL CA

RAN4 #100e GTW Agreements

GTW Session [129]

Agreement: Add new enumerated value to beam management type in [Rel-17] so that a UE can support both IBM and CBM, i.e., ENUMERATED {ibm, cbm, both}.

- FFS on the applicability of requirements for UE supporting both capabilities.

- FFS whether to introduce it from early release.

Agreement: The capability to indicate whether the UE support CBM under multi-chain architecture is not introduced.

Agreement:

- For core requirements applicability in relation to BMRS location,

  - CBM inter-band CA requirements apply per-band with the BMRS configured in any one of the participating bands.

    - Introduce side condition for core requirement that BMRS can only be placed on PCC for the DL CA case with a single uplink.

    - FFS whether to set side condition only for the worst case

- For test cases, further discussion on setup for testing to reduce the test burden and send LS to RAN5.

Agreement: RAN4 agree to introduce the spherical coverage requirements based on IBM inter-band CA framework.

- FFS on the values for the requirements

- FFS whether there is PSD difference and what is the difference

- FFS check the impact of frequency separation

Agreement: RAN4 agree to introduce the REFSENS core requirement based on IBM inter-band CA framework

- FFS on the values for the requirements

- FFS whether there is PSD difference and what is the difference

- FFS check the impact of frequency separation

Issue 3-1-1: How to incorporate PA-PA interaction

- Proposals

  - Option 1: Included in CA MPR

  - Option 2: Included in relaxations X and Y

  - Option 3: No need to include.

- Agreement: Down-select to Option 1 and Option 2.

- Agreement: CBM between different frequency groups is feasible and study phase can be completed at least for architecture with multiple RF chain.

GTW Session [130]

- Agreement:

- Baseline is to verify that UE correctly behave without phantom and ensure the feasible requirement gain in Rel-17 with different test methods.

Agreement:

- “P-MPR report+peak EIRP without phantom”, X dB EIRP gain and P-MPR requirement of Y when UL gap is activated should be achieved compared to the case where no gap is activated

- Decide range for X value in this meeting for making decision in future meeting

  - Option 1: at least 6dB

  - Option 2: A value between 6dB and 3dB, which is typical in the field

- Further discussion on the definition of Y in this meeting

  - Option 1: Y is absolute value

  - Option 2: Y is the relative value of gain

  - Option 3: no P-MPR requirement of Y

- FFS on the implementation margin

Agreement:

- UL gap should be explicitly activated by NW via signaling

  - How can UE indicate the NW UL gap activation is needed?

    - Option 1: UE explicitly indicates to NW by signaling

    - Option 2: UE implicitly indicate to NW by P-MPR reporting. The exact P-MPR value is FFS.

  - Network can activate UL gap without the indication from UE

- UL gap should be explicitly deactivated by NW via signaling

  - How can UE indicate the NW UL gap deactivation is needed?

    - Option 1: UE explicitly indicates to NW by signaling

    - Option 2: UE implicitly indicate to NW by [TBD] reporting.

  - Network can deactivate UL gap without the indication from UE.

Agreement: Two approaches will be considered

- #1: UL gap should be explicitly configured and activated/deactivated directly by RRC signaling

- #2: UL gap should be explicitly configured by RRC and activated and deactivated by MAC CE

Agreement:

- The switching time should be included in gap period.

GTW Session [216]

- Agreements:

  - MRTD for inter-band CA in FR2 under CBM is 3us

    - For the receive time difference below X us no performance degradation is expected

    - For the receive time difference equal or higher than X us a performance degradation is allowed

      - Degradation of UE demodulation and [RRM] performance is allowed.

        - Note: companies are encouraged to bring more analysis on Demodulation and RRM performance impacts.

      - FFS on the performance degradation including affected symbols, slots

      - FFS on solutions to reduce performance degradation and whether and how to introduce restrictions for UE Rx beam change

Email discussion summaries (For information)

- R4-2114729	Email discussion summary for [100-e][129] NR_RF_FR2_req_enh2_Part_1

- R4-2115029	Email discussion summary for [100-e][129] NR_RF_FR2_req_enh2_Part_1

- R4-2115206	Email discussion summary: [100-e][216] NR_RF_FR2_req_enh2_RRM

- R4-2115391	Email discussion summary: [100-e][216] NR_RF_FR2_req_enh2_RRM

#### 2.4.2	Remaining Open issues

RF:

DL interband CA:

•	Define UE requirements for inter-band CA common beam management (CBM)

•	Study and if feasible define UE RF requirements for inter-band CA within the same freq. group (e.g. 28GHz + 28GHz) for IBM (on hold until there is operator request)

UL interband CA:

- Specify requirements for inter-band UL CA for two bands.

- Define requirements for CA_n257A-n259A based on IBM

UL gaps for self-calibration and monitoring:

- Study and, if feasible, introduce UE specific and NW configured gap for general self-calibration and monitoring purposes for

  - UE Tx power management (BPS)

  - Coherent UL-MIMO

DC location:

- A way to indicate DC location

  - detail on signaling framework of default DC location(s) with offset (static and dynamic)

CA BW classes:

- FR2 fallback group FBG3+2

  - Fallback behavior

- FBG3 with 100 MHz CC’s

- Denotation for new CA BW class in FR2 if FBG3 is introduced

RRM:

- How to capture the performance degradation given 3us MRTD

- Define RRM requirements for inter-band CA common beam management (CBM)

- Specify interruption requirements for inter-band UL CA independent beam management (IBM).

# 4.	References

RAN4 #100e 129 contributions submitted

| R4-2112379 | Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | Apple |
| --- | --- | --- |
| R4-2112380 | Draft CR for TS 38.101-2: FR2 inter-band DL CA requirements clarification | Apple |
| R4-2112793 | TR 38.851-0.2.0 | Nokia, Nokia Shanghai Bell |
| R4-2112901 | On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation |
| R4-2112902 | Draft CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation |
| R4-2112903 | Draft CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation |
| R4-2114729 | Email discussion summary for [100-e][129] NR_RF_FR2_req_enh2_Part_1 | Moderator (Nokia) |
| R4-2114959 | WF on Simultaneous RxTx capability for FR2 inter-band CA | ZTE |
| R4-2114960 | WF on WF on FR2 DL CA based on CBM | Qualcomm |
| R4-2114961 | WF on FR2 UL CA | Samsung |
| R4-2114962 | WF on DC-location | OPPO |
| R4-2114963 | WF on new FR2 CA BW class | Nokia |
| R4-2115029 | Email discussion summary for [100-e][129] NR_RF_FR2_req_enh2_Part_1 | Moderator (Nokia) |
| R4-2115099 | Draft CR to TS 38.101-2: On Simultaneous RxTx capability for FR2 inter-band CA | ZTE Corporation |
| R4-2112369 | draftCR to 38.101-2 on the definition for inter-band DL CA CBM and Beam Management Reference Signal location for FR2 CA | Apple |
| R4-2112371 | Inter-band DL CA based on CBM for FR2 | Apple |
| R4-2114487 | inter-band CA DL CA with CBM | Huawei, HiSilicon |
| R4-2112335 | Discussion on CBM based inter-band DL CA within same frequency group | LG Electronics |
| R4-2112791 | CBM considerations on Fs_inter_CBM | Nokia, Nokia Shanghai Bell |
| R4-2112872 | UE requirements for CBM | Sony, Ericsson |
| R4-2112900 | Discussion on CBM for FR2 Inter-band DL CA | ZTE Corporation |
| R4-2113002 | Discussion on CBM requirement framework within same frequency group | vivo |
| R4-2113026 | Proposal on inter-band DL CA based on CBM within the same frequency group | MediaTek Beijing Inc. |
| R4-2113096 | Rx requirements for inter-band DL CA with CBM within same frequency group | Xiaomi |
| R4-2113901 | R17 FR2 CBM Inter-band DL CA within same frequency group | OPPO |
| R4-2112283 | Definition of FR2 MPR and spherical coverage for ULCA | Qualcomm Incorporated |
| R4-2112575 | Discussion on FR2 inter-band UL CA | Samsung |
| R4-2112873 | UE UL CA requirements based on IBM | Sony, Ericsson |
| R4-2113006 | Disscussion on inter-band UL CA | vivo |
| R4-2113097 | Tx requirements for inter-band UL CA between different frequency groups based on IBM | Xiaomi |
| R4-2112336 | RF requirements for CA_n257A_n259A based on IBM | LG Electronics |
| R4-2113031 | Proposal on min peak EIRP relaxation of Inter-band UL CA of CA_n257-n259 based on IBM | MediaTek Beijing Inc. |
| R4-2113902 | R17 FR2 IBM Inter-band UL CA peak EIRP and spherical coverage | OPPO |
| R4-2114061 | On FR2 inter-band UL CA for different frequency group based on IBM | Nokia, Nokia Shanghai Bell |
| R4-2111903 | Requirement framework for Inter-band CA with CBM | Qualcomm Incorporated |
| R4-2112337 | Discussion on feasibility for inter-band DL CA | LG Electronics |
| R4-2112576 | Requirements and UE capability discussion on inter-band DL CA | Samsung |
| R4-2112634 | RF requirements for CA_n260-n261 with CBM | NTT DOCOMO, INC. |
| R4-2112875 | Applicability for CBM UEs for different frequency group | Sony, Ericsson |
| R4-2112975 | Proposal on inter-band DL CA based on CBM between different frequency groups | MediaTek Beijing Inc. |
| R4-2113003 | Discussion on feasibility of CBM between different frequency group | vivo |
| R4-2113098 | Rx requirements for inter-band DL CA with CBM between different frequency groups | Xiaomi |
| R4-2114730 | Email discussion summary for [100-e][130] NR_RF_FR2_req_enh2_Part_2 | Moderator (Apple) |
| R4-2114964 | WF on UL gap for BPS | Apple |
| R4-2114965 | LS on UL gap in FR2 RF enhancement | Apple |
| R4-2115030 | Email discussion summary for [100-e][130] NR_RF_FR2_req_enh2_Part_2 | Moderator (Apple) |
| R4-2115107 | LS on UL gap in FR2 RF enhancement | Apple |
| R4-2113004 | Discussion on UL gap for coherent MIMO | vivo |
| R4-2113212 | Discussion on UL gap for coherent UL MIMO use case | ZTE Corporation |
| R4-2113662 | On UL calibration gaps for coherent UL MIMO | Ericsson, Sony |
| R4-2114492 | on FR2 UL gap for coherence calibration | Huawei, HiSilicon |
| R4-2114555 | Ul gaps test configuration | Qualcomm Incorporated |
| R4-2114577 | UL gaps test configuration | Qualcomm Incorporated |
| R4-2112088 | UL gaps for Tx power management RF aspect | Apple |
| R4-2112635 | RF requirements and NW configuration for UL gap | NTT DOCOMO, INC. |
| R4-2112808 | Requirements and test cases of UE FR2 UL Gap for UE Tx power enhancement | Nokia, Nokia Shanghai Bell |
| R4-2113005 | Discussion on UL gap for Tx power management | vivo |
| R4-2113211 | Discussion on UL gap for Tx power management | ZTE Corporation |
| R4-2113661 | Test considerations on UL calibration gaps for BPS | Ericsson, Sony |
| R4-2113900 | R17 FR2 UL gap for power management | OPPO |
| R4-2111772 | DC location reporting method and exception handling | Nokia, Nokia Shanghai Bell |
| R4-2113007 | Discussion on DC location for intra-band UL CA | vivo |
| R4-2113899 | R17 DC reporting | OPPO |
| R4-2114548 | DC location offset framework | Qualcomm Incorporated |
| R4-2112648 | FR2 bandwidth class in Rel-17 | Verizon, MediaTek |
| R4-2112729 | New CA BW class denotation for FR2 | ZTE Corporation |
| R4-2112730 | CR to TS 38.101-2 on extension to CA BW class for FBG3 | ZTE Corporation |
| R4-2112786 | Release independence spect of new FR2 CA BW class. | Nokia, Nokia Shanghai Bell |
| R4-2112787 | draftCR Release independence aspects of new FR2 CA BW classes R15 CATB | Nokia, Nokia Shanghai Bell |
| R4-2112788 | draftCR Release independence aspects of new FR2 CA BW classes R16 CATA | Nokia, Nokia Shanghai Bell |
| R4-2112789 | draftCR Release independence aspects of new FR2 CA BW classes R17 CATA | Nokia, Nokia Shanghai Bell |
| R4-2113099 | Discussion on FR2 new CA BW class denotation and definition | Xiaomi |
| R4-2113100 | CR for TS 38.101-2 to introduction of FR2 new CA BW classes | Xiaomi |
| R4-2115206 | Email discussion summary: [100-e][216] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) |
| R4-2115332 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia |
| R4-2115391 | Email discussion summary: [100-e][216] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) |
| R4-2112426 | Further discussion on RRM requirements for FR2 inter-band DL CA | Xiaomi |
| R4-2112052 | Discussions on Inter-band DL CA MRTD requirements for CBM | NTT DOCOMO, INC. |
| R4-2112339 | Discussion on MRTD for FR2 inter-band CA based on CBM | LG Electronics |
| R4-2112484 | Discussion on CBM MRTD requirement for FR2 inter-band DL CA | MediaTek inc. |
| R4-2112637 | Further views on RRM requirements for inter-band DL CA in NR FR2 | vivo |
| R4-2112702 | MRTD requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2113200 | Discussion on MRTD requirements for inter-band DL CA in FR2 | ZTE Corporation |
| R4-2113267 | Other RRM requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2113524 | Support up to 3 us MRTD | Ericsson |
| R4-2113816 | Discussion on MRTD requirements for FR2 inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2114017 | Discussion on FR2 inter-band DL CA MRTD requirements for CBM capable UE | Nokia, Nokia Shanghai Bell |
| R4-2114192 | Discussion on ways to reduce performance degradation for MRTD=3us for CBM UEs | Intel Corporation |
| R4-2112703 | RRM requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2113268 | MRTD requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2113507 | Discussion on RRM requirements for FR2 inter-band CA for CBM | Ericsson |
| R4-2113817 | Discussion on RRM requirements for FR2 inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2114018 | Discussion on RRM for FR2 RF | Nokia, Nokia Shanghai Bell |
| R4-2112704 | Inter-band UL CA for IBM | Qualcomm Incorporated |
| R4-2113508 | RRM requirements of FR2 inter-band UL CA for IBM | Ericsson |
| R4-2113818 | Discussion on RRM impacts for FR2 inter-band UL CA with IBM | Huawei, HiSilicon |
| R4-2114019 | Discussion on inter-band UL CA for IBM | Nokia, Nokia Shanghai Bell |
| R4-2112089 | UL gaps for Tx power management RRM aspect | Apple |
| R4-2112705 | UL gaps for self-calibration and monitoring | Qualcomm Incorporated |
| R4-2114016 | Network impact of UE FR2 UL Gap for UE Tx power enhancements | Nokia, Nokia Shanghai Bell |
| R4-2115206 | Email discussion summary: [100-e][216] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) |
| R4-2115332 | WF on RRM requirements for FR2 Inter-band DL CA and UL CA | Nokia |
| R4-2115391 | Email discussion summary: [100-e][216] NR_RF_FR2_req_enh2_RRM | Moderator (Nokia) |
| R4-2112426 | Further discussion on RRM requirements for FR2 inter-band DL CA | Xiaomi |
| R4-2112052 | Discussions on Inter-band DL CA MRTD requirements for CBM | NTT DOCOMO, INC. |
| R4-2112339 | Discussion on MRTD for FR2 inter-band CA based on CBM | LG Electronics |
| R4-2112484 | Discussion on CBM MRTD requirement for FR2 inter-band DL CA | MediaTek inc. |
| R4-2112637 | Further views on RRM requirements for inter-band DL CA in NR FR2 | vivo |
| R4-2112702 | MRTD requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2113200 | Discussion on MRTD requirements for inter-band DL CA in FR2 | ZTE Corporation |
| R4-2113267 | Other RRM requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2113524 | Support up to 3 us MRTD | Ericsson |
| R4-2113816 | Discussion on MRTD requirements for FR2 inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2114017 | Discussion on FR2 inter-band DL CA MRTD requirements for CBM capable UE | Nokia, Nokia Shanghai Bell |
| R4-2114192 | Discussion on ways to reduce performance degradation for MRTD=3us for CBM UEs | Intel Corporation |
| R4-2112703 | RRM requirements for CBM based Inter-band DL CA | Qualcomm Incorporated |
| R4-2113268 | MRTD requirements for FR2 inter-band DL CA enhancements | OPPO |
| R4-2113507 | Discussion on RRM requirements for FR2 inter-band CA for CBM | Ericsson |
| R4-2113817 | Discussion on RRM requirements for FR2 inter-band DL CA with CBM | Huawei, HiSilicon |
| R4-2114018 | Discussion on RRM for FR2 RF | Nokia, Nokia Shanghai Bell |
| R4-2112704 | Inter-band UL CA for IBM | Qualcomm Incorporated |
| R4-2113508 | RRM requirements of FR2 inter-band UL CA for IBM | Ericsson |
| R4-2113818 | Discussion on RRM impacts for FR2 inter-band UL CA with IBM | Huawei, HiSilicon |
| R4-2114019 | Discussion on inter-band UL CA for IBM | Nokia, Nokia Shanghai Bell |
| R4-2112089 | UL gaps for Tx power management RRM aspect | Apple |
| R4-2112705 | UL gaps for self-calibration and monitoring | Qualcomm Incorporated |
| R4-2114016 | Network impact of UE FR2 UL Gap for UE Tx power enhancements | Nokia, Nokia Shanghai Bell |