3GPP TSG RAN Meeting #93e				        	                   RP-212240

Electronic meeting, September 13 - 17, 2021

Source:	CATT

Title:	Motivation on Further RRM enhancement for NR and MR-DC in Rel-18

Agenda Item:	9.0.5

Document for:	Discussion

Introduction

Referring to the R17 RRM package discussion, due to the limitation of scope and tight timeline, some important RRM requirements were not included in Rel-17, e.g. CSI-RS measurement enhancement and TCI switching enhancement etc. And during the R17 RRM enhancement discussion, some requirements were found missing in the current specification e.g. RRM requirements for FR1+FR1 band combination. To make the specification more complete and clear, these RRM requirements shall be considered and defined in R18.

In this paper, we provide the justification of new WI to specify further RRM requirements in Rel-18.

Discussion

Leftovers from Rel-16/17

2.1.1 Enhancement for CSI-RS based L3 measurement

In R16, CSI-RS based L3 measurement requirements were defined under some conditions e.g. single FFT assumption, time domain restriction of 5ms time window etc. And in R17 RRM enhancement, due to the tight timeline and limited scope, the CSI-RS enhancement was not included in R17 even though there are extensive support on some aspects. So we think it can be considered in R18 RRM enhancement at least for the following aspects.

- Due to lack of RAN2 involvement in R16, the time domain restriction for CSI-RS based L3 measurement is defined as applicability of requirements in RAN4 i.e. the requirements are applied when the CSI-RS resources in the same frequency layer are configured within up to two separate windows for intra-frequency measurement and within the a same up to 5ms time window for inter-frequency measurement. But it is not a systematic solution for network to configure the CSI-RS resources. It is better to introduce the signaling indication for a clear configuration window (e.g. CMTC).

- Due to the tight timeline in R16, the collision case between CSI-RS based L3 measurement of neighbor cell and the SSB/CSI-RS based RLM/BFD or other SSB/CSI-RS based L1measurements of serving cell is not considered in CSI-RS based L3 measurement requirements. The cases should be clarified in R18 RRM enhancement.

- In the current specification, the CSI-RS based L3 measurement requirements are defined only for intra-frequency without gap and inter-frequency with gap. However, the network configuration is not always so restrictive, there are more possibilities existing in the network. So more practical cases shall be considered for CSI-RS based L3 measurement in R18 which has also been included in SSB and PRS measurement, e.g.

  - intra-frequency measurement with gap（e.g. The BW of the target CSI-RS resources is not within the active BWP of the UE for intra-frequency measurement）

  - inter-frequency measurement without gap （e.g. The BW of the target CSI-RS resources is within the active BWP of the UE for inter-frequency measurement）

In summary, the CSI-RS based L3 measurement enhancement need to be further discussed in Rel-18 at lease for the following aspects:

  - RRM requirements for the case when CSI-RS based L3 measurement have collision with SSB or CSI-RS based L1 measurement

  - Further study the time domain restriction of the CSI-RS resources configuration and identify solutions to address the limitation e.g.

    - Introduce CMTC for CSI-RS based L3 measurement (including RRM requirements and signaling).

  - Further study some practical cases for CSI-RS based L3 measurement.

Proposal 1: Further study and specify the RRM requirements for CSI-RS based L3 measurement in Rel-18, including but not limited to:

  - RRM requirements for the case when CSI-RS based L3 measurement have collision with SSB or CSI-RS based L1 measurement.

  - Further study the time domain restriction of the CSI-RS resources configuration and identify solutions to address the limitation e.g.

    - Introduce CMTC for CSI-RS based L3 measurement (including RRM requirements and signaling).

  - Further study some practical cases for CSI-RS based L3 measurement.

2.1.2 Enhanced indication of UE per-FR gap capabilities

Per-FR gap capability was introduced in Rel-15 and is per-UE reported. When UE support per-FR gap, the interruption can only apply to the same frequency range. But with more FR1+FR2 band combinations introduced, the processing complexity to support per-FR gap is increased as it is not purely the issue of RF architecture. If UE have problems to support this per-FR measurement gap only in certain FR1+FR2 band combinations, then UE can only report the capability of per-UE gap which will definitely impact the UE performance. In previous meeting, one possible solution raised for this issue is to have an additional per-BC indication. Based on this solution, RAN4 discussed the feasibility and the possible impact on the existing RRM requirements but no conclusions are reached. In our understanding, this issue can be further studied in R18 on the top of the extensive discussion in R16.

Proposal 2: Further study the enhanced indication of per-FR gap capability in Rel-18.

2.1.3 TCI switching enhancement

In Rel-16 TCI switching requirement, there was a time gap from “slot n+ THARQ +(3 ms) / NR slot length” to “slot n+ THARQ +(3 ms+TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length”. And UE is not required to receive DL data during this time gap which probably impact the throughput performance in case the first SSB arrives 160ms later. Due to limited scope of RRM requirements in R17, this enhancement was not included in R17. In our understanding, RAN4 can further discuss if UE behavior can be enhanced to receive DL data during the time gap of Tfirst-SSB in R18.

Proposal 3: Further study and specify the RRM requirements for TCI witching enhancement in Rel-18.

2.1.4 RLM requirement enhancement

The function of RLM is to monitor the DL signal quality of the primary cell for the purpose of determining UE out-of-sync or in-sync status. The RLM design has been extensively discussed in RAN1 during many meetings. Reference [3-19] shows the broad interest of this topic in RAN1 from different companies. One of the most important aspects discussed for RLM is whether to support multiple IS/OOS PDCCH BLER pairs for supporting RLM optimization of VoNR and URLLC. Finally RAN1 agreed to support configurability between two pairs of IS/OOS BLER values for RLM purposes for VoNR and a LS was also sent to RAN4 to let RAN4 to decide the second BLER pair.

From the LS in R1-1809853, very clear background information on the purpose of the second IS/OOS BLER pair was included. It has been observed in LTE, when semi-persistent scheduling is used, the service quality can be adequate also when the SINR is below the level corresponding to a PDCCH BLER of 10%. The most prominent example is for VoLTE. In these cases, it has been observed that the UE triggers RLF also when the voice quality is quite adequate. In other words, RLF may be triggered prematurely. We think RLM is a very important feature which impact operators’ network performance and deployment cost. We propose to have some discussion on this feature in Rel-18.

Proposal 4: Study on how to introduce the second IS/OOS BLER pair for VoNR in Rel-18.

New identified issues

2.2.1 RRM requirements for FR1+FR1 NR-DC and FR2+LTE NE-DC

When discussing the R17 RRM enhancement, it was found that even though in TS 38.101-1/3, FR1+FR1 and FR1+FR2 band combinations are both defined for inter-band NR-DC, in current RRM specification, some requirements for FR1+FR1 are missing (e.g. for PSCell addition delay requirement, only FR2 PSCell is defined). Since the FR1+FR1 requirements are supposed to be the baseline in some scenarios, complete RRM requirements for FR1+FR1 NR-DC are expected. So the RRM requirements for FR1+FR1 shall be studied and specified in R18.

Also for HO with PSCell requirements in R17, considering the timeline and urgency, only FR1+LTE NE-DC scenario was considered and FR2+LTE NE-DC scenario was deprioritized. In our understanding, this scenario can be considered in R18.

Proposal 5: Further study the RRM requirements for FR1+FR1 NR-DC and FR2+LTE NE-DC in Rel-18.

2.2.2 HO with PSCell for new scenarios

For HO with PSCell requirements in R17 RRM enhancement, following the WID, only the following scenarios are defined:

- from NR SA to EN-DC

- from EN-DC to EN-DC

- from NE-DC to NE-DC

- from NR-DC to NR-DC

But from the discussing, there can be other scenarios that operators are interested in (e.g. from NR SA to NE-DC, from NR SA to NR-DC and from LTE SA to EN-DC etc.). Considering the market demand and commercial deployment, these scenarios can be considered in R18.

Proposal 6: Further study the new scenarios for HO with PSCell requirements in Rel-18.

2.2.3 RRM enhancement for gap-less measurement

In the current specification, some requirements are always gap based, e.g. CSI-RS based inter-frequency measurement and inter-RAT NR measurement on non-serving cell in EN-DC. To increase the UE throughput and reduce the measurement latency, it is proposed to define RRM requirements without gap for inter-RAT NR measurement and inter-frequency CSI-RS based L3 measurement if the SSB/CSI-RS is fully confined within the UE active BWP to save gaps and align with inter-frequency SSB based measurement.

And in another aspect, for the gap indication ‘NeedForGap’ introduced by RAN2, the ‘no gap’ indication can also bring benefits for UE and NW side. But the possible interruption or RRM requirements impact need further study which can be also considered in R18 and categorised as the enhancement for gap-less measurement.

Proposal 7: Further study the RRM enhancement for gap-less measurement in Rel-18 including but not limited to:

  - CSI-RS based inter-frequency measurement without gaps

  - Inter-RAT NR measurement without gaps

  - RRM requirements for ‘no gap’ indication of ‘NeedForGap’

Conclusion

In this contribution, we provide the justification to specify some enhanced RRM requirements in Rel-18 and the following proposals are given:

Proposal 1: Further study and specify the RRM requirements for CSI-RS based L3 measurement in Rel-18, including but not limited to:

  - RRM requirements for the case when CSI-RS based L3 measurement have collision with SSB or CSI-RS based L1 measurement.

  - Further study the time domain restriction of the CSI-RS resources configuration and identify solutions to address the limitation e.g.

    - Introduce CMTC for CSI-RS based L3 measurement (including RRM requirements and signaling).

  - Further study some practical cases for CSI-RS based L3 measurement.

Proposal 2: Further study the enhanced indication of per-FR gap capability in Rel-18.

Proposal 3: Further study and specify the RRM requirements for TCI witching enhancement in Rel-18.

Proposal 4: Study on how to introduce the second IS/OOS BLER pair for VoNR in Rel-18.

Proposal 5: Further study the RRM requirements for FR1+FR1 NR-DC and FR2+LTE NE-DC in Rel-18.

Proposal 6: Further study the new scenarios for HO with PSCell requirements in Rel-18.

Proposal 7: Further study the RRM enhancement for gap-less measurement in Rel-18 including but not limited to:

  - CSI-RS based inter-frequency measurement without gaps

  - Inter-RAT NR measurement without gaps

  - RRM requirements for ‘no gap’ indication of ‘NeedForGap’

References

RP-191027,  WF for Rel-16 RAN4 RRM, Demod, HST and URLLC scope, RAN4 chairman, RAN#84

RAN4_96e_RRM_chairman_report_12_EOM

R1-1717038, “RLM in NR”, ZTE, Sanechips

R1-1717055, “Remaining details on Radio link monitoring in NR”, Huawei, HiSilicon

R1-1717360, “Radio Link Monitoring for NR”, Intel Corporation

R1-1717464, “IMR configuration for Radio link monitoring in NR”, Vivo

R1-1717584, “Radio link monitoring”, Samsung

R1-1717742, “Discussions on the IS and OOS counting procedure”, Spreadtrum Communications

R1-1717805, “NR Radio Link Monitoring”, CATT

R1-1717933, “Discussion on Radio Link Monitoring in NR”, LG Electronics

R1-1718061, “Remaining details on NR radio link monitoring	Guangdong”, OPPO Mobile Telecom

R1-1718186, “Discussion on remaining details on radio link monitoring for mobility management”, NTT DOCOMO

R1-1718321, “Discussion on Radio Link Monitoring,”	MediaTek Inc.

R1-1718383, “Radio link monitoring procedure and thresholds	,”AT&T

R1-1718534, “Radio link monitoring consideration”, Qualcomm Incorporated

R1-1718803, “Radio Link Monitoring in NR”, Nokia, Nokia Shanghai Bell

R1-1718718, “Radio link monitoring for mobility management”, Ericsson

R1-1718794, summary of discussion for NR Radio link monitoring, Intel

R1-1809853, LS on second BLER pair for RLM

RP-211579, Moderator's summary for email discussion [92-e-23-RRM-Enh]