3GPP TSG RAN Meeting #93e	RP-212251

Electronic Meeting, Sep. 13 - 17, 2021

# Source:	CATT

# Title:	New WID on NR Expanded and Improved Positioning

# Document for:	Approval

Agenda Item:	9.0.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	NR Expanded and Improved Positioning

Acronym: NR_exp_pos

Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-18

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. NOTE: In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  |  |
| Don't know |  |  |  | X |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860034 | Study on NR Positioning Enhancements | Preceding Study Item |
| 900160 | NR Positioning Enhancements | Preceding Work Item |
| 880075 | Study on scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases | Preceding Study Item |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

3	Justification

3GPP is currently working on the Study Item on “Scenarios and requirements of in-coverage, partial coverage, and out-of-coverage positioning use cases”, where the positioning requirements for V2X and public safety are defined in TR38.845, based on TS 22.261 and TS 22.186. In addition, SA1 has recently completed the Work Item “Ranging based services” with the positioning requirements for ranging service being captured in TS 22.261. The ranging and relative/absolute positioning for V2X, public safety, and other commercials are required to be provided when a UE is inside the network coverage as well as outside the network coverage. Thus, there is a need for 3GPP to develop sidelink positioning solutions for supporting these use cases and corresponding requirements with the consideration of the ITS and other spectrums, for supporting ranging, relative and absolute sidelink positioning.

For supporting 5G stringent performance requirements in TS 22.261 and to make NR positioning to be competitive with other positioning techniques (e.g., GNSS positioning), 3GPP needs to develop more advanced positioning solutions for further enhancements of the positioning accuracy. Two promising techniques can be considered in Rel-18: one is to take the advantage of the rich 5G spectrum to increase the bandwidth for the transmission and reception of the positioning reference signals through the carrier aggregation, and the other is to use the NR carrier phase measurements. Intra-band carrier aggregation of PRS/SRS was discussed with limited evaluation in Rel-17 SI on NR positioning enhancements. The work was not completed mainly due to the time limitation of the SI. It is highly desirable for 3GPP to continue the study in Rel-18. GNSS carrier phase positioning has been used very successfully for cm-level positioning, but is limited to outdoor. NR carrier phase positioning has many advantages over GNSS carrier phase positioning, especially it is not limited to outdoor scenarios, since NR reference signals can be available indoor environment.

Power efficiency while maintaining high accuracy should also be a focus for positioning enhancements in Rel-18. Use cases and requirements of LPHAP (low power high accuracy positioning) are captured in TS 22.104. There is clear commercial demand for devices reaching these requirements, which cannot be achieved by NR UEs based on Rel-15/16/17 specifications. A study would be needed to determine the power consumption model allowing to evaluate achievable battery life with positioning, and to study the technical solutions for supporting LPHAP.

3GPP has introduced RedCap UE in Rel-17, but there is no discussion of the corresponding positioning requirements. The positioning solutions developed in Rel-16 and Rel-17 for eMBB UEs should also be applicable to RedCap UEs. However, the impacts of the reduced UE capability on NR positioning, including the coverage and the accuracy needs to be evaluated, and based on the evaluation, there may need to have further consideration on whether further enhancement is needed to support RedCap positioning, including the configuration of the DL PRS and UL SRS for positioning, the physical layer procedures, the UE capability, etc.

GNSS positioning integrity is scheduled to be completed in Rel-17. RAT-dependent positioning integrity was dropped off from Rel-17 due to the time limitation. It is important and necessary to further work on the RAT-dependent positioning integrity in Rel-18. Most of the accomplishments made in Rel-17 for GNSS integrity can be used for supporting RAT-dependent positioning integrity, including the signalling and procedures, the KPI definitions, etc. The main work for RAT-dependent integrity in Rel-18 may be the identification of the error sources, threat models, occurrence rates and failure modes requiring RAT-dependent positioning solutions.

4	Objective

4.1	Objective of SI or Core part WI or Testing part WI

The objective of this work item is to specify solutions to enable: sidelink positioning, further enhancements of NR positioning accuracy, RAT-dependent positioning integrity, and improvement of power efficiency. The specific objectives of this work are:

- Study/specify solutions (including reference signals, measurements, procedures, etc.) for sidelink positioning considering the following: [RAN1, RAN2]

  - To include ranging, relative positioning, and absolute positioning

  - Coverage scenarios to cover: in-coverage, partial-coverage and out-of-coverage

  - Requirements: Based on requirements identified in TR38.845 and TS22.261 and TS22.104

  - Use cases: V2X (TR38.845), public safety (TR38.845), commercial (TS22.261), IIOT (TS22.104)

Note: The next phase of discussion will include the selection of use cases to be used for evaluation and design. This does not restrict the use of the solutions for other use cases.

  - Spectrum: ITS, licensed

  - Coordination with SA2 as required (e.g. architecture aspects)

  - Study and specify the solutions for UE-based and UE-assisted cooperative sidelink positioning by integrating SL measurements with existing DL, UL, DL+UL measurements [RAN1, RAN2, RAN3]

- Improved accuracy, integrity, and power efficiency:

  - Study/Specify solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1, Coordination with SA2 as required]

Including the identification of the error sources, threat models, occurrence rates and failure modes for RAT-dependent positioning integrity

  - Study/Specify solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4, RAN2]

including the solutions for the mitigation of the multipath and the network time/frequency synchronization errors for NR carrier phase positioning

  - Study/Specify solutions for accuracy improvement based on intra-band PRS/SRS bandwidth aggregation [RAN1, RAN4, RAN2]

including the feasibility study of PRS/SRS bandwidth aggregation under the impact of timing alignment error, phase continuity, etc.

  - Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, identify potential enhancements to help address any limitations [RAN1, RAN2]

- Define positioning support for RedCap UEs, considering the following:

  - Evaluate the performance of existing positioning procedures and measurements with RedCap UEs[RAN4]

  - Based on the evaluation, identify potential enhancements to help address possible limitations associated with RedCap UEs [RAN1, RAN2]

  - Define performance requirements for positioning by RedCap UEs [RAN4]

4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

Define corresponding performance requirements and test cases for NR Expanded and Improved Positioning [RAN4]

4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TR | TBD | Study on NR Positioning Enhancements | 97 | 98 | Including the study results of the WI |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.211 | NR positioning enhancements | 100 | Core part |
| 38.212 | NR positioning enhancements | 100 | Core part |
| 38.213 | NR positioning enhancements | 100 | Core part |
| 38.214 | NR positioning enhancements | 100 | Core part |
| 38.215 | NR positioning enhancements | 100 | Core part |
| 38.305 | NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN | 101 | Core part |
| 37.355 | Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP) | 101 | Core part |
| 38.331 | NR; Radio Resource Control (RRC); Protocol specification | 101 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | 101 | Core part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | 101 | Core part |
| 38.300 | NR; NR and NG-RAN Overall Description; Stage 2 | 101 | Core part |
| 38.401 | NG-RAN; Architecture description | 101 | Core part |
| 38.413 | NG-RAN; NG Application Protocol (NGAP) | 101 | Core part |
| 38.423 | NG-RAN; Xn application protocol (XnAP) | 101 | Core part |
| 38.455 | NG-RAN; NR Positioning Protocol A (NRPPa) | 101 | Core part |
| 38.473 | NG-RAN; F1 application protocol (F1AP) | 101 | Core part |
| 38.133 | NR; Requirements for support of radio resource management -Core | 101 | Core part |
| 38.133 | NR; Requirements for support of radio resource management -Performance | 103 | Performance part and test cases |
| 38.171 | NR; Requirements for support of Assisted Global Navigation Satellite System (A-GNSS) | 103 | Performance part and test cases |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>.}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s).}

7	Work item leadership

Primary: RAN1

Secondary: RAN2, RAN3, RAN4

8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only to WGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

9	Supporting Individual Members

Supporting IM name

CATT