3GPP TSG RAN Meeting #93-e	RP-212286

Electronic Meeting, Sep 13-17, 2021

Source:	Huawei, HiSilicon

Title:	Views on Rel-18 proposals on UAV

Agenda item:	9.0.3

Document for:	Discussion and Decision

# Introduction

In the last RAN workshop discussion, UAV is identified as one of the potential topics for Rel-18 future discussion [1] and during the summary stage [2] common interest was observed on importing the Rel-15 LTE Aerial features in NR system. In addition, there were some other enhancements proposed, e.g., supporting PC5 enhancement for remote ID broadcast. During the RAN Email discussion, the support of UAV in NR was discussed as a potential topic in the Additional RAN1/2/3 candidate topics, Set 2 and a list of objectives were drafted for further discussion. In this paper, we would like to have more discussion on the objectives.

# Discussion

## Moderator summary and proposal

Here we just copy the moderator summary and proposal for reference.

There is broad support to cover LTE UAV functionality and the new requirements on ID broadcasting (from FAA) in a new work item. There is support for additional enhancements but also some views that these are not necessary.

The following areas/objectives can be considered as a starting point for further discussions on UAV

- Measurement reports (RAN2)

  - UE-triggered measurement report based on configured height thresholds

  - Reporting of height, location and speed in measurement report

  - Flight path reporting

  - Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

- Signaling to support subscription-based aerial-UE identification (RAN2/RAN3/SA2 interaction)

- Note: Work done in LTE is a starting point for the above objectives intended to cover LTE UAV functionality including any NR-specific enhancements as necessary

- Support for broadcast/groupcast of drone identification over PC5 dependent on SA2 outcome (RAN2)

  - [Applicable to both LTE and NR]

- Study and specify if needed

  - [Additional RRM enhancements to control volume of reports (RAN2)]

  - [Mobility enhancements, e.g., for CHO]

  - [Beam management enhancements]

## Considerations on PC5 for NR UAV

Companies proposed to support UAV to ground identification (e.g., to authorized third parties such as police devices) over PC5 to satisfy the FAA requirements [3]. We can find the details of the requirements in FAA website [4] and there we can find that the information to be broadcasted includes drone ID, location and altitude, velocity and so on.

Figure 1: Illustration of drone remote identification in [4]

We can understand the requirement of drone remote identification, however, it was not clarified what the format should be when the information is broadcasted over PC5. For example, whether it is assumed to be broadcasted as PC5 AS signalling, PC5-S signalling, or service data (similar as V2X broadcast service). In our understanding, this will decide what the enhancement should be expected from RAN perspective.

Proposal 1: It should be clarified whether remote identification is assumed to be broadcast in PC5 AS signalling, PC5-S signalling, or service data (similar as V2X broadcast service).

In fact, although the objective of supporting the broadcast of the drone remote identification over PC5 is provided, it is quite unclear to us what kind of enhancement is expected. During the email discussion, PC5 enhancement and system information enhancement were mentioned by different companies. With respect to the mentioned PC5 enhancement, it aims to achieve a range as large as possible. However, no detailed KPI of the broadcast range is provided and it is not clear whether the requirement of range can be satisfied by the current sidelink capability or not. Instead, from our perspective, we consider it as a more generic requirement to enlarge the sidelink communication range and it is more appropriate to discuss it in the scope of pure sidelink enhancement topic.

Regarding the system information enhancement, no more detailed explanation was provided during the email discussion. From our point of view, we wonder whether only enhancements on system information is expected, i.e., no enhancement is required on sidelink mechanism. If so, it needs to be clarified what kind of system information enhancement is needed to support remote identification broadcast over PC5 without any change on sidelink communication mechanism. From RAN perspective, even if it is supposed to be dependent on SA2 outcome, a clarification in this respect is needed.

Proposal 2: It should be clarified what kind of enhancement is expected to support broadcast of drone remote identification from RAN perspective.

In [4], it is described that the identification and location information should be broadcasted. However, in the moderator proposal, groupcast is also listed as required by some companies. In our understanding, the motivation of groupcast needs more justification.

Proposal 3: The motivation of groupcast needs justification.

One more generic question is about the security and privacy. In PC5, the data and signalling transmitted in broadcast manner is not ciphered or integrity protected. However, as described in [4],“Remote ID is the ability of a drone in flight to provide identification and location information that can be received by other parties”. It is therefore questionable whether it is acceptable to broadcast the remote identification without security protection.

Proposal 4: The security risk needs to be assessed if/when the remote identification is broadcasted over PC5 without protection.

## Considerations on other potential objectives

### Additional RRM enhancement to control volume of reports

During the email discussion, companies commented that this objective is not clear and also some companies understood it as one of the Rel-15 LTE UAV feature, i.e., it is covered by the sub-objective “Measurement reporting based on a configured number of cells (i.e., larger than one) fulfilling the triggering criteria simultaneously”. After checking the discussion, we tend to agree with this understanding.

Proposal 5: RAN to confirm that the objective “Additional RRM enhancements to control volume of reports” has been covered by the sub-objective “Measurement reporting based on a configured number of cells (i.e., larger than one) fulfilling the triggering criteria simultaneously”.

### Mobility enhancement, e.g. for CHO

Companies proposed CHO enhancement based on flight path, which sounds quite straightforward. In Rel-15 LTE UAV, the flight path reporting is supported for mobility optimization. In details, it informs the network of the UAV’s locations in specific time points. Based on this flight path, CHO execution can be triggered based on timer or location. In our understanding, it is the potential CHO enhancement. However, as commented by companies during the email discussion, such timer/location based CHO enhancement have been discussed and agreed in Rel-17 NTN. Then clarification is needed on what additional enhancements are expected for CHO.

Proposal 6: It should be clarified what additional enhancements are expected for CHO besides the timer/location based CHO agreed in Rel-17 NTN.

### Beam management enhancements

Beam management is a specific aspect in NR system, compared to LTE, therefore by nature the UAV in NR system should support beam management. In addition, considering the UAV flies on high altitude, the uptilt beam should be deployed to provide better service. However, more justification is needed on the motivation to introduce UAV specific beam management enhancement.

Proposal 7: Justification is needed on the motivation to introduce UAV specific beam management enhancement from RAN1 perspective.

# Conclusions

In this contribution, potential objectives for NR UAV are discussed, where we have provided following observations and proposals:

Proposal 1: It should be clarified whether remote identification is assumed to be broadcast in PC5 AS signalling, PC5-S signalling, or service data (similar as V2X broadcast service).

Proposal 2: It should be clarified what kind of enhancement is expected to support broadcast of drone remote identification from RAN perspective.

Proposal 3: The motivation of groupcast needs justification.

Proposal 4: The security risk needs to be assessed if/when the remote identification is broadcasted over PC5 without protection.

Proposal 5: RAN to confirm that the objective “Additional RRM enhancements to control volume of reports” has been covered by the sub-objective “Measurement reporting based on a configured number of cells (i.e., larger than one) fulfilling the triggering criteria simultaneously”.

Proposal 6: It should be clarified what additional enhancements are expected for CHO besides the timer/location based CHO agreed in Rel-17 NTN.

Proposal 7: Justification is needed on the motivation to introduce UAV specific beam management enhancement from RAN1 perspective.

# Reference

- RWS-210659, RAN Rel-18 workshop summary

- RWS-210655, Email discussion summary for [RAN-R18-WS-non-eMBB-Overall]

- RP-202158, LS on Aerial Features for Unmanned Aerial Vehicles

- UAS Remote Identification Overview (faa.gov)