3GPP TSG RAN meeting #93e 		    RP-212384

e-Meeting, Sep. 13th – 17th, 2021

Source: 	ZTE, Sanechips

Title:	Discussion on network energy saving for 5G Advanced

Agenda item:	9.0.2

Document for:	Discussion

Introduction

In Rel-18 workshop [1], the network energy saving was identified as one of the potential enhancement areas for 5G advanced.

13. Network energy savings, with the following example areas: - KPIs and evaluation methodology, focus areas and potential solutions

Furthermore, according to the follow-up email discussion, the following scope was suggested by the moderator in [2].

Potential scope of a study on network energy savings: 1. Definition of a network energy consumption model a) Note 1: it is suggested to adapt the framework of the power consumption methodology of TR38.840 to the network side, utilizing information from known models / external references and other relevant information, including relative energy consumption for DL and UL (considering factors like PA efficiency, number of TxRU, etc), sleep states and the associated transition times, and one or more reference parameters/configurations b) Note 2: an absolute power consumption model could also be studied in addition to the relative power consumption model, or a method could be studied for allowing to derive absolute power consumption metric from the relative power consumption metric and additional information available outside 3GPP 2. Definition of an evaluation methodology, including studying potential KPIs a) Note: evaluation methodology should not focus on a single KPI, but allow evaluating how to enable system-level network energy savings while balancing impact to network/user performance (e.g. spectral efficiency, capacity, UPT, latency) and UE power consumption/complexity 3. Study techniques and features to enable network energy saving a) Note 1: the focus areas include how to achieve more efficient dynamic and/or semi-static and finer granularity adaptation of transmissions and/or receptions in one or more of time, frequency, spatial, and power domains, with potential support/feedback from UE. Additional areas of the study may include UE assistance information and intra-network information exchange/coordination. b) Note 2: legacy UEs should be able to continue accessing a network implementing Rel-18 network energy savings techniques, with the possible exception of techniques developed specifically for greenfield deployments. As a starting point, the following scenarios can be considered as targets for system-level studies on network energy savings: − Urban micro in FR1, including TDD massive MIMO − FR2 beam-based scenarios with massive MIMO − Urban/Rural macro in FR1 with/without DSS (no impact to LTE expected in case of DSS) − EN-DC/NR-DC macro with FDD anchor band and TDD/Massive MIMO on higher FR1 frequency − Other scenarios, e.g., small cell deployment, can be considered

In this contribution, the aspects such as the motivation, scenarios, evaluation methodology and potential techniques for network energy saving are discussed.

Motivation

5G can significantly improve the user experience with higher throughput, massive connections, and various services, for example, eMBB, URLLC, redcap, XR, etc. However, the energy consumption and greenhouse gas emission caused by the 5G base stations and user devices have become an acute issue with the deployment of 5G.

To build a sustainable communication system, techniques including BWP adaptation, wake-up signal in Rel-16/17, cross-slot scheduling, PDCCH monitoring reduction, etc. have been specified to improve UE energy efficiency. Meanwhile, the necessity to reduce network power consumption is still outstanding. Compared with 4G base stations, the power consumption of 5G base stations is 3-4 times that of 4G base stations due to the increased number of transmission/receive antennas, frequency band, etc. According to Figure 1 [3], the BS power consumption accounts for about 58% of cellular network power consumption. Besides the impact on climate change, the network opex tends to account for around 25% of the operator cost base, wherein over 90% of network costs are spent on energy [4].

Figure 1 Cellular network power consumption profile

Therefore, the network energy saving is important for the development of a sustainable and green communication system, and reduction of the operation expense. The study and specification of solutions to network energy saving in Rel-18 is essential.

Network energy saving techniques should be studied and specified in Rel-18.

Network power consumption model

The network power consumption model is beneficial for the provision of in-depth understanding of different network energy saving techniques. Hence, a generic network power consumption model should be determined first. According to the discussion in [2], the power consumption model can be classified into two categories, i.e., absolute power consumption model and relative power consumption.

For the absolute power consumption model in [5], the detailed model of individual component, such as power amplifier, analog front-end, digital baseband/control, and so on, is provided in terms of watt. However, as it is stated in [5], the absolute power model depends on the technology evolution at component level and the base station implementation. In this sense, even an absolute power consumption is not accurate to reflect the implementation.

In the email discussion, the relative energy consumption model (similar to that of TR38.840 [4]) is preferred by many companies for the study on network energy savings. It allows for better alignment avoiding dependency on implementation details, which is simpler for standardization work and future-proof to future evaluations of power saving techniques. Therefore, the relative energy consumption model is sufficient for the purpose of improving network energy efficiency.

To derive the relative network energy consumption model, the following aspects need to be considered.

- A reference configuration based on assumption of

- FR1/FR2;

- Downlink/uplink transmission

- Carrier configuration;

- Subcarrier spacing (SCS);

- RX/TX antenna configuration;

- System Bandwidth.

- Sleep modes, and corresponding time/energy required by sleep mode transition

- Deep sleep;

- Light sleep;

- Micro sleep;

- Time/energy of Ramp up/down for sleep;

- Time/energy of cell activation/de-activation;

- Power consumed by different states

- Data transmission (e.g. PDCCH, PDSCH);

- Data reception (e.g. PUCCH, PUSCH);

- Reference signal transmission (e.g. SSB, CSI-RS);

- Reference signal reception (e.g. SRS, PRACH preamble).

- Scaling rules when the configuration of network transmission/reception is adapted in relative to the reference configuration.

To derive a relative network energy consumption model, the following factors need to be considered

- A reference configuration;

- Possible sleep modes and corresponding time/energy required by sleep mode transition;

- Power consumed by different states;

- Scaling rules.

Potential enhancements for network energy saving

In the legacy scheme, the cell can be de-activated for network energy saving, which has large impact on system performance and user experience. Therefore, more dynamic and finer granularity schemes are preferred to achieve a better tradeoff between network energy saving and system performance.

The network power saving techniques can be studied in multiple aspects, such as, spatial domain, frequency domain, time domain, UE assistance information and network coordination, etc.

- Spatial domain

AAU contributes 80%~90% of the base station power consumption, where the increase of antennas in NR is an important reason for network power consumption. In the case when the traffic load is small and the resource utilization is low, some of the antennas is not required to be active. The network can switch off some transmission/receiving antenna to save power as shown in Figure 2. The dynamic antenna adaptation can effectively reduce the network power consumption. Similarly, the dynamic TX/RX port, panel and beam adaptation can also be considered.

Figure 2 Antenna adaptation for network power saving

What’s more, for spatial domain adaptation, impact on CSI acquisition needs to be considered. Hence, the CSI-RS enhancement, SRS transmission enhancement should also be considered.

For spatial domain adaptation, the dynamic TX/RX antenna/ beam adaptation and the corresponding impact on CSI-RS/SRS transmission can be studied.

- Time domain

To obtain the power saving gain, the network can deactivate some components (e.g. cell, carrier, band) to enter the sleep states. However, the semi-static activation/de-activation mechanism may cause a large delay and affect the user experience. To improve user experience, a finer granularity time domain on-off operation, such as the switch off for several milliseconds, sub-frames, slots or symbols, has less impact on system performance, which should be also supported.

What’s more, the network cannot go into deep sleep mode because of the transmission always-on signals, for example, SSB. In order to achieve the network energy saving, the always-on signals periodicity adaptation should be taken into consider.

In the previous discussion, network wake-up mechanism is also proposed by some companies. For the wake-up mechanism, the functionality or implication of the wake-up signal needs to be discussed. And mechanism to avoid frequent transmission of wake-up signal also needs to be considered.

For time domain adaptation, the following solutions need to be considered

- A fine granularity time domain on-off operation;

- The always-on signals periodicity adaptation.

- UE assistance information

UE assistance information, such as the UE mobility, UE’s traffic pattern, preferred configuration, etc, can help network to better acquire UE’s requirements, so that the configuration can be adjusted more accurately to reduce the impact on user experience and assist network power saving. For example, for a UE with low data rate, the number of antennas or bandwidth can be dynamically adjusted to save energy for both network and UE.

Therefore, the UE assistance information report is a useful approach to accurate and flexible network energy saving.

The UE assistance information should be considered for network energy saving.

- Network coordination

Figure 3 Network coordination for cell de-activation/activation

As shown in Figure 3, the enhancement on the autonomous cell de-activation/activation mechanism should be considered for network energy saving, which can be implemented through network coordination.

Through network coordination, the information such as the sleep mode, beam adaptation, etc, can be exchanged among cells, which can help network energy saving.

Network coordination should be used for network energy saving.

Suggested objectives

According to the above discussion, the scope for the network energy saving is suggested as below.

- Study the evaluation methodology and KPIs for network energy saving.

- Study network energy saving techniques in spatial domain including

  - Dynamic TX/RX antenna/ beam adaptation

  - Corresponding impact on CSI-RS/SRS transmission.

- Study network energy saving techniques in time domain including

  - Time domain millisecond/ sub-frame/slot/symbol-level on-off operation;

  - Adaptation always-on signals periodicity.

- Study UE assistance information for network energy saving

- Study network coordination for network energy saving

The scope for the study of network energy saving is suggested as below.

- Study the evaluation methodology and KPIs for network energy saving.

- Study network energy saving techniques in spatial domain including

  - Dynamic TX/RX antenna/ beam adaptation

  - Corresponding impact on CSI-RS/SRS transmission.

- Study network energy saving techniques in time domain including

  - Time domain millisecond/ sub-frame/slot/symbol-level on-off operation;

  - Adaptation always-on signals periodicity.

- Study UE assistance information for network energy saving

- Study network coordination for network energy saving

Conclusion

In this contribution, we discuss the network energy saving schemes. We have the following proposals.

Proposal 1:	Network energy saving techniques should be studied and specified in Rel-18.

Proposal 2:	To derive a relative the network energy consumption model, the following factors need to be considered

- A reference configuration;

- Possible sleep modes and corresponding time/energy required by sleep mode transition;

- Power consumed by different states;

- Scaling rules.

Proposal 3:	For spatial domain adaptation, the dynamic TX/RX antenna/ beam adaptation and the corresponding impact on CSI-RS/SRS transmission can be studied.

Proposal 4:	For time domain adaptation, the following solutions need to be considered

- A fine granularity time domain on-off operation;

- The always-on signals periodicity adaptation.

Proposal 5:	UE assistance information should be considered for network energy saving.

Proposal 6:	Network coordination should be used for network energy saving.

Proposal 7:	The scope for the study of network energy saving is suggested as below.

- Study the evaluation methodology and KPIs for network energy saving.

- Study network energy saving techniques in spatial domain including

  - Dynamic TX/RX antenna/ beam adaptation

  - Corresponding impact on CSI-RS/SRS transmission.

- Study network energy saving techniques in time domain including

  - Time domain millisecond/ sub-frame/slot/symbol-level on-off operation;

  - Adaptation always-on signals periodicity.

- Study UE assistance information for network energy saving

- Study network coordination for network energy saving

References

3GPP RAN Rel-18 Workshop, RWS-210657, Summary of RAN Rel-18 Workshop.

3GPP RAN #93, RP-211663, Moderator's summary for discussion [RAN93e-R18Prep-13] Network energy savings.

Dutta U K, Razzaque M A, Al-Wadud M A, et al. Self-adaptive scheduling of base transceiver stations in green 5g networks.

5G energy efficiencies: green is the new black.

Bjorn Debaillie, et al. A Flexible and Future-Proof Power Model for Cellular Base Stations.

TS 38.840-g00.