3GPP TSG-RAN WG2 Meeting #115-e	R2-2107959

# Electronic, 16th Aug. – 27th Aug. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.305 | CR | 0105 | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on user-plane positioning support by SUPL |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_LTE |  |  |  |  |  | Date: |  |  | 2021-08-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The latest version of SUPL ULP is 2.0.6. Thus, the reference [18] should be updated. - In Table 4.3-1, the column SUPL indicates which positioning methods are supported by SUPL ULP as discussed in R2-2104512. Considering that SUPL can support user-plane positioning with two different ways (i.e., by SUPL ULP itself or via tunnelling LPP), we need to add some note to clarify the distinction between a SUPL ULP position method and an LPP position method. - In the first 36.305 version, the current description for “LteCellInformation” and “MeasResultListEUTRA” IE was captured from a DRAFT SUPL spec and copied into the Annex, but never updated later. The same Annex was copied into the first version of 38.305, and later fixed with CR #0011 (R2-1908253). However, the correcponding correction in 36.305 has been forgetten, and thus we need to update it according to the latest version of SUPL ULP (v2.0.6). Regarding the following description for “MeasResultListEUTRA” IE, it is not necessary to describe the mirroring relation here. Moreover, in the case of the updated “MeasResultListEUTRA”, it does not seem to be well matched with the equivalent IE from the RRC spec. Thus, it would be better to just remove the part for simplicity and consistency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In References, update [18] with the latest version of SUPL ULP (i.e., OMA-TS-ULP-V2_0_6). - In Table 4.3-1, add the note to clarify the meaning of the column SUPL. - In section B.1, updated the description for “LteCellInformation” IE and “MeasResultListEUTRA” IE according to the latest version of SUPL ULP. Impact analysis Impacted functionality: Positioning Inter-operability: If only the network is implemented according to the CR, interoperability problems are not foreseen. If only the UE is implemented according to the CR, interoperability problems are not foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - TS 36.305 refers to the old version of SUPL ULP. - TS 36.305 describes the misleading table regarding SUPL support of positioning methods. - TS 36.305 includes the outdated descriptions for “LteCellInformation” and “MeasResultListEUTRA” IE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3, B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

------------------------------------------------- Start of 1st change ------------------------------------------------

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)"

[3]	3GPP TS 22.071: "Location Services (LCS); Service description, Stage 1".

[4]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[5]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); "User Equipment (UE) radio access capabilities".

[6]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[7]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[8]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[9]	Galileo OS Signal in Space ICD (OS SIS ICD), Draft 0, Galileo Joint Undertaking, May 23rd, 2006.

[10]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5, 2002.

[11]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.0, June 17, 2008.

[12]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[13]	RTCM 10402.3, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[14]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[15]	3GPP TS 25.331: " Radio Resource Control (RRC); Protocol Specification".

[16]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[17]	OMA-AD-SUPL-V2_0: "Secure User Plane Location Architecture Approved Version 2.0".

[18]	OMA-TS-ULP-V2_0_6: "UserPlane Location Protocol Approved Version 2.0.6".

[19]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[20]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Physical layer – Measurements".

[21]	3GPP TS 36.302: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Services provided by the physical layer ".

[22]	3GPP TS 25.305: "Stage 2 functional specification of User Equipment (UE) positioning in UTRAN".

[23]	3GPP TS 43.059: "Functional stage 2 description of Location Services in GERAN".

[24]	3GPP TR 23.891: "Evaluation of LCS Control Plane Solutions for EPS".

[25]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[26]	3GPP TS 24.171: "Control Plane Location Services (LCS) procedures in the Evolved Packet System (EPS)".

[27]	3GPP TS 29.171: "Location Services (LCS); LCS Application Protocol (LCS-AP) between the Mobile Management Entity (MME) and Evolved Serving Mobile Location Centre (E-SMLC); SLs interface".

[28]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[29]	IEEE 802.11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[30]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[31]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[32]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical channels and modulation".

[33]	RTCM 10403.3, RTCM Recommended Standards for Differential GNSS Services (v.3.3), October 7, 2016.

[34]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[35]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, August 2017.

[36]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

----------------------------------------------------End of 1st change ---------------------------------------------

--------------------------------------------------- Start of 2nd change ---------------------------------------------

4.3	Standard UE Positioning Methods

The standard positioning methods supported for E-UTRAN access are:

-	network-assisted GNSS methods;

-	downlink positioning;

-	enhanced cell ID method;

-	uplink positioning;

-	WLAN method;

-	Bluetooth method;

-	Terrestrial Beacon System method;

-	Sensor based methods:

-	Barometric Pressure Sensor;

-	Motion sensor.

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted/E-SMLC-based, eNB-assisted, and LMU-assisted/E-SMLC-based versions. Table 4.3-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, E-SMLC-based | eNB- assisted | LMU-assisted/ E-SMLC-based | SUPL Note 3 |
| --- | --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | No | Yes (UE-based and UE-assisted) |
| Downlink Note1 | No | Yes | No | No | Yes (UE-assisted) |
| E-CID | No | Yes | Yes | No | Yes (UE-assisted) |
| Uplink | No | No | No | Yes | No |
| Sensor | Yes | Yes | No | No | No |
| WLAN | Yes | Yes | No | No | Yes |
| Bluetooth | No | Yes | No | No | No |
| TBS Note 2 | Yes | Yes | No | No | Yes (MBS) |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only for TBS positioning based on MBS signals. NOTE 3: This shows whether the positioning method is supported by SUPL ULP [18]. |  |  |  |  |  |

Sensor, WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding clauses.

------------------------------------------------End of 2nd change -------------------------------------------------

------------------------------------------------Start of 3rd change ------------------------------------------------

B.1	SUPL 2.0 Positioning Methods and Positioning Protocols

The following table shows how the 3GPP positioning protocols are supported in SUPL 2.0.

Table B.1-1: SUPL support of positioning methods

| Positioning Protocol: | RRLP (GSM/GPRS/WCDMA/ LTE/WLAN/WiMAX) | RRC (WCDMA) | LPP (LTE) |
| --- | --- | --- | --- |
| Positioning Method: |  |  |  |
| A-GPS (A-GANSS) SET Assisted |  |  |  |
| A-GPS (A-GANSS) SET Based |  |  |  |
| Autonomous GPS/GANSS |  |  |  |
| Enhanced Cell ID |  |  |  |
| Enhanced Observed Time Difference (E-OTD) | (GSM only) | NA | NA |
| Observed Time Difference of Arrival (OTDOA) NOTE 1 | NA |  |  |
| Sensor | NA | NOTE 3 |  |
| WLAN | NA |  |  |
| Bluetooth | NA |  |  |
| TBS NOTE 2 | NA |  |  |
| NOTE 1: This includes TBS positioning based on PRS signals, which is only supported in LPP (LTE). NOTE 2: TBS positioning based on MBS signals. NOTE 3: Only barometric pressure sensor is supported. |  |  |  |

Note:	What is referred to in the SUPL specifications as "Enhanced Cell ID is a UE-Assisted positioning mode where the neighbouring cell measurements are carried at the SUPL layer (in the SUPL_POS_INIT for example). For LTE, the ASN.1 container for this mode is defined as follows:

LteCellInformation ::= SEQUENCE {

cellGlobalIdEUTRA 		CellGlobalIdEUTRA,

physCellId				PhysCellId,

trackingAreaCode 		TrackingAreaCode,

rsrpResult 				RSRP-Range 		OPTIONAL,

rsrqResult 				RSRQ-Range 		OPTIONAL,

ta 						INTEGER(0..1282) OPTIONAL, -- Currently used Timing Advance value (N_TA/16 as per [3GPP 36.213])

measResultListEUTRA 	MeasResultListEUTRA	 OPTIONAL, --Neighbour measurements

...,

earfcn 					INTEGER(0..65535)	 OPTIONAL, -- see Table 37

earfcn-ext 				INTEGER (65536..262143) OPTIONAL, -- see Table 37

rsrpResult-ext 		RSRP-Range-Ext 		OPTIONAL,

rsrqResult-ext 		RSRQ-Range-Ext 		OPTIONAL,

rs-sinrResult 		RS-SINR-Range 	OPTIONAL,

servingInformation5G 	ServingInformation5G 	OPTIONAL

}

MeasResultListEUTRA ::= SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::= SEQUENCE {

physCellId 		PhysCellId,

cgi-Info SEQUENCE {

cellGlobalId 		CellGlobalIdEUTRA,

trackingAreaCode 	TrackingAreaCode

} OPTIONAL,

measResult SEQUENCE {

rsrpResult RSRP-Range 		OPTIONAL, -- Mapping to measured values

rsrqResult RSRQ-Range 		OPTIONAL, -- in 3GPP TS 36.133

...,

earfcn INTEGER(0..65535) 	OPTIONAL, -- see Table 37

earfcn-ext INTEGER (65536..262143) 		OPTIONAL, -- see Table 37

rsrpResult-ext RSRP-Range-Ext 		OPTIONAL,

rsrqResult-ext RSRQ-Range-Ext 		OPTIONAL,

rs-sinrResult RS-SINR-Range 		OPTIONAL,

neighbourInformation5G NeighbourInformation5G OPTIONAL

}

}

It should be noted that in addition to the container provided by SUPL itself, any E-CID positioning methods defined within LPP proper can be supported in SUPL, via tunneling LPP as shown in this annex (in the same manner that A-GNSS and OTDOA are supported).

------------------------------------------------End of 3rd change -------------------------------------------------

END OF CHANGE